3GPP TSG RAN WG1 Meeting #122	R1-2505274

Bengaluru, India, August 25th – 29th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on improvement of SRS capacity and coverage

Agenda Item:	10.2.1

Document for:	Discussion and Decision

Introduction

In RAN#108 meeting, a new work item on MIMO evolution in phase 6 was approved in [1]. As listed below, the enhancements on SRS capacity and coverage is one of the objectives for this WI:

- On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

In this contribution, we present our views to fulfill the enhancements for SRS capacity and coverage in NR MIMO Phase 6.

Multiple frequency-domain starting positions of SRS repetition with R>1 symbols for RPFS

According to the current specifications, when RB-level partial frequency sounding is configured for a given SRS repetition with R>1 consecutive symbols, the same frequency domain starting position, i.e. noffsetRPFS=NSCRB mSRS, BSRS kF+khopmod PFPF, is specified to all of R>1 consecutive symbols of the SRS repetition.

Given that the sounding bandwidth for each SRS repetition within each SRS FH for RPFS is mSRS, BSRSPF RB(s), it needs to be clarified first that the granularity of frequency-domain starting position for a given SRS repetition with R>1 symbols is in RB-level.

Proposal 1: Support to enable different frequency domain starting points of SRS repetition with R>1 OFDM symbols in RB-level.

In order to facilitate multiple frequency-domain starting positions for SRS repetition with R>1 consecutive symbols within each SRS FH for RPFS, the legacy mechanism for determining the location of mSRS, BSRSPF RB(s) for a given SRS repetition within one SRS FH period can be reused, e.g., which was determined by the values of kF, khop and PF. More precisely, the following three parameters can be introduced for a given SRS repetition with R>1 consecutive symbols to achieve this purpose:

- R': A subset of R' consecutive symbol(s) of the given SRS repetition with R symbols that have the same frequency-domain starting position

  - Where, RR' is an integer value that is smaller than or equal to PF, i.e., RR'∈2, 4. More precisely,

    - RR'=2, if PF=2

    - RR'=2 or 4, if PF=4

- rF: Starting RB index of the first set of R' consecutive symbols for the given SRS repetition,

  - Where, rF∈0, 1, …, RR'−1.

- rhop: Hopping RB index of other set(s) of R' consecutive symbols for the given SRS repetition,

  - where rhop∈0, 1, …, RR'−1.

  - For each set of R' consecutive symbol(s) for the given SRS repetition, the hopping pattern of rhop is given as follows:

    - For RR'=2, rhop= 0, 1

    - For RR'=4, rhop= 0, 2, 1, 3

Consequently, the resource allocation of multiple frequency-domain starting positions for SRS repetition with R = 4 symbols and RR'∈1, 2, 4 in case of CSRS=4 (aka mSRS, 0=4, mSRS, BSRS=4, b'=bhopBSRSNb'=4), PF=4 is illustrated in the following Figure 1.

- R'=4, RR'=1					b) R'=2, RR'=2				c) R'=1, RR'=4

Figure 1	Multiple frequency-domain starting positions for SRS repetition with R = 4 symbols and RR'∈1, 2, 4 in case of CSRS=4 (aka mSRS, 0=4, mSRS, BSRS=4, b'=bhopBSRSNb'=4), PF=4

For a harmonized approach, we have the following proposal to facilitate multiple frequency-domain starting positions for SRS repetition with R>1 consecutive symbols within each SRS FH for RPFS.

Proposal 2: For a given SRS repetition with R>1 consecutive symbols, support to specify the following parameters to facilitate multiple frequency-domain starting positions:

- A subset of R' consecutive symbol(s) of the given SRS repetition that have the same frequency-domain starting position

  - Where, RR' is an integer value that is smaller than or equal to PF, i.e., RR'∈2, 4. More precisely,

    - RR'=2, if PF=2

    - RR'=2 or 4, if PF=4

- Starting RB index rF of the first set of R' consecutive symbols for the given SRS repetition

  - Where, rF∈0, 1, …, RR'−1.

- Hopping RB index rℎop of each set of R' consecutive symbols for the given SRS repetition

  - Where rℎop∈0, 1, …, RR'−1.

  - For each set of R' consecutive symbol(s) for the given SRS repetition, the hopping pattern of rℎop is given as follows:

    - For RR'=2, rℎop= 0, 1

    - For RR'=4, rℎop= 0, 2, 1, 3

SRS transmission across two adjacent S+U slots

Time-domain resource allocation related parameters

According to the current specifications, a set of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) is configured for a given SRS resource in a slot, where the index of each SRS symbol was specified as l'=0, 1, …, NsymbSRS−1.

In order to facilitate SRS transmission across two adjacent S+U slots, the following two options on time-domain resource allocation related parameters configuration can be taken into consideration:

- Option-1: One set of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) is configured to the SRS resource transmitted across two adjacent S+U slots

  - For the number of consecutive OFDM symbols NsymbSRS as well as the repetition factor R of the SRS resource that configured by nrofSymbols and repetitionFactor respectively, the following candidate values need to be supported:

    - NsymbSRS=16, R∈1, 2, 4, 8, 16 when two UL symbols allocated in the tail of S slot

    - NsymbSRS=18, R∈1, 2, 3, 6, 9, 18 when four UL symbols allocated in the tail of S slot

  - For the index of each SRS symbol l', it is the same as the legacy, i.e., l'=0, 1, …, NsymbSRS−1.

  - For time-domain starting position l0 of the SRS resource, it still can be determined by the legacy formula l0=Nsymbslot−1−loffset. In what follows, SRS symbols should be counted backwards from the end of the S slot in case of SRS transmission across two adjacent S+U slots, i.e.:

    - loffset∈0, 1 if NsymbSRS=16 in response to two UL symbols allocated in the tail of S slot.

    - loffset∈0, 1, 2, 3 if NsymbSRS=18 in response to four UL symbols allocated in the tail of S slot.

- Option-2: Two sets of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) need to be configured for the SRS resource transmitted in S slot and U slot, respectively.

  - Although the legacy candidate values of NsymbSRS, R and loffset can be reused in this way, the following aspects need to be considered in addition:

    - Firstly, when SRS repetition is enabled, the configured value of R should be the same between S slot and U slot. Furthermore, the value of R can be equal to or larger than that of NsymbSRS in S slot or U slot, e.g., the SRS repetition across two adjacent S+U slots.

    - Secondly, in order to keep the consistency and continuity of SRS frequency hopping pattern across two adjacent S+U slots (which is counted by the number of SRS counter nSRS and further determined by the index of SRS symbol l'in legacy), it should be that l'=0, 1, …, Nsymb,  SSRS−1 in S slot and l'=Nsymb, SSRS, Nsymb, SSRS+1, …, Nsymb, SSRS+Nsymb,USRS−1 in U slot. Where, Nsymb, SSRS and Nsymb, USRS are the number of consecutive SRS symbols configured in S slot U slot, respectively.

In light of the above, we have the following proposal to facilitate SRS transmission across two adjacent S+U slots.

Proposal 3: For the time-domain resource allocation related parameters configuration of SRS transmission across two adjacent S+U slots, support to down-select one of the following two options:

- Option-1: One set of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) is configured to the SRS resource transmitted across two adjacent S+U slots

  - For the number of consecutive OFDM symbols NsymbSRS as well as the repetition factor R of the SRS resource,

    - NsymbSRS=16, R∈1, 2, 4, 8, 16 when two UL symbols allocated in the tail of S slot

    - NsymbSRS=18, R∈1, 2, 3, 6, 9, 18 when four UL symbols allocated in the tail of S slot

  - For the index of each SRS symbol l', it is same to the legacy, i.e., l'=0, 1, …, NsymbSRS−1.

  - For time-domain starting position l0 of the SRS resource,

    - loffset∈0, 1 if NsymbSRS=16 in response to two UL symbols allocated in the tail of S slot.

    - loffset∈0, 1, 2, 3 if NsymbSRS=18 in response to four UL symbols allocated in the tail of S slot.

- Option-2: Two sets of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) are configured for the SRS resource transmitted in S slot and U slot, respectively.

  - The configured value of R should be same between S slot and U slot, and the value of R can be equal to or larger than that of NsymbSRS in S slot or U slot.

  - For the index of each SRS symbol l', it should be that l'=0, 1, …, Nsymb,  SSRS−1 in S slot and l'=Nsymb, SSRS, Nsymb, SSRS+1, …, Nsymb, SSRS+Nsymb,USRS−1 in U slot.

    - Note: Nsymb, SSRS and Nsymb, USRS denoted as the number of consecutive SRS symbols configured in S slot U slot, respectively.

Determination of slot-level offset for AP-SRS

According to the current specification (as per the following highlighted excerpts in TS 38.214), for AP-SRS, the same slot-level offset is determined for all SRS resources within the SRS resource set, i.e., slot offset k that configured by slotOffset and available slot offset t that configured by availableSlotOffset. By comparison, for P/SP SRS, same slot-level periodicity but different slot-level offsets can be configured for different SRS resources within the SRS resource set, i.e., slot-level offset that configured by periodicityAndOffset-p or periodicityAndOffset-sp.

- TS 38.214

6.2.1 UE sounding procedure <Unrelated parts are omitted> The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource. <Unrelated parts are omitted> - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset, can be configured by the higher layer parameter availableSlotOffsetList. The parameter availableSlotOffsetList can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource. <Unrelated parts are omitted>

As one case depicted in Figure 2, SRS resource 1 is transmitted across S slot n+k (where assumed that SFI = 32) and U slot n+k+1 (where assumed that SFI = 1) while SRS resource 2 is transmitted in U slot n+k+1 only, where SRS resource 1 and SRS resource 2 are configured within the same SRS resource set and scheduled in response to the DCI to trigger AP-SRS that received in D slot n. In this case, it is intuitive that two different slot-level offsets need to be separately determined for SRS resource 1 and SRS resource 2, which is different from the current rule that specified in TS 38.214 as above.

Figure 2	Slot-level offset for the case of SRS transmission across two adjacent S+U slots

In order to address this issue, the following two options can be taken into consideration:

- Option-1: To configure the current slot-level offset (i.e., slot offset k and available slot offset t) per SRS resource for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slot.

- Option-2: Introduce a newly additional slot offset (denoted as t') per SRS resource on top of slot offset k and available slot offset t that configured for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slot.

In light of the above, we have the following proposal to determine the slot-level offset for AP-SRS in case of SRS transmission across two adjacent S+U slots.

Proposal 4: For the slot-level offset determination of AP-SRS in case of SRS transmission across two adjacent S+U slots, support to down-select one of the following two options:

- Option-1: To configure the current slot-level offset (i.e., slot offset k and available slot offset t) per SRS resource for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slots.

- Option-2: Introduce a newly additional slot offset (denoted as t') per SRS resource on top of slot offset k and available slot offset t that is configured for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slot.

Besides, for the available slot of AP-SRS, according to the current specification (as per the following highlighted excerpts in TS 38.214), it can be noted that the available slot is a single slot that specified for all the SRS resources in a resource set.

- TS 38.214

6.2.1 UE sounding procedure <Unrelated parts are omitted> - An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. From the first symbol carrying the SRS request DCI to the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of available slot. <Unrelated parts are omitted>

When it comes to the case of SRS transmission across S+U slots, it is natural that the terminology of “available slot” needs to be re-interpreted. Consequently, the following three options can be taken into consideration:

- Option-1: Available slot is determined per SRS resource

  - An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources (rather than all the SRS resources) in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

    - In this way, the available slot is independently determined to each of SRS resources. Consequently, if one SRS resource cannot be transmitted in the slot, the slot still can be available to other SRS resources.

- Option-2: Available slot is determined per slot

  - An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resources with the same available slot offset value in the SRS resource set (rather than all the SRS resources in the resource set) and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

    - In this way, the available slot is respectively determined to the subset of SRS resources with the same available slot offset value. Consequently, if at least one of the subset of SRS resources cannot be transmitted in the slot, then the slot cannot be available to other SRS resources in the subset either.

- Option-3: Available slot is determined per SRS resource set

  - The available slot(s) can include one or more slot(s) satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

    - In this way, the one or more available slot(s) should be jointly determined to all the SRS resources in the SRS resource set. Consequently, if at least one SRS resource cannot be transmitted in the one or more slot(s), then the one or more slot(s) cannot be available to all the other SRS resources in the set either..

Among the above three options, it can be noted that Option-1 provides the best flexibility of AP-SRS transmission, while Option-3 is the most aligned with the legacy definition of “available slot”. For Option-2, it can be the trade-off between SRS flexibility and specification impact.

In light of the above elaboration, we have the following proposal to re-interpret the terminology of “available slot” for AP-SRS in case of SRS transmission across two adjacent S+U slots.

Proposal 5: For a given AP-SRS resource set in case of SRS transmission across two adjacent S+U slots, support to down-select one of the following three options to re-interpret the terminology of “available slot”:

- Option-1: Available slot is determined per SRS resource

  - An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

- Option-2: Available slot is determined per slot

  - An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resource(s) with same available slot offset value in the SRS resource set, and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

- Option-3: Available slot is determined per SRS resource set

  - The available slot(s) can include one or more slot(s) satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

Definition of SRS transmission occasion

According to the current specifications (as per the highlighted part in TS 38.214 as follows), the transmission power of SRS transmission, it was specified and counted based on the definition of SRS transmission occasion.

- TS 38.213

- 7 Uplink Power control <Unrelated parts are omitted> A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214]. <Unrelated parts are omitted> ### 7.3.1 UE behaviour If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) of each SRS resource in SRS transmission occasion i as [dBm] <Unrelated parts are omitted>

Consequently, for SRS transmission across two adjacent S+U slots, it is natural that the SRS transmission power needs to take the consecutive SRS symbols allocated in both S slot and U slot into account. In this sense, the transmission occasion for SRS transmission across two adjacent S+U slots should be defined by the total of consecutive SRS symbols allocated in S slot and U slot together, hence we have the following proposal.

Proposal 6: Support to specify that the transmission occasion for SRS transmission across two adjacent S+U slots should be defined by the total of consecutive SRS symbols allocated in S slot and U slot together.

Requirements of phase continuity for SRS transmission

As stated in the WID, the phase continuity for SRS transmission may need to be justified particularly to facilitate the enhancements of SRS transmission in Rel-20, in which the applicable conditions and requirements for DMRS bundling should be retained as much as possible. As in the current specifications, several events were specified in response to which cause power consistency and phase continuity not to be maintained for DMRS bundling within the nominal TDW.

Similarly, such kind of events which cause the phase continuity not to be maintained for SRS transmission across two adjacent S+U slots or SRS repetition symbols with different frequency-domain starting positions for RPFS can be justified accordingly. Nevertheless, in procedure-wise, it seems to be proper to figure this out after the fundamental design on the enhancements of SRS transmission in Rel-20 is fulfilled.

Proposal 7: Support to further justify whether/how to specify events which cause phase continuity not to be maintained for the enhancements of SRS transmission in Rel-20.

Conclusion

In this contribution, we present our views to fulfill the enhancements for SRS capacity and coverage in NR MIMO Phase 6 with the following proposals.

Proposal 1: Support to enable different frequency domain starting points of SRS repetition with R>1 OFDM symbols in RB-level.

Proposal 2: For a given SRS repetition with R>1 consecutive symbols, support to specify the following parameters to facilitate multiple frequency-domain starting positions:

- A subset of R' consecutive symbol(s) of the given SRS repetition that have the same frequency-domain starting position

  - Where, RR' is an integer value that is smaller than or equal to PF, i.e., RR'∈2, 4. More precisely,

    - RR'=2, if PF=2

    - RR'=2 or 4, if PF=4

- Starting RB index rF of the first set of R' consecutive symbols for the given SRS repetition

  - Where, rF∈0, 1, …, RR'−1.

- Hopping RB index rℎop of each set of R' consecutive symbols for the given SRS repetition

  - Where rℎop∈0, 1, …, RR'−1.

  - For each set of R' consecutive symbol(s) for the given SRS repetition, the hopping pattern of rℎop is given as follows:

    - For RR'=2, rℎop= 0, 1

    - For RR'=4, rℎop= 0, 2, 1, 3

Proposal 3: For the time-domain resource allocation related parameters configuration of SRS transmission across two adjacent S+U slots, support to down-select one of the following two options:

- Option-1: One set of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) is configured to the SRS resource transmitted across two adjacent S+U slots

  - For the number of consecutive OFDM symbols NsymbSRS as well as the repetition factor R of the SRS resource,

    - NsymbSRS=16, R∈1, 2, 4, 8, 16 when two UL symbols allocated in the tail of S slot

    - NsymbSRS=18, R∈1, 2, 3, 6, 9, 18 when four UL symbols allocated in the tail of S slot

  - For the index of each SRS symbol l', it is same to the legacy, i.e., l'=0, 1, …, NsymbSRS−1.

  - For time-domain starting position l0 of the SRS resource,

    - loffset∈0, 1 if NsymbSRS=16 in response to two UL symbols allocated in the tail of S slot.

    - loffset∈0, 1, 2, 3 if NsymbSRS=18 in response to four UL symbols allocated in the tail of S slot.

- Option-2: Two sets of time-domain resource allocation related parameters (i.e., NsymbSRS, R and loffset) are configured for the SRS resource transmitted in S slot and U slot, respectively.

  - The configured value of R should be same between S slot and U slot, and the value of R can be equal to or larger than that of NsymbSRS in S slot or U slot.

  - For the index of each SRS symbol l', it should be that l'=0, 1, …, Nsymb,  SSRS−1 in S slot and l'=Nsymb, SSRS, Nsymb, SSRS+1, …, Nsymb, SSRS+Nsymb,USRS−1 in U slot.

    - Note: Nsymb, SSRS and Nsymb, USRS denoted as the number of consecutive SRS symbols configured in S slot U slot, respectively.

Proposal 4: For the slot-level offset determination of AP-SRS in case of SRS transmission across two adjacent S+U slots, support to down-select one of the following two options:

- Option-1: To configure the current slot-level offset (i.e., slot offset k and available slot offset t) per SRS resource for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slots.

- Option-2: Introduce a newly additional slot offset (denoted as t') per SRS resource on top of slot offset k and available slot offset t that is configured for a given aperiodic SRS resource set in case of SRS transmission across two adjacent S+U slot.

Proposal 5: For a given AP-SRS resource set in case of SRS transmission across two adjacent S+U slots, support to down-select one of the following three options to re-interpret the terminology of “available slot”:

- Option-1: Available slot is determined per SRS resource

  - An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for at least one of the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

- Option-2: Available slot is determined per slot

  - An available slot is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for a subset of SRS resource(s) with same available slot offset value in the SRS resource set, and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

- Option-3: Available slot is determined per SRS resource set

  - The available slot(s) can include one or more slot(s) satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set.

Proposal 6: Support to specify that the transmission occasion for SRS transmission across two adjacent S+U slots should be defined by the total of consecutive SRS symbols allocated in S slot and U slot together.

Proposal 7: Support to further justify whether/how to specify events which cause phase continuity not to be maintained for the enhancements of SRS transmission in Rel-20.

References

RP-251856, New WID: NR MIMO Phase 6, Moderator (Samsung)