3GPP TSG RAN WG1 #122	        	R1-2505291

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:					    11.5

Source:	Sony

Title:	Consideration on 6GR Energy Efficiency

Document for:	Discussion

Introduction

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

According to the SID of 6GR [1], energy efficiency and energy saving: both for network and device would be studied.

Additionally, according to the agenda shared by RAN1 chair [2], it would be discussed about NW energy saving, UE power saving, and joint mechanisms taking both NW and UE into account for power saving targeting to categorize the proposals by RAN1#123.

In this contribution, our considerations on energy efficiency on 6GR are discussed.

Discussion

General Principle of 6GR

To maximize the energy efficiency of the 6GR system, optimization from both the perspectives of network energy saving (NES) and UE power saving (UEPS) is essential.

To achieve NES and UEPS, it is necessary to minimize the opportunities for signal transmission and reception as much as possible in order to mitigate energy consumption by networks and devices. However, reducing the transmission opportunities of signals from the network for the purpose of NES may lead to additional power consumption at the UE side due to extra processing, e.g., transmission of wake-up signals or increasing of monitoring opportunities to receive the reduced signals. Similarly, reducing the transmission opportunities of signals from the UE for the purpose of UEPS may lead in additional energy consumption at the network side.

In other words, NES and UEPS are in a trade-off relationship, making it difficult to achieve both simultaneously. Optimization of one may affect the performance of the other.

To appropriately manage such trade-offs, it is necessary to design with consideration of the differences in requirements and capabilities depending on the use case, network deployment, and UE type. For example, in terms of UE types, mobile terminals (e.g., IoT devices, smartphones, and XR devices) have limited battery capacity, making power saving at the UE side a critical factor. On the other hand, connected cars are equipped with sufficient battery capacity, and the power consumed by communication is negligible compared to other power consumption, making UE power saving less important. Similarly, fixed wireless access (FWA), Customer Premises Equipment (CPE), and industrial robots typically don’t have limited power, so the power consumed by communication may not be a major concern from the device’s communication lifetime

From the network perspective, in urban areas with high cell density, the network power consumption per unit area is large, making network energy saving important. Conversely, in rural areas or NTN (Non-Terrestrial Networks), where a single base station covers a wide area, the network energy consumption per unit area is low, and network energy saving is less critical.

Therefore, it is necessary to define the differences in NES and UEPS requirement levels according to the use case, network deployment, and UE type, and to study optimization methods for maximizing energy saving gain between the network and UE based on these definitions. This approach will contribute to realizing an energy-efficient 6GR system.

Proposal 1: Optimization methods for energy efficiency between the network and UE should be studied based on the differences in NES and UEPS requirement levels according to the use case, network deployment, and UE type.

Joint Mechanisms between NW and UE Energy Saving on 6GR

Based on the discussion in section 2.1, it is quite clear that one solution may not solve both NES and UEPS requirements. The system may operate in more than one state depending on variables like UE type and its operating conditions, cell deployment type and energy source, and MNO policy. Figure 1 lists a few such possible states/modes of operation and transition between these states/modes. These are not actual states to be implemented in the specifications but should be used as guiding or design principles. For example, NES state may allow network energy saving and might do at the expense of slightly increased UE power consumption. UE power saving may result in slightly increased energy consumption on the network side. Joint power saving state should be the ultimate goal allowing the best outcome for both NES and UEPS. But this may not be achievable under all conditions. There could be triggers to switch between these states or mode of operations.

Proposal 2: Capture Figure 1 in the TR exhibiting different energy saving and power saving states and transition between these states.

Fig.1 Configurable network and UE energy saving modes

Network Energy Saving on 6GR

Design Principle of Network Energy Saving

To achieve energy saving in the network, it is essential to increase opportunities for base stations to enter sleep mode. To this end, it is necessary to reduce the transmission occasions of always-on signals. In NR, the SSB (Synchronization Signal Block) and SIB1 (System Information Block Type 1) are representative examples of the always-on signals. While these signals are essential for initial access, their constant transmission prevents base stations from staying in sleep mode for a long duration.

In 6GR, revisiting the structure and transmission periodicity of synchronization signal and minimum system information can contribute to network energy saving. However, such changes directly affect the initial access procedure and therefore should be considered in the initial specification phase. In particular, the synchronization signal and minimum system information are physical signals/channels provided to idle mode UEs. If changes are made after the initial specification is finalized, idle mode UEs based on the initial specification will not be able to recognize these changes, resulting in impacts on existing UEs.

Therefore, the design of synchronization signal and minimum system information should be addressed in the early stages of 6GR specification development.

Proposal 3: Physical signals/channels related to the initial access procedure, e.g., synchronization signal and minimum system information, should be designed with consideration for network energy saving.

Potential technologies

The transmission scheme of synchronization signal and minimum system information is a key factor in achieving network energy saving. In 5G, on-demand SSB is applicable only to SCell, limiting the energy saving gains to a restricted set of cells. Similarly, on-demand SIB1 is only applicable when assistance from other cells is available (specifically, when Cell A provides the UL WUS configuration to the UE) and is not functional in standalone cell operations.

Furthermore, time-domain SSB adaptation in 5G is limited to connected mode UEs in SCell, and only allows adaptation within legacy SSB periodicities. It is not possible to configure a periodicity longer than the legacy periodicity settings. Due to this limitation, sleep opportunities for cells do not increase, and the energy saving gains remain limited.

Considering these limitations, studying on-demand synchronization signal and on-demand minimum system information transmission in the 6GR specification would allow more cells to operate in an energy-efficient manner, thereby realizing an energy-efficient network.

Additionally, if time-domain adaptation is enabled to allow synchronization signal to be transmitted at longer periodicities for idle mode UEs, with dynamic switching to shorter periodicities as needed, sleep opportunities for cells would increase, enabling the realization of an energy-efficient network.

Proposal 4: On-demand transmission of synchronization signal and minimum system information, as well as time-domain adaptation should be studied in 6GR.

UE Power Saving on 6GR

Design Principle of UE Power Saving

In 6G, similar to other generations of wireless connectivity, different device types are to be considered including IoT, XR, eMBB devices. All these devices are battery operated with different energy storage capacity. In order to support as long-time period operation as possible for all device types, not only the 6GR design needs to be energy efficient, but also it is necessary that 6GR design includes UEPS from the start to guarantee a long battery life-time device and network operation. In our view, UEPS techniques can be categorized to,

- common techniques independent of device type,

- techniques dedicated to device types (and their use-case).

Proposal 5: UE power saving technique to be categorized to i) common mechanisms independent of device type ii) techniques dedicated to device types (and their use-case).

In general, one of the main sources of the energy consumption is to unnecessarily keep the device circuitry ON when there is low or no activity in the uplink or the downlink. It is therefore important to adjust/optimize the UE circuitry operation according to its communication activity. Different power saving techniques have been developed during both 4G/LTE and 5G/NR to address this issue. In our view, several of the power saving mechanisms which were developed during 5G/NR design can be inherited and used in 6GR from the start as common UEPS techniques. There may, however, be needs to improve or adjust these techniques to have them optimized for 6GR design features and new services. There may also be a need to develop new power saving techniques to address computation complexity and power consumption that may be introduced through new 6G services and solutions such as sensing and AI/ML

Below we provide a list of common UEPS mechanisms that can be inherited from NR UE power saving features and be used from day one of 6GR introduction. We then discuss potential additional power saving techniques that needs to be developed/designed dedicated to device types.

Potential technologies

Common UEPS techniques independent of device type

The UEPS mechanism can be done in different domains: time, frequency, space and power domains.

- Low-power WUS with Discontinuous reception (DRX) - This is a power saving technique in both time and power domains. An ultra-low power receiver, in addition to the main receiver, listens to the channel for potential wake-up signal periodically following a discontinuous reception. The main receiver is in sleep state. This technique can reduce the cost of channel monitoring significantly.

- Deep-, light- and micro-sleep RX – This is a power saving technique in power domain. A UE can switch off its circuitry to reduce the UE power consumption when there is no activity on the channel but also adapt its sleep level in accordance with its expected subsequent operation.

- BWP adaptation – 6GR need to support wide bandwidth for eMBB or XR. These devices however do not always need to monitor the entire bandwidth. Through BWP adaption the UE switches to smaller bandwidth suitable for the operation in hand. This feature is a power saving feature in frequency domain.

- Antenna adaptation - UE adapts the number of MIMO layers or number of Tx/Rx antenna (panels) to the traffic to achieve the UE power saving. This is a power saving in space domain.

- Adaption mechanism of RRM measurement activities – For low mobility deployment, UE speed, or favorable level received signal level, the number of measurement samples for a given duration (e.g., measurement period / evaluation period) can be relaxed with negligible impact on accuracy.

Proposal 6: 6GR includes UEPS techniques from 5G/NR from day one, including LP-WUS with discontinuous reception, different levels of RX sleep levels, BWP adaptation, antenna adaptation, adaption mechanism of RRM measurement.

In 5G, the UE reports its preferred mechanisms to the network via UE assistance information message, allowing the NW to adjust the UE configuration based on the reported information. Since the final decision to provide such power saving features is by the NW, the UE radio resources configuration may not always result in optimized UE power consumption reduction. As mentioned above, AI/ML techniques can be applied to make the operations of the above features optimally and autonomously adaptable to the use-case scenario and requirements in hand. It is however necessary to make sure that the computation and signaling introduced for AI/ML do not contribute to an increase of power consumption and is suitable/compatible to different device types, for instance sensor or IoT type devices which have limited energy storage. Therefore, the impact of signaling and computation complexity of AI/ML used for the optimization of power saving features need to be studied.

The UE assistance information for any UE power saving scheme is for UE to provide the assistance information to the network in configuration for UE adaptation to achieve power saving gain. UE assistance information reported by UE for power saving is a UE recommendation and the network shall make the final decision on whether and how to use that information.

Proposal 7: Study use of AI/ML for the optimization of power saving mechanisms and the impact of its corresponding signaling, power consumption and computation complexity on both device and network.

Techniques dedicated to device types (and their use-case)

The device category of the 6G is not decided and yet to be studied. Some type of device categorization is, however, expected with the IoT-type devices at the low tier, eMBB and FWA at mid-tier and XR-type applications at the top-tier. As discussed earlier, depending on the device battery capacity and use-case in hand, there may be need for other UEPS mechanisms suitable for the use-case and device type in hand.  For example, in some IoT use-cases used as tracking devices, performing the full cell search always in all frequency bands leads to drainage of the device battery and reduced communication lifetime e.g. for devices in a container ship. Another example is a XR head mounted, or wearable device used in an immersive reality use-case. In many cases the size of the device is relatively large to able to accommodate a large battery or energy storage component for its demanding communication and computation operations. Dedicated power saving techniques suitable for such a device are needed if the requirement is to reduce the size of a bulky head mounted device and complexity. It is therefore important to study power saving and optimization techniques dedicated to device types and their corresponding use-cases.

Proposal 8: In addition to common UEPS techniques, study power saving and optimization techniques dedicated to device types and their corresponding use-cases.

Conclusions

The following proposals were made in this contribution.

Proposal 1: Optimization methods for energy efficiency between the network and UE should be studied based on the differences in NES and UEPS requirement levels according to the use case, network deployment, and UE type.

Proposal 2: Capture Figure 1 in the TR exhibiting different energy saving and power saving states and transition between these states.

Proposal 3: Physical signals/channels related to the initial access procedure, e.g., synchronization signal and minimum system information, should be designed with consideration for network energy saving.

Proposal 4: On-demand transmission of synchronization signal and minimum system information, as well as time-domain adaptation should be studied in 6GR.

Proposal 5: UE power saving technique to be categorized to i) common mechanisms independent of device type ii) techniques dedicated to device types (and their use-case).

Proposal 6: 6GR includes UEPS techniques from 5G/NR from day one, including LP-WUS with discontinuous reception, different levels of RX sleep levels, BWP adaptation, antenna adaptation, adaption mechanism of RRM measurement.

Proposal 7: Study use of AI/ML for the optimization of power saving mechanisms and the impact of its corresponding signaling, power consumption and computation complexity on both device and network.

Proposal 8: In addition to common UEPS techniques, study power saving and optimization techniques dedicated to device types and their corresponding use-cases.

References

- RP-251881, “Study on 6G Radio,” 3GPP TSG RAN Meeting #108, Prague, June 9-13, 2025.

- Agenda item by RAN1 chair.