3GPP TSG-RAN WG1 Meeting #122	R1-2505347

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	9.10

Source:	Huawei, HiSilicon

Title:	UE features for IoT-NTN TDD mode

Document for:	Discussion and Decision

Introduction

In RAN1#121 meeting, some agreements about UE features for IoT-NTN TDD mode is achieved [1]. In this contribution, we will continue to discuss and provide an updated list in the Appendix A.

Discussion

There are two highlight parts as the remaining issues in the UE feature list: the need of FDD/TDD differentiation and capability interpretation for mixture of FDD/TDD. For the need of FDD/TDD differentiation, because this feature is for IoT-NTN TDD mode, it’s nature to support TDD only. Therefore, RAN1 should remove the bracket of “TDD only”. Similarly, for the capability interpretation for mixture of FDD/TDD, RAN1 should remove the bracket of “N/A”.

Proposal 1: Support to remove the brackets of “TDD only” and “N/A”.

In RAN1#121 meeting [2], RAN1 achieved the following agreement:

Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation

In Rel-17, the segmented UL transmission (Rel-17 FG2-1c) is mandatory supported for NGSO. Thus, RAN2 did not assign separate IODT bit for it and merged it into ntn-Connectivity-EPC-r17 corresponding to Rel-17 FG2-1b without TDD/FDD differentiation. Considering Rel-17 FG2-1b is prerequisite of Rel-19 FG2-1, it forced a TDD only UE to support segmented UL transmission which is not used according to RAN1 specifications. Therefore, we think it should be clarified that Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG should support segmented UL transmission on IoT NTN TDD band.

Proposal 2: Add a note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG should support segmented UL transmission on IoT NTN TDD band.

Conclusions

The proposals made in this contribution are summarized below:

Proposal 1: Support to remove the brackets of “TDD only” and “N/A”.

Proposal 2: Add a note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG should support segmented UL transmission on IoT NTN TDD band.

- References

[1] R1-2504668, Session Notes of AI 9.15.10, Ad-Hoc Chair (NTT DOCOMO).

[2] Chair’s notes for RAN1 #121, St Julian’s, Malta, May 19th – 23rd, 2025.

# Appendix A

Proposed revision for RAN1 UE features list for IoT-NTN TDD mode after RAN1#121 in [2].

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_TDD | 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH/SIB1-NB transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. 7. Support of NPRACH periodicities of 90ms and 180ms, instead of 40ms and 80ms. | Rel-17 2-1b | Yes | N/A | IoT-NTN TDD mode is not supported | N/A | [TDD only] | [N/A] | Applicable only for 1616-1626.5 MHz MSS allocated band Note: UE supporting band 249 must support this FG Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG must support IoT NTN FDD band Note: Rel-17 2-1b as prerequisite FG doesn’t imply the UE supporting this FG support segmented UL transmission on IoT NTN TDD band. | Optional without capability signalling |