3GPP TSG RAN WG1 #122	R1-2505370

Bengaluru, India, Aug 25th – 29th, 2025

Source:	vivo

Title:	Maintenance on UE-initiated/event-driven beam management

Agenda Item:	8.2.1

Document for:	Discussion and Decision

Introduction

According to the discussion in the previous meetings, the whole procedure of UE-initiated/event-driven beam report, including measurement RS determination, event definition and evaluation, report contents and reporting procedure, is clear, and has been captured in the first version of Rel-19’s RAN1 specification [1][2][3].  In this contribution, we provide our views on the description of UE-initiated/event-driven beam report in the above specification.

Discussion

Details on multiplexing 1-bit indication on PUSCH

Besides collision handling between 1-bit first PUCCH and PUCCH carrying SR/CSI/HARQ-ACK, agreement on collision handling between 1-bit first PUCCH and PUSCH was achieved in the last meeting below [4].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support Option-3 of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

However, details on how to multiplex 1-bit indication in the PUSCH are unclear and the following issues need further discussion.

Issue-1: The determination of the 1-bit indication value and corresponding beta offset value when multiplexed on PUSCH

According to the above agreement, the 1-bit indication is always multiplexed in the PUSCH, regardless that the UE-initiated/event-driven beam report procedure is triggered. Therefore, the value of the 1-bit indication should be further determined to assist the network in identifying whether the corresponding report(s) are triggered. If at least one report associated with the PUCCH resource satisfies the corresponding trigger condition, the 1-bit indication transmitted on the PUSCH is set to 1; else, the 1-bit indication transmitted on the PUSCH is set to 0.

If at least one report associated with the PUCCH resource satisfies the corresponding trigger condition, the 1-bit indication transmitted on the PUSCH is set to 1; else, the 1-bit indication transmitted on the PUSCH is set to 0.

Similar to HARQ-ACK/CSI, the beta offset value of the 1-bit indication can be determined by two modes, i.e., dynamic and semi-static. To achieve it, the most straightforward way is to follow the beta offset value of CSI part I with up to 11 bits or HARQ-ACK with up to 2 bits.

The beta offset value of the 1-bit indication is the same as that for CSI part I with up to 11 bits or HARQ-ACK with up to 2bits.

Issue-2: How to multiplex the 1-bit indication into PUSCH if it is only present on the PUSCH without CG-UCI/UTO-UCI?

As the number of the indication bits is 1, the loss of data transmission caused by puncturing to achieve multiplexing transmission of the 1-bit indication can be negligible. Similar to the case of HARQ-ACK bits up to 2, multiplexing transmission of the 1-bit indication by puncturing is preferred if only the 1-bit indication is present on the PUSCH. In this case, separate coding is applied for the 1-bit indication naturally.

If only the 1-bit first PUCCH is overlapped with PUSCH in the time domain, the 1-bit indication should be multiplexed on the PUSCH by puncturing and coded separately.

Issue-3: Whether to consider the co-existence of the 1-bit indication and other UCI types (e.g. HARQ-ACK, CSI, CG-UCI, UTO-UCI) and if yes, how to multiplex?

For CG-UCI/UTO-UCI, it is carried by CG-PUSCH. As CG-PUSCH and PUCCH resources carrying the 1-bit indication are NW-configured uplink resources, to simplify system design, such channel collision can be avoided by the network. Thus, there is no need to consider the co-existence of the 1-bit indication and CG-UCI/UTO-UCI. For CSI, the uplink resource to carry it can be configured and scheduled flexibly. In addition, both the CSI report and UE-initiated/event-driven beam report have requirements for low latency. Therefore, the collision between the PUCCH resource carrying the 1-bit indication and the PUSCH carrying CSI may not be avoided, and hence how to multiplex CSI and the 1-bit indication on PUSCH needs to be resolved. According to the current specification, CSI is always separately coded and transmitted on PUSCH by rate matching, regardless of which other UCI types (e.g., HARQ-ACK, UTO-UCI, CG-UCI) and CSI are present for transmission on PUSCH.  As a new UCI type, it is the most straightforward for the 1-bit indication to follow the same principle. Hence, when the 1-bit indication and CSI are present for transmission on PUSCH, they are coded separately. Furthermore, CSI is transmitted on PUSCH by rate matching and the 1-bit indication is transmitted on PUSCH by puncturing. For HARQ-ACK, it follows downlink scheduling and reflects the transmission status of PDCCH/PDSCH in time. In some use cases, e.g., URLLC, HARQ-ACK is present for transmission in each slot. Therefore, the co-existence between the 1-bit indication and HARQ-ACK should be considered. To simplify and accelerate the coding procedure, joint coding is preferred for the 1-bit indication and HARQ-ACK. If the number of jointly coded bits for HARQ-ACK and the 1-bit indication is no more than 2, same as HARQ-ACK, the jointly coded bits are transmitted on PUSCH by puncturing. Otherwise, the jointly coded bits are transmitted on PUSCH by rate matching. Based on the above analysis, we summarize the coexistence situations in Table 1.

Co-existence with other UCI types

|  | 1-bit indication | HARQ-ACK | CG-UCI/ UTO-UCI | CSI | Consider or not | Separate/joint coding | Puncture or rate match |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case-1 | ✔ | ✔ | ❌ | ❌ | ✔ | Joint coding | If number of jointly coded bits is no more than 2=>puncture; else, =>rate match |
| Case-2 | ✔ | ❌ | ❌ | ✔ | ✔ | Separate coding | 1-bit indication=> puncture; CSI=>rate match |
| Case-3 | ✔ | ✔ | ❌ | ✔ | ✔ | Joint coding for 1-bit indication and HARQ-ACK; Separate coding for CSI | If number of jointly coded bits is no more than 2=>puncture; else, =>rate match; CSI=>rate match |
| Case-4 | ✔ | ❌ | ✔ | ❌ | ❌ |  |  |
| Case-5 | ✔ | ❌ | ✔ | ✔ | ❌ |  |  |
| Case-6 | ✔ | ✔ | ✔ | ❌ | ❌ |  |  |
| Case-7 | ✔ | ✔ | ✔ | ✔ | ❌ |  |  |

Support co-existence between the 1-bit indication and other UCI types as the following Table.

|  | 1-bit indication | HARQ-ACK | CG-UCI/ UTO-UCI | CSI | Consider or not | Separate/joint coding | Puncture or rate match |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case-1 | ✔ | ✔ | ❌ | ❌ | ✔ | Joint coding | If number of jointly coded bits is no more than 2=>puncture; else, =>rate match |
| Case-2 | ✔ | ❌ | ❌ | ✔ | ✔ | Separate coding | 1-bit indication=> puncture; CSI=>rate match |
| Case-3 | ✔ | ✔ | ❌ | ✔ | ✔ | Joint coding for 1-bit indication and HARQ-ACK; Separate coding for CSI | If number of jointly coded bits is no more than 2=>puncture; else, =>rate match; CSI=>rate match |
| Case-4 | ✔ | ❌ | ✔ | ❌ | ❌ |  |  |
| Case-5 | ✔ | ❌ | ✔ | ✔ | ❌ |  |  |
| Case-6 | ✔ | ✔ | ✔ | ❌ | ❌ |  |  |
| Case-7 | ✔ | ✔ | ✔ | ✔ | ❌ |  |  |

Issue-4: whether to consider the case of more than one indication multiplexing with PUSCH?

When multiple PUCCH resources used to carry the 1-bit indication are overlapped with PUSCH in the time domain, if the multiple indications multiplexed on the PUSCH are allowed, a bit sequence (i.e., bitmap) with a length of N needs to be transmitted on the PUSCH, where N is the number of the PUCCH resources overlapped with the PUSCH. As shown in Figure 1, a bitmap with a length of 4 is required. Furthermore, if the trigger condition for the 1-bit first PUCCH is satisfied, the corresponding bit within the bitmap should be set to 1; else, set 0. Thus, the network could identify which report is triggered and perform subsequent scheduling.

Multiple PUCCH resources used to carry the 1-bit indication overlap with PUSCH

As mentioned above, if considering the case of more than one indication multiplexing with PUSCH, the bit length of indications transmitted on the PUSCH is determined based on the number of PUCCH resources used to carry the trigger indication and overlapped with the PUSCH. In addition, the multiplexing method (i.e., puncture or rate match) is associated with the number of bits. So, many cases need to be considered, leading to complicated system design. Considering it is in the maintenance phase, such a complex case should be avoided. From the other perspective, the case of more than one indication multiplexing with PUSCH should be avoided by network scheduling.

The case of more than one indication multiplexing with PUSCH should be avoided by the network.

TPs

TP on the determination of the CC on which the indicated TCI state is applied

In the previous meetings, the determination of measurement RSs corresponding to the current beam and new beam(s) was discussed for the multi-CC case respectively, and the following agreement was achieved.

RAN1#118bis meeting agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - For new beam measurement, in a CSI report configuration, configure legacy RRC parameter carrier that indicates the CC that the RS resource set associated with the CSI reporting configuration can be found - FFS: Whether the current beam RS and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement. - FFS: Whether the indicated TCI state and new beam RS(s) can be in the same CC or in different CCs, regarding cross-CC beam measurement. RAN#120 meeting agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied. RAN#120bis meeting agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary

- According to the above agreements, the indicated TCI state used to determine the current beam RS is applied on the CC where the new beam RS(s) are or the CC indicated by ServCellIndex. Furthermore, the CC where the new beam RS(s) are is the same as the CC on which CSI-ReportConfig is configured or the CC indicated by carrier. However, according to the current specification [3], if ServCellIndex is not provided, the indicated TCI state used to determine the current beam RS is only determined by the indicated TCI state, which is applied on the CC of CSI-ReportConfig. The case that the indicated TCI state used to determine the current beam RS can be determined by the indicated TCI state, which is applied on the CC indicated by carrier is absent. To supplement this case, the following TP is proposed.

Support the following TP#1.

| Reason for change | In TS38.214 [3], the indicated TCI state is applied to the same CC of CSI-ReportConfig, or the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig. However, according to the above agreements, the CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) can be the same or different. The point that needs clarification is that the CC of the CSI-ReportConfig may not be equal to the CC in which the new beam RS is configured, especially when the RRC parameter carrier is configured in the CSI-ReportConfig. The corresponding text proposal is provided accordingly. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the determination of CC on which the indicated TCI state is applied in the multi-CC case. |
| Consequences if not approved | Previous agreement is not captured in the specification. |
| TS38.214 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. < Unchanged parts are omitted > 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event1’, an event instance is determined if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet, is lower than eventThreshold-Event1. < Unchanged parts are omitted > 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal with the valueOfQ highest L1-RSRP out of the reference signals among the activated TCI states (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH block s QCLed with the reference signals among the activated TCI states (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. < Unchanged parts are omitted > |  |

TP on CSI report configuration indicator within report content

The TP in this section is related to report content. To assist the network in identifying the reported results, UE needs to report a CSI report identifier when multiple CSI report configurations are associated with one PUCCH resource. The CSI report identifier is determined based on the order of the CSI report configuration in the CSI report configurations associated with the same PUCCH resource.  Furthermore, the CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId.

RAN1#120 agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19

However, in the current specification for TS38.214 [3], the CSI report identifier is captured by CSI-ReportConfigId directly.  To reflect the agreement, we propose the following TP.

Support the following TP#2.

| Reason for change | In TS38.214 [3], the CSI report identifier in the beam report format is captured as CSI-ReportConfigID. However, according to the related agreement, the CSI report identifier is determined based on the order of the triggered CSI report in the CSI report configurations associated with the same PUCCH resource. Thus, the specification description is aligned with the agreement only when all CSI report configurations in a CC are associated with the same PUCCH resource. However, it can introduce many restrictions to the network configuration. In addition, the CSI report identifier is captured in TS38.212 [1] as follows: Table 6.3.2.1.2-14: CRI, SSBRI, RSRP, CapabilityIndex, CSI report configuration indicator and Condition met indicator Field; Bitwidth. CRI; log2(KsCSI-RS). SSBRI; log2(KsSSB). RSRP; 7. Differential RSRP; 4. CSI report configuration indicator; log2(KsCSI_config). Condition met indicator; 1. NOTE: KsCSI-RS is the number of CSI-RS resources in the corresponding resource set, KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Index', KsCSI_config is the number of CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR.. Table 6.3.2.1.2-3I: Mapping order of CSI fields of one report for CSI report configuration indicator/CRI/RSRP/Condition met indicator, or CSI report configuration indicator/SSBRI/RSRP/Condition met indicator CSI report number; CSI fields. CSI report #n; CSI report configuration indicator as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #1 as in Table 6.3.2.1.2-14. CRI or SSBRI #2 as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #3 as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #4 as in Table 6.3.2.1.2-14, if reported. RSRP #1 as in Table 6.3.2.1.2-14. Differential RSRP #2 as in Table 6.3.2.1.2-14, if reported. Differential RSRP #3 as in Table 6.3.2.1.2-14, if reported. Differential RSRP #4 as in Table 6.3.2.1.2-14, if reported. Condition met indicator #1 in Table 6.3.2.1.2-14, if reported. Condition met indicator #2 in Table 6.3.2.1.2-14, if reported. Condition met indicator #3 in Table 6.3.2.1.2-14, if reported. Condition met indicator #4 in Table 6.3.2.1.2-14, if reported. RSRP as in Table 6.3.2.1.2-14 for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state if eventType is set to ‘event1’ and enabledCurrentBeamReport is configured; Differentiable RSRP as in Table 6.3.2.1.2-14 for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state if eventType is set to ‘event2’ and if enabledCurrentBeamReport is configured; Differentatial RSRP as in Table 6.3.2.1.2-14 for the reference signal with the valueOfQ highest L1-RSRP out of reference signals among the activated TCI states or the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of refence signals among the activated TCI states if eventType is set to ‘event7’ and if enabledCurrentBeamReport is configured;. Zero padding bits , if needed. NOTE: The CSI report configuration indicator with value indicates the (+1)-th CSI report configuration among the CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR, which are ordered in ascending order of CSI-ReportConfigID. The condition met indicators are reported only when the UE is configured with higher layer parameter PresenceOfConditionMetIndicator. If the reference signal associated with CRI or SSBRI #j satisfies the condition for transmitting UEIRI as defined in Clause 5.2.1.5.4.1 in [6, TS 38.214] when eventDetectionTimeWindowLength is configured, the value of the condition met indicator #j is set to 1; otherwise set to 0.. The above description in TS38212 is aligned with the agreement. Therefore, to align the specification description and agreement, the description of the CSI report identifier in TS38.214 should be revised. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1d, clarify the determination of a CSI report identifier in the report format when multiple CSI report configurations are associated with one PUCCH resource. |
| Consequences if not approved | The spec is not aligned with the agreement. |
| TS38.214 5.2.1.5.4.1d UE Initiated CSI reporting for multiple CSI configurations For a UE configured with multiple CSI-ReportConfig with same higher layer parameters eventType and PUCCHResource, the UE expects - the multiple CSI-ReportConfig to be configured in the same CC, - the multiple CSI-ReportConfig to be associated with a same CSI trigger state if reportTransmissionMode is configured as ‘ModeA’, else - the same configuredPUSCHResourceOfModeB if reportTransmissionMode is configured as ‘ModeB’. The UE reports in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet in a CSI-ReportConfig that satisfies the event. The CSI report includes the corresponding CSI-ReportConfigId CSI report configuration indicator as defined in Table 6.3.2.1.2-14 in [5, TS38.212] and is zero padded to a fixed payload size (when needed), with the fixed payload size given by the maximum payload size among all the multiple CSI-ReportConfig. |  |

TP on multiplexing the 1-bit indication on PUCCH format 2/3/4 carrying CSI

The TP in this section is related to collision handling for the first PUCCH indication for UE-initiated/event-driven beam reporting. In the last meeting, the following multiplexing rule was agreed below.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing a number of L (L>=1) first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ/CSI/SR. - Option-1: Extend the SR field of log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI - The value in the field is based the ascending order of SR ID first and then the ascending order of the PUCCH resource ID associated with the first PUCCHs. - If one of the SRs is a positive LRR, the value of the log2K+L+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+L+1 bits indicates the positive UEIBR. - An all-zero value for the log2K+L+1 bits represents a negative SR and UEIRI value across all K LRR/SRs and/or L UEIRIs.

However, in  TS38.213, the case where one of the UEIBRs is a positive UEIBR is absent when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR. To align with the agreement, the following TP is proposed.

Support the following TP#3.

| Reason for change | In the last meeting, when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR, the following multiplexing rule was agreed. - Extend the SR field of log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI - The value in the field is based the ascending order of SR ID first and then the ascending order of the PUCCH resource ID associated with the first PUCCHs. - If one of the SRs is a positive LRR, the value of the log2K+L+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+L+1 bits indicates the positive UEIBR. - An all-zero value for the log2K+L+1 bits represents a negative SR and UEIRI value across all K LRR/SRs and/or L UEIRIs. However, in the current specification, only the green highlighted part is captured when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR. To align with the agreement, the yellow-highlighted part should be added. |
| --- | --- |
| Summary of change | In TS38.213 section 9.2.5.1, add the case of no positive LRR and at least one positive UEIBRs to the multiplexing rule of PUCCH with UEIBR. |
| Consequences if not approved | UE behavior is unclear for the case of no positive LRR and at least one positive UEIBRs, when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR.. |
| TS38.213 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH < Unchanged parts are omitted > If a UE would transmit a PUCCH with OACK HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in clauses 9.2.1 and 9.2.3, log2K+1 bits representing a negative or positive SR/UEIRI, in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell for SR, and then in ascending order of the values of pucch-ResourceId for UEIRI, are appended to the HARQ-ACK information bits and the UE transmits the combined OUCI=OACK+log2K+1 UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR; else, if one of the UEIRIs is a positive UEIRI, the value of the log2K+1 bits indicates the positive UEIRI. An all-zero value for the log2K+1 bits represents a negative SR/UEIRI value across all K SRs and UEIRIs. If a UE would transmit a PUCCH with CSI report bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, log2K+1 bits representing corresponding negative or positive SR/UEIRI, in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell for SR, and then in ascending order of the values of pucch-ResourceId for UEIRI, are prepended to the CSI information bits as described in clause 9.2.5.2 and the UE transmits a PUCCH with the combined OUCI=log2K+1+OCSI UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR; else, if one of the UEIRIs is a positive UEIRI, the value of the log2K+1 bits indicates the positive UEIRI. An all-zero value for the log2K+1 bits represents a negative SR/UEIRI value across all K SRs and UEIRIs. If a UE transmits a PUCCH with OACK HARQ-ACK information bits, OSR=log2K+1 SR/UEIRI bits, and OCRC CRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs MRB,minPUCCH for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs provided by nrofPRBs in PUCCH-format2 or nrofPRBs in PUCCH-format3 and starts from the first PRB from the number of PRBs, that results to OACK+OSR+OCRC≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r and, if MRBPUCCH>1, OACK+OSR+OCRC>MRB,minPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, where Nsc,ctrlRB, Nsymb-UCIPUCCH, Qm, and r are defined in clause 9.2.5.2. For PUCCH format 3, if MRB,minPUCCH is not equal 2α2⋅3α3⋅5α5 according to [4, TS 38.211], MRB,minPUCCH is increased to the nearest allowed value of nrofPRBs [12, TS 38.331]. If OACK+OSR+OCRC>MRBPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the PUCCH over the MRBPUCCH PRBs. If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation and transmits a PUCCH with OACK HARQ-ACK information bits, OSR=log2K+1 SR/UEIRI bits, and OCRC CRC bits using PUCCH format 2 or PUCCH format 3, the UE transmits the PUCCH over the first interlace if OACK+OSR+OCRC≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r; otherwise, if the UE is provided a second interlace by interlace1 in PUCCH-format2 or PUCCH-format3, the UE transmits the PUCCH over the first and second interlaces. < Unchanged parts are omitted > |  |

TP on the intra-UE multiplexing/prioritization rules of UEI-BR for Mode A

In the last meeting, collision handling between CSI reports was discussed and following agreement was achieved.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding priority rules for CSI report multiplexing/dropping, UEI beam report for both mode-A and mode-B is prioritized over Semi-persistent CSI reports on PUCCH and Periodic CSI reports on PUCCH - UEI beam report for mode-A > Aperiodic CSI report > UEI beam report (for mode-B) > Semi-persistent CSI reports on PUSCH Note-1: The intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. Note-2: How to capture the above is up to Editor.

According to the agreement, the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. However, in the current TS38.213, the corresponding description is absent. Hence, the UE behavior is not clear when PUSCH carrying UEI-BR for Mode A overlaps with other uplink channels/RSs. Therefore, we propose the following TP.

Support the following TP#4.

| Reason for change | In the last meeting, reusing the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH for UEI-BR for Mode A was agreed. However, it is not captured in the current specification. |
| --- | --- |
| Summary of change | In TS38.213 section 9, clarify the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. |
| Consequences if not approved | UE behavior is not clear when PUSCH carrying UEI-BR for Mode A overlaps with other uplink channels/RSs. |
| TS38.213 9 UE procedure for reporting control information < Unchanged parts are omitted > For the remaining of this clause, when a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs, and is provided coresetPoolIndex with a value of 1 for second CORESETs, on active DL BWPs of serving cells, and - is provided sTx-2Panel the UE separately determines and resolves time overlapping among first PUSCH transmissions that use respective first spatial domain filters corresponding to first TCI-State or TCI-UL-State associated with the first CORESETs, and among second PUSCH transmissions that use respective second spatial domain filters corresponding to second TCI-State or TCI-UL-State associated with the second CORESETs. For the remaining of this clause, reference to PUSCH with semi-persistent CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeB’ in the CSI report configuration [6, TS 38.214]. For the remaining of this clause, reference to PUSCH with aperiodic CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeA’ in the CSI report configuration [6, TS 38.214]. For the remaining of this clause, for a UE operating on an NTN serving cell, the timeline conditions for resolving time overlapping between a PUSCH transmission with repetitions in an OCC group [6, TS 38.214] and PUCCH transmissions are applicable with respect to the first repetition of the PUSCH transmission in the OCC group - if the UE would multiplex UCI from the PUCCH transmissions in the PUSCH, the UE multiplexes the UCI in all repetitions of the PUSCH transmission in the OCC group - if the UE would not transmit a repetition of the PUSCH transmission, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group - the UE does not expect to transmit in different slots more than one PUCCHs that provide HARQ-ACK information or CSI reports and would overlap with the PUSCH transmission in the OCC group When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in clause 9.2.5 and 9.2.6 before resolving the overlapping for PUCCH transmissions without SL HARQ-ACK or the overlapping for PUCCH transmissions and PUSCH transmissions. < Unchanged parts are omitted > |  |

TP on editorials

In this section, TP on some editorial issues proposed as below.

The following TP#5 is proposed for editorial corrections.

| Reason for change | Editorial corrections. |
| --- | --- |
| Summary of change | - In TS38.214 section 5.2.1.5.4.1, add “state” after “indicated TCI”. - In TS38.214 section 5.2.1.5.4.1b, revise “L1-RSRPs” to “L1-RSRP”. - In TS38.214 section 5.2.1.5.4.1c, clarify the current beam RS determination when enabledCurrentBeamReport is configured for event-7. |
| Consequences if not approved | It can cause inconsistency in the specification description. |
| TS38.214 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. < Unchanged parts are omitted > 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event1’, an event instance is determined if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet, is lower than eventThreshold-Event1. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode-r19 is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. < Unchanged parts are omitted > 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal with the valueOfQ highest L1-RSRP out of the reference signals among the activated TCI states (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH block s QCLed with the reference signals among the activated TCI states (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH blocks QCLed with the activated TCI state reference signals which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. < Unchanged parts are omitted > |  |

Conclusion

For UE-initiated/event-driven L1/L2 beam reporting, we have the following proposals:

- If at least one report associated with the PUCCH resource satisfies the corresponding trigger condition, the 1-bit indication transmitted on the PUSCH is set to 1; else, the 1-bit indication transmitted on the PUSCH is set to 0.

The beta offset value of the 1-bit indication is the same as that for CSI part I with up to 11 bits or HARQ-ACK with up to 2bits.

If only the 1-bit first PUCCH is overlapped with PUSCH in the time domain, the 1-bit indication should be multiplexed on the PUSCH by puncturing and coded separately.

Support co-existence between the 1-bit indication and other UCI types as the following Table.

|  | 1-bit indication | HARQ-ACK | CG-UCI/ UTO-UCI | CSI | Consider or not | Separate/joint coding | Puncture or rate match |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case-1 | ✔ | ✔ | ❌ | ❌ | ✔ | Joint coding | If number of jointly coded bits is no more than 2=>puncture; else, =>rate match |
| Case-2 | ✔ | ❌ | ❌ | ✔ | ✔ | Separate coding | 1-bit indication=> puncture; CSI=>rate match |
| Case-3 | ✔ | ✔ | ❌ | ✔ | ✔ | Joint coding for 1-bit indication and HARQ-ACK; Separate coding for CSI | If number of jointly coded bits is no more than 2=>puncture; else, =>rate match; CSI=>rate match |
| Case-4 | ✔ | ❌ | ✔ | ❌ | ❌ |  |  |
| Case-5 | ✔ | ❌ | ✔ | ✔ | ❌ |  |  |
| Case-6 | ✔ | ✔ | ✔ | ❌ | ❌ |  |  |
| Case-7 | ✔ | ✔ | ✔ | ✔ | ❌ |  |  |

The case of more than one indication multiplexing with PUSCH should be avoided by the network.

Support the following TP#1.

| Reason for change | In TS38.214 [3], the indicated TCI state is applied to the same CC of CSI-ReportConfig, or the CC indicated by CCofIndicatedTCI-r19 in the CSI-ReportConfig. However, according to the above agreements, the CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) can be the same or different. The point that needs clarification is that the CC of the CSI-ReportConfig may not be equal to the CC in which the new beam RS is configured, especially when the RRC parameter carrier is configured in the CSI-ReportConfig. The corresponding text proposal is provided accordingly. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1, clarify the determination of CC on which the indicated TCI state is applied in the multi-CC case. |
| Consequences if not approved | Previous agreement is not captured in the specification. |
| TS38.214 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. < Unchanged parts are omitted > 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event1’, an event instance is determined if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet, is lower than eventThreshold-Event1. < Unchanged parts are omitted > 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal with the valueOfQ highest L1-RSRP out of the reference signals among the activated TCI states (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH block s QCLed with the reference signals among the activated TCI states (applied to the same CC of the reference signal configured by the newBeamResourceSet CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. < Unchanged parts are omitted > |  |

Support the following TP#2.

| Reason for change | In TS38.214 [3], the CSI report identifier in the beam report format is captured as CSI-ReportConfigID. However, according to the related agreement, the CSI report identifier is determined based on the order of the triggered CSI report in the CSI report configurations associated with the same PUCCH resource. Thus, the specification description is aligned with the agreement only when all CSI report configurations in a CC are associated with the same PUCCH resource. However, it can introduce many restrictions to the network configuration. In addition, the CSI report identifier is captured in TS38.212 [1] as follows: Table 6.3.2.1.2-14: CRI, SSBRI, RSRP, CapabilityIndex, CSI report configuration indicator and Condition met indicator Field; Bitwidth. CRI; log2(KsCSI-RS). SSBRI; log2(KsSSB). RSRP; 7. Differential RSRP; 4. CSI report configuration indicator; log2(KsCSI_config). Condition met indicator; 1. NOTE: KsCSI-RS is the number of CSI-RS resources in the corresponding resource set, KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Index', KsCSI_config is the number of CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR.. Table 6.3.2.1.2-3I: Mapping order of CSI fields of one report for CSI report configuration indicator/CRI/RSRP/Condition met indicator, or CSI report configuration indicator/SSBRI/RSRP/Condition met indicator CSI report number; CSI fields. CSI report #n; CSI report configuration indicator as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #1 as in Table 6.3.2.1.2-14. CRI or SSBRI #2 as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #3 as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #4 as in Table 6.3.2.1.2-14, if reported. RSRP #1 as in Table 6.3.2.1.2-14. Differential RSRP #2 as in Table 6.3.2.1.2-14, if reported. Differential RSRP #3 as in Table 6.3.2.1.2-14, if reported. Differential RSRP #4 as in Table 6.3.2.1.2-14, if reported. Condition met indicator #1 in Table 6.3.2.1.2-14, if reported. Condition met indicator #2 in Table 6.3.2.1.2-14, if reported. Condition met indicator #3 in Table 6.3.2.1.2-14, if reported. Condition met indicator #4 in Table 6.3.2.1.2-14, if reported. RSRP as in Table 6.3.2.1.2-14 for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state if eventType is set to ‘event1’ and enabledCurrentBeamReport is configured; Differentiable RSRP as in Table 6.3.2.1.2-14 for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state if eventType is set to ‘event2’ and if enabledCurrentBeamReport is configured; Differentatial RSRP as in Table 6.3.2.1.2-14 for the reference signal with the valueOfQ highest L1-RSRP out of reference signals among the activated TCI states or the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of refence signals among the activated TCI states if eventType is set to ‘event7’ and if enabledCurrentBeamReport is configured;. Zero padding bits , if needed. NOTE: The CSI report configuration indicator with value indicates the (+1)-th CSI report configuration among the CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR, which are ordered in ascending order of CSI-ReportConfigID. The condition met indicators are reported only when the UE is configured with higher layer parameter PresenceOfConditionMetIndicator. If the reference signal associated with CRI or SSBRI #j satisfies the condition for transmitting UEIRI as defined in Clause 5.2.1.5.4.1 in [6, TS 38.214] when eventDetectionTimeWindowLength is configured, the value of the condition met indicator #j is set to 1; otherwise set to 0.. The above description in TS38212 is aligned with the agreement. Therefore, to align the specification description and agreement, the description of the CSI report identifier in TS38.214 should be revised. |
| --- | --- |
| Summary of change | In TS38.214 section 5.2.1.5.4.1d, clarify the determination of a CSI report identifier in the report format when multiple CSI report configurations are associated with one PUCCH resource. |
| Consequences if not approved | The spec is not aligned with the agreement. |
| TS38.214 5.2.1.5.4.1d UE Initiated CSI reporting for multiple CSI configurations For a UE configured with multiple CSI-ReportConfig with same higher layer parameters eventType and PUCCHResource, the UE expects - the multiple CSI-ReportConfig to be configured in the same CC, - the multiple CSI-ReportConfig to be associated with a same CSI trigger state if reportTransmissionMode is configured as ‘ModeA’, else - the same configuredPUSCHResourceOfModeB if reportTransmissionMode is configured as ‘ModeB’. The UE reports in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet in a CSI-ReportConfig that satisfies the event. The CSI report includes the corresponding CSI-ReportConfigId CSI report configuration indicator as defined in Table 6.3.2.1.2-14 in [5, TS38.212] and is zero padded to a fixed payload size (when needed), with the fixed payload size given by the maximum payload size among all the multiple CSI-ReportConfig. |  |

Support the following TP#3.

| Reason for change | In the last meeting, when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR, the following multiplexing rule was agreed. - Extend the SR field of log2K+1 bits to a field of log2K+L+1 bits of representing at most one of positive SR or positive UEIRI - The value in the field is based the ascending order of SR ID first and then the ascending order of the PUCCH resource ID associated with the first PUCCHs. - If one of the SRs is a positive LRR, the value of the log2K+L+1 bits indicates the positive LRR, else, if one of the UEIBRs is a positive UEIBR, the value of the log2K+L+1 bits indicates the positive UEIBR. - An all-zero value for the log2K+L+1 bits represents a negative SR and UEIRI value across all K LRR/SRs and/or L UEIRIs. However, in the current specification, only the green highlighted part is captured when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR. To align with the agreement, the yellow-highlighted part should be added. |
| --- | --- |
| Summary of change | In TS38.213 section 9.2.5.1, add the case of no positive LRR and at least one positive UEIBRs to the multiplexing rule of PUCCH with UEIBR. |
| Consequences if not approved | UE behavior is unclear for the case of no positive LRR and at least one positive UEIBRs, when the PUCCH resource for UE-initiated/event-driven beam reporting is collided/overlapped with a PUCCH format 2/3/4 carrying CSI/SR.. |
| TS38.213 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH < Unchanged parts are omitted > If a UE would transmit a PUCCH with OACK HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in clauses 9.2.1 and 9.2.3, log2K+1 bits representing a negative or positive SR/UEIRI, in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell for SR, and then in ascending order of the values of pucch-ResourceId for UEIRI, are appended to the HARQ-ACK information bits and the UE transmits the combined OUCI=OACK+log2K+1 UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR; else, if one of the UEIRIs is a positive UEIRI, the value of the log2K+1 bits indicates the positive UEIRI. An all-zero value for the log2K+1 bits represents a negative SR/UEIRI value across all K SRs and UEIRIs. If a UE would transmit a PUCCH with CSI report bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, log2K+1 bits representing corresponding negative or positive SR/UEIRI, in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell for SR, and then in ascending order of the values of pucch-ResourceId for UEIRI, are prepended to the CSI information bits as described in clause 9.2.5.2 and the UE transmits a PUCCH with the combined OUCI=log2K+1+OCSI UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. If one of the SRs is a positive LRR, the value of the log2K+1 bits indicates the positive LRR; else, if one of the UEIRIs is a positive UEIRI, the value of the log2K+1 bits indicates the positive UEIRI. An all-zero value for the log2K+1 bits represents a negative SR/UEIRI value across all K SRs and UEIRIs. If a UE transmits a PUCCH with OACK HARQ-ACK information bits, OSR=log2K+1 SR/UEIRI bits, and OCRC CRC bits using PUCCH format 2 or PUCCH format 3 in a PUCCH resource that includes MRBPUCCH PRBs, the UE determines a number of PRBs MRB,minPUCCH for the PUCCH transmission to be the minimum number of PRBs, that is smaller than or equal to a number of PRBs provided by nrofPRBs in PUCCH-format2 or nrofPRBs in PUCCH-format3 and starts from the first PRB from the number of PRBs, that results to OACK+OSR+OCRC≤MRB,minPUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r and, if MRBPUCCH>1, OACK+OSR+OCRC>MRB,minPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, where Nsc,ctrlRB, Nsymb-UCIPUCCH, Qm, and r are defined in clause 9.2.5.2. For PUCCH format 3, if MRB,minPUCCH is not equal 2α2⋅3α3⋅5α5 according to [4, TS 38.211], MRB,minPUCCH is increased to the nearest allowed value of nrofPRBs [12, TS 38.331]. If OACK+OSR+OCRC>MRBPUCCH-1⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r, the UE transmits the PUCCH over the MRBPUCCH PRBs. If a UE is provided a first interlace of MInterlace,0PUCCH PRBs by interlace0 in InterlaceAllocation and transmits a PUCCH with OACK HARQ-ACK information bits, OSR=log2K+1 SR/UEIRI bits, and OCRC CRC bits using PUCCH format 2 or PUCCH format 3, the UE transmits the PUCCH over the first interlace if OACK+OSR+OCRC≤MInterlace,0PUCCH⋅Nsc,ctrlRB⋅Nsymb-UCIPUCCH⋅Qm⋅r; otherwise, if the UE is provided a second interlace by interlace1 in PUCCH-format2 or PUCCH-format3, the UE transmits the PUCCH over the first and second interlaces. < Unchanged parts are omitted > |  |

Support the following TP#4.

| Reason for change | In the last meeting, reusing the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH for UEI-BR for Mode A was agreed. However, it is not captured in the current specification. |
| --- | --- |
| Summary of change | In TS38.213 section 9, clarify the intra-UE multiplexing/prioritization rules of PUSCH with A-CSI for PUSCH is reused for UEI-BR for Mode A. |
| Consequences if not approved | UE behavior is not clear when PUSCH carrying UEI-BR for Mode A overlaps with other uplink channels/RSs. |
| TS38.213 9 UE procedure for reporting control information < Unchanged parts are omitted > For the remaining of this clause, when a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs, and is provided coresetPoolIndex with a value of 1 for second CORESETs, on active DL BWPs of serving cells, and - is provided sTx-2Panel the UE separately determines and resolves time overlapping among first PUSCH transmissions that use respective first spatial domain filters corresponding to first TCI-State or TCI-UL-State associated with the first CORESETs, and among second PUSCH transmissions that use respective second spatial domain filters corresponding to second TCI-State or TCI-UL-State associated with the second CORESETs. For the remaining of this clause, reference to PUSCH with semi-persistent CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeB’ in the CSI report configuration [6, TS 38.214]. For the remaining of this clause, reference to PUSCH with aperiodic CSI reports includes a PUSCH with UE initiated reports when reportTransmissionMode is configured as ‘ModeA’ in the CSI report configuration [6, TS 38.214]. For the remaining of this clause, for a UE operating on an NTN serving cell, the timeline conditions for resolving time overlapping between a PUSCH transmission with repetitions in an OCC group [6, TS 38.214] and PUCCH transmissions are applicable with respect to the first repetition of the PUSCH transmission in the OCC group - if the UE would multiplex UCI from the PUCCH transmissions in the PUSCH, the UE multiplexes the UCI in all repetitions of the PUSCH transmission in the OCC group - if the UE would not transmit a repetition of the PUSCH transmission, the UE does not transmit all repetitions of the PUSCH transmission in the OCC group - the UE does not expect to transmit in different slots more than one PUCCHs that provide HARQ-ACK information or CSI reports and would overlap with the PUSCH transmission in the OCC group When a UE determines overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of larger and/or smaller priority index, the UE resolves the overlapping for PUCCH transmissions with SL HARQ-ACK reports and PUCCH of each priority index as described in clause 9.2.5 and 9.2.6 before resolving the overlapping for PUCCH transmissions without SL HARQ-ACK or the overlapping for PUCCH transmissions and PUSCH transmissions. < Unchanged parts are omitted > |  |

The following TP#5 is proposed for editorial corrections.

| Reason for change | Editorial corrections. |
| --- | --- |
| Summary of change | - In TS38.214 section 5.2.1.5.4.1, add “state” after “indicated TCI”. - In TS38.214 section 5.2.1.5.4.1b, revise “L1-RSRPs” to “L1-RSRP”. - In TS38.214 section 5.2.1.5.4.1c, clarify the current beam RS determination when enabledCurrentBeamReport is configured for event-7. |
| Consequences if not approved | It can cause inconsistency in the specification description. |
| TS38.214 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. < Unchanged parts are omitted > 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event1’, an event instance is determined if the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet, is lower than eventThreshold-Event1. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode-r19 is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. < Unchanged parts are omitted > 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal with the valueOfQ highest L1-RSRP out of the reference signals among the activated TCI states (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH block s QCLed with the reference signals among the activated TCI states (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH blocks QCLed with the activated TCI state reference signals which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. < Unchanged parts are omitted > |  |

References

3GPP TS38.212: “NR; Multiplexing and channel coding (v19.0.0)”

3GPP TS38.213: “NR; Physical layer procedures for control (v19.0.0)”

3GPP TS38.214: “NR; Physical layer procedures for data (v19.0.0)”

RAN1#121 chairman notes.