3GPP TSG RAN WG1 #122	R1-2505504

Bengaluru, India, Aug 25th – 29th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues on IoT-NTN TDD mode

Agenda Item:	8.11.5

Document for: Discussion

- Introduction

              - In RAN1#121 [1], the solution design of IoT-NTN TDD mode is completed. In this contribution, the remaining issues are discussed.

- Discussion on frame structure for IoT-NTN TDD mode

For IoT-NTN TDD mode, a TDD frame structure with 90 ms periodicity has been introduced, where D=8 consecutive DL subframes and U=8 consecutive UL subframes are separated by G=50 guard period subframes. In current specification, the TDD frame structure is expected to be defined in TS 36.211 and cited by TS 36.213. However, the wordings in 36.211 and 36.213 are not aligned, which may cause confusion.

In TS 36.211, the frame structure is as specified as below, where “frame structure for NTN-TDD” is used to describe the frame structure of IoT-NTN TDD mode.

4.4 Frame structure type 1 for NTN-TDD The frame structure for NTN-TDD, at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [4] consists of 8 consecutive downlink subframes, followed by 50 consecutive guard period subframes, followed by 8 consecutive uplink subframes in each 90 ms interval. - The UE shall not assume any signal or channel being transmitted in subframes other than downlink subframes 3, 4, 5, 6, 7, 8, 9, and 0 across two consecutive radio frames. - The UE shall not transmit any signal or channel on a subframe other than the 8 consecutive uplink subframes.

While in TS 36.213, the frame structure related specifications are shown below, where the wording “TDD pattern” is used and the definition in TS 36.211 is cited to determine the value of consecutive downlink and uplink subframes.

16 UE Procedures related to narrowband IoT Throughout this clause, - for a NB-IoT UE in a NTN TDD serving cell, - the UE shall not assume any downlink physical signal or physical channel is present in any subframe other than within the D consecutive downlink subframes according to the TDD pattern and the value of D defined in [3], - the UE shall not transmit any uplink physical signal or physical channel in any subframe other than within the U consecutive uplink subframes according to the TDD pattern and the value of U defined in [3].

Different terminologies are used in TS 36.211 and TS 36.213, which makes it more difficult to find the referenced specification. Therefore, updates on TS 36.211 or TS 36.213 are expected to align the specification, e.g., by replacing “TDD pattern” in TS 36.213 with “frame structure for NTN-TDD”.

Proposal 1: Adopt the following TP in TS 36.213 V19.0.0.

| Reason for change: | The frame structure for IoT-NTN TDD mode is defined in TS 36.211 and cited in TS 36.213. However, the terminology “TDD pattern” is used in TS 36.213, which is not aligned with definition in TS 36.211. |
| --- | --- |
| Summary of change: | The terminology “TDD pattern” is changed to “frame structure for NTN-TDD” to align with the definition in TS 36.211. |
| Consequences if not approved: | Misaligned specification between TS 36.213 and cited TS 36.211. |

- --------------------Start of TP for TS 38.213 V19.0.0 --------------------------------- <Unchanged parts are omitted> ## 16 UE Procedures related to narrowband IoT Throughout this clause, - for a NB-IoT UE in a NTN TDD serving cell, - the UE shall not assume any downlink physical signal or physical channel is present in any subframe other than within the D consecutive downlink subframes according to the TDD pattern frame structure for NTN-TDD and the value of D defined in [3], - the UE shall not transmit any uplink physical signal or physical channel in any subframe other than within the U consecutive uplink subframes according to the TDD pattern frame structure for NTN-TDD and the value of U defined in [3]. - for a NB-IoT UE, the value of Koffset is given by, - if the UE is configured with the higher layer parameter k-Offset, - Koffset= Kcell_offset-KUE_offset where Kcell_offset is the parameter k-Offset provided by higher layers, and KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0 - otherwise, - Koffset=0. <Unchanged parts are omitted> ## 16.4 Narrowband physical downlink shared channel related procedures A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows - If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe. - Else if the UE is in a NTN TDD serving cell and the UE determines the subframe is not one of the D consecutive downlink subframes according to the TDD pattern frame structure for NTN-TDD and the value of D defined in [3], then the subframe is not assumed as a NB-IoT DL subframe. <Unchanged parts are omitted> ## 16.5 Narrowband physical uplink shared channel related procedures For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter npusch-MultiTB-Config, there shall be a maximum of 2 uplink HARQ processes. For a NB-IoT UE and NPUSCH transmission using preconfigured uplink resource, there shall be 1 uplink HARQ process. A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows - If higher layer parameter resourceReservationConfigUL is configured - for NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space including NPUSCH format 1 transmission without a corresponding NPDCCH - if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT UL subframe - else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe). - for NPUSCH format 2 transmission - the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe). - In all other cases, - for TN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers - for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe - for NTN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if it is one of the U consecutive uplink subframes according to the TDD pattern frame structure for NTN-TDD and the value of U defined in [3]. <Unchanged parts are omitted> --------------------End of TP for TS 38.213 V19.0.0 ---------------------------------

- Conclusions

      - According to the analysis given above, we have the following proposal:

Proposal 1: Adopt the following TP in TS 36.213 V19.0.0.

| Reason for change: | The frame structure for IoT-NTN TDD mode is defined in TS 36.211 and cited in TS 36.213. However, the terminology “TDD pattern” is used in TS 36.213, which is not aligned with definition in TS 36.211. |
| --- | --- |
| Summary of change: | The terminology “TDD pattern” is changed to “frame structure for NTN-TDD” to align with the definition in TS 36.211. |
| Consequences if not approved: | Misaligned specification between TS 36.213 and cited TS 36.211. |

- --------------------Start of TP for TS 38.213 V19.0.0 --------------------------------- <Unchanged parts are omitted> ## 16 UE Procedures related to narrowband IoT Throughout this clause, - for a NB-IoT UE in a NTN TDD serving cell, - the UE shall not assume any downlink physical signal or physical channel is present in any subframe other than within the D consecutive downlink subframes according to the TDD pattern frame structure for NTN-TDD and the value of D defined in [3], - the UE shall not transmit any uplink physical signal or physical channel in any subframe other than within the U consecutive uplink subframes according to the TDD pattern frame structure for NTN-TDD and the value of U defined in [3]. - for a NB-IoT UE, the value of Koffset is given by, - if the UE is configured with the higher layer parameter k-Offset, - Koffset= Kcell_offset-KUE_offset where Kcell_offset is the parameter k-Offset provided by higher layers, and KUE_offset is the parameter Differential Koffset provided by higher layers, otherwise KUE_offset=0 - otherwise, - Koffset=0. <Unchanged parts are omitted> ## 16.4 Narrowband physical downlink shared channel related procedures A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows - If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe. - Else if the UE is in a NTN TDD serving cell and the UE determines the subframe is not one of the D consecutive downlink subframes according to the TDD pattern frame structure for NTN-TDD and the value of D defined in [3], then the subframe is not assumed as a NB-IoT DL subframe. <Unchanged parts are omitted> ## 16.5 Narrowband physical uplink shared channel related procedures For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter npusch-MultiTB-Config, there shall be a maximum of 2 uplink HARQ processes. For a NB-IoT UE and NPUSCH transmission using preconfigured uplink resource, there shall be 1 uplink HARQ process. A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows - If higher layer parameter resourceReservationConfigUL is configured - for NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space including NPUSCH format 1 transmission without a corresponding NPDCCH - if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT UL subframe - else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe). - for NPUSCH format 2 transmission - the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe). - In all other cases, - for TN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers - for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe - for NTN TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if it is one of the U consecutive uplink subframes according to the TDD pattern frame structure for NTN-TDD and the value of U defined in [3]. <Unchanged parts are omitted> --------------------End of TP for TS 38.213 V19.0.0 ---------------------------------

- Reference

- Chair’s notes for RAN1 #121