3GPP TSG RAN WG1 #122		R1-2505545

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.6.1

Source:	Samsung

Title:	Remaining issues on LP-WUS and LP-SS design

Document for:	Discussion and Decision

- Introduction

In WID for Rel-19 LP-WUS [1], objectives for LP-WUS and LP-SS design have been agreed as follow:

Objective for LP-WUS design - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported

Objective for LP-SS design - For IDLE/INACTIVE modes - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

In addition, there are notes for the target coverage and priority of LP-WUS signal design as follow:

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

This contribution discusses some remaining issues for LP-WUS and LP-SS design.

- LP-WUS targeting to one LP-WUR type in a cell

In RAN1#121, the scenario that the gNB enables LP-WUS feature only for one LP-WUR type in a cell was discussed as follows [2].

Scenarios mentioned by companies include 1) gNB enables LP-WUS feature only for one LP-WUR type in the cell, e.g., only OFDM-based receiver is to be served - A) For IDLE/INACTIVE mode, gNB configures the LP-WUS entry/exit conditions for OFDM-based receiver only - B) For CONNECTED mode, gNB provides the LP-WUS configuration for OFDM-based receiver only 2) gNB enables LP-WUS feature for both LP-WUR types in the cell - A) For IDLE/INACTIVE mode, gNB configures the LP-WUS entry/exit conditions for both LP-WUR types. However, it is possible that only OFDM-based receiver is to be paged by a WUS transmission at a given time - B) For CONNECTED mode operation, gNB configures LP-WUS separately for OFDM-based receiver and OOK-based receiver, e.g., with different MOs and/or codepoints For above scenario when a LP-WUS targets OFDM-based receiver only, the LP-WUS transmission duration can be determined by the information bits without channel coding, number of overlaid OFDM sequences per OOK ON chip and the repetition factor, without dependency of OOK ON-OFF pattern targeting OOK receiver. In above scenarios, there are two cases for LP-WUS transmission from gNB perspective, and gNB can switch between case 1 and case 2 depending on the targeting WUR type. Considering it may not be always possible to indicate which case is used for a WUS transmission especially in RRC idle/inactive, and further, it is not necessary to indicate it neither. Case 1 and case 2 is agnostic to UE. - Case 1: when a LP-WUS transmission targets only OFDM-based receiver, gNB transmits LP-WUS with duration determined according to the number of OFDM symbols X required to deliver the full information bits configured for the OFDM-based receiver. - Case 2: when a LP-WUS transmission targets both OFDM-based receiver and OOK-based receiver, gNB transmits LP-WUS with duration determined according to the number of OFDM symbols Y required to deliver the full information bits configured for the OOK-based receive. - For case 2, from gNB perspective, block-level repetition of overlaid OFDM sequence is performed in the Y symbols. From UE perspective, the LP-WUS duration shall be known to the UE. Given that WUS information bits are carried by overlaid OFDM sequences, a shorter WUS duration is possible no matter case 1 or case 2. And thus, WUS duration length for OFDM receiver and OOK receiver could be different

The benefits to support above scenario enabling LP-WUS targeting to OFDM-based LP-WUR is to allow shorter transmission duration for LP-WUS, and it provides LP-WUS overhead reduction from the gNB point of view.

Specifically, when M is set to 2 and the code block length is 32, the number of OFDM symbols required to transmit an LP-WUS is 32 after applying Manchester coding. In this case, a UE supporting OOK-based LP-WUS must monitor all OFDM symbols to obtain wake-up information. However, a UE supporting OFDM-based LP-WUS—i.e., capable of obtaining wake-up information from an overlaid OFDM sequence—can acquire the wake-up information and terminate LP-WUS monitoring after monitoring only 2–3 OFDM symbols. Even if the overlaid OFDM sequence is received twice for improved reception performance, the UE can still determine the wake-up status by monitoring only 4–6 OFDM symbols.

From the perspective of a UE with OFDM-based LP-WUR, such early termination enables power savings. However, the gNB must transmit the LP-WUS over all 32 OFDM symbols to support UEs in the cell with OOK-based LP-WUR. Therefore, when the gNB does not have sufficient resources to support OOK-based LP-WUS, it may enable the LP-WUS operation feature by targeting only UEs with OFDM-based LP-WUR.

How the gNB targets a specific LP-WUR type when enabling the LP-WUS feature can be further discussed. For example, the gNB may implicitly indicate this through configuration of LP-SS or entry conditions according to the LP-WUR type, or explicitly indicate the LP-WUR types it supports through higher layer parameters.

On the other hand, targeting only UEs with OOK-based LP-WUR does not offer clear benefits for either the gNB or the UE. Therefore, when the gNB supports OOK-based LP-WUR, it would be preferable to configure LP-WUS-related settings by considering both LP-WUR types.

Proposal 1: Support the scenario that the gNB enable LP-WUS feature only for OFDM-based LP-WUR type in the cell.

- FFS: implicit configuration or explicit configuration.

- Discussion on higher layers parameters

In [3], higher layer parameters for LP-WUS are listed, but some discussion points remain.

One issue is that, for LP-WUS operation in RRC IDLE/INACTIVE, two options can be considered regarding LP-SS-related configuration. The specific parameters are “LP-SS_Binary_Seq”, “LP-SS_Binary_Seq_Length”, and “LP-SS_periodicityoffset”. According to [3], these parameters (i) shall be configured for OOK-based LP-WUR for LP-WUS operation in IDLE/INACTIVE (Option 1), or (ii) shall be configured only when LP-SS is configured (Option 2). The meaning of Option 1 is not entirely clear; we have interpreted it as meaning that LP-SS-related parameters must always be configured when the gNB enables LP-WUS feature.

If the scenario discussed in Section 2—where LP-WUS transmission targets only OFDM-based LP-WUR—is supported, these parameters are not mandatory when the gNB enables the LP-WUS feature. This is because LP-SS is originally designed for OOK-based LP-WUR, and if the gNB supports LP-WUS only for OFDM-based LP-WUR, LP-SS transmission is not required. Moreover, as specified in FG 62-1a, UEs with OFDM-based LP-WUR support SSB-based RRM measurements and can perform synchronization using SSB, meaning that transmitting LP-SS provides no clear benefit for either the gNB or the UE. Therefore, if the scenario of targeting only OFDM-based LP-WUR is supported, the LP-SS-related configuration parameters described above may be specified to be configured only when LP-SS transmission is configured—i.e., when the gNB enables a support of OOK-based LP-WUR.

In other words, in such cases, the additional LP-SS configuration can indicate whether the gNB targets only OFDM-based LP-WUR or both LP-WUR types. Alternatively, the absence of these parameters could implicitly indicate that LP-SS is not transmitted, and thus that only the OFDM-based LP-WUR type is supported in the cell. On the other hand, a case where the gNB supports OOK-based LP-WUR but does not configure or transmit LP-SS-related parameters is not considered.

Proposal 2: For higher layer parameters related to LP-SS, these parameters shall be configured only when the gNB enables a support for OOK-based LP-WUR.

- LP-SS_Binary_Seq

- LP-SS_Binary_Seq_Length

- LP-SS_periodicityoffset

Proposal 3: The configuration status of LP-SS-related parameters can be used to indicate whether the gNB targets only OFDM-based LP-WUR or all LP-WUR types.

Another issue is the configuration of LP-WUS_overlaidSeq_roots_IDLE/INACTIVE. It is still FFS whether this parameter shall be configured for OFDM-based LP-WUR for LP-WUS operation in IDLE/INACTIVE. From our understanding, this parameter is essential for an OFDM-based LP-WUR to obtain wake-up information. As described in Section 2, supporting LP-WUS operation solely for UEs with OOK-based LP-WUR offers no clear benefit. Therefore, when the gNB enables the LP-WUS feature, this parameter shall be configured.

Proposal 4: LP-WUS_overlaidSeq_roots_IDLE/INACTIVE shall be configured when the gNB enables the LP-WUS feature.

- Conclusion

The proposals and observation made in this contribution are summarized below:

Proposal 1: Support the scenario that the gNB enable LP-WUS feature only for OFDM-based LP-WUR type in the cell.

- FFS: implicit configuration or explicit configuration.

Proposal 2: For higher layer parameters related to LP-SS, these parameters shall be configured only when the gNB enables a support for OOK-based LP-WUR.

- LP-SS_Binary_Seq

- LP-SS_Binary_Seq_Length

- LP-SS_periodicityoffset

Proposal 3: The configuration status of LP-SS-related parameters can be used to indicate whether the gNB targets only OFDM-based LP-WUR or all LP-WUR types.

Proposal 4: LP-WUS_overlaidSeq_roots_IDLE/INACTIVE shall be configured when the gNB enables the LP-WUS feature.

Reference

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

[2] R1-2504908, Final summary of discussion on LP-WUS and LP-SS design (RAN1#121), vivo

[3] R1-2503244, Collection of Rel-19 higher layers parameters list Post RAN1#121