3GPP TSG RAN WG1 #122	R1-2505658

Bengaluru, India, August 25th - 29th, 2025

Agenda item:		8.2.1

Source:		Ofinno

Title:			Discussion on UE-initiated/event-driven beam management

Document for:	Discussion and Decision

Introduction

The RAN1 core part of the WI on UE-initiated/event-driven beam management [1] was completed in the last RAN plenary meeting (RAN#108) and the corresponding RAN1 CRs to TS 38.211, 38.212, 38.213, and 38.214 [2-5] were approved. The agreements and the discussion at the last RAN1 meeting (RAN1#121) were captured in the moderator summary [6]. In this paper we discuss some of the remaining issues as part of the maintenance of the work item.

Counter Reset and Cancellation conditions

To ensure that event evaluation is always based on up-to-date measurement results and configuration status, counter reset condition is the most important thing to accomplish it. Without such conditions, event triggering may rely on stale or inconsistent beam information, leading to unnecessary suppression of valid triggers, delayed reporting of better beams, and potential misalignment between UE and gNB. The discussion first addresses candidate reset conditions for Event 2 that were considered during the Work Item phase but not agreed, followed by reset conditions for Event 1 and Event 7.

Though we consider other candidates worthwhile to consider, we are open to start from FL proposal [7] on Candidate #1 and Candidate #7.

Proposal 1.2: Regarding triggering event determination for Event 2, besides for Candidate#2, at least Candidate #1 and Candidate#7 are additionally supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - The RRC parameter(s) at least comprises the threshold for event evaluation - In such case, the UE need to reset the counting for all new beams. - FFS: other parameters, e.g., the length of the time window or maximum counting number M.

Candidate#1: Reconfiguration/update of the new beams or MAC-CE signaling for new beam is received.

If the counter is not reset when the beam RS list is updated by an RRC reconfiguration message, the UE may continue to maintain counters for beams that are no longer configured. This can cause two issues: obsolete beams may unnecessarily influence event evaluation, and newly added beams may not trigger promptly because their counters are not restarted from a clean state. Resetting the counter for removed beams eliminates their influence on event detection, and starting a new counter for newly added beams ensures that they are evaluated based on fresh measurements.

Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured.

When RRC parameters associated with UE-initiated beam reporting, for example, the time window or maximum event instance count, are reconfigured and the counter is not reset, the UE may continue to evaluate events under outdated configuration values. This can lead to unintended triggering frequency or suppression that does not match the intended reporting behavior. Resetting the counters ensures that the evaluation immediately reflects the updated configuration, maintaining the intended triggering logic.

We propose the following modification to FL proposal to agree.

Proposal 1. Propose the revised FL proposal, updated as follows:

Proposal 1.2: Regarding triggering event determination for Event 2, besides for Candidate#2, at least Candidate #1 and Candidate#7 are additionally supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - Reset a new beam removed by the RS reconfiguration/update (the new beam not in the updated list any longer). - Do not reset a new beam in both old and new list (the new beam is not changed in the updated list). - Start counting for a new beam added to the list (the new beam not in the updated list). - Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - The RRC parameter(s) at least comprises the threshold, time window and maximum event instance count for event evaluation - In such case, the UE need to reset the counting for all new beams.

Proposal 2. Adopt Text proposal #1 in Annex A to TS 38.214.

Since the network cannot predict exactly when the UE will perform a UE-initiated beam reporting, situations may arise where the triggered report becomes invalid before completion. In addition to defining counter reset conditions, it is also necessary to specify conditions for cancelling an ongoing UE-initiated beam reporting procedure. In certain situations, continuing a triggered report is no longer meaningful, and cancelling it can prevent unnecessary signaling (e.g., the first PUCCH and the second PUSCH transmissions) and reduce UE overhead. For example, after the UE triggers a UE-initiated beam reporting, the indicated TCI state may be updated by the NW before the first PUCCH transmission. If the UE performs the first PUCCH transmission even though the indicated TCI has been updated, upon receiving the PUCCH transmission, the NW may incorrectly assume that the new updated TCI state has a poor quality and may update the indicated TCI state again. Similarly, the UE may trigger UEIBR for a TCI state of BWP 1 but switch to BWP 2 with a different TCI state before the first PUCCH transmission, leading the network to wrongly conclude that the TCI state of the new BWP is problematic.

Proposal 3. The following conditions can be considered for cancellation of a triggered UE-initiated/event-driven beam reporting for Event 2:

- Update of the indicated TCI state

- Transmission of UE-initiated CSI report

- BWP switching

Proposal 4. Adopt Text proposal #2 in Annex A to TS 38.214.

Similar to Event 2, counter reset conditions for Event 1 and Event 7 also need to be clarified. Without such rules, the UE may continue triggering UE-initiated CSI reports once the counter reaches its maximum value, leading to redundant or inconsistent reporting.

For Event 1, an update of the indicated TCI state should trigger a counter reset, ensuring alignment between the UE’s event evaluation and the current beam configuration.

For Event 7, since it was agreed that additional events (i.e., Event 1 and Event 7) follow the same design as Event 2, the event instance counting should be per new beam. In particular, a change of the activated TCI state with the Q-th best quality due to factors such as reception of Unified TCI States Activation/Deactivation MAC CE or updated UE measurements should trigger a counter reset. Without this, the UE-initiated beam report may not reflect the latest active TCI state set, preventing the network from managing beam activation effectively.

Proposal 5. Regarding the triggering event determination for Event 1 and Event 7, the following conditions can be considered for resetting the counter for Event 1 and Event 7:

- For Event 1: Update of the indicated TCI state

- For Event 7: Update/change of the activated TCI state with the Q-th best quality

  - In Event 7, the event instance(s) counting is per new beam.

Proposal 6. Adopt Text proposal #3 in Annex A to TS 38.214.

UE behavior in relation to Beam Failure Recovery

When a UE detects a beam failure on a cell (e.g., PCell, SCell), the indicated TCI state (e.g., TCI state 1) of the cell is likely to have a low radio link quality. When the indicated TCI state has a low radio link quality, the conditions of Event 1 and/or Event 2 are satisfied frequently during the ongoing beam failure recovery (e.g., between t1 and t2 in Figure 1), and the counter used for counting event instances of a new beam for UE-initiated beam reporting may reach the maximum value within the time window quickly. In another words, the UE may keep triggering the UE-initiated/event-driven beam report and transmit the first PUCCH and/or the second PUSCH multiple times leading to increased signaling. Figure 1 shows an existing beam failure recovery procedure. In our view, the ongoing beam failure recovery procedure is already trying to recover (or indicate) the potential blockage, and there is no need for a UE-initiated CSI report during the ongoing beam failure recovery. Especially, in the case of no carrier aggregation (e.g., only PCell is configured), when there is a beam failure on the PCell, the uplink channels are also likely to fail (the reason for resetting the uplink channels after beam failure recovery), so the first PUCCH and the second PUSCH for the UE-initiated beam reporting may fail.

- : Beam failure recovery procedure

In the legacy systems, 28 symbols after a beam failure recovery is completed, the UE applies the candidate reference signal identified for the beam failure recovery to PDCCH/PDSCH/A-CSI-RS receptions until application of a new indicated TCI state (e.g., between t2 and t5 in Figure 1). However, since the indicated TCI state for the cell is still TCI state 1 between t2 and t5 though it is not used, the conditions of Event 1 and/or Event 2 are still satisfied frequently even after successful completion of a beam failure recovery, and thus the UE triggers UE-initiated/event-driven beam report. One solution can be to restrict evaluation of the conditions for Event 1 and/or Event 2 after beam failure recovery and until a new TCI state is indicated (e.g., no trigger-event condition evaluation between t2 and t5), or another solution can be to evaluate the conditions Event 1 and/or Event 2 based on the candidate reference signal instead of the indicated TCI state.

Proposal 7. Specify necessary enhancements to address beam failure recovery when UE is configured with UE-initiated/event-driven beam report procedure at least based on Event 1 or Event-2.

- For example, the UE-initiated/event-driven beam report procedure is not performed (or is suspended) during an ongoing beam failure recovery procedure for a cell. (e.g., no condition evaluation, no first PUCCH and/or second PUSCH transmissions) until the beam failure recovery is completed or a new TCI is applied for the cell.

Proposal 8. Adopt Text proposal #4 in Annex A to TS 38.214.

Proposal 9. Specify UE behavior of UE-initiated/event-driven beam report procedure (e.g., condition evaluation, first PUCCH transmission, second PUSCH transmission, and so on) from a completion of a beam failure recovery procedure until the indicated TCI is updated.

- The candidate beam of the beam failure recovery is used for current beam in UEBIR.

Proposal 10. Adopt Text proposal #5 in Annex A to TS 38.214.

Multiplexing rules

In the legacy systems, for PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition (see the text copied below from 38.214 Section 6.1.2.1).

#### 6.1.2.1 Resource allocation in time domain … For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration.

Moreover, in the legacy systems, when a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH (see the text copied below from 38.214 Section 5.2.3).

### 5.2.3 CSI reporting using PUSCH … When a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

In Mode A of UE-initiated CSI reporting, as the DCI indicating an uplink grant for UE-initiated CSI report can schedule repetitions or multiple PUSCH transmissions, it is not clear which repetition or the PUSCH the UE-initiated CSI report is multiplexed in. In our view, the same behaviour as the aperiodic CSI report should be followed.

Proposal 11. On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, reuse the multiplexing rules of PUSCH with aperiodic CSI report for PUSCH with UE-initiated CSI report.

- For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and UE-initiated CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition.

- When a DCI format 0_1 schedules two PUSCH allocations, the UE-initiated CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the UE-initiated CSI report is carried on the penultimate scheduled PUSCH.

Proposal 12. Adopt Text proposal #6 in Annex A to TS 38.214.

UE behavior for resources outside active UL BWP

When the configured PUCCH resource and/or the Type 1 CG PUSCH resource for UE-initiated reporting is located in an uplink BWP that is not currently active, the UE cannot transmit the report due to the absence of uplink resources in the active BWP. Until the network activates the relevant uplink BWP configured with uplink resources for the report, the UE is unable to proceed with the CSI reporting procedure, resulting in increased latency for transmitting the UE-initiated CSI report. Additional clarification is needed on the expected UE behavior in this scenario to ensure consistent handling across implementations and avoid unnecessary delay in reporting.

For example, in Figure 2 below, the PUCCH resource for the first PUCCH transmission is configured on Uplink BWP 2. However, when the UE triggers the UE-initiated CSI reporting, Uplink BWP 3 may be active. If the UE waits for Uplink BWP 2 to become active, this can lead to increased reporting delay. One possible solution is to allow the UE to switch to Uplink BWP 2, which contains the configured PUCCH resource. Another approach is to configure multiple PUCCH resources, one per uplink BWP, so that the UE can immediately transmit the first PUCCH transmission without switching BWPs or waiting for the appropriate BWP (e.g., Uplink BWP 2) to be activated. Note that in current legacy periodic CSI reporting, the UE is configured with a set of PUCCH resources (e.g., pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource).

- : BWP switching for first PUCCH transmission

Similarly, in Figure 3 below, the Type 1 CG PUSCH resource is configured on Uplink BWP 2 of CC 0. However, when the UE transmits the first PUCCH transmission for the UE-initiated CSI reporting on CC 1, Uplink BWP 3 on CC 0 may be active. If the UE waits for Uplink BWP 2 to become active on CC 0, this can lead to increased reporting delay. One possible solution is to allow the UE to switch to Uplink BWP 2, which contains the configured Type 1 CG PUSCH resource. Another approach is to configure multiple Type 1 CG PUSCH resources, one per uplink BWP, so that the UE can immediately transmit the report without switching BWPs or waiting for the appropriate BWP (e.g., Uplink BWP 2) to be activated.

- : BWP switching for second PUSCH transmission in Mode-B of UEIBR

Therefore, UE behavior in scenarios where the associated PUCCH resource and/or the Type 1 CG PUSCH is located on an uplink BWP that is not currently active needs to be further clarified. Two possible approaches to resolve this issue are, allowing the UE to perform BWP switching to access the required uplink resources, or skipping event-instance evaluation.

Proposal 13. RAN1 agrees to support one of the following approaches to ensure that the UE can transmit the first PUCCH and/or second PUSCH for UEI-based CSI reporting:

- Alt 1: When the UE triggers UE-initiated CSI reporting, if the PUCCH resource and/or Type 1 CG PUSCH resource used for UE-initiated CSI reporting are not configured in the active UL BWP(s), the UE performs BWP switching to an UL BWP where the first PUCCH and/or second PUSCH are configured.

- Alt. 2: If the PUCCH resource and/or Type 1 CG PUSCH resource used for UE-initiated CSI reporting are not configured in the active UL BWP(s), UE does not perform event-instance evaluation.

Proposal 14. Adopt Text proposal #7 in Annex A to TS 38.214.

Prioritization of UEIRI in UL Tx power reduction

When the transmission powers of multiple uplink transmissions exceed the UE’s maximum transmit power, TS 38.213 specifies a prioritization mechanism to determine which transmissions should be reduced or dropped. This prioritization covers all conventional UCI types, such as Scheduling Request (SR), CSI reports, and HARQ-ACK, ensuring consistent behavior across implementations when power reduction is needed. However, the UE-initiated CSI reporting indicator (UEIRI), introduced as a new UCI type to support UE-initiated CSI reporting, is not currently included in the priority order defined in TS 38.213. Without explicit prioritization, UE behavior may vary between implementations, potentially causing inconsistent reporting of UE-initiated CSI under power-limited conditions. This could result in the unintended suppression of UEIRI or, conversely, excessive reduction of other UCI types, impacting system performance and interoperability. To maintain consistent behavior across all UEs and align the UEIRI treatment with existing UCI types, it is necessary to add UEIRI to the prioritization rules in TS 38.213.

Proposal 15. Add UEIRI to the prioritization order for transmission power reduction to ensure consistent handling with other UCI types when the maximum transmit power is exceeded.

Proposal 16. Adopt Text proposal #8 in Annex A to TS 38.213.

UE behavior when Current beam is the highest

When the UE is configured with an event-triggered report for Event 2 or Event 7 with enabledCurrentBeamReport-r19, there can be situations where the measured current beam has the highest quality compared to the new beams after UE determining event triggering according to event evaluation. This situation can occur under several configurations and operating conditions. For example, the configurable threshold value for Event 2 ranges from 0 to 31 dB, the window length from 4 to 1280 ms, and the number of counters from 2 to 16. It may also arise when timeRestrictionForChannelMeasurements is configured, or when only one new beam (N = 1) is configured, in which case it may happen frequently. Furthermore, at a center frequency of 3.5 GHz with 30 kHz SCS (μ = 1) and a UE velocity of 60 km/h, the coherence time is roughly 2 to 3 slots, making such occurrences more likely. In such a case, the reported differential value for the current beam may be incorrect, as the reporting procedure is designed under the assumption that the current beam is not the highest.

A possible solution without dramatic change is to introduce a 1-bit “out-of-range” indicator to inform the gNB when the reported measurement on the current beam is out-of-range. When the UE is configured with an event-triggered report for Event 2 or Event 7 with enabledCurrentBeamReport-r19, the UE reports the indicator as 1 when the measured current beam is the highest, indicating that the reported differential value is incorrect, and reports the indicator as 0 when the measured current beam is not the highest, indicating that the reported differential value is correct. This explicit signaling allows the gNB to correctly interpret the reported measurement and maintain accurate beam quality tracking even when the current beam outperforms all new beams.

Proposal 17. Introduce a 1-bit “out-of-range” indicator in UE-initiated/event-driven CSI reporting to indicate whether the measured current beam is the highest compared to the new beams when enabledCurrentBeamReport-r19 is configured for Event 2 or Event 7.

Proposal 18. Adopt Text proposal #9 in Annex A to TS 38.214 and TS 38.214.

Proposal 19. Adopt Text proposal #10 in Annex A to TS 38.214 and TS 38.212.

Conclusion

This contribution has discussed the remaining issues related to the physical layer aspects of enhancement for UE-initiated/event-driven beam management. The following are our proposals:

Proposal 1. Propose the revised FL proposal, updated as follows:

Proposal 1.2: Regarding triggering event determination for Event 2, besides for Candidate#2, at least Candidate #1 and Candidate#7 are additionally supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - Reset a new beam removed by the RS reconfiguration/update (the new beam not in the updated list any longer). - Do not reset a new beam in both old and new list (the new beam is not changed in the updated list). - Start counting for a new beam added to the list (the new beam not in the updated list). - Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - The RRC parameter(s) at least comprises the threshold, time window and maximum event instance count for event evaluation - In such case, the UE need to reset the counting for all new beams.

Proposal 2. Adopt Text proposal #1 in Annex A to TS 38.214.

Proposal 3. The following conditions can be considered for cancellation of a triggered UE-initiated/event-driven beam reporting for Event 2:

- Update of the indicated TCI state

- Transmission of UE-initiated CSI report

- BWP switching

Proposal 4. Adopt Text proposal #2 in Annex A to TS 38.214.

Proposal 5. Regarding the triggering event determination for Event 1 and Event 7, the following conditions can be considered for resetting the counter for Event 1 and Event 7:

- For Event 1: Update of the indicated TCI state

- For Event 7: Update/change of the activated TCI state with the Q-th best quality

  - In Event 7, the event instance(s) counting is per new beam.

Proposal 6. Adopt Text proposal #3 in Annex A to TS 38.214.

Proposal 7. Specify necessary enhancements to address beam failure recovery when UE is configured with UE-initiated/event-driven beam report procedure at least based on Event 1 or Event-2.

- For example, the UE-initiated/event-driven beam report procedure is not performed (or is suspended) during an ongoing beam failure recovery procedure for a cell. (e.g., no condition evaluation, no first PUCCH and/or second PUSCH transmissions) until the beam failure recovery is completed or a new TCI is applied for the cell.

Proposal 8. Adopt Text proposal #4 in Annex A to TS 38.214.

Proposal 9. Specify UE behavior of UE-initiated/event-driven beam report procedure (e.g., condition evaluation, first PUCCH transmission, second PUSCH transmission, and so on) from a completion of a beam failure recovery procedure until the indicated TCI is updated.

- The candidate beam of the beam failure recovery is used for current beam in UEBIR.

Proposal 10. Adopt Text proposal #5 in Annex A to TS 38.214.

Proposal 11. On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, reuse the multiplexing rules of PUSCH with aperiodic CSI report for PUSCH with UE-initiated CSI report.

- For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and UE-initiated CSI report(s) on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition.

- When a DCI format 0_1 schedules two PUSCH allocations, the UE-initiated CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the UE-initiated CSI report is carried on the penultimate scheduled PUSCH.

Proposal 12. Adopt Text proposal #6 in Annex A to TS 38.214.

Proposal 13. RAN1 agrees to support one of the following approaches to ensure that the UE can transmit the first PUCCH and/or second PUSCH for UEI-based CSI reporting:

- Alt 1: When the UE triggers UE-initiated CSI reporting, if the PUCCH resource and/or Type 1 CG PUSCH resource used for UE-initiated CSI reporting are not configured in the active UL BWP(s), the UE performs BWP switching to an UL BWP where the first PUCCH and/or second PUSCH are configured.

- Alt. 2: If the PUCCH resource and/or Type 1 CG PUSCH resource used for UE-initiated CSI reporting are not configured in the active UL BWP(s), UE does not perform event-instance evaluation.

Proposal 14. Adopt Text proposal #7 in Annex A to TS 38.214.

Proposal 15. Add UEIRI to the prioritization order for transmission power reduction to ensure consistent handling with other UCI types when the maximum transmit power is exceeded.

Proposal 16. Adopt Text proposal #8 in Annex A to TS 38.213.

Proposal 17. Introduce a 1-bit “out-of-range” indicator in UE-initiated/event-driven CSI reporting to indicate whether the measured current beam is the highest compared to the new beams when enabledCurrentBeamReport-r19 is configured for Event 2 or Event 7.

Proposal 18. Adopt Text proposal #9 in Annex A to TS 38.214 and TS 38.214.

Proposal 19. Adopt Text proposal #10 in Annex A to TS 38.214 and TS 38.212.

References

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung

- R1-2504982, “Introduction of NR MIMO Phae 5”, Ericsson

- R1-2504989, “Introduction of Rel-19 NR MIMO Phae 5”, Huawei

- R1-2504973, “Introduction of NR MIMO Phae 5”, Samsung

- R1-2504998, “Introduction of UE-initiated/event-driven beam management”, Nokia

- R1-2504473, ”Moderator Summary #3 on UE-initiated/event-driven beam management”, Moderator (ZTE)

- R1-2504471, ”Moderator Summary #1 on UE-initiated/event-driven beam management”, Moderator (ZTE)

Annex A

Text proposal #1

==================================unchanged omitted=================================== 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. The counter of the event instances for such reference signal is reset and the timer for such reference signal is stopped: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated, or- if eventInstanceCount, or eventDetectionTimeWindowLength is reconfigured by higher layers, or - if newBeamResourceSet is reconfigured by higher layers and such reference signal is not among reference signals provided by the updated newBeamResourceSet. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. ==================================unchanged omitted===================================

Text proposal #2

==================================unchanged omitted=================================== 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, a UE-initiated CSI report triggered for the reference signal is considered to be pending. If at least one UE-initiated CSI report for the reference signal is pending, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, a UE-initiated CSI report triggered for such reference signal is considered to be pending. If at least one UE-initiated CSI report for the reference signal is pending, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. The UE cancels the pending UE-initiated CSI report triggered for such reference signal: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated, or - if a CSI report including such reference signal is transmitted; or - if the active downlink BWP of the CC (the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig) is switched. The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. ==================================unchanged omitted===================================

Text proposal #3

==================================unchanged omitted=================================== 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated if eventType is set to ‘event1’ or ‘event7’, or - if the reference signal with the valueOfQ highest L1-RSRP out of the reference signals among the activated TCI states or the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH blocks QCLed with the reference signals among the activated TCI states is updated if eventType is set to ‘event7’. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. ==================================unchanged omitted===================================

Text proposal #4

==================================unchanged omitted=================================== 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. Upon detection of a beam failure on the CC (the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if eventType is set to ‘event1’ or ‘event2’, the UE: - cancels, if any, pending UE-initiated CSI report; and - stops event-instance determination until a new TCI states is applied for the CC. ==================================unchanged omitted===================================

Text proposal #5

==================================unchanged omitted=================================== 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType. After 28 symbols from completion of a beam failure recovery for the CC (the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig) as described in Clause 5.17 of [11, TS 38.321] and before a new TCI state is applied for the CC , if eventType is set to ‘event1’ or ‘event2’, the candidate reference signal for the beam failure recovery as described in clause 6 of [6, TS 38.213] is used for event instance determination instead of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. ==================================unchanged omitted===================================

Text proposal #6

==================================unchanged omitted=================================== ### 5.2.3 CSI reporting using PUSCH A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state. A UE shall perform aperiodic CSI reporting using PUSCH on the serving cell with the smallest serving cell index scheduled by DCI format 0_3 which triggers an aperiodic CSI trigger state. When a DCI format 0_1 or DCI format 0_3 schedules two PUSCH allocations on the serving cell, the aperiodic CSI report or CSI report with CSI-ReportConfig with eventType and reportTransmissionMode configured as ‘ModeA’ is carried on the second scheduled PUSCH. When a DCI format 0_1 or DCI format 0_3 schedules more than two PUSCH allocations on the serving cell, the aperiodic CSI report or CSI report with CSI-ReportConfig with eventType and reportTransmissionMode configured as ‘ModeA’ is carried on the penultimate scheduled PUSCH. ==================================unchanged omitted=================================== #### 6.1.2.1 Resource allocation in time domain ==================================unchanged omitted=================================== For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or CSI report with CSI-ReportConfig with eventType and reportTransmissionMode configured as ‘ModeA’ or activates semi-persistent CSI report(s) on PUSCH with no transport block by a 'CSI request' field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) or CSI report with CSI-ReportConfig with eventType and reportTransmissionMode configured as ‘ModeA’ by a 'CSI request' field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a 'CSI request' field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213], and Clause 5.34.3 of [10, TS 38.321]. For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s), or CSI report with CSI-ReportConfig with eventType and reportTransmissionMode configured as ‘ModeA’ on PUSCH by a 'CSI request' field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration. ==================================unchanged omitted===================================

Text proposal #7

Alt. 1:

==================================unchanged omitted=================================== 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE - starts a timer for such reference signal from the initial value equal to the time window length provided by eventDetectionTimeWindowLength and sets the counter to 1, if the timer for such reference signal is not running; or - increments the counter for such reference signal by 1, if the timer for such reference signal is running. If the number of event instances determined by the counter for such reference signal is greater than or equal to eventInstanceCount, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. If the PUCCH resource configured by PUCCHResource in the CSI-ReportConfig is not configured for the active uplink BWP of the CC (provided by pucchCell, if configured, in the CSI-ReportConfig), the UE switches/sets the active uplink BWP to the uplink BWP configured with the PUCCH resource. ==================================unchanged omitted=================================== 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 ==================================unchanged omitted=================================== - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. If the type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB is not configured for the active uplink BWP of the CC as the corresponding CSI-ReportConfig, the UE switches/sets the active uplink BWP to the uplink BWP configured with the type 1 CG-PUSCH. The reference signal in the indicated TCI state is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. The UE does not expect that the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is in a different CC from the reference signals configured by newBeamResourceSet. 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 ==================================unchanged omitted=================================== - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode-r19 is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. If the type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB is not configured for the active uplink BWP of the CC as the corresponding CSI-ReportConfig, the UE switches/sets the active uplink BWP to the uplink BWP configured with the type 1 CG-PUSCH. The reference signal in the indicated TCI state is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. The UE does not expect that the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is in a different CC from the reference signals configured by newBeamResourceSet. 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 ==================================unchanged omitted=================================== - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. If the type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB is not configured for the active uplink BWP of the CC as the corresponding CSI-ReportConfig, the UE switches/sets the active uplink BWP to the uplink BWP configured with the type 1 CG-PUSCH. The reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in such activated TCI state. The UE does not expect that the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals is in a different CC from the reference signals configured by newBeamResourceSet. ==================================unchanged omitted===================================

Alt. 2:

==================================unchanged omitted=================================== 5.2.1.5.4.1 UE Initiated CSI reporting For a UE configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, without eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList, when an event instance is determined (as described in the following clauses), - for a reference signal configured by the newBeamResourceSet, if eventType is set to ‘event2’ or ‘event7’, or - for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI, if eventType is set to ‘event1’, the UE transmits UEIRI on a PUCCH format 0 or format 1 in the PUCCH resource (in the CC provided by pucchCell, if configured, in the CSI-ReportConfig) configured by PUCCHResource in the CSI-ReportConfig. The UE performs event instance determination when: - the PUCCH resource configured by PUCCHResource in the CSI-ReportConfig is configured for the active uplink BWP of the CC (provided by pucchCell, if configured, in the CSI-ReportConfig), and - if reportTransmissionMode is configured as 'ModeB, the type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB is configured for the active uplink BWP of the CC as the corresponding CSI-ReportConfig. ==================================unchanged omitted===================================

Text proposal #8

==================================unchanged omitted=================================== - 7.5 Prioritizations for transmission power reductions For single cell operation with two uplink carriers or for operation with carrier aggregation or for operation with a candidate cell configured by LTM-Config or for operation with a single cell configured with sTx-2Panel, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cell(s) or on a candidate cell, if any, in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cell(s) or on a candidate cell, if any, in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If the UE transmits SRS on multiple SRS resources according to Clause 6.2.1.4 of [6, TS 38.214], the UE allocates power so that all REs of the SRS transmission have same power. For the purpose of power allocation in this clause, if a UE is provided uci-MuxWithDiffPrio and the UE multiplexes HARQ-ACK information in a PUSCH, a priority index of the PUSCH is the larger of (a) the priority index of the PUSCH according to clause 9 and (b) the larger priority index of the HARQ-ACK information. When determining a total transmit power for serving cells or a candidate cell, if any, as described in Clause 21 in a frequency range in a symbol of transmission occasion i, the UE does not include power for transmissions starting after the symbol of transmission occasion i. The total UE transmit power in a symbol of a slot is defined as the sum of the linear values of UE transmit powers for PUSCH, PUCCH, PRACH, and SRS in the symbol of the slot. - PRACH transmission on a candidate cell, if any, as described in Clause 21 - PRACH transmission on the PCell - PUCCH or PUSCH transmissions with larger priority index - For PUCCH or PUSCH transmissions with same priority index - PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, and/or UEIRI, or PUSCH transmission with HARQ-ACK information of the priority index - PUCCH transmission with CSI or PUSCH transmission with CSI - PUSCH transmission without HARQ-ACK information of the priority index or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell - If the UE is configured with prioSCellPRACH-OverSP-PeriodicSRS-r17 - Aperiodic SRS transmission or PRACH transmission on a serving cell other than the PCell - Semi-persistent and/or periodic SRS transmission - otherwise, - SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell ==================================unchanged omitted===================================

Text proposal #9

==================================unchanged omitted=================================== 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state and out-of-range indicator, indicating whether the reported differential RSRP for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is out-of-range within the reporting range, as defined in Table 10.1.6.1-2 of [TS 38.133]. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. The reference signal in the indicated TCI state is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. The UE does not expect that the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is in a different CC from the reference signals configured by newBeamResourceSet. ==================================unchanged omitted=================================== 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal with the valueOfQ highest L1-RSRP out of the reference signals among the activated TCI states (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block with the valueOfQ highest L1-RSRP out of the SS/PBCH block s QCLed with the reference signals among the activated TCI states (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals and out-of-range indicator, indicating whether the reported differential RSRP corresponding to the reference signal with valueQfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueQfQ highest L1-RSRP out of activated TCI state reference signals is out-of-range within the reporting range or, as defined in Table 10.1.6.1-2 of [TS 38.133]. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH. The reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals is the reference signal w.r.t. QCL-TypeD, if there are two QCL RSs in such activated TCI state. The UE does not expect that the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals is in a different CC from the reference signals configured by newBeamResourceSet. ==================================unchanged omitted===================================

Text proposal #10

==================================unchanged omitted=================================== - 6.3.2.1.2 CSI ==================================unchanged omitted=================================== The bitwidth for CRI, SSBRI, RSRP, differential RSRP, CSI report configuration indicator, Condition met indicator and Out-of-range indicator are provided in Table 6.3.2.1.2-14. Table 6.3.2.1.2-14: CRI, SSBRI, RSRP, CapabilityIndex, CSI report configuration indicator, Condition met indicator and Out-of-range indicator Field; Bitwidth. CRI; log2(KsCSI-RS). SSBRI; log2(KsSSB). RSRP; 7. Differential RSRP; 4. CSI report configuration indicator; log2(KsCSI_config). Condition met indicator; 1. Out-of-range indicator; 1. NOTE: KsCSI-RS is the number of CSI-RS resources in the corresponding resource set, KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP' or 'ssb-Index-RSRP-Index', KsCSI_config is the number of CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR.. ==================================unchanged omitted=================================== Table 6.3.2.1.2-3I: Mapping order of CSI fields of one report for CSI report configuration indicator/CRI/RSRP/Condition met indicator, or CSI report configuration indicator/SSBRI/RSRP/Condition met indicator CSI report number; CSI fields. CSI report #n; CSI report configuration indicator as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #1 as in Table 6.3.2.1.2-14. CRI or SSBRI #2 as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #3 as in Table 6.3.2.1.2-14, if reported. CRI or SSBRI #4 as in Table 6.3.2.1.2-14, if reported. RSRP #1 as in Table 6.3.2.1.2-14. Differential RSRP #2 as in Table 6.3.2.1.2-14, if reported. Differential RSRP #3 as in Table 6.3.2.1.2-14, if reported. Differential RSRP #4 as in Table 6.3.2.1.2-14, if reported. Condition met indicator #1 in Table 6.3.2.1.2-14, if reported. Condition met indicator #2 in Table 6.3.2.1.2-14, if reported. Condition met indicator #3 in Table 6.3.2.1.2-14, if reported. Condition met indicator #4 in Table 6.3.2.1.2-14, if reported. RSRP as in Table 6.3.2.1.2-14 for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state if eventType is set to ‘event1’ and enabledCurrentBeamReport is configured; Differentatial RSRP as in Table 6.3.2.1.2-14 for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state if eventType is set to ‘event2’ and if enabledCurrentBeamReport is configured; Differentatial RSRP as in Table 6.3.2.1.2-14 for the reference signal with the valueOfQ highest L1-RSRP out of reference signals among the activated TCI states or the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of refence signals among the activated TCI states if eventType is set to ‘event7’ and if enabledCurrentBeamReport is configured;. Out-of-range indicator as in Table 6.3.2.1.2-14, if reported. Zero padding bits OP, if needed. NOTE: The CSI report configuration indicator with value i indicates the (i+1)-th CSI report configuration among the CSI report configurations associated with a same PUCCH resource configured by firstPUCCHResourceConfig-UEIBR, which are ordered in ascending order of CSI-ReportConfigID. The condition met indicators are reported only when the UE is configured with higher layer parameter PresenceOfConditionMetIndicator. If the reference signal associated with CRI or SSBRI #j satisfies the condition for transmitting UEIRI as defined in Clause 5.2.1.5.4.1 in [6, TS 38.214] when eventDetectionTimeWindowLength is configured, the value of the condition met indicator #j is set to 1; otherwise set to 0. The out-of-range indicator is reported only when the UE is configured with higher layer parameter enabledCurrentBeamReport and eventType is set to ‘event2’ or ‘event7’. If the RSRP for the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is the highest within the report, the value of the out-of-range indicator is set to 1; otherwise set to 0.. ==================================unchanged omitted===================================