3GPP TSG RAN WG1 #122                                                   	 R1-2505755

Bangalore, India, August 25th – 29th, 2025

Source:	OPPO

Title:	Overview of 6GR: A unified air interface with modular design

Agenda Item:	11.1

Document for:	Discussion

Introduction

The study objectives on 6G Radio are defined in SID [1].

In this paper, we present our views on high-level design principles, enabling technologies and key aspects of the 6GR study.

High-level design principles for 6GR

6GR air interface needs to be properly enhanced to support requirements and services of a new generation of mobile communication and information system.

In a word, the 6GR shall be able to balance between performance and cost/complexity with sufficient flexibility. To achieve this, the 6GR should provide scalability and modularity to efficiently accommodate existing and future services, especially the ones with vastly different usage scenario, performance requirements and life cycles e.g., eMBB and IoT. Additionally, while it is still vital to preserve the quality of existing services like voice and data, the capability to natively support unprecedented new services e.g., AI, sensing and data services will be pivotal to the business success of 6G.

Some high-level design principles taking the goals of 6GR into consideration are discussed in the following part.

- Minimization of options - 5G compatible PHY parameters as a starting point

6GR should strive to accommodate different usage scenarios within the same (flexible) technical framework. For example, 6G MBB and 6G IoT should aim for reusing 6GR baseline waveform and frame structure. This principle also applies to 6G Sensing and 6G NTN. And there is no need to design a dedicated technical solution for 6G HRLLC. Additional waveforms could be considered only in case of significant gain.

5G NR compatible PHY parameters including waveform, numerology, modulation, channel coding, etc. could form a stable basis for 6GR to minimize workload and thus allow companies to focus on technologies which could bring substantial gain. 6G infra and device implementation could also benefit from existing and proven designs to greatly reduce R&D cost, time to market etc.

- Enhance performance with reasonable complexity

Performance, especially spectrum efficiency, is still a key target for 6GR design. In contrast, the peak data rate does not reflect real-world user experience and thus is not a good performance indicator.

Some foreseeable performance enhancements include but not limited to: MIMO enhancements, Coding/modulation enhancements, Duplex enhancements, and AI/ML for 6G RAN.

High-order MIMO with reduced RS overhead may be one of the main approaches to achieve high 6G throughput, while balance between number antenna ports and device complexity is the key.

AI empowered enhancements e.g. Superimposed pilot design (SIP), Joint source channel coding for CSI feedback (JSCC-CSI) and Joint Modulation and precoding seem very promising to improve performance over traditional approaches.

- Provisions on native PHY support of new services

New services like AI/ML and sensing are unprecedented to 3GPP systems and involve fundamental changes to capabilities the PHY needs to offer. For example, AI/ML services may involve physical layer data collection and transfer between PHY entities for the purposes of model training and fine-tuning, and sensing (mono-static, bi-static or cooperative) may require enhancements on cancellation and duplexing. Thus, provisions on the native support of these candidate new services need to be taken into consideration from the early phase of 6G study to ensure a highly-efficient and cost-effective PHY design.

- Scalability and modularity as a foundation of PHY design

Lessons learned from 5G tell us that an all-out broadband system deployment approach may not be cost-effective or is even financially unsustainable considering the interaction between network coverage, new device penetration rate and the time needed for new services to popularize. Thus 6GR, as the largest investment needed to provide mobile information services, needs to think “out of the box” and find a way to fulfill various service requirements while maintaining a reasonable cost/complexity of both the network and the UE, as well as be able to provide sufficient scalability/expandability for future possibilities.

Therefore, the “modular design” concept takes shape. Here different features provided by the 6GR are organized into different “functionality sets” in a bottom-up way. For example, a basic functionality set may be used to support fundamental services and low-end IoT, and when higher performance is required or different usage scenarios emerge, additional functionality sets like the MBB or vertical may be applied on top of the basic functionality set. Thus, the requirements of cost/complexity and scalability/expandability could be met.

Therefore, a modular design can be taken for 6GR air interface, as shown in Figure 1. A low-cost and energy-efficient “Basic function set” can be designed for 6GR air interface, which serves as a "common basis" for various usage scenarios. For a specific usage scenario, a full functionality set can be designed which “grows up” from Basic functionality set.

Figure 1. Modular design of 6GR

From our perspective, 6G MBB (i.e. Immersive Communication) and 6G IoT (i.e. Massive Communication) are most important usage scenarios for the 1st release of 6G system. And each 6G MBB device are naturally a 6G IoT device in “low-data rate” mode. For example, when a smartphone is in a “screen off” status or communicates with IoT devices, only the low-rate “background data” are received and transmitted, only “IoT-like” traffic is conveyed for the smartphone. Therefore, the 6G IoT provides a proper design benchmark for 6G Basic functionality set. And the Basic functionality set should be designed to fully support 6G IoT operation. Meanwhile, the Basic functionality set is also designed to support low-data rate energy-efficient mode of 6G MBB. With this, the Basic functionality set is the common basis shared by 6G IoT and 6G MBB. Beyond it, a MBB full functionality set can be designed to support high-data rate mode of 6G MBB operation. This a unified 6GR air interface can be designed, scalable from 6G IoT to 6G MBB.

The design principle for modular 6GR air interface can be:

- Unified and scalable from 6G IoT to 6G MBB, by designing a Basic functionality set and an MBB full functionality set.

- The Basic functionality set is designed to fully support 6G IoT operation and support low-data rate energy-efficient mode of 6G MBB. Growing up from the Basic functionality set, an MBB full functionality set can be designed for high-data rate mode of 6G MBB operation.

- The Basic functionality set also serves as common basis shared by all other 6G vertical usage scenarios. Growing up from the Basic functionality set, corresponding Vertical full functionality sets can be designed for each 6G verticals, e.g., 6G sensing, 6G NTN, 6G HRLLC.

- The Basic functionality set is mandatory for all types of UEs over all 6G usage scenarios. The mandatory features in MBB full functionality set the are mandatory for all 6G MBB UEs. The mandatory features in a Vertical full functionality set the are only mandatory for the corresponding 6G vertical UEs (e.g. mandatory features in Sensing full functionality set are only mandatory for 6G Sensing UEs).

More detailed, two types of Vertical full functionality sets can be considered:

- Type 1: Vertical full functionality set growing up directly from the Basic functionality set (as shown in right hand side of Figure 1).

For example, 6G Sensing and 6G NTN (i.e. Ubiquitous Connectivity), due to their different targets in air interface design compared to 6G MBB (i.e. Immersive Communication), their air interface needs to be designed and optimized for sensing performance and the satellite communication performance respectively. The two Vertical full functionality sets may not completely reuse MBB full functionality set, although some designs can be shared if applicable.

- Type 2: Vertical full functionality set growing up from the MBB full functionality set (as shown in left hand side of Figure 1).

For example, 6G HRLLC (High Reliability Low Latency Communication), due to its similar targets in air interface design with 6G MBB (such as higher transmission data rate), can be designed with a 6G HRLLC full functionality set based on 6G MBB full functionality set, by enhancements for low latency and high reliability. But considering the 5G URLLC has not be large-scaled deployed, which shows the unclear market demand for 6G HRLLC, the enhancements may not be treated in the 1st release of 6G standards. The basic 6GR air interface design (e.g. waveform, numerology/frame structure, channel coding, modulation, basic MIMO operation, initial access, basic DL/UL control channels) should not be impacted by 6G HRLLC requirements. The potential enhancements, if justified by future market demand, can be considered in later releases of 6G standards, e.g., by improving control channel/scheduling design.

Proposal 1: The design principle for modular 6GR air interface can be:

- Unified and scalable from 6G IoT to 6G MBB, by designing a Basic functionality set and an MBB full functionality set.

- The Basic functionality set is designed to fully support 6G IoT operation and support low-data rate energy-efficient mode of 6G MBB. Growing up from the Basic functionality set, an MBB full functionality set can be designed for high-data rate mode of 6G MBB operation.

- The Basic functionality set also serves as common basis shared by all other 6G vertical usage scenarios. Growing up from the Basic functionality set, corresponding Vertical full functionality sets can be designed for each 6G verticals, e.g., 6G sensing, 6G NTN, 6G HRLLC.

- The Basic functionality set is mandatory for all types of UEs over all 6G usage scenarios. The mandatory features in MBB full functionality set the are mandatory for all 6G MBB UEs. The mandatory features in a Vertical full functionality set the are only mandatory for the corresponding 6G vertical UEs (e.g. mandatory features in Sensing full functionality set are only mandatory for 6G Sensing UEs).

As the usage scenario providing the Basic functionality set in 6GR, the 6G IoT should provide the following functionalities/capabilities:

- Fulfil the requirements of low-cost IoT.

- Higher than NB-IoT and lower than RedCap, e.g., support [3-5]MHz operation.

- Basic functionalities for all usage scenarios, e.g.,

  - Full coverage,

  - Basic access and control channels,

  - Low data rate transmission,

  - Basic QoS (basic latency, reliability performance).

- Native low-power, and naturally serve as energy-efficient mode of 6G MBB.

Figure 2: Definition of 6G IoT capability (i.e. Basic functionality set)

Proposal 2: 6G IoT should provide the following functionalities/capabilities (i.e. Basic functionality set):

- Fulfil the requirements of low-cost IoT.

- Higher than NB-IoT and lower than RedCap, e.g., support [3-5]MHz operation.

- Basic functionalities for all usage scenarios, e.g.,

  - Full coverage,

  - Basic access and control channels,

  - Low data rate transmission,

  - Basic QoS (basic latency, reliability performance).

- Native low-power, and naturally serve as energy-efficient mode of 6G MBB.

The candidate technologies enabling the 6GR can be shown in Figure 3. A stable basis is desired for leveraging technical innovations in other areas of 6GR, which includes waveform, numerology/frame structure, channel coding and modulation reusing 5G NR as the baseline. Beyond that, the better performance can be achieved with enhancements on MIMO, channel coding, modulation, duplex and supports of AI/ML algorithm. Besides necessary technologies enabling new services (e.g. AI/ML and sensing), the better cost and power efficiency are also essential for 6GR. To realize this, a “modular” system design building up from the Basic functionality set is needed, in which the initial access and control signaling should be designed in “modular” manner.

Figure 3: Potential enabling technologies for 6GR

Proposal 3: Consider to design a stable basis to leverage technical innovations in other areas of 6GR.

- Reuse 5G NR as the baseline for 6GR waveform, numerology/frame structure, channel coding and modulation.

Physical Layer basis for 6GR

Waveform for 6GR

6G waveform, as the basis of 6GR physical layer, should be designed following the above overall design principle for 6GR air interface. That is:

- Design a baseline waveform for unified 6GR to fulfil the requirements of 6G MBB and 6G IoT, which show clear market demand for 1st release 6G.

- 6G HRLLC is not in the consideration for designing the baseline waveform due to unclear market demand. 6G HRLLC in 1st release completely reuses the baseline waveform designed for 6G MBB and 6G IoT.

- Reusing the baseline waveform for other 6G verticals is highly desired. However, the necessity of introducing an additional waveform to fulfil the performance requirement for the verticals (e.g. Sensing) can be studied. If significant gain can be justified for the vertical scenario, the additional waveform can be considered. But the additional waveform is only supported by the vertical BS/UE, not other types of BS/UE.

Figure 4: Unified 6GR baseline waveform

Proposal 4: A unified 6GR baseline waveform is studied to fulfil the requirements of 6G MBB (Immersive Communication) and 6G IoT (Massive Communication).

- The baseline waveform is used for 6G HRLLC.

Proposal 5: Study waveforms to fulfil the requirement of 6G Sensing and 6G NTN (Ubiquitous Connectivity).

- Strive for reusing the 6GR baseline waveform for 6G Sensing and 6G NTN.

- An additional waveform can be considered if significant gain over the baseline waveform can be justified for a specific vertical scenario, but only supported by the vertical BS/UE.

Potential proposals for 6GR baseline waveform could be categorized into two types:

- Enhanced OFDM-based waveform for higher spectrum efficiency:

Proposals in this category may lead to a “joint design” combining waveform with other components in the PHY processing chain, e.g., joint OFDM and MIMO transmission. The concerns about this type of proposals would be the constraints to MIMO and modulation/coding.

- Enhanced OFDM-based waveform for better coverage:

Proposals in this category may improve the power efficiency of the OFDM transmission by lowering the PAPR (Peak-to-Average Power Ratio). Some low-PAPR OFDM scheme can be implemented without specification impacts, and can be supported in a specification-transparent way. If the proposal needs to bring some specification impacts, OFDM with windowing/filtering may sacrifice the scheduling flexibility in time-domain or frequency-domain, which leads to spectrum efficiency loss on system level.

Proposal 6: For studying the 6GR baseline waveform, evaluate waveform proposals using 5G NR waveform (i.e. CP-OFDM for DL and CP-OFDM/DFT-s-OFDM for UL) as the benchmark, with the consideration of:

- Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc.

- Support flexible frequency-domain (e.g. RB-level) and time-domain (e.g. symbol-level) resource allocation.

- Support of efficient 5G/6G spectrum sharing.

Numerology and frame structure for 6GR

With the similar consideration with that for waveform, the following proposals can be made for 6GR numerology and frame structure.

Figure 5: 6GR baseline numerology

Proposal 7: Study a set of baseline numerology and a baseline frame structure to fulfil the requirements of 6G MBB (Immersive Communication) and 6G IoT (Massive Communication).

- The baseline numerology is used for 6G HRLLC and 6G NTN (Ubiquitous Connectivity) in Rel-21.

Proposal 8: Study numerology and frame structure to fulfil the requirement of 6G Sensing.

- Study the necessity of additional numerology or frame structure for 6G Sensing in case the same waveform as for 6G MBB and 6G IoT is used.

  - Substantial gain over the baseline numerology should be justified.

For the baseline SCS which supports 6G IoT and 6G MBB, the performance of 15kHz*2n has been justified in the 4G and 5G network deployments. Reusing the SCS set for 6GR is the reasonable approach for baseline numerology, and can easily support 5G/6G coexistence. Other SCS values can be studied, and potentially be considered only if substantial gain and 5G/6G coexistence solution are justified.

Considering the reality that URLLC services have not yet deployed in 5G network, which shows the unclear market demands on low-latency 6GR, the service-dependent SCS should not be considered in the 1st release of 6G standard. So, the only valid consideration is to keep the band-dependent flexible SCS. But granularity for SCS flexibility needs to be considered.

For the maximum Channel Bandwidth (CBW), a larger value than that in 5G NR (i.e. 100MHz) is expected to increase the data rate for 6GR. Meanwhile, complexity on UE-side needs to be considered, taking account of HW capability of 6G devices. We see 200MHz is a practical maximum CBW for 6GR in FR1 and FR3. A larger CBW requires excessive UE complexity and cost, which should not be considered in first release of 6GR.

The minimum CBW depends on the capability of 6G IoT. According to the analysis in Section, the 6G IoT needs to support [3-5]MHz operation bandwidth. Thus the 6GR should support [3-5]MHz for minimum CBW.

Proposal 9: For 6GR baseline numerology,

- Subcarrier spacing (SCS) compatible with 5G NR (i.e. 15kHz*2^n) is considered as starting point.

- Study necessity of extended CP (incl. review of deployment scenarios).

- Only a single subcarrier spacing (SCS) is supported for PDSCH/PDCCH or PUSCH/PUCCH after RRC connection per [FFS: carrier, CC, band, frequency range].

- Consider 200MHz for maximum CBW and up to 8K for FFT size, and [3-5] MHz for minimum CBW.

For 6GR, support of multi-beam transmission and the flexible multiplexing between channels are still valid motivations for symbol-level resource allocation and configuration. And both semi-static and dynamic symbol-level scheduling should be supported for designing the slot structure.

In 5G NR, a simple frame structure is defined for a simple function – timing and synchronization. We do not the need to re-design it for 6GR. The only potential change is to remove the sub-frame concept.

For study of 6G frame structure, the impacts of advanced duplexing should be taken into consideration to strive for unified frame structure design for various duplex types and enabling forward compatibility for more advanced duple types. But considering network and UE implementation complexity and the work load, only semi-static SBFD is introduced for 5G NR.

Proposal 10: For 6GR baseline frame structure,

- Slot consisting of 14 symbols is considered as starting point, with symbol-level resource allocation and configuration supported.

- Reuse the 5G NR frame structure design as a starting point. Study the necessity of defining sub-frame and its use case.

- Study unified frame structure design for various duplex types, e.g., TDD, FDD and BS-side SBFD. Forward compatibility for other advanced duplex types, e.g., FD, can be considered.

  - Support semi-static assignment for transmission direction, e.g., semi-static TDD/FDD/SBFD.

- Reuse the baseline frame structure for 6G Sensing in case 6GR baseline waveform is reused.

  - New symbol structure can be studied for sensing-specific waveform (e.g. pulse wave) for mono-static sensing.

Channel coding for 6GR

Channel coding is one area we do not expect many modifications from its 5G version, due to superior coding performance and mature chipset implementations of 5G channel coding. Meanwhile, there are indeed some aspects of 6G channel coding that are desirable to have generation-level improvements. The 6G channel coding study should focus on key improvements on LDPC code and Polar code.

Proposal 11: The 6G channel coding study should focus on potential improvements on 5G LDPC code and 5G Polar code.

LDPC coding

One aspect of LDPC improvement is to increase the LDPC code block size to enable higher traffic data throughput. One viable solution to increase the LDPC code block size is to increase the lifting size Z (currently no larger than 384). To balance between throughput improvement and implementation complexity, we target to increase the upper-bound of Z to the double of existing 5G value for BG1 and the 4-times of existing 5G value for BG2.

Proposal 12: Study the increase of LDPC code block size by allowing larger lifting sizes, e.g. up to a value that is 2~4 times of existing upper-bound (=384) in 5G.

Polar coding

The maximum number of information bits equal to 140 bits already caused some design restriction or led to additional design complexity in 5G, e.g., in the work-item of multi-carrier enhancement by using single DCI to schedule multiple cells. Therefore, it can be beneficial in 6G to increase this upper-bound of 140 bits. However, how large this new upper-bound needs to be should, in our view, take into account the study in control channel design, especially the DL/UL scheduling functionalities, which however can take places only starting from RAN1 #124.

Observation 1: The RAN1 experience in 5G shows it can be beneficial for 6G channel coding study to look at the increase of upper-bound of Polar coding information bits (currently equal to 140 bits).

- This study may take inputs from the Agenda Item 11.9 (Physical layer control, data scheduling and HARQ), which starts from RAN1 #124.

Modulation for 6GR

Modulation is one of the essential components of 6GR facing the next decade and beyond. As a consequence, modulation should be carefully studied in the early stage, leaving the room for future potential proposals and innovations. The bit-interleaved coded modulation (BICM) scheme has proved its superiority in achieving independent design between channel coding module and modulation module in existing system like LTE and 5G NR. For a long period of time, the classical Quadrature Amplitude Modulation (QAM) proposed in [2] has served as an attractive benchmark due to its balanced performance, controllable mapper/demodulation complexity and high energy efficiency. The classical QAM modulation with BICM is adopted in various cellular network and wireless local area network in past several decades, with thorough evaluations across all layers of radio interface, mature hardware implementations and algorithms. Therefore, the modulation scheme in 5GR NR, up to 1024QAM for DL and up to 256QAM for UL following BICM architecture, should be considered as baseline for the study of 6GR modulation scheme.

In 5G, up to 1024QAM is supported for DL and up to 256QAM is supported for UL. Higher order QAM can be considered for 6GR. It is straightforward that higher order QAM can boost the peak data rate. However, it shall be carefully studied with considering the feasibility of practical product implementation and chance of supporting higher order modulation in real field deployment.  For example, can the tight EVM requirement of higher order QAM be achieved by practical base station and UE implementation? What is the probability of channel condition in a real-field deployment cell that can support higher order QAM? All those factors shall be studied carefully for introducing higher order modulation.

Proposal 13: Modulation specified in 5G shall be the starting point for 6GR and study supporting higher order QAM for 6GR with considering the implementation feasibility of higher order modulation, e.g., EVM requirement and practical application chance.

For 6GR, the QAM modulation supported in 5G shall be starting point and baseline. On top of that, we can study non-uniform modulation to further enhance the modulation performance. Among the technologies for non-uniform constellation, the geometric shaping (GS) and probabilistic shaping (PS) have gained much attention due to the superior performance and controllable implementation complexity.

GS designs the non-uniformly spaced constellation points, while keeping the same probability of all modulation symbols. The GS relaxes the constraint of uniform spaced QAM constellation to non-uniform constellation (NUC). The distribution of transmitted signals can be approximately the optimal distribution (such as Gaussian distribution over AWGN channel) for the channel condition, which would improve the channel capacity and the error probability at each particular SNR. Our study showed that the performance gain of GS-based modulation compared with QAM grows with modulation order, from marginal gain for 16QAM to about 1.2 dB gain for both 256QAM and 1024QAM.

PS designs a non-equiprobable distribution of the constellation points while maintaining the uniformly spaced QAM constellation points. The basic idea of PS is simple, but the implementation of PS is pretty complicated. One additional distribution matcher (DM) module, which maps uniform equiprobable bit stream to non-equiprobable constellation points (symbols), is needed to realize PS-based modulation. PAS (probabilistic amplitude shaping) is one way to implement PS with DM, where the mapped bits of each modulation symbol are split into sign bits and amplitude bits. Our study showed that the PAS-based modulation scheme can achieve superior performance compared with uniform 256QAM modulation scheme in both AWGN channel and i.i.d. Rayleigh channel. Meanwhile, the implementation complexity and potential standardization efforts for PS shall also be considered carefully.

Proposal 14: Study the following modulation schemes as potential enhancement for modulation scheme in 6GR:

- GS-based modulation;

- PS-based modulation;

  - PAS-based modulation as one implementation method of PS.

- Note: performance gain, spec impact and hardware complexity should be taken into account for further study.

MIMO for 6GR

MIMO is essential part for 6GR system to provide commercial service. Compared to 5G, it is expected that larger antenna array and higher order MIMO transmission can be supported in 6GR. In Figure.4, we compare the spectrum efficiency of different antenna port number with different CSI types. It can be found that with straight extension of antenna port number, significant gain can be obtained in both cell-average and cell edge spectrum efficiency.

Figure 6: Potential performance gain with large antenna array in 6GR

In our view, the design of 6G MIMO shall take into the following criterions:

- The MIMO design shall be commercial application oriented. Only the features with commercial application prospect can be supported in 6G day 1;

- 6G MIMO shall only support the necessary configuration and option for each feature, and minimize the supported options;

- Strive to have a unified and simplified design for associated features, avoiding introducing related functions through different framework;

- Last but not least, new features for 6G should focus on new deployment scenarios (FR3) and new technology (AI), rather than solutions not supported in NR.

Proposal 15: 6G MIMO design shall be commercial needs oriented and only supports the necessary configuration/option. Consider unified design for all the associated features and focus on new scenarios and new technologies.

6G MIMO design shall include the DL and UL transmission schemes, reference signal design, CSI feedback and multi-beam and mTRP operation.

For DL and UL transmission scheme design, 6G MIMO shall support massive MIMO targeting higher spectral efficiency. For DL, at least 256 Tx and 8 Rx ports shall be considered. For UL, 4/8 Tx ports shall be considered. However, practical hardware restriction at both base station and UE shall be taken into account in the transmission scheme design.  The MIMO transmission schemes supported in 5G can be starting point and we shall strive to reuse the 5G transmission schemes. Uplink coherent transmission, which was not practically applied in 5G UEs, is expected to be supported in 6G UEs for better uplink coverage, considering both practical UE RF implementation and possible standardization methods. For uplink, the number of ports can be 1, 2, 3 and 4 (and possible 8) with or without full power transmission. In 5G, uplink has two transmission schemes: codebook-based and non-codebook based, which can be reused for 6G, considering possible signaling optimization, e.g., DCI overhead reduction or unified signaling design. In 5G, many mTRP transmission schemes were specified in multiple releases. In 6GR, we shall check those schemes closely and restrict the number of mTRP schemes. In our view, coherent joint transmission can be prioritized while the mTRP scheme in non-ideal backhaul scenarios can be deprioritized, considering the implementation issue.

Proposal 16: MIMO transmission schemes in 5G are the starting point for 6G MIMO and strive to limit the number of mTRP schemes.

Reference signal in 6GR shall include CSI-RS for DL CSI measurement, SRS for UL CSI and DMRS for demodulation. First of all, we shall avoid supporting too many options for each RS type, which is one lesson learnt in 5G. In 6GR, a unified design shall be supported for different number of ports of CSI-RS. FDM across PRBs can be considered for large number of ports, for example 128/256 ports. Since large number of ports would be supported for CSI-RS, overhead reduction shall be supported in 6GR. Both AI-based and non-AI-based methods can be considered. And overhead reduction in both frequency domain and spatial domain could be the starting point.

6G MIMO shall support both DL and UL DMRS for PDSCH and PUSCH. With increasing of DL antenna ports (e.g. 256 ports), up to 48 ports can be considered for DMRS design for MU-MIMO. A single DMRS type is preferred for DL/UL DMRS to simplify the DMRS design. One same design is used for both DL and UL DMRS to minimize the specification effort. Nested structure shall be considered to accommodate different number of DMRS ports. Overhead reduction shall also be considered for DMRS to improve the throughput, e.g., frequency sparse or time sparse DMRS.

Proposal 17: For CSI-RS, SRS and DMRS, RAN1 shall strive to unified design and single option for each RS type. Overhead reduction shall be considered for CSI-RS and DMRS considering potential large number of ports.

6G MIMO shall support a unified CSI feedback framework. It is essential to design the codebook for larger number of antenna ports and achieving better performance. We can consider unified codebook for both single TRP transmission and coherent joint transmission, and restrict the number of codebooks. Downloadable codebook/basis can be further studied, which is expected to provide potential performance gain. The CSI report procedure and signaling can be simplified to support more efficient system operation. 5G supported periodic, semi-persistent and aperiodic CSI reporting based on RRC/MAC CE/DCI through PUSCH or PUCCH. For 6G, if some simplification is needed, semi-persistent and aperiodic CSI reporting in PUSCH through DCI triggering can be prioritized.

Proposal 18:  For 6G MIMO CSI feedback:

- Consider codebook design for large number of antenna ports and unified design for both single TRP transmission and coherent joint transmission;

- Consider to simplify the CSI reporting procedure and signaling.

6G MIMO can support multi-beam operation and multi-TRP system based on unified TCI state framework. Unified TCI framework shall support the smooth mobility and flexible multi-TRP operation. A lean design for unified TCI framework is considered and all the non-essential optimizations for example default QCL shall be avoided. 5G specified many different multi-TRP schemes in different releases and they were implemented with different framework, which should be avoided in 6G. For multi-beam operation, the resource overhead and time used for beam sweeping would consume large amount of system resource and also might cause large latency to multi-beam operation. Mechanisms to minimize the resource and time requirement for multi-beam measurement can be considered for 6G. Beam failure recovery and UE-initiated beam reporting were supported in 5G. For 6G, a single framework can be considered for UE-initiated beam management, which at least includes both functions of beam failure recovery and UE-initiated beam reporting.

Proposal 19: 6G MIMO shall consider unified TCI for smooth mobility and flexible multi-TRP operation. Strive to support low overhead/latency beam measurement/reporting and a unified UE-driven multi-beam operation.

AI/ML for 6GR

The field of wireless communication has recently witnessed a surge of interest in AI-Intrinsic (AII) approaches, with both academic and industry efforts to explore its potential.

Figure 7: AI/ML new use cases on 6GR transceiver chain

When selecting AI/ML use cases for 6GR, several critical factors should be seriously evaluated to ensure meaningful advancements. ​​First, AI/ML-Intrinsic design fundamentally reimagines the transceiver chain (the core Tx-Rx components illustrated in Figure 7) by integrating AI/ML technologies at its foundation. Second​​, we should prioritize use cases that deliver ​​non-marginal performance improvements​​, as incremental gains may not justify the transition from legacy approaches. ​​Third​​, it is necessary to balance performance benefits, computational complexity, and power consumption, particularly for UE-side models. Specifically, we need to ensure that the computational load (in FLOPs) remains feasible for real-world deployment.

Proposal 20: Consider the following principles to select AI/ML use cases for 6GR study:

- Prioritization of AI/ML-Intrinsic design that impacts the basic components of transceiver chain.

- Significant performance benefits for intermediate metrics (e.g. SGCS, NMSE, or predication accuracy) and final metrics (e.g. BLER or throughput) over legacy non-AI schemes.

- Well-balanced tradeoff among performance benefits, computation complexity and power consumption.

In Rel-20, the study of AI/ML for 6GR can be conducted in two stages. For the 1st stage, RAN1 could take the first 3 meetings (i.e. 25’ Q3 and Q4) to briefly select the categories of AI/ML use cases. For instance, assume the RS overhead reduction as a category, but we don’t have to limit its AI/ML solutions which can be left for the 2nd stage. Due to limited time in Stage 1, there seems no sufficient time for companies to carry out comprehensive performance evaluations. Hence, we will do the selection based on popularity and potentiality in RAN1. For the 2nd stage, RAN1 will carefully study those use cases selected in Stage 1 and possibly carry out down selection at the end of Rel-20 study. Hence, we would like to suggest the following timeline.

Proposal 21: Suggested timeline to select AI/ML use cases for Rel-20 6GR

- Stage 1: 25’Q3 & 25’ Q4

  - Briefly select generic categories of AI/ML use cases by popularity and potentiality in RAN1

- Stage 2: 26’Q1 to 27’ Q2:

  - Comprehensive study of all use cases selected in Stage 1 and carry out potential down selections

When it comes to 6GR, we expect that the use cases selected would be more diverse than NR. Not only CSI-related use case(s), but also some transmission schemes or RS overhead reduction may be studied in Rel-20. From this sense, we should avoid building LCM framework to facilitate CSI-related use cases only. As the diversity of AI/ML use cases matters, one may expect a few different aspects of 6GR to be studied. It would be complicated to study LCM-related procedures for each use case. Therefore, it calls for an unified LCM framework.

Proposal 22: Strive for unified LCM framework for 6GR, including at least the following aspects:

- Data collection for model training and fine-tuning.

- UE-side and/or NW-side model monitoring.

- Model paring (for two-sided model) and model identification.

- LCM-related operation, e.g. model switch, fallback, activation/deactivation.

Initial access for 6GR

Specific requirements for 6GR initial access

As discussed in Section 2, the better cost and power efficiency are also essential for 6GR. To realize this, a “modular” system design building up from the Basic functionality set is needed, in which the initial access and control signaling should be designed in “modular” manner. Regarding the 6GR modular initial access, the Basic functionality set provides the basic initial access for 6G IoT and energy-efficient mode of 6GR MBB, e.g., an energy-efficient initial access in a smaller bandwidth. While the 6GR MBB Full functionality set provides the high-performance initial access for 6GR MBB, e.g., a low-latency initial access.

According to the GSMA report [2], the energy cost of mobile networks accounts for ~23% of the total cost of operators. Energy consumption has become a critical part of operators' operating expenses. Therefore, in the later stage of 5G, 3GPP has realized the importance of network energy conservation for environmental sustainability, and successively introduced a series of network energy saving functions in Rel-18 and Rel-19 standards, including, supporting on-demand SSB (On-demand SSB) in SCell, on-demand SIB1 transmission, cell and UE DRX/DTX alignment, etc. In addition, the energy efficiency of UE is crucial for a mobile communication system frequent charging causes inconvenience to users.

Therefore, joint energy saving of networks and terminals is one of the main design goals of 6GR networks, which should be taken as the design goal of modular 6GR initial access at Day 1 as well. Meanwhile this goal should be considered moderately and in balance in 6GR initial access design. For example, reducing or even completely turning off the transmission of always-on signals can save the energy consumption of the network to the greatest extent, but this will lead to the time and energy consumption of the initial cell search of the UE. Therefore, in the design process of 6GR initial access, network energy conservation and terminal energy conservation should be balanced, and at least the energy consumption and complexity of UE initial access will not increase with the introduction of network energy saving technology.

In addition, in LTE and 5G, the initial access is designed with reference to the capabilities of consumer-grade UE, and the final initial access mechanism cannot effectively support IoT UE, or even cannot be applied to IoT UE. To prevent this situation from happening again initial access mechanism for 6GR should be able to support all UE types foreseen. However, it is also important to avoid another extreme case, where the system is over-optimized for IoT UE at the expense of the efficiency of consumer-grade UEs.

Enhanced overall coverage, focus on cell-edge performance and UL coverage, is one of RAN requirement of 6GR as stipulated in the 6G SID RP-251881, which should also be considered in initial access. But the schemes for cell-edge performance enhancement should not undermine the performance of cell center UEs, or, sacrifice too much resource efficiency. Coverage performance considers both TN and NTN.

Proposal 23: The following specific requirements should be moderately considered in 6GR initial access:

- Energy efficiency for both network and device.

- Enhanced overall coverage, focus on cell-edge performance and UL coverage.

- Target scalable and forward compatible design for diverse device types.

SSB design

In 5G communication systems, SSB (Synchronization Signal Block) is the key for synchronization between UE and base station. In order to simplify the cell searching process and improve network access efficiency, SSB in 5G can only be transmitted on sync raster. The granularity of synchronization raster has to satisfy: synchronization channel raster = minimum channel bandwidth – SSB bandwidth + channel raster, so as to ensure that there is at least one SSB in any channel. The subcarrier spacing of SSB in 5G is typically 15kHz or 30kHz in FR1, and the bandwidth of SSB corresponds to 3.6 MHz and 7.2 MHz, respectively. Before Rel-18, the minimum channel bandwidth supported by 5G was 5 MHz, so the channel bandwidth was greater than the SSB bandwidth. Although 3GPP supports 3 MHz channel bandwidth in Rel-18, the structure of SSB was not redefined, instead the SSB is directly punctured to adapt to the 3 MHz channel bandwidth.

In order to satisfy the design goals of the initial 6GR access, as a next-generation mobile communication system, the SSB in 6GR needs to consider the following requirements:

- In terms of the basic functions of SSB, including time synchronization, frequency synchronization, PCI indication, and MIB transmission, etc., the overall performance of 6GR SSB should be better than 5G SSB, at least close to the performance of 5G SSB.

- As mentioned above, the joint energy saving of network and terminal is one of the important goals of 6GR network, and the basic functions of SSB should be ensured in the design process of 6GR SSB without increasing the energy consumption of network and terminals.

- Since the initial stage of 6GR may not be able to accurately predict the exact capabilities of IoT type UEs required by the future market, the design process of 6GR SSB should leave more room for the introduction of lower capacity UEs in the future.

- It can fit into the smallest possible channel bandwidth, avoiding the situation of punching SSB to accommodate smaller channel bandwidth as in 5G.

Based on the above considerations, the bandwidth of SSB can be further reduced on the basis of 5G SSB in 6GR. For example, assuming that the 15 kHz SCS can still be used on the operating frequency band of the 6GR system, the bandwidth of the SSB can be reduced to 3 MHz or 2.16 MHz, and the sequence length of PSS/SSS will not be affected, so as to ensure time-frequency synchronization performance and PCI indication capability, and the smaller SSB bandwidth will surely provide more room for supporting 6G IoT UE and smaller channel bandwidths. In terms of energy consumption, according to the sync raster division rules mentioned above, when the bandwidth of SSB is reduced to 3 MHz or 2.16 compared to 3.6 MHz in NR, the granularity of synchronization raster can be increased to achieve an energy-saving gain of 33% or 54% at UE side [3], assuming that the transmission period of the SSB remains unchanged. The energy-saving gain of the UE is mainly achieved by reducing the number of SSB detections and the cell access delay, so it is also possible to appropriately increase the duty cycle of the SSB to convert some of the UE energy-saving gains into network energy-saving gains. However, if the 15 kHz subcarrier spacing is not available on the main operating frequency band of 6GR, such as in the ~7 GHz band, the length of PSS/SSS and the bandwidth of SSB need to be evaluated more if minimum channel bandwidth smaller than 5MHz is pursued.

After reducing the bandwidth of SSB, the number of OFDM symbols of PBCH can still be maintained as 2, as shown in the left figure in Figure 8, which is conducive to increasing the flexibility of 6GR system deployment. As the resources in the SSB for basic PBCH become less than NR, more compact MIB than NR should be pursued, otherwise, suffering from ~3dB loss as shown in Figure 8 (right) below. “Additional PBCH” can be transmitted in addition to basic PBCH to enable faster cell searching for consumer-grade UE or improve the cell edge performance.

Figure 8: Candidate SSB structure with smaller BW for 6GR (left) and PBCH performance comparing to NR (right)

Proposal 24: The following requirements should be considered for 6GR SSB design:

- The overall performance of 6GR SSB should be at least close to the performance of 5G SSB.

- Pursue high energy efficiency, at least not worse than 5G NR.

- Support operation of 6G IoT UEs.

- It can fit into the minimum CBW.

Random access

In the 6GR communication system, the number of terminals in general will further increase, especially IoT terminals, which will periodically or occasionally trigger random access, and insufficient PRACH capacity will lead to access conflicts and latency surges. Therefore, further increasing the capacity of PRACH channels in 6GR should be considered.

In addition, the operating frequency band of 6GR will generally be higher than that of 5G, so it will face greater signal attenuation. If NTN needs to be supported in 6GR Day 1, PRACH also needs to adapt to ultra-long distances (satellite-ground links) and high Doppler frequency shifts. These new demands mean that further PRACH coverage will be needed in 6GR.

In the LTM (Lower-layer Triggered Mobility) introduced by Rel-18, as long as the target base station and the source base station are synchronized, the UE can measure the TA relative to the target base station according to the time advance (TA) in the source cell. 5G NTN also supports this feature. So, the UE can access the network without sending PRACH (i.e., PRACH-free random access), and the uplink resources of the UE are provided through dynamic scheduling or semi-static scheduling. PRACH-free random access is also a feasible feature for 6GR systems, for example, for UEs residing in small cells, there is no need to add TA in uplink transmission. If 6NB can manage resource conflicts between UEs, random access without PRACH can also reduce latency and improve resource efficiency for random access. For UEs that allow random access without PRACH, it can transmit the first message (message U1) through a randomly selected PUSCH resource, in which the information in Msg.3 can be sent directly.

Proposal 25: The following requirements should be considered for 6GR random access:

- Increased capacity of PRACH channels.

- Enhanced coverage.

- Fast and resource-efficient access, e.g., with PRACH-less random access.

Physical layer control, scheduling and HARQ for 6GR

As discussed in Section 2, the better cost and power efficiency are also essential for 6GR. To realize this, a “modular” system design building up from the Basic functionality set is needed, in which the initial access and control signaling should be designed in “modular” manner. Regarding the 6GR modular control, the Basic functionality set provides the basic control for 6G IoT and energy-efficient mode of 6GR MBB, e.g., an energy-efficient PDCCH/PUCCH. While the 6GR MBB Full functionality set provides the high-performance PDCCH/PUCCH for 6GR MBB, e.g., a high-capacity control channel/signaling.

DL control

UE energy consumption for DL control reception is always the key part for energy efficiency. Reducing its power is particularly how 5G enhancements try to realized, especially for connected mode.

The DL control for 6G should be a more energy efficient channel instead of making the detection of it with too much unnecessary power waste. In the traditional control channel, multiple detection hypotheses severed for user multiplexing, link adaptation and different DCI sizes. Multiple purposes of the hypotheses cost lot of UE detection power. For 6G down link control, that detection effort can be significantly reduced. Then, the new design of DL control can achieve much better energy efficiency.

Regarding the exact design of energy efficient DL control, we can consider 2 stage schemes. The full detection of DCI is indicated by compact information. That information could be simple as ON/OFF. It could be carried by waveform for lower power receiver. It is noted that the, 5G Low Power wake-up signal already had good study on the waveform. It is feasible to integrate the similar signa with DL control to make a simple and energy efficient channel.

Proposal 26: The design of DL control in 6G shall natively support the modular structure to meet Energy efficient operations. PDCCH should consider 2 stage indications.

- First stage with compact indication and lower power consumption for detection, corresponding to low power module.

- Second stage is corresponding to full power module and detect full information of control, by facilitation of the First stage.

UL control

Up to five PUCCH formats, dynamic HARQ-ACK timing and symbol level UCI transmission are supported in NR. The original intention of these designs is to improve the flexibility and adaptability of the system, but it also brings some complexity and challenges which will result in higher costs and increased power consumption for NW and UEs, including:

- Complex PUCCH resource allocation and determination to adapt to multiple PUCCH formats, variable UCI payloads, the number of DCI indication bits, etc.

- Complex HARQ-ACK codebook determination based on dynamic timing set(s).

- Complex UCI multiplexing/collision handling.

  - 5G define very complex and dynamic multiplexing timeline to ensure that UE can implement the multiplexing without introducing much scheduling restrictions. The dynamic multiplexing timeline complicates both NW and UE implementation.

  - 5G designs lengthy pseudo codes to determine overlapping PUCCH group, which is very complex for both UE/NW implementation and specification.

  - When there is overlapping between PUCCHs and PUSCHs, UCI is to be multiplexed on PUSCH. 5G dynamic timeline leads to large dynamic range of UCI payload size and as a result, NW may always wait for the critical deadline to know how many resources are needed for UCI transmission and in such a case pre-planning and pre-scheduling of PUSCH would be impossible.

- Larger DCI overhead to indicate information for PUCCH transmission in each DL grant, including HARQ-ACK timing, PUCCH resource, DAI, etc.

- Define multiple kinds of scheduling and HARQ order to avoid out-of-order processing in UE while supporting flexible scheduling and HARQ-ACK timing.

- Complex scheduling implementation for NW to simultaneously allocate the resource for downlink data and UCI.

The design of UL control in 6G should achieve low complexity and low power consumption for NW and UEs. First, 6G should consider defining an appropriate number of uplink control channel formats to balance flexibility while avoiding the complexity issues.

Second, for 6G HARQ-ACK feedback design, the following principles can be considered:

- Sparse HARQ-ACK transmission to improve UL efficiency and reduce UE/NW power consumption.

- Decoupling HARQ-ACK resource allocation from downlink scheduling.

Third, in order to reduce the NW/UE implementation and specification complexity brought by dynamic multiplexing timeline and overlapping based UCI multiplexing, it is proposed to consider simplified UCI multiplexing mechanism, e.g., slot-level multiplexing without overlapping judgment in 6G.

Proposal 27: The design of UL control in 6GR should achieve low complexity and low power consumption for NW and UEs.

- The number of PUCCH formats in 6G should be less than that in 5G;

- 6G HARQ-ACK transmission mechanisms satisfying the following principles:

  - Sparse HARQ-ACK transmission to improve UL efficiency and reduce UE/NW power consumption.

  - Decoupling HARQ-ACK transmission from downlink scheduling.

- Slot-level UCI multiplexing should be supported in 6GR.

HARQ operation

In NR, one TB can be segmented into multiple CBs and uniform CB grouping is supported, where consecutive CBs are grouped into one CBG, and each CBG corresponds to one HARQ-ACK bit. In practice, the decoding results of each CB in one TB have correlation which is impacted by: resource mapping types, physical resources occupied by CBs, frequency selection and time-varying characteristics of channels, etc. However, the correlation between decoding results among multiple CBs has not been fully explored in NR CBG.

To minimize the probability of correctly decoded CBs being expressed as NACK within a group, non-uniform CBG can be considered in 6G, thereby achieving:

- Improve the retransmission efficiency by reducing the redundant retransmissions.

- AMC adjustment based on more accurate CBG-level HARQ-ACK can improve transmission efficiency and reduce the impact of cliff effect.

Based on the simulation results shown in Figure 9, compared to NR TB based feedback and retransmission, non-uniform CBG based feedback and retransmission (i.e. 8-bit codebook) can achieve 7%~9% gain, and compared to NR uniform CBG based feedback and retransmission, non-uniform CBG based feedback and retransmission can achieve up to 3% gain.

Figure 9: Simulation results of non-uniform CBG based retransmission

Based on the simulation results shown in Figure 10, compared to NR TB based AMC, non-uniform CBG based AMC can achieve 3%~7% gain, and compared to NR uniform CBG based AMC, non-uniform CBG based AMC can achieve up to 4% gain.

Figure 10: Simulation results of non-uniform CBG based AMC

Proposal 28: Non-uniform CBG should be considered in 6GR to achieve higher transmission efficiency.

Small-payload DL Data optimization

Also, for efficient data scheduling/transmission, the data with small payload would be better to directly transmitted. Small data packet is more typical for asymmetric traffic scenarios. For higher layer feedback packet, it could be possible that few hundreds of bits only be used. With that payload, it is inefficient to have both control and scheduled data resource to finish single packet transmission. Instead, the packet can be carried by control resource. That will surely reduce the total efforts for small packet transmission/reception. In the same time, power efficiency and latency can be improved.

Proposal 29: Consider the small-payload DL data transmission with higher transmission efficiency. The DL small packet can be directly transmitted over control resources.

Duplexing for 6GR

Background of 5G duplex

5G supports TDD and FDD since the first release, i.e., release 15. In addition, to achieve flexible transmission direction change for time resources to adapt to various use cases, deployment scenarios and traffic load, dynamic TDD is also supported in 5G Rel-15. However, in conventional TDD operation, the time resource is split between downlink and uplink, and a limited time duration is allocated for the uplink, resulting in insufficient UL coverage/capacity and large UL latency. Based on that, 5G studied and supported subband non-overlapping full duplex (SBFD) at the gNB side in Rel-18 and Rel-19. Considering network and UE implementation complexity and the work load, only semi-static SBFD is introduced.

Key Considerations for 6GR duplex

For study of various duplex modes in 6G, many aspects should be considered, e.g.,

- Deployment scenarios and traffic variations: The motivation to study the duplexing mode in 6G is mainly to adapt to various deployments and traffic variations. For example, the DL and UL traffic is always unevenly distributed over different geographic areas, and even within same geographic area, the traffic is also variable in time domain. In addition, different DL/UL traffic patterns maybe observed in different UE types including the traditional mobile phone, XR devices, IoT devices and so on. Moreover, both wide-area macro deployments and local area deployments need to be considered and same or different duplex mode can be deployed for different scenarios.

- Cross-operator interference: The study of duplex mode should take coexistence of frequency resources among multiple operators into consideration such that potential cross-operator interference causing by duplex is controllable.

- Transmission performance, e.g., throughput, coverage and latency: The study of duplexing mode in 6G should take both UL and DL transmission performance into account, from both network perspective and UE perspective.

- Self-interference and cross link interference management: for some advanced duplexing mode, e.g., SBFD, full duplex, simultaneous uplink transmissions and downlink receptions are allowed in a same time duration, and in such a case, self-interference management in the corresponding device should be considered in the feasibility analysis. Moreover, both gNB-to-gNB and UE-to-UE cross link interference are observed and the related mechanisms to reduce CLI needs to be studied accordingly.

- Network and UE implementation complexity: Last but not least, in order to support some advanced duplexing mode, network and UE implementation complexity is obviously increased to achieve enough isolation capability and meet the RF/demodulation requirement. Considering the 6G design should target at green and energy-saving design, the device implementation complexity becomes a very important aspect, and maybe the bottleneck for the duplexing study.

Proposal 30: Study potential duplex mode for 6GR with consideration of the following aspects:

- Deployment scenarios and traffic variations.

- Cross-operator interference.

- Transmission performance, e.g., throughput, coverage and latency.

- Self-interference and cross link interference management.

- Network and UE implementation complexity.

From our perspective, at least the following duplex modes should be supported in 6G radio:

- TDD operation and SBFD operation from gNB perspective on an unpaired spectrum.

- FDD operation on a paired spectrum.

For SBFD operation at UE side, it is proposed to achieve FDD-like latency and coverage and further boost capacity for unpaired spectrum. Careful feasibility analysis is necessary to evaluate whether UE can mitigate the self-interference leakage via isolation techniques in many domains, e.g., antenna domain/spatial domain, RF domain, digital domain and frequency domain. The following targets should be considered:

- Target 1: To make sure that the UE receiver front end is not saturated: this is because there is always a limit on the maximum mean power that can be received at the UE antenna to guarantee that the received signal is within the linear working range of the LNA such that the RF front end is not blocked. Only the antenna domain isolation and potential RF domain isolation contributes to this target since this focus on the received signal before LNA.

- Target 2: To make sure that the SINR for the downlink signal is enough for data decoding: this is because the UL signal acts as the interference at baseband, which impacts UE maximum tolerance desensitization.

Proposal 31: Support the following duplex modes for 6G radio:

- TDD operation and SBFD operation from gNB perspective on an unpaired spectrum.

- FDD operation on a paired spectrum.

Proposal 32: If SBFD operation at UE is to be studied, careful feasibility analysis is necessary with consideration of the following targets:

- Target 1: To make sure that the UE receiver front end is not saturated.

- Target 2: To make sure that the SINR for the downlink signal is enough for data decoding.

Spectrum utilization and aggregation in 6GR

In FR1, operators have many narrow bandwidths as summarized in Table 5.5A.2-1 (intra-band) and Table 5.5A.3.1-1 ~ Table 5.5A.3.3-1 (inter-band) in TS 38.101 [4]. In 5G, the mentioned non-contiguous narrow bandwidths are used in legacy CA manner, that is, the fragmented resources will be treated as multiple cells, which will result in:

- Low spectrum efficiency due to large cell identification/management overhead, at least including multiple synchronization signals, multiple system information, and more DCI.

- UE capability is wasted by the inefficient handling of fragmented resources, as shown in Figure 11.

Figure 11: An example of inefficient consumption of UE capability

In addition, NR supports Tx/Rx switching based on CA capability to reuse RF chain resources, but the base band processing chain, e.g., the PDCCH monitoring capability, PDSCH processing capability, is not shared among multiple cells/carriers.

With the above analysis, in 6G design, it is necessary to explore effective strategies to optimize the utilization of these fragmented resources with limited UE processing chains (both RF and baseband). The following mechanisms can be considered:

Mechanism #1: Treat multiple non-contiguous frequency resources as one cell (i.e., single cell with multiple carriers) with the following benefits observed:

- Less system overhead: Less SS/PBCH/system information can be transmitted on fragmented carriers.

- Initial access offloading:

  - Flexible association of DL carrier(s) and UL carrier(s); and

  - Access can be flexibly performed on one or more carrier(s).

- Efficient scheduling:

  - Less DCI overhead.

  - Less PHY resource fragmentation caused by resource allocation.

  - Less L2 header/padding and L1 CRC overhead when larger TB is scheduled.

- Enhanced transmission performance: A single HARQ entity shared by carrier(s) within one cell can achieve lower latency.

- Optimal utilization of UE capability: The transmission on multiple carrier(s) can share UE capability at different levels, e.g., one RF chain, PDCCH detection capability, PDSCH processing capability, etc.

- Coverage enchantment: Flexible association of DL carrier(s) and UL carrier(s).

For single cell with multiple carrier(s), the following aspects need to be further studied:

- Whether single cell with multiple carrier(s) is applicable to intra-band scenario and/or inter-band scenario.

- The potential restrictions which should be satisfied for single cell with multiple carrier(s), e.g., the maximum bandwidth span for all the non-contiguous carrier(s), the maximum number of inter-bands if inter-band scenario is supported.

Mechanism #2: Carrier switching with single carrier processing capability and function shared by switched carriers, especially for:

- RF chain.

- Base band processing, e.g., Single PDCCH monitoring capability, PDSCH processing capability, etc.

- HARQ entity.

Figure 12: An example of carrier switch

Proposal 33: Study the following mechanisms to optimize the utilization of non-contiguous frequency resources for 6GR:

- Single cell with multiple carriers (SCMC).

- Carrier switch for UEs with single-carrier processing capability, for SCMC as well as CA scenarios.

  - Study functionalities/capability shared by switched carriers.

Proposal 34: For study of SCMC, the following aspects needs to be considered:

- The applicable scenario(s) for SCMC, e.g., intra-band, inter-band;

- The potential restrictions which should be satisfied for single cell with multiple non-contiguous frequency resources, e.g.,

  - The maximum bandwidth span for all the non-contiguous frequency resources.

  - The maximum number of inter-bands if inter-band scenario is supported.

- Consider to support operation of non-SCMC-capable UEs in a SCMC cell.

NTN for 6GR

Background of 5G NTN

With the standardization of 5G, Non-Terrestrial Networks (NTN) have become an integral part of global wireless communication systems. 5G NTN supports various satellite configurations, including geostationary (GEO), medium-earth orbit (MEO), and low-earth orbit (LEO) satellites, using transparent or regenerative payloads and typically operating in FDD duplex mode. Notable features standardized in 5G Release 17, 18 and 19 include uplink (UL) synchronization based on GNSS-capable UEs, HARQ-less transmission, timing relationship alignment for satellite delay, UL/DL coverage enhancements, network verified UE location and support for RedCap UEs. These features enable NTN to provide basic eMBB and IoT services over satellites.

However, several technical challenges persist. First, the coverage for GEO is still challenging in particular for initial access. In 5G LEO deployment was prioritized, which leaves this challenge to 6GR to address. Second, UE power wasting issue has been identified for NTN operation when the satellite has a small number of active beams but still intends to provide a wide coverage. 5G NTN applies a beam hopping operation, which mainly was up to satellite implementation. However, due to the limit of backward compatibility, such implementation will result in quite a lot of UE power wasting for monitoring and measurement. Moreover, with the constraint of limited number of active satellite beams, wide-beam SSBs are often considered in practice. This results in poor beam granularity during cell search, making it difficult for the network to direct the UE to a suitable DL beam for RACH operation. Third, the reliance on GNSS-based UL synchronization excludes UEs without GNSS capability, limiting service availability in GNSS-denied environments such as indoors or underground. Finally, deployment flexibility is constrained. To ensure cell discovery, operators often need to configure large coverage cells with fixed and frequent SSB periodicities (e.g., 20ms), which may not be optimal for all deployment scenarios and leads to inefficient spectrum and power usage.

Key Considerations and features for 6G NTN

First, we suggest that 6GR should consider the baseline NTN deployment scenarios. This includes supporting GEO/MEO/LEO satellites and multi/single-satellite beam per cell layout, with both transparent and regenerative payload transmissions. Regarding the duplex mode, FDD should be targeted at first. While new deployment scenarios such as TDD-NTN and spectrum sharing between TN and NTN or between 5G NTN and 6GR NTN are still lack of clear justifications. Not only will these new scenarios touch technical aspects, but also they may touch spectrum regulation aspects. As RAN1 is a WG dealing with specific low layer specifications, we prefer that RAN1 will not trigger the discussions on these new deployment scenarios before receiving a more clear confirmation on the feasibility or applicability from RAN plenary.

Proposal 35: The following deployment scenarios are considered for 6GR NTN as a starting point:

- FDD, GEO/MEO/LEO, multi/single satellite beam per cell layout with both transparent and regenerative payload.

  - No discussion for TDD-NTN in RAN1 until the deployment scenarios are identified on RAN plenary level.

- No discussion for spectrum sharing between TN and NTN in RAN1 until receiving the confirmation on their feasibility/applicability from RAN plenary.

- No discussion for spectrum sharing between 5G NTN and 6G NTN in RAN1 until deployment scenarios are identified.

Second, regarding the key features to be considered from Day-1 specification of 6G NTN, some NTN baseline features can be studied for the 6GR first release. The baseline features include uplink (UL) synchronization based on GNSS, HARQ-less transmissions, DL/UL timing relationship to avoid collision due to large satellite RTT.  and coverage enhancement via repetitions during initial access. To overcome the limitations observed in current 5G NTN implementations and to build a more efficient and inclusive non-terrestrial communication system, we recommend supporting:

Proposal 36: The following features should be studied for 6GR NTN:

- Refined beam management, enabling more accurate DL beam selection for random access procedure.

- Efficient beam hopping operations, which reduce UE wait time and power consumption.

- Robust DL/UL coverage for considered NTN deployment scenarios, in particular for GEO. Repetition and new waveform can be studied.

Third, to support GNSS-incapable UEs, we propose extending and generalizing the 5G R20 study on GNSS resilience. Specifically, 6G NTN could introduce network-based UL synchronization techniques, such as broadcasting positioning reference signals (PRS) to idle UEs to facilitate location estimation and timing acquisition without GNSS dependency.

Proposal 37: Study support of GNSS-incapable UE for 6GR NTN.

Last, 5G treats NB-IoT NTN in a separate RAT. Given that the IoT NTN service will be more likely present in the next 5-10 years compared to NR NTN service, the crucial question is whether IoT NTN service will be covered by 6GR or leave it to 4G evolution similar to 5G. From our perspective, the same decision should be taken, i.e., the services supported by 4G IoT NTN should be left to 4G evolution.

Proposal 38: The services supported by 4G IoT NTN should be left to 4G evolution.

Sensing for 6GR

For the study of ISAC in day-1 of 6GR, the relationship between the sensing and communication should be clarified at first. Since communication and sensing are about to be integrated into the same cellular system of 6GR, which is fundamentally a communication system, it is more appropriate to consider sensing as an add-on to the communication system. As a result, the commercialization of sensing will be facilitated if shared hardware design can be used for both communication and sensing. Specifically, to reuse communication for sensing is preferred.

Proposal 39: For 6G sensing study, consider the need of sharing common hardware for 6G communication and 6G sensing.

- Reuse 6G communication HW for 6G sensing is preferred.

Although a sensing signal not usable for communication purpose is allowable in 6G ISAC, cautions should be given for the potential impacts to the communication functions, procedures and even hardware. The potential impact may be exhibited in interference, resource competition, analog hardware (like ADC/AGC) distortion/blocking due to overwhelming high power signal entering RF unit, and etc.

Proposal 40: To ensure ​​coexistence of communication and sensing, strive to reduce impact on 6G communication from 6G sensing signal.

Moreover, as an add-on to the communication system, sensing may not have its own frame structure. Study for ISAC should assume the sensing signals to be multiplexed in the frame structure originally designed for communication. Note that sensing can have its own waveform/RS(s) that are different from communication ones, in order to get better coverage/sensing performance. These sensing specific waveform/RS(s) should smoothly fit into the communication frame structure with artful design. It is also assumed that communication frame structure is flexible enough to support the various sensing waveform/RS configurations.

The multiplexing compatibility between sensing signal and communication signal in a communication-oriented frame structure does not necessarily mean the sensing signal has to be used for communication purpose as well. In fact, it is not precluded to have radar-alike pulse signal to be transmitted in a communication frame structure and not usable by any communication functionality.

Another consideration for 6G ISAC is signal processing procedure for sensing measurement and report. Different sensing task require varying levels of sensing signal processing. Generally, sensing measurements can be systematically acquired at three hierarchical stages of the signal processing. First, full channel state information (CSI) is estimated as the foundational input for sensing operations. Second, range/doppler/angle profiles are generated via transform-domain processing. From this stage, inevitably, the signal processing procedure for sensing diverges from that for communication, but it is beneficial to share common basic processing unit between communication and sensing, e.g., FFT/IFFT, for receiver complexity and cost. Third, detection algorithms extract refined target parameters. All of the measurements obtained from these three stages can be listed as candidate measurements and reporting quantities, which may cause significant overhead, necessitating efficient compression or selective reporting methods.

Proposal 41: Study at least followings on physical layer design for ISAC:

- The 6GR study for ISAC is not targeted to design sensing-specific frame structure. Instead, it should be assumed that the sensing signals need to be multiplexed in the frame structure designed for 6G communication.

- The followings should be studied (at least including waveform and reference signal design):

  - A sensing signal that is exclusively used for sensing purpose, i.e., not being able to support any communication functionality.

  - A sensing signal that can be used for sensing purpose and communication purpose at the same time.

- Different levels of sensing signal processing in sensing receiver (e.g. raw data or processed data) should be considered to satisfy requirement of different sensing tasks.

Conclusion

The overview of our consideration on 6GR study is presented in this paper. The proposals in Section 2 - 11 are suggested to be considered in the 6GR study.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator),3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9th -13th, 2025.

- https://www.gsmaintelligence.com/research/5g-energy-efficiencies-green-is-the-new-black-the-sequel

- R1-2505761, Energy Saving for 6GR air interface, OPPO, 3GPP TSG RAN WG1 #122, Bangalore, India, August 25th - 29th, 2025.

- TS 38.101, NR; User Equipment (UE) radio transmission and reception.