3GPP TSG RAN WG1 #122	R1- 2505773

Bengaluru, India, August 25th – May 29th, 2025

Agenda item: 		8.6.1

Title:			Remaining issues relating LP-WUS and LP-SS design

Source: 			Nokia

Document for: 		Discussion and Decision

# Introduction

In this document, we will discuss about the remaining issues related to the design of LP-WUS and LP-SS design.

# LP-WUS Waveform Design

## DC Centering

During work item, it was agreed that if ZC sequence is used as an overlay sequence, then the need for OOK1 can be ignored as ZC sequence in time domain is the same as ZC sequence in frequency domain but with different root index.

Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4 Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) ,, M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, - where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement For M=2, 4, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any.

As the generation of LP-WUS is based on OOK4, which involves the DFT/FFT operation on the desired time domain signal, the size of the DFT used to find the frequency domain samples before IFFT processing along with adjacent NR signals must be fixed. As agreed in the previous meeting, two alternatives are considered for further selection in the next meeting, namely, the size of the DFT is matched to power of two or it can be the same as that of the number of tones used for LP-WUS useful BW.

It is typical that the sequences are pre-calculated at the gNB and stored in a look-up table (LUT) that can then be populated on to the desired tones dynamically depending on the underlying information bits, the DFT size does not impact the real-time performance of the gNB implementation, unless FFT/DFT processing is performed at each LP-WUS symbol instants, which is noticeably costlier. On the other hand, if LR intends to perform FFT based processing, the closest FFT size with a matching power of two can be used without affecting the detection performance significantly.

In RAN1#120bis meeting, there was a discussion on the use of DFT shift that ensures DC centering before mapping the corresponding DFT output samples to frequency domain tones. Even though it may not be needed in practice due to the presence of overlay sequence, it may be needed if the sequence is generated only using time domain representation with corresponding interpolation factor. Thus, it favors simple implementation and generation of LP-WUS overlays sequences in time domain.

- The sequence is specified in the time domain before DFT operation in 38.211 specification

- Depending on the sampling rate employed at the LR, the sequence as specified in 38.211 can be used with the interpolation factor determined as α=132MWUS×3.84e6Fs, where Fs is the LR sampling rate.

The sequence as specified in the specification 38.211 can be used to generate the desired sequence for correlation in the LR if DFT shift is employed, otherwise additional operation of -1n should be used before performing interpolation that is not specified.

RAN1 shall consider performing DFT-shift operation before mapping the LP-WUS samples to frequency domain REs after DFT operation.

Following text proposal for LP-WUS generation

##### ------------------- Text Omitted ------------------ ##### 7.4.4.1.2 Generation of rWUS(n) The block of complex-valued symbols rWUS(0),…,rWUS(MbitMZC-1) is defined by rWUSlNscWUS+k=1NscWUSi=0NscWUS-1-1i×rWUSlNscWUS+ie-j2πikNscWUS k=0,1,…,NscWUS-1 l=0,1,…,MbitMWUS-1 NscWUS=11NscRB ##### ------------------- Text Omitted ------------------

Correspondingly, LP-SS generation can be shown as

##### ------------------- Text Omitted ------------------ ##### 7.4.5.1.3 Generation of rLPSS(n) The block of complex-valued symbols rLPSS(0),…,rLPSS(NOOKMZC-1) is defined by rLPSSlNscWUS+k=1NscWUSi=0NscWUS-1-1i×rLPSSlNscWUS+ie-j2πikNscWUS k=0,1,…,NscWUS-1 l=0,1,…,NOOKMLPSS-1 NscWUS=11NscRB ##### ------------------- Text Omitted ------------------

## WUS actual duration and nominal MO duration for IDLE/INACTIVE and CONNECTED mode

In RAN1#122 it was agreed to support following ranges for the codeword lengths:

Agreement For code block (coded bits) length L after rate matching as defined in 38.212 section 7.4.2.1, support - For 1 information bit, L is in 1~32 - For 2 information bits, L is in 2~32 - For 3 information bits, L is in 3~32 - For 4 information bits, L is in 5~32 - For 5 information bits, L is in 6~32 - For M=4, the code block (coded bits) length L is even number. - Further restriction on the L value within the above range is not precluded. - Do not support repetition after rate matching

It was left open whether to limit further the possible range of the codeword lengths. As the codeword length determines together with the applied M value the actual LP-WUS duration (and vise-versa), this would relate to the actual WUS duration parameter (LP_WUS_Actual_WUS_duration_{IDLE/INACTIVE and CONNECTED}) as well as to the nominal MO duration (LP_WUS_NominalMO_duration_{IDLE/INACTIVE and} CONNECTED}).

In [1], we presented some analysis from minimum Hamming distance perspective for the codeword lengths (bits), and concluded that for information bits lengths {3,4,5}, codeword lengths Using [7,11,15,19,20,23,25,27,29,31] bits can achieve the maximum Hamming distance with shortest codeword length. While evidently longer LP-WUS duration provides more energy, and hence improved performance, this gives some indication that all codeword lengths are not necessary.

Another aspect to consider the possible limitation of the valid codeword/actual LP-WUS durations should consider the NR slot grid and other signals to facilitate the actual time domain multiplexing. While the UE can accumulate the (non-contiguous) symbols within the nominal MO duration, it would seem preferable to maintain configuration flexibility so that codeword length/LP-WUS actual duration would be consistent with the number of symbols within a slot e.g. accounting also the possibility reserve some symbols e.g. for PDCCH.

As the parameters (actual LPWUS duration for IDLE/INACTIVE and CONNECTED) are determined to be configured at symbol level, also the applied M-value and the Manchester encoding would need to be considered. Hence, accounting all possible information bit sizes, the considered L range would be from 1 symbol to 64 symbols (5 bits).

Therefore, we propose that at least following actual LP-WUS duration values (in OFDM symbols) are supported {2, 4, 6, 7, 8, 10, 12, 14, 16, 18, 20, 24, 28, 36, 42, 48, 56, 60, 64}.

Consider at least values {2, 4, 6, 7, 8, 10, 12, 14, 16, 18, 20, 24, 28, 36, 42, 48, 56, 60, 64} symbols for the actual WUS duration configuration (in LP_WUS_Actual_WUS_duration_{IDLE/INACTIVE and CONNECTED})

For nominal MO duration, accounting the actual WUS duration, considering at least full slot lengths (multiple of 14 symbols) would seem practical. In addition, for M=2 and M=4, nominal duration of less than slot could also be considered. Thus, for nominal MO duration the following values could be considered: {4, 7, 14, 28, 42, 56, 70, 84}

Consider at least values {4, 7, 14, 28, 42, 56, 70, 84} symbols for nominal MO duration (LP_WUS_NominalMO_duration_{IDLE/INACTIVE and CONNECTED})

# Conclusion

In this document, we presented our initial thoughts on the WI scope and the topics that are relevant to be considered and discussed during the WI phase.

## List of Proposals

Proposal 1:	RAN1 shall consider performing DFT-shift operation before mapping the LP-WUS samples to frequency domain REs after DFT operation.

Proposal 2:	Consider at least values {2, 4, 6, 7, 8, 10, 12, 14, 16, 18, 20, 24, 28, 36, 42, 48, 56, 60, 64} symbols for the actual WUS duration configuration (in LP_WUS_Actual_WUS_duration_{IDLE/INACTIVE and CONNECTED})

Proposal 3:	Consider at least values {4, 7, 14, 28, 42, 56, 70, 84} symbols for nominal MO duration (LP_WUS_NominalMO_duration_{IDLE/INACTIVE and CONNECTED})

## List of Observations

Observation 1:	The sequence as specified in the specification 38.211 can be used to generate the desired sequence for correlation in the LR if DFT shift is employed, otherwise additional operation of -1n should be used before performing interpolation that is not specified.

# References

- R1-2503694, LP-WUS and LP-SS design, Nokia, 3GPP TSG RAN WG1 #121, Valetta, Malta.