3GPP TSG-RAN WG1 Meeting #122	R1-2505799

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		8.1.1

Source:	Nokia

Title:	Maintenance on AI/ML Beam Management

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss our considerations for maintenance of Rel-19 AI/ML for beam management.

# UE-sided BM-Case1 & BM-Case 2

## Inference Operation

### Issue #1: Set B for BM-Case2

In RAN1#120bis meeting it was concluded that for BM-Case 2 inference, AP CSI-RS for Set B is not supported. However, AP CSI report for inference is still possible with SSB and P/SP-CSI-RS. For BM Case 2, the UE may use past SSB/CSI-RS measurements as input of the AI/ML model and may up to UE implementation to determine number of measurements on Set B. However, this approach may not be fully practical with AP CSI report, where the UE may require measuring and maintain past SSB/CSI-RS measurements without being triggered for AP report.

Reason for change: To avoid unnecessary UE measurements logging, it was proposed that the NW can configure/indicate the UE with measurement RS set (Set B) to measure at multiple-time instances, where it is still to be clarified how the UE is configured/indicated the number of measurements for Set B.

Summary of change: Introduce a configuration to indicate the UE the number of measurements (N) for Set B in BM Case-2 AP CSI report scenario.

Consequence if not approved: If this issue is not addressed, there may be inefficiencies in UE operations due to unnecessary logging of measurements, potentially leading to increased resource consumption.

Proposal 1: For UE-sided BM Case-2 AP CSI report configuration, endorse the following text proposal to 38.214 Clause 5.2.1.4.1.

5.2.1.4.1 Resource Setting configuration < Unchanged parts are omitted > For aperiodic CSI, a UE may be configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' and when nroftimeinstance-r19 is configured, or a UE is configured with a CSI-ReportConfig with reportQuantity-r19 is set to 'rs-pai-r19', the UE is not expected to be configured with aperiodic CSI Resource Setting. For a UE configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' , 'p-ssb-index-RSRP-r19', or 'none-bm-r19', the UE is not expected to be configured with more than 64 NZP CSI-RS resources and/or SS/PBCH block resources in the second Resource Setting given by resourcesForSetA-r19. For a UE configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE is not expected to measure the CSI-RS or SSB resources in the second Resource Setting given by resourcesForSetA-r19. For aperiodic CSI with periodic or semi-persistent CSI Resource Setting, when the UE configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and nroftimeinstance-r19 is configured, the UE shall consider at least N most recent measurement occasions, no later than the CSI reference resource, of CSI-RS or SS/PBCH resources associated with the First Resource Setting, where the value N is given by the higher layer parameter nroftimeinstanceSetB-r19. < Unchanged parts are omitted >

### Issue #2: AP-CSI-RS for Set A

AP CSI reporting for configuring Resource Set IDs may require a first CSI-RS Resource Set for RS resources to be measured (Set B) and a second CSI-RS Resource Set for RS resources not to be measured (Set A). Current AP trigger cannot carry information for both Resource Set IDs, leading to potential ambiguity.

Reason for change: It is not clear how the trigger for AP CSI Report can carry the information for two Resource Set IDs.

Summary of change: Support in AP CSI triggering state of an additional indication with Resource Set ID for RS prediction for Set A selection. The associated CSI-ReportConfig corresponding to a trigger state in CSI-AperiodicTriggerStateList shall indicate, in addition to the Resource Set ID for channel measurement, a second Resource Set ID for RS prediction.

Consequence if not approved: If this issue is not addressed, the UE may face ambiguity in determining RS resources to be predicted.

Proposal 2: For UE-sided BM Case-1 and 2, for AP CSI reporting for inference, endorse the following text proposal to 38.214 Clause 5.2.1.

5.2.1 Channel state information framework < Unchanged parts are omitted > For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR, CapabilityIndex, TDCP, L1-SRS-RSRP, L1-CLI-RSSI, SRS-RSRP-MRI, CLI-RSSI-MRI, CSI-PAI, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI, CJTC-Dd, CJTC-F, CJTC-Dd-F, CJTC-P, a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings and/or X≥1 ltm-CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings and/or Y≥1 LTM-CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs or one associated LTM-CSI-ReportConfig indicating the Resource Set IDs for channel and optionally for interference where a Resource Set for interference can only be present for a Report Setting given by a CSI-ReportConfig and a trigger state additionally contains one or more reportSubConfigId if the associated CSI-ReportConfig configured with a list of sub-configurations, as described in Clause 5.2.1.1. For P-CRI, P-SSBRI, and P-L1-RSRP, the associated CSI-ReportConfig corresponding to a trigger state in CSI-AperiodicTriggerStateList shall indicate, in addition to the Resource Set ID for channel measurement, a second Resource Set ID for RS prediction. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig or LTM-CSI-ReportConfig, and a trigger state additionally contain one or more reportSubConfigId if the associated CSI-ReportConfig is configured with a list of sub-configurations, as described in Clause 5.2.1.1. < Unchanged parts are omitted >

## Performance monitoring

In the latest version 19.0.0 of 38.214, the following text is captured.

##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: - for each of the nroftransmissionOccasion-r19 latest transmission occasion(s), where the latest transmission occasion of nroftransmissionOccasion-r19 latest transmission occasion(s), and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting, the UE shall: - perform L1-RSRP measurements for the configured CSI-RS resources, or SS/PBCH Block resources, associated with the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on L1-RSRP(s) measured CSI-RS resources, or SS/PBCH Block resources of the corresponding Resource Set; - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19).

### Issue #3: Periodicity of CSI-RS in a Resource Set for Set A

The current 3GPP standard specification 38.214 mandates that all CSI-RS resources within a single set must share the same periodicity, which may limit performance monitoring operations for AI/ML beam management.

Reason for change: The same-periodicity requirement restricts the flexibility needed for AI/ML beam management, particularly when dealing with subsets of resource sets that require different transmission periodicities.

Summary of change: Allow different CSI-RS resources within a single set to have varied time-domain transmission behaviors when the resources belong to resourcesForSetA-r19, which are used for prediction.

Consequence if not approved: If this issue is not addressed, there could be ambiguity in determining the periodicity of CSI-RS resources in Set A.

Proposal 3: For UE-sided BM Case-1 and 2, for CSI-RS resources in Set A, endorse the following text proposal to 38.214 Clause 5.2.2.3.1.

##### 5.2.2.3.1 NZP CSI-RS < Unchanged parts are omitted > The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s). The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration: - nzp-CSI-RS-ResourceId determines CSI-RS resource configuration identity. - periodicityAndOffset defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS. Except resources in resourcesForSetA-r19 used for predicted RS quantities reporting, aAll the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources. - resourceMapping defines the number of ports, CDM-type, and OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211]. - nrofPorts in resourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. - density in resourceMapping defines CSI-RS frequency density of each CSI-RS port per PRB, and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. For density 1/2, the odd/even PRB allocation indicated in density is with respect to the common resource block grid. - cdm-Type in resourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. < Unchanged parts are omitted >

### Issue #4: Aperiodic Reporting Settings for monitoring and inference reports

The current specifications allow monitoring for aperiodic CSI report inference using only the AP CSI report monitoring configuration. However, in the case where M > 1 (given by nroftransmissionOccasion-r19), the NW must trigger the AP report for inference M times in order for the UE to report the RS-PAI.

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

Reason for change: To ensure that monitoring results are properly calculated at the UE, it is to clarify that for AP Report for inference, the number of transmission occasions triggered by AP report for monitoring should be limited to M = 1. With this limitation, AP monitoring of AP inference is not useful, though there may be alternative ways to address this issue.

Summary of change: For AP Report for inference, limit the number of transmission occasions triggered by AP report for monitoring to M = 1.

Consequence if not approved: If this change is not implemented, there could inconsistencies in determining monitoring results at the UE.

Proposal 4: For UE-sided BM Case-2 AP CSI report configuration, endorse the following text proposal to 38.214 Clause 5.2.1.4.3b.

##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: < Unchanged parts are omitted > - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). When both the first and the second CSI Reporting Settings are aperiodic, the UE is not expected to configure with a value larger than 1 for nroftransmissionOccasion-r19. < Unchanged parts are omitted >

## Other aspects

### Issue #5: Associated ID for AP-CSI-RS resource sets

The current specification may create ambiguity when determining applicability report for Aperiodic CSI RS. Following legacy aperiodic Resource Settings, it can contain multiple Resource Sets and which Resource Set to use for a certain AP measurement and report is indicated in the aperiodic trigger state. As there can be multiple resource sets in CSI-ResourceConfig, assuming one associated ID may not be sufficient.

In this case, there could be two options:

- Option 1: limit the AP CSI-RS to a single Resource Set. With this limitation NW can configure one or two associate ID in the AP CSI-ReportConfig, similarly to AP CSI report with P/SP-CSI-RS.

- Option 2: assume multiple resource sets in CSI-ResourceConfig as in legacy. In this case, NW can configure multiple associated IDs each corresponding to a resource set in CSI-ResourceConfig. Some limitations may apply as NW may configure the multiple associated IDs with the same one or two values, corresponding to Set A/Set B.

Without this clarification, the UE may not accurately determine which A-CSI-RS resource set(s) are applicable and therefore there could be ambiguity in determining the applicability of the CSI-reportConfig for AP CSI report.

Proposal 5: For the configuration of the associated ID for AP-CSI-Report with AP-CSI-RS resource sets, support configuring multiple associated IDs each corresponding to a resource set in CSI-ResourceConfig.

### Issue #6: Counting of active CSI-RS resources

During the inference stage, the UE is not expected to measure resources in Set A, which are typically defined as CSI-RS resource sets intended for reporting purposes. However, under legacy rules, the UE still considers CSI-RS resources in Set A, along with their associated ports, as active. because there is no legacy behaviour defined for the active time durations of aperiodic, semi-persistent, or periodic CSI-RS resources configured as Set A. This lack of definition leads to ambiguous or undefined behaviour in the UE.

Summary of change: Given above points, in this text proposal we would aim to: i) address the challenges posed by the legacy rules for active CSI-RS resources in CSI reporting framework extended to AI/ML-related beam prediction and ii) eliminate inefficiencies in counting active NZP-CSI-RS resources to fully clarify the UE capabilities indication.

Consequence if not approved: Without this clarification, the gNB ability to utilize other CSI reports may be restricted by the presence of resources in Set A. Considering the Set A may include a large number of CSI-RS resources (e.g. up to 256) this limitation is significant.

Proposal 6: For the counting of the active CSI-RS resources, endorse the following text proposal to 38.214 Clause 5.2.1.6.

#### 5.2.1.6 CSI processing criteria < Unchanged parts are omitted > In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19' and the case in which corresponding aperiodic CSI-RS resources are only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When aperiodic CSI-RS resources are for reporting ‘p-CRI’ or ‘p-CRI-RSRP’, the UE does not expect to assume aperiodic CSI-RS resources as active. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS ((excluding the case in which semi-persistent CSI-RS resources are only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’), starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. The UE does not expect to receive an activation command for semi-persistent CSI-RS resource set if the CSI-RS resource set is only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’. For periodic CSI-RS (excluding the case in which periodic CSI-RS resources are only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’), starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. When the periodic CSI-RS resources are only for reporting ‘p-CRI’ or ‘p-CRI-RSRP’, the UE does not expect to assume periodic CSI-RS resources as active. < Unchanged parts are omitted >

# Conclusion

In this contribution, we discuss our considerations for maintenance of Rel-19 AI/ML for beam management, and we have the following proposals:

Proposal 1: For UE-sided BM Case-2 AP CSI report configuration, endorse the following text proposal to 38.214 Clause 5.2.1.4.1.

5.2.1.4.1 Resource Setting configuration < Unchanged parts are omitted > For aperiodic CSI, a UE may be configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' and when nroftimeinstance-r19 is configured, or a UE is configured with a CSI-ReportConfig with reportQuantity-r19 is set to 'rs-pai-r19', the UE is not expected to be configured with aperiodic CSI Resource Setting. For a UE configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' , 'p-ssb-index-RSRP-r19', or 'none-bm-r19', the UE is not expected to be configured with more than 64 NZP CSI-RS resources and/or SS/PBCH block resources in the second Resource Setting given by resourcesForSetA-r19. For a UE configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE is not expected to measure the CSI-RS or SSB resources in the second Resource Setting given by resourcesForSetA-r19. For aperiodic CSI with periodic or semi-persistent CSI Resource Setting, when the UE configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and nroftimeinstance-r19 is configured, the UE shall consider at least N most recent measurement occasions, no later than the CSI reference resource, of CSI-RS or SS/PBCH resources associated with the First Resource Setting, where the value N is given by the higher layer parameter nroftimeinstanceSetB-r19. < Unchanged parts are omitted >

Proposal 2: For UE-sided BM Case-1 and 2, for AP CSI reporting for inference, endorse the following text proposal to 38.214 Clause 5.2.1.

5.2.1 Channel state information framework < Unchanged parts are omitted > For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR, CapabilityIndex, TDCP, L1-SRS-RSRP, L1-CLI-RSSI, SRS-RSRP-MRI, CLI-RSSI-MRI, CSI-PAI, P-CRI, P-SSBRI, P-L1-RSRP, RS-PAI, CJTC-Dd, CJTC-F, CJTC-Dd-F, CJTC-P, a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings and/or X≥1 ltm-CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings and/or Y≥1 LTM-CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs or one associated LTM-CSI-ReportConfig indicating the Resource Set IDs for channel and optionally for interference where a Resource Set for interference can only be present for a Report Setting given by a CSI-ReportConfig and a trigger state additionally contains one or more reportSubConfigId if the associated CSI-ReportConfig configured with a list of sub-configurations, as described in Clause 5.2.1.1. For P-CRI, P-SSBRI, and P-L1-RSRP, the associated CSI-ReportConfig corresponding to a trigger state in CSI-AperiodicTriggerStateList shall indicate, in addition to the Resource Set ID for channel measurement, a second Resource Set ID for RS prediction. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig or LTM-CSI-ReportConfig, and a trigger state additionally contain one or more reportSubConfigId if the associated CSI-ReportConfig is configured with a list of sub-configurations, as described in Clause 5.2.1.1. < Unchanged parts are omitted >

Proposal 3: For UE-sided BM Case-1 and 2, for CSI-RS resources in Set A, endorse the following text proposal to 38.214 Clause 5.2.2.3.1.

##### 5.2.2.3.1 NZP CSI-RS < Unchanged parts are omitted > The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s). The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration: - nzp-CSI-RS-ResourceId determines CSI-RS resource configuration identity. - periodicityAndOffset defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS. Except resources in resourcesForSetA-r19 used for predicted RS quantities reporting, aAll the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources. - resourceMapping defines the number of ports, CDM-type, and OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211]. - nrofPorts in resourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. - density in resourceMapping defines CSI-RS frequency density of each CSI-RS port per PRB, and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. For density 1/2, the odd/even PRB allocation indicated in density is with respect to the common resource block grid. - cdm-Type in resourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. < Unchanged parts are omitted >

Proposal 4: For UE-sided BM Case-2 AP CSI report configuration, endorse the following text proposal to 38.214 Clause 5.2.1.4.3b.

##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: < Unchanged parts are omitted > - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). When both the first and the second CSI Reporting Settings are aperiodic, the UE is not expected to configure with a value larger than 1 for nroftransmissionOccasion-r19. < Unchanged parts are omitted >

Proposal 5: For the configuration of the associated ID for AP-CSI-Report with AP-CSI-RS resource sets, support configuring multiple associated IDs each corresponding to a resource set in CSI-ResourceConfig.

Proposal 6: For the counting of the active CSI-RS resources, endorse the following text proposal to 38.214 Clause 5.2.1.6.

#### 5.2.1.6 CSI processing criteria < Unchanged parts are omitted > In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS (excluding the case in which the corresponding aperiodic CSI Reporting Setting is configured with the higher layer parameter [RRC_name-r19] and is linked to another aperiodic CSI Reporting Setting with reportQuantity-r19 set to 'csi-pai-r19' and the case in which corresponding aperiodic CSI-RS resources are only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’), starting from the end of the PDCCH containing the request and ending at the end of the scheduled PUSCH containing the report associated with this aperiodic CSI-RS. When aperiodic CSI-RS resources are for reporting ‘p-CRI’ or ‘p-CRI-RSRP’, the UE does not expect to assume aperiodic CSI-RS resources as active. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinkingId, for the purpose of determining the NZP CSI-RS resource active duration, the PDCCH candidate that ends later in time among the two linked PDCCH candidates is used. For semi-persistent CSI-RS ((excluding the case in which semi-persistent CSI-RS resources are only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’), starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. The UE does not expect to receive an activation command for semi-persistent CSI-RS resource set if the CSI-RS resource set is only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’. For periodic CSI-RS (excluding the case in which periodic CSI-RS resources are only configured for reporting ‘p-CRI’ or ‘p-CRI-RSRP’), starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. When the periodic CSI-RS resources are only for reporting ‘p-CRI’ or ‘p-CRI-RSRP’, the UE does not expect to assume periodic CSI-RS resources as active. < Unchanged parts are omitted >