3GPP TSG RAN WG1 #122	      	R1-2505877

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.5.1

Source:	Apple

Title:	Remaining issues on on-demand SSB SCell operation

Document for:	Discussion/Decision

# Introduction

This contribution discusses the remaining issues for OD-SSB SCell operation according to the objective in [1].

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

# Implicit OD-SSB deactivation in Case #2 (w. AO-SSB)

Both implicit and explicit deactivation of OD-SSB transmission are supported. Implicit deactivation for Case #2 involves deactivating OD-SSB transmission after N times of OD-SSB burst transmissions, since  OD-SSB has been activated or adapted by MAC-CE. If N times of OD-SSB transmission are completed before SCell’s deactivation, there are several options available:

- Option 1: UE expects the OD-SSB transmission is not deactivated while the SCell is activated.

  - Note: This can be achieved by gNB guaranteeing to send MAC-CE adaptation signal properly, or to send OD-SSB until SCell is deactivated.

- Option 2: UE assumes the OD-SSB transmission is deactivated.

For Option 2, it is not desirable for UE to change configurations for measurement depending on whether OD-SSB exists or not during SCell activation period. Therefore, Option 1 is preferred.

Proposal 1: For Case#2 (with AO-SSB), in case that N times of OD-SSB transmission are completed before SCell is deactivated, RAN1 to adopt Option 1.

- Option 1: UE expects the OD-SSB transmission is not deactivated while the SCell is activated.

  - Note: This can be achieved by gNB guaranteeing to send MAC-CE adaptation signal properly, or to send OD-SSB until SCell is deactivated.

- Option 2: UE assumes the OD-SSB transmission is deactivated.

# Signaling for OD-SSB

One agreement in RAN1 #120 is:

Agreement (RAN1#120) For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

If SSB positions in burst indicates [0 1 1 0], e.g., it would be good to clarify “the first on-demand SSB burst”:

- Interpretation 1: The first on-demand SSB burst indicates the first on-demand SSB index of on-demand SSB burst regardless of actual transmission according to SSB positions in burst.

- Interpretation 2: The first on-demand SSB burst indicates the first actually transmitted SSB index of on-demand SSB burst according to SSB positions in burst.

Since RRC complete message is up to UE, we think interpretation 2 makes more sense and also aligns with the agreement for MAC CE based signaling. This has been illustrated in Figure 1.

Proposal 2: On the following agreement in RAN1 #120, it is to proposed to clarify:

- For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

  - Interpretation 2: The first on-demand SSB burst indicates the first actually transmitted SSB index of on-demand SSB burst according to SSB positions in burst.

Figure 1 Clarification of RRC based OD-SSB indication

RAN1 agreed on the time instances A and B at which UE can determine whether there is an OD-SSB or not for certain scenarios. However, we believe that the following cases are still not accounted for:

- 1/ Time instance B for RRC based deactivation

- 2/ Time instance A for MAC CE based adaptation

As for 1/ Time instance B for RRC based deactivation, we propose the following:

- For RRC based OD-SSB deactivation, UE expects that on-demand SSB is NOT transmitted after receiving RRC carrying deactivation of OD-SSB transmission.

Proposal 3: For RRC based OD-SSB deactivation, UE expects that on-demand SSB is NOT transmitted after receiving RRC carrying deactivation of OD-SSB transmission.

As for 2/ Time instance A for MAC CE based adaptation, we think reasonable to have the same principle as MAC-CE based OD-SSB activation.

Proposal 4: Time instance C for MAC CE based OD-SSB adaptation is the same as time instance A for OD-SSB activation as agreed.

From the agreement:

Agreement (RAN1 #118bis) The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology. Agreement (RAN1#120bis) Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B. - Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’. - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB. - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification. - (Working assumption) T=T_min - RAN4 to confirm the above two working assumptions - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication RAN4 agreed the following: Agreement (RAN4#115) UE applies the additional processing timing only in deactivated SCell measurement. · Note: The additional UE processing timing is not applied to SCell activation procedure. The additional processing timing is T_processing, the value of T_processing: · Introduce different values with UE capability: [2 or 3]ms and 5ms. Agreement (RAN4#115) o RAN4 should introduce UE capability with values of 2 ms and 5 ms.

Proposal 5: On processing time for MAC-CE based indication of OD-SSB operation, update the working assumptions as follow:

- (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - => To confirm the working assumption to agreement

- (Working assumption) T=T_min

  - => Revised to the following

    - T=T_min+T_processing

      - T_processing = 2 or 5ms subject to UE capability for deactivated SCell measurement

      - T_processing = 0 for other cases

# OD-SSB Periodicity

As for Case #2 (with AO-SSB), main use case is for network to send AO-SSB with large periodicity for network energy saving and additionally to send OD-SSB as per needed. For instance, the periodicities of AO-SSB and OD-SSB can be, respectively, 160ms and 5ms, where network energy saving gain can be obtained when only AO-SSB is transmitted without OD-SSB. If the periodicity of AO-SSB is smaller than that of OD-SSB, there is no instance to save network energy saving.

Observation 6: There is no network energy saving gain in Case #2 when the periodicity of AO-SSB is smaller than that of OD-SSB.

Proposal 6: The case when the periodicity of AO-SSB is smaller than that of OD-SSB is not supported.

# L1 Measurement

Agreement (RAN1 #120bis) For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), - A CSI report configuration can be associated with - both of AO-SSB and OD-SSB - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”. - OD-SSB only - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

In 38.214, the UE behavior is described if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

We prefer the same principle as for {timeRestrictionForChannelMeasurements = notConfigured} should be applied.

Proposal 7:

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - If a CSI report configuration is associated with both of AO-SSB and OD-SSB,

    - if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”,

      - which SSB to measure between the most recent AO-SSB no later than the CSI reference resource and the most recent OD-SSB no later than the CSI reference resource is up to UE implementation as long as RAN4 requirements are met.

In RAN2 #129bis,

Working assumption:

=> When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

In case that SCell is configured but not activated, RF bandwidth at UE can be only for receiving SSB. Even though both AO-SSB and OD-SSB are located in the same BWP during SCell configured but not activated period, UE might need to retune center frequency depending on UE implementation if both AO-SSB and OD-SSB needs to be measured. Therefore, we propose:

Proposal 8:

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - A CSI report configuration can be associated with

    - OD-SSB only

      - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

# Summary of Observations/Proposals

< Implicit OD-SSB deactivation in Case #2 (w. AO-SSB)>

Proposal 1: For Case#2 (with AO-SSB), in case that N times of OD-SSB transmission are completed before SCell is deactivated, RAN1 to adopt Option 1.

- Option 1: UE expects the OD-SSB transmission is not deactivated while the SCell is activated.

  - Note: This can be achieved by gNB guaranteeing to send MAC-CE adaptation signal properly, or to send OD-SSB until SCell is deactivated.

- Option 2: UE assumes the OD-SSB transmission is deactivated.

< Signaling for OD-SSB >

Proposal 2: On the following agreement in RAN1 #120, it is to proposed to clarify:

- For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

  - Interpretation 2: The first on-demand SSB burst indicates the first actually transmitted SSB index of on-demand SSB burst according to SSB positions in burst.

Proposal 3: For RRC based OD-SSB deactivation, UE expects that on-demand SSB is NOT transmitted after receiving RRC carrying deactivation of OD-SSB transmission.

Proposal 4: Time instance C for MAC CE based OD-SSB adaptation is the same as time instance A for OD-SSB activation as agreed.

Proposal 5: On processing time for MAC-CE based indication of OD-SSB operation, update the working assumptions as follow:

- (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - => To confirm the working assumption to agreement

- (Working assumption) T=T_min

  - => Revised to the following

    - T=T_min+T_processing

      - T_processing = 2 or 5ms subject to UE capability for deactivated SCell measurement

      - T_processing = 0 for other cases

< OD-SSB Periodicity >

Observation 6: There is no network energy saving gain in Case #2 when the periodicity of AO-SSB is smaller than that of OD-SSB.

Proposal 6: The case when the periodicity of AO-SSB is smaller than that of OD-SSB is not supported.

< L1 Measurement >

Proposal 7:

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - If a CSI report configuration is associated with both of AO-SSB and OD-SSB,

    - if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”,

      - which SSB to measure between the most recent AO-SSB no later than the CSI reference resource and the most recent OD-SSB no later than the CSI reference resource is up to UE implementation as long as RAN4 requirements are met.

Proposal 8:

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

  - A CSI report configuration can be associated with

    - OD-SSB only

      - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

# Reference

- RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson

- R1-2503155, Consolidated Rel-19 higher layers parameters list Post RAN1#120bis, Moderator (Ericsson)