3GPP TSG RAN WG1 #122			R1-2505890

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.11.4

Source:	Apple

Title:	Remaining issues on IoT-NTN Uplink Capacity Enhancement

Document for:	Discussion/Decision

# Introduction

In the RAN #102 meeting, the WID of Release 19 IoT-NTN was agreed [1]. One objective of the WID is uplink capacity/throughput enhancement for IoT-NTN. In addition, the following agreements were made in the last RAN1 meeting [2].

Agreement Confirm the following working assumption from RAN1#120 Athens: “For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.” Agreement The total number of slots in the NPUSCH transmission after OCC is applied is MrepNPUSCHNslotsULNRU , where the parameters have the legacy definitions in TS36.211 and MrepNPUSCH≥2. Agreement For 3.75kHz SCS, the TDM DMRS positions that are activated within the TDM DMRS pattern are associated at least with the OCC sequence index. Agreement When the OCC is configured by RRC, if the number of repetitions of NPUSCH Format 1 is equal to 1, the DCI is interpreted as per legacy, otherwise: - For 15kHz SCS: - the reserved states in the subcarrier indication field are used to indicate the location of subcarriers, OCC sequence index and OCC activation / deactivation - For 3.75kHz SCS: - The redundancy version field is repurposed to indicate [OCC activation / deactivation or OCC sequence index] - Down-select between: - Option 1: The fields “subcarrier indication” and “modulation and coding scheme” are jointly encoded to indicate the location of the subcarriers, the value of ITBS and [OCC activation / deactivation or OCC sequence index] - Option 2: the MSB bit of “modulation and coding scheme” is used to indicate [OCC activation / deactivation or OCC sequence index] Agreement For 3.75kHz SCS OCC for NPUSCH format 1, the following mappings between DMRS sequence samples and active TDM DMRS slots is applied: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots Agreement For the TDM DMRS positions that are activated within the TDM DMRS pattern: For an NPUSCH format 1 with 3.75kHz SCS allocated to start in slot m, the NPUSCH is postponed to start in the next slot, whose index satisfies (SFN * 5 + ns) mod 4 = 0. The UE transmits TDM DMRS in the nth slot after the start of its NPUSCH transmission according to: Criterion; DMRS activity. OCC sequence index 0 [1 1]; OCC sequence index 1 [1 -1]. n mod 4 = 0; ON; OFF. n mod 4 = 1; ON; OFF. n mod 4 = 2; OFF; ON. n mod 4 = 3; OFF; ON. Agreement For 3.75kHz SCS: - RV indicates activation / deactivation. - The UE initiates NPUSCH format 1 transmission with rvDCI=0 Agreement For 3.75kHz SCS: The fields “subcarrier indication” and “modulation and coding scheme” are jointly encoded to indicate the location of the subcarriers, the value of ITBS and OCC sequence index. The joint encoding does not support indication of ITBS = 10. Agreement For NPUSCH Format1, Symbol-level OCC applied to 3.75kHz SCS single tone and Slot-level OCC applied to 15kHz SCS single-tone, the same RV value is used within an OCC group for MNRUNslotsUL slots, where M is the OCC length. RV cycling is performed across the OCC groups. Note: the number of RVs is MrepNPUSCH/M. Note: the RV sequence is as in legacy specifications.

In this contribution, we provide our views on the remaining issues of IoT-NTN uplink capacity enhancement.

# Discussion

### UL transmission gap

In previous RAN1 meeting, several UL transmission gaps were agreed to study further any specification impacts.

Agreement RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity - UL gaps for synchronization (from Rel-13) - Gaps around NPRACH occasions - UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17) - TDM DMRS that are muted - Guard periods for 3.75kHz UL transmissions

- UL gaps for synchronization

According to current specification, the UL transmission gap is 40ms for every 256 subframe transmission. If two UEs are paired for transmissions with OCC spreading, and UE2 NPUSCH transmissions is overlapped with UL transmission gap of UE1, the OCC sequences of two UEs are still orthogonal before and after the UL transmission gap as shown in Figure 1. The two slots within the OCC group doesn’t not cross the UL gap. Therefore, the orthogonality is kept within the OCC group, and OCC performance is not impacted by the UL transmission gap.

Figure 1: Example of UL transmission gap

- TDM DMRS that are muted

For 3.75kHz SCS, TDM DMRS pattern is supported. As only one DRMS symbol is muted and not transmitted, one symbol gap will not impact the phase continuity. In addition, RAN4 replied LS [3] on 3.75kHz SCS TDM DMRS concluded that it is feasible for the UE to maintain the phase continuity across the blank DMRS symbol across 2 slots in TDM DMRS scheme.

- Guard periods for 3.75kHz UL transmissions

The guard period in one slot is 2304Ts or 0.075ms in a 3.75kHz SCS slot. As RAN4 indicated in Rel-17 coverage enhancement work item that phase continuity is assumed if gap between two PUSCH/PUCCH transmissions does not exceeds 13 symbols. This 2304Ts gap will not impact the phase continuity in OCC design.

Observation: The following gaps do not impact the OCC design for IoT-NTN

- UL gaps for synchronization

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

- Gaps around NPRACH occasions

As indicated by the specification 36.211, the NPUSCH transmission will be delayed in the event of a collision with NPRACH.

If a mapping to slots or a repetition of the mapping contains a resource element which overlaps with - any configured NPRACH resource according to nprach-ParametersList in SystemInformationBlockType2-NB, or - …… then, - for the NPUSCH transmission in overlapped slots is postponed until the next slots not overlapping with any configured NPRACH resource. - for the NPUSCH transmission in overlapped slots is postponed until the next slots starting with the first slot satisfying ns mod 2=0 nsmod 2=0and not overlapping with any configured NPRACH resource.

For 15kHz SCS NPUSCH transmission, if the transmission overlaps with NPRACH resource, it is postponed, and restart form the first slot of a subframe after the gap. Since we have agreed that the OCC length of 2 is supported and OCC spreading is applied per slot. Therefore, NPRACH gap has no impacts on OCC orthogonality of 15kHz SCS NPUSCH transmission.

For 3.75kHz SCS NPUSCH transmission, according to current specification, the overlapped transmission is postponed on a per-slot basis. TDM DMRS pattern was agreed for 3.75kHz SCS NPUSCH transmission with OCC, one slot postpone would cause the larger separation between two activated DMRS symbols, then NPUSCH performance is impacted. To avoid the postponement impacts over NRACH occasion, NPUSCH transmissions with OCC spreading are postponed multiple of 2-slots.

Proposal 1: If 3.75kHz SCS NPUSCH transmission collides with NPRACH occasion, NPUSCH transmissions are postponed multiple of 2 slots.

- UL timing adjustment gaps and segmentation

The UL segment and compensation gap is used for TA pre-compensation, the TA pre-compensation gap could be 1symbol, 1slot, or 2 slots. Each NPUSCH segment could transmit with duration of 2ms, 4ms, up to 256ms. The UL timing adjustment gap will impact both slot-level OCC and symbol-level OCC which depends on the TA pre-compensation gap is symbol level or slot level.

In principle, if the performance of NPUSCH with OCC is impacted by TA pre-compensation gap, the whole PUSCH transmissions covering by an OCC sequence shall be dropped.

Proposal 2: If NPUSCH transmission collides with UL timing adjustment gaps, the entire NPUSCH transmissions covered by an OCC sequence are dropped.

# Conclusion

In this contribution, we provided our views on remaining issues of IoT-NTN uplink capacity enhancements. Our observation and proposals are as follows:

Observation: The following gaps do not impact the OCC design for IoT-NTN

- UL gaps for synchronization

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

Proposal 1: If 3.75kHz SCS NPUSCH transmission collides with NPRACH occasion, NPUSCH transmissions are postponed multiple of 2 slots.

Proposal 2: If NPUSCH transmission collides with UL timing adjustment gaps, the entire NPUSCH transmissions covered by an OCC sequence are dropped.

# References

- RP-234077, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3,” Dec. 2023.

- R1-2505102, “Report of RAN1#121 meeting”, August 2025

- R1-2503211, “LS reply on TDM DMRS”, RAN4 (Sony), May 2025