3GPP TSG RAN WG1 #122		                                                                                                                                                                                                 	R1- 2505895

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	9.3

Source: 	Apple Inc.

Title: 	Views on UE features for Rel-19 SBFD

Document for:	Discussion/Decision

- Views on UE features for Rel-19 NR duplex operation: SBFD

In this contribution, we present our high-level views on UE features for Rel-19 SBFD, based on the progress achieved in previous meeting. The FGs are organized in the following way to handle different objectives/agendas in NR Rel-19 NR SBFD.

- FG 60-1-x, and 60-2-y covers SBFD TX/RX/measurement procedures

- FG 60-3 to FG 60-6 covers SBFD random access operation

- FG 60-7 to FG 60-9 covers CLI handling

## SBFD TX/RX/measurement procedures

## Discussion

Proposal 1: For SBFD Tx/Rx/measurement procedures, the followings are supported

- For FG 60-1, add the following candidate values for component 2

  - Candidate values can be {DUD, DU, both}

- Split FG 60-2 into separate features, 60-2-a and 60-2-b for respectively DL reception and UL transmission

- Additional transient time from non-SBFD symbol to SBFD symbol for DL reception

  - Candidate values can be {1, 2, 3} symbols in TDD SCS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | Component 2 has the following candidate values {DUD, DU, both} Component 7 applies only to UEs supporting FG 23-1-1h Note: Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement | Optional with capability signalling |
| 60. NR_duplex_evo | 60-2-a | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support Configuration 2 for PDSCH with repetitions 2. Support Configuration 2 for SPS PDSCH 3. Support Configuration 2 for multi-grant PDSCH 4. Support Configuration 2 for CSI-RS measurement | 60-1 | YES | n/a | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-2-b | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support Configuration 2 for PUSCH with repetitions Type A 2. Support Configuration 2 for PUSCH with repetitions Type B 3. Support Configuration 2 for CG-PUSCH (Type1 or 2) 4. Support Configuration 2 for multi-grant PUSCH 5. Support Configuration 2 for P/SP-PUCCH | 60-1 | YES | n/a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-2 | Transition gap between non-SBFD symbols and SBFD symbols and vice versa | 1. number of symbols to switch from non-SBFD symbol to SBFD symbol 2. number of symbols to switch from SBFD symbol to non-SBFD symbol 3. number of symbols to switch from UL sub-band to DL sub-band 4. number of symbols to switch from DL sub-band to UL sub-band |  | Yes | n/a |  | Per band | n/a | n/a | n/a | Component 1 candidate values in number of symbols {0, 2, 7, 14, 28} symbols Component 2 candidate values in number of symbols {0, 2, 7, 14, 28} symbols | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-3 | FDRA Type 0 in SBFD slot | 1. FDRA Type 0 for PDSCH 2. FDRA Type 0 for PUSCH | 5-1 | Yes | n/a | UE cannot be scheduled with Type 0 PXSCH in SBFD | Per band | n/a | n/a | n/a | Note: SBFD aware UE must support at least one FDRA type | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-3a | # of partial PRG in SBFD | 1. Number of partial PRG for DL reception in SBFD slot |  | Yes | n/a | UE cannot be scheduled more than 2 partial PRG | Per band | n/a | n/a | n/a | Candidate values are {2,3,4} | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-4 | FDRA Type 1 in SBFD slot | 1. FDRA Type 1 for PDSCH 2. FDRA Type 1 for PUSCH | 5-1 | Yes | n/a | UE cannot be scheduled with Type 1 PXSCH in SBFD | Per band | n/a | n/a | n/a | Note: SBFD aware UE must support at least one FDRA type | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-5 | Frequency hopping in SBFD slot | 1. intra-slot frequency hopping in SBFD slot 2. Inter-slot frequency hopping in SBFD slots 3. Inter-slot frequency hopping between SBFD and non-SBFD slots |  | Yes | n/a | UE ignores FH flag for PUSCHs in SBFD | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-5a | RB-Offset for PUSCH in SBFD | 1. Applying RB-Offset for PUSCH in SBFD symbols |  | Yes | n/a | UE assumes RB-Offset =0 | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-6 | Separate freq. config for PUCCH-Resource | 1. Support separate configurations of startingPRB 2. Support separate configurations of secondHopPRB |  | Yes | n/a | No separate PUCCH-Resource config is supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-7 | Separate SRS-ResourceSets | 1. Support two SRS-Resource set with usage Codebook for SBFD and non-SBFD transmissions 2. Support two SRS-Resource set with usage nonCodebook for SBFD and non-SBFD transmissions |  | Yes | n/a | No separate SRS-ResourceSets is supported | Per band | n/a | n/a | n/a | Note: if UE supports separate SRS-ResourceSets for Codebook, the two sets have same number of SRS resources Note: if UE supports separate SRS-ResourceSets for nonCodebook, the two sets have same number of SRS antenna ports | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-10 | CSI-RS processing across two DL subbands | 1. CSI processing timeline and CPU counting for CSI processing across DL sub-bands | 60-2 | Yes | n/a | CSI processing timeline Z/Z’ is scaled by factor of 2 | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-1-11 | Separate UL power control for SBFD and non-SBFD | 1. Support new P0AlphaSets in Uplink-powerControl for PUSCH 2. Support new P0AlphaSets in Uplink-powerControl for PUCCH 3. Support new P0AlphaSets in Uplink-powerControl for SRS |  | Yes | n/a | Same/single P0AlphaSets is applied to both SBFD and non-SBFD | Per band | n/a | n/a | n/a |  | Optional with capability signalling |

## SBFD random access operation

## Discussion

Proposal 2: For SBFD random access operation UE features, the followings are supported

- For FG60-3, the following components should be included

  - Support of reinterpretation of frequency offset of lowest RO

  - Support of separate power control parameter (i.e., preambleReceivedTargetPower) in additional-RO

  - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period

- For FG60-4, the following components should be included

  - Support of separate power control parameters in additional-ROs

  - Support of separate mapping of SSB and additional ROs

  - Support separate determination of the association period and association pattern period

  - Support of valid RO across SBFD and non-SBFD symbols

- For FG60-4, the prerequisite feature group is FG60-1

- For FG60-5 and FG60-5a, the prerequisite feature group includes FG54-1

- Introduce a new feature group for Msg3 PUSCH repetition in SBFD symbols

  - The prerequisite feature groups include 30-6, 60-3 or 60-4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | - Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) - Support of reinterpretation of frequency offset of lowest RO - Support of separate power control parameter (i.e., preambleReceivedTargetPower) in additional-RO - Support of separate mapping of SSB and additional ROs, and separate determination of the association period and association pattern period - | 60-1 | YES | n/a | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | - Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) - Support of separate power control parameters in additional-ROs - Support of separate mapping of SSB and additional ROs - Support separate determination of the association period and association pattern period - Support of valid RO across SBFD and non-SBFD symbols | 60-1 | YES | n/a | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-5 | Preamble repetition within additional-ROs for RACH configuration Option 1 | 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros 2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-3, 54-1 | YES | n/a | Preamble repetition within additional-ROs for RACH configuration Option 1 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-5a | Preamble repetition within additional-Ros for RACH configuration Option 2 | 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-Ros 2. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. | 60-4, 54-1 | YES | n/a | Preamble repetition within additional-ROs for RACH configuration Option 2 is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-6 | Msg3 PUSCH repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols | 30-6, 60-3 or 60-4 | Yes | n/a | Msg3 PUSCH repetition in SBFD symbols is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |

## CLI handling

## Discussion

Proposal 3: For CLI handling procedures, the followings are supported

- For FG 60-7, and 60-7a, add the following in the Note

  - Only one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH

- For FG 60-8, and 60-9, remove aperiodic from the []

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH | [FFS] | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | Per band | TDD only | n/a | n/a | this FG supports one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH | Optional with capability signalling |
| 60. NR_duplex_evo | 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH | [FFS] | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | Per Band | TDD only | n/a | n/a | this FG supports one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH | Optional with capability signalling |
| 60. NR_duplex_evo | 60-8 | L1 CLI-RSSI measurement and aperiodic reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs | FFS | YES | n/a | Aperiodic L1 CLI-RSSI measurement and reporting is not supported | Per band | TDD only | [n/a] | [n/a] |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource 2. Maximum number of L1 SRS-RSRP measurement resources across all CCs | FFS | YES | n/a | Aperiodic L1 SRS-RSRP measurement and reporting is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |

- References

- R1-2504673, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #121”, Moderators (AT&T, NTT DOCOMO, INC.), May 2025