3GPP TSG-RAN WG1 Meeting #122	R1-2505953

Bengaluru, India, Aug 25th – Aug 29th 2025

Agenda Item:	8.6.3

Source:	Ericsson

Title:	Maintenance on LP-WUS operation in CONNECTED modes

Document for:	Discussion

# 1	Introduction

In this contribution, we discuss text proposals for the corresponding specifications and value range for RRC parameters.

# 2	Text proposals

In this section, we provide TPs related to Connected mode operation for Clause 10.4D of TS 38.213 ‎[2].

Regarding the minimum time gap reported by the UE, the following agreement was reached in RAN1 #121:

Agreement For the UE capability report on the minimum time gap between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, the same candidate values {V1, V2, V3} are supported for different receiver types - V1=5ms - V2=13ms - V3=37ms

The unit of time gap is ms based on the agreements. Therefore, we propose that the following TP is adopted.

Adopt the ms unit of minimum time gap for Clause 10.4D in TS 38.213.

- Reason for change: The unit of minimum time gap in the specification is incorrect

- Summary of change: Change the unit of minimum time gap in sub-clauses 10.4D of TS 38.213

- Consequences if not approved: Incorrect UE report in the specification

---------------------------------Start of Text Proposal on 3GPP TS 38.213 V19.0.0----------------------- ## 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED <Unchanged parts are omitted> The UE reports a number of slots ms [18, TS 38.306] where the UE is not required to monitor WUS prior to the slot where the drx-onDurationTimer would start. <Unchanged parts are omitted> ---------------------------------End of Text Proposal on 3GPP TS 38.213 V19.0.0-----------------------

Regarding Option 1-2, the following was agreed in RAN1 #120

Agreement - For LP-WUS MOs in connected mode for Option 1-2, support Approach 1: - LP-WUS MOs, including a periodicity and a time offset3, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE is configured with a number of MOs for the periodicity by RRC - UE monitors LP-WUS in the LP-WUS MOs and, if triggered to wake up, starts a new timer for PDCCH monitoring triggered by LP-WUS - Alt1: - A time offset4 configured by the network indicating a time from the first LP-WUS MO per periodicity, after which the UE starts PDCCH monitoring via starting the new timer. - The time gap from the last LP-WUS MO per periodicity and the start of new timer is not smaller than the minimum time gap given based on UE capability

Based on the agreement, periodicity and offset are used to determine the start point of the monitoring occasions, from where the UE will monitor a number of monitoring occasions, as shown in the figure below.

Therefore, we propose that the following TP is adopted.

Adopt the TP provided in this paper for Clause 10.4D in TS 38.213 for UE behavior on WUS monitoring in option 1-2.

- Reason for change: The description of UE behavior on WUS monitoring in option 1-2 in the specification is not clear

- Summary of change: Change the description of UE behavior on WUS monitoring in option 1-2 in sub-clauses 10.4D of TS 38.213

- Consequences if not approved: Misleading UE behavior on WUS monitoring in option 1-2.

---------------------------------Start of Text Proposal on 3GPP TS 38.213 V19.0.0----------------------- ## 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED <Unchanged parts are omitted> A UE can be provided by lpwus-Mo12 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity for the UE to determine the first WUS monitoring occasion of a number of WUS monitoring occasions per periodicity, provided by lpwus-NumOfMo12. <Unchanged parts are omitted> ---------------------------------End of Text Proposal on 3GPP TS 38.213 V19.0.0-----------------------

Also, based on the latest version of RAN2 running CR, updated RRC parameter names are provided in the TP.

- Reason for change: RRC parameters in the specification are incorrect

- Summary of change: Fixed RRC parameter names in sub-clauses 10.4D of TS 38.213

- Consequences if not approved: Incorrect RRC parameter names in the specification

---------------------------------Start of Text Proposal on 3GPP TS 38.213 V19.0.0----------------------- <Unchanged parts are omitted> ## 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED A UE configured with DRX mode operation and operating in the RRC_CONNECTED state can be provided for WUS reception on the primary cell of a cell group - a number of OOK symbols per OFDM symbol, a first RB, and overlaid OFDM sequences per OOK symbol for WUS reception [4, TS 38.211], and - a number of codepoints provided for the UE by the WUS [6, TS 38.212], by WUS-codepoint_CONNECTED A UE assumes that a WUS is quasi co-located with an SS/PBCH block or a CSI-RS with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable. If a UE is provided ABC, the UE receives WUS based on the quasi co-location information of the TCI states indicated by a most recent DCI format or MAC CE, after a respective application time; otherwise, the UE receives WUS based on the quasi co-location information of the TCI states for a CORESET with controlResourceSetId value that is same as the one indicated by WUS_TCI_states_CONNECTED lpwus-TCI-States. A UE assumes that an SCS configuration for WUS receptions is same as an SCS configuration for the active DL BWP. A UE does not monitor a WUS during Active Time [11, TS 38.321]. A UE does not monitor WUS during DTX inactive period for the primary cell. A UE can be provided by WUS-MOCONNECTED-Option1-1 lpwus-Mo11 a periodicity, by periodicityMO-Option 1-1, and a time offset, by offsetMO-Option 1-1, relative to the start of a system frame with SFN 0, for the UE to determine WUS monitoring occasions. The UE starts to monitor WUS in a first WUS monitoring occasion that is not earlier than a first slot that is prior to a second slot where the drx-onDurationTimer would start by a time provided by timeOffsetCONNECTEDOption1-1, and monitors WUS for a number of monitoring occasions provided by numMO-Option 1-1 lpwus-NumOfMo11. The UE reports a number of slots ms [18, TS 38.306] where the UE is not required to monitor WUS prior to the slot where the drx-onDurationTimer would start. The UE is not required to monitor WUS within the reported number of slots prior to the slot where the drx-onDurationTimer would start. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts the drx-onDurationTimer [11, TS 38.321]. A UE can be provided by WUS-MOCONNECTED-Option1-2 lpwus-Mo12 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity for the UE to determine the first WUS monitoring occasion of a number of WUS monitoring occasions per periodicity, provided by numMO-perPeriodicity-Option 1-2 lpwus-NumOfMo12. The UE reports a number of slots ms [18, TS 38.306] and expects that a time gap, from a last WUS monitoring occasion from the number of WUS monitoring occasions per periodicity to the slot where the wus-PDCCHMonitoringTimer would start, is no smaller than the reported number of slots. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts wus-PDCCHMonitoringTimer [11, TS 38.321] after a time, provided by timeOffsetCONNECTEDOption1-2, with respect to the start of the first WUS monitoring occasion from the number of WUS monitoring occasions per periodicity. A UE can be provided, by WUS_available_slot_CONNECTED lpwus-AvailableSlot, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. The UE can be additionally provided, by WUS_available_symbol_CONNECTED lpwus-AvailableSymbol, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided WUS_available_slot_CONNECTED lpwus-AvailableSlot, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided WUS_available_symbol_CONNECTED lpwus-AvailableSymbol, the UE assumes that, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when - the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon or tdd-UL-DL-ConfigurationDedicated - the symbol is indicated for transmission of SS/PBCH blocks, by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon A WUS monitoring occasion is over a first number of symbols, provided by WUS_NominalMO_duration_CONNECTED lpwus-NominalMoDuration. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion is smaller than a second number of symbols, provided by WUS_ActualMO_duration_CONNECTED lpwus-ActualDuration, the UE does not monitor WUS in the WUS monitoring occasion. The UE monitors WUS in a WUS monitoring occasion over the earliest available WUS_ActualMO_duration_CONNECTED lpwus-ActualDuration symbols in the WUS monitoring occasion. If a UE detects a codepoint in a WUS reception, from the number of codepoints, on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group. <Unchanged parts are omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.213 V19.0.0 ------------------

# 3	Value range for existing RRC parameters

Some existing RRC parameters in ‎[3] are not fully complete in terms of value range. In this section, we provide our view on the corresponding value ranges.

| Parameter name in the spec | Description | Value range | Our comment |
| --- | --- | --- | --- |
| NumOfMO_Monitoring_Option 1-1 | For CONNECTED mode optoin 1-1, UE monitors the first Z LP-WUS MO(s) at or after time offset prior to a slot where the drx-onDurationTimer would start. Z is configured by RRC. | FFS: INTEGER {1..4} | Up to 4 MOs is sufficient for handling the cases where the resources for LP-WUS are used for other NR channels/signals or UL slot occurs. |
| NumOfMO_per_periodicity_Option 1-2 | For Option 1-2, UE can be configured with a number of MOs per periodicity. | FFS: INTEGER {1..4} | Up to 4 MOs is sufficient for handling the cases where the resources for LP-WUS are used for other NR channels/signals or UL slot occurs. |
| LP-WUS_NominalMO_duration_CONNECTED | To configure the nominal MO duration for LP-WUS in CONNECTED, in number of OFDM symbols | FFS M=1: - FR1: {2:1:14, 14* [2-10]} OFDM symbols - FR2: {2:2:14, 14*[2:25]} OFDM symbols M=2: - FR1: {1-14, 14* [2-6]} OFDM symbols - FR2: {1-14, 14*[2:22]} OFDM symbols M=4: - FR1: {1-14, 14* [2-5]} OFDM symbols - FR2: {1-14, 14*[2:21]} OFDM symbols | Considering that code-block length (before Manchester encoding) can be any value in the range 1-32, the actual WUS duration should be: M=1: 2-64 OFDM symbols M=2: 1-32 OFDM symbols M=4: 1-16 OFDM symbols Within a nominal MO duration, some symbols are reserved at least for PDCCH/SSB and cannot be used for WUS. For example, 3 symbols of each slot can be allocated for PDCCH, and within an SSB periodicity 4 symbols are used for SSB per beam (so 32 symbols for 8 beams). The number of SSB beams in a burst can be up to 8 in FR1 and 64 in FR2. Also, the MO duration should cover cases with short WUS durations (i.e., a few symbols). The MO duration is determined considering actual WUS duration and unusable symbols. |
| LP-WUS_Actual_WUS_duration_CONNECTED | To configure the actual duration for LP-WUS in CONNECTED, in number of OFDM symbols | FFS M=1: 2-64 OFDM symbols M=2: 1-32 OFDM symbols M=4: 1-16 OFDM symbols | Considering that code-block length (before Manchester encoding) can be any value in the range 1-32, the actual WUS duration should be: M=1: 2-64 OFDM symbols M=2: 1-32 OFDM symbols M=4: 1-16 OFDM symbols Agreement For code block (coded bits) length L after rate matching as defined in 38.212 section 7.4.2.1, support - For 1 information bit, L is in 1~32 - For 2 information bits, L is in 2~32 - For 3 information bits, L is in 3~32 - For 4 information bits, L is in 5~32 - For 5 information bits, L is in 6~32 - For M=4, the code block (coded bits) length L is even number. |

Adopt the proposed value ranges for existing RRC parameters in Table 2 of this paper.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the ms unit of minimum time gap for Clause 10.4D in TS 38.213.

Proposal 2	Adopt the TP provided in this paper for Clause 10.4D in TS 38.213 for UE behavior on WUS monitoring in option 1-2.

Proposal 3	Adopt the proposed value ranges for existing RRC parameters in Table 2 of this paper.

# References

RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR),” 3GPP TSG RAN meeting #105, Sept. 2024.

3GPP TS 38.213-j00, “NR: Physical layer procedures for control”, June 2025.

R1-2503243	Consolidated Rel-19 higher layers parameters list Post RAN1#121	Moderator (Ericsson).