3GPP TSG RAN WG1 Meeting #122	         R1-2505995

Bangalore, India, August 25-29, 2025

Agenda Item:	11.5

Source:	Lenovo

Title:	Discussion on 6GR Energy Efficient design

Document for:	Discussion

# Introduction

As work on 6G minimum TPR based on ITU-R M.2160 is underway and captured in TR 38.914, the 3GPP RAN#108 plenary concluded and approved the 6G Radio SID in RP-251881. which defines the study objectives towards the next-generation radio access technology and its enabling components. This contribution highlights key technical areas that should be considered to achieve Energy efficient radio design in 6G.

# Discussion

## 6G Energy Efficient Radio

6G Radio comes with a once-in-a-decade opportunity to refresh the radio design without needing to address backward compatibility. The goal for 6G is to embed energy efficiency into network and devices from day-1, thereby reducing end-to-end energy usage and addressing one of the primary targets of 6G to reduce the total cost of ownership (TCO) for the MNO by operating their network in an energy efficient method. In an era where MNO are still struggling to make revenues from value added services, energy efficient 6G radio could significantly reduce their energy bills. [1].

In addition, device power saving functionality should cater to the needs of different device form factors with the supported features depending on the corresponding use case of each device type envisioned in 6G. Since 6G is introducing native network energy saving features, their impact on device power saving should also be analyzed. 6G should aim to address both, TCO reduction using network energy saving and improved user experience using device power saving, using joint network and device power saving design.

### 6G Energy Efficient Radio at Network

The opportunity for 6G radio to ensure energy efficiency design from day-1 should target the following functional areas:

- Time domain techniques

- Energy efficient transmission techniques

- Network Circuit level optimizaton and its impact

- Frequency domain techniques

- Spatial domain techniques

Figure 1: Illustration of various Network Energy saving techniques

Proposal 1: 6GR should aim to reduce TCO by utilizing the non-backward compatible opportunity provided by once in a decade new generation refresh by designing native energy efficient solution using various techniques such as time, frequency, spatial, power domain.

#### Network Energy Saving Techniques in 6G

Common channel transmission and reception is one of the key issues to be addressed in non-backward compatible 6GR design to ensure energy efficiency, spectral efficiency etc., at various cell loads. Many of the 5G NR network energy saving features studied during 5G Advanced Rel18-19 had backward compatibility limitations thereby limiting the choice and end result in obtainable network energy saving. 6GR is a non-backward compatible release providing an opportunity to bring in native energy efficient initial access and common channel design.

6GR initial access, including corresponding signals and channels such as synchronization signals, raster configurations, required minimum system information, paging channel, random access etc., and associated procedures such as cell detection, system information acquisition, random access, paging etc., should be optimized relative to 5G NR primarily focusing on improving energy-efficiency, delay reduction, enhanced coverage and reliability in connectivity establishment.

Proposal 2: 6GR should enhance common signals/channels transmission and periodicity for network energy saving under various cell load.

##### Energy Efficient common channel design

6GR non-backward compatible synchronization signals design starts from SS/PBCH signal block and multiplexing structure, bandwidth, time domain location of SS/PBCH symbols, periodicity optimized for energy efficiency. Current 5G NR SS/PBCH signal block structure of 20 PRBS over 4 symbols takes into consideration the minimum system bandwidth and MNO channel bandwidth of 5MHz. However, latter part of 5G introduced optimization for 3MHz MNO channel bandwidth by puncturing the PBCH signal to 12 PRBs. Hence, 6GR SS/PBCH signal block and multiplexing structure design needs to consider minimum MNO channel and system bandwidth in each frequency range as mentioned in Table 1 and its impact on energy efficiency.

| Frequency Range | Minimum CBW (MHz) | Maximum BW (MHz) |
| --- | --- | --- |
| 600 MHz – 2 GHz | [3/5] | 20 |
| 2 GHz – 6 GHz | 5 | 200 |
| 6 GHz - 24 GHz | [20] | 200 |
| 24.25 GHz – 52.6 GHz | 100 | 400 |

Table 1: Minimum and Maximum bandwidth in each frequency range

The number of SS/PBCH beams supported for XL-MIMO is increased to sufficiently cover all spatial regions from ~200 AE in 5G NR to ~1000 AE in 6GR, resulting in 3dB beam width change from 1.60 (angle/deg.) for the ~200 AE to 0.41 (angle/deg.) for the ~1000 AE. The conclusion is that beam width for XL-MIMO is reduced by a factor of four otherwise four times more beams are needed to cover the same area when co-sited with 5G. The time domain location of SS/PBCH symbols and the number of SS/PBCH blocks in a slot by reducing the gaps between SS/PBCH blocks is another important factor to be considered to enable compact SS/PBCH transmission in a SSB burst.

Proposal 3: The number of candidate SS/PBCH beams from XL-MIMO in the upper mid-band spectrum and its impact on the network energy consumption needs to be studied and evaluated.

Another aspect is the time domain symbol and frequency domain resource need to define the SS/PBCH block structure considering various aspects such as the minimum common bandwidth depending on the frequency range, etc..,. Various SS/PBCH block structure can be envisioned considering 3MHz or 20MHz minimum channel bandwidth, increased/reduced time domain symbols. These SS/PBCH block structure needs to be compared against 5G NR SS/PBCH block structure in terms of performance e.g., detection reliability, residual time/frequency errror and network power consumption needs at various frequency ranges. The reduced PBCH bandwidth with 12 PRBs and 6 OFDM symbols is suitable at lower frequency range to meet the 3MHz carrier bandwidth requirement and the lean FDMed SS/PBCH block maybe suitable at wider bandwidth and higher frequency range. Lastly, SS repetitions within SS/PBCH block maybe suitable to increase the reliability of detection considering the increase in the SS/PBCH periodicity.

a) Reduced PBCH bandwidth 		b) FDMed SS/PBCH block       c) SS/PBCH block - SS repetition

Figure 2: Various SS/PBCH block structure envisioned for 6GR

Significant energy saving at higher frequency range can be achieved by minimizing the number of time domain symbols defining SS/PBCH block structure together with compact SS/PBCH block transmission by reducing the gaps between SS/PBCH blocks in a SSB burst. However, for the lower frequency due to limitation in the minimum supported common carrier bandwidth, increasing the periodicity of SSB transmission is the way to gain network energy saving.

Figure 3: Network energy saving gains comparing FDMed SSB Vs Regular SSB for different SSB periodicity assuming no UE specific load

Observation 1: Energy saving gains of 20% to 30% from lean FDMed SS/PBCH block structure can be achieved in the upper mid band spectrum compared to the existing 5G NR SS/PBCH block structure.

Proposal 4: Evaluate the network energy consumption of various SS/PBCH block structures with different periodicity envisioned for all frequency ranges.

Figure 4: Illustration of Compact SSB structure in 6GR compared to 5G NR

Figure 5: Network energy saving gains comparing compact SSB and 5G NR SSB for different SSB periodicity assuming no UE specific load

The gain is not constant across SSB periodicity, because as the periodicity gets larger, let’s say 2ms difference from the compact SSB compared to the regular SSB becomes a smaller fraction of the period hence, the percentage gain only slightly decreases with increased periodicity.  Another reason is that the deep sleep state begins >= 50m + burst size, however with changing periodicities, bursts get pushed across these thresholds hence a slight variation is observed however this variation is not significant.

Observation 2: Optimizing gaps between SS/PBCH block transmission within a SSB burst can bring 9% to 10% energy saving gains compared to 5G NR SS/PBCH.

Proposal 5: Study the feasibility and impact of reducing gaps between SS/PBCH block transmission within a SSB burst.

In 5G, uniform SSB burst periodicity was defined e.g., 5ms, 10ms…,160ms. However, increased periodicity of SSB burst e.g., 160ms, results in few transmissions of SSBs. Although increasing the periodicity of transmitted cell defining SSB brings in significant energy saving, further evaluations are needed to understand its impact on AGC tracking, cell selection time at UE, mobility, other measurements etc., Also, number of SSBs needed for a UE to get time-frequency synchronization to decode other common channel e.g., paging channel can depend on the UE geometry within a cell e.g., SINR. Also, UE in not good geometry needs three or four SSBs resulting in significant delay and power consumption to achieve finer synchronization. Hence, a joint mechanism ensuring network energy saving while simultaneously minimizing the acquisition time at the device side should be studied.

Figure 6: Comparison of Network Energy saving gain for various SSB periodicities – No load

Some of the above drawbacks can be addressed by clustering/grouping SSB bursts with reduced inter-SSB burst gap between them and increasing the periodicity of these clusters resulting in significant network power saving. One benefit of these clustered transmission is that it can maintain sufficient SSB density in the time domain together with increased periodicity. The significant network power saving comes from the fact that the Cell enter the deep sleep state because of 160ms cluster periodicity while Cell can enter only the light sleep state with uniformly spaced SSB periodicity. Furthermore, number of SSB bursts in a cluster also has an impact on power consumption, reducing from four to three SSB bursts reduces the network energy consumption by 15% (TE consumption of 160ms clustered SSB periodicity with four SSB bursts per cluster compared with three SSB bursts per cluster). Although our evaluation assumption assumes only deep sleep state of one relative power value and 50ms transition state, with longer periodicity, network can enter even lower power saving sate e.g., ultra-deep sleep state with even lower relative power value and longer transition state. Such enhancement to power saving state should also be taken into considering as it reduces significant power consumption for longer periodicity. As you can see from the Figure 9, both maximum and average latency is significantly reduced comparing to similar regularly spaced SSB periodicity. Cluster based common channel structure reduces the need of additional RS overhead in terms of power consumption and spectral efficiency. Hence, cluster based SSB transmission can replace regularly spaced SSBs in many ways and under different cell load conditions and should be evaluated.

Figure 7: Illustration of SSB burst clusters/groups Vs Regular 5G NR SSB

Figure 8: Comparing the network energy consumption of uniformly spaced 5G NR SSB Vs SSB Cluster

Figure 9: Comparing the SSB acquisition latency of uniformly spaced 5G NR SSB Vs SSB Cluster

Observation 3: Clustering/grouping SSB bursts with minimum inter-SSB burst gap and larger inter-cluster gap can reduce SSB acquisition latency compared to the uniformly spaced 5G NR SSB.

Proposal 6: Study the feasibility of clustering/grouping of SSBs and other common signal/channel and compare pros and cons in terms of acquisition latency and network energy consumption.

Besides SSBs transmission design, reducing the gaps between paging resources, e.g., PFs and/or POs, could be considered to enable compact paging transmission for network energy saving. This principle has been used in NR Rel-19 by associating 8 POs with a PF, compared with 4 POs associated with a PF before Rel-19. However, PFs are still dispersive within a paging cycle and legacy POs exists. Both may contribute quite a lot of energy consumption. The non-backward compatible 6G design could provide an alternate design to achieve more energy saving for paging from the beginning. The compact paging frames must be calculated such that it is within a Cell active time periods and in addition to the time domain paging occasion configuration, frequency domain paging occasions increases the availability of overall paging occasions reducing temporal occasions within a Cell active time period to benefit network energy saving.

Proposal 7: Evaluate the compact paging frame against dispersive paging frame of 5G NR with respect to network energy saving and latency.

##### Adaptation of common channel transmission / reception

Adaptation of the common channel transmission / reception according to the cell load can bring quite a lot of energy saving. While the adaptation can be done individually for each of the common channels, adaptation can also be done jointly exploiting the linkage between these common channels. Although these adaptations were discussed in 5G NR, some of these adaptations were restricted for e.g., NCD-SSB and SCell to prevent backward compatibility problems. In 6GR, adaptation for the synchronization signals according to various cell load can be studied for the cell defining SSB, which includes muting the SS/PBCH candidate resource within a SSB burst together with adapting the position of SS/PBCH block within a SSB burst reduces the gap between the transmission. Secondly, adapting the periodicity of the SSB burst transmission studied in 5G NR can be extended to the Cell defining SSB design.

The common channel resources e.g., SSBs, RACH occasions, paging occasions can be clustered or condensed in the time/frequency region thereby allowing cell to enter deep sleep or a light sleep state more often for energy saving. In the 6GR, adaptation in the time domain and in the spatial domain, including non-uniform allocation of RACH resource occasion according to the density of UEs and adaptively turning on and off RACH resources associated with a SSB beam, can jointly optimize the resource utilization and energy efficiency. RACH occasions can be adapted in the time domain and in the spatial domain for each of the associated SSB beam depending on the density of UEs within each SSB beam. The RACH load can be determined at the network side using e.g., implementation methods by monitoring the RACH request on each of the associated RACH occasions.

In 6G, a common adaptation framework considering a linkage between these common channels is useful to be studied. The 6GR should incorporate design enabling muting and adaptation of a common channel and its dependent common channel exploiting its linkage. Whenever a SS/PBCH block is not transmitted, corresponding RACH occasion, paging occasion and RMSI need not be monitored at the UE, also whenever a SS/PBCH block is not transmitted, corresponding paging occasion and RMSI need not be transmitted by the network, also its time domain positions can be dynamically shifted providing a considerable energy saving gain for both UE and network.

Proposal 8: Study a common adaptation framework to jointly adapt common signal/channel by exploiting the linkage between them.

##### Adaptation of Cell power state

5G NR introduced Cell DTX/DRX feature to allow the cell to sleep for extended time, however the effectiveness of the feature depends on switching off the transmission of periodic common channels needed by the UE such as synchronization signals and other related common channels. 5G NR allowed such signals to be transmitted by the cell outside the cell active time periods e.g.., using DCI formats 2_6/2_7, DCI for RAR/RAR window, DCI for Msg4/Contention resolution timer, SSBs, POs. The non-backward compatible 6G design must align the DRX cycle of the UE to that of Cell’s DTX/DRX cycle and minimize the transmission of signals outside the cell active time. The network can also optimize the scheduling decisions by reducing the temporal scheduling and subsequent reception as much as possible to be within the Cell active time.

Figure 10: Alignment between DRX of UE with that of the DTX/DRX at NW

The network power states in 5G NR evaluations includes micro sleep, light sleep and deep sleep states. Deep sleep state is the lowest power state considered for evaluation in 5G NR with a transition time of 50ms and a relative power of value 1. 6GR should extend the evaluation assumptions to include ultra-deep sleep state with more transition time and even less relative power value, the reason is to do proper evaluation of common channel supporting longer periodicity in 6GR beyond the deep sleep state. As you can see from figure 5, the network energy saving gains saturates as the common channel periodicity increases, because the percentage of the time network spends in the deep sleep state gets saturated. A new ultra deep sleep state can accurately quantify the network energy saving gain estimates for longer periodicity as the network enters a new state when its idle period is more than the transition time period of the new state consuming fraction of power compared to the deep sleep state. 6G BS power model can include such lower power state from beginning in the evaluation purpose to define candidate techniques to wake up the BS from that state.

Figure 11: Percentage of time in each power state for each SSB periodicity

Proposal 9: Study on how alignment between Cell DTX/DRX and UE DTX/DRX cycles can be achieved by minimizing the transmission of signals outside the cell active time.

Proposal 10: Consider a new relative power value and transition duration for a new ultra-deep sleep state (lower than the deep sleep state) to envision network energy saving for low load cases.

##### Impact of SSB and RMSI periodicity on Cell selection Latency

Firstly, a UE needs to achieve sufficient synchronization by acquiring three to four SSBs depending on the UE geometry in the cell to decode RMSI, paging etc.. The latency to select a suitable cell increases with increased SSB and RMSI periodicity is shown in Figure 9. Since, UE typically acquires cell quality thresholds transmitted in SIB1 as part of cell selection process during initial cell search, the impact from the longer periodicity of SSB and RMSI containing cell quality thresholds to the cell selection time needs to be carefully evaluated and mitigated to avoid delayed emergency call set up, attach latency, registration.

Figure 12: Illustrating Cell selection procedure

Observation 4: The longer SSB acquisition time and latency in acquiring cell quality thresholds from RMSI affects the cell selection time.

Proposal 11: Analyze the impact to the cell selection latency from increased SSB and RMSI periodicity.

##### Network Energy saving using PAPR/CM reduction Techniques

Techniques to reduce PAPR/CM in the existing DL waveform i.e., CP-OFDM, includes reducing the back-off at Power Amplifier (PA) leading to more efficiency in terms of output power while, reduction in power amplifier back-off is conceptually equivalent to power boosting technique. Hence improving PAPR/CM in DL waveform increases the energy efficiency per bit. PAPR reduction methods usually depend on signal clipping, digital pre-distortion or companding (compression at Tx and expansion at Rx). Although these methods can achieve very good reduction in PAPR/CM, they usually lead to signal distortion and SNR degradation at Rx. Therefore, trade-off between required PAPR/CM reduction and signal quality at Rx needs to be considered. On the other hand, methods such as tone reservation and selective mapping result in moderate reduction in PAPR/CM while preserving signal quality with distortion less signal recovery at Rx.

As explained in our companion contribution [2], Selective mapping on CP-OFDM can achieve ~1.5dB PAPR gain with increased complexity at Tx side and Tone reservation can achieve ~2dB gain in PAPR while leading to some loss of spectral efficiency.

DFT-s-OFDM has the potential to enhance the coverage and energy saving at the transmitter, making it particularly well-suited for certain 6G applications, where coverage and not high data throughput is the primary KPI, such as in IoT/LPWA and NTN. However, DFT-s-OFDM limits the physical channel multiplexing in the frequency domain, MIMO support etc., A single DFT applied for a group of UEs can achieve lower PAPR/CM to enhance both energy saving and downlink coverage. For the IoT/LPWA use case, DFT-s-OFDM at the BS offers benefits due to low PAPR and CM. These benefits translate into improved coverage for cell-edge IoT/LPWA UEs, fewer required repetitions (needed to reach deep coverage for such device types), and thus reduced energy consumption, making DFT-s-OFDM a strong candidate for low-complexity downlink transceiver. The PA back off reduction using PAPR/CM reduction techniques translates into energy efficient transmission as the PA can work linearly at higher output power resulting improved DL coverage.

Proposal 12: Study the feasibility of network energy saving from various PAPR/CM reduction techniques such as DL waveform using DFT-s-OFDM, selective mapping, tone reservation.

##### Multi-Carrier operation for Network Energy Saving

Figure 13: Multi-carrier environment

6GR can leverage multi-carrier environment by minimizing the number of always-on cell/carriers. Although, configuring the multi-carrier environment is not in the scope of this agenda item, studying candidate techniques utilizing the multiple carriers can be considered within network energy saving evaluation. The serving carrier can be on the lowest frequency providing wider coverage enabling initial access needs of all device types while anchor carrier provides assistance information in terms of reference timing information to a non-anchor carrier. Most of the 5G NR work item focused on network energy saving at the Secondary Cell(s) leveraging the presence of anchor cell providing assistance configuration information and reference timing for the transmission of uplink wake- up signal requesting on-demand signals. Often a Secondary Cell(s) associated to one UE may be a Primary Cell for another UE questioning the usefulness of such on-demand in 5G NR. Other features such as inter band SSB-less Secondary Cell(s) needs re-evaluation again together with supported CA band combination as it comes with hardware constraints both at the network and UE side.

Proposal 13: Consider multi-carrier environment to minimize the number of always-on cell/carrier extending the on-demand common channel framework.

#### Energy Efficient XL-MIMO operation in new upper-mid-band spectrum

The BS power consumption model defined in TR 38.864 needs to be extended to include aspects related to XL-MIMO operation in upper mid band spectrum. Wider system bandwidth needs to be considered for evaluation in the upper mid band spectrum with increased Tx/Rx RUs and ~1000 AEs. Then, aspects related to sub-band duplex configuration such as power consumption estimation for the sub-band duplex slots needs to be defined as these slots includes both downlink and uplink non-overlapping and overlapping sub-bands.

|  | Set 4 FR3 |
| --- | --- |
| Duplex | TDD, SBFD |
| System BW | 200MHz |
| SCS | 30kHz |
| #Tx & Rx RUs | 256 |
| Total DL power | TBA |

Table 2: BS reference configuration for XL-MIMO and upper-mid-band spectrum for power model evaluation

Proposal 14: Consider in the Evaluation assumption for the network energy saving in the new upper-mid-band spectrum:

- Wider bandwidth, 200MHz

- Increased Tx & Rx RUs – 256, considering XL-MIMO operations

- Various Duplexing operations i.e., TDD, SBFD

##### Spatial domain Network energy saving

In 5G NR, multiple MNOs and Network vendors advocated in reducing network power consumption noticing the high computation and transmission power cost of a fully switched-on network as well as strong fluctuations in the cost of energy supplies. Features with the objective of improving Network Energy saving were specified during NR Rel-18, one of these NES enhancements is spatial-domain (SD) adaptation enabling network to configure UE with measuring/reporting multiple CSIs corresponding to multiple DL transmission hypotheses with different subsets of activated antennas/antenna ports to enable network selecting the most efficient operating point of the performance-energy consumption trade-off. These solutions had multiple issues/limitations:

- The scope was limited to Type-I single-panel (SP) codebook (CB) corresponding to a low-resolution beamforming. While Type-I SP CB is the basic feature, high-resolution CBs are key enablers of Spatial domain (SD-NES), since they provide much higher beamforming gains to improve performance.

- Reporting multiple CSI-reports leads to substantial increase in CSI feedback overhead consuming a large portion of the UL resources.

- Spatial domain (SD-NES) is disadvantageous to UE power consumption as it is configured to measure multiple CSIs to enhance NES. This issue extends to Rel-19 on the CSI-RS resource/port counting process when the CSI-RS resources/ports are associated with multiple measurements.

Observation 5: Spatial domain (SD-NES) framework defined in 5G NR has multiple issues/limitations, including limitation in supported CB types, large CSI feedback overhead, and increased UE power consumption.

These drawbacks will be further pronounced in 6G where support for a larger number of antenna ports is expected and therefore a new SD NES framework is needed in 6G. This framework should support a flexible configuration, enabling the UE to measure/report multiple CSIs corresponding to a common configuration that is associated with CSI measurements resources that can be used for multiple SD patterns, and hence reduce CSI computational complexity at UE. In addition, CSI feedback of a full antenna array can be used by the NW to infer precoders for SD patterns corresponding to selected sub-arrays.

Aside from SD NES, the NR Rel-19 channel model study for 7-24 GHz, has noted the impact of spatial non-stationarity (SnS) in this frequency range, leading to a varying channel impulse response across different antennas of a large antenna array, e.g., due to partial blockage. SnS manifests due to a blocker/scatterer being in the near-field region with respect to either the BS or UE. The issue is generally more relevant from BS perspective due to the much larger antenna aperture size esp. in XL-MIMO. As a simple example, assume a case in which the BS antenna array comprises two subsets of antenna elements: the first subset of antennas are “visible” to the UE, and the second subset of antennas are invisible/blocked to the UE, e.g., yellow-highlighted antennas and grey-highlighted antennas in Figure. 15., respectively. From BS perspective, it is necessary to identify the two subsets of antennas to avoid extreme variations in channel coefficients across antennas to reduce decoding errors, as well as to save energy via disabling blocked antennas in the second subset.

Observation 6: Identifying spatial non-stationarity (SnS) behaviour across BS antennas is necessary to maintain uniform channel behaviour across transmissions from BS, as well as saving energy in case a subset of BS antennas are blocked from the UE perspective.

To address the observations above, a new 6G framework that enables flexible MIMO configuration needs to be studied, including configuration, measurement and reporting that enables the inference of a precoding coefficients corresponding to an antenna sub-array of a full precoding matrix corresponding to the full antenna array, without violating the codebook basis orthogonality to avoid inter-layer interference.

Proposal 15: Study the necessity/feasibility of deriving a new framework for 6G that enables flexible MIMO configuration by the BS, while neither increasing UE complexity nor the corresponding CSI feedback overhead.

| X | X | X | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X | X | X | X | X | X | X | X |
| X | X | X | X | X | X | X | X |
| X | X | X | X | X | X | X | X |
| X | X | X | X | X | X | X | X |
| X | X | X | X | X | X | X | X |
| X | X | X | X | X | X | X | X |
| X | X | X | X | X | X | X | X |

Figure 14: Spatial non-stationarity causing non-uniform channel correlation across antenna elements, wherein antenna elements with different colors are mutually uncorrelated

##### Network Circuit energy consumption reduction for large number of antenna ports

The expansion of the antenna port dimension leads to an inevitable increase in the network circuit power consumption especially power from RF components such as ADC, DAC etc., assuming the same MIMO hardware technology. This is, at least in part, due to the increased number of energy-hungry components which scale linearly with the number of the digital MIMO ports. As part of the transceiver chain process, data conversion relates the baseband analog signal to its sampled digital representation. While impact of a high-resolution conversion (e.g., 14 bits and above) is usually neglected from the signal quality perspective due to the negligible effect compared to the other signal impairments, e.g., thermal noise and interference, the convertor may have a significant contribution in the overall transceiver power consumption, especially with the increase of processing bandwidth, convertor resolution, and number of the radio chains. It is well known that the power consumption of the data convertors increases super-linearly with the increase of sampling rate and increase exponentially with the number of the quantization bits (convertor resolution). From the popular DAC components, a quick observation can be made where DACs with 500 MS/sec with 16 quantization bits consume power (an average reported value for the available products) of approx. 930 mW, whereas the consumption level is approx. 370 for the DACs with a reduced resolution of 10 bits. Such levels of convertor power consumption are prohibitively large, when used in a MIMO deployment with large array dimensions.

In Figure 15, it is observed that the additional gain in spectral efficiency achieved by higher-resolution convertors diminishes with resolution (i.e., spectral efficiency saturates beyond a certain resolution). Therefore, to save energy without sacrificing considerable spectral efficiency, the network should be able to switch between different ADC/DAC (e.g., 2 or more) resolutions based on link condition, resulting in an energy-efficient solution, as motivated by Figure 16. 6G should therefore study MIMO architectures with both low- and high-resolution Tx/Rx chains and enable adaptive switching between them. Here several problems need to be considered, such as evaluating EVM and other RAN4 requirements with distortion caused by low-resolution ADCs/DACs and evaluating beam management/RS solutions and schemes that are robust against low-resolution ADC/DAC quantization distortions.



Figure 15: Spectral efficiency analysis of a MIMO system with various ADC resolutions across different Tx power operating points illustrates the limited SE available gains with increasing ADC resolutions.

Figure 16: Energy efficiency analysis of a MIMO system with various ADC/DAC resolutions across different Tx power operating points illustrating optimal ADC resolution for energy efficiency.

Observation 7: Given the lower per element cost and energy consumption and the expected larger array dimensions for FR3 deployments, it is of interest to study the impact of LR transmission and receiver chains from the perspective of feasibility, spectral and energy efficiency and to identify potential physical layer enhancements.

Proposal 16: RAN1 to study MIMO architectures with support of low-resolution convertors from the perspective of compliance with RAN4 requirements, as well as the potential enhancements associated with beam management and reference signals adjusted to the low-resolution MIMO operation.

### 6G device power saving

UE power saving in 5G was mostly an add-on feature introduced in the later releases of 5G NR resulting in non-deployment of many of the UE power saving features. 6G should consider UE power saving to improve the user experience as a Day-1 feature. Although many UE power saving features have been specified in 5G for idle and connected mode, 6G Day-1 should identify features that brings in substantial energy saving while at the same time trim down the feature list from 5G NR power saving.

| Release | Power Saving Feature | Description |
| --- | --- | --- |
| Rel-15 | Cross-slot scheduling with time gap | UE can switch off RF between PDCCH and PDSCH when scheduled later in the slot. |
|  | BWP adaptation | UE switches to narrower Bandwidth Parts (BWPs) in low traffic to save power. |
|  | Antenna/MIMO layer adaptation | Dynamically reduce MIMO layers and active antennas at UE. |
|  | Search space set group switching (baseline) | Reduce PDCCH monitoring by switching search space sets. |
| Rel-16 | Wake-up signaling (WUS) via DCI 2_6 (connected DRX) | Enables UE to skip DRX on-duration timer if no scheduling is expected. |
|  | SCell dormancy | Secondary cells (SCells) can be deactivated for long periods to save power. |
|  | Assistance Information (Preferred DRX, BWP, SCell config, etc.) | UE preference to optimize energy. |
| Rel-17 | Early Paging Indication | UE can be informed earlier whether paging is present, allowing more sleep time. |
|  | TRS (Tracking Reference Signal) availability signaling | Reduces UE measurement frequency by signaling TRS presence explicitly. |
| Rel-18 | PDCCH monitoring skipping | Allows skipping PDCCH monitoring occasions without impacting reliability. |
|  | RRM measurement relaxation | Further relaxes idle/inactive measurements for energy efficiency. |
| Rel-19 | LP-WUR/LP-WUS (Low Power Wake-Up Receiver & Signal) | A low-power receiver monitors a simplified wake-up signal, avoiding main RF activation unless needed. Supports paging & connected DRX. |

Table 3: UE power saving features per 3GPP Release

Some of the features can be broadly classified into the following categories shown in Figure 17 and can be directly from 5G NR as baseline for further optimization. Since 6GR considers unified RAT supporting multiple device types e.g., MBB, IoT, a common power saving technique to address various device type can be envisioned for day-1.

Proposal 17: Study a unified device power saving mechanism using following techniques to support diverse device types from day-1

- Time domain technique

- Frequency domain technique

- Spatial domain technique

- Measurement relaxations

- Processing domain technique

Figure 17: Illustration of various device power saving techniques

#### Synchronization Raster definition

Synchronization signal detection complexity including cell-search time and energy depends on the number of synchronization raster hypotheses and should be one of the key points to be addressed at the early stage of the 6G study item as it is related both to the network deployments and energy saving at both BS and UE. The Synchronization signal detection is a function of number of synchronization frequency raster hypotheses to be searched in a frequency range and search time for each such synchronization raster point depends on the synchronization signal periodicity. The synchronization raster points to be defined for the existing frequency bands and as well as for the new upper mid band frequency.

Proposal 18: Discuss synchronization raster definition in 6GR for both existing and new frequency range considering device power saving.

#### Considerations on Low power radio architecture for 6GR

Low power radio architecture e.g., LPWUR/LPWUS has potential to bring substantial device power saving at low paging rate around 1% which is possible at low cell load. At higher paging rate, frequent wake-up leads to increased device power consumption. A unified low power architecture design across device types can be beneficial from Day-1. In 5G, a new set of low-power signals design is needed, thereby increasing the energy consumption at the BS. To avoid introducing additional network energy consumption at BS, 6GR design should consider unified physical channel design for both Main radio and Low power radio from day-1.

In 5G it was found that ON-OFF keying increased the transmission/reception duration of RF both at the network and device side. Furthermore, the addition of Manchester coding, repetitions, Reed-Muller coding specified to improve the coverage of ON-OFF keying finally resulted in increased transmission/reception duration thereby increasing the network and device energy consumption. The coverage of Low power radio architecture compared to Main radio is one of the bottlenecks due to different receiver architecture, noise figure, waveform. The receiver using sequence-based detection provides improved coverage for the low power radio compared to the energy detection. The tradeoff between different receiver architecture is whether to support better signal detection thereby improved coverage of Low power radio using sequence based receiver or minimized energy consumption of low power radio using energy detection receiver. The component between low power radio and MR should be reused as much as possible to reduce the bill of materials.

Low power radio architecture should revisit the choice of waveform in 6GR to improve coverage and reduce transmission/reception duration to save both network and device energy consumption. The DFT-spread sequence can be starting point without considering the overlaid sequence design used in 5G NR. The DFT-spread sequence provides sufficient coverage resulting in overall energy saving at both NW and device side

Figure 18: DFT-s-spreaded LPWUS sequence generation

Proposal 19: Study low power radio architecture in 6GR by considering

- A unified design catering to diverse device types

- Minimize the impact on network power consumption

- Improved coverage and reduced transmission/reception duration

In 6GR, the low power radio architecture usage can be extended to extract device power saving beyond envisioned in 5G NR. In one such aspects, the paging monitoring occasions can be offloaded to the low-power radio thereby avoiding power hungry main radio from not monitoring paging occasions and extending its ultra deep sleep/deep sleep state. The paging occasions can be instead monitored and received at the low-power radio waking up the main radio after successful paging reception to transmit random access preamble.

Figure 19: Illustration of using low power radio architecture to monitor idle mode paging occasion

Proposal 20: Study the feasibility of low power radio usage in 6GR for idle mode paging occasion monitoring.

#### Considerations on Connected mode device power saving

The connected mode device power saving includes reduced PDCCH blind decoding attempts, since it consumes a lot of power to blindly decode PDCCH at UE side. In addition, PDCCH monitoring indication, skipping and resumption indication can be utilized to avoid unnecessary PDCCH monitoring to save device power.

In 6G systems, a two-stage Downlink Control Information (DCI) framework can improve energy efficiency and signaling flexibility for a wide range of devices and use cases. In the first stage, a compact, high-reliability control message conveys only essential parameters—such as waveform configuration, time-frequency scheduling, or common signaling for a set of UEs—while minimizing processing complexity (e.g., reducing the number of blind decoding attempts). The second stage carries a more extensive control payload, with its detection parameters (e.g., PDCCH format, search space) indicated by the first-stage message. UEs not addressed in the first stage can immediately discontinue monitoring, thereby avoiding the energy cost of decoding a longer second-stage message. From the network perspective, transmitting a common compact first-stage control message to multiple UEs, potentially with repetitions to improve coverage, can enhance control channel efficiency and reduce transmission energy, supporting both enhanced capacity and extended coverage scenarios.

Low-power radio can be efficiently utilized when the main radio is not monitoring PDCCH, to trigger the monitoring of PDCCH at the main radio, skip the PDCCH monitoring etc., Yet another approach is the configurable number of candidate blind decoding attempts similar to the search space switching group switching as it greatly reduces the maximum number of blind decode for a UE during low activity period. BWP adaptation, MIMO layer adaptation, SCell dormany indication, RRM measurement relaxation from 5G NR must also be considered in 6GR day-1 functionality to reduce the power consumption and improve the user experience.

Proposal 21: Study the feasibility of reducing the device power consumption in the connected mode by avoiding unnecessary PDCCH monitoring, minimizing blind decoding attempts.

# Conclusions

Proposal 1: 6GR should aim to reduce TCO by utilizing the non-backward compatible opportunity provided by once in a decade new generation refresh by designing native energy efficient solution using various techniques such as time, frequency, spatial, power domain.

Proposal 2: 6GR should enhance common signals/channels transmission and periodicity for network energy saving under various cell load.

Proposal 3: The number of candidate SS/PBCH beams from XL-MIMO in the upper mid-band spectrum and its impact on the network energy consumption needs to be studied and evaluated.

Observation 1: Energy saving gains of 20% to 30% from lean FDMed SS/PBCH block structure can be achieved in the upper mid band spectrum compared to the existing 5G NR SS/PBCH block structure.

Proposal 4: Evaluate the network energy consumption of various SS/PBCH block structures with different periodicity envisioned for all frequency ranges.

Observation 2: Optimizing gaps between SS/PBCH block transmission within a SSB burst can bring 9% to 10% energy saving gains compared to 5G NR SS/PBCH.

Proposal 5: Study the feasibility and impact of reducing gaps between SS/PBCH block transmission within a SSB burst.

Observation 3: Clustering/grouping SSB bursts with minimum inter-SSB burst gap and larger inter-cluster gap can reduce SSB acquisition latency compared to the uniformly spaced 5G NR SSB.

Proposal 6: Study the feasibility of clustering/grouping of SSBs and other common signal/channel and compare pros and cons in terms of acquisition latency and network energy consumption.

Proposal 7: Evaluate the compact paging frame against dispersive paging frame of 5G NR with respect to network energy saving and latency.

Proposal 8: Study a common adaptation framework to jointly adapt common signal/channel by exploiting the linkage between them.

Proposal 9: Study on how alignment between Cell DTX/DRX and UE DTX/DRX cycles can be achieved by minimizing the transmission of signals outside the cell active time.

Proposal 10: Consider a new relative power value and transition duration for a new ultra-deep sleep state (lower than the deep sleep state) to envision network energy saving for low load cases.

Observation 4: The longer SSB acquisition time and latency in acquiring cell quality thresholds from RMSI affects the cell selection time.

Proposal 11: Analyze the impact to the cell selection latency from increased SSB and RMSI periodicity.

Proposal 12: Study the feasibility of network energy saving from various PAPR/CM reduction techniques such as DL waveform using DFT-s-OFDM, selective mapping, tone reservation.

Proposal 13: Consider multi-carrier environment to minimize the number of always-on cell/carrier extending the on-demand common channel framework.

Proposal 14: Consider in the Evaluation assumption for the network energy saving in the new upper-mid-band spectrum:

- Wider bandwidth, 200MHz

- Increased Tx & Rx RUs – 256, considering XL-MIMO operations

- Various Duplexing operations i.e., TDD, SBFD

Observation 5: Spatial domain (SD-NES) framework defined in 5G NR has multiple issues/limitations, including limitation in supported CB types, large CSI feedback overhead, and increased UE power consumption.

Observation 6: Identifying spatial non-stationarity (SnS) behaviour across BS antennas is necessary to maintain uniform channel behaviour across transmissions from BS, as well as saving energy in case a subset of BS antennas are blocked from the UE perspective.

Proposal 15: Study the necessity/feasibility of deriving a new framework for 6G that enables flexible MIMO configuration by the BS, while neither increasing UE complexity nor the corresponding CSI feedback overhead.

Observation 7: Given the lower per element cost and energy consumption and the expected larger array dimensions for FR3 deployments, it is of interest to study the impact of LR transmission and receiver chains from the perspective of feasibility, spectral and energy efficiency and to identify potential physical layer enhancements.

Proposal 16: RAN1 to study MIMO architectures with support of low-resolution convertors from the perspective of compliance with RAN4 requirements, as well as the potential enhancements associated with beam management and reference signals adjusted to the low-resolution MIMO operation.

Proposal 17: Study a unified device power saving mechanism using following techniques to support diverse device types from day-1

- Time domain technique

- Frequency domain technique

- Spatial domain technique

- Measurement relaxations

- Processing domain technique

Proposal 18: Discuss synchronization raster definition in 6GR for both existing and new frequency range considering device power saving.

Proposal 19: Study low power radio architecture in 6GR by considering

- A unified design catering to diverse device types

- Minimize the impact on network power consumption

- Improved coverage and reduced transmission/reception duration

Proposal 20: Study the feasibility of low power radio usage in 6GR for idle mode paging occasion monitoring.

Proposal 21: Study the feasibility of reducing the device power consumption in the connected mode by avoiding unnecessary PDCCH monitoring, minimizing blind decoding attempts.

# References

[1] NGMN RPAF Paper: https://www.ngmn.org/publications/ngmn-radio-performance-assessment-framework.html

[2] R1-2505792, ‘Discussion on 6GR waveform’, RAN1-122, India meeting

[3] 3GPP TR 38.864 ‘ Study on network energy savings for NR’, Release 18