3GPP TSG RAN WG1 #122	R1-2506052

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.3.1

Source: 	ETRI

Title:	Maintenance on SBFD TX/RX/measurement procedures

Document for:	Discussion/Decision

Introduction

This contribution presents ETRI’s views on remaining details for SBFD TX/RX/measurement procedures.

Discussion

In RAN1#121 meeting, the following agreements on valid symbol type of CMR and IMR for CSI derivation were made [1]:

Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation configured in CSI-ReportConfig applies to both CMRs and IMRs (limited to NZP-CSI-RS) associated with the CSI report.

The corresponding TPs in TS 38.214 is shown by the table below [2]:

For a CSI report associated with semi-persistent or periodic CSI-RS, [or with semi-persistent or periodic SRS-RSRP measurement resources, or with semi-persistent or periodic CLI-RSSI measurement resource, and] if a UE is configured with SBFD symbols and a CSI-ReportConfig with the higher layer parameter symbolType, the UE only considers the CSI-RS occasions within either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for the CSI derivation for the CSI report. The symbolType configured in CSI-ReportConfig applies to NZP-CSI-RS resources that are associated with the CSI report and configured for both channel measurement and interference measurement.

As a consequence, only the IMR configured with ZP CSI-RS (i.e., CSI-IM) cannot be associated with valid symbol type for SBFD/non-SBFD operations. Thus, the interference and noise measured by ZP CSI-RS shall/could be mixed up together. From our understanding, it causes several issues, such as:

- UE shall aggregate all the interference and noise from different SBFD/non-SBFD symbol types, although the SBFD/non-SBFD symbols are exclusive to each other and never be shared from network perspectives, and

- the current specification does not clearly capture that “the IMR configured with ZP CSI-RS cannot be associated with valid symbol type for SBFD/non-SBFD operations”

Therefore, the following TPs are suggested to address the issues above:’

Proposal 1. Support valid symbol type for CSI-IM, and agree the following TP for TS 38.214:

##### 5.2.1.4.1 Resource Setting configuration <unchanged texts omitted> For a CSI report associated with semi-persistent or periodic CSI-RS, [or with semi-persistent or periodic SRS-RSRP measurement resources, or with semi-persistent or periodic CLI-RSSI measurement resource, and] if a UE is configured with SBFD symbols and a CSI-ReportConfig with the higher layer parameter symbolType, the UE only considers the CSI-RS occasions within either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for the CSI derivation for the CSI report. The symbolType configured in CSI-ReportConfig applies to NZP-CSI-RS resources that are associated with the CSI report and configured for both channel measurement and interference measurement. The symbolType configured in CSI-ReportConfig applies to CSI-IM that are associated with the CSI report. <unchanged texts omitted>

| Reason for change: | SBFD/non-SBFD valid symbol type is not applicable for CSI-IM. |
| --- | --- |
| Summary of change: | Clause 5.2.1.4.1: Clarify that valid symbol type is also applicable for CSI-IM |
| Consequences if not approved: | The UE behavior on CSI derivation for SBFD symbol is ambiguous. |

Conclusion

In this contribution, ETRI’s views on SBFD were shown and the following observation and proposals were made:

Proposal 1. Support valid symbol type for CSI-IM, and agree the following TP for TS 38.214:

##### 5.2.1.4.1 Resource Setting configuration <unchanged texts omitted> For a CSI report associated with semi-persistent or periodic CSI-RS, [or with semi-persistent or periodic SRS-RSRP measurement resources, or with semi-persistent or periodic CLI-RSSI measurement resource, and] if a UE is configured with SBFD symbols and a CSI-ReportConfig with the higher layer parameter symbolType, the UE only considers the CSI-RS occasions within either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for the CSI derivation for the CSI report. The symbolType configured in CSI-ReportConfig applies to NZP-CSI-RS resources that are associated with the CSI report and configured for both channel measurement and interference measurement. The symbolType configured in CSI-ReportConfig applies to CSI-IM that are associated with the CSI report. <unchanged texts omitted>

| Reason for change: | SBFD/non-SBFD valid symbol type is not applicable for CSI-IM. |
| --- | --- |
| Summary of change: | Clause 5.2.1.4.1: Clarify that valid symbol type is also applicable for CSI-IM |
| Consequences if not approved: | The UE behavior on CSI derivation for SBFD symbol is ambiguous. |

References

- RAN1#121 chair notes

- 3GPP TS 38.214 V19.0.0