3GPP TSG RAN WG1 #122			R1-2506096

Bengaluru, India, August 25th – 29th, 2025

Source:	CMCC

Title:	Discussion on evaluation assumptions for 6GR air interface

Agenda item:	11.2

Document for:	Discussion & Decision

- Introduction

In RAN#108 meeting, a new SI of study on 6G Radio was approved [1], one of the objectives is to evaluate the performance of 6GR physical layer including energy efficiency, spectrum efficiency and coverage. The related objective is as following:

The detailed objectives of the study are: (2) Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1

In this contribution, we discuss evaluation assumptions for 6GR air interface, including deployment scenario, evaluation of 6G coverage, evaluation of NTN, evaluation assumptions of power model, evaluation assumptions for initial access and evaluation of sensing.

- Discussion on deployment scenarios

In the RAN#108 meeting, the 6G TR for scenarios and requirements was updated [2] with 6G deployments scenarios and key performance indicators (KPIs). In the updated TR, more than 10 candidates of deployment and scenario are identified to evaluate for 6G services. However, the following tasks need to be done in RAN1 to start performance evaluation:

- RAN1 needs to identify the deployment scenarios to evaluate

- For some deployment scenarios, some parameters such as carrier frequencies, aggregated system level bandwidth still have many options

- More detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. need to be defined

In this section, we discuss the following aspects:

-	Views on the deployment scenarios to evaluate and general evaluation methodology

-	Proposals on evaluation parameters including carrier frequency, aggregated system level bandwidth, BS antenna elements

Besides, regarding carrier frequency, aggregated system bandwidth, BS antenna elements, we suggest taking the highlight values toward the down selection of evaluation options.

Indoor hotspot (InH)

Table 1-1:Indoor hotspot (InH) (System-level simulation)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz Around 7 GHz Around 4GHz+Around 7GHz Around 30 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1 GHz (DL+UL) |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m (Equivalent to 12TRxPs per 120m x 50m) |
| BS antenna elements | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 1024 Tx and Rx antenna elements Around 30 GHz: Up to 1024Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, 10 users per TRxP |
| Service profile | Non-full buffer traffic as baseline. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced. |

Dense Urban

Table 1-2: Dense Urban (System-level simulation)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz |
| Aggregated system bandwidth | Around 2GHz: Up to 120 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz |
| ISD | Macro: 200m Micro: 3 micro TRxPs per macro TRxP Micro: 100m All micro TRxPs are all outdoor |
| BS antenna elements | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 1024 Tx and Rx antenna elements Around 30 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | Two layers: Step1: Uniform/macro TRxP, 10 users per TRxP Step2: Uniform/macro TRxP + Clustered/micro TRxP, 10 users per TRxP Single layer: 10 users per TRxP 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | Non-full buffer traffic as baseline. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced. |

Urban macro

Table 1-3: Urban macro (System-level simulation)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 2GHz Around 4 GHz Around 7 GHz Around 4 GHz + Around 30GHz Around 4 GHz + Around 7 GHz |
| Aggregated system bandwidth | Around 2GHz: Up to 40 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: 1GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers for Around 4 GHz + Around 30 GHz: - Macro layer: Hex. Grid, 4GHz - Micro layer: Random drop, 30GHz |
| ISD | Macro: 500m |
| BS antenna elements | Around 2 GHz: Up to 256 Tx and Rx antenna elements Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 1024 Tx and Rx antenna elements Around 30 GHz: Up to 1024 Tx and Rx antenna elements |
| UE antenna elements | Around 2 GHz: Up to 8 Tx and Rx antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 50% outdoor vehicles (120km/h) and 50% indoor (3km/h), 10 users per TRxP |
| Service profile | Non-full buffer traffic as baseline. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced. |

Rural

Table 1-4: Rural (System-level simulation)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7GHz (ISD 1) Around 700MHz + Around 7GHz (for ISD 1) |
| Aggregated system bandwidth | Around 700 MHz: Up to 20 MHz (DL+UL) Around 2GHz: Up to 120 MHz (DL+UL) Around 4 GHz: Up to 200 MHz (DL+UL) Around 7GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732m ISD 2: 5000m |
| BS antenna elements | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 7 GHz: Up to 1024 Tx and Rx antenna elements Around 700 MHz: Up to 64 Tx and Rx antenna elements |
| UE antenna elements | Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 7 GHz: Up to 8 Tx and Rx antenna elements Around 700 MHz: Up to 4 Tx and Rx antenna elements |
| User distribution and UE speed | 50% outdoor vehicles (120km/h), 50% indoor (3 km/h) 10 users per TRxP |
| Service profile | Non-full buffer traffic as baseline. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced. |

High speed

Figure 1. Layout illustration of deployment scenario of “High speed”

Table 1-5: High speed (Link-level simulation)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro NOTE2 only: Around 4 GHz Macro NOTE2+ relay nodes: 1) For BS to relay: Around 4 GHz For relay to UE: Around 30 GHz or Around 70 GH or Around 4 GHz 2) For BS to relay: Around 30 GHz For relay to UE: Around 30 GHz or Around 70 GHz or Around 4 GHz |
| Aggregated system bandwidth | Around 4 GHz: Up to 200 MHz (DL+UL) Around 30 GHz or Around 70 GHz: Up to 1 GHz (DL+UL) |
| Layout | Macro only: -Around 4 GHz: Dedicated linear deployment along the railway line as in Figure 6.1.5-1 in TR 38.913. RRH site to railway track distance: 100m Macro + relay nodes: -Around 4 GHz: Dedicated linear deployment along the railway line as in Figure 6.1.5-1 in TR 38.913. RRH site to railway track distance: 100m -Around 30 GHz: Dedicated linear deployment along the railway line as in Figure 6.1.5-2 in TR 38.913. RRH site to railway track distance: 5m. |
| ISD | -Around 4 GHz: ISD 1732m between RRH sites, two TRxPs per RRH site. See Figure 6.1.5-1 in TR 38.913. -Around 30 GHz: 1732m between BBU sites, 3 RRH sites connected to 1 BBU, one TRxP per RRH site, inter RRH site distance (580m, 580m, 572m). See Figure 6.1.5-2 in TR 38.913. -Small cell within carriages: ISD = 25m. |
| BS antenna elements | Around 4 GHz: Up to 256 Tx and Rx antenna elements Around 30 GHz: Up to 256 Tx and Rx antenna elements |
| UE antenna elements | Relay Tx: Up to 256 antenna elements Relay Rx: Up to 256 antenna elements Around 4 GHz: Up to 8 Tx and Rx antenna elements Around 30 GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% of users in train For non-full buffer, 300 UEs per macro cell (assuming 1000 passengers per high-speed train and at least 10% activity ratio) Maximum mobility speed: 500km/h |

Proposal 1: For simulation assumptions, RAN1 to take Table 1-1 / 1-2 / 1-3 / 1-4 in R1-2506096 as deployment scenarios for Indoor hotspot (InH), Dense Urban, Urban macro, and Rural respectively.

Proposal 2: For simulation assumptions, RAN1 to take Table 1-5 in R1-2506096 as deployment scenarios for High speed scenario.

- Discussion on the evaluation of 6G coverage

Deployment scenarios

f) Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

According to the SID [1], reusing of the existing 5G mid-band(~3.5GHz) site grid for 6G deployments is listed as one of the design targets, which can save the costs for renting new sites for 6G base stations. Also it implies that though the higher operation band (around 7GHz) would be used in 6G, operators can only purchase the same amount of base stations as in 5G. The coverage performance is required for the comparison with 5G NR.

Proposal 3: Compared with 5G, similar or better coverage performance can be considered for 6G design.

k) Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. i. RAN4 can be involved, if necessary, based on the LS from RAN1

For the evaluation of the coverage performance, the deployment scenarios should be defined or selected first. Based on the latest study on 6G scenarios [2], at least 10 scenarios were proposed and captured, including indoor hotspot, dense urban, rural, urban macro, sub-urban macro and etc. Not all of them require the coverage performance evaluations. Urban macro and rural scenarios which are targeted for providing continuous services to the normal UEs can be considered in first.

Urban macro deployment is assumed to have 500m inter-site distance of providing continuous coverage in the main cities. And the rural scenario is assumed to have 1732m inter-site distance (in legacy evaluations) and provides continuous coverage in the rural area. Both of them can be considered in the first phase evaluations. But considering that 5G was first deployed in the main cities in many countries, then urban macro has higher priority for the coverage evaluation.

Proposal 4: Both urban macro and rural scenarios can be considered for coverage performance evaluation. And urban macro scenario has higher priority.

The scenarios such as indoor hotspot and dense urban which are targeting for the high capacity and may not have issues of the coverage can be deprioritized. And other scenarios, such as sub-urban macro, high speed, urban coverage of massive connection can reuse the similar deployment strategy and technologies as urban macro and rural, then a dedicated evaluation may not be needed. The Extreme long distance coverage in low density area can be discussed later when the definition of this scenario is clear. NTN scenarios definitely requires enhancements or design for the coverage considering the long propagation distance between satellites and UEs. The coverage performance of the NTN can be discussed when specific deployment scenarios e.g. LEO, GEO are concluded.

Traffic

The coverage performance is usually evaluated with an assumed traffic and required data rate. In Rel-16 SI of coverage enhancements, DL 10Mbps and UL 1Mbpsa was assumed for the urban cells. And 1Mbps for DL and 100kbps for uplink were considered in rural area [3]. Rural with long distance was optionally required for the evaluations with DL 1Mbps and UL 30kbps. VoIP traffic was considered with 320bits packet size and 20ms arrival time. In 6G both wideband traffic with a requirement for data rate and VoIP traffics should be considered for the coverage evaluations.

The LPWA (low power and wide area) traffic is considered to provide IoT services in the 6G. The specific required data rate and coverage of LPWA can be discussed considering the deployment scenario and operation frequencies.

Proposal 5：Both requirements of wideband traffic with high data rate and VoIP, LPWA traffic should be considered for the coverage performance evaluations.

Working frequency band

As mentioned in the SID, 7GHz can be considered for the coverage evaluations at least for the wideband traffic. And if 7GHz is assumed, the antenna element numbers at base station side and UE side can also be discussed for a more realistic assumption. Since 7GHz is higher than 2.6GHz and 3.5GHz, the propagation loss would be even worse than 5G. Additional propagation loss and penetration loss compare with the 5G frequency should be considered in the link budget calculation.

Proposal 6: 7GHz can be considered as main operation frequency for the coverage evaluations.

Channels for evaluations

Since the coverage evaluation is for the design of new generation technologies, both uplink and downlink channels in idle mode and connected model should be considered. In the idle mode, PBCH, PDCCH and PDSCH for the system information, PRACH, PUSCH and PUCCH used for Msg 3, Msg 5 and HARQ-ACK for Msg 4 should be considered for the evaluation to guarantee the performance of initial access. And the in the connected mode, the PDCCH, PDSCH, PUCCH and PUSCH should also be considered.

Proposal 7: Both uplink and downlink channels in the idle mode and connected mode should be considered for the coverage evaluations.

Evaluation methodologies

Coverage performance is usually carried out with a link budget template table considering both parameters from based station side and UE side. The transmit power and the antenna assumptions are important for the coverage evaluation. And the required SINR of the specific channels can also determine the coverage performance. Link level simulations can be used to evaluate the required SINR for specific channels with specific traffic. And the link budget template from Rel-16 CE and IMT-2020 self-evaluation can be the starting point.

Proposal 8: Link level simulation can be carried out to evaluate the SINR requirements for the specific channel and traffic. Link budget template from Rel-16 CE and IMT-2020 self-evaluations can be considered as a starting point.

Performance metrics

In Rel-16 coverage performance evaluation, different performance metrics were proposed as performance metrics, including MCL(Maximum Coupling Loss), MIL(Maximum Isotropic Loss), MPL(Maximum Pathloss). MCL considers transmit power, receiver sensitivity and base stations antenna gain. MIL is similar as MCL but excluding some detailed components for antenna gains calculation. MPL further excludes the shadow fading gain in the propagation loss and penetration loss and diversity gain at transmitter and receiver. MCL can be considered as the outputs from the link budget template which is more close to that from IMT-2020 self-evaluations.

TR38.830: Definition of MCL: - MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2). - More details can be found in the link budget template shown in Annex A.3 Definition of MIL: - MIL = Total transmit power – Receiver sensitivity – Tx loss – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain. - More details can be found in the link budget template shown in Annex A.3 Definition of MPL: - MPL = MIL – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gains. - More details can be found in the link budget template shown in Annex A.3

Proposal 9: A unified coverage target can be considered for all the channels and signals. Define a minimum MCL target for 6G coverage enhancement for around 7GHz considering: penetration loss, antenna gain. The MCL target aims to re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. For traffic channels, at least 1Mbps data rate at cell edge for 6G should be considered as the coverage targets.

- Discussion on the evaluation of NTN

As 6G is expected to provide heterogeneous services and seamless network coverage, the integrated 6G TN-NTN network architecture and air interface design is considered to promise coverage extension to underserved areas. We can evaluate the performance of different NTN deployment scenarios and configurations through KPIs such as throughput, latency, reliability, coverage, and connection density. And it is suggested to first evaluate the coverage performance based on the new 6G structure.

Deployment scenarios

To comprehensively study multiple NTN scenarios with enhanced service capability, various deployment scenarios should be studied and identified for NTN. Regarding the satellite orbit, both GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit) based NTN scenarios and corresponding satellite parameters are assumed in TR 38.821. From our perspective, the evaluation of 6G NTN can still focus on the above scenarios, and detailed satellite parameters (e.g., Set-1/2 specified in TR 38.821) for each scenario can be considered as a start point.

Proposal 10: Support the deployment scenarios including at least the following aspects.

- GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO).

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- Transparent/regenerative satellite payload with single/multiple beams

Target service and data rates

Regarding the target services and target data rate, the NTN-specific coverage evaluation in Rel-18/Rel-19 focus on the handset terminals (including smartphones with -5.5 dBi antenna gain) and S-band. From our perspective, the target services (i.e., VoIP and low-data rate services for commercial handset terminals) and the target data rate for each service can be used to evaluate the coverage performance in 6G NTN.

Proposal 11: For the evaluation of NTN scenarios, the following target services (i.e., VoIP and low-data rate services for commercial handset terminals) and the target data rate assumptions can be considered.

- For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- For low-data rate service, both 3 kbps and 1Mbps can be considered as DL date rate, 3 kbps and 100 kbps can be considered for UL.

Evaluation methodology

For the evaluation in 6G NTN scenarios, the evaluation methodology needs to be defined at first. And for link level coverage evaluation, At least the evaluation methodology and performance metrics defined in Rel-18/Rel-19 NTN, i.e. 3-step procedure, can be reused.

Proposal 12: Link-level DL coverage performance of NTN can be evaluated based on the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821.

- Step 2: Required SNR of target service is evaluated by LLS.

- Step 3: The CNR and the required SNR are compared to identify the coverage gap

Channel model and evaluated channels

To obtain the required SNRs of channels/signals the channel models defined in TR 38.811 can be applied for link-level simulation of 6G NTN.

Proposal 13: For coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread: NTN-TDL-A (NLOS) and NTN-TDL-C (LOS) defined in TR 38.811

Furthermore, to identify the bottleneck channels of DL coverage in NR NTN, both UL and DL channels/signals designed for 6G deployment can be evaluated. In addition, the synchronization signal and broadcast channels in initial access procedure are important for UE to access to the satellite, if these signals/channels cannot be detected by UE correctly, the UE has no chance to access to the satellite. And the following channels/signals can be considered for evaluation.

- Synchronization signal

- Broadcast signals/channels

- Initial access channels/signals

- Other UL/DL channels/signals

Proposal 14: For 6G NTN evaluation, the following channels/signals can be evaluated as a baseline.

- Synchronization signal

- Broadcast signals/channels

- Initial access channels/signals

- Other UL/DL channels/signals

- Discussion on evaluation assumptions of 6G power model

In NR, the power model for both network and UE are well discussed and captured in [4-7]. Specifically, current power model for NW and UE has the following characteristics:

- For NW power model (as defined in 3GPP TR38.864):

  - Multiple sleep modes are defined by considering

    - deep sleep/light sleep/micro sleep

    - corresponding power value for each sleep mode

    - the transition time and additional transition energy are set between different type of sleep modes.

  - Scaling method by considering resource utilization (RU) on frequency domain, spatial domain and power domain.

  - The impact of DL and UL are jointly considered during the evaluation period.

- For UE power model (as defined in 3GPP TR 38.840 (UE Power saving), TR38.869 (LP-WUS/WUR) and TR 38.865 (Redcap)):

  - Multiple sleep modes are defined by considering

    - deep sleep/light sleep/micro sleep as defined in TR 38.840 and TR38.865

    - ultra-deep sleep as defined in TR38.869

    - corresponding power value for each sleep mode

    - the transition time and additional transition energy are set between different type of sleep modes.

  - Scaling method by considering only bandwidth, or antenna and other specific cases, e.g., PDCCH only, cross-slot scheduling, PDSCH-only, short PUCCH and etc.

  - Separate DL or UL evaluation is considered during the evaluation period.

  - RRM power model and scaling law by considering different number of cells, intra/inter-frequency, synchronous/asynchronous case

By reviewing the widely used power models in 5G era, it is sophisticated to regard 6G power evaluation as building upon these models as start point and further consider their amendments.

- TR 38.864 as start point for network power model

- TR 38.840 as start point for UE power model for smart phone

- TR 38.865 as start point for LPWA only UE as supplementary to the TR38.840 UE power model

- TR 38.869 as start point for LP-WUR power model

Proposal 15: 6GR power evaluation can take followings as start point and further consider their amendments.

- TR 38.864 as start point for network power model

- TR 38.840 as start point for UE power model for smart phone

- TR 38.865 as start point for LPWA only UE power model as supplementary to the TR38.840 UE power model

- TR 38.869 as start point for LP-WUR power model

Based on the existing foundations, we have the following considerations on further amendments for 6GR power model:

The general structure for different sleep mode, transition time and additional transition energy in NR can be reused with the following improvements,

- The specific values of relative power and transition time shall be further discussed.

  - The relative power values and transition time should be set according to the product plan on time point of the commercial use for 6GR (e.g. in the year of 2030). The system design may be impacted by the setting of parameters. For instance, the transition time between different sleep modes may impact the periodicity of common signal/channel.

- Extension of UE power model to consider joint scaling of time/bandwidth/antenna and other related aspects.

  - For relative power in Active DL/UL transmission, our understanding is that the current definition and scaling method in NW power model is quite comprehensive, while the definition and scaling method in UE power consumption is relatively rough and is not suitable for the case that resources in multiple dimensions are jointly changed. Therefore, we suggest to have a unified design for the scaling method on both NW and UE power model, that is, the methodology in TR 38.864 [4] can be considered as a starting point and the component in UE side can be further discussed. From our understanding, the following methodology can be considered as the starting point. To be specific, for DL, UE power consumption can be modelled similar as the UL power model in TR 38.864 [4], which is:

      - PDL=PstaticDL+PdynamicDL

      - PstaticDL is a static part of power for UE in active, which is not scaled based on reference configurations.

      - PdynamicDL=sa*Pdyn,anteDL is a dynamic part of power for UE in active, which is scaled based on reference configuration, and sa is the fraction of active TxRUs.

    - For UL, UE power consumption can be generally modelled similar as the DL power model in TR 38.864 [4], while some difference on hardware component between NW and UE need to be considered, e.g. no intermediate frequency part for UE power model. Therefore, we suggest the following design:

      - PUL=PstaticUL+PdynamicUL

      - PstaticUL is a static part of power for UE in active, which is not scaled based on reference configurations.

      - PdynamicUL=sa*sf*spηsf,  sp*Pdyn,  PA is a dynamic part of power for UE in active, where sa, sf, sp is the fraction of active TxRUs, the ratio between the RF bandwidth and the maximum system BW, and the ratio of PSD per TxRU between the UL transmission and reference configuration, respectively, and ηsf,  sp is the PA efficiency (PAE) related to sf, sp.

- On top of the scaling method, a further issue is the modelling of PAE. In TR 38.864 [4], PAE is a simple, two-value model and cannot accurately reflect the power consumption of RF part. Current TR 38.840 [5] only assumes two Tx power values 0dB and 23dB, and it is very coarse. Thus, a more accurate PAE model shall be considered for NW and UE energy consumption in 6GR.

- Another point worth discussing is whether to introduce a LP-WUS/LP-WUR for NW side. From our point of view, it is worth studying in current SI stage and requires a specific power model.

Proposal 16: For both NW and UE power model for 6GR, consider the following tables as the starting point and further discuss the specific value:

- Note that the setting of parameter may closely related to the system design. For instance, the transition time between different sleep modes may strongly impact the periodicity of common signal/channel.

Table 5-1: Power states and relative power values of BS or UE power consumption model

| Power state | Characteristic | FR1 TDD | FR1 FDD | FR2 TDD |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. |  |  |  |
| Deep sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. |  |  |  |
| Light sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. |  |  |  |
| Micro sleep | There is neither DL transmission nor UL reception. Immediate transition is assumed for energy saving study purpose from or to a non-sleep state. |  |  |  |
| Active DL | There is only DL transmission. |  |  |  |
| Active UL | There is only UL reception. |  |  |  |

Table 5-2: Total transition time and additional transition energy of BS or UE power consumption model

| Power state | Total transition time | Additional transition energy |
| --- | --- | --- |
| Ultra-deep sleep |  |  |
| Deep sleep |  |  |
| Light sleep |  |  |

Proposal 17: For NW power model in 6GR, consider the scaling method in TR 38.864 as the starting point.

Proposal 18: For UE power model in 6GR, in order to scale among multiple aspects, consider the following scaling method as an example for further discussion:

- for DL:

  - PDL=PstaticDL+PdynamicDL

  - PstaticDL is a static part of power for UE in active, which is not scaled based on reference configurations.

  - PdynamicDL=sa*Pdyn,anteDL is a dynamic part of power for UE in active, which is scaled based on reference configuration, and sa is the fraction of active TxRUs.

- For UL:

  - PUL=PstaticUL+PdynamicUL

  - PstaticUL is a static part of power for UE in active, which is not scaled based on reference configurations.

  - PdynamicUL=sa*sf*spηsf,  sp*Pdyn,  PA is a dynamic part of power for UE in active, where sa, sf, sp is the fraction of active TxRUs, the ratio between the RF bandwidth and the maximum system BW, and the ratio of PSD per TxRU between the UL transmission and reference configuration, respectively, and ηsf,  sp is the PA efficiency (PAE) related to sf, sp.

Proposal 19: RAN1 to study power model for NW and UE by taken PA Efficiency into account.

- Joint Tx and Rx power consumption evaluation

For 5G UE power evaluation, most of the evaluation results only consider DL or UL transmission solely. However, for example, for UE power model, the Tx of HARQ-ACK can be of much higher power consumption than the PDSCH reception. Hence, the power consumption evaluation should consider both Tx and Rx at the same time.

Proposal 20: Power consumption evaluation should consider both Tx and Rx at the same time.

- Discussion on evaluation assumptions for initial access

In NR, evaluation assumptions for initial access have been well defined, as captured in TR 38.802. Assumptions on link level simulation for synchronization signal, RACH and PBCH, are discussed and captured in Table A.1.5-1, Table A.1.5-2, and Table A.1.5-3. System level simulation assumptions for synchronization signals/channels are also provided in Table A.2.3-1. In addition, performance metric of sequence detection, such as miss detection rate, false alarm rate, etc., and of common channel reception, e.g., BLER target, are defined.

In general, the link level and system level simulation methodology and assumptions for 6GR initial access should reuse that defined for NR initial access as a starting point.

Proposal 21: The link level and system level simulation methodology and assumptions for 6GR initial access should reuse that defined for NR initial access, as captured in TR 38.802 Table A.1.5-1, Table A.1.5-2, Table A.1.5-3, and Table A.2.3-1, as a starting point.

## 6.1 Link level simulation

In the following, we provide detailed explanation on the proposed changes marked by (*) from Table 6-1 to 6-3.

The link level simulation assumptions are summarized in the following table.

Table 6-1: Link level simulation assumptions for synchronization signal and PBCH

| Common for synchronization signal and PBCH |  |  |
| --- | --- | --- |
|  | Below 6GHz | Above 6GHz |
| Carrier Frequency | 4 GHz | 7GHz |
| Channel Model | CDL-C or TDL-C(*) model with delay scaling values of 100 ns |  |
| Subcarrier Spacing(s) | 15 kHz, 30 kHz (other values are not precluded and reported by companies) |  |
| Transmission bandwidth(*) | 3 MHz (with 15 kHz SCS) or 5 MHz (with 15 kHz and 30 kHz SCS) |  |
| SNR range | > -6dB |  |
| Search window(*) | The time window to search (correlate) PSS. It depends on the periodicity of SS transmission. Candidate values such as 40 ms, 80 ms, 160 ms, can be taken as a starting point. |  |
| Antenna Configuration at the TRP | (1,1,2) with omni-directional antenna element |  |
| Antenna Configuration at the UE | (1,1,2) with omni-directional antenna element |  |
| Antenna port virtualization | Clarified by each proponent in simulation assumptions (e.g. the beamforming method, beam directions, number of beams) |  |
| Frequency Offset | - Initial acquisition - TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 5, 10, 20 ppm (each company to choose one) - Non-initial acquisition - TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 0.1 ppm |  |
| Number of interfering TRPs(*) | Case 1: (Non-cell free case) - No interferences assumed: mandatory - 2 interfering TRPs (1st SIR = 0dB, 2nd SIR = -3dB; SIR is defined as the ratio of power between a reference cell and interfered cell) – timing arrival differences from TRPs are provided by each proponent: optional Case 2: (Cell free case) - Multiple SS are received from at least 3 TRPs. (0dB, 0dB, 3dB) is assumed as the ratio of power between the strongest signal and the 2nd ,3rd signal. Timing arrival differences from TRPs are provided by each proponent: optional |  |
| For PBCH |  |  |
| Channel coding scheme for PBCH(*) | Polar coding |  |
| Performance Target | 1% BLER @ -6dB (average received SNR) |  |

Proposal 22: Consider Table 6-1 in R1-2506096 as link-level simulation assumption for initial access simulation.

Channel Model

In NR, link level simulation for synchronization signals/channels uses CDL-C channel model. In our views, for link level simulation, it is sufficient to simulate TDL-C channel and the simulation complexity can be simplified.

Proposal 23: TDL-C channel model can also be considered for link level simulation for synchronization signals/channels.

Transmission bandwidth

For NR SSB, PSS/SSS occupies 12 PRB and PBCH occupies 20 PRB, which is equivalent to 2.16 MHz and 3.6 MHz for SCS of 15 kHz, and 4.32 MHz and 7.2 MHz for SCS of 30 kHz. For NR PRACH, the transmission bandwidth corresponds to different preamble format and SCS are summarized as below:

Table 6-2: Transmission bandwidth for NR PRACH

| Format | Sequence length | SCS | Bandwidth |
| --- | --- | --- | --- |
| 0~3 | 839 | 1.25 kHz | 1.25 MHz |
|  | 839 | 5 kHz | 5 MHz |
| A1/2/3, B1/2/3/4, C0/2 | 139 | 15 kHz | 2.5 MHz |
|  | 139 | 30 kHz | 5 MHz |

From the current design, transmission bandwidth of common signals/channels in initial access, especially in case of 30 kHz SCS, the transmission bandwidth can be larger than 5 MHz, and is only feasible for a system bandwidth larger than 5 MHz.

For 6GR initial access, targeting harmonized and forward compatible design for diverse device types including LPWA devices, the transmission bandwidth should be revisited in the evaluation. In our view, transmission bandwidth of 3 MHz (with 15 kHz SCS) and 5 MHz (with 15 kHz and 30 kHz SCS) can be additionally considered.

Proposal 24: Transmission bandwidth of 3 MHz (with 15 kHz SCS) and 5 MHz (with 15 kHz and 30 kHz SCS) should be considered in the link level simulation of synchronization signal, PBCH and RACH.

Search window

In NR, the UE assumes that the default periodicity of initial cell search is 20 ms. Therefore, the search window for PSS correlation, which depends on the SSB periodicity, is 20 ms.

As illustrated in our companion’s contribution regarding initial access [8] and energy efficiency [9], extending the periodicity of SSB is a promising solution for network energy saving. Therefore, for 6GR initial access evaluations, search window longer than 20 ms can be considered. Candidate values such as 40 ms, 80 ms, and 160 ms can be further discussed and investigated. It is noted that with the increase of the SSB periodicity for initial cell search, UE complexity of PSS detection is increased, the evaluation can take into account the UE PSS blind detection complexity.

Proposal 25: Search window for PSS correlation with a duration longer than 20 ms should be considered in the link level simulation. Candidate values such as 40 ms, 80 ms, 160 ms, can be taken as a starting point.

- UE complexity of PSS detection with increased duration of search window should be considered.

Number of interfering TRPs

On top of the assumption regarding number of interfering TRPs in link level simulation for NR initial access. We additionally consider the assumption for cell free deployment.

As discussed in our companion’s contribution [8], cell free deployment is proposed to be studied in 6GR initial access, where all cells/TRPs in a cell free area (CFA) transmit a common synchronization signal in a SFN manner, as illustrated in the following figure.

Figure 2: An example of Cell free deployment

When compared to conventional cell-based deployment, multiple SS from different cells/TRPs are interferences, however, in cell free deployment, as all cells/TRPs in a cell free area can transmit a common synchronization signals/channels in a SFN manner, which transfers interference into interested signals. To evaluate the cell free deployment, the following assumption of number of interfering TRPs are considered:

- Multiple SS are received from at least 3 TRPs. (0dB, 0dB, 3dB) is assumed as the ratio of power between the strongest signal and the 2nd ,3rd signal. Timing arrival differences from TRPs are provided by each proponent: optional

Proposal 26: The following assumption on number of interfering TRPs for cell free case is used:

- Multiple SS are received from at least 3 TRPs. (0dB, 0dB, 3dB) is assumed as the ratio of power between the strongest signal and the 2nd ,3rd signal. Timing arrival differences from TRPs are provided by each proponent: optional

Channel coding for PBCH

Simulation assumptions for NR PBCH uses LTE TBCC as baseline channel coding scheme. The specification eventually supports Polar coding for NR PBCH. Therefore, in 6GR evaluation assumptions, the channel coding for PBCH should use Polar coding as baseline.

Proposal 27: Use Polar coding as the baseline channel coding scheme for 6GR PBCH evaluation.

## 6.2 System level simulation

The system level simulation assumptions are summarized in the following table.

Table 6-3: System level simulation assumptions for synchronization signals/channels

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Scenario | Urban Macro (ISD = 500m) Dense urban (ISD = 200m) |  |
| Carrier Frequency | 4 GHz | 7GHz |
| Channel Model | TR 38.901 |  |
| Subcarrier Spacing(s) | 30 kHz (other values are not precluded and reported by companies) |  |
| UE dropping | According to TR 38.901 |  |
| Search window | The time window to search (correlate) PSS. It depends on the periodicity of SS transmission. Candidate values such as 40 ms, 80 ms, 160 ms, can be taken as a starting point. |  |
| Network Synchronization | TRPs are synchronized, propagation delay difference between TRPs is modelled based on the free space assumption. |  |
| Antenna Configuration at the TRP | (8,1,2) with directional antenna element (HPBW=65, directivity 8dB) |  |
| Antenna Configuration at the UE | (1,1,2) with omni-directional antenna element |  |
| Antenna port virtualization | Clarified by each proponent in simulation assumptions (e.g. the beamforming method, beam directions, number of beams) |  |
| Frequency Offset | - Initial acquisition - TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 5, 10, 20 ppm (each company to choose one) - Non-initial acquisition - TRP: uniform distribution +/- 0.05 ppm - UE: uniform distribution +/- 0.1 ppm |  |
| TRP interferences(*) | Case 1: (Non-cell free case) - Each TRP is deployed as a cell, the interference is calculated according to neighbouring cells Case 2: (Cell free case) - All cells/TRPs in a cell free area (CFA) transmit a common synchronization signal in a SFN manner (see below section) |  |
| Coordination cell for cell free case(*) | The nearest 21 cells for each UE can be taken as a starting point. Other values are not precluded and reported by companies. |  |

TRP interferences

Considering cell free deployment, as discussed in the link level simulation, as all cells/TRPs in a cell free area can transmit a common synchronization signal/channel in a SFN manner, which transfers interference into interested signals, the assumptions on TRP interferences are considered as follows:

- All cells/TRPs in a cell free area transmit a common synchronization signal in a SFN manner.

Proposal 28: The following assumption on TRP interferences for cell free case is used:

- All cells/TRPs in a cell free area transmit a common synchronization signal in a SFN manner.

Coordination cell for cell free case

For system level simulation for cell free deployment, assumptions regarding coordination cell for such deployment should be additionally considered. We propose to coordinate the nearest 21 cells per each UE as a cell free area, and other values can be further discussed or report by each proponent.

Proposal 29: A cell free area coordinating the nearest 21 cells for each UE can be taken as a starting point.

- Other values are not precluded and reported by companies.

Proposal 30: Consider Table 6-3 in R1-2506096 as system-level simulation assumption for initial access simulation.

## 6.3 Performance Metric

In this section, we discuss the performance metric for 6GR initial access.

Link-level simulation performance metrics

At least the following performance metric should be considered, which is same as that defined in NR:

- Signal detection rate

- Misdetection probability

- Residual timing and frequency errors after synchronization

- Other metrics (e.g., PAPR) are not precluded

System-level simulation performance metrics

The following performance metric should be further considered:

- Distribution of acquisition time (i.e., cell search time). From network energy saving perspective, a longer periodicity of SSB can provide significant gain, but it is apparent that such extension will have direct impact on UE cell search time, and therefore it can be further considered as a performance metric.

- Network power consumption for supporting UE initial acquisition and after initial acquisition in IDLE mode.

- UE power consumption after initial acquisition in IDLE mode.

- RACH blocking probability. In case of cell free deployment, potential benefits including reduction of RACH blocking rate can be obtained. To further justify the gain on cell free deployment, RACH blocking rate as a new performance metric is proposed.

- Distribution of SINR. In case of cell free deployment, due to the benefits of SFN transmission of a common synchronization signal over a single wide beam, it is still possible to provide seamless coverage in a cell free area. To justify the gain on cell free deployment, distribution of SINR can be considered as a performance metric.

Proposal 31: The following link level simulation performance metric can be used for 6GR initial access simulation:

- Signal detection rate

- Misdetection probability

- Residual timing and frequency errors after synchronization

- Other metrics (e.g., PAPR) are not precluded

Proposal 32: The following system level simulation performance metric can be used for 6GR initial access simulation:

- Distribution of acquisition time (i.e., cell search time)

- Network power consumption for supporting UE initial acquisition and after initial acquisition in IDLE mode.

- UE power consumption after initial acquisition in IDLE mode.

- RACH blocking probability.

- Distribution of SINR.

- Discussion on the evaluation of sensing

As mentioned in the SID for NR ISAC, evaluation on TRP-mono-statics UAV sensing is now under discussion [10]. Metrics and evaluation methodology are expected to be determined before 6G ISAC studies. Due to possible new services and scenarios in 6G, new metrics and methodologies are required, taking agreement in NR ISAC as a starting point.

Deployment scenarios and sensing modes

Deployment scenarios including detection and tracking of UAV, automotive, human, hazard are supported in TR 38.901 [11]. In our view, evaluation on these scenarios still needs to be discussed with new sensing modes and requirement on performance. Meanwhile, new scenarios considering further integration between sensing and communication should also be included for 6G.

Proposal 33: Support the deployment scenarios including at least the following aspects:

- UAV

- Automotive

- Human

- Hazard

- Environment awareness/reconstruction to assist communication

All sensing modes including monostatic and bistatic with both TRP and UE involved are in the scope of SID for 6G ISAC. Different from TRP monostatic mode in NR studies, performance of bi-statics sensing is sensitive to synchronization on frequency, time, and beam, especially with UE involved. From our perspective, issues of synchronization should be fully considered for 6G ISAC evaluation.

Proposal 34: Issues of synchronization on frequency, time, and beam should be fully considered in evaluation of bi-static sensing.

Metrics and evaluation methodology

Metrics defined in NR studies with corresponding evaluation methodologies can be reused for detection purposes in 6G ISAC evaluation. Based on our observation, metrics and requirements of tracking differ from that of detection in many aspects. According to deployment requirement of tracking, partial interruptions of the trajectory resulting from false alarms are acceptable, while continuous interruptions of the trajectory caused by false alarms are unacceptable even with a lower false alarm probability. And from aspect of deployment, sensing capacity, which describes the maximum number of the targets detected or tracked within a serving zone, is vital. Sensing performance degrades with increasing number of targets.

Proposal 35: RAN1 can use metrics and methodologies in NR as a starting point for detection requirement in 6G ISAC evaluation.

Proposal 36: RAN1 should study new metrics and evaluation methodologies according to different scenarios and requirements.

For scenario of assisting communication, existing metrics for communication can be used as a starting point.

Proposal 37: RAN1 can use existing metrics for communication as a starting point for scenario of assisting communication.

Channel model

Existing ISAC channel model in TR 38.901 [11] is established for detection requirement. Several additional features developed for different requirements of communication are reused. The compatibility among these additional features needs to be re-examined especially for new sensing scenarios and requirements. Current statistical channel model for background is not enough for requirements including adaptive clutter suppression and environment awareness. At least spatial consistency in background channel should be studied considering both Tx and Rx movements. If all these additional features for sensing lead to huge increment in simulation complexity, ray-tracing-based channel modelling can also be considered.

Proposal 38: RAN1 should extend NR ISAC channel model for new scenarios and requirements in 6G.

Proposal 39: Compatibility among additional features of channel model for 6G ISAC should be fully considered.

- Conclusions

In this contribution, we discuss evaluation assumptions for 6GR air interface, and the following proposals are made:

For deployment scenarios:

Proposal 1: For simulation assumptions, RAN1 to take Table 1-1 / 1-2 / 1-3 / 1-4 in R1-2506096 as deployment scenarios for Indoor hotspot (InH), Dense Urban, Urban macro, and Rural respectively.

Proposal 2: For simulation assumptions, RAN1 to take Table 1-5 in R1-2506096 as deployment scenarios for High speed scenario.

For evaluation of 6G coverage:

Proposal 3: Compared with 5G, similar or better coverage performance can be considered for 6G design.

Proposal 4: Both urban macro and rural scenarios can be considered for coverage performance evaluation. And urban macro scenario has higher priority.

Proposal 5：Both requirements of wideband traffic with high data rate and VoIP, LPWA traffic should be considered for the coverage performance evaluations.

Proposal 6: 7GHz can be considered as main operation frequency for the coverage evaluations.

Proposal 7: Both uplink and downlink channels in the idle mode and connected mode should be considered for the coverage evaluations.

Proposal 8: Link level simulation can be carried out to evaluate the SINR requirements for the specific channel and traffic. Link budget template from Rel-16 CE and IMT-2020 self-evaluations can be considered as a starting point.

Proposal 9: A unified coverage target can be considered for all the channels and signals. Define a minimum MCL target for 6G coverage enhancement for around 7GHz considering: penetration loss, antenna gain. The MCL target aims to re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. For traffic channels, at least 1Mbps data rate at cell edge for 6G should be considered as the coverage targets.

For evaluation of NTN:

Proposal 10: Support the deployment scenarios including at least the following aspects.

- GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO).

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- Transparent/regenerative satellite payload with single/multiple beams

Proposal 11: For the evaluation of NTN scenarios, the following target services (i.e., VoIP and low-data rate services for commercial handset terminals) and the target data rate assumptions can be considered.

- For VoIP, AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval is used in the evaluations.

- For low-data rate service, both 3 kbps and 1Mbps can be considered as DL date rate, 3 kbps and 100 kbps can be considered for UL.

Proposal 12: Link-level DL coverage performance of NTN can be evaluated based on the following steps.

- Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821.

- Step 2: Required SNR of target service is evaluated by LLS.

- Step 3: The CNR and the required SNR are compared to identify the coverage gap

Proposal 13: For coverage performance evaluation, the following are assumed for all channels/signals:

- Channel model/Delay spread: NTN-TDL-A (NLOS) and NTN-TDL-C (LOS) defined in TR 38.811

Proposal 14: For 6G NTN evaluation, the following channels/signals can be evaluated as a baseline.

- Synchronization signal

- Broadcast signals/channels

- Initial access channels/signals

- Other UL/DL channels/signals

For evaluation assumptions of 6G power model:

Proposal 15: 6GR power evaluation can take followings as start point and further consider their amendments.

- TR 38.864 as start point for network power model

- TR 38.840 as start point for UE power model for smart phone

- TR 38.865 as start point for LPWA only UE power model as supplementary to the TR38.840 power model

- TR 38.869 as start point for LP-WUR power model

Proposal 16: For both NW and UE power model for 6GR, consider the following tables as the starting point and further discuss the specific value:

- Note that the setting of parameter may closely related to the system design. For instance, the transition time between different sleep modes may strongly impact the periodicity of common signal/channel.

Table 5-1: Power states and relative power values of BS or UE power consumption model

| Power state | Characteristic | FR1 TDD | FR1 FDD | FR2 TDD |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. |  |  |  |
| Deep sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. |  |  |  |
| Light sleep | There is neither DL transmission nor UL reception. Time interval for the sleep should be larger than the total transition time entering and leaving this state. |  |  |  |
| Micro sleep | There is neither DL transmission nor UL reception. Immediate transition is assumed for energy saving study purpose from or to a non-sleep state. |  |  |  |
| Active DL | There is only DL transmission. |  |  |  |
| Active UL | There is only UL reception. |  |  |  |

Table 5-2: Total transition time and additional transition energy of BS or UE power consumption model

| Power state | Total transition time | Additional transition energy |
| --- | --- | --- |
| Ultra-deep sleep |  |  |
| Deep sleep |  |  |
| Light sleep |  |  |

Proposal 17: For NW power model in 6GR, consider the scaling method in TR 38.864 as the starting point.

Proposal 18: For UE power model in 6GR, in order to scale among multiple aspects, consider the following scaling method as an example for further discussion:

- for DL:

  - PDL=PstaticDL+PdynamicDL

  - PstaticDL is a static part of power for UE in active, which is not scaled based on reference configurations.

  - PdynamicDL=sa*Pdyn,anteDL is a dynamic part of power for UE in active, which is scaled based on reference configuration, and sa is the fraction of active TxRUs.

- For UL:

  - PUL=PstaticUL+PdynamicUL

  - PstaticUL is a static part of power for UE in active, which is not scaled based on reference configurations.

  - PdynamicUL=sa*sf*spηsf,  sp*Pdyn,  PA is a dynamic part of power for UE in active, where sa, sf, sp is the fraction of active TxRUs, the ratio between the RF bandwidth and the maximum system BW, and the ratio of PSD per TxRU between the UL transmission and reference configuration, respectively, and ηsf,  sp is the PA efficiency (PAE) related to sf, sp.

Proposal 19: RAN1 to study power model for NW and UE by taken PA Efficiency into account.

Proposal 20: Power consumption evaluation should consider both Tx and Rx at the same time.

For evaluation assumptions for initial access:

Proposal 21: The link level and system level simulation methodology and assumptions for 6GR initial access should reuse that defined for NR initial access, as captured in TR 38.802 Table A.1.5-1, Table A.1.5-2, Table A.1.5-3, and Table A.2.3-1, as a starting point.

Proposal 22: Consider Table 6-1 in R1-2506096 as link-level simulation assumption for initial access simulation.

Proposal 23: TDL-C channel model can also be considered for link level simulation for synchronization signals/channels.

Proposal 24: Transmission bandwidth of 3 MHz (with 15 kHz SCS) and 5 MHz (with 15 kHz and 30 kHz SCS) should be considered in the link level simulation of synchronization signal, PBCH and RACH.

Proposal 25: Search window for PSS correlation with a duration longer than 20 ms should be considered in the link level simulation. Candidate values such as 40 ms, 80 ms, 160 ms, can be taken as a starting point.

- UE complexity of PSS detection with increased duration of search window should be considered.

Proposal 26: The following assumption on number of interfering TRPs for cell free case is used:

- Multiple SS are received from at least 3 TRPs. (0dB, 0dB, 3dB) is assumed as the ratio of power between the strongest signal and the 2nd ,3rd signal. Timing arrival differences from TRPs are provided by each proponent: optional

Proposal 27: Use Polar coding as the baseline channel coding scheme for 6GR PBCH evaluation.

Proposal 28: The following assumption on TRP interferences for cell free case is used:

- All cells/TRPs in a cell free area transmit a common synchronization signal in a SFN manner.

Proposal 29: A cell free area coordinating the nearest 21 cells for each UE can be taken as a starting point.

- Other values are not precluded and reported by companies.

Proposal 30: Consider Table 6-3 in R1-2506096 as system-level simulation assumption for initial access simulation.

Proposal 31: The following link level simulation performance metric can be used for 6GR initial access simulation:

- Signal detection rate

- Misdetection probability

- Residual timing and frequency errors after synchronization

- Other metrics (e.g., PAPR) are not precluded

Proposal 32: The following system level simulation performance metric can be used for 6GR initial access simulation:

- Distribution of acquisition time (i.e., cell search time)

- Network power consumption for supporting UE initial acquisition and after initial acquisition in IDLE mode.

- UE power consumption after initial acquisition in IDLE mode.

- RACH blocking probability.

- Distribution of SINR.

For evaluation of sensing:

Proposal 33: Support the deployment scenarios including at least the following aspects:

- UAV

- Automotive

- Human

- Hazard

- Environment awareness/reconstruction to assist communication

Proposal 34: Issues of synchronization on frequency, time, and beam should be fully considered in evaluation of bi-static sensing.

Proposal 35: RAN1 can use metrics and methodologies in NR as a starting point for detection requirement in 6G ISAC evaluation.

Proposal 36: RAN1 should study new metrics and evaluation methodologies according to different scenarios and requirements.

Proposal 37: RAN1 can use existing metrics for communication as a starting point for scenario of assisting communication.

Proposal 38: RAN1 should extend NR ISAC channel model for new scenarios and requirements in 6G.

Proposal 39: Compatibility among additional features of channel model for 6G ISAC should be fully considered.

- References

- RP-251881, Moderator (NTT DOCOMO), New SID: Study on 6G Radio

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements

- 3GPP TR 38.830, Study on NR coverage enhancements

- 3GPP TR 38.864, Study on network energy savings for NR

- 3GPP TR 38.840, Study on User Equipment (UE) power saving in NR.

- 3GPP TR 38.869, Study on low-power wake up signal and receiver for NR

- 3GPP TR 38.865, Study on further NR RedCap UE complexity reduction

- R1-2506095, CMCC, Overview of 6GR air interface

- R1-2506101, CMCC, Discussion on Energy Efficiency for 6GR interface

- RP-251861, Moderator (AT&T), New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz