3GPP TSG RAN WG1 #122	             			            		                                            R1-2506233

Bengaluru, India, Aug 25th –29th, 2025

Agenda Item:	11.6

Source:	AT&T

Title:	Views on AI/ML in 6GR

Document for:	Discussion/Decision

# Introduction

The study on 6G Radio was agreed in RAN#108 in RP-251881 [1]. One of the main objectives of the study relates to AI/ML for 6GR:

AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… 
Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution we discuss the requirements and the core design principles for enabling a native integrated AI/ML framework in 6G. We then discuss how having such a standardized framework help reduce the barrier to entry for AI/ML use cases on the air interface, we finally present our views on selection of AI/ML use cases for 6GR air interface.

# The need for an Integrated and Native AI/ML for 6G

In an AI native 6G, AI models are expected to be embedded in different layers of the network architecture across different domains at the UE/gNB/OAM/CN. Models are deployed simultaneously across layers of the protocol stack, to optimize network performance. Cross layer AI/ML models are possible where inputs from one or multiple layers are used to train another model, and the AI/ML model output inference can be used by one or multiple layers.

Observation 1: An AI native 6G allows simultaneous deployment of multiple models across layers and across domains in 6GR.

To enable an AI/ML native 6G, collaboration and interoperability between the network vendor and different user equipment vendors or among  different vendors in the network infrastructure is an important aspect that needs to be addressed to ensure that the designed AI/ML model can perform well and be supported across different vendors.

Observation 2: Interoperability is needed for AI native 6G

Similarly, the network operator should be able to manage and control the different data collection aspects such as when to collect data, how to ensure privacy and security of the collected data and how to store and share the collected data (with the relevant parties through an SLA).

Observation 3: Network visibility and control over different data collection aspects is needed for AI/ML models in 6GR.

The native AI/ML framework should further be flexible to allow for quick updates of AI/ML model and to enable new features based on the current requirements by the network. Finally, the native AI/ML framework should be flexible and scalable to allow for the AI/ML use cases to be quickly developed and deployed with limited framework enhancements, and if possible, to also have backward compatibility with the 5G framework.

Observation 4: Native AI/ML framework design is flexible and scalable to continuously evolve and deploy AI/ML in 6GR.

It is thus natural to think of an integrated scalable framework for data management for the different use cases rather than taking a per-use case approach to data management and lifecycle management, which often results in a bulky non flexible design, and might lead to fragmentation or segmentation in the data collection across different use cases. An integrated framework for data collection, data transfer and delivery, and performance monitoring improves network efficiency, interoperability, vendor uniformity, and scalability, while addressing critical security and privacy concerns.

## Comparison with 5G AI/ML Framework

AI/ML use cases and their related lifecycle management and corresponding data collection framework were not considered as part of the day 1 5G design.  As AI/ML models and use cases emerge at the UE and/ or the network side, their specification in 5G-Advanced is being retrofitted into the 5G architecture and design. The approach taken to standardization of AI/ML in the air interface in 5G-Advanced is very similar to that taken traditionally in 3GPP, to specify, enable, test and potentially deploy features based on specific use cases. The AI/ML field, however, is constantly evolving and having a “one and done” approach is not amenable to continuous innovation and disruption.

The 5G standardization effort for AI/ML use cases on the air interface focuses on the existing 5G radio design and allows for only limited enhancements to it to support AI/ML use cases. This results in a design which is not supportive of any native AI/ML methodology as we will need new enhancements whenever there is a new AI/ML use case. The AI/ML traffic consists of all data related to the AI/ML activities in the network and is not generated or terminated at any end user application.

Observation 5: The 5G framework for AI/ML on the air interface does not allow for native AI/ML integration in 6GR.

In the 5G framework, each use case separately discusses the enhancements to the interfaces to support AI/ML data transfer. This approach is not scalable as for every new use case, the discussion is restarted on how to transfer AI/ML related data across different termination points. This makes it difficult to design a native architecture as it is exceedingly difficult to get performance metrics for models at different layers and design them in a manner such that they do not impact each other’s performance.

Observation 6: The 5G framework for AI/ML is not scalable for AI/ML data transfer

Another issue with the existing framework is the limitation regarding the LCM operations and monitoring metrics that can be done on the control plane. Due to the limited bandwidth of the CP only short monitoring metrics can be reported to the user. Furthermore, using CP for AI/ML LCM is also not very scalable in case of frequent configuration/functionality activation/deactivation/switching/fallback as the load on the CP will increase with number of use cases.

Observation 7: The 5G framework for AI/ML limits LCM operations and metrics used for monitoring on the control plane

## Integrated Framework Principles and Design Requirements

An integrated AI/ML framework enables MNOs to manage network capacity effectively, fostering innovation, and unlocking new value opportunities. A unified approach to training and deploying AI/ML models supports continuous innovation in model development, ensuring the network evolves dynamically. Furthermore, a uniform life cycle management (LCM) framework is crucial for running multiple models simultaneously across different layers, meeting diverse service requirements end-to-end.

To realize this vision, the following core principles should be used for AI/ML framework design in 6GR:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

Proposal 1: The following core principles are followed to design an AI/ML framework for 6GR air interface:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

To support these core principles, a universal framework to avoid fragmentation and per-use case specification and ensure scalability to new AI/ML use cases is needed. For this framework, multiple termination points for AI/ML data within the network should be supported, with MNO visibility. In addition, 6GR should be designed to differentiate AI/ML data management traffic from user plane traffic and control plane traffic.

Proposal 2: AI/ML framework in 6GR should support multiple termination points for AI/ML data within the network with MNO visibility

Proposal 3: 6GR is designed to differentiate AI/ML data management traffic from user plane traffic and control plane traffic

In the 5G architecture the control plane (CP) is responsible for signaling and control functions to establish, maintain, and terminate communication sessions between the user and the network, while the user plane (UP) is responsible for all the user data traffic. As the CP contains only the control related information to maintain and manage the communication link between the user and the network, CP by design has a limited amount of bandwidth, low latency requirements and any data transfer on CP is not charged to the users. The UP, however, contains all the data traffic requested by the user and user applications, therefore it has high bandwidth, variable latency/QoS requirements (based on user application) and the users are charged for the data usage on the UP.

The AI/ML models are typically trained by collecting a large real-world dataset and are then transferred and deployed to the users. The size of training dataset can be in range of 100k to 100M data sample, depending on various aspects such as use case, type of model, generalization aspects etc. Similarly, the model size can vary from 10k to 100M parameter model depending on the use case, model type and other aspects. Due to the large amount of data involved, it is important that the network has full visibility and control when the data is shared, to avoid any network impact. Additionally, the users should not be charged for the AI/ML model training data or model transfer.

Having a CP based solution does not work due to the large amount of data being involved and an UP based solution will not give any visibility to the network and can impact its performance.

There are 3 potential approaches that can be used to design data collection and data management for the native AI/ML in 6G

- Continue with the current framework and have a hybrid of CP and UP

- CP overhaul and redesign

- A new plane: AMP (AI/ML plane) in parallel with the current CP and UP

The first option is not well suited for a native AI/ML approach and borrows from the 5G framework. A hybrid approach using CP and UP requires significant specification impact and coordination across multiple work groups across RAN and CN. It also increases the signaling load on the CP due to reporting of when and what data is being shared. The increase in signaling load also makes such an approach not scalable with the number of use cases. In addition, it is very difficult to optimize data collection for multiple use cases in case some of the input data is common among the use cases.

The second option would require a significant specification impact and would result in the 6G framework to be entirely non compatible with the legacy 5G framework. Such a design can allow for several of the core issues with the AI/ML framework to be properly addressed. However, there will still be an issue of increased load on the CP with the number of use cases. In addition, unless we allow the new CP to have higher or adaptable bandwidth there will still be an issue with configuring resource utilization over the new CP, and with unnecessarily using the CP for latency tolerant data transfer/delivery.

The third option is to have an entirely new plane which we can label AMP for all AI/ML related traffic. The AMP is responsible for the AI/ML related data traffic for the data collection and model transfer. We can also utilize AMP for transfer of AI/ML model monitoring metrics between UE side and network side. Furthermore, the AMP can be designed to manage the AI/ML model LCM aspects such as AI/ML model/functionality activation/deactivation /switching/fallback. This will allow all AI/ML related aspects to be managed from within the new plane AMP, which enables us to design new use cases quickly and efficiently for native AI/ML.

The new AMP, by design, combines benefits of both CP and UP solutions when needed as all the traffic in the AMP is intended to be originated and terminated at UE device or at the different entities in the network like the CP traffic. However, the traffic characteristics of the AMP with regards to model transfer/delivery and data collection requirements are like those of the UP traffic. Like the CP traffic, traffic at AMP will not be charged to the user.

The AMP will allow for much better data management as the network can collect data for multiple use cases simultaneously improving efficiency (as there may be some common inputs for different AI/ML models operating at different layers or even at different domains/network functions). Furthermore, the network can add relevant additional information and labels to the data. The network can also ensure that the privacy is preserved, and proprietary and security aspects of the data are properly addressed before the data is shared with the network or UE vendors to develop UE models. Additionally, it gives network control over when to collect data or transfer models so that there is minimal impact on system performance. Finally, the AMP architecture provides us with a good framework that can be used to quickly develop and deploy AI/ML use cases based on the network requirements, reducing the standardization effort to onboard new use cases based on the established framework.

Proposal 4: for the AI/ML framework in 6GR, study the introduction of an AI/ML data management plane to manage data collection, model transfer/delivery and LCM aspects of different AI/ML use cases on 6GR interface

# 6GR Interface Use Cases

Having an integrated standardized framework for AI/ML data management and a unified framework for LCM allows for a scalable and flexible introduction of AI/ML models and use cases in the 6GR air interface. The integrated framework reduces the overhead of duplicate data collection for various air interface use cases and optimizes network capacity. Such an integrated framework further naturally allows to aggregate data across use cases when needed.

An example of this can be seen in the 5G-A use cases considered so far in the 5G specifications. All the use cases use channel state information, estimation and resources, be it CSI prediction, beam management, mobility or CSI compression. An integrated framework for data collection for training, monitoring, and data transfer, and network assistance information, when needed, streamlines the AI/ML data management process, and allows for a seamless integration and maintenance of these models in the network.

For 6GR, within the integrated framework for data management, it is important to consider use cases that are of practical relevance and can resolve problems that are of high impact to the network operators. AI/ML models should generally be used to resolve complex problems with high dimensionality for which a low complexity model-based solution is unknown or inadequate.

Proposal 5: For 6GR design, consider use cases that provide high impact practical relevance to the operators

6GR AI/ML models need to be evaluated in terms of benefits versus complexity and compute tradeoffs, as well as overall integration across the network architecture for an AI/ML native 6G. Performance evaluation needs to consider final performance metrics such as throughput for example, for communication performance improvements rather than intermediate metrics such as SGCS which was often used in 5G advanced to assess the benefits of AI/ML models.

Proposal 6: For 6GR design, final performance metrics (e.g. throughput) are used for performance evaluation of AI/ML use cases

Proposal 7: For 6GR design, consider complexity and performance tradeoffs for evaluating AI/ML use cases

Use cases that address improved RAN efficiency should be prioritized for selection for 6GR design. In addition to spectral efficiency, energy efficiency can also be considered.

One of the main pain points for example from the operators right now, as we move into extremely large massive MIMO, is improving the penetration of multi-user MIMO in the field deployments. This relates to scheduling, traffic considerations but also to availability of reliable channel knowledge from the users at the gNBs. The lack of dependable SRS for channel reciprocity remains a significant challenge in current deployments. Additionally, employing Type II CSI involves considerable costs and creates substantial overhead.

Use cases that bring improvements on channel estimation on the DL, either via SRS enhancements or CSI including but not limited to SRS-less DL enhancements, CSI-RS overhead reduction, receiver enhancements, traffic and channel prediction, are of high value and should be considered with high priority.

Proposal 8: for 6GR AI/ML, consider use cases related to SRS enhancements, CSI-RS overhead reduction, receiver enhancements, traffic and channel prediction with high priority

Another important practical enhancement to the network lies in the enhancements on the UL coverage and UL capacity. This is crucial as we witness increased traffic on the uplink driven by new services such as AI/ML and XR.  Such use cases can include AI/ML based receiver enhancements, uplink buffer prediction.

Proposal 9: For 6GR AI/ML, consider use cases related to UL enhancements such as AI/ML based receiver enhancements, and uplink buffer prediction with high priority

# Conclusion

In this contribution, we presented our views on AI native 6GR and we discussed an integrated framework for data management and LCM. We also presented our views on AI/ML use case selection for 6GR. We made the following observations and proposals.

Observation 1: An AI native 6G allows simultaneous deployment of multiple models across layers and across domains in 6GR.

Observation 2: Interoperability is needed for AI native 6G.

Observation 3: Network visibility and control over different data collection aspects is needed for AI/ML models in 6GR.

Observation 4: Native AI/ML framework design is flexible and scalable to continuously evolve and deploy AI/ML in 6GR.

Observation 5: The 5G framework for AI/ML on the air interface does not allow for native AI/ML integration in 6GR.

Observation 6: The 5G framework for AI/ML is not scalable for AI/ML data transfer

Observation 7: The 5G framework for AI/ML limits LCM operations and metrics used for monitoring on the control plane

Proposal 1: The following core principles are followed to design an AI/ML framework for 6GR air interface:

- A unified flexible LCM framework for model management, model transfer, model training, and model testing

- A unified data collection framework to enhance management efficiency

- Network visibility to drive innovation while proactively addressing security and privacy concerns

- Network control over data collection to ensure network performance is not impacted while providing potential new value opportunities via hosting/routing/augmenting the data

- Scalability to accommodate various emerging and future use cases.

Proposal 2: AI/ML framework in 6GR should support multiple termination points for AI/ML data within the network with MNO visibility

Proposal 3: 6GR is designed to differentiate AI/ML data management traffic from user plane traffic and control plane traffic

Proposal 4: for the AI/ML framework in 6GR, study the introduction of an AI/ML data management plane to manage data collection, model transfer/delivery and LCM aspects of different AI/ML use cases on 6GR interface

Proposal 5: For 6GR design, consider use cases that provide high impact practical relevance to the operators

Proposal 6: For 6GR design, final performance metrics (e.g. throughput) are used for performance evaluation of AI/ML use cases

Proposal 7: For 6GR design, consider complexity and performance tradeoffs for evaluating AI/ML use cases

Proposal 8: for 6GR AI/ML, consider use cases related to SRS enhancements, CSI-RS overhead reduction, receiver enhancements, traffic and channel prediction with high priority

Proposal 9: For 6GR AI/ML, consider use cases related to UL enhancements such as AI/ML based receiver enhancements, and uplink buffer prediction with high priority

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT Docomo, Prague Czech Republic