3GPP TSG RAN WG1 #122		                                                       	R1-2506284

Bengaluru, India, August 25th – 29th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for AI/ML for NR Air Interface

Agenda Item:	9.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#107 meeting, WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved with the objective as follows [1]:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above three use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. NOTE: offline training is assumed for the purpose of this project. NOTE: Coordination with SA/SA WGs of the ongoing work as it may relate to their required work.

In this contribution, we present our view on UE features for AI/ML for NR Air Interface.

- Discussion on FGs for all CSI-related use cases

At the RAN1#121 meeting, FG 58-0-1 is introduced for all CSI-related AI/ML use cases based on the agreements about the APU occupation. At the RAN #108 meeting, the following were agreed to allow UE reporting up to 2 resource pools for UE inference with AI/ML.

A UE can report support for N (up to 2) AI/ML PU pools for AI/ML features - For each of the N AI/ML PU pools, UE reports the maximum number of simultaneous AI/ML PUs, respectively. - If N = 2, for each AI/ML use case feature, UE reports which AI/ML PU pool it belongs to.

To accommodate this agreement, FG 58-0-1 should be updated to allow report the number of APU pools and the maximum APU resources for each pool. Since for 5GA, the APU pools are only for CSI-related use cases, the prerequisite of FG 58-0-1 can be FG 2-32, the basic CSI feedback.

For CSI-related use cases, UE will also report the APU occupation for the CSI report configuration that requires UE inference, e.g., the following for AI/ML beam management.

Agreement For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, - For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied

When both maximum number of APU and APU occupation per configuration are reported by UE, there will be no common understanding about the meaning of one unit for APU between the NW and UE. To alleviate this issue, we suggest limiting the range of the maximum number of APU and candidate values for APU occupation reporting, with which the NW can have some understanding about the AI/ML capability of a UE.

Proposal 1: Update FG 58-0-1 as follows.

- Add number of APU pools and update the corresponding parts for the reporting.

- The prerequisite is FG 2-32.

- The range of the maximum number of APUs for each APU pool is [0, 8].

| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Number of APU pools. 2. Maximum number of APUs for each APU pool for all types of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APUs for each APU pool for all types of UE-sided inference for CSI report(s) simultaneously across all CCs | 2-32 | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | Per UE | No | No |  | Component 1 candidate values: {0, 1, 2} Component 2 candidate values: [0, 8] Component 3 candidate values: [0, 8] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Discussion on FGs for AI/ML in beam management

  - NW side beam prediction

At the previous RAN1 meetings, agreements were made about FG for increasing the number of beams in L1-RSRP reporting. For this feature, there is no strong motivation to introduce finer granularity, such as per BC/FC/FCPC. Unless any justification is brought, per UE (or per band) should be sufficient. Also, the prerequisite of this FG can be FG 2-24.

Proposal 2: Update FG 58-1-1 as follows.

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1.Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | 2-24 | yes | n/a | Increased number of reported beams for beam management is not supported | Per UE | No | No |  | Component 3 candidate values: {6, 8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - UE side data collection

At the previous RAN1 meetings, it was agreed to introduce FG for data collection for UE-sided beam prediction. As this feature is basically the measurement of RSRP, the prerequisite FG can be FG 2-24. Also, this feature is not related to CA, so the type should be per UE (or per band).  We don’t think it is necessary to highlight the usage of the collected data since it is up to UE or UE-side implementation. Therefore, the “for BM case 1” parts can be removed.

Proposal 3: Update FG 58-1-7 as follows.

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of data collection for UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] [6: Supported maximum number of resources for Set B] [7: Supported maximum number of resources for Set A] 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | 2-24 | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | Per UE | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Inference result reporting

At the previous RAN1 meetings, it was agreed to introduce separate FGs for only beam indication and/or predicted RSRP for BM-case1.  If the predicted RSRP is available, the top-K predicted beams can be obtained from the predicted RSRP. Hence, the FG for reporting only beam indication can be the prerequisite of the FG for reporting predicted RSRP.

According to the AI/ML model deployed at UE, the applicable combination of the number of Set A and the number of Set B is different. For example, some UE is capable of beam prediction “from 4 Set B beams to 16 beams” and “from 8 Set B beams to 64 beams”, while the prediction “from 4 Set B beams to 64 beams” cannot be performed. Taking it into consideration, the combination of the maximum number of Set A and the minimum number of Set B should be reported as component in these FG.

Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

In the previous RAN1 meeting, it was agreed that the maximum number of configured/activated/triggered CSI reporting for UE side model is up to UE capability, as above. In the legacy CSI reporting, the number of processed activated CSI reporting is restricted by the maximum CSI processing units UE supports. This principle can be reused in AI/ML too. Also, the number of activated AI/ML-based CSI reporting can be covered by 58-0-1 regarding maximum number of APUs. Hence, there is no need for UE to report the maximum number of activated/triggered CSI reporting even for AI/ML-based CSI reporting.

Another issue is the APU occupation for the beam prediction, a new component should be introduced to reflect the following agreement from RAN1#121.

Agreement For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, - For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied

It is noted in the agreements that detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature. For APU (i.e., CPU_2 in current specification) occupations, if a UE can report arbitrary number, there will be no consensus on the understanding of the APU between NW and UE. The maximum APU resources reported by UE is meaningless then. The NW cannot manage the configurations for UEs to fully utilize the AI/ML capability of UEs in the network. To avoid this and meanwhile to reflect the different UE implementations with different model size, we suggest the restrictions on the candidate values of O_APU. For example, N_small and N_large besides the value 0 for UE using small- or large-scale models. For CPU occupation, the number of candidate values should also be restricted. Besides O_CPU=0, O_CPU can be 1 when both CPU and APU are occupied and the AI/ML models run on the APU; and O_CPU can be a large number, e.g., 8, for CPU based prediction.

Also, this feature is not related to CA, so the type should be per UE (or per band).

Proposal 4: Update FG 58-1-2 and 58-1-3 as follows.

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per PWB] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported of CSI-RS resource types for Set A: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instance 13. A list of supported combinations, each combination is {minimum required number of SSB/CSI-RS resources to be measured, maximum number of SSB/CSI-RS resources to be predicted} 14. Index about which APU resource pool is CPU_2. 15. Value of CPU_1 occupation. 16. Value of CPU_2 occupation. | 2-24, 58-0-1 | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference]is not supported | Per UE | No | No |  | Component 12 Candidate value set: {1, 2, 3, 4} Candidate value of the minimum required number of SSB/CSI-RS resources to be measured in Component 13: {4, 8, 16, 32} Candidate value of the maximum number of SSB/CSI-RS resources to be predicted in Component 13: {16, 32, 64, 128} Candidate values for component 14: {1,2} Candidate values for component 15: {0, 1, 8} Candidate values for component 16: {0, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] 2. Supported maximum number of predicted beams with predicted RSRP in each reporting instance | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference]is not supported | Per UE | No | No |  | Candidate values for Component 2 candidate values: {1, 2, 3, 4} FFS: CPU/AIMLPU related information | Optional with capability signalling |

At the previous RAN1 meetings, it was agreed to introduce separate FGs for only beam indication and/or predicted RSRP for BM-case2. BM-Case 2 can be categorized into two types: “only temporal beam prediction” and “spatial domain and temporal beam prediction”. As the implementation difficulties are different depending on these types, it is better to differentiate. However, this differentiation can be reported via the combination of minimum required Set B and maximum Set A. Hence, no dedicated component is necessary for this differentiation.

In the temporal beam prediction, the prediction is based on multiple measurement occasions of Set B. According to the UE capability, the required number of measurement occasions could be different. For example, if UE has advanced capability, only one measurement occasion may be sufficient. On the other hand, if UE does not have the advanced capability, UE may need several measurement occasions to predict future beam quality. Hence, the required number of measurement occasions of Set B should be reported by UE.

Also, in the same manner as BM-Case1, the following design should be considered for BM-Case 2 too.

- the combination of the number of Set A and number of Set B should be reported as component

- the type should be per UE (or per Band)

- Add the components about which APU is CPU_2 and CPU_1/CPU_2 occupations. Restrict the number of candidate values for CPU_1 and CPU_2 occupation, e.g., only 3 candidates {0, 1, 8} for CPU_1 and {0, 2, 4} for CPU_2.

Proposal 5: Update FG 58-1-4 and 58-1-5 as follows.

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported CSI-RS resource types for Set [A/B]: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s)] 21. A list of supported combinations, each combination is {minimum required number of SSB/CSI-RS resources to be measured, maximum number of SSB/CSI-RS resources to be predicted} 22. Required minimum number of measurement occasions of Set B 23. Index about which APU resource pool is CPU_2. 24. Value of CPU_1 occupation. 25. Value of CPU_2 occupation. | 2-24, 58-0-1 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference]is not supported | Per UE | No | No |  | Component 11 Candidate value set: {1, 2, 3, 4} Component 12 candidate value set: {1, 2, 4, 8} Candidate value of the minimum required number of SSB/CSI-RS resources to be measured in Component 21: {4, 8, 16, 32} Candidate value of the maximum number of SSB/CSI-RS resources to be predicted in Component 21: {16, 32, 64, 128} Component 22 Candidate value set: {1, 2, 4} Candidate values for Component 23: {1,2} Candidate values for component 24: {0, 1, 8} Candidate values for component 25: {0, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference]is not supported | Per UE | No | No |  | FFS: CPU/AIMLPU related information | Optional with capability signalling |

  - Performance monitoring

At RAN1#117 meetings, supporting UE assisted performance monitoring was agreed. More details for this feature were agreed in the subsequent meetings.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues UE measures the dedicated resource set(s) for monitoring. Agreement For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as: - At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling) - Where K is the number of predicted beam(s) in the corresponding inference report per time instance - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring - M is configured by NW in CSI report configuration for monitoring - M= 1, 2 - FFS: detailed rule or signalling Agreement For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model: - Support the size of a set for monitoring is the same as the size of Set A, - The n-th resource in the set for monitoring is linked to the n-th resource in Set A. - Support the size of a set for monitoring is smaller than the size of Set A Working Assumption At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring, - for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report. Working Assumption For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring - wherein N (N>=1) is configured in CSI-ReportConfig - FFS on additional rule for counting N linked pair For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring. Working Assumption For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance FFS on other options.

Performance monitoring can be categorized into NW side performance monitoring and UE assisted performance monitoring.

The feature used for NW side performance monitoring is identical to the one for NW side beam prediction. Hence, the UE capability discussion for performance monitoring can focus on only UE assisted performance monitoring. For UE assisted performance monitoring, a new FG should be introduced. Based on the above agreement for this feature, the following FG should be introduced for UE assisted performance monitoring.

Proposal 6: Introduce the following FG for UE assisted performance monitoring.

| 58. NR_AIML_air | 58-1-6 | UE-assisted performance monitoring for UE-sided beam prediction | 1. Support of UE-assisted performance monitoring 2. Supported BM-Case(s) 3. Maximum value N for monitoring window (number of monitoring instances) 4. Support of M =1, 2 5. Maximum value of minimal slot offset X | 58-1-2, 58-1-4 | yes | n/a | UE-assisted performance monitoring for UE-sided beam prediction is not supported | Per UE | No | No |  | Component 2 candidate value set: {BM-Case1, BM-Case2} Component 3 Candidate value set: {1, 2, 4, 8, 16} Component 5 Candidate value set: {1, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Discussion on FGs for AI/ML for positioning accuracy enhancement

  - FG58-2-1: UE-based positioning Case 1 [for inference]

FG 58-2-1 was captured as a basic capability for AI/ML based positioning of case 1 in RAN1#120bis meeting. Our views are shown below.

- FG name

  - In our view, if FG 58-2-1 can also be considered as a basic capability of UE-based positioning Case 1, inference operation is an essential LCM procedure for UE-based positioning Case 1. In terms of that, “for inference” can be added and the bracket can be removed for a clarification for further discussion. However, some companies think other LCM procedures (e.g., model monitoring) are highly related to model inference, and a common FG should be considered. If this FG includes other LCM procedures, [for inference] is not necessary.

- Prerequisite feature groups

  - FG 13-3 series, which can be considered as basic capabilities for legacy DL-TDOA of L1 UE features, have FG 13-1 as prerequisite feature groups. If FG 58-2-1 can also be considered as a basic capability of UE-based positioning Case 1, FG 13-1 can be considered as prerequisite feature groups.

- Type

  - The support of Case 1 positioning is not related to band/band combinations in our understanding. Therefore, the capability of reporting per UE is sufficient.

Based on the discussion, we made a following proposal.

Proposal 7: FG 58-2-1 should be modified as follows.

| 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | 13-1 | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG58-2-2: Support reception of AD for UE-based positioning Case 1

Regarding the support reception of AD for UE-based positioning Case 1, for prerequisite feature groups, we think any UE feature groups are not necessary as prerequisite feature groups. It is because UE may receive assistance data for some LCM procedures such as model inference or data collection. The use case of reception of assistance data should not be limited by prerequisite feature groups. Moreover, we think the structure of existing FGs can be followed by FGs for UE-based positioning Case 1. Considering that the existing FG regarding reception of assistance data (e.g., FG 27-12) does not have any prerequisite feature groups, there is no need to introduce any UE feature groups as prerequisite feature groups.

Proposal 8: FG 58-2-2 should be modified as follows.

| 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG58-2-3/3a/3b: DL PRS Resources for UE-based positioning Case 1 (on a band/BC)

In Rel-19 discussion, there is no agreement to enhance DL PRS Resources for AI/ML based positioning. Therefore, we think it is straightforward to follow the elements of existing FGs 13-2/3/4 series.  We also think that the relationship between FG 58-2-4 and 58-2-3 series is similar to the relationship between FG13-1 and FG 13-3 series of a basic capabilities for legacy DL-TDOA of L1 UE features. Based on them, prerequisite feature group is FG 58-2-4 or FG 13-1.

Proposal 9: FG 58-2-3, 58-2-3a, and 58-2-3b should follow the elements of existing FGs 13-2/3/4 series. The FGs can be modified as follows.

| 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | 58-2-4; otherwise 13-1 | N/A | N/A |  | Per UE | n/a | n/a | n/a | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | 58-2-4; otherwise 13-1 | N/A | N/A |  | Per Band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | 58-2-4; otherwise 13-1 | N/A | N/A |  | Per BC | n/a | n/a | n/a | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

  - FG58-2-4: DL PRS Processing Capability for UE-based positioning Case 1

Regarding DL PRS Processing Capability for UE-based positioning Case 1, our views are shown below.

- Prerequisite group

  - Since DL PRS Processing Capability for legacy positioning (FG 13-1) does not have any prerequisite feature group, it is unnecessary for FG 58-2-4 to introduce any prerequisite feature groups. Regarding FG 13-1, in our understanding, FG 58-2-4 is independent from FG 13-1. Therefore, FG 13-1 is not needed for prerequisite feature groups.

- Note

  - The bracketed part describes how process DL PRS when FG 13-1 is provided with/without FG 58-2-4. It is important to clarify which capability is applied to avoid collusion and these notes are align with our view. Therefore, these brackets can be removed.

Proposal 10: FG 58-2-4 can be modified as follows.

| 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. | FFS | No | N/A |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. [Note: If the UE reports both FG 13-1 and [this FG], then the UE is not expected to perform simultaneous processing of legacy methods and Case 1 positioning method. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG58-2-5: Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1

For Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1, following the legacy capability “Support of SSB from neighbour cell as QCL source of a DL PRS,” prerequisite feature groups should be DL PRS Processing Capability. Following this principle, prerequisite feature group is FG 58-2-4 or FG 13-1. Therefore, we are fine to remove the bracket.

Proposal 11: FG 58-2-5 can be modified as follows.

| 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise 13-1] | No | N/A | SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG58-2-6: Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1

Same as FG 58-2-5, FG 58-2-6 follows the legacy capability “Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS,” i.e., the bracket in prerequisite feature groups can be removed.

Proposal 12: FG 58-2-6 can be modified as follows.

| 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise 13-1] | No | N/A | DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Discussion on FGs for AI/ML for CSI prediction

  - FGs for inference result report

In the last meeting, a common FG (i.e., FG 58-0-1) for all CSI-related AI/ML use cases was defined, and it should be the prerequisite for all FGs of the CSI-related AI/ML use cases where UE-side inference is involved. Therefore, the prerequisite of FG58-3-1 should include FG 58-0-1, the newly introduced FG.

Another issue is the APU occupation for the UE-side CSI prediction, a new component should be introduced to reflect the following agreement from RAN1#121.

Agreement For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook, - For PU occupancy, support - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied

It is noted in the agreements that detailed values of N and M can be further discussed in UE feature. For APU (i.e., CPU_2 in current specification) occupations, if a UE can report arbitrary number, there will be no consensus on the understanding of the APU between NW and UE. The maximum APU resources reported by UE is meaningless then. The NW cannot manage the configurations for UEs to fully utilize the AI/ML capability of UEs in the network. To avoid this and meanwhile to reflect the different UE implementations with different model size, we suggest the restrictions on the candidate values of O_APU. For example, N_small = 2 and N_large = 4 besides the value 0 for UE using small- or large-scale models.  Therefore, we have similar proposals about the APU/CPU occupations as the beam management part.

Proposal 13: Update the FG58-3-1 as the following,

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU_1 occupation. when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. Index about which APU pool is CPU_2 13. Value for CPU_2 occupation. | 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and per BC] | n/a | n/a | n/a | Candidate values for component 9: {0, 1, 8} Candidate values for component 12: {1,2} Candidate values for component 13: {1,2,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FGs for data collection

The prerequisite of this FG can be FG2-35 since the training data collection is based on the legacy CSI framework and no AI/ML model is involved.

Proposal 14: Update the FG58-3-4 as the following,

- Use FG 2-35 as the prerequisite (UEs without AI/ML capability can support the data collection).

| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | 2-35 | yes | n/a | UE side data collection for CSI prediction is not supported | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FGs for performance monitoring

Based on the agreements, an additional FG should be introduced for performance monitoring. To reflect the current progress, we propose the following FG,

Proposal 15:Introduce the following FG for the performance monitoring of AI/ML CSI prediction.

| 58. NR_AIML_Air | 58-3-5 | UE side performance monitoring for CSI prediction for UE-sided inference | Support the CSI report with reportQuantity set to csi-pai-r19 | 58-3-1 | yes | n/a | UE side performance monitoring for CSI prediction for UE-sided inference is not supported | Per band and Per BC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, we discussed on UE features for AI/ML for NR Air Interface. Based on the discussion, we made following proposals.

FGs for all CSI-related use cases

Proposal 1: Update FG 58-0-1 as follows.

- Add number of APU pools and update the corresponding parts for the reporting.

- The prerequisite is FG 2-32.

- The range of the maximum number of APUs for each APU pool is [0, 8].

| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Number of APU pools. 2. Maximum number of APUs for each APU pool for all types of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APUs for each APU pool for all types of UE-sided inference for CSI report(s) simultaneously across all CCs | 2-32 | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | Per UE | No | No |  | Component 1 candidate values: {0, 1, 2} Component 2 candidate values: [0, 8] Component 3 candidate values: [0, 8] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FGs for AI/ML in beam management

Proposal 2: Update FG 58-1-1 as follows.

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1.Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | 2-24 | yes | n/a | Increased number of reported beams for beam management is not supported | Per UE | No | No |  | Component 3 candidate values: {6, 8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Update FG 58-1-7 as follows.

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of data collection for UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] [6: Supported maximum number of resources for Set B] [7: Supported maximum number of resources for Set A] 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | 2-24 | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | Per UE | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Update FG 58-1-2 and 58-1-3 as follows.

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per PWB] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported of CSI-RS resource types for Set A: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instance 13. A list of supported combinations, each combination is {minimum required number of SSB/CSI-RS resources to be measured, maximum number of SSB/CSI-RS resources to be predicted} 14. Index about which APU resource pool is CPU_2. 15. Value of CPU_1 occupation. 16. Value of CPU_2 occupation. | 2-24, 58-0-1 | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference]is not supported | Per UE | No | No |  | Component 12 Candidate value set: {1, 2, 3, 4} Candidate value of the minimum required number of SSB/CSI-RS resources to be measured in Component 13: {4, 8, 16, 32} Candidate value of the maximum number of SSB/CSI-RS resources to be predicted in Component 13: {16, 32, 64, 128} Candidate values for component 14: {1,2} Candidate values for component 15: {0, 1, 8} Candidate values for component 16: {0, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] 2. Supported maximum number of predicted beams with predicted RSRP in each reporting instance | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference]is not supported | Per UE | No | No |  | Candidate values for Component 2 candidate values: {1, 2, 3, 4} FFS: CPU/AIMLPU related information | Optional with capability signalling |

Proposal 5: Update FG 58-1-4 and 58-1-5 as follows.

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported CSI-RS resource types for Set [A/B]: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s)] 21. A list of supported combinations, each combination is {minimum required number of SSB/CSI-RS resources to be measured, maximum number of SSB/CSI-RS resources to be predicted} 22. Required minimum number of measurement occasions of Set B 23. Index about which APU resource pool is CPU_2. 24. Value of CPU_1 occupation. 25. Value of CPU_2 occupation. | 2-24, 58-0-1 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference]is not supported | Per UE | No | No |  | Component 11 Candidate value set: {1, 2, 3, 4} Component 12 candidate value set: {1, 2, 4, 8} Candidate value of the minimum required number of SSB/CSI-RS resources to be measured in Component 21: {4, 8, 16, 32} Candidate value of the maximum number of SSB/CSI-RS resources to be predicted in Component 21: {16, 32, 64, 128} Component 22 Candidate value set: {1, 2, 4} Candidate values for Component 23: {1,2} Candidate values for component 24: {0, 1, 8} Candidate values for component 25: {0, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference]is not supported | Per UE | No | No |  | FFS: CPU/AIMLPU related information | Optional with capability signalling |

Proposal 6: Introduce the following FG for UE assisted performance monitoring.

| 58. NR_AIML_air | 58-1-6 | UE-assisted performance monitoring for UE-sided beam prediction | 1. Support of UE-assisted performance monitoring 2. Supported BM-Case(s) 3. Maximum value N for monitoring window (number of monitoring instances) 4. Support of M =1, 2 5. Maximum value of minimal slot offset X | 58-1-2, 58-1-4 | yes | n/a | UE-assisted performance monitoring for UE-sided beam prediction is not supported | Per UE | No | No |  | Component 2 candidate value set: {BM-Case1, BM-Case2} Component 3 Candidate value set: {1, 2, 4, 8, 16} Component 5 Candidate value set: {1, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FGs for AI/ML for positioning accuracy enhancement

Proposal 7: FG 58-2-1 should be modified as follows.

| 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | 13-1 | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8: FG 58-2-2 should be modified as follows.

| 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 9: FG 58-2-3, 58-2-3a, and 58-2-3b should follow the elements of existing FGs 13-2/3/4 series. The FGs can be modified as follows.

| 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | 58-2-4; otherwise 13-1 | N/A | N/A |  | Per UE | n/a | n/a | n/a | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | 1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | 58-2-4; otherwise 13-1 | N/A | N/A |  | Per Band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | 58-2-4; otherwise 13-1 | N/A | N/A |  | Per BC | n/a | n/a | n/a | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

Proposal 10: FG 58-2-4 can be modified as follows.

| 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. | FFS | No | N/A |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. [Note: If the UE reports both FG 13-1 and [this FG], then the UE is not expected to perform simultaneous processing of legacy methods and Case 1 positioning method. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 11: FG 58-2-5 can be modified as follows.

| 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise 13-1] | No | N/A | SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 12: FG 58-2-6 can be modified as follows.

| 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise 13-1] | No | N/A | DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FGs for AI/ML for CSI prediction

Proposal 13: Update the FG58-3-1 as the following,

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU_1 occupation. when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. Index about which APU pool is CPU_2 13. Value for CPU_2 occupation. | 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and per BC] | n/a | n/a | n/a | Candidate values for component 9: {0, 1, 8} Candidate values for component 12: {1,2} Candidate values for component 13: {1,2,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 14: Update the FG58-3-4 as the following,

- Use FG 2-35 as the prerequisite (UEs without AI/ML capability can support the data collection).

| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | 2-35 | yes | n/a | UE side data collection for CSI prediction is not supported | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 15:Introduce the following FG for the performance monitoring of AI/ML CSI prediction.

| 58. NR_AIML_Air | 58-3-5 | UE side performance monitoring for CSI prediction for UE-sided inference | Support the CSI report with reportQuantity set to csi-pai-r19 | 58-3-1 | yes | n/a | UE side performance monitoring for CSI prediction for UE-sided inference is not supported | Per band and Per BC | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Reference

[1] RP-250792, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, Mar. 2025.