3GPP TSG RAN-WG1 Meeting #122			R1-2506458

Bengaluru, India, 25-29 Aug, 2025

Source: 	Moderator (Huawei)

Title:	FL summary #1 for Ambient IoT: “10.3.1 Evaluations”

Document for:	Discussion and decision

Agenda item:	10.3.1

Introduction

The outcome for the study of Ambient IoT in Rel-19 for indoor scenarios can be referred in TR38.769 [1]. The outcome for the RAN1 normative work of Ambient IoT in Rel-19 can be referred in TS38.291 [2]. For the study of Ambient IoT in Rel-20 for outdoor scenarios, a set of objectives have been approved and updated in the SID [3]. According to the chair’s agenda and contributions [4-26] submitted to this meeting from companies, this feature lead summary is organized to discuss the evaluation assumptions and results for Ambient IoT study in Rel-20. Please companies indicate your contact list for this agenda into the table in the end of Section 1.

The draft FL proposals will be found in each section with the following way of naming:

Proposal 3.1a-v1: XXX … - ‘3.1’ is the section number in this document - ‘a’ is marked as the serial of proposals under this section, e.g. different proposals in this section will be numbered as {a, b, c, …} - ‘v1’ is the version of a proposal, and will be numbered as {v1, v2, v3, …} to update

Please follow the guidelines for offline discussions and inputs to FL summary

- Proposals/questions marked as [open] are candidates to be discussed in this meeting.

- Once a proposal/question marked as [closed] then no need further discussion in this meeting.

- Proposals/questions marked as [H] are assumed to be in high priority for the upcoming discussion round in this meeting.

- Proposals/questions marked as [M] are assumed to be in medium priority for the upcoming discussion round in this meeting.

- Please kindly provide your views by 20:00 Monday local time for the 1st round discussion. Early inputs before 13:00 Monday local time (before 1st online) are highly appreciated.

Please companies indicate your contact lists for this agenda as follows:

| Company | Contact person | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Zhe Jin (Feature lead) | jinzhe.jinzhe@huawei.com |
| Huawei, HiSilicon | Xiang Mi | shawn.mixiang@huawei.com |
| Huawei, HiSilicon | Sarun Selvanesan | sarun.selvanesan@huawei.com |

Related proposals

| Source | Observations/Proposals |
| --- | --- |
| FUTUREWEI | Observation 1: The active A-IoT devices (Device 2b and Device C) have channel selecting capability, which can enable the devices to support FDM on the R2D link and FDMA on the D2R link. Observation 2: With CFO being reduced to 10s to 100s ppm range after carrier frequency offset calibration, the device’s SFO should be reduced to the same ppm range. Observation 3. In addition to RF harvesting, other forms of energy harvesting are supported in active A-IoT devices. Observation 4: It is expected that both Budget-Alt1 and Budget-Alt2 are used in evaluations. Observation 5: The delay spread values are 129 ns for UMi and 363ns for UMa. Observation 6: The nominal delay spread for FR1 is 100 ns. Proposal 1: In calculating SINR the noise and interference bandwidth should be the same as the signal bandwidth when using Budget-Alt2 to calculate receiver sensitivity. Proposal 2: For link budget evaluation, adopt UMi and UMa for pathloss evaluation. Proposal 3: For link level simulations, select a delay spread value of 100 ns. Proposal 4: For link level simulations, adopt TDL-B as the channel model. |
| Spreadtrum, UNISOC | Proposal 1: The pathloss model of UMa-NLOS in TS 38.901 is adopted for coverage evaluation. Proposal 2: For evaluation purpose, - the Tx power of A-IoT BS reader is assumed as 46dBm. - the Tx power of Device 2b and Device C is be assumed as -10dBm and 0 dBm, respectively. Proposal 3: The link budget parameters and values agreed in R19 SI should be considered as the starting point for link budget calculation in R20 SI. Proposal 4: TDL-C channel model is used for outdoor simulation assumption. Proposal 5: The simulation assumption of initial SFO and CFO agreed in R19 is reused for device 2b/C. |
| Huawei, HiSilicon | Observation 1: The energy source used by the device for energy harvesting and the size of the capacitor for energy storage have an impact on the operation time of the device. Proposal 1: For the study of outdoor deployment scenario in Rel-20, Table 1 is adopted as deployment scenario assumptions. Proposal 2: For the study of outdoor deployment scenario in Rel-20, the maximum distance between reader and device is calculated as 2*(ISD/3). Proposal 3: For the study of outdoor deployment scenario in Rel-20, RAN1 uses the pathloss model formula from TR 38.901 to derive the coverage distance based on the link budget. Proposal 4: Adopt the link budget template in Table 3 with updated items and values for the Rel-20 Ambient IoT study. Proposal 5: The “Penetration margin (dB)” is introduced in the link budget, and the value is 26.25 dB for UMa NLOS pathloss model, and 17.46 dB or 25 dB for RMa NLOS pathloss model. Proposal 6: For the study of outdoor deployment scenario in Rel-20, the channel model for link level simulation is TDL-C, with an RMS delay spread of 300 ns. Proposal 7: For the study of outdoor deployment scenario in Rel-20, reference data rate can be 0.1~1 kbps for both R2D and D2R coverage evaluation. Proposal 8: For the study of outdoor deployment scenario in Rel-20, the CFO and SFO values for device 2b/C can be set as follows - For device 2b (as per TR38.769) - The initial SFO can randomly select a value from the range of [0.1 ~ 1] *104 ppm, and the initial CFO can be set as 1000 ppm. - The SFO after clock sync / calibration at device side is 103 ppm, and CFO after clock sync / calibration at device side is 100 ppm. - For device C - The initial SFO and initial CFO can be set as 50 ppm. - The SFO and the CFO after clock sync / calibration at device side can be set as 10 ppm. Proposal 9: Link level simulation is evaluated according to the assumptions in Table 4. Proposal 10: Companies are suggested to report the evaluation results based on Table 5. Proposal 11: Companies to report assumptions on the energy source and capacitor size for energy harvesting/storage used to obtain the reported data rates. Our assumptions are as follows: - Energy source: solar power/light - Capacitor size: ~200 mF |
| Ericsson | - Observation 1: For Rel-20 A-IoT outdoor devices, the coverage target should reflect the differentiation from 6G LPWA, consider the SID guidelines and the mentioned distance range of 50 to 500 m. - Observation 2: As a starting point and before considering any coverage enhancement technique, the initial evaluations should consist in performing an analysis (e.g., link-budget) assuming the higher transmitting power available for devices C to determine if the coverage target can be fulfilled. - Observation 3: For maximum transmit power for coverage evaluation of device C in sensor applications, where the transmission power budget is lower than that in other use cases, at least small values such as 0 dBm or -3 dBm can be considered to evaluate whether the coverage targets can still be met. Proposal 1: The Rel-19 SI A-IoT link budget methodology and template, Table 4.3.2-1 in 38.769, can be taken as starting point and adapted accordingly for evaluating coverage of active A-IoT devices in outdoor scenarios. Proposal 2: The so-called Budget-Alt2 can be used for calculating receiver sensitivity and coverage evaluation for both reader-to-device (R2D) and device-to-reader (D2R) links. Proposal 3: For coverage evaluation of active devices in D4T1 scenario, RAN1 should consider UMa NLOS pathloss models. Proposal 4: For pathloss models UMa NLoS, the shadow fading margins can be considered as 6 dB. Proposal 5: Consider the following parameters for the reader: transmit power spectrum density of 36 (dBm/MHz), TX/Rx antenna gain of 8 dBi, and vertical height of 25 m. Proposal 6: Consider the following maximum transmit power for coverage evaluation of device C: -3 dBm, 0 dBm +5 dBm. Proposal 7: Use the adapted version of Table 4.3.2-1 from 3GPP TR 38.769, shown in Table 1, to evaluate the coverage of active A-IoT devices in outdoor scenarios (the needed updates are marked by red color). Proposal 8: Consider TDL‑C channel with a delay spread of 300 ns to link-level evaluations. Proposal 9: Use the adapted version of Table 4.2.2-1 from 3GPP TR 38.769, shown in Table 2, to evaluate the BLER performance of active A-IoT devices in outdoor scenarios (the updates are marked by red). |
| CATT | Observation 1: For R2D link, there will be no error floor only when M=2. Observation 2: For the R2D link in TDL-C channel: - The values of required SNR at BLER=10% when M=2, 6, 12, 24 are 4.2 dB, 9.7 dB, 12.7 dB, 18.2 dB respectively. - The values of required SNR at BLER=1% when M=2 is 16.3 dB. Observation 3: The values of required SNR at BLER=10% and 1% for the D2R link in TDL-C channel are -5.8 dB and -4.7 dB respectively. Observation 4: For device 2b with R2D data rate 7.5kbps and D2R data rate 5kbps, the coverage distance can achieve: - 232.8m when the target BLER is 10% - 218.2m when the target BLER is 1% Observation 5: For device C with R2D data rate 7.5kbps and D2R data rate 5kbps, the coverage distance can achieve: - 756.5m when the target BLER is 10% - 709.0m when the target BLER is 1% Proposal 1: For the coverage evaluation in Rel-20, the link budget template defined in Rel-19 should be used as baseline. Proposal 2: UMa NLOS and [RMa LOS] can be used as the pathloss model for the Rel-20 A-IoT study of outdoor deployment scenarios. Proposal 3: Both of the hardware capabilities of A-IoT device implementation and the value derived from the Budget-Alt2 need to be considered in the determination of receiver sensitivity for A-IoT devices. Proposal 4: Use the link budget template in Annex A to evaluate the coverage performance for device 2b and device C in outdoor scenario. Proposal 5: The following channel models can be considered in LLS for Rel-20 A-IoT: - TDL-C, 300ns, 3km/h - [TDL-E, 30ns, 3km/h] Proposal 6: Reference data rate higher than 100kbps can be considered in the LLS for Rel-20 A-IoT. Proposal 7: Use the assumptions in Annex B for the LLS for device 2b and device C. |
| vivo | Observation 1: The cell edge data rate, derived according to a certain coverage range, is not equivalent to user-experience data rate actually needed in AIoT use cases. - For example, if the cell edge data rate is ~5kbps assuming X = [500] meters coverage, while the user experienced data rate required in AIoT use case is typically less than 1kbps, the achievable coverage for Rel-20 AIoT is higher than X = [500] meters. Observation 2: BPSK and FSK are constant-envelope waveforms that enable much higher PA efficiency and transmitter efficiency compared with that for OFDM waveform widely used in cellular networks. Observation 3: initial clock accuracy is about 103ppm and residual CFO of 30ppm can be achieved after calibration. Observation 4: 500m coverage cannot be always achieved in UMa scenario and in RMa scenario based on Rel-19 signal channel design with minimum data rate with M=2 for R2D and ~5kbps for D2R. Observation 5: Remarkable coverage improvement can be achieved with enhancement solutions on top of Rel-19 air interface design. Observation 6: For AIoT-SSB signal/channel, 500m in UMa and 1000m in RMa scenario can be achieved respectively if 2 block level repetitions are used. Proposal 1: 500 meters can be assumed as starting point to determine cell edge data rate for UMa and RMa scenario - Different data rates can be evaluated to determine cell edge data rate, and to determine actual coverage can be achieved under user-experience data requirements, e.g., <1kbps. Proposal 2: For coverage evaluation for outdoor and active device, reuse evaluation methodologies used for device 2b for indoor scenario in Rel-19, with necessary changes, e.g., BS/AIoT device Tx power, channel model, and etc., applicable to outdoor scenario. Proposal 3: IF envelop detector or ZIF envelop detector should be assumed for receiver of active device - R2D receiver sensitivity is derived using budget-Alt2, i.e., receiver sensitivity is calculated based on required SNR obtained from LLS results. Proposal 4: For evaluation purpose for Rel-20 AIoT, 3.84Msps sampling rate at device can be considered as starting point. Proposal 5: For evaluation purpose for Rel-20 AIoT, 1~3 RB channel bandwidth can be considered as starting point for both D2R and R2D. Proposal 6：For evaluation purpose, 46dBm/43dBm Tx power and 8dBi antenna gain can be assumed for outdoor BS. Proposal 7：For evaluation purpose, - -10 dBm Tx power is assumed for device 2b; - 4 dBm Tx power is assumed for device C. Proposal 8: UMa-NLOS/RMa-NLOS can be considered as a pathloss model for outdoor, and shadow fading margin is 6dB and 8dB for UMa and RMa, respectively. Proposal 9: 10 km/h device speed is assumed in LLS. Proposal 10: TDL-C with 300ns for NLOS channel can be used in LLS for outdoor scenario. Proposal 11: Following coverage enhancement solutions on top of Rel-19 air interface design can be considered in coverage evaluation for Rel-20 AIoT - Repetition and/or FEC can be considered for R2D - Modulation schemes robust to CFO and enhancements on repetition/FEC, can be considered for D2R. Proposal 12: Evaluate coverage of periodic sync signal, aka., AIoT SSB considering large initial CFO. Proposal 13: Evaluate multiple data rates, and achievable cell edge data rate can be determined as the highest data rate which can achieve coverage better than selected maximum distance between Reader and Device e.g., 500 meters. Proposal 14: TR captures a conclusion that the reported data rates can be supported by energy sources with high energy conversion efficiency, e.g., solar, vibration, thermal, etc., with sufficient energy storage, which is up to implementation. - Reporting of detailed assumptions of energy source/storage corresponding to the data rate is not needed. Proposal 15: Adopt the Table 4 in R1-2505410 as evaluation assumptions for coexistence. - Detailed cases for co-existence evaluation, e.g., NR/AIoT aggressor AIoT/NR victim, etc., is up to RAN4 discussion. |
| Nokia | Observation 1: For the representative use cases rUC5, rUC6, and rUC8, more power budget for longer range communications and active data transmission capability is necessary like Device 2b and Device C. Observation 2: The only evaluation scenario in scope is “D4T1-C” (Deployment scenario 4 with topology 1 with no external carrier wave) which can be characterized as follows (following the format in Table 4.2.1-1 of TR 38.769): Scenario; CW Inside/outside topology; Diagram of the scenario; Description of the scenario; Device; CW spectrum; D2R spectrum; R2D spectrum. D4T1-C; No CW; No CW Node.; Device 2b, C; N/A; UL; DL. Observation 3: According to the SID, device 2b and device C use IF-ED or ZIF receiver. Proposal 1: RAN1 conclude that Device 2b and Device C are necessary and applicable to support the representative use cases rUC5, rUC6, and rUC8. Proposal 2: Select UMa (urban macro) as scenario for evaluation. Proposal 3: Select UMa-NLOS as pathloss model for link budget calculation. Proposal 4: Select TDL-C with 300 ns delay spread as channel model for link level simulation. Proposal 5: Consider 3 km/h and 10 km/h for device velocity. Proposal 6: Adopt initial SFO 104 ppm for device 2b and device C. Proposal 7: Adopt 800 bits as additional message size for LLS. Proposal 8: Use the link budget template (Table 4.3.2-1 of TR 38.769) agreed in Rel-19 as starting point. Proposal 9: Adopt Budget-Alt2 to derive the R2D receiver sensitivity. Proposal 10: Adopt 3-sector base station antenna pattern with 8 dBi. Proposal 11: Adopt base station antenna height of 25 m. Proposal 12: Adopt base station TX power of 46 dBm. |
| Xiaomi | Observation 1: The KPIs for sensor data collection in TS 22.369 can be applied as references when designing and determining the evaluation assumptions of Rel-20 A-IoT. Observation 2: The co-existence simulation methodology warrants re-examination to accommodate certain novel circumstances within the DT41-C scenario. Observation 3: The assumptions for coexistence simulation can be adaptively adjusted by referring to the existing simulation assumptions in Rel-19. Proposal 1: For device 2b in D4T1, the applicable maximum distance target value is 500m. For device C in D4T1, the applicable maximum distance value can reuse 500m or can be defined separately. Proposal 2: The procedures for sensor use case can be defined by the following steps: - Step 1: R2D synchronization signal transmission. Reader transmits synchronization signal to be received by A-IoT device(s). - Step 2: A-IoT random access. Synchronized A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure. - Step 3: Energy harvesting. A-IoT device(s) perform energy harvesting until the stored power is higher than the pre-defined threshold. - Step 4: Possible D2R data transmission. A-IoT device(s) perform the sensing result(s) transmission(s). Proposal 3: The definition of latency of sensor use case can be refined as the time interval between the time that R2D synchronization signal transmission and time that D2R data transmission completed or successfully received by reader. Proposal 4: For the purpose of potential evaluation, D4T1 can be added to the Table 4.2.1-1 of Evaluation scenarios in TR 38.769. Table 4.2.1-1: Evaluation scenarios (update) Scenario; CW Inside/outside topology; Diagram of the scenario; Description of the scenario; Device 1/2a/2b/C; CW spectrum; D2R spectrum; R2D spectrum. D4T1-C; No CW; No CW Node.; Device 2b/C; N/A; UL; At least DL. Proposal 5: In Rel-20 A-IoT for evaluation assumptions of both R2D and D2R, the channel model can consider: - D4T1: UMa NLOS with TDL-A and UMa LOS with TDL-D as baseline. - D4T1: RMa and SMa can be optionally considered. Proposal 6: In Rel-20 A-IoT, for evaluation assumption, the message size can additionally consider about 800 bits for outdoor sensor. Proposal 7: In Rel-20 A-IoT, for evaluation assumption, R2D FEC with 1/3 CC can be optionally considered in addition to “No FEC”, in order to achieve larger coverage. Proposal 8: For Rel-20 A-IoT, the coverage evaluation assumptions for Link-level simulation in TR 38.769 can be updated to support new deployment and active device with the following Table as reference. Proposal 9: The UMa, RMa (optional) and SMa (optional) scenario in TR 38.901 could be considered as the fundamental assumptions for the purpose of formulating the assumptions regarding the layout of D4T1 in Rel-20 A-IoT. Proposal 10: In Rel-20 A-IoT, for evaluation purpose, the following Link budget template and coverage calculation are defined for outdoor scenario and device 2b/C. Proposal 11: At least case 7 should be considered for coexistence evaluation in Rel-20 A-IoT. Proposal 12: The coexistence cases for Rel-20 A-IoT can be down selected from the definitions provided in TR 38.769 as show in the following Table. Proposal 13: The distribution of NR UE can be either within the coverage area of the A-IoT reader or solely within the coverage area of the NR BS, without overlapping with the area where the A-IoT devices are situated. Proposal 14: RAN1 needs to clarify whether the transmission power of the A-IoT reader is independent of the transmission power of the NR BS. Proposal 15: RAN1 needs to clarify whether to study the method for interference suppression to provide reasonable assumptions for the coexistence simulation work. |
| China Telecom | Proposal 1: LLS-based link budget methodology for A-IoT in indoor scenarios can be reused for A-IoT in outdoor scenarios for coverage evaluation methodology. Proposal 2: For an outdoor evaluation scenario, - For R2D/D2R link, - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements. - Step 2: Obtain the receiver sensitivity derived by the required SINR which is given by LLS results. - Step 3: Obtain the coverage performance for link R2D/D2R based on the receiver sensitivity from step 2 and link budget template. Proposal 3: For outdoor scenarios, MPL and distance is used as performance evaluation metric for link budget calculation, where the distance is derived from MPL and corresponding pathloss model. Proposal 4: The following pathloss models for outdoor scenarios are used in the coverage evaluation. - TDL-C channel model for NLOS component - TDL-D channel model for LOS component Proposal 5: Regarding the carrier frequency, at least 900MHz and 2GHz need to be considered for FDD spectrum for outdoor scenarios. Proposal 6: Support the following BS layouts for UMa and RMa scenarios. Proposal 7: For LLS evaluation in outdoor scenarios, the total transmit power of BS can be configured as 36dBm as a starting point. Proposal 8: The Tx antenna gain of BS for A-IoT in outdoor scenarios should be considered. - The detailed values can be FFS. Proposal 9: For outdoor scenarios, link performance can be evaluated by predefining reference data rates and then deriving the achievable coverage range. - The detailed values of reference data rate can be FFS. Proposal 10: Coherent receiver is recommended in D2R transmission in outdoor A-IoT scenarios. |
| Samsung | Observation 1: For D2R link level simulation with payload size of 20 bits and 96 bits, after adding CFO=100 ppm with drift rate of 0.1ppm/s, both payload size could reach target BLER of 10% and 1%. The performance degradation for payload size of 20 bits are negligible, and for payload size of 96 bits there are 1dB degradation at BLER of 10% and ~3dB degradation at BLER of 1%. Proposal 1: Study applicable maximum distance target values for device 2b and device C, respectively, within the range of 50-500m, considering - Define separate target values of Device 2b and C, study the performance gap considering at least Tx power difference and R2D receiver sensitivity - Target value of device 2b is larger than 50m to achieve improvement from Rel-19 - Target value of device C can start with typical 3GPP outdoor UE coverage Proposal 2: The definition of latency in Rel-19 is reused for Rel-20 evaluation at least for DO-DTT and DT traffic types. - Study if new definition of latency are needed for DO-A traffic type. Proposal 3: On the basis of evaluation assumptions in Rel-19 SI, study at least the following assumptions for D4T1 scenarios: - Channel model - BS related assumptions, e.g. total transmit power, antenna gain, number of antenna elements, number of TXRUs - Layouts Proposal 4: On the basis of evaluation assumptions in Rel-19 SI, study at least the following refinements according to Rel-19 outcomes: - Block structure, e.g. X-ambles, CRC, FEC, repetitions - Reference data rate - Transmission bandwidth for R2D and D2R - Modulation and line encoding Proposal 5: On the basis of evaluation assumptions in Rel-19 SI, study at least the following assumptions according to Device 2b/C capabilities: - D2R transmit power - R2D receiver aspects, e.g. receiver noise figure, receiver sensitivity - ED bandwidth for R2D reception, transmission bandwidth and frequency resource allocation methods for D2R transmission - CFO aspects, e.g. CFO values, feasible assumptions on reader handling of CFO estimation and correction, D2R FDM related assumptions and restrictions, assumptions on phase offset, etc. Moreover, SFO aspects to reflect chip length drift - Assumptions related to self-generated D2R signal, e.g. waveform, power control Proposal 6: The layouts for coexistence evaluation can be up to RAN4. Proposal 7: Study the feasibility and appropriate assumptions for R2D narrowband filtering for coverage and coexistence evaluations. |
| ZTE | - Proposal 1: For coverage evaluation, the cell-edge data rate is investigated and the following methodology can be considered: - Step 1: Obtain the MPL based on the maximum coverage distance via pathloss model; - Step 2: Derive the required SNR from link budget according to the MPL; - Step 3: Determine the data rate corresponding to the required SNR via link-level simulations. - Proposal 2: Clarify the coverage target of outdoor deployment. The coverage distance of 500 m is assumed as the starting point and larger coverage distance can be studied to support rural use cases. - Proposal 3: Both in-band and standalone deployments should be considered in A-IoT coverage evaluation. - Proposal 4: For pathloss model, UMa NLOS is recommended as mandatory and the RMa NLOS model can be treated as optional. - Proposal 5: An additional penetration margin of 20 dB should be accounted in link budget based on O2I Car penetration loss model in 38.901. - Proposal 6: The link budget template in Table 2 can be considered as a starting point for A-IoT coverage evaluation. - Proposal 7: Coverage enhancements for both R2D and D2R transmissions should be studied for Device 2b/C to support the outdoor scenario. - Proposal 8: For energy storage, the maximum PRDCH reception time and the maximum PDRCH transmission time that Device 2b/C supports can be assessed and the above assessment methodology can be considered as a starting point. - Proposal 9: Device (un)availability solutions need be studied for Device 2b/C for the following cases: - Device (un)availability prior to receiving the target paging message. - Device (un)availability during the random access procedure. - Device (un)availability after the random access procedure. - Proposal 10: For the coexistence between Rel-20 A-IoT and NR/LTE, the cases in Table 3 and 4 can be considered and the details are determined by RAN4. |
| Transsion | Proposal 1: The maximum distance for each device type can be determined based on evaluation results of the link budget. Proposal 2 It is recommended to use the Budget-Alt2 method for calculating receiver sensitivity in the link budget template. Proposal 3 For Deployment Scenario 4, it is recommended to use the UMa (Urban Macro) and UMi (Urban Micro) channel models defined in TS 38.901 as the basis for pathloss modeling. |
| OPPO | - Proposal 1: The UMa scenario is considered for A-IoT outdoor scenarios. - Proposal 2: A-IoT devices drop uniformly distributed over the horizontal area in D4T1. - Proposal 3: D4T1-C is defined for A-IoT outdoor scenarios for the purpose of evaluation. - Proposal 4: The UMa NLOS model is considered as pathloss model in coverage evaluation for D4T1. - Proposal 5: The total Tx power of macro-BS is assumed to be 46 dBm. - Proposal 6: The total Tx power of Device C is assumed to be -3 dBm. - Proposal 7: The Tx antenna gain of BS for outdoor is assumed to be 8 dBi. - Proposal 8: The shadow fading margin is assumed to be 7.8 dB for the UMa NLOS model. Proposal 9: The link budget template for Device 2b can be reused for Device C, with the update of the total Tx power only. Proposal 10: For D4T1, TDL-C channel model with delay spread of 300 ns is considered for link-level simulation. Proposal 11: The LLS assumptions of Device 2b can be reused for Device C. Proposal 12: Consider -30dBm as RF energy harvesting sensitivity in Rel-20 A-IoT outdoor study. Proposal 13: Consider energy storage of A-IoT device <=200μJ in Rel-20 A-IoT outdoor study. |
| Tejas | Observation 1: To evaluate the pathloss model for outdoor Ambient IoT, the NLOS pathloss model for outdoor urban macro BS and urban micro BS are considered from TR 38.901. It has been observed that urban macro BS provides better coverage than urban micro BS. Observation 2: The coverage enhancement depends on multiple factors such as, the receiver sensitivity of device and reader, transmit power of device and reader, communication bandwidth, BS/reader and device height, etc. Observation 3: Considering urban macro BS and UE height of 5 m, the coverage values of Device 2b for different device transmit power are, 55m for -20dBm, 75m for -15dBm, and 103m for -10dBm device transmit power. Observation 4: Considering urban macro BS and UE height of 5 m, the coverage values of Device C for different device transmit power are, 157m for -3dBm, 188m for 0dBm, 225m for 3dBm, and 253m for 5dBm device transmit power. Observation 5: It has been observed that the 3D distance from reader to device increases with the device antenna height. Proposal 1: A suitable transmit power of device and Reader (base station) can be chosen to achieve the maximum target coverage which is the minimum value of the coverage achieved for the physical channels PRDCH, PDRCH for monostatic scenario. Proposal 2: A suitable and feasible device antenna height of 5-20m can be chosen to achieve the coverage target. |
| LGE | Proposal 1: Support at least 500m as the maximum target distance to support outdoor scenarios for Rel-20 Ambient IoT Proposal 2: Support 38 dBm as the basic evaluation assumption for maximum Tx power of base station for outdoor scenarios Proposal 3: At least assume following values for maximum Tx power of active devices for Rel-20 Ambient IoT study: - 5 dBm for device C - -10 dBm for device 2b Proposal 4: For Rel-20 Ambient IoT study, reuse as much as possible the link budget parameters for IF/ZIF receiver assumed in Rel-19 Proposal 5: MPL can be used for link budget calculation Proposal 6: UMa scenario as described in TR 38.901 can be assumed for path-loss model Proposal 7: TDL-C channel can be used as the mandatory link level simulation channel model Proposal 8: For LLS, parameters not specific to deployment scenarios (e.g. indoor/outdoor scenario) can be reused for Rel-20 evaluation assumptions |
| Apple | Proposal 1: Following LLS assumptions can be considered as the starting point for Rel-20 Ambient IoT Study on active devices for outdoor scenario DS4: Table 1: LLS assumptions for DS4 Proposal 2: Following link budget template can be considered as the starting point for Rel-20 Ambient IoT Study on active devices for outdoor scenario DS4: Table 2: Link budget template for DS4 Proposal 3: In terms of design targets, coverage study should aim to determine separate coverage range for device 2b and device C (within the recommended range of 50-500m) - For determining the respective coverage range comparison between device 2b and device c, same air interface design assumed as baseline and those can be feasible within device’s complexity and power requirements Proposal 4: Following deployment assumption can be considered as the starting point for Rel-20 Ambient IoT Study on active devices for outdoor scenario DS4: Table 3: DS4 deployment assumptions |
| NEC | Observation 1: For device with maximum transmit power higher than or equal to -10dBm, R2D would be the bottleneck for pathloss calculation. Observation 2: For device with maximum transmit power as -20dBm, D2R could be bottleneck only when 46dBm maximum transmit power adopted by gNB. Proposal 1: For outdoor channel model, UMi-street and UMa can be used for evaluation, while RMa and SMa may not be suitable due to large intersite distance. Proposal 2: For evaluation purpose, R2D would be the bottleneck of pathloss when transmit power of active device is higher or equal to -10dBm. Proposal 3: To meet the maximum distance of 50-500m, at least 83dB pathloss should be provided, which means the maximum transmit power of gNB should be higher than or equal to 38dBm, and device should use FEC and repetition when adopting -20dBm transmit power. |
| CMCC | Proposal 1: The MPL and coverage distance is used as performance metric for link-budget calculation. Proposal 2: For R2D link and D2R link in the coverage evaluation, Budget-Alt2 is used for device 2b and device C. Proposal 3: For an evaluation scenario, the coverage evaluation methodology is based on the following steps. - For R2D link or D2R link, - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements - Step 2: Obtain the receiver sensitivity using Budget-Alt2 (receiver sensitivity is derived by required SINR which is given by LLS results). - Step 3: Obtain the coverage performance based on the receiver sensitivity from step 2 and link budget template. Proposal 4: For energy harvesting of device 2b and device C, various power sources provided by external environment other than RF energy harvesting can be considered. Proposal 5: Link budget for communications between reader and device can be calculated respectively as below， - MPLR2D = Transmitter Tx power – Device receive sensitivity + Transmitter antenna gain + Receiver antenna gain – shadowing fading margin – polarization loss - MPLD2R = Device Tx power – Receiver sensitivity+ Transmitter antenna gain + Receiver antenna gain – shadowing fading margin – polarization loss Proposal 6: The following pathloss model can be used in the coverage evaluation of Ambient IoT - For urban scenarios, UMA_A NLOS model can be used. - For rural scenarios, RMA_A NLOS model can be used. Proposal 7: For outdoor A-IoT BS readers, total Tx power of 38 dBm/46 dBm and Tx antenna gain of 15dBi can be considered for coverage evaluation. Proposal 8: The item of receiver interference density can be added in the link budget template considering the impact of outdoor cell interference, the candidate values can be reported by companies. Proposal 9: Adopt the link budget template in Table 2.1.1-1 for link budget evaluation in Rel-20 Ambient IoT. Proposal 10: For antenna configuration, 1Tx/1Rx for Ambient IoT devices is considered. Proposal 11: For link level performance evaluation, the following channel models are assumed, - Chanel models TDL-C as in TR 38.901, assuming a delay spread of 300ns and speed of 3km/h. Proposal 12: The following parameters of sampling frequency are considered in the evaluations, Parameter; Values. Sampling Frequency; Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial CFO for device 2b/C: 100 ppm (M) 200 ppm (O) 1000 ppm (O, only as initial CFO) Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. CFO after calibration for device 2b/C: 10 ppm - Note: Above assumptions are for LLS evaluation purpose only. Proposal 13: The BB LPF reported by companies in assumptions for LLS can be referred as receiver bandwidth for calculation of noise power for R2D in link budget template. Proposal 14: Adopt the LLS assumptions in Table 2.1.2-1 for coverage evaluation in Rel-20 Ambient IoT. |
| Sony | Observation 1 – Assuming Urban Marco pathloss model, the maximum distance 50-500 m is equivalent to LOS pathloss of 64.6-96 dB and NLOS of 79-118 dB. Observation 2 – With 5dBm Tx power, a link budget of 115dB can be achieved and the cell edge coverage can be realized. Observation 3: Device 2b may run without recovery periods on small batteries even though recovery period may be needed for devices relying on energy harvesting. Observation 4: Recovery periods may be needed for Device C when it performs its transmission when operating on constrained batteries, depending on device power consumption. Proposal 1 – RAN1 to use link budget template of Device 2b in the TR 38.769 when calculating link budget for device types of 2b and C. Proposal 2 – RAN1 to update/include the following parameters to address deployment scenario 4 and Device type C - Tx power for both BS and Device types, - Noise figure for Device types 2b and C, - Channel model to represent outdoor Macro, - CFO and SFO. Proposal 3 – For comparing applicability of Device 2b and Device C, RAN1 to include latency and spectral efficiency as performance metrics in addition to data rate at the cell edge. Proposal 4: Consider including energy harvesting and battery characteristics when choosing possible output power levels and system parameters. Proposal 5: For traffic type of DO-A, study feasibility of using code-based non-orthogonal multiple access in D2R. |
| InterDigital | Observation 1: Using the Rel-19 PRDCH design with wider receive BW (e.g., 2 MHz) may not be sufficient to achieve coverage at link distances of 500 m. Observation 2: Using the Rel-19 PDRCH design may achieve coverage at 500 m for device C, but not for Device 2b. Proposal 1: Use the evaluation assumptions from Table 1 as a starting point for R2D link level simulation. Proposal 2: Use the evaluation assumptions from Table 2 as a starting point for D2R link level simulation. Proposal 3: Study methods to improve R2D coverage such as channel coding, repetition. Proposal 4: Larger number of repetitions and lower code rate should be supported to improve D2R coverage if it is concluded that coverage enhancement is necessary. Proposal 5: Develop a revised link budget for Device 2b and Device C in Rel-20 based on the evaluation assumptions in Table 6. |
| Qualcomm | Observation 1: The A-IoT use cases with target coverage of 500m (i.e., use cases requiring large cells, wide area outdoor) have overlap that for LPWA, e.g., smart grid operation, gas meter, water meter, large smart farm sensors, infrastructure monitoring, etc. Observation 2: Low D2R data rate case - Device 2b - Device 2b coverage is D2R limited. - Max coverage of Device 2b (-20dBm, D2R 7bkps) is 61.8m. - Max coverage of Device 2b (-10dBm, D2R 7bkps) is 118.6m. - Device C - Device C (-3dBm) coverage is D2R limited. - Max coverage of Device C (-3dBm, D2R 7/4bkps) is 409.5m. - Device C (5dBm) coverage is R2D limited. - Max coverage of Device C (+5dBm, D2R 7/4bkps) is 690m. Observation 3: Higher data rate case - Device 2b - Device 2b link is D2R limited. - Max coverage of Device 2b (-20dBm, 42bkps) is 54.6m. - Max coverage of Device 2b (-10dBm, 42bkps) is 104.8m. - Device C - Device C (-3dBm) coverage is D2R limited. - Max coverage of Device C (-3dBm, 42/4bkps) is 361.7m. - Device C (5dBm) coverage is R2D limited. - Max coverage of Device C (+5dBm, 42/4bkps) is 609.6m. Observation 4: For active device, long tx/rx duration should be avoided due to higher power consumption. Observation 5: Higher data rate can reduce energy consumption by reducing tx/rx duration. Observation 6: Rel-20 devices should support a higher data rate to energy storage requirement. Observation 7: In outdoor (TDL-C, 300ns), the BLER performance of PRDCH of large M = 24, 32 degrade significantly due to long delay spread in outdoor environment compared indoor (TDL-A, 30ns). Observation 8: Overlaid sequence could provide diversity gain for the performance of PRDCH with M values <=12. Observation 9: For PRDCH detection, the energy detection method provides the significant gain of a few dB to more than 10dB compared to edge detection method. Observation 10: For SFO=1000ppm, the CP handling Method 1-1 works reasonably well for all M values up to M=32. Proposal 1: For coverage and coexistence evaluation, RAN1 to study at least two target use cases in outdoor scenarios in terms of coverage. - Target 1:50-100m (small cell/local area outdoor – small smart farm sensors, temperature/humidity monitoring sensor, outdoor inventory for small business, small size infrastructure monitoring) for device 2b - Target 2: 400-500m (large cell, wide area outdoor – smart grid operation, gas meter, water meter, large smart farm sensors, infrastructure monitoring) for device C Proposal 2: At least for evaluation purposes, RAN1 to investigate the following active device architectures with ZIF/IF receivers with further details regarding their potential effects on system design - Whether Tx RF filter / Rx RF filter is necessary or not for outdoors - Sampling clock speed faster than that for Device 1, e.g., 5.76 MHz or 11.52MHz - The accuracy assumption for Rx/Tx LO for generating carrier frequency at 900MHz and absence/presence of crystal - Whether I/Q branches are needed or not depending on the modulation scheme, waveform, etc. - Noise figure for 2b/C - Feasibility of power amplifier for 2b/C Proposal 3: For coverage evaluation purpose, consider the outdoor scenario D4T1-C. Proposal 4: For [1D]-R2D Number of Tx antenna/element TxRU/tx chain modelled in LLS, consider (M,N,P,Mg,Ng) = (4,2,2,1,1) or (2,4,2,1,1) with two TxRUs. Proposal 5: For coverage evaluation purpose, assume total R2D BS tx power of [44, 49]dBm with PSD limit of 36+X dBm/MHz and where X is a power boosting factor. X=0 for NR signal and X=6dB for A-IoT signal. Proposal 6: For [1F] R2D transmission bandwidth, consider 180kHz ~ 4.32MHz. For D2R transmission bandwidth, consider 15kHz as starting point. Proposal 7: For [1G]-R2D BS Tx antenna gain consider 17dBi (based on 16 elements, 2 Tx RU). For [1G]-D2R device tx antenna gain, consider -3dBi to capture practical value. Proposal 8: For [1N]-R2D Cable loss, consider 3dB considering outdoor BS. Proposal 9: For [2B]-R2D Bandwidth used for the evaluated channel, companies to report rx bandwidth considering tx signal bandwidth, Rx LO uncertainty, and BB LPF bandwidth. Proposal 10: For [2B]-D2R Bandwidth used for the evaluated channel, companies to report rx bandwidth considering, tx signal bandwidth (with 1SB/2SB assumption), and Tx LO uncertainty. Proposal 11: For [2D]-R2D Receiver Noise Figure, consider 18dB for device 2b and 15dB for device C. For [2D]-D2R Receiver Noise Figure, consider 5dB. Proposal 12: For [2L]-R2D/D2R Receiver sensitivity, consider Budget-Alt2 method only. Proposal 13: For [3A] R2D/D2R Shadow fading margin, consider 7.82dB for UMi and 6dB for UMa. Proposal 14: For [0d] Channel model, for NLOS consider TDL-C(M) and TDL-A(O). For LOS, consider TDL-D. Proposal 15: For [0d]-R2D/D2R Delay spread, consider following delay spread values for channel models. - 300ns(M) and 150ns(O) for TDL-C(M) - 150ns(M) and 30ns(O) for TDL-A(O) - 150ns(M) and 30ns(O) for TDL-D(O) Proposal 16: For [0f]-R2D/D2R Device velocity of 3km/h. Proposal 17: For [0h1] BS number of antenna elements, consider (M,N,P,Mg,Ng) = (4,2,2,1,1) or (2,4,2,1,1) with 2 TxRUs. Proposal 18: For [0m] Reference data rate, consider 7, 14, 42, 84, and 168kbps as a starting point. Proposal 19: For [0q-a] Sampling frequency offset, consider following options. - SFO before calibration: 10^3 - 10^4 ppm (M) - SFO after calibration: 10^3 ppm (M), 10^2 ppm (O) Proposal 20: For [0q2] Carrier frequency offset, consider following options. - CFO before calibration: 10^2 ~ 10^3 ppm (M), 10 - 100 ppm (O) - CFO after calibration: 100ppm (M), 10ppm(O) Proposal 21: For [1b] ED bandwidth, companies to report considered ED bandwidth considering tx signal BW and LO uncertainty. Proposal 22: For [1d] Waveform, OOK-4 or DFT-spread-OFDM could be baseline. Proposal 23: For [1e] Modulation, consider OOK as baseline. Other options are not precluded. Proposal 24: For [1j] Detection/decoding method for line coding, for line coding decode, if exist, RAN1 to assume “Energy detection” method (comparing energy of first chip and that of the second chip) as baseline. Companies to report schemes, if different scheme is considered. Proposal 25: For [2a1] Transmission bandwidth, support 15kHz, 30kHz, …, 1.44MHz, 2.88MHz for 1SB waveform. For 2SB waveform two times of 1SB signal is assumed. Proposal 26: RAN4 performs coexistence study considering outdoor multi-cell environment with inter-cell interference. |
| NTT DOCOMO | Proposal 1: For LLS assumptions for coverage evaluation in Rel-20, Rel-19 LLS assumptions for device 2b in Table 4.2.2-1 of TR38.769 should be the baseline. Proposal 2: For coverage evaluation in Rel-20, UMa NLOS pathloss model should be assumed for both R2D/D2R. Proposal 3: For LLS assumptions for coverage evaluation in Rel-20, TDL-C with 300ns delay spread should be assumed for both R2D/D2R. Proposal 4: For LLS assumptions for coverage evaluation in Rel-20, 1kbps and 5-7kbps can be kept as mandatory value for reference data rate. Proposal 5: For LLS assumptions for coverage evaluation in Rel-20, discuss whether the assumption on SFO for device C should be the same as device 2b or can be smaller. Proposal 6: For LLS assumptions for coverage evaluation in Rel-20, OOK-4 should be assumed for R2D modulation. M value can be 2 or 1 as baseline and other values are up to company’s report. Proposal 7: For LLS assumptions for coverage evaluation in Rel-20, Manchester coding specified in Rel-19 should be assumed for R2D line coding scheme. Proposal 8: For LLS assumptions for coverage evaluation in Rel-20, neither channel coding nor repetition should be assumed for R2D as baseline. Proposal 9: For LLS assumptions for coverage evaluation in Rel-20, Manchester coding or no line coding with square wave modulation as specified in Rel-19 should be assumed for D2R. Proposal 10: For LLS assumptions for coverage evaluation in Rel-20, TBCC specified in Rel-19 should be assumed for D2R as baseline. Proposal 11: For LLS assumptions for coverage evaluation in Rel-20, whether to apply block-level repetition for D2R can be up to company depending on the corresponding reference data rate. Proposal 12: For LLS assumptions for coverage evaluation in Rel-20, updates in red in Table 1 should be applied to Table 4.2.2-1 in TR38.769. Proposal 13: For link budget template for coverage evaluation in Rel-20, Rel-19 link budget for device 2b in Table 4.3.2-1 of TR38.769 should be the baseline. Proposal 14: For link budget template for coverage evaluation in Rel-20, total transmission power for BS in R2D should be 33dBm. Proposal 15: For link budget template for coverage evaluation in Rel-20, total transmission power for A-IoT device in D2R should be - -20dBm and -10dBm for device 2b - -3dBm and 5dBm for device C Proposal 16: For link budget template for coverage evaluation in Rel-20, transmission antenna gain for BS in R2D should be [17] dBi assuming 2 Tx antenna elements / TxRU and 8dBi antenna element gain. Proposal 17: For link budget template for coverage evaluation in Rel-20, receiver sensitivity for both R2D/D2R should be based on Budget-Alt2 in TR38.769. Proposal 18: For link budget template for coverage evaluation in Rel-20, shadow fading margin for both R2D and D2R can be 6 dB or 7dB if O2I scenario. Proposal 19: For link budget template for coverage evaluation in Rel-20, updates in red in Table 2 should be applied to Table 4.3.2-1 in TR38.769. |
| IIT Kanpur | Proposal 1: Use the link budget template defined in Rel-19 as the baseline for coverage evaluation in Rel-20. Proposal 2: Use at least UMa NLOS and RMa LOS pathloss models for the Rel-20 A-IoT study of outdoor deployment scenarios. Proposal 3: For studying the outdoor deployment scenarios, update to the Rel-19 link budget should be made to incorporate the evaluation assumptions for outdoor deployment including deployment layout, UE distribution, and additional BS-related parameters such as totasl Tx power, Tx/Rx antenna gain, number of antenna elements, and number of TXRUs. Proposal 4: Study the applicable maximum distance target values for Device 2b and Device C separately, within the range of 50–500 m, while considering the transmit power difference and receiver design aspects. Proposal 5: Use the Rel-19 link budget alt 2 for coverage evaluation in Rel-20 of the active device types with IF/ZIF receiver. Proposal 6: For the Rel-20 study on active device types, reuse the link budget parameters for IF/ZIF receivers assumed in Rel-19. Proposal 7: For link-level performance evaluation, use the TDL-C (DS-300 ns) and TDL-D (DS-100ns) channel profiles as defined in TR 38.901. |

Discussion

Based on inputs from papers, companies are mainly focusing on evaluation assumptions for this meeting. Most companies express views in their papers that the link budget template and LLS table in Rel-19 can be used as starting point for Rel-20 study with adapting to active device and outdoor scenarios. FL plans to discuss link budget and LLS assumptions in this way:

- Highlight in red texts on the potential update of items/values in link budget template table and LLS assumptions table for companies to see things to be discussed. (See in the end of section 3.3 and 3.4)

- Pick those things in red texts to be discussed and make corresponding individual FL proposals in section 3.3 and 3.4

- Once agreements achieved after discussion on those individual FL proposal, later in this week, FL will try to update the link budget template table and LLS assumptions table accordingly and ask if RAN1 could agree updated tables for further evaluation work.

In addition, FL has the following observations:

- For general aspects of evaluation, companies are mainly talking about the followings which can be found in section 3.1,

  - Rx sensitivity alternatives

  - Evaluation methodology

  - Applicable maximum distance

  - Others

- For necessary further assumptions of deployment scenarios for coverage and coexistence evaluations, companies are mainly talking about the followings which can be found in section 3.2,

  - Pathloss model

  - Deployment assumptions

  - Others

- For link budget template, some companies give detail updates on items and values in their papers. Companies are mainly talking about updates on the followings which can be found in section 3.3,

  - BS and Device Tx power

  - BS and Device antenna gain

  - BS and Device Rx noise figure

  - Bandwidth assumption

  - Interference density

  - Penetration margin/loss

- For LLS table, some companies give detail updates on items and values in their papers. Companies are mainly talking about updates on the followings which can be found in section 3.4,

  - Channel model and delay spread for outdoor

  - Device velocity

  - Payload size

  - Reference data rate

  - Device SFO/CFO assumptions

  - R2D assumption on FEC/repetition

- For energy harvesting/storage, companies are mainly talking about the followings which can be found in section 3.5,

  - Energy source

  - Energy storage

General

### >>Rx sensitivity alternatives

For Rx sensitivity Budget-Alt1 and Budget-Alt2 defined in R19, FL observes the followings

- 10 companies [Huawei, Ericsson, CATT, vivo, Nokia, Transsion, CMCC, Qualcomm, DCM] propose to use Budget-Alt2 for the Rel-20 study.

- 2 company [FUTUREWEI, Xiaomi] mentioned both Budget-Alt1 and Budget-Alt2.

Given that, FL propose the followings:

### [Open][H] Proposal 3.1a-v1:

Proposal: For Rel-20 study, Budget-Alt2 (as per TR38.769) is used to derive Rx sensitivity in the evaluation for both R2D and D2R.

| Company | Views |
| --- | --- |

### >>Evaluation methodology

FL observes the followings,

- Most companies reuse Rel-19 methodology to do LLS for obtaining required SINR under certain reference data rate assumption, then derives Rx sensitivity according to that required SINR and do link budget calculation. FL copy the following from section 7.1 of TR 38.769

- On the other hand, 2 companies [vivo, ZTE] propose different methodology than Rel-19 which is to get required SNR from link budget by MPL/maximum distance first, then do LLS to determine data rate according to that required SNR.

Thus, FL would like to check views from companies which option is preferred from the followings.

### [Open][H] Proposal 3.1b-v1:

Proposal: For Rel-20 study, RAN1 to decide which of the followings is used to perform coverage evaluation for R2D and D2R:

- Option 1 (as Rel-19):

  - Step 1: Obtain the required SINR by link-level simulations under different reference data rates

  - Step 2: Obtain the receiver sensitivity based on the required SINR if Budget-Alt2 is used

  - Step 3: Obtain the MPL/distance based on the receiver sensitivity and link budget

- Option 2

  - Step 1: Obtain the MPL based on the distance via pathloss model

  - Step 2: Derive the required SINR from link budget according to the MPL

  - Step 3: Determine the data rate corresponding to the required SINR via link-level simulations.

| Company | Views |
| --- | --- |

### >>Applicable maximum distance

Several companies mentioned applicable maximum distance or coverage target, the views are diverse and FL summarize the followings:

- Co-site outdoor BS coverage (ISD based): Huawei

- Typical 3GPP outdoor UE coverage: Samsung (Device C)

- 500m: vivo (starting point), Xiaomi, ZTE (starting point), LGE (at least 500m)

- >500m: Xiaomi (Device C), ZTE

- >50m: Samsung (Device 2b)

- 50-100m: Qualcomm (Device 2b)

- 400-500m: Qualcomm (Device C)

- Separate coverage range for Device 2b and C: Samsung, Qualcomm, Apple

- The maximum distance for each device type can be determined based on evaluation results of the link budget: Transsion

From above, FL observes some companies think Device 2b and Device C should have separate applicable maximum distance target within 500m. Some companies think at least Device C may have applicable maximum distance larger than 500m, or achieve typical 3GPP outdoor UE coverage or co-site deployment should be considered as ISD based maximum distance between reader and device.

### [Open][M] Question 3.1c-v1:

Question: For Rel-20 study, please companies share views on the coverage target in the following table.

- Q1: For outdoor coverage, is the applicable maximum distance target within 50-500m or could it be larger than 500m (e.g. consider co-site deployment and typical 3GPP UE outdoor coverage)?

- Q2: Should the applicable maximum distance target be investigated after seeing the evaluation results?

| Company | Views |
| --- | --- |

### >>Others

FL observes that some companies mentioned to discuss assumptions on active device with ZIF/IF receivers (e.g filter etc.), FL plans to discuss them together with parameters in link budget or LLS table where it will be used in the evaluation.

FL observes 3 companies [Nokia, Xiaomi, OPPO] suggest for Rel-20 study RAN1 could define evaluation scenario based on the update of evaluation scenarios defined in section 4.2.1 of TR38.769. FL understand the proposal from companies are straight forward according to the SID. Thus, FL plans to focus on evaluation assumptions in terms link budget and LLS which would be fundamental for evaluation work.

FL observes that most companies are talking about coverage evaluation, among them 3 companies [CMCC, CTC, LGE] propose MPL/distance is the metric for coverage evaluation in link budget calculation. FL understand MPL and distance is already in the TR and no body propose to change, thus seems no need to re-agree.

In addition, FL noticed that 3 companies [Xiaomi, Samsung, Sony] discuss latency definition while 1 company [Sony] discuss spectral efficiency.

Deployment scenarios

### >>Pathloss model

FL observes companies propose the following pathloss models for outdoor deployment evaluation:

- UMa: 18 companies [FUTUREWEI, Spreadtrum, Huawei, Ericsson, CATT, vivo, Nokia, Xiaomi, CTC, ZTE, Transsion, OPPO, Apple, NEC, CMCC, InterDigital, DCM, IITK]

  - 12 companies indicate NLOS while 1 company indicates LOS

- UMi: 3 companes [FUTUREWEI, Transsion, NEC]

- RMa: 8 companies [Huawei, CATT, vivo, Xiaomi, CTC, ZTE, CMCC, IITK]

  - 5 companies indicate NLOS while 3 company indicate LOS

- SMa: 1 company [Xiaomi]

Based on above, FL propose the followings

### [Open][H] Proposal 3.2a-v1:

Proposal: For Rel-20 study, the following path-loss model is used in the coverage evaluation:

- Option 1: UMa NLOS defined in TR38.901

- Option 2: RMa NLOS defined in TR38.901

| Company | Views |
| --- | --- |

### >>Deployment assumptions

FL observes companies talking in their papers on the followings:

- Companies indicate BS and Device antenna height which will be used in path-loss calculation.

- Many companies indicate outdoor BS layout and ISD in their papers for the evaluation while 1 company [Samsung] propose the layouts for coexistence evaluation can be up to RAN4.

### [Open][H] Proposal 3.2b-v1:

Proposal: For Rel-20 study, the antenna height for BS and Device used for evaluation purpose are as follows:

- BS antenna height

  - UMa-NLOS: 25m

  - RMa-NLOS: 35m

- Device antenna height

  - 1.5m

| Company | Views |
| --- | --- |

FL understands as Samsung pointed out, in the TR38.769 RAN4 already have assumptions on outdoor BS layout when did Rel-19 coexistence work with indoor A-IoT without RAN1 inputs at that time. For the study of Rel-20, RAN1 may take that into consideration when discuss the deployment assumption. FL collect the inputs into the table and further check views from companies.

With above, FL propose the followings and please companies to share your views

### [Open][H] Proposal 3.2c-v1:

Proposal: For Rel-20 study, the following layout assumptions is used for evaluation purpose:

- Hexagonal grid, 19 macro sites, 3 sectors per site

- ISD is considered as follows

  - UMa NLOS: [200m or 500m], 750m

  - RMa NLOS: [1732 m or 5000m]

| Company | Views |
| --- | --- |

### >>Others

A few companies talked coexistence in their papers, FL have the following observations:

- Coexistence cases

  - 1 company [vivo] propose that detailed cases for co-existence evaluation, e.g., NR/AIoT aggressor AIoT/NR victim, etc., is up to RAN4 discussion.

  - 2 companies [Xiaomi, ZTE] propose coexistence cases in their paper.

- Inter-cell interference

  - 1 company [Qualcomm] propose that RAN4 performs coexistence study considering outdoor multi-cell environment with inter-cell interference.

  - 1 company [Xiaomi] propose that RAN1 needs to clarify whether to study the method for interference suppression to provide reasonable assumptions for the coexistence simulation work.

  - 1 company [FUTUREWEI] mentioned with the RF LO and IF filter, an active A-IoT device can also makes it possible coexistence with other 5G/NR system(s) on downlink.

  - 1 company [Samsung] propose to study the feasibility and appropriate assumptions for R2D narrowband filtering for coverage and coexistence evaluations.

FL understands the coexistence cases were not determined in RAN1 and up to RAN4. For the second point FL plans to discuss narrowband filtering for Rx bandwidth in link budget and LLS assumptions.

Link budget

Companies express views in their papers that the link budget template used in Rel-19 (as per TR 38.769) can be used as starting point for Rel-20 study with adapting to active device and outdoor scenarios. FL plans to discuss those key adapting parts from inputs first, then try to formulate the link budget template table later in this meeting for company further checking the update.

### >>BS and Device Tx power

For BS Tx power, FL observes the followings from inputs

- 49dBm: Qualcomm

- 46dBm: Spreadtrum, CATT, vivo, Nokia, Xiaomi, OPPO, CMCC

- 44dBm: Qualcomm

- 43dBm: vivo, ZTE (for standalone deployment), Apple

- 38dBm: Huawei, Xiaomi, LGE, NEC(>=38dBm), CMCC

- 36dBm/MHz: Ericsson

- 36+XdBm/MHz where X is 6dB for A-IoT signal and 0dB for NR: Qualcomm

- 33dBm: Xiaomi, ZTE (for in-band deployment), InterDigital, DCM

- 24dBm: Xiaomi, InterDigital

- Xiaomi propose that RAN1 needs to clarify whether the transmission power of the A-IoT reader is independent of the transmission power of the NR BS.

FL notices that:

- The values companies provided in wide range and some companies mentioned in-band and standalone case may have different BS Tx power.

- Most companies propose Tx power in dBm while 2 companies propose power spectral density dBm/MHz

  - Note that for 36dBm/MHz which contains maximum 4PRBs and approximately equivalent to ~30dBm/1PRB and ~46dBm/50PRBs

  - Note that for 36+6dBm/MHz power boosting which contains maximum 4PRBs and approximately equivalent to ~36dBm/1PRB and ~46dBm/10PRBs

Thus, based on above, FL propose the followings with understanding at least two values are needed for in-band and standalone deployment respectively. FL would like companies to share views which value(s) to pick.

### [Open][H] Proposal 3.3a-v1:

Proposal: For Rel-20 study, BS Tx power assumption for evaluation purpose is from the followings:

- For standalone: [46, or 43] dBm

- For in-band:

  - Option 1: [38, or 33] dBm

  - Option 2: (36+X) dBm/MHz

    - X = [0 or 6] dB

| Company | Views |
| --- | --- |

For Device Tx power, FL observes the followings from inputs

- Device 2b

  - -10dBm: Spreadtrum, CATT, vivo, Xiaomi, LGE, Apple, CMCC, DCM

  - -20dBm: CATT, Xiaomi, Apple, CMCC, DCM

- Device C

  - 5dBm: Ericsson, Xiaomi, LGE, Apple, CMCC, DCM

  - 4dBm: vivo

  - 0dBm: Spreadtrum, Ericsson, CATT, Apple, CMCC

  - 0-5dBm: Huawei

  - [-3, 5]dBm: ZTE

  - -3dBm: Ericsson, Xiaomi, OPPO, Apple, DCM

Thus, FL propose the followings based on the inputs.

### [Open][H] Proposal 3.3b-v1:

Proposal: For Rel-20 study, Device Tx power assumption for evaluation purpose is from the followings:

- Device 2b: {-10, -20} dBm

- Device C: {-3, 0, 5} dBm

| Company | Views |
| --- | --- |

### >>BS and Device antenna gain

For BS antenna gain, FL observes the followings:

- 17dBi: Huawei, Apple, Qualcomm (based on 16 elements, 2 Tx RU), DCM (2*8)

- 15dBi: CMCC

- 8dBi: Ericsson, CATT, vivo, Nokia, OPPO

- 6dBi: Interdigital

FL understands that the main difference between large value and small value may be from BS antenna gain model. As per Qualcomm and DoCoMo mentioned the proposed 17dBi gain including 16 elements (i.e. 8 elements per Tx RU). It will similar as those proposed 8dBi but need to set 8 elements per Tx RU in link level simulation. FL understands in that sense, they should be same in principle but results in two different options to do the evaluation. Thus, FL propose the followings:

### [Open][H] Proposal 3.3c-v1:

Proposal: For Rel-20 study, BS antenna gain assumption in link budget for evaluation purpose is from the followings:

- Option 1: 17dBi

  - This assumes in the LLS table (as per TR38.769), the values in “[0h1] Number of antenna elements” and “[0h2] the Number of TXRUs” would be same (as Rel-19)

- Option 2: 8dBi

  - This assumes in the LLS table (as per TR38.769), the value in “[0h1] Number of antenna elements” would be 8 times of the value in “[0h2] the Number of TXRUs”

| Company | Views |
| --- | --- |

For Device antenna gain, FL observes most companies remain to use 0 dBi in their papers while 1 company Qualcomm propose -3dBi instead.

- 0dBi: Huawei, FUTUREWEI, Spreadtrum, Ericsson, CATT, Xiaomi, ZTE, Tejas, CMCC, InterDigital, Apple

- -3dBi: Qualcomm

Thus, FL propose the following,

### [Open][H] Proposal 3.3d-v1:

Proposal: For Rel-20 study, Device antenna gain assumption in link budget for evaluation purpose is 0dBi for both Device 2b and C.

| Company | Views |
| --- | --- |

### >>BS and Device Noise figure

For BS noise figure, FL observes that most companies use 5dB in their papers while 1 company [Huawei] uses 3dB. Thus, FL propose the following,

### [Open][H] Proposal 3.3e-v1:

Proposal: For Rel-20 study, BS noise figure assumption in link budget for evaluation purpose is 5dB.

| Company | Views |
| --- | --- |

For Device noise figure, FL observes that some companies use 15dB for both Device 2b and C. Some companies think the noise figure for Device 2b can be larger than 15dB (although in TR 38.769, Device 2 is assumed to be 15dB). Some companies think the noise figure for Device C can be better than Device 2b. Based on the values mentioned from papers, FL propose the following,

### [Open][H] Proposal 3.3f-v1:

Proposal: For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose is from the followings:

- Option 1:

  - 15dB for both Device 2b and Device C

- Option 2:

  - Device 2b: [15, 18, 15-20, or 40] dB

  - Device C: [10-15, 15, or 30] dB

| Company | Views |
| --- | --- |

### >>Bandwidth assumption

FL understand the following bandwidth will be used in evaluation for link budget and LLS:

- R2D Tx bandwidth

- D2R Tx bandwidth

- R2D Rx bandwidth

- D2R Rx bandwidth

For R2D Tx bandwidth, FL observes the following values discussed in papers:

- 180kHz: Huawei, FUTUREWEI, Spreadtrum, vivo, Tejas, CMCC, InterDigital, Ericsson, Xiaomi, ZTE, CATT, Qualcomm, Apple

- 360kHz (O): Ericsson, vivo, ZTE

- 540kHz (O): vivo, ZTE

- M*180kHz: CATT

- 180kHz ~ 4.32MHz: Qualcomm

- 1.08MHz: Ericsson, Xiaomi

Thus, FL propose the followings

### [Open][H] Proposal 3.3g-v1:

Proposal: For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 180kHz as starting point

  - Other values of multiple 180kHz are not precluded and up to company report

| Company | Views |
| --- | --- |

For R2D Rx bandwidth, FL observes the following values discussed in papers:

- Many companies use 180kHz as ED bandwidth or BB LPF bandwidth

- CMCC propose the Rx BW the BB LPF reported by companies in assumptions for LLS can be referred as receiver bandwidth for calculation of noise power for R2D in link budget template.

- Qualcomm propose the for [2B]-R2D Bandwidth used for the evaluated channel, companies to report rx bandwidth considering tx signal bandwidth, Rx LO uncertainty, and BB LPF bandwidth.

FL understands as Qualcomm mentioned, due the Rx LO uncertainty of active device, ZIF/IF receiver filter BW will be impact by e.g. CFO ppm assumption. Thus, FL propose

### [Open][H] Proposal 3.3h-v1:

Proposal: For Rel-20 study, R2D receiver bandwidth assumption in link budget and LLS for evaluation purpose is up to company report with considering:

- R2D transmission bandwidth

- Device Rx LO uncertainty (e.g. CFO ppm assumption)

- Note: ED bandwidth and BB LPF bandwidth in LLS are assumed to be same as R2D receiver bandwidth in link budget for active device with ZIF/IF receiver

| Company | Views |
| --- | --- |

For D2R Tx bandwidth, FL observes the followings discussed in papers:

- Many companies use same assumption as Rel-19 as per TR 38.769

- 1 company Huawei propose to use 10kHz instead of 15kHz

- 1 company Ericsson propose 180kHz (M) and 15kHz (O)

- 1 company Qualcomm propose to use values 15kHz, 30kHz, … 2.88MHz corresponding to Rel-19 D2R chip/bit duration and change to SSB bandwidth.

FL propose the following,

### [Open][H] Proposal 3.3i-v1:

Proposal: For Rel-20 study, D2R transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings:

- 15kHz as starting point

  - Other values are not precluded and up to company report

  - Companies to report which of DSB or SSB modulation is assumed for D2R transmission bandwidth

| Company | Views |
| --- | --- |

For D2R Rx bandwidth, it refers to LLS and describe as company to report. Thus, FL think may be only need to clarify the Rx bandwidth needs to consider Tx LO uncertainty compared to Rel-19. Thus, FL will not make an individual proposal here and the update can be found in LLS table update.

### >>Interference density

4 companies [Huawei, CMCC, Xiaomi, Qualcomm] discussed interference/multi-cell interference while among them 2 companies [Huawei, CMCC] propose to add a row/item into link budget template as receiver interference density which is similar as TR38.830 or TR38.910 for outdoor coverage evaluation. In addition, CMCC propose values of 0dB or XdB up to company report (FL understands this would require further clarify from CMCC the meaning of proposed value since the unit of interference density is dBm/Hz rather relative dB). Huawei propose to reuse values from TR 37.910 which is -169.3dBm/Hz for R2D and -165.7dBm/Hz for D2R.

Thus, FL propose the followings

### [Open][H] Proposal 3.3j-v1:

Proposal: For Rel-20 study, add a new item of “Receiver interference density (dBm/Hz)” into the link budget template and the value of X dBm/Hz for evaluation purpose is from the followings:

- For R2D, X = {-169.3, other value(s)}

- For D2R, X = {-165.7, other value(s)}

| Company | Views |
| --- | --- |

### >>Penetration margin

4 companies [Huawei, Xiaomi, ZTE, Apple] propose to consider penetration loss/margin for outdoor scenario coverage evaluation and hence propose to add a new item to link budget template. Among of them, 1 company Xiaomi propose company to report with justification and other 3 companies propose details values. ZTE and Apple propose 0 or 20 dB while Huawei propose 26.25 for UMa NLOS, 17.46 or 25 for RMa NLOS. Note that companies indicate these values mentioned in their papers are based on TR38.901.

Thus, to simplify the discussion, FL propose the following based on the collection of values provided from companies.

### [Open][H] Proposal 3.3k-v1:

Proposal: For Rel-20 study, add a new item of “Penetration margin (dB)” into link budget template and the value of X dB for evaluation purpose is from the followings:

- For both R2D and D2R

  - X = {0, 15, 20, 25} dB

| Company | Views |
| --- | --- |

### >>Link budget template

For information at this time, link budget template table is assumed to be updated as follows according to the discussion in above sections,

### [Open][M] Proposal 3.3l-v1:

Proposal: For Rel-20 study, “Link budget template” is expected to be as followings,

Link budget template for Rel-20 study

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | Deployment scenario 4 with topology 1 with no external carrier wave | Deployment scenario 4 with topology 1 with no external carrier wave |
| [0B] | Device 2b/C | Device 2b/C | Device 2b/C |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Pathloss model | To be discussed | To be discussed |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: 2(M) or 4(O) antenna elements for 0.9 GHz | 1 |
| [1E] | Total Tx Power (dBm) | To be discussed | To be discussed |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) To be discussed | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | To be discussed | For A-IoT device, 0dBi |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0.9dB or 4.7dB |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth To be discussed | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For IF/ZIF receiver 15dB, Device 2 Other values can be optionally used and reported by companies To be discussed | For BS as reader: 5dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [[2E1]] | [Receiver interference density (dBm/Hz)] | [To be discussed] | [To be discussed] |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1/ Budget-Alt2 (see note1) To be discussed | Budget-Alt2 |
| [2L] | Receiver Sensitivity (dBm) | To be discussed | To be discussed |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | To be discussed | To be discussed |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| [[3E]] | [Penetration margin (dB)] | [To be discussed] | [To be discussed] |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

FL notes: Calculated details in Note 1 will be updated later accordingly. Above those cells which contain red highlighted texts will be discussed in section 3.3 with individual proposals and that cell will be updated once corresponding agreement achieved. Please companies kindly indicate whether you are fine or have comments with the remaining cells which only have black texts.

| Company | Views |
| --- | --- |

LLS

Companies express views in their papers that the LLS assumption table used in Rel-19 (as per TR 38.769) can be used as starting point for Rel-20 study with adapting to active device and outdoor scenarios. FL plans to discuss those key adapting parts from inputs first, then try to formulate the LLS assumption table later in this meeting for company further checking the update.

### >>Channel model and delay spread

For channel model in link level simulation, FL observes the followings:

- TDL-A: InterDigital, Qualcomm (O)

- TDL-B: FUTUREWEI

- TDL-C: Spreadtrum, Huawei, Ericsson, CATT, vivo, Nokia, CTC (for NLOS), OPPO, LGE, CMCC, Qualcomm (M), DCM, IITK

- TDL-D: CTC (for LOS), Qualcomm (for LOS), IITK

- TDL-E: CATT

For delay spread in link level simulation, FL observes the followings:

- 100ns: FUTUREWEI, IITK (TDL-D)

- 150ns: Qualcomm (TDL-A/D)

- 300ns: Huawei, Ericsson, CATT (TDL-C), vivo, Nokia, OPPO, CMCC, Qualcomm, DCM, IITK (TDL-C)

- 30ns: CATT(TDL-E), InterDigital (TDL-A), Qualcomm (Option for TDL-A/D)

Based the inputs, FL propose the following,

### [Open][H] Proposal 3.4a-v1:

Proposal: For Rel-20 study, channel model TDL-C with an RMS delay spread of 300 ns delay spread is used for link level simulation.

| Company | Views |
| --- | --- |

### >>Reference data rate

FL observes the followings companies used in their papers:

- Reuse Rel-19: Spreadtrum, Xiaomi, CMCC, NTT DOCOMO, CATT, InterDigital

- 0.1-1kbps: Huawei

- 5-7kbps: Ericsson, DoCoMo

- 50kbps: Ericsson

- >100kbps: CATT

- 7, 14, 42, 84, and 168kbps: Qualcomm

- 1kbps-640kbps: FUTUREWEI

FL copies existing values from TR38.769 here

FL propose the followings,

### [Open][H] Proposal 3.4b-v1:

Proposal: For Rel-20 study, reference data rate in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether to add additional larger value(s)

| Company | Views |
| --- | --- |

### >>Message size

2 companies [Nokia, Xiaomi] propose to add 800 bits in addition to the 3 values of 20 bits, 96 bits and 400 bits in current TR 38.769.

### [Open][H] Proposal 3.4c-v1:

Proposal: For Rel-20 study, message size in LLS assumption for evaluation purpose is from the followings:

- Reuse the values in Rel-19 (as per TR 38.769) as starting point

  - FFS whether needs further update to add additional value of 800 bits.

| Company | Views |
| --- | --- |

### >>Device CFO/SFO

For CFO and SFO assumption for LLS assumption, FL observes the followings

  - Device 2b:

    - Initial SFO:

      - R19: Spreadtrum, Huawei, CATT, Qualcomm

      - 10^4 ppm: Nokia

    - After SFO:

      - R19: Spreadtrum, Huawei, Ericsson, Qualcomm

      - 10s-100s ppm: FUTUREWEI

    - Initial CFO

      - R19: Spreadtrum, Huawei, CMCC

      - 200 ppm: Apple (O), CMCC (O)

      - 100 ppm: Apple (M), CMCC (M)

      - 10^2-10^3 ppm: Qualcomm (M)

      - 10-100 ppm: Qualcomm (O)

    - After CFO

      - R19: Spreadtrum, Huawei

      - 10s-100s ppm: FUTUREWEI

      - 10 ppm: CMCC, Qualcomm (O)

      - 100 ppm: Qualcomm (M)

  - Device C:

    - Initial SFO

      - R19: Spreadtrum, Qualcomm

      - 10^4 ppm: Nokia

      - 10^3 ppm: CATT

      - 50ppm: Huawei

    - After SFO

      - R19: Spreadtrum, Qualcomm

      - 10s-100s ppm: FUTUREWEI

      - 10 ppm: Huawei

      - 100 or 1000 ppm: Ericsson

    - Initial CFO

      - R19: Spreadtrum,

      - 200 ppm: Apple (O), CMCC (O)

      - 100 ppm: Apple (M), CMCC (M)

      - 50ppm: Huawei

      - 80 ppm: CATT

      - 10^2-10^3 ppm: Qualcomm (M)

      - 10-100 ppm: Qualcomm (O)

    - After CFO

      - R19: Spreadtrum

      - 10s-100s ppm: FUTUREWEI

      - 10 ppm: Huawei, CMCC

      - 10 ppm: CMCC, Qualcomm (O)

      - 100 ppm: Qualcomm (M)

Based on the inputs, FL propose the followings,

### [Open][H] Proposal 3.4d-v1:

Proposal: For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings:

- Option 1: Device C is better than Device 2b

| Option 1 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | 10^3~10^4 ppm | [50, 80, 10~100] ppm |
| SFO after clock sync/calibration | 10^3 ppm | [10] ppm |
| Initial CFO | 10^2~10^3 ppm | [50, 80, 10~100] ppm |
| CFO after clock sync/calibration | 100 ppm | [10] ppm |

- Option 2-1: Device C and Device 2b are same (with large ppm)

| Option 2-1 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | 10^3~10^4 ppm | 10^3~10^4 ppm |
| SFO after clock sync/calibration | 10^3 ppm | 10^3 ppm |
| Initial CFO | 10^2~10^3 ppm | 10^2~10^3 ppm |
| CFO after clock sync/calibration | 100 ppm | 100 ppm |

- Option 2-2: Device C and Device 2b are same (with small ppm)

| Option 2-2 | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | [50, 80, 10~100] ppm | [50, 80, 10~100] ppm |
| SFO after clock sync/calibration | [10] ppm | [10] ppm |
| Initial CFO | [50, 80, 10~100] ppm | [50, 80, 10~100] ppm |
| CFO after clock sync/calibration | [10] ppm | [10] ppm |

| Company | Views |
| --- | --- |

### >>FEC and repetition

For R2D FEC, some companies e.g. Ericsson, DoCoMo propose no FEC as baseline, some other companies e.g. CMCC and Huawei propose companies to report FEC. FL propose the followings,

### [Open][H] Proposal 3.4e-v1:

Proposal: For Rel-20 study, R2D FEC in LLS assumption for evaluation purpose is from the followings:

- Company to report no FEC or using FEC in their evaluations

| Company | Views |
| --- | --- |

For R2D repetition, some companies e.g. Ericsson, DoCoMo propose no repetition as baseline, some other companies e.g. vivo, Huawei propose to use repetition. Ericsson propose to add a new row for R2D repetition. FL understand in Rel-19 D2R already support repetition but missing in LLS table. FL propose the followings,

### [Open][H] Proposal 3.4f-v1:

Proposal: For Rel-20 study, add new item of “Repetition” for both R2D and D2R into LLS assumptions for evaluation purpose

- Company to report no repetition or using repetition in their evaluations

| Company | Views |
| --- | --- |

### >>LLS assumptions table

For information at this time, LLS assumption table is assumed to be updated as follows according to the discussion in above sections,

### [Open][M] Proposal 3.4g-v1:

Proposal: For Rel-20 study, “Coverage evaluation assumptions for link-level simulation” is expected to be as followings,

Coverage evaluation assumptions for link-level simulation for Rel-20 study

| No. | Parameters |  | Assumptions |
| --- | --- | --- | --- |
| R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |
| [0b] | SCS |  | 15 kHz as baseline |
| [0c] | Block structure |  | Blocks as agreed in Rel-19 as starting point, other blocks reported by companies |
| [0d] | Channel model |  | To be discussed |
| [0e] | Delay spread |  | To be discussed |
| [0f] | Device velocity |  | 3 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 or 4 |
| [0h2] |  | Number of TXRUs | 2 or 4 |
| [0m] | Reference data rate |  | 1 kbps (M) 5 - 7 kbps (M) 48 - 60 kbps (O) 0.1 kbps for message size of 20 bits or 96 bits (O) Other data rates can be reported by companies Note 1: Companies to report the exact data rate. Note 2: The exact data rate is close to the values listed above. Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads(e.g., CRC, pre/mid/post-ambles if present). Note 4: The exact data rate may be related to coding scheme, repetition and etc. Note 5: All data rates considered are for evaluation purpose only To be discussed |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size To be discussed |
| [0p] | BLER target |  | 1%, 10% |
| [0q] | Sampling frequency |  | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. CFO for device 2b: 100 ppm (M) 200 ppm (O) 1000 ppm (O, only as initial CFO) Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only To be discussed |
| [0r] | Device 2b/C |  | ZIF/IF-ED |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz as baseline. Other larger values are not precluded. |
| [1b] | ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. To be discussed |
| [1c] | BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth. Companies to report X = {3, 5}. To be discussed |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-4 Companies to report M chips per OFDM symbol |
| [1f] | Line code |  | Manchester |
| [1g] | FEC |  | No FEC as baseline To be discussed |
| [1h] | ADC bit width |  | 4-bit |
| [1j] | Detection/decoding method for Line code |  | Companies to report |
| [[1k]] | [Repetition] |  | [To be discussed] |
| D2R specific parameters |  |  |  |
| [2a1] | Transmission bandwidth |  | [2a1]-Alt1 (M): DSB X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz [2a1]-Alt2: SSB X kHz is considered for D2R transmission bandwidth. The value is for one sideband, i.e., the total transmission bandwidth for SSB is X kHz. Not applicable for at least device 1 X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies To be discussed |
| [2a2] | Chip rate |  | Companies to report |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value, and further down-selection of the values and DSB/SSB is not precluded. |
| [2b] | Waveform (CW) |  | RF LO |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, no line coding |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit, 14-bit |
| [2j] | D2R receiver |  | Companies to report, e.g., coherent receiver / non-coherent receiver |
| [[2k]] | [Repetition] |  | [To be discussed] |
|  | Other assumptions |  |  |
| [3a] | Other assumptions |  | To be reported by company |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |

FL notes: Above those cells which contain red highlighted texts will be discussed in section 3.4 with individual proposals and that cell will be updated once corresponding agreement achieved. Please companies kindly indicate whether you are fine or have comments with the remaining cells which only have black texts.

| Company | Views |
| --- | --- |

Energy harvesting/storage

7 out of [23] companies discuss energy harvesting/storage in their papers. FL observes the followings main interests on assumptions:

- Energy sources

  - RF energy source: 2 companies [OPPO, Qualcomm]

  - Other energy source than RF: 5 company [FUTUREWEI, Huawei, vivo, CMCC, Qualcomm]

    - e.g., solar/light, vibration, thermal, etc.

- Energy storage

  - In addition to capacitor, 1 company [Sony] also discuss small rechargeable battery.

  - Companies discuss energy storage size in different volumes

    - e.g. tens of uF, <=200μJ, ~200mF

  - 1 company [ZTE] propose that for energy storage, the maximum PRDCH reception time and the maximum PDRCH transmission time that Device 2b/C supports can be assessed and the above assessment methodology can be considered as a starting point.

  - 1 company [Qualcomm] discuss data rate and energy consumption/storage for coverage evaluation.

  - 1 company [vivo] propose that TR captures a conclusion that the reported data rates can be supported by energy sources with high energy conversion efficiency, e.g., solar, vibration, thermal, etc., with sufficient energy storage, which is up to implementation.

Based on above inputs, FL propose the followings,

### [Open][M] Proposal 3.5a-v1:

Proposal: For Rel-20 study, RAN1 assumes companies to report energy source/storage assumptions with their evaluations.

| Company | Views |
| --- | --- |

Evaluation results

FL observes some companies have preliminary results as early inputs in their paper for this meeting. FL will not process evaluation results at this moment. Once evaluation assumptions are going to be determined, FL will kick off results collection for companies to fill into and update their results in future meetings.

Offline/Online proposals

Online - Monday afternoon

RAN1 agreements achieved during Rel-20 SI

- RAN1#122 (Bengaluru)

Reference

- TR 38.769	Study on solutions for ambient IoT (Internet of Things)

- TS 38.291		Ambient IoT Physical layer

- RP-251884	New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

- R1-2505154	Discussion on Evaluation Methodology for R20 A-IoT	FUTUREWEI

- R1-2505166	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum, UNISOC

- R1-2505212	Evaluation for active A-IoT device in outdoor scenario	Huawei, HiSilicon

- R1-2505246	Evaluations	Ericsson

- R1-2505304	Evaluation methodology for A-IoT outdoor deployment scenarios	CATT

- R1-2505410	Evaluation methodologies, assumptions and results for R20 AIoT	vivo

- R1-2505423	Initial views on Evaluation Methodology for Rel-20 AIoT	Nokia

- R1-2505457	Discussion on evaluation methodology for Ambient IoT in NR outdoor for active devices	Xiaomi

- R1-2505513	Discussion on evaluations for Ambient IoT	China Telecom

- R1-2505578	Evaluation methodology and assumptions for Rel-20 Ambient IoT	Samsung

- R1-2505603	Discussion on evaluation of active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2505615	Discussion on evaluation for Ambient IoT	Transsion Holdings

- R1-2505751	Discussion on EVM for R20 A-IoT	OPPO

- R1-2505793	Evaluation assumptions for outdoor Ambient IoT 	Tejas Network Limited

- R1-2505852	Evaluations for Rel-20 Ambient IoT SI	LG Electronics

- R1-2505907	On Rel-20 Ambient IoT evaluation assumptions	Apple

- R1-2505923	Evaluations for Ambient IoT	NEC

- R1-2506091	Discussion on evaluation assumptions	CMCC

- R1-2506112	Ambient IoT evaluations for outdoor active devices	Sony

- R1-2506158	Evaluations for Active Device in Ambient IoT	InterDigital, Inc.

- R1-2506212	Evaluations	Qualcomm Incorporated

- R1-2506299	Study on evaluations for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2506373	Discussion on evaluation methodology for Rel-20 AIoT	IIT Kanpur