3GPP TSG RAN WG1 #122		R1-2506576

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	10.2.2

Source: 	Moderator (MediaTek)

Title: 	Moderator summary on enhancing DL CSI acquisition (Round 2)

Document for:	Discussion and Decision

- Introduction

In RAN#108, Rel-20 WID for NR MIMO Phase 6 (RP-251856) is approved. According to the approved WID, the detailed scope in this agenda item (AI 10.2.2) includes the following objective:

2. On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

- Plan

Based on the contributions from companies [1]-[36], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – Early SRS/CSI/CSI-RS triggering

    - Issue 1.1 – Transition from IDLE/INACTIVE to CONNECTED mode

    - Issue 1.2 – SCell transition from deactivated to activated state

    - Issue 1.3 – SCell transition from dormant to non-dormant BWP

  - Issue 2 – CSI-RS frequency-domain density reduction

- Moderator’s observations and recommended proposals based on the companies’ views on each issue

This summary will be used for our 3rd online discussion on Thursday (08:30 local time). Please upload your inputs to the draft folder, if any, by 06:30 local time.

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| MediaTek | Rebecca Chen (Moderator) | rebecca.chen@mediatek.com |
| MediaTek | Darcy Tsai | Darcy.Tsai@mediatek.com |
| Qualcomm | Jing Dai | jindai@qti.qualcomm.com |
| InterDigital | Afshin Haghighat | Afshin.Haghighat@InterDigital.com |
| Samsung | Youngrok Jang | yr.jang@samsung.com |
| Samsung | Dattaraj Dileep Raut Mulgaonkar | dileep.raut@samsung.com |
| Nokia | Sanjay Goyal | Sanjay.goyal@nokia.com |
| Nokia | Mihai Enescu | Mihai.enescu@nokia.com |
| vivo | Tamrakar Rakesh | rakesh@vivo.com |
| vivo | Kaili Zheng | kaili.zheng@vivo.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| Apple | Haitong Sun | haitong_sun@apple.com |
| ETRI | Wooram Shin | w.shin@etri.re.kr |
| ETRI | Woncheol Cho | woncheol@etri.re.kr |
| Futurewei | Weimin Xiao | weimin.xiao@futurewei.com |
| Futurewei | Zhigang Rong | zrong@futurewei.com |
| Sony | Jose Flordelis | jose.flordelis@sony.com |
| LG | Seongwon Go | sw.go@lge.com |
| LG | Hyungtae Kim | ht.kim@lge.com |
| TCL | Didi Zhang | didi.zhang@tcl.com |
| TCL | Minqiang Zou | mingqiang.zou@tcl.com |
| CEWiT | Pardh | pardhasarathy.j@cewit.org.in |
| CEWiT | Goutham | gouthamjutke@cewit.org.in |
| Spreadtrum | Dawei Ma | Dawei.ma@unisoc.com |
| Spreadtrum | Yu Yang | Yu.Yang2@unisoc.com |
| Rakuten | Duc To | duc.to@rakuten.com |
| Sharp | Taro | kumamoto.taro@mail.sharp |
| Sharp | Taka | fukui.takahisa@mail.sharp |
| NTTDOCOMO | Yuki Matsumura | yuuki.matsumura.vz@nttdocomo.com |
| NTTDOCOMO | Naoya Shibaike | naoya.shibaike.eg@nttdocomo.com |
| NTTDOCOMO | Jing Wang | wangj@docomolabs-beijing.com.cn |
| NTTDOCOMO | Teppei Otsuka | teppei.ootsuka.vy@nttdocomo.com |
| Google | Alex Liou | alexliou@google.com |
| Lenovo | Bingchao Liu | liubc2@lenovo.com |
| Lenovo | Mahdi Barzegar | mkhalilsarai@lenovo.com |
| NEC | Yukai Gao | gao_yukai@nec.cn |
| NEC | Peng Guan | guan_peng@nec.cn |
| Fujitsu | Xin Wang | wangxin@fujitsu.com |
| Huawei | Fanbo | asen.fanbo@huawei.com |
| Huawei | Yubo Yang | Yangyubo1@huawei.com |
| NICT | Kenichi Takizawa | takizawa@nict.go.jp |
| ZTE | Guangyu Jiang | jiang.guangyu@zte.com.cn |
| ZTE | Ling Yang | yang.ling17@zte.com.cn |
| AT&T | Ahmed Hindy | ahmed.hindy@att.com |
| AT&T | Salam Akoum | salam.akoum@att.com |
| Tejas | Sheeba Kumari M | sheebak@tejasnetworks.com |
| IIT Kanpur | Robin Singh | robins@iitk.ac.in |

- Proposals to be discussed in the 3rd online session

(Offline Consensus) Proposal 2.1B:

CSI-RS frequency-domain density ρ = 1/3 and 1/6 can be configured to the K NZP CSI-RS resources at least for the following cases:

  - K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports

FFS: K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports

Note: It’s not precluded that the frequency-domain density configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation can be different

Proposal 2.1C:

CSI-RS frequency-domain density ρ = 1/8 can be configured to the K NZP CSI-RS resources at least for the following cases:

  - K=4 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports

  - K=4 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 128 CSI-RS ports

  - K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports

  - K=2 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports

FFS: K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports

Note: It’s not precluded that the frequency-domain density configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation can be different

Not support: ZTE (concern the cases with K=2), ETRI (concern the cases with K=2), Qualcomm (concern the cases with K=2)

Proposal 1.1.3:

For UE transition from IDLE to CONNECTED mode, study the following three options for Step-1 and Step-2:

- Option-1: NW can provide the resource/report configuration in SIBx based on only one UE capability assumption, and UE can report through MSG3 whether the resource/report configuration received in SIBx is supported.

- Option-2: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 which resource/report configuration(s) received in SIBx is/are supported.

- Option-3: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 the supported capability(s) of early SRS/CSI/CSI-RS triggering (e.g., whether to support this feature, max number of CSI-RS ports, xTyR for SRS-AS, max bandwidth of the CSI-RS/SRS-AS etc.).

Note: It’s not precluded to support more than one options depending on the use cases (e.g., early SRS-AS, early CSI-RS for tracking, and CSI-RS for CSI, early aperiodic CSI reporting)

FFS: Whether the UE capability assumption(s) is predefined in the specification or is up to NW implementation

Note: The term “capability” or “UE capability” above does not mean legacy RRC based UE capability.

Proposal 1.2.1A:

For SCell transition from deactivation to activation, support aperiodic SRS-AS transmission on a SCell, triggered via based on legacy SCell activation command activating the SCell.

- FFS: Timeline of the aperiodic SRS-AS transmission, except RAN4-related issue(s)

- FFS: Whether/how to enhance the Rel-15 and/or Rel-17 SCell activation MAC-CE

FL note: Whether to support aperiodic SRS-AS “before” SCell activation completion can be a part of timeline issue for the aperiodic SRS-AS transmission.

Proposal 1.2.1B:

For SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell and corresponding aperiodic CSI-RS for CSI on the SCell, triggered via based on legacy SCell activation command activating the SCell.

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI, except RAN4-related issue(s)

- FFS: Whether/how to enhance the Rel-15 and/or Rel-17 SCell activation MAC-CE

Proposal 1.3.1A:

For SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field and resizing the existing DCI field

  - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic SRS-AS transmission, except RAN4-related issue(s)

Proposal 1.3.1B:

For SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy.

- Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field and resizing the existing DCI field

  - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported

- FFS: Triggering mechanism(s)

- FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI, except RAN4-related issue(s)

Proposal 1.1.1C:

For UE transition from IDLE/INACTIVE to CONNECTED mode, down-select one from the following alternatives to support early CSI-RS for tracking:

- Alt1: Support aperiodic CSI-RS for tracking

- Alt2: Support periodic CSI-RS for tracking

- Alt-3: Support both aperiodic and periodic CSI-RS for tracking

Alt1 supported by: China Telecom, LG, Huawei/HiSilicon, Futurewei

Alt2 supported by: OPPO, ZTE,

Alt3 supported by ETRI

Not support all the above alternatives: Qualcomm

Table 0 – Company inputs for above recommended proposals before the 3rd online session

| Company | Input |
| --- | --- |
| ETRI | Proposal 2.1C: We have concerns about including the last two bullets (i.e., the cases with K=2). Samsung stated that CSI-RS power boosting with K=2 can provide performance gain in the last offline, but we doubt its validity when the delay spread is relatively high, as this would diminish the power boosting gain while also incurring inefficiency in rate matching due to legacy ZP CSI-RS. Therefore, we prefer to keep them as FFS at this stage. Proposal 1.1.3: Support Proposal 1.2.1A: Support Proposal 1.2.1B: Support Proposal 1.3.1A: Support. @Mod, for clarification, is it open whether to include SRS request in a DCI indicating switching out of SCell dormancy? Proposal 1.3.1B: Support. @Mod, for clarification, is it open whether to include CSI request in a DCI indicating switching out of SCell dormancy? [Mod] If a DCI format includes SRS/CSI request field in legacy, it is not precluded to use them for triggering SRS/CSI. |
| Futurewei | Proposal 1.1.3: - It should be ‘… study the following three options …’ - For Option-3, suggest adding ‘maximum bandwidth of the CSI-RS/SRS’. Proposal 1.2.1A/B: - We are not sure if ‘deactivated/activated states’ are defined for SCell in the specs. Suggest changing to ‘For SCell transition from deactivation to activation, …’ or simply ‘For SCell activation, …’ - For the FFS on AP TRS triggering, we believe AP TRS is critical to CSI-RS based DL MIMO CSI acquisition and hence the reduced latency for the SCell activation. If it takes a couple of tens of ms to receive P TRS, early CSI enhancements may bring very little benefit. We suggest changing the FFS (at least for Proposal 1.2.1B of CSI reporting) to ‘The same SCell activation command includes a trigger for aperiodic CSI-RS for tracking for the SCell’. [Mod] Captured. Thanks. Proposal 1.3.1A/B: Support. |
| Mod | - Please share your further comment on the proposals provided in Section 4, if any. - Please share your comment on Proposal 1.1.1C |
| OPPO | Re 1.2.1A and 1.2.1B: the 1st FFS on timeline. RAN1 should discuss and decide the timeline, no matter whether RAN4 involes or not. Because the RAN1 spec would need to capture timeline for determining the slot location of SRS, CSI-RS transmisison and CSI reporting. [Mod] Yes, how to determine the transmission slot is in the RAN1 scope. Please check the update to avoid confusion. 1.2.1B: the 2nd FFS should be removed. That seems to discuss the MAC CE design. We only dicuss and decide the function. The design of MAC CE message, including how to enahce the current MAC CE messge is RAN2 work. [Mod] That is the exact intention of 2nd FFS to discuss the functionality only. Please check the update to avoid confusion. 1.1.1C: we support Alt2. The concern on alt1 is trigger a AP TRS transmission during that short period does not provide benefit to the system. Re periodic TRS, we do not need a ‘triggering’ from the MSG4. The UE can start to track the TRS as long as the UE receives the configruaiton of TRS in SIB1. |
| ZTE | Proposal 2.1C: Do NOT support, the combinations of ρ = 1/8 and K = 2 should be further studied. As we pointed before, legacy UE need to perform rate matching based on legacy ZP CSI-RS resource, to prevent data transmission on the REs occupied new low-frequency-density NZP CSI-RS. However, legacy UEs only support ZP CSI-RS resource with frequency density 1 or 1/2. Then a lot of physical resources would be wasted if the combinations of ρ = 1/8 and K = 2 are supported. The following figures gives three examples, - In the first example, the K = 2 NZP CSI-RS resources with frequency density 1/8 are aggregated in an FDM manner, with one RB gap between the two resources. Then legacy UEs need to perform rate matching based on one ZP CSI-RS resource with frequency density 1/2. In 2RBs of every 8RBs, the REs occupied by ZP CSI-RS resource are wasted (no data and no RS is transmitted over these REs). - In the second example, the K = 2 NZP CSI-RS resources with frequency density 1/8 are aggregated in an FDM manner without RB gap between the two resources. Then legacy UEs need to perform rate matching based on one ZP CSI-RS resource with frequency density 1. In 6RBs of every 8RBs, the REs occupied by ZP CSI-RS resource are wasted (no data and no RS is transmitted over these REs). - In the third example, the K = 2 NZP CSI-RS resources with frequency density 1/8 are aggregated in a TDM manner. Then legacy UEs need to perform rate matching based on two ZP CSI-RS resources with frequency density 1. In 6RBs of every 8RBs, the REs occupied by the two ZP CSI-RS resources are wasted (no data and no RS is transmitted over these REs). So, we suggest to put FFS before the two combinations. Proposal 2.1C: CSI-RS frequency-domain density ρ = 1/8 can be configured to the K NZP CSI-RS resources at least for the following cases: - K=4 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports - K=4 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 128 CSI-RS ports FFS: K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports FFS: K=2 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports FFS: K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports Note: It’s not precluded that the frequency-domain density configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation can be different. Proposal 1.1.1C: Support Alt2, i.e., periodic TRS. As periodic TRS is already supported (although is not for early SRS/CSI-RS/CSI triggering), the periodic TRS can be directly reused for tracking in the early SRS/CSI-RS/CSI triggering case. We didn’t see the necessity of introducing a dedicated aperiodic TRS, and it would cause additional RS overhead. Regarding reusing existing periodic TRS, we didn’t see any issues, because we introduced many new report quantities based on already existing periodic TRS in Rel-18 and Rel-19, e.g., TDCP, CJTC-Dd, CJTC-FO. |
| China Telecom | Proposal 1.1.3: Support and prefer Option-2. Proposal 1.1.1C: At least Alt 1 should be supported, Alt 2 needs further discussion. According to the legacy specification of Clause 5.1.6.1 in TS 38.214, a UE could be configured with both aperiodic and periodic CSI-RS for tracking with the same bandwidth (with same RB location) and the aperiodic CSI-RS being configured with qcl-Type set to 'typeA' and 'typeD'. If the aperiodic CSI-RS for tracking is triggered, the associated aperiodic CSI-RS for tracking is valid as well. So we prefer to support Alt.1 as a baseline, and conduct more discussion on whether to support Alt.2 or not. |
| Mod | - Please share your further comment on the proposals provided in Section 4, if any. - Please share your comment on Proposal 1.1.1C - Please check the update to Proposal 1.2.1A and Proposal 1.2.1B |
| LG | Proposal 2.1C: Support. Proposal 1.1.3: OK with the current direction. Proposal 1.2.1A/B: We have similar view as Futurewei that TRS is essential part for CSI-RS based DL CSI acquisition. Rel-20 UE should not be mandated to support Rel-17 aperiodic CSI-RS for tracking for the SCell, since specific Rel-17 feature is not prerequisite for Rel-20 early CSI triggering. Also, support of triggering of CSI-RS for tracking is clearly mentioned in WID. So we propose to revise Proposal 1.2.1B as below: Updated Proposal 1.2.1B: For SCell transition from deactivation to activation, support aperiodic CSI-RS for tracking, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI for a SCell, triggered via SCell activation command activating the SCell. - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI, except RAN4-related issue(s) - FFS: Whether the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI can be triggered together with aperiodic CSI-RS for tracking for the SCell supported in Rel-17 [Mod] To my understanding on WID, Rel-20 enhancement on supporting CSI-RS for tracking is only for IDLE/INACTIVE to CONNECTED mode. Proposal 1.1.1C: Since early triggering of CSI-RS for tracking should be supported based on WID, we support Alt 1. For Alt 2, same comment as above, i.e., Rel-20 UE is not mandated to support Rel-17 periodic TRS resources for IDLE/INACTIVE UEs. |
| Samsung | Proposal 1.1.3 Support in principle. Regarding Option 3, we think that overhead of capability via Msg3 would be larger than Option 1 and 2. If we go with Option 3, then gNB can trigger by considering/selecting from the common set of report/resource configuration via SIBx and Msg3 reported capability. We don’t see any benefit from Option 3 in terms of neither gNB flexibility nor UE flexibility & reporting overhead. So, we would like to suggest deleting Option 3. Also, considering comment from QC on UE capability, we suggest to delete “UE” (based on only one UE capability assumption, based on one or multiple UE capability assumptions) and add a note as below, as we added in the previous agreement. Note: The term “capability” above does not mean legacy RRC based UE capability. [Mod] Adding the note seems sufficient for clarification. Let’s keep the term of “UE capability assumption“ here and add the note. Proposal 1.2.1A: Support in principle. Regarding 2nd FFS, we would like to highlight that “based on legacy SCell activation MAC CE” should be considered per WID. Then, it means that, although detail MAC-CE design is up to RAN2, legacy (Rel-15/17) MAC-CE should be baseline and updated. If we have this FFS, then it may imply that separate MAC-CE can be considered, which should not be the case. Hence, we prefer to delete 2nd FFS. [Mod] Please check the update based on your suggestion. Proposal 1.2.1B: Support in principle. Similar with Proposal 1.2.1A, we prefer to delete 2nd FFS. Regarding AP CSI-RS for CSI, it is “on” a SCell which is measured from the SCell. But for corresponding AP CSI report, it can be “on a SCell” meaning AP CSI report on PUSCH is transmitted “from the SCell”, or it can be “for a SCell” meaning AP CSI report on PUSCH is transmitted either “from the SCell” or “from the other active cell”. Our view is that what the intended enhancement is AP CSI report for a SCell and AP CSI-RS on a SCell. So we would like to clarify as follows: Updated Proposal 1.2.1B: For SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell and corresponding aperiodic CSI-RS for CSI for on athe SCell, triggered via SCell activation command activating the SCell. - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI, except RAN4-related issue(s) - FFS: Whether the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI can be triggered together with aperiodic CSI-RS for tracking for the SCell supported in Rel-17 [Mod] Thanks for good suggestion on wording. Please check the update. Proposal 1.3.1A: Support in principle. We think that similar FFS for timeline can be added in this proposal. Regarding “without DCI enhancement”, since the intention is not to introduce additional DCI bits/fields (as FL kindly confirmed), we would like to revise as “without additional DCI overhead”. This is because, if we introduce additional rule or enhancement on trigger state for SRS request/CSI request, this also seems DCI enhancement. Proposal 1.3.1A: For SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - Legacy DCI format(s) for switching out of SCell dormancy is reused without DCI enhancement additional DCI overhead - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic SRS-AS transmission Proposal 1.3.1B: Support in principle. Similar comment with Proposal 1.3.1A, so updating the proposal accordingly as follows: Proposal 1.3.1B: For SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting and the corresponding aperiodic CSI-RS for CSI on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - Legacy DCI format(s) for switching out of SCell dormancy is reused without DCI enhancement additional DCI overhead - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI [Mod] Okay. Please check the update for those two proposals Proposal 1.1.C: Fine with studying both Alt1 and Alt2. For Alt1, we think that AP-TRS in fast SCell activation (in Rel-17) can be a baseline which can have up to two bursts, and also aligned with WID. For Alt2, it is indeed beneficial from gNB to inform UE the information of periodic TRS which will be configured to the UE after RRC CONNECTED mode. But we are not sure whether periodic TRS can be “triggered”. Also, we are not sure what the meaning of sub-bullet (taking Rel-17 periodic TRS resources ~). If we take this as a baseline, then the corresponding resource configuration is based on SIB17? We think this is not needed. To reflect (and add some more wording to align with WID), we would like to revise the proposal as follows: Proposal 1.1.1C: For UE transition from IDLE/INACTIVE to CONNECTED mode, down-select one from the following alternatives to support early CSI-RS for tracking triggered via MSG4 of 4-Step RACH: - Alt1: Support aperiodic CSI-RS for tracking - Taking aperiodic TRS for fast SCell activation as baseline. - Alt2: Support periodic CSI-RS for tracking - Taking Rel-17 periodic TRS resources for IDLE/INACTIVE UEs as baseline. [Mod] Then I suggest not mentioning any legacy scheme here. In Rel-17 periodic TRS resources for IDLE/INACTIVE UEs, there is a L1-based ‘TRS availability indication’ to indicate the first and following measurement occasion(s) within validity time. The L1-based ‘TRS availability indication’ seems to be a triggering indication. |
| Nokia | Proposal 1.1.3: The difference between the three options is not clear in terms of what the UE reports. In particular, for Option 2, it may be necessary for the UE to indicate which resource/report configuration is supported. Therefore, there are multiple aspects that need further discussion. [Mod] Sorry for the confusion due to missing some words in the proposals. Please check the proposal again. We would suggest discussing the UE capability reporting part later and first focusing only on the resource/report configuration. |
| Samsung | Proposal 2.1B/C : We support all the configurations. From our understanding, all of the above configuration is possible with appropriate set of ZP CSI-RS configuration in conjunction with NZP CSI-RS. It can be left to implementation to select the CSI-RS densities and appropriate sets of ZP CSI-RS based on condition of channel. As shown in our tdoc, with more ZP CSI-RS resources and proper power boosting for CSI-RS REs, throughput gain is possible. Proposal 2.2: We do not support different CSI frequency densities for K NZP CSI RS in CSI Resource set. The different CSI RS densities and unequal number of CSI-RS frequency pilot samples across the resources can have impact on channel estimation and throughput. Also from our understanding, different densities can have significant impact on specification. Also, it seems an out-of-scope issue, since WID mentions “For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification” where only same density for all CSI-RS resources within a same CSI-RS resource set is possible. Proposal 2.3: Support |
| Huawei, HiSilicon | Proposal 1.1.3: Don’t support Option-1. There are different types of UE with different capability in a cell. How can gNB assume only one UE capability assumption? The restriction is too strong and unrealistic. Proposal 1.2.1A: For the first FFS, suggest removing “including whether it is up to RAN1 or RAN4”. The timeline will surely have some RAN1 spec impact, e.g., the slot offset and reference slot design to make sure the triggered SRS is transmitted after DL synchronization, and thus it is not possible to leave all the discussion to RAN4. Indeed, some RAN4 impact may be involved. But that should be in the later stage after RAN1 have clear outcome on the triggering mechanism. [Mod]The wording of FFS has been revised. Please check the update Proposal 1.2.1B: Same comment as 1.2.1A Proposal 1.3.1A: Support. Prefer implicit triggering via legacy DCI without introducing new DCI field. Multiple SCell groups can be indicated by the DCI. If we introduce explicit indication of AP SRS for each SCell, dozens of new fields need to be introduced. That is unacceptable. Proposal 1.3.1B: Same comment as 1.2.1B. Proposal 1.1.1C: Since it is triggering, there is no way to support periodic TRS. The only choices may be: support aperiodic TRS, or not support aperiodic TRS, in Rel-20. Proposal 2.1B/2.1C: We share similar view with Samsung that no restriction is needed. Anyway, these are configured by network, network scheduler can avoid any configuration with potential problems. We don’t see the benefits to have these restrictions. |
| Xiaomi | Proposal 1.1.3: regarding option 2, if NW provide resource/report configuration(s) based on multiple UE capability assumptions, how to work if UE reports whether the resource/report configuration is supported? NW doesn’t know which one capability assumption is supported by the UE. [Mod] Sorry for the confusion due to missing some words in the proposals. Please check the proposal again. Proposal 1.3.1A/B: share same view as Samsung, if reinterpret ‘SRS request’ or ‘CSI request’ is not precluded, it is better to update ‘without DCI enhancement’ to ‘without additional DCI overhead’. |
| Panasonic | Proposal 1.1.3: Support Proposal 1.2.1A and Proposal 1.2.1B: We do not support. We find the design in these proposals ambiguous, so we would want to have the discussion about “TIMELINES” jointly with this discussion to understand the performance gains that we are after. Proposal 1.3.1A and Proposal 1.3.1B: We do not support. For the dormant SCell, the UE stops PDCCH monitoring but continues to perform CSI measurements and beam management (Periodic and semi persistent Reporting). We raised the question of why these reports are not sufficient for CSI acquistion. Isn’t this the purpose of specifying the dormancy state in legacy? [Mod] I think key point of this Rel-20 enhancement for switching out of SCell dormancy is how to acquire the CSI for non-dormant BWP after BWP switching as early as possible. Proposal 1.1.1C: Do not support. We prefer to study rather than downselect. |
| NEC | Proposal 1.1.3: General fine with the proposal. While for option-2, we share similar view with Nokia and Xiaomi, in case to cover multiple UE capability assumptions, multiple resource/report configurations are needed? If so, we suggest to add “whether/which of the resource/report configuration(s)”, as UE needs to indicate which one to be supported. [Mod] Sorry for the confusion due to missing some words in the proposals. Please check the proposal again. In addition, a wording suggested to update “both options”, as now there are three listed. [Mod] Thanks for the suggestion. Please check the update. Proposal 1.1.3: For UE transition from IDLE to CONNECTED mode, study the following three options for Step-1 and Step-2: - Option-1: NW can provide the resource/report configuration in SIBx based on only one UE capability assumption, and - Option-2: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 whether/which the resource/report configuration received in SIBx is supported. - Option-3: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 the supported capability(s) of early SRS/CSI/CSI-RS triggering (e.g., whether to support this feature, max number of CSI-RS ports, xTyR for SRS-AS, max bandwidth of the CSI-RS/SRS-AS etc.). Note: It’s not precluded to support at least one option depending on the use cases (e.g., early SRS-AS, early CSI-RS for tracking, and CSI-RS for CSI, early aperiodic CSI reporting) FFS: Whether the UE capability assumption(s) is predefined in the specification or is up to NW implementation |
| Mod | Please check the update to Proposals |
| ETRI | Proposal 2.1B: Support Proposal 2.1C: We have concerns about including the last two bullets (i.e., the cases with K=2). Samsung stated that CSI-RS power boosting with K=2 can provide performance gain in the last offline, but we doubt its validity when the delay spread is relatively high, as this would diminish the power boosting gain while also incurring inefficiency in rate matching due to legacy ZP CSI-RS. We further checked Samsung's tdoc on the LLS, which assumes 30 ns delay spread. However, we think that for UEs at the low SNR regime (i.e., cell-edge UEs), the delay spread would be around 100 ns. In such cases, while the power boosting gain could be beneficial, the larger frequency selectivity may cause performance degradation. Of course, the NW can configure such settings if performance gain is expected, but we believe such cases are less probable. Therefore, we prefer to keep them as FFS at this stage. Proposal 1.1.3: Support Proposal 1.2.1A/1.2.1B: Support Proposal 1.3.1A/1.3.1B: Support Proposal 1.1.1C: We think that rushing to down select among two alternatives is too early. We support both Alt1 and Alt2 at this stage. |
| ZTE2 | Proposal 1.1.3: Re Samsung’s comment, we do NOT agree, and we think which option requires less signaling overhead will be determined by specific design. For example, the candidate capabilities that UE may report may be limited, and one UE capability may correspond to more than one resource/report configurations. In this case, option 3 would require less signaling overhead. As this is a framework for further discussion, we do NOT think it is time to do down-selection. Proposal 1.2.1A/B: As pointed by Samsung, the WID sates that ‘… based on legacy SCell activation MAC CE’. So, some refinement may be needed to enable triggering SRS/CSI-RS/CSI by the SCell activation MAC CE. Accordingly, we suggest to directly use the wording ‘based on legacy SCell activation MAC CE’, and add an additional FFS ‘Whether/how to refine the SCell activation command MAC CE’. Proposal 1.2.1A: For SCell transition from deactivation to activation, support aperiodic SRS-AS transmission on a SCell, triggered via based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission, except RAN4-related issue(s) - FFS: Whether the aperiodic SRS-AS can be triggered together with aperiodic CSI-RS for tracking for the SCell supported in Rel-17 - FFS: Whether/how to refine the SCell activation command MAC CE Updated Proposal 1.2.1B: For SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell and corresponding aperiodic CSI-RS for CSI on the SCell, triggered via based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI, except RAN4-related issue(s) - FFS: Whether the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI can be triggered together with aperiodic CSI-RS for tracking for the SCell supported in Rel-17 - FFS: Whether/how to refine the SCell activation command MAC CE. [Mod] Thanks for the suggestion. Please check the update. |
| Futurewei | Proposal 1.1.3: Support. Proposal 1.2.1A: Support, but the wording ‘legacy SCell activation command’ may need to be revised to ‘SCell activation command activated the SCell enhanced from legacy SCell activation command’. Proposal 1.2.1B: TRS cannot be left out. MIMO CSI-RS for CSI acquisition requires TRS as its QCL source, i.e., TRS must precede CSI-RS. Without TRS, we are not even sure what kind of MIMO CSI can be acquired or reported. We are fine with LG’s version above with some small revisions: Updated Proposal 1.2.1B: For SCell transition from deactivation to activation, support aperiodic CSI-RS for tracking for a SCell, aperiodic CSI reporting for a the SCell and corresponding aperiodic CSI-RS for CSI on the SCell, triggered via legacy SCell activation command activating the SCell enhanced from legacy SCell activation command’. - FFS: Timeline of the aperiodic CSI-RS for tracking, aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI, except RAN4-related issue(s) - FFS: Whether the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI can be triggered together with aperiodic CSI-RS for tracking for the SCell supported in Rel-17 [Mod] Thanks for the suggestion. Please check the update. Proposal 1.3.1A/B: Support. Proposal 1.1.1C: Alt1. Again, in our view, TRS should precede CSI-RS even in IDLE mode, so since we now support AP CSI/CSI-RS, AP TRS is the most suitable one, otherwise the AP CSI-RS has to wait after a number of P TRS transmissions, which will be very slow. |
| Sony | Proposal 2.1B: Support. Proposal 2.1C: Support. Proposal 1.1.3: Okay with the current proposal with the three options for down-selection. Proposal 1.2.1A, 1.2.1B: Okay. |
| Qualcomm | Proposal 2.1C: Not support. For the two highlighted cases (2x24 and 2x32), we think at least it should be FFS before the claimed benefit is convincing to companies (while the drawback is clear: None-backward compatibility for legacy rate-matching – which is quite clearly required per WID) Proposal 1.1.3: OK for listing the options. Although our first preference is Option1 (to make things simple), we are open to Option2/3 for some cases (especially multiple choices of xTyR SRS-AS, which makes sense to us). Proposal 1.2.1B: Looks still a bit unclear to us. We are in favor of the spirit of “except RAN4-related issue(s),” and would like to clarify it more clearly: - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI, except RAN4-related issue(s) satisfying RAN4 requirements. [Mod] Whether the timeline designed by RAN1 satisfies RAN4 requirements is an RAN4-related issue. Original wording is more general. Proposal 1.2.1A: OK. For SRS, we can be more flexible (than Proposal 1.2.1B), since it was something not already existed in 38.133. Proposal 1.3.1A/B: Looks OK to us (although we think it should be deprioritized). Proposal 1.1.1C: Not support (After more thinking, we changed our view on this issue) We don’t see the necessity of Alt1 (AP-TRS), while Alt2 (P-TRS) does not need to be re-specified. - The logic of AP-TRS, seems to copy the similar thing from SCell activation case – however, the RACH scenario is totally different - For SCell activation, AP-TRS can reduce the DL T/F synchronization time (especially when SSB periodicity is long e.g. 160 msec); - However, after Msg4 of RACH, the DL T/F synchronization have already been very long (starts from cell search, a lot of SSB and PDSCH/PDCCH DMRSs have been received). - A very strong demonstration is needed, since it works fine for years in the field today w/o this Msg4-triggered AP-TRS. - For P-TRS, we don’t see the logic of “Rel-20 UE does not necessarily support Rel-17 features” – anyway Rel-20 features and Rel-17 features are UE-optional, why we need to re-design another optional given an existing optional? |

- Discussion

- Issue 1 – Early SRS/CSI/CSI-RS triggering

- Issue 1.1 – Transition from IDLE/INACTIVE to CONNECTED mode

Table 1-1 Summary for Issue 1.1

| # | Issue | Companies’ view and Moderator’s summary/proposal |
| --- | --- | --- |
| 1.1.1 | Use case of early triggering | Early CSI-RS for tracking - Aperiodic - Support: Huawei/HiSilicon, CATT, vivo, Samsung, OPPO, Rakuten, AT&T, Qualcomm, Xiaomi, MediaTek, Nokia, Panasonic, ETRI, LG, Fujitsu, Sharp - Concern: - Periodic - Support: Spreadtrum, ZTE, Samsung (open), Ericsson, ETRI, LG, Fujitsu, Sharp - Concern: vivo FL Note: Diverse companies’ views on the types of CSI-RS for tracking are observed. No proposal is recommended for now, and companies are encouraged to share your view on them. As a side information, periodic CSI-RS for tracking (P-TRS) measurement in IDLE/INACTIVE mode is supported in Rel-17 for UE power saving, which is configured by TRS-ResourceSetConfig in SIB17 and triggered by paging DCI or PEI. However, it is not used for UE transition from IDLE/INACTIVE to CONNECTED mode, and spec change may be needed. |
| 1.1.2 | Procedure of early triggering for INACTIVE → CONNECTED | Question 1: Whether the same procedure for the case of transition from IDLE to CONNECTED mode can be reused for transition from INACTIVE to CONNECTED mode? - Yes: Spreadtrum, MediaTek, Samsung (but skipping Step-2), Google, Fujitsu, Huawei/HiSilicon, - No: Ericsson, Panasonic |
| 1.1.3 | Step-1: Resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering in SIBx Step-2: Capability on early SRS/CSI-RS/CSI triggering through MSG3 | Which one of the following alternatives is supported for Step-1 and Step-2? - Alt1: NW can provide the resource/report configuration in SIBx for early triggering based on only one UE capability assumption, and UE can report through MSG3 whether the resource/report configuration received in SIBx is supported. - Support: Qualcomm (CSI-RS/CSI), Ericsson, Nokia (CSI-RS/CSI), Apple, Futurewei, LG, Spreadtrum (CSI-RS/CSI) - Alt2: NW can provide resource/report configuration in SIBx for early triggering based on one or multiple UE capability assumptions, and UE can report through MSG3 which resource/report configuration(s) received in SIBx is/are supported. - Support: Docomo, ETRI, NEC, Samsung, Spreadtrum, Xiaomi, ZTE, Qualcomm (SRS-AS), vivo, Nokia (SRS-AS), Panasonic, ETRI, Docomo, CATT, OPPO. TCL, Ericsson (with limited number), Fujitsu, Sharp FL note: It can be discussed later whether/how to separate the configurations and corresponding UE capabilities provided/reported for different use cases (e.g., early SRS-AS, early CSI-RS for tracking, early aperiodic CSI with PMI-based and PMI-less reporting), respectively. The UE capability assumption(s) is predefined in the specification or is up to NW implementation? - Alt1: Pre-defined in the specification - Support: Qualcomm, vivo, MediaTek, Apple, Docomo. Futurewei, LG, Spreadtrum, Sharp - Alt2: Up to NW implementation - Support: Nokia, Samsung, ZTE, ETRI, OPPO, TCL, Ericsson, Fujitsu, Huawei/Hisilicon ------------------------------------------------------------------------------------------------------------------ Which SIB is used carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering? - Alt1: SIB1 - Support: Huawei/HiSilicon, Spreadtrum, InterDigital, Lenovo, OPPO, Rakuten, MediaTek, Xiaomi, Apple, Docomo, - Alt2: New SIB - Support: CATT, vivo, NEC, HONOR, Futurewei, Fujitsu, Sharp FL note: No proposal is recommended for SIB issue in this round. |
| 1.1.4 | Step-3: NW triggers early SRS-AS/CSI-RS/CSI via MSG4 | The signaling medium for triggering early SRS-AS/CSI-RS/CSI - Alt1: MAC-CE in MSG4 - Support: Huawei/HiSilicon, InterDigital, ZTE, CATT, TCL, Xiaomi, MediaTek, Samsung, OPPO, NEC, Fujitsu, ETRI, Ericsson, ITRI/Acer, NICT, vivo, Nokia, Apple, Futurewei, Ericsson, Fujitsu, Sharp - Concern: Qualcomm - Alt2: DCI scheduling MSG4 - Support: Spreadtrum, China Telecomm, Qualcomm, Nokia, Panasonic, Docomo - Concern: vivo ------------------------------------------------------------------------------------------------------------------ How to determine the PUSCH for carrying aperiodic CSI reporting triggered by MSG4? - Alt1: The PUSCH is pre-configured along with the reporting configuration for the early CSI triggering. - Support: vivo, Samsung, OPPO, NEC, Apple, ETRI, Sony (1st) - Alt2: The PUSCH is scheduled by MSG4 along with the early CSI triggering. - Support: ZTE, CATT, vivo, MediaTek, TCL, Ericsson , Huawei/HiSilicon, Sony (2nd) - Alt3: The first DG or CG PUSCH after HARQ-ACK transmission for MSG4 (the same design as Rel-19 LTM). - Support: Spreadtrum, Xiaomi, Lenovo - Alt4: The first DG PUSCH after HARQ-ACK transmission for MSG4. - Support: Qualcomm, Docomo, Nokia, Spreadtrum, Fujitsu, Sharp - Alt5: ONLY the PUSCH carrying ‘RRC Setup Complete’ message. - Support: Sharp, vivo, Nokia, Sharp FL note: No proposal is recommended for this issue in this round. The triggering mechanisms and signaling contents can be discussed once we have reached conclusions on the above issues. |

Table 1-2 Company inputs for Issue 1.1

| Company | Input |
| --- | --- |
| Mod | No input in this round |

- Issue 1.2 – SCell transition from deactivated to activated state

Table 1-3 Summary for Issue 1.2

| # | Issue | Companies’ view and Moderator’s summary/proposal |
| --- | --- | --- |
| 1.2.1 | Use case of early triggering | Whether to support enhancement for triggering SRS-AS transmission on a SCell before SCell activation completion, via SCell activation command - Support aperiodic: Futurewei, Spreadtrum, MediaTek, ZTE, TCL, Samsung, Lenovo, OPPO, ETRI, NTT DOCOMO, vivo, Xiaomi, Apple, CATT, OPPO, Sharp - Support semi-persistent: TCL - Concern: Qualcomm FL note: Early SRS enhancement shall focus on enabling SRS transmission during SCell activation. According to current specifications Clause 5.9 of TS38.321 and Clause 4.3 of TS38.213, the SRS transmission on SCell shall start after the activation completion. ------------------------------------------------------------------------------------------------------------------ Based on FL’s understanding of the current specifications, periodic/semi-persistent CSI-RS measurement and reporting for a target SCell transitioning from deactivated to activated can be supported via a CSI report configuration provided on another cell (e.g., PCell). By the same approach, an aperiodic CSI report configuration can be also provided on another cell and is associated with an aperiodic CSI-RS resource configuration provided on the target SCell. In this case, the aperiodic CSI reporting is triggered by the DCI scheduling the cell provided with the aperiodic CSI report configuration, rather than by the DCI scheduling the target SCell. Furthermore, according to current RAN2 specifications TS38.321, the first active BWP configured for the target SCell should be activated once UE receives the SCell activation by RRC/MAC-CE. Therefore, it seems the aperiodic CSI-RS measurement on the target SCell is not precluded by no active BWP. Then, I’d like to check with companies whether you have the understanding same as or different from above? - Yes: Qualcomm, Samsung, Nokia, MediaTek, vivo, ZTE, ETRI, CATT, OPPO, LG, TCL, Spreadtrum, Fujitsu, Spreadtrum, Huawei/Hisilicon, Sharp - No: Docomo, Futurewei If your understanding is the same, whether you still support enhancement for triggering early CSI by SCell activation command? - Yes: Samsung, ZTE, ETRI, Docomo, Huawei/Hisilicon, Sharp - No: Qualcomm, MediaTek, vivo, LG, TCL, Fujitsu, Spreadtrum ------------------------------------------------------------------------------------------------------------------ Whether the early CSI/SRS triggering (if supported) for SCell transition from deactivated to activated state is applicable only if the first active BWP for the SCell is not a dormant BWP? - Yes: vivo, MediaTek, Samsung, Nokia, ETRI, Docomo, TCL, Huawei/HiSilicon - No: Apple, Ericsson |
| 1.2.2 | Step-1: Early triggering via SCell activation command | How to trigger aperiodic SRS-AS via SCell activation command? - Alt1: Explicit indication of an SRS trigger state in the SCell activation command - Support: MediaTek, Samsung, Apple, ZTE, ETRI, CATT, Sharp - Alt2: Explicit indication of an SRS resource set for antenna switching in the SCell activation command - Support: Xiaomi, Lenovo, LG, Huawei/Hisilicon - Alt-3: Implicit indication of an SRS resource set for AS, which is RRC-configured to a SCell for early SRS-AS triggering. When the activation command for the SCell is received, the SRS resource set for early SAS-AS triggering is triggered. - Support: NTT DOCOMO, Spreadtrum, Xiaomi, Nokia, ETRI - Alt4: Explicit indication of a CSI report configuration ID associated with SRS-AS in the SCell activation command - Support: vivo FL note: No proposal is provided in this round |

Table 1-4 Company inputs for Issue 1.2

| Company | Input |
| --- | --- |
| Mod | No input in this round |

- Issue 1.3 – SCell transition from dormant to non-dormant BWP

Table 1-5 Summary for Issue 1.3

| # | Issue | Companies’ view and Moderator’s summary/proposal |
| --- | --- | --- |
| 1.3.1 | Use case of early triggering | Whether to support enhancement for triggering aperiodic SRS-AS transmission on the non-dormant BWP, via the DCI switching SCell from the dormant BWP to the non-dormant BWP? - Support: Futurewei, Huawei/HiSilicon, ZTE, Xiaomi, MediaTek, Samsung, Lenovo, OPPO, ETRI, NTT DOCOMO, Qualcomm (low priority), MediaTek (prefer no DCI enhancement), vivo, Xiaomi, Ericsson, Fujitsu, Spreadtrum ------------------------------------------------------------------------------------------------------------------ Whether to support enhancement for triggering aperiodic CSI-RS for CSI on the non-dormant BWP, via the DCI switching SCell from the dormant BWP to the non-dormant BWP? - Support: Futurewei, Huawei/HiSilicon, ZTE, TCL, CATT, Samsung, Lenovo, OPPO, ETRI, NTT DOCOMO, Xiaomi, Qualcomm (low priority), Nokia, vivo, Xiaomi, Ericsson, Fujitsu, Spreadtrum |
| 1.3.2 | Step-1: Early triggering via the DCI indicating switching out of SCell dormancy | For SCell transition from dormant to the non-dormant BWP, to trigger the aperiodic SRS-AS transmission on the non-dormant BWP, whether the enhancement to legacy UE-specific DCI format 0_1/0_3/1_1/1_3 containing ‘SCell dormancy indication’ and ‘SRS request’ fields is necessary? - Yes: Qualcomm, Samsung, Xiaomi (only DCI format 0_1) - No: Nokia (prefer implicit triggering), MediaTek (no DCI enhancement), Fujitsu, Spreadtrum (prefer implicit triggering), Huawei/HiSilicon (prefer implicit triggering) FL note: The DCI indicating switching out of SCell dormancy can be only received on PCell, thus cross-carrier SRS triggering has to be supported for triggering the aperiodic SRS-AS transmission on SCell in this case. In RAN1 specification, cross-carrier SRS triggering is only specified explicitly for the SRS carrier switching based on DCI format 1_1/1_2. It is unclear whether cross-carrier SRS triggering is supported w/o SRS carrier switching. On the other hand, legacy UE-specific DCI format 0_1/0_3/1_1/1_3 containing “SCell dormancy indication” can switch out of SCell dormancy for multiple SCells. Then, it is unclear whether it is necessary/possible to use the DCI to simultaneously trigger SRS-AS transmissions on multiple SCells. Clause 6.2.1.3 in TS38.214 For an aperiodic SRS triggered in DCI format 1_1 or 1_2, if the UE is configured by SRS-CarrierSwitching, it transmits SRS on one serving cell not configured for PUSCH/PUCCH transmission scheduled by the DCI and the UE in the serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.. ------------------------------------------------------------------------------------------------------------------ For SCell transition from dormant to the non-dormant BWP, to trigger aperiodic CSI reporting and aperiodic CSI-RS measurement on the non-dormant BWP, whether the enhancement to legacy UE-specific DCI format 0_1/0_3 containing ‘SCell dormancy indication’ and ‘CSI request’ fields is necessary? - Yes: Qualcomm, Xiaomi (only DCI format 0_1) - No: Samsung, Nokia (prefer implicit triggering), MediaTek, Fujitsu, Spreadtrum (prefer implicit triggering), Huawei/HiSilicon (prefer implicit triggering) FL note: Legacy DCI format 0_1/0_3 with ‘SCell dormancy indication’ and ‘CSI request’ fields can support simultaneous switching out of dormancy and aperiodic CSI reporting for one or multiple SCells. The DCI format 0_1/0_3 received on PCell can trigger multiple aperiodic CSI reporting configurations provided on PCell (by linking these CSI report configurations to the same aperiodic trigger state) but associated with the CSI-RS resource configurations on the non-dormant BWPs of these SCells, and the aperiodic CSI report is transmitted on the PCell. To FL’s understanding, it seems we don’t need any enhancement for triggering aperiodic CSI/CSI-RS based on UE-specific DCI format 0_1/0_3 containing ‘SCell dormancy indication’ and ‘CSI request’ fields. ------------------------------------------------------------------------------------------------------------------ For SCell transition from dormant to the non-dormant BWP, to trigger aperiodic CSI, CSI-RS, or SRS-AS, whether the enhancement to legacy UE-common DCI format 2_6 containing ‘SCell dormancy indication’ is necessary? - Yes: - No: Qualcomm, Nokia (prefer implicit triggering), MediaTek, Xiaomi (prefer implicit triggering), Fujitsu, Spreadtrum (prefer implicit triggering), Huawei/HiSilicon (prefer implicit triggering) FL note: Legacy DCI format 2_6 doesn’t have ’SRS request’ or ‘CSI request’ field, and it cannot be used for scheduling a PUSCH for aperiodic CSI reporting. |

Table 1-6 Company inputs for Issue 1.3

| Company | Input |
| --- | --- |
| Mod | No input in this round |

- Issue 2 – CSI-RS frequency-domain density reduction

Table 2-1 Summary for Issue 2

| # | Issue | Companies’ view and Moderator’s summary/proposal |
| --- | --- | --- |
| 2.1 | New frequency-domain density 1/3, 1/4, 1/6, 1/8 | Whether to support the new frequency-domain densities for the following use cases in the table? Density Use Case; ρ = 1/3; ρ = 1/6; ρ = 1/4; ρ = 1/8. Aggregating K=3 16-port NZP CSI-RS resources for Ntot=48 CSI-RS ports; Support: Huawei/HiSilicon, ZTE, Spreadtrum, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, TCL, Ericsson, Fujitsu, CEWiT, Rakuten Concern:; Support: Huawei/HiSilicon, ZTE, Spreadtrum, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, TCL, Ericsson, Fujitsu, CEWiT, Rakuten Concern:; Support: Huawei/HiSilicon, Spreadtrum, CATT, Docomo, Samsung, Apple, OPPO, Ericsson, CEWiT, Rakuten Concern: ZTE, LG, Sony, QC, ETRI, Sony, TCL, Fujitsu; Support: Huawei/HiSilicon, Spreadtrum, CATT, Docomo, Samsung, Apple, OPPO, Ericsson, CEWiT, Rakuten Concern: ZTE, LG, Sony, QC, ETRI, Sony, TCL, Fujitsu. Aggregating K=2 24-port NZP CSI-RS resources for Ntot=48 CSI-RS ports; Support: Huawei/HiSilicon, Spreadtrum, CATT, Docomo, Xiaomi, Samsung, Apple, OPPO, Ericsson, CEWiT Concern: ZTE, LG, Sony, QC, ETRI, Sony, TCL, Fujitsu, Rakuten; Support: Huawei/HiSilicon, Spreadtrum, CATT, Docomo, Xiaomi, Samsung, Apple, OPPO, Ericsson, CEWiT Concern: ZTE, LG, Sony, QC, ETRI, Sony, TCL, Fujitsu, Rakuten; Support: Huawei/HiSilicon, Spreadtrum, ZTE, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, TCL, Ericsson, Fujitsu, CEWiT, Rakuten Concern:; Support: Spreadtrum, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, TCL, Ericsson, Fujitsu, CEWiT, Rakuten Concern: Huawei/HiSilicon, ZTE, ETRI, Spreadtrum. Aggregating K=4 16-port NZP CSI-RS resources for Ntot=64 CSI-RS ports; Support: Huawei/HiSilicon, Spreadtrum, ZTE, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, Ericsson, Fujitsu, CEWiT, Rakuten Concern:; Support: Huawei/HiSilicon, Spreadtrum, ZTE, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, Ericsson, Fujitsu, CEWiT, Rakuten Concern:. Aggregating K=2 32-port NZP CSI-RS resources for Ntot=64 CSI-RS ports; Support: Huawei/HiSilicon, Spreadtrum, ZTE, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, Ericsson, Fujitsu, CEWiT, Rakuten Concern:; Support: Huawei/HiSilicon, Spreadtrum, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, Ericsson, Fujitsu, CEWiT, Rakuten Concern: ZTE, ETRI, Spreadtrum. Aggregating K=4 32-port NZP CSI-RS resources for Ntot=128 CSI-RS ports; Support: Huawei/HiSilicon, Spreadtrum, ZTE, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, Ericsson, Fujitsu, Rakuten Concern:; Support: Huawei/HiSilicon, Spreadtrum, ZTE, CATT, LG, Sony, Docomo, Xiaomi, Samsung, Apple, ETRI, OPPO, Sony, Ericsson, Fujitsu, Rakuten Concern:. FL note: Based on the contributions from companies, for some of above use cases if configured with particular frequency-domain densities, the RE mapping patterns of the CSI-RS cannot match the RE mapping patterns supported by legacy ZP CSI-RS. Thus, these companies show concern on supporting the particular frequency-domain densities for those use cases. |
| 2.2 | Frequency-domain density among the K NZP CSI-RS resources in a CSI-RS resource set aggregating 48/64/128 CSI-RS ports | Whether the frequency-domain density configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation should be the same, or can be different? - Alt1: The K NZP CSI-RS resources should be configured with the same density - Support: MediaTek, CATT, TCL, Qualcomm, Samsung, Nokia, Xiaomi, Apple, ZTE, ETRI, TCL, LG, Sony, NEC, Ericsson, Fujitsu, CEWiT, Rakuten, Sharp, Docomo, Fraunhofer IIS/HHI, IIT, Lekha, Google - Alt2: The K NZP CSI-RS resources can be configured with different densities - Support: vivo, InterDigital, Huawei, Spreadtrum FL note: In legacy NR specification, “all CSI-RS resources within one set are configured with same density and same nrofPorts”. Question 1: Whether to support the following recommended proposal based on current input from companies? Proposal 2.2 Different frequency-domain densities can be configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation. Support: China Telecom Not support: Samsung, ETRI, ZTE, IIT Kanpur |
| 2.3 | RB positions for NZP CSI-RS resource with the new frequency-domain density | Whether to introduce RRC-configured RB-level offset per NZP CSI-RS resource (analogous to evenPRBs/oddPRBs for ρ=0.5) to indicate the occupied RBs for each NZP CSI-RS resource? - Yes: Apple, China Telecomm, HONOR, OPPO, Samsung, Spreadtrum, Xiaomi, ETRI, MediaTek, Docomo, Rakuten, CEWiT, Qualcomm, Nokia, vivo, Xiaomi, Apple, OPPO, TCL, Sony, EC, Ericsson, Fujitsu, CEWiT, Rakuten, Sharp, Docomo, IIT, Lekha, Google - No: Lenovo (joint indication), LG Whether we need to introduce the specification restriction to the RB positions for the K aggregated NZP CSI-RS resources (e.g., the K aggregated NZP CSI-RS resources must be configured in consecutive RBs or distributed evenly in every 1/ρ RBs), or the RB positions can be configured with full flexibility? - Alt1: Restriction to the RB positions for the K aggregated NZP CSI-RS resources is needed - Support: CATT, Qualcomm, ETRI, ZTE, Apple, NEC, Fujitsu, Docomo - Alt2: Full flexibility w/o restriction to the RB positions for the K aggregated NZP CSI-RS resources - Support: Samsung, vivo, OPPO, TCL, Sony, Ericsson, Spreadtrum, CEWiT, Huawei, Rakuten, Spreadtrum, Sharp, Fraunhofer IIS/HHI, IIT, Lekha Question 1: Whether to support the following recommended proposal based on current input from companies? Proposal 2.3: If CSI-RS frequency-domain density ρ = 1/3, 1/4, 1/6, and 1/8 is configured to the K NZP CSI-RS resources aggregating 48/64/128 CSI-RS ports, RB-level offset is RRC-configured per NZP CSI-RS resource (analogous to evenPRBs/oddPRBs for ρ = 1/2) to indicate the occupied RBs for each of the K NZP CSI-RS resources - FFS: How to indicate the RB-level offset and whether/how to introduce the specification restriction to the RB positions for the K aggregated NZP CSI-RS resources |
| 2.4 | Cyclic RB position in different transmission occasions | Whether to support cyclic RB position for the K aggregated NZP CSI-RS resources in different transmission occasions? - Yes: Huawei/HiSilicon, Docomo, CEWiT, Rakuten, IIT, Lekha, Fujitsu - No: Qualcomm, MediaTek, Nokia, vivo, Xiaomi, Apple, ZTE, CATT, OPPO, TCL, Ericsson, Google FL note: No proposal is provided in this round. |
| 2.5 | Enhancement on CSI subband size, PRG size, ARC, CPU, and CSI processing timeline | Question 1: Whether to introduce new CSI subband size (e.g., the CSI subband size that is an integer of multiple 1/ρ) for the frequency-domain density ρ = 1/3 and 1/6? - Yes: Huawei/HiSilicon, Samsung, LG, ETRI, AT&T (also for ρ = 1/4 and 1/8), Fujitsu, Docomo, CEWiT - No: OPPO Question 2: Whether to introduce new PRG size? - Yes: Lenovo, Docomo, Samsung - No: OPPO Question 3: Whether to enhance ARC, CPU, and CSI timeline for the aggregated 48/64/128 CSI-RS ports configured with the new frequency-domain density? - Yes: Huawei/HiSilicon, ZTE - No: OPPO, Samsung, MediaTek, Docomo |

Table 2-2 Company inputs for Issue 2

| Company | Input |
| --- | --- |
| Mod | Please share your comment on Proposal 2.2 and Proposal 2.3, if any |
| China Telecom | Proposal 2.2: We are fine with the proposal, since such configuration can be beneficial for gNB from the perspective of increasing the flexibility for gNB according to the channel condition. Proposal 2.3: Support. We think even if there are restrictions on the RB-level offset, RRC configuration per CSI-RS resource is still needed anyway. |
| ETRI | Proposal 2.2: We share similar view as Samsung. Proposal 2.3: Support |
| ZTE2 | Proposal 2.2: Do NOT support. We failed to see the necessity. Proposal 2.3: Do NOT support. We think the principles of aggregating the low-frequency-density NZP CSI-RS resources should be discussed first before the configuration signaling. For example, whether these CSI-RS resources should be allocated in FDM or TDM manner, and whether RB gap is needed if these CSI-RS resources are FDMed. If these CSI-RS resources need to be FDMed evenly, one possible way to introduce such restriction is to configured ‘whether even or odd RBs are occupied by these CSI-RS resources’ in resource set level. So, this proposal precludes some possible ways to introduce restrictions for CSI-RS resource aggregation. That is why we suggest to discuss it later. |
| IIT Kanpur | Proposal 2.2: We do not support it now but fine with FFS. Proposal 2.3: Support |

- Appendix: Agreements/conclusions before/in RAN1#122

RAN1#122

Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support at least aperiodic SRS-AS transmission triggered via MSG4 of 4-Step RACH. Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, support aperiodic CSI reporting triggered via MSG4 of 4-Step RACH based on the followings: - The aperiodic CSI reporting is transmitted on PUSCH. - Support at least aperiodic CSI-RS for CSI associated with the aperiodic CSI reporting - Support PMI-based reporting with wideband PMI based on Rel-15 Type-I SP codebook and wideband CQI - FFS: Which report quantity(s) can be configured - Support PMI-free reporting with wideband CQI - FFS: Which report quantity can be configured Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately Agreement: CSI-RS frequency-domain density ρ = 1/4 can be configured to the K NZP CSI-RS resources at least for the following cases: - K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports - K=4 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports - K=2 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports - K=4 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 128 CSI-RS ports FFS: K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports Note: It’s not precluded that the frequency-domain density configured to the K NZP CSI-RS resources in the same CSI-RS resource set for 48/64/128 CSI-RS ports aggregation can be different

- References

| 1 | R1-2505577 | Views on enhancing DL CSI acquisition | Samsung |
| --- | --- | --- | --- |
| 2 | R1-2505409 | Discussion on enhancing DL CSI acquisition | vivo |
| 3 | R1-2505636 | Enhancing DL CSI acquisition | Tejas Network Limited |
| 4 | R1-2505647 | Enhancing DL CSI acquisition | Lenovo |
| 5 | R1-2505750 | Discussions on Enhancing DL CSI Acquisition | OPPO |
| 6 | R1-2505284 | Discussion on enhancing DL CSI acquisition | TCL |
| 7 | R1-2505289 | Enhancing DL CSI acquisition | MediaTek Inc. |
| 8 | R1-2505303 | On enhancements for DL CSI acquisition | CATT |
| 9 | R1-2505456 | Discussion on enhancing DL CSI acquisition | Xiaomi |
| 10 | R1-2505165 | Discussion on enhancing DL CSI acquisition | Spreadtrum, UNISOC |
| 11 | R1-2505152 | Enhancing DL CSI acquisition | FUTUREWEI |
| 12 | R1-2505210 | DL CSI acquisition enhancment | Huawei, HiSilicon |
| 13 | R1-2505243 | NR MIMO Phase 6: DL CSI Enhancement | InterDigital, Inc. |
| 14 | R1-2505275 | Discussion on enhancing DL CSI acquisition | ZTE Corporation, Sanechips |
| 15 | R1-2506145 | Discussion on Enhancement of CSI DL Acquisition | Rakuten Mobile, Inc |
| 16 | R1-2506166 | On Rel-20 Enhanced DL CSI acquisition | Ericsson |
| 17 | R1-2506137 | Discussion on Early DL CSI Acquisition Enhancements | Panasonic |
| 18 | R1-2506211 | DL CSI acquisition enhancements in 5G MIMO Phase 6 | Qualcomm Incorporated |
| 19 | R1-2506266 | Enhancing DL CSI acquisition | Sharp |
| 20 | R1-2506234 | DL CSI Enhancements for NR Rel-20 | AT&T |
| 21 | R1-2506298 | Discussion on Enhancing DL CSI acquisition | NTT DOCOMO, INC. |
| 22 | R1-2505998 | Discussion on enhancing DL CSI acquisition | HONOR |
| 23 | R1-2506015 | Discussions on enhancing DL CSI acquisition | China Telecom |
| 24 | R1-2506016 | DL CSI acquisition enhancements for Rel. 20 MIMO | Fraunhofer IIS, Fraunhofer HHI |
| 25 | R1-2506111 | Discussion on DL CSI acquisition enhancements | Sony |
| 26 | R1-2506060 | Discussion on enhancing DL CSI acquisition for NR MIMO Phase 6 | ETRI |
| 27 | R1-2505906 | On Rel-20 MIMO CSI enhancement | Apple |
| 28 | R1-2505864 | On DL CSI Acquisition Enhancements for FR1 | Nokia |
| 29 | R1-2505822 | Discussion on enhancing DL CSI acquisition | LG Electronics |
| 30 | R1-2506319 | Discussion on DL CSI acquisition | ITRI, Acer Incorporated |
| 31 | R1-2506370 | Discussion on enhancing DL CSI acquisition | NICT |
| 32 | R1-2506371 | Discussion on Enhancing DL CSI acquisition | IIT Kanpur |
| 33 | R1-2505931 | Discussion on Enhancing DL CSI acquisition | NEC |
| 34 | R1-2505966 | Discussion on enhancing DL CSI acquisition | Fujitsu |
| 35 | R1-2506355 | Views on DL Channel acquisition enhancements | CEWiT, IITM |
| 36 | R1-2506351 | Discussion on enhancing DL CSI acquisition | Google |