3GPP TSG RAN WG1 #122		                               R1-2505294

Bengaluru, India, Aug. 25th – 29th, 2025

Source:	CATT

Title:	Maintenance on channel modelling enhancements for 7-24GHz

Agenda Item:	8.8

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our views on antenna radiation pattern of the handheld UT and the 	model of the antenna element-wise antenna field patterns with several TPs. We also provide the calibration results of BS side spatial non-stationarity using the parameters updated in RAN1#121 meeting.

- Discussions

  - Calibration for BS side spatial non-stationarity

In RAN1 #121 meeting, the following agreements on unified VP and VR based approach were achieved for BS side spatial non-stationarity.

Agreement For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted: - To calculate the power attenuation factor for the clusters with SNS, the definition of xs, ys in RAN1#120bis agreement is revised as: - The xs, ys denotes the coordinate of the antenna element out of VR region. - To generate the visibility region, the Pn refers to the power of cluster generated by (7.5-6) in dB scale. In the case of LOS condition, the LOS path is considered as an additional cluster, and the power ratio of the LOS path to NLOS clusters follows the Ricean K-factor generated in Section 7.5. Agreement The following value in the agreement of RAN1#120bis is revised as: - The probability of VR at the upper part of antenna array is [0.8] 0.5 and the probability of VR at the lower part of antenna array is [0.2] 0.5; Agreement Scenario; A; B; R; δ; C; PrSNS(μ,σ2). μ; σ. UMa; 0.15; 0.45; 33; Nμ,σ2~N0, 0.0015; 13; 0.56; 0.20. UMi; 0.12; 0.48; 50; N(μ,σ2)~ N(0, 0.001); 13; 0.49; 0.18. InH; 0; 0.60; NA; N(μ,σ2)~N0, 0.0011; 13; 0.31; 0.08.

The above agreements clarify antenna element coordinates outside the visibility region, specify cluster power calculation (including LOS as an additional cluster with Ricean K-factor), and update VR probabilities for the upper and lower antenna array parts to 0.5 each. Based on the above agreements, we conducted UMi scenario calibration. The results are provided in the attached excel file, and the detailed simulation parameters are summarized in Table 1

Table 1 Simulation assumptions for initial calibration results for BS side spatial non-stationarity

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz |
| BS Antenna Configuration and port mapping | Config 1 for 7 GHz: (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| UT Antenna Configuration | Config 1: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| Spatial non-stationarity modeling at the BS | Case 2: Stochastic-based approach |

  - Antenna radiation pattern of the handheld UT

| Reason for change: | - Clause 7.3.0 of TR 38.901 [1] introduces the antenna array structures for UT considered for calibration in this SI. However, the isotropic UT antenna pattern used in calibration in clauses 7.8.1, 7.8.2, and 7.8.3 is not specified, leading to inconsistency between these clauses and clause 7.3.0. - An unintended subscript “max” appears in the range of values of ϕ″ in the formula for the horizontal cut of the radiation power pattern (dB) in Table 7.3-2. |
| --- | --- |
| Summary of change: | - Add a description of the isotropic UT antenna pattern in clause, 7.3.0 to align with the calibration assumptions in clauses 7.8.1, 7.8.2 and 7.8.3. - Delete the subscript “max” from the range of values of ϕ″ in the formula for the horizontal cut of the radiation power pattern (dB). |
| Consequences if not approved: | - Calibration would be limited to directional UT antenna patterns, causing inconsistency across clauses and potentially impacting the reproducibility of simulation results. - The range of values of ϕ″ in the formula of horizontal cut of the radiation power pattern (dB) is unclear. |

Proposed TP for TR 38.901

- 7.3.0 Antenna array structure <Unrelated parts are omitted> UT antenna model: <Unrelated parts are omitted> - The antenna radiation power pattern of each antenna element for handheld UT is isotropic or the antenna radiation power pattern of each antenna element for handheld UT is generated according to Table 7.3-2. Table 7.3-2: Radiation power pattern of a single antenna element for handheld UT Parameter; Values. Vertical cut of the radiation power pattern (dB); AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with θ3dB=125°,SLAV=22.5dB and θ″∈0°, 180°. Horizontal cut of the radiation power pattern (dB); AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with ϕ3dB=125°,Amax=22.5 dB and ϕ″∈-180°, 180°max. 3D radiation power pattern (dB); AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax. Maximum directional gain of an antenna element GE,max; 5.3 dBi. NOTE: For UT antenna modelling of handheld devices, optional antenna imbalance can be modelled. If modelled, randomized loss is applied per UT antenna port and randomized loss can be applied independently for the UL and DL directions. No imbalance is modelled by default.. <Unrelated parts are omitted>

  - Model of the antenna element-wise antenna field patterns

| Reason for change: | In the current TR 38.901 [1], antenna element-wise angular-domain parameters are introduced to additionally model the antenna element-wise field patterns for the NLOS channel impulse response in the near-field channel model. However, the definitions of these antenna element-wise angular-domain parameters are missing, leading to ambiguity in their interpretation. |
| --- | --- |
| Summary of change: | Add definitions for the antenna element-wise angular-domain parameters in equation 7.6-49 of TR 38.901 [1], which are introduced for modelling the antenna element-wise field patterns. |
| Consequences if not approved: | The expression for the angular-domain parameters in equation 7.6-49 of TR 38.901 [1] remains undefined, resulting in ambiguity and potential inconsistency in implementation. |

Proposed TP for TR 38.901

- 7.6.13 Near-field channel model <Unrelated parts are omitted> - To model the antenna element-wise antenna field patterns additionally, the Hu,s,n,mNLOSt is given by: Hu,s,n,mNLOSt=PnMFrx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s expj2πd2,n,m-d2,n,m∙ rrx,n,m-drx,uλ0expj2πd1,n,m-d1,n,m∙ rtx,n,m-dtx,sλ0expj2πrrx,n,mT∙vλ0t (7.6-49) where θn,m,ZOA,u and ϕn,m,AOA,u are the respective antenna element-wise elevation arrival angles and azimuth arrival angles for ray m of cluster n between the reference point at TRP side and receive antenna element u, and θn,m,ZOD,s and ϕn,m,AOD,s are the respective antenna element-wise elevation departure angles and azimuth departure angles for ray m of cluster n between the transmit antenna element s and the reference point at UT side. <Unrelated parts are omitted>

- Conclusions

In this contribution, we present our views on the maintenance of channel modelling enhancements for 7-24GHz, with 2 TPs provided in previous sections. Besides, the calibration results of BS side spatial non-stationarity using the parameters updated in RAN1#121 meeting are provided.

Proposal 1: For the Rel-19 channel modelling enhancements for 7-24GHz, adopt the following TP for TR 38.901:

--------------------------------------- Start of TP for TR 38.901------------------------------------------

- 7.3.0	Antenna array structure

<Unrelated parts are omitted>

UT antenna model:

<Unrelated parts are omitted>

-	The antenna radiation power pattern of each antenna element for handheld UT is isotropic or the antenna radiation power pattern of each antenna element for handheld UT is generated according to Table 7.3-2.

Table 7.3-2: Radiation power pattern of a single antenna element for handheld UT

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with θ3dB=125°,SLAV=22.5dB and θ″∈0°, 180° |
| Horizontal cut of the radiation power pattern (dB) | AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with ϕ3dB=125°,Amax=22.5 dB and ϕ″∈-180°, 180°max |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax |
| Maximum directional gain of an antenna element GE,max | 5.3 dBi |
| NOTE: For UT antenna modelling of handheld devices, optional antenna imbalance can be modelled. If modelled, randomized loss is applied per UT antenna port and randomized loss can be applied independently for the UL and DL directions. No imbalance is modelled by default. |  |

<Unrelated parts are omitted>

--------------------------------------- End of TP for TR 38.901------------------------------------------

Proposal 2: For the Rel-19 channel modelling enhancements for 7-24GHz, adopt the following TP for TR 38.901:

--------------------------------------- Start of TP for TR 38.901------------------------------------------

- 7.6.13	Near-field channel model

<Unrelated parts are omitted>

-	To model the antenna element-wise antenna field patterns additionally, the Hu,s,n,mNLOSt is given by:

Hu,s,n,mNLOSt=PnMFrx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s

expj2πd2,n,m-d2,n,m∙ rrx,n,m-drx,uλ0expj2πd1,n,m-d1,n,m∙ rtx,n,m-dtx,sλ0expj2πrrx,n,mT∙vλ0t	(7.6-49)

where θn,m,ZOA,u and ϕn,m,AOA,u are the respective antenna element-wise elevation arrival angles and azimuth arrival angles for ray m of cluster n between the reference point at TRP side and receive antenna element u, and θn,m,ZOD,s and ϕn,m,AOD,s are the respective antenna element-wise elevation departure angles and azimuth departure angles for ray m of cluster n between the transmit antenna element s and the reference point at UT side.

<Unrelated parts are omitted>

--------------------------------------- End of TP for TR 38.901------------------------------------------

- References

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, v19.0.0, 2025-06.