3GPP TSG RAN WG1 #122		                                                      R1-2505331

Bengaluru, India, Aug.25th – 29th, 2025

Source:	CATT

Title:	Remaining issues on LP-WUS operation in IDLE/Inactive modes

Agenda Item:	8.6.2

Document for:	Discussion and Decision

Introduction

The LP-WUS operation of UE wakeup in IDLE/Inactive modes has been finalized with the Rel-19 LP-WUS/WUR work item completed in RAN1#121. The physical layer procedures of using LP-WUS for waking up UE in IDLE/Inactive modes include the LP-WUS MO configuration, LP-WUS monitoring procedure, RRM measurement and entry/exit condition in determining the LP-WUS monitoring, etc. A set of agreements were achieved in RAN1#121 [1]. In this contribution, remaining issues and error corrections of physical layer specification of LP-WUS operation procedures for UEs in IDLE/Inactive modes are discussed.

Discussion

Agreement@RAN1#118bis In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells. Agreement@RAN1#120bis For RRC connected mode, maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, - support maximum 16 candidates overlaid sequences for M=1 - support maximum 8 candidates overlaid sequences for M=2 - support maximum 4 candidates overlaid sequences for M=4 (agreed in RAN1#119) - For candidate overlaid sequences across all OOK ON chips of LP-WUS, the number of roots (in specification) is up to [FFS: X], FFS whether the number of roots can be different for different M value. - FFS: The number of overlaid sequences applicable for a UE is no more than 2 per OOK ON chip. - FFS: Whether or not the candidate set of overlaid sequences change across OOK ON chips

Based on the RAN1#121 agreements [2], one or more overlaid OFDM sequences are configured to a UE with one sequence indicating the given UE or other sequences with each sequence indicating a group of UEs including the given UE. Only one overlaid OFDM sequence is configured per OOK symbol. UE would be configured to decode an overlaid OFDM sequence from one or more configured OFDM sequences.

Proposal 1: To make it more clear, the ‘one or more overlaid OFDM sequences’ can be changed to ‘an OFDM sequence from one or more configured overlaid OFDM sequences’.

Adopt the TP#1 for Subclause 10.4C of TS 38.213.

- TP#1

| Reason for change: | The current specification is in error in capturing the overlaid OFDM sequence. Only one overlaid OFDM sequence would be configured per OOK symbol. UE would be configured to decode an overlaid OFDM sequence from one or more configured OFDM sequences. |
| --- | --- |
| Summary of change: | Change ‘one or more overlaid OFDM sequences’ to ‘an OFDM sequence from one or more configured overlaid OFDM sequences’. |
| Consequences if not approved: | The configuration of overlaid OFDM sequence is not unaligned with agreements. |

-------------------------------------------- Start of text proposal to TS 38.213 v19.0.0 ---------------------------------------

A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception - a number of OOK symbols per OFDM symbol, a first RB, and an overlaid OFDM sequence per OOK symbol for LPSS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211], - a number of OOK symbols per OFDM symbol, the first RB, and an OFDM sequence from one or more configured overlaid OFDM sequences per OOK symbol for WUS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211], and A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304].

-------------------------------------------- End of text proposal to TS 38.213 v19.0.0 ---------------------------------------

In [2], the ‘LP-SS reception occasions’ is used for the occasion of UE LP-SS reception and is different to the LP-SS occasion, which is the LP-SS transmission occasion configured by the gNB to the UEs. Similar to the LP-WUS monitoring occasion and LP-WUS occasion, the specification should align with the agreement to the ‘LP-SS occasions’ as the configured occasions of the LP-SS transmissions.

Agreement@RAN1#121 For UE to determine the LP-SS occasions, - One or two start symbol locations within a slot are configured. - Candidate value range for each start symbol location: {0, 1, …, 10}. - If one value is configured, there is one LP-SS occasion in a slot. - If two values are configured, there are two LP-SS occasions in a slot. - Starting from the slot for the first LP-SS occasion (determined from the periodicity/offset configuration for LP-SS), LP-SS occasions are present in this slot and the next (ceil(X/Y)-1) DL slots, where X is the number of SSBs determined according to ssb-PositionsInBurst in SIB1, and Y is the number of LP-SS occasions in a slot. - UE expects the slot for the first LP-SS occasion is a DL slot. - Note: a slot is a DL slot if all the symbols in the slot are indicated as DL symbols in tdd-UL-DL-ConfigurationCommon. - Note: UE follows the configured resources for LP-SS processing.

Proposal 2: To align with the agreement, ‘LPSS occasions’ can replace ‘LPSS reception occasions’.

Adopt the TP#1 for Subclause 10.4C of TS 38.213.

- TP#2

| Reason for change: | LP-SS reception occasion is incorrect. The RAN1 agreement of LP-SS occasion is the occasion of configured LP-SS transmission. The LP-SS occasions should be changed in the TS. |
| --- | --- |
| Summary of change: | Change ‘LPSS reception occasions’ to ‘LPSS occasions’. |
| Consequences if not approved: | The LPSS reception occasions are not aligned with agreement. |

-------------------------------------------- Start of text proposal to TS 38.213 v19.0.0 ---------------------------------------

A UE receives an LPSS in consecutive symbols within a slot. The UE can be provided one or two first symbols for respective one or two LPSS reception occasions in the slot by lpss-StartSymbol. The UE determines slots for LPSS reception occasions based on a periodicity and a time offset, relative to a system frame with SFN 0, provided by lpss-periodicityoffset. Within a period of LPSS reception occasions, LPSS reception occasions are in a set of KL consecutive slots that have all symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon, if provided, and start from the first slot provided by the time offset in the period, where K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and L is the number of LPSS reception occasions in a slot. LPSS reception occasions are indexed sequentially in time. An LPSS reception at the k-th LPSS reception occasion is quasi co-located with the k-th transmitted SS/PBCH block, with respect to quasi co-location 'typeC' or 'typeD' properties when applicable, where 1≤k≤K.

-------------------------------------------- End of text proposal to TS 38.213 v19.0.0 ---------------------------------------

In RAN1#121, it was agreed that the LP-SS occasion periodicity was configured with the candidate value set of {160ms, 320ms} [1]. It is noted that the time offset is configured for the first LP-SS occasion of one or more LP-SS occasions at each period with reference to the start of the SFN0. This should also reflect on the TS to make it clear.

Agreement—RAN1#121 If LP-SS is configured, the periodicity is configured with the candidate value set of {160ms, 320ms}. - A time offset is configured for the first LP-SS occasion with reference to SFN0 - If the periodicity is 160ms, the candidate value set for the time offset is {0, 1, …, 159}ms. - If the periodicity is 320ms, the candidate value set for the time offset is {0, 1, …, 319}ms.

Proposal 3: To make it more clear, the time offset is configured for the first LP-SS occasion per periodicity with reference to the start of the SFN0 should be reflected in TS.

Adopt the TP#2 for Subclause 10.4C of TS 38.213.

- TP#3

| Reason for change: | To make it more clear for the configured periodicity and time offset of LP-SS occasions. |
| --- | --- |
| Summary of change: | Capture agreement on detailed a time offset is configured for the first LP-SS occasion with reference to SFN0. |
| Consequences if not approved: | The details of the configured periodicity and time offset of LP-SS occasions are unclear. |

-------------------------------------------- Start of text proposal to TS 38.211 v19.0.0 ---------------------------------------

A UE receives an LPSS in consecutive symbols within a slot. The UE can be provided one or two first symbols for respective one or two LPSS reception occasions in the slot by lpss-StartSymbol. The UE determines the first slots of the configured number of LP-SS occasions at each period for LPSS reception occasions based on a periodicity and a time offset, relative to the start of a system frame with SFN 0, provided by lpss-periodicityoffset. Within a period of LPSS reception occasions, LPSS reception occasions are in a set of KL consecutive slots that have all symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon, if provided, and start from the first slot provided by the time offset in the period, where K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and L is the number of LPSS reception occasions in a slot.

-------------------------------------------- End of text proposal to TS 38.211 v19.0.0 ---------------------------------------

Conclusion

In this contribution, we share our views on remaining issues on LP-WUS operation in IDLE/Inactive modes. The proposals are summarized as follows:

Proposal 1: To make it more clear, the ‘one or more overlaid OFDM sequences’ can be changed to ‘an OFDM sequence from one or more configured overlaid OFDM sequences’.

Adopt the TP#1 for Subclause 10.4C of TS 38.213.

Proposal 2: To align with the agreement, ‘LPSS occasions’ can replace ‘LPSS reception occasions’.

Adopt the TP#1 for Subclause 10.4C of TS 38.213.

Proposal 3: To make it more clear, the time offset is configured for the first LP-SS occasion per periodicity with reference to the start of the SFN0 should be reflected in TS.

Adopt the TP#2 for Subclause 10.4C of TS 38.213.

References

- Chairman notes RAN1#121, St Julian’s, Malta, May 19th – 23rd, 2025.

- TS 38.213, NR, Physical later procedures for control, v19.0.0.