3GPP TSG RAN WG1 #122                                                                                           R1-2505366

Bengaluru, India, Aug 25th – 29th, 2025

Source:	vivo, Spreadtrum

Title:	Discussions on RAN simulation assumptions for ULBC

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

3GPP SA WG4 has an ongoing feasibility study on Ultra Low Bitrate Speech Codec (FS_ULBC) and has agreed to do the evaluation within SA4 under certain assumptions on, e.g., GEO NTN NB-IoT channel simulation parameters, the packet header overhead and the mouth-to-ear delay over the GEO channel. SA4 has sent an LS [1] to check RAN1’s view on the simulation assumptions.

To RAN1: ACTION: Q1: SA4 kindly asks RAN1 to confirm the evaluation assumptions in the attachment, and provide feedback, if any. Q2: In Table 6.1.3.3-1 of TR 38.821, how the RX G/T value (-31.6 dB/T) in the table or equivalently the antenna gain and noise figure for DL for NB-IoT with GEO are determined, whether it is a worst-case scenario, and whether SA4 can assume this value in the simulation?

In this contribution, we would provide the relevant background and discussion on the two questions.

- Discussions

Regarding the Q1

After checking the evaluation assumptions, we do not see any problems on the assumptions in the attachments. Nevertheless, we note that there are multiple target BLERs, i.e. 1%, 2%, 6%, 10% are considered for the evaluation. Generally, in the RAN1 evaluation, 2% rBLER is used as target BLER for VoIP or voice service, e.g. R17 coverage enhancement [2] and R18 NTN coverage evaluations [3]. It is suggested that SA4 takes this information into account.

Proposal 1: Answer to Q1: RAN1 does not observe any other problems regarding the evaluation assumptions in the attachment. However, RAN1 would like to inform SA4 that in general 2% rBLER is used as target BLER for evaluation of VoIP or voice service, for example, in the R17 coverage enhancement (TR38.830) and in the R18 NTN coverage evaluations (R1-2205212).

Regarding the Q2:

In TR38.821 Section 6.1.3.1, the calculation of G/T is given as follows. It can be observed that the key factors for determining the G/T are rx antenna gain, noise figure and antenna temperature, which mainly represent the performance of the receiver.

Antenna-gain-to-noise-temperature GT can be derived by [2] GTdB=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB] (6.1.3.1-3) where GR is receive antenna gain, Nf is noise figure, T0 is ambient temperature and Ta is antenna temperature. Receive antenna gain GR can be obtained by GR dBi=GR,edBi+10log10NR,a-Lp[dB], &array antenna10log10η∙π2∙D[m]2λ[m]2, &parabolic antenna (6.1.3.1-4) where GR,e is receive antenna element gain, NR,a is the number of receive antenna elements, Lp is polarization loss, η is the antenna aperture efficiency (a dimensionless parameter with typical values for parabolic antennas from 0.55 to 0.70), D is the equivalent antenna diameter, and λ is the wavelength.

Regarding the Rx antenna gain and noise figure, the values used to derive the G/T for various UE types are listed in the table 6.1.1.1-3 in TR 38.821, as cited below.

Table 6.1.1.1-3: UE characteristics for system level simulations Characteristics; VSAT (Note 2); Handheld; Other (Note 1). Frequency band; Ka band(i.e. 30 GHz UL and 20 GHz DL); S band (i.e. 2 GHz); Ka band(i.e. 30 GHz UL and 20 GHz DL). Antenna type and configuration; Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter; (1, 1, 2) with omni-directional antenna element; Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg). Polarisation; circular; Linear: +/-45°X-pol; Linear: +/-45°X-pol. Rx Antenna gain; 39.7 dBi; 0 dBi per element; TBD dBi per element. Antenna temperature; 150 K; 290 K; TBD K. Noise figure; 1.2 dB; 7 dB; TBD dB. Tx transmit power; 2 W (33 dBm); 200 mW (23 dBm); [TBD W (TBD dBm)]. Tx antenna gain; 43.2 dBi; 0 dBi per element; TBD dBi per element. NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna.

Based on the calculation and UE characteristics, the G/T for DL with 7dB noise figure could be derived by

GTdB=0 dBi-7 dB-10log10290 K+290 K-290 K10-0.1*7 dB=-31.62 [dBK].

From the link budget results in TR 38.821 Table 6.1.3.3-1, the calculated -31.62 [dBK] is matched with the G/T results of case 4 and case 5 for GEO set 1 and handheld UE.

Besides, some different UE characteristics were assumed, e.g. noise figure with 9 dB for NB-IoT/eMTC UE in TR 36.763 for R17 IoT NTN and for handheld or IoT devices in table 4.4-1 in TR38.811 for NTN as follows, respectively. For existing commercial high-end smartphones that support propriety solutions for voice call over GEO satellite, a better noise figure, e.g. 4dB, is already achievable.

### TR 36.763 ### 6.2.1 Link budget parameters The following assumptions are agreed for a common set of link budget parameters: - UE power class (PC5=20 dBm) - UE Noise Figure (NF=9 dB) - Channel Bandwidth for NB-IoT and eMTC as was included in IoT NTN reference scenario parameters agreed in RAN1#103e: - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz (UL) - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations, including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) - Other losses: Table 6.2-1: Other losses Other Losses; GEO (35786 km); LEO (1200 km); LEO (600 km); MEO (10000 km). Scintillation losses; 2.2; 2.2; 2.2; 2.2 dB. Atmospheric losses; 0.2; 0.1; 0.1; 0.04 dB. Polarization loss; 3; 3; 3; 3 dB. Shadow margin; 3; 3; 3; 3 dB. NOTE 1: With PC3 (23 dBm) there is a 3dB gain compared to the PC5 (20 dBm) assumption on UL. NOTE 2: With NF=7 dB, there is a 2 dB improvement compare to NF=9 dB on DL. NOTE 3: Link budgets with other link budget parameters are not excluded from being captured in the TR. NOTE 4: These parameters are only for the purpose of link budget calculations. NOTE 5: Atmospheric losses are a function of elevation angle. Link budget analysis assumes 3 dB polarization loss for DL and 3 dB polarization loss on UL for satellite parameters Set 1, Set 2, Set 3, and Set 4 For the satellite parameter sets Set-3 and Set-4, the 3 dB beam width (HPBW), central beam center elevation and central beam edge elevation in the satellite parameter set(s) to be used in link budget calculations are given in Tables 6.2-2 and 6.2-3. These parameters correspond to the satellite parameter Set 3 and Set 4 given in Tables 6.2-6 and 6.2-7 respectively.

TR 38.811 Table 4.4-1: Typical minimum RF characteristics of UE in satellite and aerial access networks Very Small Aperture Terminal (fixed or mounted on moving platforms); Handheld or IoT devices (3GPP class 3, see [2]). Transmit Power; 2 W (33 dBm); 200 mW (23 dBm). Antenna type; 60 cm equivalent aperture diameter (circular polarisation); Omnidirectional antenna (linear polarisation). Antenna gain; Tx: 43.2 dBi Rx: 39.7 dB; Tx and Rx: 0 dBi. Noise figure; 1.2 dB; 9 dB. EIRP; 45.75 dBW; -7 dBW. G/T (NOTE 1); 18.5 dB/K; -33.6 dB/K. Polarisation (NOTE 2); Circular; Linear. NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB NOTE 2: For S band, we assume that the User Equipment has an omni-directional antenna of linear polarization, while the antenna on board space-borne or airborne platforms features typically employs circular polarization. Hence a polarization mismatch of 3 dB has to be taken into account for the radio link budget computation. This will impact the UE RF characteristics as below: - Equivalent EIRP of 20 dBm (-10 dBW) under satellite coverage. - Equivalent G/T of -36,6 dB/K under satellite coverage..

For comparison,  G/T with different UE characteristics are listed in the following table.

Table 1. G/T for some combinations

| UE Rx Antenna gain | 0 dBi | 0 dBi | 0 dBi |
| --- | --- | --- | --- |
| Antenna temperature | 290K | 290K | 290K |
| Noise figure | 4 dB | 7 dB | 9 dB |
| DL G/T | -28.62 | -31.62 | -33.62 |

In summary, the calculated RX G/T value (-31.62 dB/K) is a moderate case, while a better value (e.g., -28.62 dB/K) is possible for commercial smartphone devices.

Proposal 2: Answer to Q2: The calculation of RX G/T is defined in the section 6.1.3.1 of TR 38.821. The RX G/T value of -31.6 dB/K is a moderate case. While a better RX G/T value of up to -28.62 dB/K can also be considered considering the available implementation of high-end devices.

- Conclusion

In the contribution, we provide our views and answers on the LS on RAN simulation assumptions for ULBC, with the following proposals.

Proposal 1: Answer to Q1: RAN1 does not observe any other problems regarding the evaluation assumptions in the attachment. However, RAN1 would like to inform SA4 that in general 2% rBLER is used as target BLER for evaluation of VoIP or voice service, for example, in the R17 coverage enhancement (TR38.830) and in the R18 NTN coverage evaluations (R1-2205212).

Proposal 2: Answer to Q2: The calculation of RX G/T is defined in the section 6.1.3.1 of TR 38.821. The RX G/T value of -31.6 dB/K is a moderate case. While a better RX G/T value of up to -28.62 dB/K can also be considered considering the available implementation of high-end devices.

Appendix: Draft reply LS on the RAN simulation assumptions for ULBC to R1-2505140 (S4-251584)

1 Overall description RAN1 thanks SA4 for the LS on the RAN simulation assumptions for ULBC, RAN1 can provide the following answers from RAN1’s perspective. Q1: SA4 kindly asks RAN1 to confirm the evaluation assumptions in the attachment, and provide feedback, if any. Answer: RAN1 does not observe any other problems regarding the evaluation assumptions in the attachment. However, RAN1 would like to inform SA4 that in general 2% rBLER is used as target BLER for evaluation of VoIP or voice service, for example, in the R17 coverage enhancement (TR38.830) and in the R18 NTN coverage evaluations (R1-2205212). Q2: In Table 6.1.3.3-1 of TR 38.821, how the RX G/T value (-31.6 dB/T) in the table or equivalently the antenna gain and noise figure for DL for NB-IoT with GEO are determined, whether it is a worst-case scenario, and whether SA4 can assume this value in the simulation? Answer: The calculation of RX G/T is defined in the section 6.1.3.1 of TR 38.821. The RX G/T value of -31.6 dB/K is a moderate case. While a better RX G/T value of up to -28.62 dB/K can also be considered considering the available implementation of high-end devices. 2 Actions To SA4 RAN1 respectfully asks SA4 to take the above information into account in related works.

References

- S4-251584, “LS on the RAN simulation assumptions for ULBC”, TSG SA WG4 #133-e, July 18th – 25th, 2025.

- 3GPP TR38.830: “Study on NR coverage enhancements (Release 17)”.

- R1-2205212: “Summary #3 on 9.12.1 Coverage enhancement for NR NTN”, TSG RAN WG1 #109-e, May 9th – 20th, 2022.