3GPP TSG RAN WG1 #122			R1-2505497

Bengaluru, India, Aug 25th – 29th, 2025

Source: 	     Xiaomi

Title:	NR-NTN uplink capacity/throughput enhancement

Agenda item:       8.11.3

Document for:    Discussion and Decision

Introduction

During RAN1#121 meeting, the following agreements were achieved for PUSCH capacity enhancement.

Agreement Remove the FFS in sub-bullet of 2nd bullet in RAN1#120bis agreement on UCI multiplexing. - If the PUSCH repetition is dropped according to legacy rules and the updated timeline conditions for PUSCH dropping are satisfied, UCI is transmitted on PUCCH and all PUSCH repetitions within the OCC group that overlaps with the PUCCH are dropped (Option 2) - FFS: if PUCCH is overlap with PUSCH repetition in both time and frequency domain. Agreement When OCC is applied on the PUSCH with UL-SCH with repetition type A and A-CSI report is triggered, the following applies: - The A-CSI report(s) is multiplexed on all PUSCH repetitions within the first OCC group. Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, support: - Option 3: Candidates values are configured by RRC higher-layer parameter as part of the TDRA table with a new column for configuring the OCC length, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - No additional rows for the TDRA table Conclusion RAN1 will not specify enhancements for support of TBoMS with inter-slot OCC for PUSCH in Rel-19. Conclusion There is no consensus in RAN1 to introduce a restriction on the number of PRBs to support inter-slot OCC for PUSCH. R1-2503685 Feature lead summary #2 of AI 9.11.3 on NR-NTN uplink capacity and throughput MediaTek Inc. From Thursday session Agreement For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI Format 0_1 and Format 0_2. Support implicit DCI indication with re-use of antenna port field. Association between antenna ports and OCC sequence is defined by re-using the legacy tables with two new columns added for OCC sequence. Note: the tables could directly reference the OCC sequence index (Table 6.3.2.5A-1 and Table 6.3.2.5A-2 in TS38.211) or spell out the sequence as in the example below. Table 7.3.1.1.2-6: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=1, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2-BPSK modulation is used Value; Number of DMRS CDM group(s) without data; DMRS port(s); OCC sequence for OCC length =2; OCC sequence for OCC length =4. 0; 2; 0; [1 1]; [1 1 1 1]. 1; 2; 1; [1 -1]; [1 -1 1 -1]. 2; 2; 2; [1 1]; [1 1 -1 -1]. 3; 2; 3; [1 -1]; [1 -1 -1 1]. Table 7.3.1.1.2-6A: Antenna port(s), transform precoder is enabled, dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2-BPSK modulation is used, dmrs-Type=1, maxLength=1 Value; Number of DMRS CDM group(s) without data; DMRS port(s); OCC sequence for OCC length =2; OCC sequence for OCC length =4. 0; 2; 0, nSCID= 0; [1 1]; [1 1 1 1]. 1; 2; 0, nSCID= 1; [1 -1]; [1 -1 1 -1]. 2; 2; 2, nSCID= 0; [1 1]; [1 1 -1 -1]. 3; 2; 2, nSCID= 1; [1 -1]; [1 -1 -1 1]. Table 7.3.1.1.2-7: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=2, except that dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured and π/2-BPSK modulation is used Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; OCC sequence for OCC length =2; OCC sequence for OCC length =4. 0; 2; 0; 1; [1 1]; [1 1 1 1]. 1; 2; 1; 1; [1 -1]; [1 -1 1 -1]. 2; 2; 2; 1; [1 1]; [1 1 -1 -1]. 3; 2; 3; 1; [1 -1]; [1 -1 -1 1]. 4; 2; 0; 2; [1 1]; [1 1 1 1]. 5; 2; 1; 2; [1 -1]; [1 -1 1 -1]. 6; 2; 2; 2; [1 1]; [1 1 -1 -1]. 7; 2; 3; 2; [1 -1]; [1 -1 -1 1]. 8; 2; 4; 2; [1 1]; [1 1 1 1]. 9; 2; 5; 2; [1 -1]; [1 -1 1 -1]. 10; 2; 6; 2; [1 1]; [1 1 -1 -1]. 11; 2; 7; 2; [1 -1]; [1 -1 -1 1]. 12-15; Reserved; Reserved; Reserved; Reserved; Reserved. Table 7.3.1.1.2-7A: Antenna port(s), transform precoder is enabled, dmrs-UplinkTransformPrecoding and tp-pi2BPSK are both configured, π/2-BPSK modulation is used, dmrs-Type=1, maxLength=2 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; OCC sequence for OCC length =2; OCC sequence for OCC length =4. 0; 2; 0, nSCID= 0; 1; [1 1]; [1 1 1 1]. 1; 2; 0, nSCID= 1; 1; [1 -1]; [1 -1 1 -1]. 2; 2; 2, nSCID= 0; 1; [1 1]; [1 1 -1 -1]. 3; 2; 2, nSCID= 1; 1; [1 -1]; [1 -1 -1 1]. 4; 2; 0, nSCID= 0; 2; [1 1]; [1 1 1 1]. 5; 2; 0, nSCID= 1; 2; [1 -1]; [1 -1 1 -1]. 6; 2; 2, nSCID= 0; 2; [1 1]; [1 1 -1 -1]. 7; 2; 2, nSCID= 1; 2; [1 -1]; [1 -1 -1 1]. 8; 2; 4, nSCID= 0; 2; [1 1]; [1 1 1 1]. 9; 2; 4, nSCID= 1; 2; [1 -1]; [1 -1 1 -1]. 10; 2; 6, nSCID= 0; 2; [1 1]; [1 1 -1 -1]. 11; 2; 6, nSCID= 1; 2; [1 -1]; [1 -1 -1 1]. 12-15; Reserved; Reserved; Reserved; Reserved; Reserved. Conclusion Explicit configuration / indication of OCC enabler is not supported. Agreement Revise the first bullet in RAN1#120bis agreement on UCI multiplexing with “without A-CSI reports” and FFS removed as below: - If the UCI is multiplexed on the PUSCH repetition according to legacy rules and the updated timeline conditions for UCI multiplexing are satisfied, UCI is multiplexed on all PUSCH repetitions without A-CSI reports within an OCC group with inter-slot OCC overlaps with the PUCCH. (Option 3-a) - FFS: PUSCH repetition with A-CSI reports Agreement Remove the FFS in the sub-bullet of 3rd bullet in RAN1#120bis agreement on UCI multiplexing. - UE does not expect there are multiple PUCCHs without repetitions in different PUCCH slots with a same or different UCI types other than SR overlapping with multiple PUSCH repetitions in the same OCC group. - FFS: whether the above applies only when at least one of the overlapping PUCCHs result in a UCI being multiplexed on the PUSCH Agreement Confirm RAN1#120bis Working Assumption 2 on UCI multiplexing with revised text: - Working Assumption 2: The above agreement applies to PUCCH with repetitions if no additional specification impact is identified. Agreement Confirm RAN1#120bis Working Assumption 1 on UCI multiplexing with revised text: Working assumption 1: The above agreement applies to different priority indexes for PUCCH/PUSCH if no additional specification impact is identified. Agreement A UE is not expected to be configured with frequency hopping for PUSCH repetitions with inter-slot OCC. Agreement Alt 2. For PUSCH grouping with inter slot OCC, the integer number of OCC groups is determined as M =N/L, where N is the number of repetitions of PUSCH and L is the OCC length. An OCC group is defined by L consecutive PUSCH repetitions. OCC Group m includes PUSCH repetition mxL, mxL+1, .., (m+1)xL-1, where m=0,1, .., M-1

In this contribution, we will focus on several left-over issues on NR-NTN PUSCH capacity enhancement and share our considerations.

Discussion

Applicable for UEs operating in TN

As illustrated in the WID below, a key issue under discussion is whether to apply inter-slot OCC multiplexing for UEs operating in TN based on a common design framework. From a normative work standpoint, there is currently no evidence of specialized inter-slot OCC multiplexing designs targeting explicitly for TN scenarios. Therefore, it is reasonable to conclude that inter-slot OCC multiplexing can be applied to UEs operating in TN under the existing common design principles. However, this conclusion may have impact on the UE feature description and require corresponding updates.

Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

Proposal 1: Support inter-slot OCC multiplexing for UEs operating in TN based on a common design.

RV counting upon OCC group(s) dropping

In RAN1 #120bis meeting, it was agreed that OCC group(s) may be dropped according to the legacy rules for single-UE transmission, when there is overlapping between PUCCH and PUSCH. For example, when the PUCCH has a higher priority than the overlapping PUSCH, or the PUCCH carrying SR occurring with the PUSCH without UL-SCH data, the whole number of repetitions within the OCC group will be dropped. In this way, one issue needs to be considered is how to determine the RV for the subsequent OCC group(s) transmission. Given the limited number of repetitions, and the extended duration of dropped PUSCH repetitions, we prefer to defer the RV for the follow-up OCC group(s) to maximize coding gain.

Proposal 2: Defer the RV for the follow-up OCC group(s) when OCC group(s) dropping is occurred.

Conclusion

In this contribution, we discuss the normative work of NR-NTN PUSCH capacity enhancement. Based on the discussion, our views are summarized as follows.

Proposal 1: Support inter-slot OCC multiplexing for UEs operating in TN based on a common design.

Proposal 2: Defer the RV for the follow-up OCC group(s) when OCC group(s) dropping is occurred.