3GPP TSG RAN WG1 Meeting #122	R1-2505523

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.3.3

Source:	Ericsson

Title:	Maintenance on SBFD CLI Handling

Document for:	Discussion, Decision

# 1	Introduction

The WID for SBFD [1] contains objectives for CLI handling in the following areas:

- UL resource muting for PUSCH

- L1-based UE-to-UE CLI measurement and reporting

In this contribution we discuss the RAN1 maintenance aspects for CLI handling.

# L1-Based UE-UE CLI Measurement and Reporting

For L1 based UE-UE CLI measurement, three methods were agreed: Method#1: UE measures RSSI within DL subband, Method#2: UE measures RSRP of aggressor UE within UL subband, Method#3: UE measures RSSI within UL subband[2]. Based on existing RAN1 agreements on L1-CLI measurement configuration, the method#1(RSSI measurement on DL subband) and method#3 (RSSI measurement on UL subband) for L1-CLI-RSSI cannot be performed on the same OFDM symbol based on RAN1#119 agreement. Similarly, method# 3 (RSSI measurement on UL subband) and method#2 (SRS-RSRP within UL subband) cannot be performed on the same OFDM symbol. Furthermore, a separate UE feature for allowing FDM-ed reception of DL and SRS-RSRP measurement was agreed based on the following agreement.

RAN1 #118 Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband RAN1 #119 Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig. RAN1# 120bis Agreement[3] For the following question from the RAN4 LS in R1-2501698: - RAN4 respectfully asks RAN1 to clarify whether the NW configuration of timeRestrictionForChannelMeasurement or a similar configuration will apply for L1-SRS-RSRP and/or L1-CLI-RSSI measurement. - RAN1 informs RAN4 that the NW configuration of timeRestrictionForChannelMeasurement will apply for L1-SRS-RSRP and L1-CLI-RSSI measurement. - RAN4 kindly asks whether RAN1 will define optional UE capability for FDM-ed DL reception and SRS measurement. - RAN1 informs RAN4 that a new optional UE capability will be defined for FDM-ed DL reception and SRS measurement. RAN1#121 Agreement[4] A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol.

Based on the agreements above, it is not clear whether the method#2 (SRS-RSRP within UL subband) and method#1 (RSSI measurement on DL subband) can be performed in the same OFDM symbol.

Furthermore, it is not clear if the FDM-ed DL reception and SRS measurement includes measurement of CLI-RSSI in the DL subband. It was defined with DL physical signals and channels and not considering measurements.

RAN1 to clarify if the UE feature of FDM-ed DL reception and SRS measurement include reception of the CSI-RS resources for CLI-RSSI measurement in the DL subband.

Additionally, due to the difference in measurement timings of SRS-RSRP and CLI- RSSI, we don’t see the technical feasibility to perform the two sets of measurements at the same time.

If RAN4 introduce measurement restrictions between method#1 and method#2, clarification in needed for which method have the priority at what scenario. Also, it is upto the network configuration to decide whether SRS and RSSI measurement resources will be on the same OFDM symbol, we think it is better to clarify this from RAN1 whether the SRS within UL subband and RSSI measurement resources within DL subband can be configured within the same OFDM symbol. Hence, we propose the following-

RAN1 agrees that measurement resource for method#1 and method#2 cannot be configured in the same OFDM symbol.

# Other maintenance issues

## 38.214 Section 5.2.1.5.1

-	For each aperiodic CLI-RSSI resource in a CLI-RSSI resource set associated with each CSI triggering state or for each aperiodic SRS-RSRP resource in a SRS-RSRP resource set associated with each CSI triggering state, the UE is optionally indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location typeD, as described in clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CLI resources associated with the CSI triggering state. [If a State referred to in the list is configured with a reference to an RS configured with qcl-Type set to 'typeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.] The UE is not expected to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CLI measurement resources in a CLI-RSSI-MeasurementResourceSet or in a SRS-RSRP-MeasurementResourceSet smaller than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48}∙2max(0, μCLI-3) and enableBeamSwitchTiming is not provided, or smaller than 48∙2max(0, μCLI-3) when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided.

-	[If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CLI measurement resources in a CLI-RSSI-MeasurementResourceSet or in a SRS-RSRP-MeasurementResourceSet is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48}∙2max(0, μCLI-3) and enableBeamSwitchTiming is not provided, or is equal to or greater than 48∙2max(0, μCLI-3) when the UE provides beamSwitchTiming-r16 and enableBeamSwitchTiming is provided, and]

-	if the UE is configured with a list of TCI states in CSI-AssociatedReportConfigInfo and it is configured with unifiedTCI-StateType, the UE is expected to apply the QCL assumptions indicated by qcl-Info for the aperiodic CLI measurement resources in the CSI triggering state indicated by the CSI trigger field in DCI.

-	if the UE is not configured with a list of TCI states in CSI-AssociatedReportConfigInfo and if the UE is configured with unifiedTCI-StateType, the UE is expected to apply the indicated DL TCI state or joint TCI state.

-	if the UE is not configured with a list of TCI states in CSI-AssociatedReportConfigInfo and if the UE is not configured with unifiedTCI-StateType, the UE is expected to assume that the aperiodic CLI measurement resources are the QCL ‘typeD’ to one of the latest received PDSCH and the latest monitored CORESET.

-	The UE is not expected to receive aperiodic CSI-RS and PDSCH/aperiodic CSI-RS associated with different values of coresetPoolIndex in overlapped symbol(s). The UE is not expected to receive aperiodic CSI-RS and semi-persistent/periodic CSI-RS with different 'QCL-type D' in overlapped symbol(s).

-	If dl-OrJointTCI-StateList-r17 is provided, the UE may assume that a CSI-RS resource in an aperiodic CSI-RS resource set configured without trs-Info is quasi co-located with the RS(s) in the indicated TCI state.

-	A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

      - It should be noted that the qcl-info is the wrong parameter name; 38.331 running CR uses qcl-InfoCLI-r19.

[[

resourcesForChannelCLI-r19      SEQUENCE {

resourceSetCLI-r19              INTEGER (1..maxNrofCLI-MeasResourceSetsPerConfig-r19)                        OPTIONAL,  -- Need R

qcl-infoCLI-r19                 SEQUENCE (SIZE(1..maxNrofCLI-MeasResourceSetsPerConfig-r19)) OF TCI-StateId  OPTIONAL  -- Need R

}

]]

      - Square brackets need to be resolved in RAN1.

      - Similar issues for CSI-RS switching time in 38.214 section

RAN1 considers the following issues for discussion and modifies the TS 38.214 section 5.2.1.5.1 and 5.2.1.5.1a accordingly.

  - Use qcl-InfoCLI-r19 instead of qcl-info, wherever applicable.

  - Square brackets need to be resolved.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to clarify if the UE feature of FDM-ed DL reception and SRS measurement include reception of the CSI-RS resources for CLI-RSSI measurement in the DL subband.

Proposal 2	RAN1 agrees that measurement resource for method#1 and method#2 cannot be configured in the same OFDM symbol.

Proposal 3	RAN1 considers the following issues for discussion and modifies the TS 38.214 section 5.2.1.5.1 and 5.2.1.5.1a accordingly.

	Use qcl-InfoCLI-r19 instead of qcl-info, wherever applicable.

	Square brackets need to be resolved.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Rapporteur (Huawei), June 2024.

R1-2501547, “Summary #2 of CLI handling,” Moderator (Huawei), RAN1#120, February 2025.

R1-2503036, “Summary #2 of CLI handling,” Moderator (Huawei), RAN1#120bis, April 2025.

R1-2504799, “Summary #2 of CLI handling, “ Moderator (Huawei), RAN1#121, May 2025.