3GPP TSG-RAN WG1 Meeting #122	R1- 2505593

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	11.2

Source:	NVIDIA

Title:	Evaluation assumptions for 6GR air interface

Document for:	Discussion and decision

# 1	Introduction

3GPP Release 20 represents both a capstone in the evolution of 5G-Advanced and a stepping stone toward 6G [1]. The Release-20 package includes a study item on 6G radio (6GR), aiming to develop a non-backward compatible radio access technology. The detailed study item description can be found in [2].

In this contribution, we present views on evaluation assumptions for 6GR air interface.

# 2	Key topics

## 2.1	Energy consumption model for base station (BS)

In order to align with the global climatic goal of sustainability and reduced carbon footprint, energy consumption modeling of base stations (BSs) that are projected to incur more than 50% of 6G networks’ total energy consumption is of critical importance. In the past, 3GPP introduced models and evaluation scenarios for estimating energy consumption of 5G gNodeB during Release 18 [3]. 3GPP’s BS energy consumption model classified gNB operation into multiple power states, each with distinct energy profiles – 1) Deep sleep (no DL transmission/UL reception; the lowest power state), 2) Light sleep (similar to deep sleep but shorter transition times), 3) Micro sleep (immediate transition to/from active states and short sleep between signals), 4) Active DL (DL only transmissions) and 5) Active UL (UL only transmissions). For each energy profile, the energy consumption model specifies the relative power consumed in each state and the energy/time needed for transitions. The model is scaled based on load and traffic profile considerations such as - slot and symbol level dynamic resource occupancy, bandwidth used, number of transmit/receive antennas, the direction of transmission (DL/UL), sleep modes (deep, light, micro) and transition times/energies for entering and/or exiting sleep states [4].

While the existing 5G BS energy consumption model could be the foundational steppingstone for 6G, increased complexity and data demands for 6G make its energy consumption model more complicated than 5G’s static rule-based approach. The AI-native framework of 6G, where AI serves both as a means for intelligently optimizing network’s energy savings and as a source of energy consumption by itself, triggers a fundamental shift in 6G BS’s energy modeling, including how energy consumption as well as energy savings are evaluated, measured and compared.

- The present rule-based energy modeling approach in 5G, where BS’s energy consumption is estimated based on ‘fixed’ power consumption levels at different operational states or energy profiles (deep sleep vs. active) should evolve to take into consideration AI-driven ‘dynamic and predictive’ network energy optimization in 6G, making the power consumption levels of various energy profiles as ‘variables’ with appropriate parameterization.

- The 6G BS energy model should accurately capture the dynamism of fine-grained energy management in evolving ‘simple’ sleep modes of 5G, wherein an AI-native 6G BS can adaptively control power consumption of its radiofrequency (RF) and compute resources by agile scaling of antenna arrays, beamforming and carrier activation/deactivation of individual radio units and their sub-components (like power amplifiers), based on real-time traffic demand.

- AI-driven energy optimization mechanisms of 6G may involve cross-layer joint coordination, wherein 6G BSs would manage energy at a network level instead of at a single BS in silo. The energy modeling of 6G BS needs to evolve considering possibilities of such cross-layer energy optimization.

In addition to AI-based energy savings, application specific signaling aspects and their impacts on energy consumption of the BS should be captured in 6G BS energy modeling as well. As one example, integrated sensing and communication (ISAC) would require processing of sensing signals, which may consume additional energy on top of energy consumed for communications signal processing. But the frequency of sensing signals reception/processing may dynamically vary over time depending on sensing requirements. This, in turn, will determine BS’s energy consumption, depending on factors like power amplifier on/off period, sensing signal processing chain activation/deactivation, sensing signal transmission frequency etc. The energy modeling of 6G BS should be flexible to cater to such application specific aspects of energy optimization.

Alongside modeling the energy savings from AI-driven and other optimizations in 6G network, the energy consumption of running AI models themselves needs to be taken into consideration in the energy modeling of 6G BSs. In this context, the following factors should be considered-

- Computational overhead (AI processors’ energy consumption in running complex AI algorithms for various workloads),

- Data processing (energy consumed for data collection, processing and transfer for AI models’ training/inference), and

- Model lifecycle management (energy footprint of AI models with the continuous operational cycle of ‘training-validation-deployment-monitoring-retraining-adaptation’).

Observation 1: Energy consumption model of AI-native 6G base stations will be a complex function, reflecting both energy-savings from AI-driven network optimizations and the energy cost of AI algorithms execution.

Proposal 1: Study energy consumption model of base stations in 6G during 6GR study, taking into consideration the energy savings from AI-driven network energy optimizations and the energy costs of running AI models.

## 2.2	Traffic model

Network traffic modeling is an essential part of communications systems design and performance evaluation to accurately predict network loads in realistic use cases and deployment scenarios. User data traffic models in a real network can significantly vary based on traffic types. Generally speaking, these traffic models can be broadly categorized into periodic and aperiodic traffic models. Periodic, fixed-size traffic models are usually used to simulate applications like Voice over IP (VoIP) that generates fixed-size packets at regular time intervals (e.g., every 20 ms) during the application runtime. Such deterministic, periodic traffic models are crucial for evaluating network performance for services with stringent latency and jitter requirements. Aperiodic traffic model, on the other hand, simulates applications like File Transfer Protocol (FTP), video streaming or gaming where the traffic pattern is unpredictable and often bursty. This type of traffic model uses statistical distributions to define traffic parameters. Aperiodic traffic models are generally used to evaluate network capacity and throughput for network services like web browsing and video streaming, where traffic is intermittent.

One way of simulating periodic traffic patterns is to use a full buffer traffic model, where it is assumed that data is always available for transmission. This is an ideal (but non-realistic) condition considering network to be fully loaded, primarily used to evaluate maximum throughput and spectral efficiency of the communications system. A more practical way of simulating periodic/aperiodic traffic patterns reflecting real network conditions is to use a non-full buffer traffic model, with bursty data transmission and traffic arriving either periodically or according to certain patterns (e.g., Poisson distribution). Non-full buffer traffic model is used for latency sensitive and low-to-moderate traffic-load scenarios like IoT, V2X or industrial automation [4]. Alongside full buffer and non-full buffer traffic models, 3GPP has also defined service or application specific traffic models, e.g., media and XR service models [6], tailored for media streaming, cloud gaming, and extended reality (XR). These traffic models feature detailed packet size distributions, transmission intervals, and dynamic characteristics of various types of traffic [7].

With the projected rise of UL-heavy AI traffic in 6G [8], 3GPP traffic models need to evolve beyond the conventional VoIP or FTP based traffic models mentioned above, which are traditionally DL-heavy and may not be ‘rich’ enough to accurately reflect the unique characteristic of AI-traffic. In addition to being bursty and often asymmetric between DL and UL, AI-traffic patterns will likely be influenced by the data type and the location of its processing (e.g., on-device versus far edge versus near edge/cloud). Built upon the service/application specific traffic modeling that 3GPP has already introduced in 5G, the AI-traffic models in 6G may further evolve to cater to additional requirements and characteristic unforeseen in the existing 5G applications.

As one example, in the case of federated learning, where data is kept ‘on-device’ to preserve privacy, only model updates are transferred from individual devices to the server (instead of the actual data), whereas the AI model training happens in a ‘distributed’ fashion (i.e., locally at each device). In such a scenario, a commensurate traffic model should reflect periodic traffic patterns with – 1) Burstiness (considering scenarios where multiple devices may send their model updates simultaneously), 2) Variable payload size (depending on the model update size), and 3) Higher UL traffic volume than DL. As another example, many AI applications in 6G will require real-time inference at the edge for low-latency requirements (e.g. AR/VR/XR) and this, in turn, may create a specific traffic pattern – which is asymmetrical and periodic, but with bursty data volume. More specifically, for some applications, UL traffic may be characterized by high data volume due to devices continuously uploading raw data (e.g., high-resolution video streaming) to the edge server for processing, while the DL traffic may reflect low-latency data in lighter volume stemming from edge servers’ short, delay-sensitive responses.

In addition to AI-traffic, there are other sources of non-AI traffic that are predicted to surge UL capacity demand in 6G networks, including growing trends of image/video uploads by mobile data consumers, and increasing users of cloud gaming and immersive applications over cellular networks. With the emergence of applications beyond communication, e.g., ISAC, 6G networks’ UL traffic will not only carry communication data packets but also sensing data, which would add to the UL traffic volume. This uptick in UL capacity demand should be properly reflected in the ISAC specific traffic models.

To address the unique requirements of aforementioned use cases (and more), 3GPP traffic models for 6G should reflect at least the following characteristics as its foundational principles –

- Uplink-centric traffic model - prioritizing UL direction, focusing on UL throughput, latency, and peak/average data rate needed for continuous upload,

- Bursty traffic pattern - incorporating statistical models reflecting unpredictability aspects of AI traffic patterns,

- Asymmetrical UL/DL traffic profile - explicitly defining UL/DL traffic ratio with key focus on scenarios entailing UL-heavy traffic,

- Context awareness of traffic model - considering data types (e.g., video/sensor data vs. data related to model parameters) and AI-application specific requirements (e.g., inference latency) in the context of specific services,

- AI-related service KPIs - adding to the traffic modeling KPIs such as AI inference accuracy with deterministic and stringent latency bounds, resource utilization efficiency for distributed computing and/or model aggregation, and energy efficiency,

- Extensible traffic model - making traffic models readily updatable/upgradable with new AI-service classes.

Observation 2: 6G network traffic is expected to be uplink-heavy, bursty and highly dynamic with the uprise in immersive (e.g., XR/cloud gaming) and AI-driven applications.

Proposal 2: Study traffic models for performance evaluation during 6GR study taking into consideration the unique characteristics of UL-heavy immersive and AI applications related traffic.

## 2.3	Channel model

The physics of electromagnetic (EM) wave propagation is characterized by Maxwell’s equations. Solving Maxwell’s equations requires accurate knowledge of the boundary conditions at interfaces between different materials, as well as the EM properties of these materials, which can be challenging to characterize accurately. To address specific aspects of wireless propagation without solving the full set of Maxwell's equations, stochastic channel models and ray tracing are both techniques used in the field of wireless communication to model and simulate the behaviour of radio waves.

Stochastic channel models take a statistical and probabilistic approach. They use statistical processes to model the random variations and uncertainties in the wireless channel. While stochastic channel models may capture statistical properties, they do not represent the precise geometric details of the environment or specific signal paths, leading to a lack of physical consistency and site-specificity.

To address some of the shortcomings of the 3GPP stochastic channel models, 3GPP has conducted two studies in Release 19 focusing on channel modeling for integrated sensing and communication (ISAC) and for 7–24 GHz spectrum to prepare for 6G standardization.

- Release-19 Channel Modeling Study for ISAC: As a key 6G usage scenario, ISAC integrates sensing capabilities to obtain spatial information about connected or unconnected objects and their environments. In Release 19, 3GPP defined channel modeling for sensing using the existing 3GPP channel models as a starting point, covering the following aspects:

  - Scenarios

  - Physical object model, including a sensing target’s RCS and cross-polarization matrix

  - Reference channel models and required updates

  - Fast fading model, including target channel and background channel

  - Additional modeling components, including spatial consistency, Type-2 environment object, etc.

- Release-19 Channel Modeling Study for 7–24 GHz Spectrum: The 7–15 GHz spectrum is considered to be the main new spectrum range that can be used for 6G in addition to the existing two frequency ranges (FR): FR1 and FR2. It has the potential to provide wide-area coverage with large bandwidths. In Release 19, at least for the 7–24 GHz spectrum, 3GPP adapted the channel models, covering the following aspects:

  - Near-field channel model

  - Spatial non-stationary channel model

While Release-19 channel model studies introduced some deterministic elements, the channel models are still mostly stochastic. 6G wireless networks are becoming increasingly heterogeneous, encompassing different inter-site distances, antenna array dimensions and layout radiated powers, frequency bands, to name a few. Correspondingly, the wireless channel modeling needs to provide consistency and, above all, a correct representation of the frequency, spatial, and temporal correlation across BSs and devices. Achieving this without a propagation model grounded on the underlying physics of the scattering phenomena is prone to modeling error and possibly a huge waste of effort for the industry.

Ray tracing is a rendering and simulation technique used in computer graphics, optics, and other fields to simulate the way rays of light or other radiation travel through a virtual environment. In the context of wireless communication and radio wave propagation, ray tracing is often employed to model and simulate the paths that EM waves take as they propagate through various materials and interact with surfaces and obstacles. It provides a deterministic and physics-based modeling approach that simulates the paths of individual rays of EM waves, considering reflections, refractions, diffractions, and other interactions with objects and surfaces. It offers high-resolution simulations, capturing the specific paths of rays and the effects of the surrounding environment, making it valuable for site-specific planning, antenna design, and network optimization in the field of wireless communication.

It is worth noting that there is a new Task Group about WLAN sensing within the IEEE 802.11 working group, i.e., Task Group IEEE 802.11bf. This task group has embraced ray tracing based channel model for Wi-Fi sensing [10].

Ray tracing is essential for studying, evaluating, and developing key technologies in 6G. For example, AI/ML is becoming an essential feature of 5G-Advanced toward 6G. Using data generated by stochastic channel models to train and test AI/ML models will lead to inaccurate assessment of the models and result in failure of the AI/ML models when deployed in real networks. To illustrate this, we consider a concrete use case study, where the channel impulse response data is used as input to an AI/ML model for object localization. A small indoor office environment was selected for evaluation.

- A first set of channel impulse response data was collected based on 3GPP stochastic channel model, and then a first AI/ML model was trained using the channel impulse response training data from stochastic channel model.

- A second set of channel impulse response data was collected based on ray tracing, and then a second AI/ML model was trained using the channel impulse response training data from ray tracing.

After training, both AI/ML models were tested using channel impulse response test data from ray tracing, assuming the physics-based ray tracing data represents the ground truth of the specific indoor environment.

The figure below shows the evaluation results. We can see that the test accuracy is poor for the AI/ML model trained on stochastic channel model data, compared to the test accuracy of the AI/ML model trained on ray tracing data. This implies that using data generated from stochastic channel models to train AI/ML models will lead to degraded performance of the AI/ML models when deployed in real networks that are site specific.

Figure 1: Comparison of stochastic channel model and ray tracing for AI/ML with sensing.

The key takeaway from this case study is that a propagation model, particularly ray tracing, grounded on the underlying physics of the scattering phenomena is indispensable for 5G-Advanced toward 6G.

Observation 3: Task Group IEEE 802.11bf has embraced ray tracing based channel model for Wi-Fi sensing.

Proposal 3: Besides stochastic channel model, deterministic physics-based channel model, especially ray tracing, should be considered for 6GR study.

## 2.4	Example scenarios and parameters for simulation

For 6GR study, it is important to conduct performance evaluations in deployment scenarios that span a wide range of real‐world conditions. We propose considering at least five scenarios: rural, suburban, urban, dense urban, and indoor hotspot, in system-level simulations. The rural scenario, with wide cell spacing and minimal clutter, captures coverage-centric use cases such as wide-area IoT or aerial connectivity. The suburban case introduces moderate building density to evaluate mid-range deployments. The urban scenario represents a typical city-center macro network, where interference and user density rise sharply. The dense urban scenario, modeling high-rise districts, tests capacity-focused enhancements under the most demanding site density and blockage conditions. Finally, the indoor hotspot scenario can model office or factory environments with high BS density. Other scenarios considered in 3GPP TR 38.914 about the Study on 6G Scenarios and Requirements can be derived from these base scenarios, e.g., by setting required UE speed, inter-site distance (ISD), and device density, modeling aircraft and satellite components.

To reflect the spectrum likely to be available for 6G, we propose considering carrier frequencies around 700 MHz, 2 GHz, 4 GHz, 7 GHz, and 13 GHz. The sub-1 GHz band (e.g., 700 MHz) provides deep wide area coverage, but it has limited capacity. Mid-bands around 2 GHz and 4 GHz offer a balance of coverage and capacity. The 7 GHz band begins to introduce wider contiguous channels while still retaining reasonable link budgets, and the 13 GHz band approaches the lower edge of millimeter-wave territory, enabling even larger bandwidths but demanding more aggressive beamforming and RF technologies.

Channel bandwidth is another critical performance evaluation assumption. We suggest considering at least up to 200 MHz channel bandwidth per carrier, with further consideration of 400 MHz in higher frequency bands. This range allows examination of proposals under both moderate-width channels, where legacy spectrum coexistence and sub-200 MHz RF frontends remain relevant, and ultra-wide channels that stress next-generation baseband processing, filter design, and MIMO algorithms.

For ISD, we recommend the following values: 5 km ISD for rural, 1732 m for suburban, 500 m for urban, 100m-200 m for dense urban and 20 m for indoor hotspot. These distances reflect typical macro-cell layout in sparse deployments and progressive densification down to small-cell grids and indoor environments.

On the BS antenna side, we propose that BSs employ at least 128 transceiver chains, and in the most advanced massive MIMO studies, 256/512 chains, to enable thorough evaluation of high-order spatial multiplexing, fine-grained beamforming, and null-steering techniques in, e.g., FR3. UE should be modeled with at least 8 RXs and up to 8 TXs  in, e.g., FR3, reflecting the anticipated evolution of handset and device form factors.

Distributed MIMO, where multiple geographically separated BSs jointly process signals as if they were a single, large antenna array, has the potential of reducing inter-cell interference, boosting cell-edge throughput, and enhancing continuity of coverage, but it comes with the associated fronthaul, synchronization, and processing overhead. To systematically assess its benefits and costs, we propose considering four coordination modes: no coordination as a baseline, and distributed MIMO clusters of 3, 9, and 21 cells (i.e., 1 site with 3 sectors, 3 sites with 9 sectors, and 7 sites with 21 sectors).

Finally, mobility assumptions should span from pedestrian speeds (3 km/h) through typical vehicular speeds (30 km/h and 120 km/h), high-speed train contexts (500 km/h), and airborne platforms (1000 km/h). Covering this entire speed range guarantees that doppler resilience, channel tracking algorithms, and adaptive schemes are evaluated under both slow-fading and ultra-fast dynamics.

Table 1Table 1 summarizes the proposed scenarios and parameters for evaluating technology proposals in 6GR study.

Table 1: Example scenarios and parameters for 6GR simulation study.

| Attribute | Example Assumptions for Simulation |
| --- | --- |
| Scenario | Rural, Sub-urban, Urban, Dense urban, Indoor office/factory |
| Carrier frequency | Around 700 MHz, 2 GHz, 4 GHz, 7 GHz, 13 GHz |
| Channel bandwidth | 200 MHz, 400 MHz per carrier in FR1/FR3 |
| Inter-site distance | - Rural: 5000 m - Sub-urban: 1732 m - Urban: 500 m - Dense urban: 100m-200 m - Indoor office/factory: 20 m |
| BS antenna | 128/256/512 transceiver chains for massive MIMO in, e.g., FR3 |
| BS coordination | - No coordination - Distributed MIMO: coordination across 3, 9, 21 cells |
| UE antenna | At least 8 Rx, up to 8 Tx in, e.g., FR3 |
| UE speed | - Pedestrian: 3 km/h - Vehicular: 30 km/h, 120 km/h - High-speed train: 500 km/h - Airplane: 1000 km/h |

Proposal 4: Consider the scenarios and parameters listed in Table 1 for evaluating technology proposals in 6GR study.

## 2.5	Evaluation metrics

In the 6GR study, spectral efficiency related metrics should be the key evaluation metrics.

Average Spectral Efficiency (bits/s/Hz/TRxP)

Average spectral efficiency (SEavg) measures the aggregate network throughput per unit bandwidth and per transmission reception points (TRxPs), offering a macro-level view of spectrum utilization. It is computed by summing the correctly received bits of all N users over a period T, dividing by the channel bandwidth W and the number of active TRxPs M:

SEavg=1M⋅n=1NRnT⋅W

where Rn denotes the number of correctly received bits (i.e., the number of bits contained in the SDUs delivered to Layer 3) of user n during the period. The normalization makes SEavg comparable across deployments with different site densities or spectrum allocations.

5th-Percentile User Spectral Efficiency (bits/s/Hz)

The 5th-percentile user spectral efficiency quantifies how efficiently the network serves its cell-edge users, by normalizing their throughput (correctly received Layer 3 bits over active session time) to the channel bandwidth and expressing it in bits/s/Hz.

Formally, the normalized user throughput of user n is defined as:

rn=RnTn⋅W

where Rn denotes the number of correctly received bits (i.e., the number of bits contained in the SDUs delivered to Layer 3) of user n during its active session time Tn. The channel bandwidth W is the operating bandwidth adjusted for the uplink/downlink ratio.

The 5th-percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput.

Area Traffic Capacity (bits/s/m2)

Area traffic capacity translates average spectral efficiency into spatial throughput by accounting for site density and channel bandwidth.

Defined as the total bits delivered per square meter per second (bits/s/m²), it links the per-TRxP spectral efficiency (bits/s/Hz/TRxP) to the TRxP density ρ (TRxP/m²) and channel bandwidth W (Hz):

Carea=ρ⋅W⋅SEavg

where SEavg denotes the average spectral efficiency as defined before.

Network Energy Efficiency (bits/s/W)

Network energy efficiency metric measures the energy consumption of a RAN for delivering traffic.

The metric (bits/s/W) is defined as:

EE=load level jαjVjECj

where

- αj∈[0,1] denotes the weight for each traffic load level and jαj=1.

- Vj denotes the traffic per second served by a base station at the load level j (in bits/s).

- ECj denotes the power consumed by a base station to serve traffic Vj  (in Watt).

Radio Resource Utilization Efficiency (unit-less)

Radio resource utilization efficiency (RUE) metric aims to measure how efficiently a RAT utilizes the available radio resources [11].

Applicable to either downlink or uplink, the metric is defined as the product of five components:

RUE=1-γguardband⋅1-γCP⋅1-γOH⋅Rcode⋅Rduplex

In this equation:

- γguardband∈[0,1]  denotes the guard band ratio, i.e., total guard band divided by bandwidth.

- γCP∈[0,1]  denotes the ratio of the number of CP samples to the total number of samples in a slot.

- γOH∈[0,1]  denotes the overhead calculated as the average ratio of the number of resource elements occupied by L1/L2 control, SSB, reference signals, etc. This value heavily depends on how the system configures CORESETs for PDCCH, SSB, TRS, CSI-RS, DM-RS, etc.

- Rcode∈[0,1]  is the forward error correction (FEC) code rate.

  - Using maximum forward error correction code rate in the metric evaluates the peak performance. One can use, e.g., average forward error correction code rate to evaluate the average performance.

- Rduplex∈[0,1] is the duplex factor reflecting the fraction of time/frequency resources available in the direction (downlink or uplink) in question:

  - In FDD, Rduplex(DL)=Rduplex(UL)=0.5 because the radio resources are equally split between downlink and uplink.

  - In TDD, Rduplex(DL) and Rduplex(UL) depend on how the specific TDD configuration splits the radio resources between downlink and uplink.

  - In Full Duplex, Rduplex(DL)=Rduplex(UL)=1 because the radio resources are fully available to both downlink and uplink.

The following table provides an example calculation for the radio RUE metric in the 5G NR downlink.

| 1-γguardband | 1-γCP | 1-γOH | Rcode | Rduplex | RUE |
| --- | --- | --- | --- | --- | --- |
| 0.9828 | 0.9333 | 0.8 | 0.9258 | 0.5 | 0.3397 |
| Assume 100 MHz with 30 kHz SCS | Assume normal CP | Assume 20% overhead in downlink | Assume max NR code rate: 948/1024 | Assume FDD | 5G NR downlink |

The above example calculation is for 5G NR downlink in FDD. Similarly, considering a TDD configuration with 70% radio resources allocated to downlink, the radio RUE metric in the 5G NR downlink can be calculated as RUE = 0.9828*0.9333*0.8*0.9258*0.7 = 0.4755.

As can be seen from the above two calculation examples, the radio RUE values in 5G NR are not high, even assuming the maximum FEC code rate.

Compared to 5G NR, one may design 6GR to be more radio resource efficient along any of the five dimensions. In one example, 6GR may be designed to reduce the overhead due to L1/L2 control, SSB, reference signals, etc., thus reducing the value of γOH in the radio RUE metric.

In another example, 6GR may support full duplex, which will increase Rduplex from 0.5 (FDD) to 1 (full duplex), and accordingly double the radio RUE.

Proposal 5: Evaluation metrics should include spectral efficiency related metrics: average spectral efficiency (bits/s/Hz/TRxP), 5th-percentile user spectral efficiency (bits/s/Hz), area traffic capacity (bits/s/m2), network energy efficiency (bits/s/W), and radio resource utilization efficiency (unit-less).

# Conclusion

In the previous sections, we discuss evaluation assumptions for 6GR air interface and make the following observations:

Observation 1: Energy consumption model of AI-native 6G base stations will be a complex function, reflecting both energy-savings from AI-driven network optimizations and the energy cost of AI algorithms execution.

Observation 2: 6G network traffic is expected to be uplink-heavy, bursty and highly dynamic with the uprise in immersive (e.g., XR/cloud gaming) and AI-driven applications.

Observation 3: Task Group IEEE 802.11bf has embraced ray tracing based channel model for Wi-Fi sensing.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Study energy consumption model of base stations in 6G during 6GR study, taking into consideration the energy savings from AI-driven network energy optimizations and the energy costs of running AI models.

Proposal 2: Study traffic models for performance evaluation during 6GR study taking into consideration the unique characteristics of UL-heavy immersive and AI applications related traffic.

Proposal 3: Besides stochastic channel model, deterministic physics-based channel model, especially ray tracing, should be considered for 6GR study.

Proposal 4: Consider the scenarios and parameters listed in Table 1 for evaluating technology proposals in 6GR study.

Proposal 5: Evaluation metrics should include spectral efficiency related metrics: average spectral efficiency (bits/s/Hz/TRxP), 5th-percentile user spectral efficiency (bits/s/Hz), area traffic capacity (bits/s/m2), and radio resource utilization efficiency (unit-less).

# Reference

X. Lin, “A Tale of Two Mobile Generations: 5G-Advanced and 6G in 3GPP Release 20,” arXiv preprint arXiv:2506.11828, June 2025.

RP-251881, “New SID: Study on 6G Radio,” 3GPP TSG RAN #108, June 2025.

3GPP TR 38.864, “Study on network energy savings for NR”, v18.1.0, Mar. 2023.

L. Kundu, X. Lin and R. Gadiyar, “Toward Energy Efficient RAN: From Industry Standards to Trending Practice”, IEEE Wireless Communications, vol. 32, no. 1, pp. 36-43, Feb. 2025.

3GPP TR 38.913, “Study on scenarios and requirements for next generation access technologies”, v18.0.0, Apr. 2024.

3GPP TR 26.926: “Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems”, v19.0.0, Mar. 2025.

3GPP TR 26.925, “Typical traffic characteristics of media services on 3GPP networks”. V19.0.0, Jun. 2025.

R1-2505519, “Overview of 6GR air interface”, NVIDIA, 3GPP TSG-RAN WG1 Meeting #122, Aug. 2025.

3GPP 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,” V19.0.0, June 2025.

IEEE 802.11bf, “Channel Models for WLAN Sensing Systems,” IEEE 802.11-21/0782r5.

J. Boccuzzi, “Spectral Efficiency Considerations for 6G,” Top of Form

- , arXiv: 2508.09117, 2025.