3GPP TSG RAN WG 1 Meeting #122	R1-2505796

Bangalore, India, August 25-29, 2025

Agenda Item:	11.4.2

Source:	Lenovo

Title:	Discussion on 6GR modulation

Document for:	Discussion

# 1	Introduction

In RAN#108, the 6G Radio (6GR) study item was discussed and approved [1]. It was agreed that 5G NR modulation schemes are the default for 6GR and will also be the benchmark for potential enhancements by other proposed modulation schemes. The study will focus on enhancements aimed at improving spectral efficiency, coverage and energy efficiency and target meeting diverse requirements for future deployments. 6GR modulation study will additionally evaluate the potential improvements in comparison to current modulation schemes, including a detailed assessment of their suitability for future use cases and deployment scenarios. The results of this evaluation, along with any proposed enhancements, are expected to be finalized, reviewed and assessed during RAN#112 as part of the ongoing 6GR study.

# 2	Discussion

## 2.1 Uniform constellations and modulation

Uniform constellations are used in 5G NR and considered the benchmark and default choice for 6GR as well, as per FS_6G_Radio study objectives [1]. These constellations are uniform both in terms of the functional mapping of bits to constellation symbols in the I/Q space, but also in terms of their constellation spacing and structures. Such structures benefit from reduced demodulation complexity given regular or simpler Voronoi detection regions over which the symbol-to-bits recovery (or de-mapping) is performed. In case of uniform Quadrature Amplitude Modulation (QAM) constellations another advantage of the regular structure is the fact that the I/Q components of the signal can be realized and processed in parallel given its orthogonal characterization derived as a cartesian product of two independent one-dimensional Pulse Amplitude Modulation (PAM) constellations. This benefits both the transmitter and the receiver modulation/demodulation blocks, respectively.

Despite the ease of realization of uniform constellations (e.g., the canonical M-QAM, BPSK or π/2-BPSK), these constellations can present signalling characteristics which do not always match well desired system attributes. For instance, despite high spectral efficiency potential, higher order QAM constellations suffer in terms of Peak-to-Average Power Ratio (PAPR) metric, further highlighted when used with waveforms such as CP-OFDM. Even more generally, discrete uniform constellations cannot by design approach optimal signalling over the AWGN channel, i.e., the canonical channel approximating the discrete input continuous output memoryless equivalent channel between the modulator and demodulator after channel equalization at the receiver. In fact, an asymptotic and deterministic gap in capacity relative to continuous input signalling is present for higher order QAM constellations, resulting in core spectral inefficiencies at PHY layer. As a consequence, it is generally beneficial to study and evaluate 6GR modulation enhancements over existent 5G NR modulation schemes. Candidate solutions should provide better figures in terms of spectral efficiency, PAPR/energy efficiency, and/or detection characteristics (e.g., LLR, BER, BLER), while preserving reasonable complexity and implementations feasibility.

Proposal 1: Study 6GR modulation enhancements aiming for spectral efficiency, PAPR and/or detection characteristics (e.g., BLER/BER, LLR) improvements, while maintaining manageable complexity and implementation feasibility.

## 2.2 Modulation Enhancement Areas

### 2.2.1 Improvements for IoT

Modulations with lower spectral efficiency include less dense constellation structures that accommodate lower PAPR characteristics. Desirable in this context are I/Q constellation points displaced on the same I/Q-plane ring, or set of rings, and thus sharing same power figures (i.e., magnitudes). In 5G NR such characteristics are shared by QPSK, π/2-BPSK and BPSK modulations. These modulations, alongside PAPR-friendly modulations (e.g., DFT-s-OFDM) can account for considerable reductions in PAPR figures, which in effect can be exploited on the transmitter side to operate in the linear region of the transmit power amplifier and effectively extend coverage while avoiding signal clippings or other signal processing artifacts.

In 6GR the cell-edge performance and coverage are expected to be improved over 5G NR. For narrowband IoT applications, which do not have high spectral efficiency requirements, but rather prefer energy efficiency, it is of interest to consider such PAPR-friendly modulations not only in UL (as is the case in NB-IoT and/or 5G NR PUSCH) but also in DL for PDSCH.

For example, utilizing π/2-BPSK in DL in addition to QPSK could provide additional PA-backoff, or alternatively, lower the SNR necessary to attain same BLER, which may be leveraged and optimized upon based on system requirements and operational scenarios. Despite potentially reduced spectral efficiency, these effects can thus be used to provide communications service at larger distances and lower rates while reducing the required number of repetitions/retransmissions. Figure 2.2.1-1 displays potential reductions in number of repetitions needed to achieve a target 10% BLER in downlink for MCL 154 dB. The π/2-BPSK is compared to QPSK at same spectral efficiency (i.e., at double the coding rate for π/2-BPSK case vs. QPSK case) over the DFT-S-OFDM waveform and realistic TDL-C channel. As illustrated, π/2-BPSK can reach 27%, respectively, 38% reduction in repetitions needed to achieve 10% target BLER when compared to QPSK for 6RBs and 12RBs, respectively. In effect, this allows receiver IoT devices to perform power savings when compared to QPSK or higher spectral efficiency modulation schemes, by either avoiding tuning in on retransmissions, and/or entering earlier a lower power state (and turning off the RF and low-power noise amplifier components).

Figure 2.2.1-1: Example of potential reductions in repetitions needed to achieve DL target 10% BLER between π/2-BPSK and QPSK with DFT-S-OFDM for MCL 154 dB.

Table 2.2.1-2: Simulation Parameters for Figure 2.2.1-1

| Parameter | Value |
| --- | --- |
| Channel Model | TDL-C |
| Delay Spread | 1000 ns |
| UE speed | 0 |
| MCS | Fixed to index 0 |
| Channel Coding Scheme | 5G LDPC w/ 24-bit CRC, code rates 0.234 (π/2-BPSK) and 0.117 (QPSK), respectively. |

It is worth noting that utilizing such PAPR-friendly modulation schemes is generally beneficial to narrowband (e.g., LPWA, low-bit rate voice service) communications over terrestrial networks (TN) and non-terrestrial networks (NTN) access.

Observation 1: Utilizing π/2-BPSK for DL narrowband transmissions could offer significant reductions in repetitions (e.g., ~27%-38% for 6RBs-12RBs) needed to achieve desirable target BLER (e.g., 10%) under extended coverage.

Proposal 2: Study and evaluate benefits (e.g., BLER, EE gains) vs. caveats (e.g., SE losses) of utilizing π/2-BPSK modulation in 6GR for DL narrowband extended coverage applications.

### 2.2.2 Improvements for high-throughput UEs

In case of high throughput UEs, with potential of operating at high SINR link budgets (e.g., FWA UEs), one design option to increase spectral efficiency for DL transmission is to utilize higher order QAM constellations, e.g., 4096QAM, going beyond 1024QAM used in 5G NR. In pure capacity terms, this would increase the bit-interleaved coded modulation (BICM) capacity by up to 2 bits per channel use per Hz starting from ~30 dB SNR in comparison to the existing 1024QAM modulation scheme. A complete overview of BICM capacity curves per SNR (dB) over all possible M-QAM modulation schemes with M = {4, 16, 64, 256, 1024, 4096} is  displayed in Figure 2.2.2-1.

The 4096QAM modulation therefore would open different opportunities, together with MIMO transmissions, to operate at higher spectral efficiency than possible before, provided that the link SINR is high and relatively stable (i.e., not highly affected by temporal fading variations). Such conditions may be common in applications specific to residential FWA, where capacity and throughput are core KPIs and where the UE is not mobile and has high processing and/or MIMO capabilities. For such scenarios, or equivalent deployment use cases, it is therefore advisable to consider utilizing 4096QAM constellation signalling in DL transmissions.

Proposal 3: Study for high throughput and high capabilities UEs (e.g., residential FWA) 4096QAM modulation for DL transmissions and characterize its realistic performance (incl. BLER, BER, LLR) and achievable spectral gains over existing 5G NR schemes.

Figure 2.2.2-1: BICM capacity for Gray-coded M-QAM constellations with M = {4, 16, 64, 256, 1024, 4096}. Shannon limit for AWGN channel with continuous inputs and continuous outputs is also plotted as reference for the theoretical maximum achievable rate.

### 2.2.3 Constellation shaping

BICM is the state-of-the-art approach for combining coding and digital modulation for transmission over fading channels. BICM decouples the modulation constellation from the choice of coding and coding rate. The BICM framework consists of serial concatenation of a channel code, a bit interleaver and a modulation mapper, allowing the transmitter various operational choices (i.e., MCS values) to trade-off between capacity and reliability. Uniform QAM constellations are employed by 5G NR (and previous 3GPP generations) in BICM-style. These uniform constellations offer computational simplicity but can result in notable losses in spectral efficiency and error rate performance (e.g., BER, BLER).

It is important to note that the AWGN channel is still a good approximation for the equivalent channel observed at the receiver post-equalization, whereby the fading channel effects are resolved based on channel estimation and any outstanding residuals due to equalization errors can be further approximated on average as Gaussian, as well. In OFDM systems, each subcarrier can be modeled as experiencing an AWGN channel in the frequency domain, which supports the AWGN approximation in per-subcarrier analysis.

When uniform QAM constellations are used over AWGN channels, the SNR loss in spectral efficiency converges, asymptotically, to 1.53 dB, since the discrete and uniform nature of the constellation points do not well approximate the optimal Gaussian signaling structure for higher order modulations [2]. This (horizontal) gap is observed in Figure 2.2.2-1, whereas its vertical interpretation, i.e., at a fixed SNR point, is displayed in Figure 2.2.3-1 outlining an envelope gap to capacity ranging from 0.033 to 0.55 bits/s/Hz over the interval 0 to 35 dB across 4QAM-to-4096QAM uniform constellations. These gaps are in nature theoretical and in practical systems the experienced absolute performance is slightly less for various reasons such as non-linear components, non-Gaussian residuals, HW imperfections, arithmetic approximations, quantization losses which are not well captured or modeled canonically. Still, the theoretical values represent good indicators and provide guidance in designing practical systems.

Observation 2: When designing discrete constellations, the key objective is to approach the theoretical limit defined by the Shannon capacity. Additional criteria may include reliability metrics (e.g., BER/BLER) and power/energy efficiency measures (e.g., PAPR).

Observation 3: Conventional uniform QAM constellations have an asymptotic theoretical gap to AWGN capacity of up to 1.53dB, or alternatively, a loss of up to 0.55 bits/s/Hz. As the modulation order increases, the losses become higher.

Figure 2.2.3-1: Capacity gap to Shannon’s AWGN channel capacity limit for Gray-coded uniform M-QAM modulations with M = {4, 16, 64, 256, 1024, 4096}.

Shaping (i.e., geometric shaping and/or probabilistic shaping) of signaling constellations is a well-known strategy to improve reliability and spectral efficiency over the uniform QAM constellations and to reduce the gap to the theoretically possible channel capacity.

#### 2.2.3.1	Geometric shaping

Geometric shaping refers to changing locations of the discrete constellation points on the I-Q plane, yielding Non-Uniform Constellations (NUCs). Deviating from standard geometries (e.g., square and rectangular M-QAM, circular M-PSK) might improve the mutual information between the channel input and its output and the reliability of information transfer. With NUCs, the SNR gains in spectral efficiency can be up to 1dB with various caveats and added complexity when compared to the conventional rectangular/square QAM over AWGN channels. Such NUCs may, however, affect other modules/functions at the transmitter (such as PAPR) and may require re-designing of the signal processing algorithms at the receiver (e.g., the LLR computation module, the symbol detection and/or the symbol demapper).

There are various approaches to NUC dependent on the design space and the available degrees of freedom. 1D-NUC assumes modifying the constellation geometry only along one axis (e.g., in PAM constellation domain) and generates an NUC based on the cartesian product of the two modified one-dimensional constellations. This yields geometrically square or rectangular NUC QAM constellations. On the other hand, 2D-NUC provides two-dimensional degrees of freedom and allows NUC optimization in both I and Q directions, potentially yielding non-square/non-rectangular constellations and allowing for higher gains in spectral efficiency. However, the gains with 2D-NUC come at the cost of added complexity and lack of parallelized processing capabilities per direction, since the demapper at the receiver needs to treat the I and Q components jointly to perform symbol detection, demapping and/or LLR computation.

#### 2.2.3.2	Probabilistic shaping

Probabilistic shaping refers to shaping the probability mass function (PMF) of the discrete constellation points. This technique preserves the original constellation’s geometry as the conventional square/rectangular QAM but alters the probabilities of the constellation points non-uniformly. Applying probabilistic shaping, the SNR gains can be up to 1.4dB compared to conventional (equiprobable) QAM over AWGN channels. However, optimizing the discrete probability distribution (i.e., PMF) for improving capacity is a complex task, since it requires additional functionality/modules to be introduced at the transmitter and the receiver, e.g., a distribution matcher at the transmitter and a distribution dematcher at the receiver. Despite the additional complexity and blocks in the transceiver processing chain, one advantage of probabilistic shaping over geometric shaping is that the signal constellations and symbol detection circuitry/logic can be preserved as in conventional M-QAM or M-PSK.

#### 2.2.3.3	Evaluations for geometric and probabilistic shaping

First, spectral efficiency can be improved by geometric and/or probabilistic shaping. Figure 2.2.3-2 illustrates spectral gains achieved by a 256-point NUC designed via geometric shaping and a 256-point PS-QAM (i.e., a 256QAM whose PMF is optimized through probabilistic shaping) against a conventional uniform 256QAM. It can be observed that PS can almost attain the AWGN capacity, while NUCs achieve approximately half of the gains achieved by PS-QAM. Note that the performance by NUC and PS-QAM slightly degrades in high SNR regions. It is expected that, higher-order modulations, such as an NUC or a PS-QAM with 1024 points or 4096 points, can provide greater gains to more closely approach to the AWGN capacity at high SNR values (18dB and above in the figure).

Figure 2.2.3-2: Mutual information obtained by conventional rectangular QAM, geometrically shaped non-uniform constellation (NUC), and probabilistically shaped QAM (PS-QAM) for M=256 symbols. “Continuous-input capacity” denotes the AWGN channel capacity with continuous inputs.

Furthermore, communication reliability, e.g., block error rate (BLER), can be greatly enhanced by geometric and/or probabilistic shaping. Table 2.3.3-1 shows the simulation setup used to obtain BLER performance achieved by geometric shaping, denoted as NUC, and probabilistic shaping, denoted as PS-QAM, over AWGN channels. The baseline is the 5G NR uniform QAM constellation. Three different spectral efficiencies are considered, 3.02, 4.52, and 5.33 corresponding, respectively, to the MCS indices 12, 17, and 20 in Table 5.1.3.1-2 of 3GPP TS 38.214 [3]. For SE of 3.02 and 4.52, a 64-point constellation is used, while for SE of 5.33, a 256-point constellation is used. For simulations, two different block lengths, i.e., 1000 bits and 4000 bits, are considered.

Figure 2.2.3-3: The probabilistic amplitude shaping (PAS) framework [5] used for PS-QAM in simulations.

For NUCs, 1D-NUC is used for constellation geometry. For SE=[3.02, 4.52, 5.33], NUCs are optimized at the target SNR=[11,15,18] dB, respectively. For PS-QAM, the constant composition distribution matching (CCDM) [4] is used to obtain a desired symbol probability. The probabilistic amplitude shaping (PAS) framework [5], as shown in Figure 2.2.3-3, is considered wherein the red boxes denote the additional modules compared to the conventional transmit-receive chain using (equiprobable) uniform QAM constellations. The amplitudes of real and imaginary axes are shaped according to a desired amplitude probability.  The bit ordering module at the transmitter ensures that the parity bits and some of the systematic bits are used as sign bits, and the LLR de-ordering module at the receiver recovers the ordering. The FEC coding rate is fixed at 0.8485. The symbol distribution is as px∝exp-vx2 to mimic the optimal Gaussian signaling, where x  is a symbol of a PAM constellation. The spectral efficiency is adjusted by varying the design parameter v . For SE=[3.02, 4.52, 5.33], v =[0.136, 0.036,0.02] is set.

Figure 2.2.3-4 shows the BLER curves for three different spectral efficiencies, each evaluated at two different block lengths. Compared to the conventional uniform QAM, the SNR gains of PS-QAM ranges between 0.6dB and 1.3dB, while the gains of NUC is about 0.2dB to 0.5dB. There is room for further improvement in performance achieved by NUC and PS-QAM. The performance of PS could be further improved by jointly adjusting v and the FEC coding rate appropriately. Also, the performance of NUC can be further enhanced by using higher degrees of design freedom in constellation geometry, such as 2D-NUC.

Table 2.2.3-1: Simulation parameters for uniform QAM, NUC, and PS-QAM

| Variable | Value |
| --- | --- |
| Mother uniform QAM constellation (M) | 64QAM, 256QAM (Gray-coded) |
| Spectral efficiency (SE) | [3.02, 4.52, 5.33] |
| Channel coding | LDPC NR (Belief propagation decoding with 20 iterations) |
| Block length | [1000, 4000] bits |
| NUC geometry | 1D-NUC |
| Target SNR for NUC design | [11, 15, 18] dB |
| FEC coding rate in PS-QAM | 0.8485 (fixed) |
| Design parameter v in PS-QAM | [0.136, 0.036, 0.02] |

Figure 2.2.3-4: BLER performance obtained by uniform QAM, geometrically shaped NUC, and probabilistically shaped QAM (PS-QAM) for achieving different spectral efficiencies of [3.02, 4.52, 5.33] from left to right. The top three figures show results for a blocklength of k=1000 bits, while the bottom three figures correspond to k=4000 bits.

It is also worth noting the fact that additional constraints could be included in the shaping designs which improve other figures of merit for the shaped constellations. Such constraints may relate to constrained unitary power (i.e., typical constraint associated with digital modulation schemes), to bounded minimum distance (which can improve/characterize detection, i.e., BER/BLER, limits associated with a set of constellation symbols), to bounded PAPR (which can be a coverage enhancement enabler) and to others.

Observation 4: Constellation shaping (i.e., geometric and/or probabilistic shaping) can provide gains by approaching the theoretical spectral efficiency limits and improving the reliability while satisfying other desirable attributes of signal constellations.

From the results in Figures 2.2.3-2 and 2.2.3-4, both geometric and probabilistic shaping provide spectral efficiency gains and improve reliability (i.e., BLER) over AWGN channels. Since the AWGN channel is still a good approximation for the equivalent channel observed at the receiver in OFDM systems, similar gains can be expected in these systems as well.

To achieve the gains, however, both geometric and probabilistic shaping require additional modules/complexity in the transmit-receive chain. Geometric shaping requires changing the constellation set at the transmitter and receiver to the desired NUCs and the corresponding mapping and demapping operations. Probabilistic shaping causes greater impact, relative to the geometric shaping, on the transceiver processing chain, as it requires a probabilistic matcher module at the transmitter and a probabilistic dematcher module at the receiver (or equivalent combinations of other modules/functionalities) to realize a desired PMF.

Therefore, it is critical to study and determine the appropriate trade-offs between complexity and performance when realizing geometric and/or probabilistic shaping schemes. This can ensure sustainable performance gains with implementation-friendly techniques.

Observation 5 Realizing constellation shaping (i.e., geometric and/or probabilistic shaping) requires introducing additional modules in the transmit-receive chain and/or increasing the computational complexity of the transceiver.

Proposal 4: Study and evaluate trade-offs between performance gains and implementation complexity associated with constellation shaping. The performance metrics can include spectral efficiency, reliability (e.g., LLR, BLER/BER), and energy efficiency/power attributes (e.g., PAPR). Characterize transceiver design impacts, such as the need for additional modules and/or modification of existing processing blocks.

### 2.2.4 AI/ML approaches

The discussion on constellation shaping provided here also serves as a motivating factor for considering AI-based constellation shaping as a candidate use case for AI/ML-based PHY optimization. AI/ML-based constellation shaping may be used to unlock further gains and/or reduce complexity observed in the present contribution, given the requirements and needs for successful realizations of geometric shaping (i.e., NUCs), probabilistic shaping, or both. For further details on the scope and potential objectives on AI/ML-based constellation shaping, please refer to our contribution on agenda item 11.6 [6].

# 3	Conclusions

Modulation is a critical PHY layer component and further study for 6GR of its potential enhancements is worthwhile. The following observations and study proposals are deemed essential:

Proposal 1: Study 6GR modulation enhancements aiming for spectral efficiency, PAPR and/or detection characteristics (e.g., BLER/BER, LLR) improvements, while maintaining manageable complexity and implementation feasibility.

Observation 1: Utilizing π/2-BPSK for DL narrowband transmissions could offer significant reductions in repetitions (e.g., ~27%-38% for 6RBs-12RBs) needed to achieve desirable target BLER (e.g., 10%) under extended coverage.

Proposal 2: Study and evaluate benefits (e.g., BLER, EE gains) vs. caveats (e.g., SE losses) of utilizing π/2-BPSK modulation in 6GR for DL narrowband extended coverage applications.

Proposal 3: Study for high throughput and high capabilities UEs (e.g., residential FWA) 4096QAM modulation for DL transmissions and characterize its realistic performance (incl. BLER, BER, LLR) and achievable spectral gains over existing 5G NR schemes.

Observation 2:  When designing discrete constellations, the key objective is to approach the theoretical limit defined by the Shannon capacity. Additional criteria may include reliability metrics (e.g., BER/BLER) and power/energy efficiency measures (e.g., PAPR).

Observation 3:  Conventional uniform QAM constellations have an asymptotic theoretical gap to AWGN capacity of up to 1.53dB, or alternatively, a loss of up to 0.55 bits/s/Hz. As the modulation order increases, the losses become higher.

Observation 4: Constellation shaping (i.e., geometric and/or probabilistic shaping) can provide gains by approaching the theoretical spectral efficiency limits and improving the reliability while satisfying other desirable attributes of signal constellations.

Observation 5: Realizing constellation shaping (i.e., geometric and/or probabilistic shaping) requires introducing additional modules in the transmit-receive chain and/or increasing the computational complexity of the transceiver.

Proposal 4:  Study and evaluate trade-offs between performance gains and implementation complexity associated with constellation shaping. The performance metrics can include spectral efficiency, reliability (e.g., LLR, BLER/BER), and energy efficiency/power attributes (e.g., PAPR). Characterize transceiver design impacts, such as the need for additional modules and/or modification of existing processing blocks.

# References

[1] RP-251881: 3GPP Study on 6G Radio (FS_6G_Radio), Status: Approved at 3GPP RAN Plenary RAN#108

[2] J.M. Cioffi, Discrete Data Transmissions – Chapter 1: Modulation and Canonical Reception, available at: chap1.pdf, retrieved: July 2025.

[3] 3GPP TS 38.214, Version 16.2.0, Release 16, Jul 2020.

[4] P. Schulte and G. Böcherer, "Constant Composition Distribution Matching," in IEEE Transactions on Information Theory, vol. 62, no. 1, pp. 430-434, Jan. 2016, doi: 10.1109/TIT.2015.2499181.

[5] G. Böcherer, F. Steiner and P. Schulte, "Bandwidth Efficient and Rate-Matched Low-Density Parity-Check Coded Modulation," in IEEE Transactions on Communications, vol. 63, no. 12, pp. 4651-4665, Dec. 2015, doi: 10.1109/TCOMM.2015.2494016.

[6] R1-2505690, Discussion on AI/ML in 6GR, Source: Lenovo, Status: Available for RAN1#122.