3GPP TSG RAN WG1#122                                                              R1-2505808

Bengaluru, India, Aug 25th– 29th, 2025

Agenda Item:	8.2.1

Source:	Lenovo

Title:	Maintenance on UE-initiated/event-driven beam management

Document for:	Discussion

Introduction

In this contribution, we provide our views on the remaining issues of UE initiated beam measurement.

Discussion

L1-RSRP reporting

Regarding the L1-RSRP report format for different events, if RRC parameter enabledCurrentBeamReport is configured for event2 or event2, the L1-RSRP for the current beam is reported by a differential RSRP as specified in TS38.212, which is aligned with the agreement achieved in RAN1 by an assumption that the quality of the current beam doesn’t have the highest L1-RSRP when event2 or event7 is triggered. However, the CSI reference resource for the beam report for UEIBM is determined by the triggering DCI for mode A or by the second PUSCH carrying the beam report for mode B. Given that, the determine CSI reference resource for the UE to obtain the L1-RSRP for the beam report may be later than the slot for the first PUCCH transmission to trigger the beam report. Thus, it should be possible that the current beam has the highest L1-RSRP in the determined CSI reference resource in the beam report. The current differential RSRP scheme for current beam cannot be used for this case. The same situation may be happened for event1, for which the absolute L1-RSRP for current beam is directly quantized to a 7-bits value other than a differential value. Thus, we proposed to extend this scheme for event2 and event7.

Proposal 1: For the L1-RSRP report for event2 and event7, the L1-RSRP for current beam is absolute L1-RSRP other than a differential RSRP.

DCI for PUSCH scheduling for UEI beam report in mode A

For mode A of a UE initiated beam report, the gNB can send a DCI format 0_1/0_2 if a UE has transmitted a UEIRI in the first PUCCH resource to schedule a PUSCH for the beam report transmission, where the CSI request field indicate a CSI trigger state associated with UEI beam report configuration(s). In Rel-18, DCI format 0_1/0_2 can trigger one or more aperiodic CSI report by using a DCI scrambled by C-RNTI, and can activated one or more semi-persistent CSI report by using a DCI scrambled by SP-CSI-RNTI. For the prior case, the CSI request field codepoint is mapped with a CSI-AperiodicTriggerState, while for the latter case, the CSI request field codepoint is mapped with SP-CSI triggering state. Either of the DCI can schedule a corresponding PUSCH. During the whole discussion, we assume that the legacy DCI for aperiodic CSI triggering is used in the step 2 of mode A procedure of UEIBM. Thus, it should be restricted that the DCI format 0_1/0_2 in mode A should be scrambled by C-RNTI.

Proposal 2: DCI format 0_1/0_2 in step 2 of mode A procedure of UEIBM in response to the UEIRI is scrambled by C-RNTI.

CPU occupation for UEI beam report

In RAN1#121, it was agreed that OCPU = 1 is occupied for a CSI report configuration for UEIBM, but the occupation time is not specified. It was agreed that all the resources within the newBeamResourceSet should have a same periodicity with the RS for the current beam. For UEI beam report, the UE shall perform L1-RSRP measurement on each transmission occasion of the resources within  newBeamResourceSet as well as each transmission occasion of the resource corresponding to the current beam. Thus, the CPU should be periodically occupied for each transmission occasions of those resources.

Proposal 3: UE periodically occupies one CPU from the first symbol of the earliest one of each transmission occasion of periodic CSI-RS/SSB resources within  newBeamResourceSet and the periodic CSI-RS/SSB resource corresponding to the current beam, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resources within  newBeamResourceSet and the periodic CSI-RS/SSB resource corresponding to the current beam.

CRI/SSBRI k definition in a CSI report for UEIBM

Reason for change: CRI and SSBRI in CSI report for UEIBM are not defined.

Summary of changes: Introduce the definition of CRI and SSBRI in CSI report for UEIBM.

Consequences if not approved: UE does not know how to determine the value of CRI or SSBRI in a CSI report for UEIBM.

Proposal 4: Adopt the following TP for TS38.214V19.0.0

<omitted text> 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report <omitted text>

Conclusion

In this contribution, we provide the following proposals:

Proposal 1: For the L1-RSRP report for event2 and event7, the L1-RSRP for current beam is absolute L1-RSRP other than a differential RSRP.

Proposal 2: DCI format 0_1/0_2 in step 2 of mode A procedure of UEIBM in response to the UEIRI is scrambled by C-RNTI.

Proposal 3: UE periodically occupies one CPU from the first symbol of the earliest one of each transmission occasion of periodic CSI-RS/SSB resources within  newBeamResourceSet and the periodic CSI-RS/SSB resource corresponding to the current beam, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resources within  newBeamResourceSet and the periodic CSI-RS/SSB resource corresponding to the current beam.

Proposal 4: Adopt the following TP for TS38.214 v19.0.0

<omitted text> 5.2.1.5.4.1a UE Initiated CSI reporting for event 2 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1b UE Initiated CSI reporting for event 1 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRPs, and, when enabledCurrentBeamReport is configured, the absolute L1-RSRPs, corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report <omitted text> 5.2.1.5.4.1c UE Initiated CSI reporting for event 7 <omitted text> After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212] in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding NZP-CSI-RS-ResourceSet configured for newBeamResourceSet. SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet configured for newBeamResourceSet. For each CRI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured a condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals, or to the SS/PBCH block which is QCLed with the reference signal with the valueOfQ highest L1-RSRP out of the activated TCI state reference signals. The UE sends the CSI report <omitted text>

References

- TS38.214 v19.0.0