3GPP TSG RAN WG1 #122	R1-2506162

Bangalore, India. 25 – 29 August. 2025

Agenda item:		8.2.4

Source:	Nokia

Title:	Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Document for:		Discussion and Decision

# Introduction

# The 3GPP RAN plenary RAN#105 has amended the MIMO WID [1] and included two TA enhancements as part of the Rel-19 work item. The amended Rel-19 MIMO Phase 5 WID is in RP-242394 [2]. RAN1 completed the rel 19 work item in RAN1 #121and most of the main issues were addressed. From RAN1-#122 it will be in the maintenance phase and address left over minor issues.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

# Discussion

The UE may transmit PRACH for various reasons including PDCCH-order PRACH, RLF recovery, Re-synchronization, HO, etc. In the Asymmetric DL-sTRP UL-mTRP scenarios we only agreed CFRA PDCCH order PRACH by associating it with the PL-Offset towards the UL-Only TRP. It is good to clarify whether other PRACH types can be used with the PL-offset towards the UL-only TRP

Proposal 1: Clarify whether the PL-Offset towards UL-Only TRP can be applied to other RACH types.

In the asymmetric DL sTRP / UL mTRP scenario, as there is no DL transmission from UL-Only TRP, the UE will not have a DL reference timing based on an RS sent by the UL-only TRP. The error on the transmission timing of the PRACH preamble may be too large (larger than CP length) in some deployments, and may cause a) interference, b) the network not being able to detect such RACH preamble, and c) an increase in the PRACH latency. To avoid such situations, we are of the opinion that an explicit indication of the predefined timing offset from the DL and UL capable TRP to UE is beneficial.

Proposal 2. In the asymmetric DL sTRP / UL mTRP scenario when UE uses one reference timing for two links, the timing error in UE to UL-Only TRP link may be larger. In such a situation, we propose to have an indication of the timing offset value from the DL and UL capable TRP to UE.

In the asymmetric DL sTRP / UL mTRP scenario, The UL-Only TRP doesn’t transmit any signals in the DL. The UL/DL capable TRP indicates the pathloss offset to UE for applying more accurate pathloss towards the UL-Only TRP. In some scenarios The UL-only TRP and DL/UL capable TRP may operate in different frequency resources (CC or BWP, etc.). In such a case, a frequency correction needs to be applied to the pathloss offset to make it more accurate. We propose to add a note or clarification in TS 38.213

Proposal 3. In the asymmetric DL sTRP / UL mTRP scenario, if the DL/UL TRP and UL-Only TRPs are operating in different frequencies, for example CC or BWP, the corresponding frequency-dependent correction will be applied to the PL-Offset value.

# Conclusion

In this contribution, we make the following proposals. Please consider adopting them.

Proposal 1: Clarify whether the PL-Offset towards UL-Only TRP can be applied to other RACH types.

Proposal 2. In the asymmetric DL sTRP / UL mTRP scenario when UE uses one reference timing for two links, the timing error in UE to UL-Only TRP link may be larger. In such a situation, we propose to have an indication of the timing offset value from the DL and UL capable TRP to UE.

Proposal 3. In the asymmetric DL sTRP / UL mTRP scenario, if the DL/UL TRP and UL-Only TRPs are operating in different frequencies, for example CC or BWP, the corresponding frequency-dependent correction will be applied to the PL-Offset value.

# References

- RP-234007	New WID: NR MIMO Phase 5

[2]  RP-242394 The amended Rel-19 MIMO Phase 5 WI