3GPP TSG RAN WG1 #122						R1-2506192

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.11.5

Source: 	Qualcomm Incorporated

Title: 	IOT-NTN TDD mode

Document for:	Discussion

# Issue#1: Availability of NRS during initial access

Agreement

Confirm the working assumption of RAN1#120b with the following modifications

DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) does not apply to a NB-IoT NTN TDD cell.

- NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes.

- NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

- Up to RAN2 to decide whether DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is constrained to not being present in a NB-IoT NTN TDD cell, or it can be configured with values all set to ‘1’.

In RAN1#121, the following agreement was reached:

This agreement guarantees that NRS is present in all the D-subframes not carrying sync signals. Current TS 36.213, however, still does not allow the UE to assume NRS is present in all D-subframes before acquiring MIB or SIB1.

Before a UE obtains operationModeInfo:

-	If frame structure type 1 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #9 and in subframes #0 not containing NSSS.

On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating guardband or standalone.

-	If frame structure type 1 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, and in subframes #0 not containing NSSS, and in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions.

-	If frame structure type 1 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes.

-	If frame structure type 2 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, subframes #0 not containing NSSS, in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions, and in NB-IoT downlink subframes.

Before a UE obtains operationModeInfo:

-	If frame structure type 1 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #9 and in subframes #0 not containing NSSS.

On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating guardband or standalone.

-	If frame structure type 1 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, and in subframes #0 not containing NSSS, and in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions.

-	If frame structure type 1 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes.

-	If frame structure type 2 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, subframes #0 not containing NSSS, in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions, and in NB-IoT downlink subframes.

According to the text above, the UE can only assume that NRS are transmitted in subframes 0, 4 and 9 not containing NSSS. This contradicts the agreement above (if downlinkBitmap / downlinkBitmapNonAnchor cannot be configured, then the UE should be able to assume that NRS is present in all D-subframes not containing NPSS or NSSS. Therefore, we make the following proposal:

Proposal 1: Before acquiring operationModeInfo and before obtaining SystemInformationBlockType1-NB, the UE may assume NRS is present in all D-subframes not carrying sync signals.

A TP for this proposal is shown below:

Before a UE obtains operationModeInfo:

-	If frame structure type 1 is used in FDD, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #9 and in subframes #0 not containing NSSS.

-	If frame structure type 1 is used in NTN TDD, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0, #3, #4, #6, #7, #8 and in subframes #9 not containing NSSS within the D consecutive downlink subframes according to the TDD pattern.

On an NB-IoT carrier for which a UE receives higher-layer parameter operationModeInfo indicating guardband or standalone.

-	If frame structure type 1 is used in FDD, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4 and in subframes #9 not containing NSSS.

-	If frame structure type 2 is used, before the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, and in subframes #0 not containing NSSS, and in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions.

-	If frame structure type 1 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #0, #1, #3, #4, subframes #9 not containing NSSS, and in NB-IoT downlink subframes.

-	If frame structure type 2 is used, after the UE obtains SystemInformationBlockType1-NB, the UE may assume narrowband reference signals are transmitted in subframes #9, subframes #0 not containing NSSS, in subframes #4 if subframes #4 is configured for SystemInformationBlockType1-NB transmissions, and in NB-IoT downlink subframes.

-	If frame structure type 1 is used in NTN TDD, the UE may assume narrowband reference signals (NRSs) are transmitted in subframes #0, #3, #4, #6, #7, #8 and in subframes #9 not containing NSSS within the D consecutive downlink subframes according to the TDD pattern.

# Issue#2: Non-anchor carriers

RAN1 did not discuss in detail how to handle the NRS availability and D-subframes in non-anchor carriers. The first agreement (which should be obvious based on previous discussions in RAN1) is that the TDD structure and timing in the non-anchor carrier is the same as that in the anchor carrier. Therefore, the UE can derive the 90ms TDD structure in the non-anchor carrier, based on its observations of downlink signals from the anchor carrier and the fact that the D-subframes are transmitted at the same time in both the anchor carrier and the non-anchor carrier.

Proposal 2: The set of D-subframes in the non-anchor carrier is the same as the set of D-subframes in the anchor carrier, and is transmitted at the same time.

Regarding NRS, in addition to the issues described in the section above, there are some specifics only for non-anchor carriers during random access and paging. For instance, there are the following special cases:

- In a non-anchor carrier for paging, if nrs-NonAnchorConfig is configured, the UE can assume that NRS is available in some subframes around the NPDCCH search space.

- In a non-anchor carrier for paging, if nrs-NonAnchorConfig is not configured, the UE can only assume NRS is transmitted when there is a paging DCI.

- In a non-anchor carrier for random access, the UE can only assume NRS is transmitted in some subframes around the NPDCCH search space.

For simplicity, we propose to adopt for the non-anchor carrier the same approach as in the anchor carrier, and allow the UE to assume that NRS is present in every D-subframe.

Proposal 3: In a non-anchor carrier, the UE can assume NRSs are transmitted in subframes #3, #4, #5, #6, #7, #8, #9, #0 contained within the set of D=8 usable consecutive downlink subframes in the TDD structure

# Summary

In this contribution we provided our views on remaining issues for IOT NTN TDD. We made the following proposals:

Proposal 1: Before acquiring operationModeInfo and before obtaining SystemInformationBlockType1-NB, the UE may assume NRS is present in all D-subframes not carrying sync signals.

Proposal 2: The set of D-subframes in the non-anchor carrier is the same as the set of D-subframes in the anchor carrier, and is transmitted at the same time.

Proposal 3: In a non-anchor carrier, the UE can assume NRSs are transmitted in subframes #3, #4, #5, #6, #7, #8, #9, #0 contained within the set of D=8 usable consecutive downlink subframes in the TDD structure