3GPP TSG-RAN WG1 Meeting #122	Tdoc R1-2506256

Bengaluru, India, August 13th – 18th, 2025

Agenda Item:	10.5.1

Source:	Ericsson

Title:	Discussion on ISAC for NR

Document for:	Discussion, Decision

# Introduction

During RAN1#108, the SID for sensing in 5G Advanced was agreed [1]. RAN1 and RAN3 objectives are as follows.

Justification: TR 22.837 identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137, in particular, detection and/or tracking of UAV (Uncrewed Aerial Vehicles). SA2 also has an ongoing Rel-20 study on Architecture Enhancement to support Integrated Sensing and Communication (FS_Sensing_ARC, SP-250401). Objectives: Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In this paper, we provide our views on suitable performance evaluation metrics and measurements. In this paper, we also discuss the candidate sensing solutions to obtain final sensing results and reference signals including the implications of using non existing DL NR waveforms or reference signals. Lastly, we provide our views on evaluation methodology and assumptions.

# Performance metrics and requirements

NR sensing performance metrics and requirements have been studied in SA and are summarized in Table 6.2-1 in TS 22.137 [2] and copied below for convenience. Note that 6G sensing performance requirements are being studied in both ITU and a RAN-led SI, with different performance metrics and definitions. Definitions of performance metrics in TS 22.137 and the ITU working document are added in the Annex. We explore the performance metrics in more detail in this section.

Table 6.2-1: Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 [3] | 5 [3] | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 0.5 | 0.5 x 0.5 for factories | 100 to 5000 | 0.1 | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |
| NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. NOTE 6: Category 7 has more stringent requirements (e.g., for KPIs such as positioning accuracy and sensing resolution) compared to other categories and typically requires more radio resources. |  |  |  |  |  |  |  |  |  |  |  |  |  |

We consider a system of ISAC capable gNBs, where each gNB reports a set of sensing results. A sensing result is a pair (p, v) of one three-dimensional position p and one three-dimensional velocity vector v, describing a detected target’s position and velocity. The quality of the overall set P of sensing results is evaluated using the metrics described in Table 6.2-1 and will be discussed in the following paragraphs.

When defining the details of these metrics, the targets and sensing results must be associated in order to evaluate error magnitudes etc. Below, we propose a simple and unified method for this association that is independent of any algorithmic consideration that is being evaluated.

The estimation error of the position of target n is the difference between the true position xₙ of the target and the closest reported position pₙᵐⁱⁿ∈P:

eₙ = pₙᵐⁱⁿ - xₙ,

where

pₙᵐⁱⁿ = argmin_p∈P {||p - xₙ||}

A target n is said to be detected if its estimation error ||eₙ|| ≤ R is smaller than a given detection radius R. The detection rate is then the number of detected targets divided by the number of targets. The missed detection rate is one minus the detection rate. To obtain a statistically stable detection rate, the detection rates of multiple drops need to be averaged.

The false alarm rate is defined in [2]. A false alarm is triggered when more than zero positions are reported in a drop without targets present. The false alarm rate is then the number of drops where a false alarm is triggered divided by the total number of drops without targets present.

Because false alarm rate fails to capture the ability of a sensing system to avoid false reports in the presence of targets, we suggest introducing a new metric that captures this. Phenomena, such as reflections of targets and sidelobes of the reference signal, can cause a simplistic sensing algorithm to report spurious targets at erroneous positions. Furthermore, a system that is optimized to reduce false alarm rate and improve detection rate and accuracy might also overreport positions when it is certain that targets are present, because when targets are present the false alarm rate is not affected by overreporting. For these reasons, we suggest also evaluating sensing systems based on their false reporting rate that is defined as the number of reported positions that are outside any detection sphere (the sphere around the true target position, whose radius equals the detection radius) divided by the total number of reported positions.

The horizontal and vertical positioning error of a target n is obtained by projecting the error vector onto the horizontal plane and the normal of the horizontal plane respectively. If the matrix H=(𝐞, 𝐧)ᵀ, where 𝐞 is the unit vector pointing towards east and 𝐧 the unit vector pointing towards north, describes the projection onto the horizontal plane, the horizontal error is given by:

Δhₙ = ||Heₙ||

If the vector 𝐮 = 𝐞 ⨯ 𝐧 is the unit-length normal of the horizontal plane, the vertical error is given by:

Δvₙ = |𝐮ᵀ eₙ|

When computing the accuracy of the sensing results, estimation errors associated with detections are studied.  Hence, define the set of detected targets:

D = {n: ||eₙ|| < R}

The horizontal accuracy is then defined in terms of the set that contains the horizontal errors from the detected targets:

{Δhₙ: n∈D}

For example, for a 95% confidence level, the horizontal accuracy is the 95-percentile of the samples in {Δhₙ: n∈D}, i.e. the smallest value Δh for which 95% of the horizontal errors of detected targets are Δhₙ ≤ Δh. To gain statistical significance, the sample distribution should be based on samples from multiple drops.

The vertical accuracy is defined in the same way, just in terms of the sample distribution of the vertical errors: {Δvₙ: n∈D}.

The velocity error of target n is defined as

τₙ = vₙᵐⁱⁿ -vₙ,

where vₙᵐⁱⁿ is the velocity estimate associated with the closest position estimate pₙᵐⁱⁿ.  Similar to the positional error, the horizontal and vertical velocity errors are the following projections:

Δηₙ = ||Hτₙ||

Δβₙ = |𝐮ᵀ τₙ|

The range and velocity resolution are proportional to the inverse of the used bandwidth and the used coherent processing interval respectively. Specifically, the range resolution is c/B, where c is speed of light and B the used bandwidth. The velocity resolution is λ/T, where λ is the wavelength of the signal and T the coherent processing interval.

We note that the metrics depend on the definition of a detection and, specifically, on the detection radius.  While a discussion is needed on what detection radius is suitable, we would like to point out that it would make sense to choose a detection radius on the same order of magnitude as the desired range resolution.

Evaluating the sensing latency is complicated by several factors and we propose to remove latency as a performance metric. The fact that tracking allows the sensing system to predict the target location ahead of time might lead to negative latency, which is confusing without additional specification on what kind of prediction ahead of time is allowed. Furthermore, latency is affected by factors outside RAN1 specification work, such as hardware choices and queuing times etc. Finally, higher layer latency is historically much larger than physical layer latency and tends to largely dominate the overall system latency.

Define detection in terms of a detection radius and define the detection of a target as the event that at least one reported position is within the detection sphere whose center is the true position of the target and whose radius is the detection radius.

Associate each target with the sensing result whose position estimate is the closest to its true position and evaluate position and velocity accuracies based on the error vectors corresponding to this association and only include error vectors corresponding to detected targets in the sample distribution.

Evaluate sensing performance also based on false reporting rate, which is the number of false reports divided by the total number of reports, where a report is false if the reported position p is further away from any target than the detection radius R, i.e. if R < ||p - xₙ|| for all targets n.

RAN1 should use the following metrics for the evaluation of sensing:
Horizontal and vertical position accuracy for a given confidence level,
Horizontal and vertical velocity accuracy for a given confidence level,
False alarm rate, missed detection rate, and false reporting rate.

Do not use latency as a performance metric.

Sensing performance requirements are classified into different categories. Categories 1 and 2 are for object detection, and categories 3 and 4 for object tracking. In general, object tracking has more stringent requirements than object detection in terms of positioning/velocity accuracy, range/velocity resolution, missed detection rate, and false alarm rate. To support higher sensing performance of tracking, additional proprietary processing at BS or core network is needed, such as fusing new sensing results with historical ones. However, different implementations among companies would make their results diverse and incomparable. We suggest RAN1 focus on the study of UAV detection. Between categories 1 and 2, category 1 has less stringent requirements and can be used as the starting point.

In TS 22.137, object tracking has more stringent requirements than object detection for positioning/velocity accuracy, range/velocity resolution, missed detection rate, and false alarm.

RAN1 to focus on the study of UAV detection and consider performance requirements of category 1 from TS 22.137 as the starting point.

# Sensing methods and measurements

In this section we provide a high level description on the assumed methods for gNB based monostatic sensing. Note that the method to sense the target is up to implementation. However, a basic description of the sensing method is necessary to understand evaluation results. For the feasibility purpose, here we explain basic ways in which a target location and velocity can be acquired.

Figure 1	An illustration of monostatic sensing of a UAV with a base station

In sensing, the target is first detected, then positioned. Velocity estimation is done over multiple samples of the target measurements. Initial target detection in a monostatic TRP based setup is done by acquiring a set of impulse responses reflected by the target, summarized in following steps

- Reference signals are transmitted from a TRP. The configuration of the reference signals are determined based on prior information of the target of interest, such as range of velocity, range of possible distances of the target etc.

- Transmitted signal is received after scattering off targets or scatterers. The received signal is matched filtered to generate channel impulse response (CIR). CIR generated from subsequent transmissions of reference signal is collected.

- Delay-Doppler processing of CIR collected from several reference signal transmissions and receptions is done.

- Clutter suppression algorithms are used to suppress receptions from unwanted static or moving scatterers.

- Signal level above a threshold (based on radio cross-section of the target) within the chosen prior Delay-Doppler range would indicate a presence of object of interest. This would lead to detection of the object of interest.

Positioning a target can be done by a combination of range and angle of arrival relative to the TRP:

- For every peak within the chosen prior range of Delay-Doppler domain, angle of arrival of the signal is estimated.

- Location of the object of interest can be estimated by combining the angle of arrival and delay of the received signal after scattering from the object.

Estimation of target velocity will be based on estimation of target positions from above steps.

- The rate of change of target positions can give the target velocity.

- The velocity estimates can further be improved with smoothing filters if needed.

In monostatic sensing, a BS is able to determine the presence/absence of a target based on the amplitude of a peak in Delay-Doppler profile.

In monostatic sensing, position of a detected target can be estimated by a BS from the angle of arrival and propagation path delay of the peak.

In monostatic sensing, velocity of a detected target can be estimated from a series of target locations estimated by the base station. The rate of change of the target location will be a velocity measure.

One objective of the SI is to study measurements reported by RAN. The measurements for sensing refer to the information a BS transmits to core network upon a sensing request. With the above-mentioned methods, a BS can generate final sensing results of target position and velocity. They can be reported to core network as sensing measurements.

An advantage of using final sensing results as measurements rather than intermediate sensing data is the simplification of the reporting. The report consists only of final sensing results with general attributes such as quality and time stamps. If the reported result is an intermediate result, further information would have to be provided to core network so that it can continue the further signal processing to generate final sensing results, and sensing methods would need to be agreed upon.

Definitions of target position, velocity, and associated quality, and time stamps, have been specified in 23.032, which can be reused for sensing without the need of defining new measurements for sensing, i.e., no RAN1 specification impacts.

It is feasible for a BS to generate final sensing results of target position and velocity, which are transmitted to core network as measurements.

Support final sensing results of target position and velocity as measurements to be transmitted from BS to core network.

# Reference signals

According to the SID, existing DL NR waveform and DL NR reference signals are the baseline for RAN1 evaluation. Among DL NR reference signals, DL CSI-RS and DL PRS are natural candidates.

Note: relevant part in SID: As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts

Reference signals other than existing DL NR reference signals can be reported by companies, but cautions should be given. Using non-NR signals as sensing reference signal should ensure that non-NR signals do not interfere with NR operations, for example, orthogonality with existing NR transmissions should be ensured. It should also be guaranteed that the non-NR signals meet RF regulatory requirements, especially given RAN4 is not involved in the SI. A study on the two requirements is needed for a reported non-NR signal.

Consider NR DL PRS and DL CSI-RS as reference signal candidates for RAN1 evaluation.

The evaluation of non-NR signals as sensing reference signal should secure that non-NR signals do not interfere with NR operations and meet RF regulatory requirements, especially given RAN4 is not involved in the SI.

If non-NR signals are reported as sensing reference signal, a study is needed on whether they are orthogonal to existing NR signals and whether they meet RF regulatory requirements.

# Evaluation methodology and simulation assumptions

  - Single-TRP or multi-TRP mono-static sensing

In channel model calibration, a target is modelled in the target channel of N=4 TRP. In Figure 2, we show the cases of N=1 and N>1. If multiple TRP provide measurements related to the same target, it is worth consideration whether measurements of multiple TRPs should be fused to generate the final sensing results.

Figure 2	One or multiple TRPs selected for a target

In our view, the SID [1] mentions Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case. It clearly states a single TRP, which precludes the fusion of measurements generated by multiple TRPs. Therefore, if multiple TRPs are paired with a target, their measurements are not fused in RAN1 evaluation.

SID states a single TRP with co-located sensing transmitter and receiver, which precludes the fusion of measurements generated by multiple TRPs.

Fusion of measurements from multiple TRPs is not considered in RAN1 evaluation.

In RAN1 evaluation we assume that the system performances are based on measurement from one TRP for each target.

  - Simulation assumptions

In 38.901 [3], evaluation parameters are given in section 7.9.1 in tables for each sensing scenario. The parameter values show validity range, i.e., under which value ranges the ISAC channel model is valid. Section 7.9.6 [3] summaries parameters used for calibration. Note some parameters have different values in the two sections in order to reduce calibration efforts, as summarized in Table 1. We add our views on parameter values for RAN1 evaluation in the last column and discuss some other simulation parameters in the following sections.

Table 1	Parameter values in sections 7.9.1 and 7.9.6 of 38.901 and proposed values for evaluation

| Parameters | Table 7.9.1-1: Evaluation parameters for UAV sensing scenarios | Table 7.9.6.1-1: Simulation assumptions for large scale calibration for UAV sensing targets | Proposed value for evaluation |
| --- | --- | --- | --- |
| 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h see note 2 and 3 NOTE 2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target NOTE 3: time-varying velocity may be considered for future evaluations. | NA | Same value as in Table 7.9.1-1 Simulate horizontal mobility only, vertical mobility only, and a combination |
| 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. N = {1, 2, 3, 4, 5} see note 4 Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. | Horizontal plane: N targets uniformly distributed in the center cell Vertical plane: uniform between 1.5m and 300m N can be a random number including 0 |
| Orientation | Random in horizontal domain | NA due to isotropic RCS used for calibration | NA, because RCS of small UAV is independent from its orientation. |
| Minimum 3D distance between sensing targets | Option 1: At least larger than the physical size of a target Option 2: 10 meters | NA due to a single target used for calibration | at least 2 times detection radius, so that a position estimate will not be associated with two targets (for association, see section 2) |
| STX/SRX selection |  | Best N = 4 STX-SRX pairs to be selected for the target. see note 1 NOTE 1: Based on the STX-SRX pairs with the smallest power scaling factor of the target channel. | multiple TRP selected for a target How to select these TRPs is up to companies to decide. |
| Bandwidth |  | FR1: 100MHz FR2: 400MHz | FR1: 100MHz |
| Carrier Frequency |  | FR1: 6 GHz FR2: 30 GHz | FR1: 6 GHz, because FR1 is the prioritized frequency range according to the SID |

Low-power cluster is an optional feature of ISAC channel model [3], which purpose is to generate NLOS clusters in background channel with lower power than the legacy NLOS clusters generated according to Rel-18 38.901. Therefore, there would not be a large power gap between target channel and background channel. Low-power clusters and legacy NLOS clusters would not impact sensing performance if they are stationary, because they can easily be removed in Doppler domain. However, low-power clusters if modelled with some moving scatterers can confuse sensing algorithms on determining whether the detection is a real target or clutter. It is useful in the evaluation of false alarm where no targets are dropped.

Consider low-power cluster and moving scatterers in RAN1 evaluation, such as the evaluation of false alarm.

### BS antenna configuration

Realistic antenna configurations and realistic isolation between antenna elements for monostatic sensing should be assumed in the studied scenarios. For example, in 38.858, the following antenna configuration was agreed upon for calibration purposes of sub-band full duplex (SBFD) with two antenna panel groups: one for transmission and the other for reception. These antenna panel groups are given sufficient separation to achieve the isolation required to minimize the impact of self-interference in SBFD base stations. This consideration is also relevant to monostatic sensing and must be included in the evaluation assumptions.

| BS antenna configuration for Urban Macro/ Dense Urban Macro layer | FR1 | M,N,P,Mg,Ng;Mp,Np= (8,8,2,1,1;2,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | - SBFD antenna configuration Option 2 (Method 2-1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ |
| --- | --- | --- | --- |

For ISAC evaluation purposes, if we decide to use the same assumptions as 38.858, it would imply that there are two panel groups for monostatic sensing where one panel is used for transmitting the sensing reference signal and the other panel group for receiving the reflections with isolation in between them. The antenna dimensions used in 38.858 can be reused independently if one or two panels are used in the simulation.

RAN1 to decide whether gNB-based monostatic sensing uses a single panel group or two panel groups. If two panel groups are supported: one for sensing TX and the other for sensing RX, the required separation distance between them needs to be agreed.

Reuse the BS antenna dimensions for FR1 from TR 38.858 i,e. For each panel group: (M,N,P,Mg,Ng;Mp,Np)  = (8,8,2,1,1;2,8)  and  (dH,dV) = (0.5, 0.8)λ,  +45°/-45° polarization and with two panel group.

### Interference modelling

In order to evaluate the performance of gNB-based mono-static sensing in realistic scenarios with practical assumptions, it is important to consider modelling different kinds of possible interferences in ISAC network.

For the purpose of simplifying performance evaluations of monostatic sensing, no simultaneous communication traffic is assumed on the evaluated carrier. The resulting sensing performance serves as an upper bound relative to scenarios where sensing and communication are on the same carrier.

If no simultaneous communication traffic is assumed on the evaluated carrier, the sensing evaluation results are the upper bound without interference from communication.

Even with communication excluded, there are different kinds of interference sources in the ISAC network that can impact the performance. These are listed below.

- Self-interference (SI)

  - Leakage from the sensing TX into its own RX including direct coupling (antenna isolation limits), RF front-end leakage, non-linearity products, LO phase noise, and residual after SI cancellation. This is typically the dominant interference for mono-static sensing gNB .

- Co-site, Inter-sector interference

  - Sensing TX from one sector at the same time interferes with the reception in another sector.

- Inter-site interference

  - Sensing Tx from a gNB at a neighboring site interferes with the reception at victim gNB. This can be quite strong if the sectors are pointing toward each other.

- Adjacent channel interference

  - Tx sensing waveform in one carrier by one operator leaks into RX in an adjacent carrier of another operator due to ACLR (transmit leakage) and ACS (receiver selectivity limits)

For the self-interference modelling, during the SBFD SI, we provided [4] full-electromagnetic simulations to analyze the power leakage from the TX panel to RX branches for both FR1 and FR2. These simulations investigate the isolation performance of an antenna design with a specific isolation enhancement structure. It showed that only physical separation between TX and RX panels does not provide sufficient isolation. Structures and materials to improve isolation are needed. In addition to antenna isolation analysis between Tx and Rx panel, the leakage between different sectors on a single mast for both FR1 and FR2 was also presented [5]. It also showed that antenna performance highly depends on polarization, desired bandwidth and complexity of combined structures. In addition, the achievable isolation can degrade ~10 dB depending on the beam forming over steering range of the antennas. Since for 3.5 GHz the best case is 70dB antenna isolation and there can be 10dB reduction depending on the direction, we assume 65dB average isolation for FR1.

For self-interference modelling in evaluations, RAN1 to assume 65dB average antenna isolation between sensing Tx and Rx panels.

For co-site inter-sector interference modelling, one could assume certain isolation between the sectors and consider reusing the model in TR 38.858. It may also be worthwhile to assume that only one sector performs sensing at a given instant.

For inter-site interference modelling, interference can be modelled with TRP-TRP bi-static channel. If a TDM muting pattern is applied, where only one TRP transmits sensing reference signals at a given time, a minimum TRP-TRP distance should be sorted out so that the two TRP within the minimum distance do not sense at the same time. The study of minimum inter-BS distance can improve sensing efficiency of the sensing network. Otherwise a sensing BS has to wait for an unnecessarily long time for its turn to sense.

Study the inter-site interference between two sensing TRPs and determine a minimum distance where both TRP can sense simultaneously and meet sensing performance requirements.

For evaluating performance of TRP monostatic sensing, RAN1 to decide whether to study interference between communication and sensing and adjacent channel interference.

RAN4 provides in-band blocker requirements for Wide Area BSs. This determines the maximum received power at the LNA. The RAN4 in-band general blocker power is -43 dBm for a WA BS with 53 dBm transmit power. If we assume 65 dB antenna isolation between the Tx and Rx receiver then the maximum transmit power that can be used to do monostatic sensing can be estimated as 65-43 dBm = 22 dBm. However, RAN4 is discussing new in-band blocker power for Rel-19 SBFD ranging from -28 dBm to -16.7 dBm. While evaluating, it might be better to be on the conservative side and consider -28 dBm as the in-band blocker power. This means, any power level above -28 dBm would saturate the receiver and sensing would not be possible. Considering the above we propose the following.

RAN1 agrees to use BS transmit power of 37 dBm for the evaluations.

### Summary

All simulation assumptions discussed in section 5.2 are summarized in Table 2.

Table 2	Proposed simulation assumptions for RAN1 evaluation

| Parameters | Proposed simulation assumptions for RAN1 evaluation |
| --- | --- |
| 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h Simulate horizontal mobility only, vertical mobility only, and a combination |
| 3D distribution | Horizontal plane: N targets uniformly distributed in the center cell Vertical plane: uniform between 1.5m and 300m N can be a random number including 0 |
| Orientation | NA |
| Minimum 3D distance between sensing targets | at least 2 times detection radius |
| STX/SRX selection | multiple TRP selected for a target How to select these TRPs is up to companies to decide. |
| Bandwidth | FR1: 100MHz |
| Carrier Frequency | FR1: 6 GHz |
| BS antenna configurations | Two panels or panel groups and the configuration for a single panel is shown below. (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization |
| BS antenna isolation level | 65 dB |
| BS in-band blocker power level | -28 dBm |
| BS Tx power | 37 dBm |

RAN1 to agree to Table 2 containing the proposed evaluation assumptions for monostatic sensing.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In TS 22.137, object tracking has more stringent requirements than object detection for positioning/velocity accuracy, range/velocity resolution, missed detection rate, and false alarm.

Observation 2	In monostatic sensing, a BS is able to determine the presence/absence of a target based on the amplitude of a peak in Delay-Doppler profile.

Observation 3	In monostatic sensing, position of a detected target can be estimated by a BS from the angle of arrival and propagation path delay of the peak.

Observation 4	In monostatic sensing, velocity of a detected target can be estimated from a series of target locations estimated by the base station. The rate of change of the target location will be a velocity measure.

Observation 5	It is feasible for a BS to generate final sensing results of target position and velocity, which are transmitted to core network as measurements.

Observation 6	The evaluation of non-NR signals as sensing reference signal should secure that non-NR signals do not interfere with NR operations and meet RF regulatory requirements, especially given RAN4 is not involved in the SI.

Observation 7	SID states a single TRP with co-located sensing transmitter and receiver, which precludes the fusion of measurements generated by multiple TRPs.

Observation 8	If no simultaneous communication traffic is assumed on the evaluated carrier, the sensing evaluation results are the upper bound without interference from communication.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define detection in terms of a detection radius and define the detection of a target as the event that at least one reported position is within the detection sphere whose center is the true position of the target and whose radius is the detection radius.

Proposal 2	Associate each target with the sensing result whose position estimate is the closest to its true position and evaluate position and velocity accuracies based on the error vectors corresponding to this association and only include error vectors corresponding to detected targets in the sample distribution.

Proposal 3	Evaluate sensing performance also based on false reporting rate, which is the number of false reports divided by the total number of reports, where a report is false if the reported position p is further away from any target than the detection radius R, i.e. if R < ||p - xₙ|| for all targets n.

Proposal 4	RAN1 should use the following metrics for the evaluation of sensing: Horizontal and vertical position accuracy for a given confidence level, Horizontal and vertical velocity accuracy for a given confidence level, False alarm rate, missed detection rate, and false reporting rate.

Proposal 5	Do not use latency as a performance metric.

Proposal 6	RAN1 to focus on the study of UAV detection and consider performance requirements of category 1 from TS 22.137 as the starting point.

Proposal 7	Support final sensing results of target position and velocity as measurements to be transmitted from BS to core network.

Proposal 8	Consider NR DL PRS and DL CSI-RS as reference signal candidates for RAN1 evaluation.

Proposal 9	If non-NR signals are reported as sensing reference signal, a study is needed on whether they are orthogonal to existing NR signals and whether they meet RF regulatory requirements.

Proposal 10	Fusion of measurements from multiple TRPs is not considered in RAN1 evaluation.

Proposal 11	In RAN1 evaluation we assume that the system performances are based on measurement from one TRP for each target.

Proposal 12	Consider low-power cluster and moving scatterers in RAN1 evaluation, such as the evaluation of false alarm.

Proposal 13	RAN1 to decide whether gNB-based monostatic sensing uses a single panel group or two panel groups. If two panel groups are supported: one for sensing TX and the other for sensing RX, the required separation distance between them needs to be agreed.

Proposal 14	Reuse the BS antenna dimensions for FR1 from TR 38.858 i,e. For each panel group: (M,N,P,Mg,Ng;Mp,Np)  = (8,8,2,1,1;2,8)  and  (dH,dV) = (0.5, 0.8)λ,  +45°/-45° polarization and with two panel group.

Proposal 15	For self-interference modelling in evaluations, RAN1 to assume 65dB average antenna isolation between sensing Tx and Rx panels.

Proposal 16	Study the inter-site interference between two sensing TRPs and determine a minimum distance where both TRP can sense simultaneously and meet sensing performance requirements.

Proposal 17	For evaluating performance of TRP monostatic sensing, RAN1 to decide whether to study interference between communication and sensing and adjacent channel interference.

Proposal 18	RAN1 agrees to use BS transmit power of 37 dBm for the evaluations.

Proposal 19	RAN1 to agree to Table 2 containing the proposed evaluation assumptions for monostatic sensing.

# References

RP-251861, AT&T (moderator),	New SID: Study on Integrated Sensing And Communication (ISAC) for NR, RAN#108

TS 22.137, Service requirements for Integrated Sensing and Communication, Stage 1, V19.1.0, March 2024

TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V19.0.0, June 2025

R4-2216404, SBFD gNB Radio and antenna considerations, Ericsson, RAN4#104bis-e, October 2022

R4-2219633, BS RF feasibility considerations, Ericsson, RAN4#105, November 2022

# Annex

## Definitions of sensing performance requirements in 22.137

-  Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

-	Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity.

-	Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy.

-	Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude.

-	Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results.

NOTE 1:	An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive.

-	False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results.

- NOTE 2:	An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative.

## Definitions of Sensing-related capabilities in ITU Document 5D/TEMP/345, 11 July 2025

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error.

•	Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

•	[Sensing Resolution: TBD]