3GPP TSG RAN WG1 #122	R1-2506325

Bengaluru, India, Aug 25th -29th, 2025

Agenda item:	11.1

Source:	Nordic Semiconductor ASA

Title:	General aspects of 6G IoT and NTN

Document for:	Discussion

# Introduction

At the TSG RAN #108 SI on 6G for WGs has been agreed in Rel-20. With the following objectives, summarized in RP-251881.

In this contribution we focus on

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

Chair recommendation on 11.1 agenda is the following:

High level design proposals/principles/target and overall design of 6G air interface to illustrate/address the pain points observed from different angles, e.g., aspects of how to design a single RAT to serve diverse devices, channel bandwidth (at least minimum and maximum), aspects of overall coverage, aspects of initial access and common channel, aspects of MRSS, aspects of service/channel multiplexing/collision, including concurrence of UL transmissions,…..., aspects of concept and operation of bandwidth, aspects of supporting existing and new services, aspects of spectrum utilization and operations, aspects of spectrum efficiency, aspects of all duplex types, as well as concepts and aspects of harmonization of TN and NTN, etc.

# Discussion

## IoT device types to be supported in 6G unified RAT

Starting from LTE R13, 3GPP introduced different IoT technologies, among which the following are deployed and their KPIs summarized in Table below:

|  | Cat-NB2 R14 | Cat-M1 R14 | Cat1-bis R14 | RedCap R17 | [eRedCap R18] |
| --- | --- | --- | --- | --- | --- |
| Bitrate [Mbs] DL/UL#7 | 0.127/0.159 | 0.6/1 | 10/5 | 54-113/56-120 per layer | 10/10 |
| Mod. DL/UL#7 | QPSK/QPSK | 16QAM/QPSK | 64/16 QAM | 64/16 QAM | 64/16 QAM |
| RF BW | 180kHz | 1.44Mhz | 20Mhz | 20Mhz | 20Mhz |
| SCS | 15kHz | 15kHz | 15kHz | 15/30kHz | 15/30kHz |
| BB PRBs #4 | 1 | 6 | 100 | 106/51 | 106/51 or 25/12 |
| UE Rx ant. | 1 | 1 | 1 | 2 | 1 |
| UE Tx ant. | 1 | 1 | 1 | 1 | 1 |
| UE Tx Pow. [dBm] | 14/20/23 | 20/23 | 23 | 23+ | 23+ |
| Network Tx ant, | 2 | 4 | 4 | >=8 | >=8 |
| DL cov enh | Rep. (20dB) | Rep. (10dB) | -3dB | DL beamforming | DL beamforming |
| UL cov enh | Rep (20dB) | Rep. (10dB) | 0dB | Rep. (10dB) | Rep. (10dB) |
| Duplex | HD-only/TDD* | HD/FD*/TDD* | HD*/FD/TDD | HD*/FD/TDD* | HD[*]/FD/TDD* |
| Duplex switch (HD-FDD or TDD) | 1ms | 1ms | puncturing | 13us | 13us |
| Sched delay DL | n+5ms | n+2ms | same slot | same slot | same slot |
| Ack proc delay | n+13ms | n+4ms | n+4ms | n+1ms/0.357ms | n+1ms/0.357ms |
| Sched delay UL | n+8ms | n+4ms | n+4ms | n+1ms/0.429ms | n+1ms/0.429ms |
| Coding DL/UL/CTRL | Viterbi/TBCC/Viterbi | TBCC/TBCC/ Viterbi/ | TBCC/TBCC/ Viterbi/ | LDPC/LDPC/Polar | LDPC/LDPC/Polar |

*not deployed

Among IoT cellular technologies it can be noticed that TDD has not been deployed much for IoT, for NB-IoT as well as RedCap at the moment.  While NB-IoT, LTE-M were deployed with HD-FDD and RedCap with FD-FDD. For eRedCap the ecosystem is still forming.

Observation-1: Low-band FDD bands are the most suitable for IoT due to their good coverage. Both half-duplex and full-duplex FDD are currently deployed in cellular IoT vertical.

RedCap and LTE CAT4 are part of high-end IoT segment which requires bitrates as high as 100-200Mbits. Example use-cases could be XR wearables, low-end smartphones, video surveillance. Low-end IoT segement such as asset trackers, metering, connected cities, smart-watches would suffice with <10-20Mbits, while benefiting from small-form factor and low-complexity LPWA-like modules.  For low-end devices IMS is an overkill protocol stack, and therefore, a low-complex non-IMS voice should be considered instead.

Two IoT Device types should be considered in RAN1 unified RAT 6G design:

Proposal-1: Consider two types of IoT devices

- TYPE1-IoT: low-end IoT with battery constrain.

  - Bands below <2.5GHz

  - Peak-rates below<20Mbits

  - HD-FDD UE operated in paired and unpaired spectrum

  - non-IMS voice only

  - <10/5MHz for DL/UL

  - 1Rx baseline

- TYPE2-IoT: high-end IoT without battery constrain.

  - Bands below <6GHz

  - Peak-rates below<200Mbits

  - FD-FDD/TDD UE operated in paired and unpaired spectrum

  - IMS voice

  - <20/20MHz for DL/UL

  - 2Rx baseline

In the following sections we try to justify the Proposal-1.

## Technology verticals that should not be pursued as part of 6G unified RAT.

Unlicensed band support has been developed throughout 4G and 5G standards. However, commercialization of those standards remained limited, almost non-existent.  With a matter of fact, Wifi is less complex and cheaper technology with broad ecosystem, it is very unclear why customers would be willing to accept 3GPP unlicensed instead of Wifi.

Proposal-2:  Do not optimise 6G unified RAT for unlicensed bands.

Similarly, we are not convinced about mesh/D2D networking to be essential for 6G unified RAT. This technology, if made by 3GPP, would not be competitive in terms of complexity and cost to existing or emerging non-3GPP mesh standards. Low complex 3GPP-mesh solution would require its own RAT (like  NB-IoT or Ambient IoT), while e.g. some parts of numerology, coding, etc. could be reused from 6G, this to maximize implementation synergy for multi-RAT chips.

Proposal-3:  Do not optimise 6G unified RAT for Mesh/D2D. Consider standalone RAT for 3GPP Mesh in the future.

## Duplexing for TYPE1-IoT devices: Half-duplex bringing UE complexity down

A half-duplex implementation is an efficient way to reduce complexity on the module level. The relaxation from simultaneous transmission and reception may simplify BB implementation as well, but main advantage comes from RFFE design simplification.

Previous generations of 3GPP standard have not supported half-duplex  in its first releases (Rel-8 of LTE and R-15 of NR). Contrary, LPWA technologies added half-duplex support in their first release (i.e. R13), and field adopted half-duplex LTE-M version instead of full-duplex. For NB-IoT, half-duplex was deployed instead of TDD. Contrary, for LTE Cat1bis half-duplex has not been adopted at all, and for NR half-duplex has not been adopted, at least yet.

In addition to complexity reduction, half-duplex reduces also the form factor of the communication module, this because duplexer technology cannot be implemented in the basic silicon. Special semiconductors and large node sizes are required for duplexers, making full-duplex solution difficult to use in small form wearables.

From network point of view, half-duplex does not necessarily impact spectral efficiency of the cell, if scheduling algorithms are designed efficiently (in network scheduler) and duplexing time-gap is small (< 1 OFDM symbol). While efficient scheduler may not be difficult to develop, it may be non-trivial to introduce it only after a full-duplex scheduler has been finalized. This is because full-duplex scheduling choices can be made so that they are not compatible with half-duplex requirements introduced in later releases. Therefore, it is essential that half-duplex requirements are known to network vendors since the first release of 6G, and also IoT chips are being developed and IoDTed by eNB vendors since the first release of 6G.

Protocol should be designed so that half-duplex FDD UE may operate also on any pair or unpaired spectrum. In other words, same protocol stack should apply to TDD or FDD bands. R19 IoT NTN TDD WID showed that HD-FDD NB-IoT protocol can be operated also on the unpaired spectrum with very minor differences.

The main difference between TDD and FDD protocol (from UE point of view) is an existence of semi-static TDD pattern, which is almost the same as rate-matching pattern.   To simplify the protocol design, support configuration of UL and DL slots only. By default, all resources are valid UL and DL subframes/slots. On a slot level, it would be possible to postpone signals and channels to next available slots of DL or UL.  Very simple rules can be developed as part of unified RAT design.

Proposal-4: For TYPE1-IoT device, support only HD-FDD in initial 6G release.  6G HD-FDD protocol should be designed such that it may be operated on both paired and unpaired spectrum.

## Unified initial access in low-bands for all UE types

Yet another key component of low-complex and small-form half-duplex module is narrow-enough UL transmission bandwidth. Existing 3GPP requirements can be met with lower complexity, when UL transmission BW is <5MHz, avoiding need for additional filtering in RF front-ends.  When it comes to DL, from Nordic point of view, there is no need to restrict DL BW to within <5MHz. DL could be 10 or even 20MHz to ensure efficient and unified broadcast design in RAT. However, UL should be confined within <5MHz transmission BW in 6G IoT standard. After MSG3, indicates UE type, separate handling of remaining ATTACH procedure can be carrier for different UE types.

6G unified initial access is designed so that the minimum RF BW for unified initial access

- is 5MHz (15kHz SCS) and 10 MHz (30kHz SCS) in DL and

- 5MHz in UL for both 15 and 30kHz SCS.

This means that SIBs, Paging, Wake-up signal(s), Synch signals, DL initial access messages (PRACH, MSG2, MSG3) should be confined within 10MHz, while UL messages during initial access to 5MHz at least in bands below 2.5GHz.

Proposal-5: For TYPE1-IoT device, consider unbalance minimum RF BW between UL (e.g. 5MHz) and DL (10MHz for 30kHz SCS and 5MHz for 15kHz). This to simplify unified 6G RAT between eMBB, NTN and IoT devices.

6G unified design shall not support MSG1 early UE type and feature identification, which horribly complicates the standard. In other words, irrespective of UE-type, UL coverage of initial access assumes, single Tx antenna and <5MHz (MSG3 and MSG1).

Proposal-6: UL coverage of initial access of 6G unified RAT assumes single Tx antenna and <5MHz (MSG3 and NPRACH). Early UE-type identification by MSG1 is not supported in 6G.

In 6G, compared to 5G, do not support mixed SCS between synchronisation channels/signals and control/data channels/signals. This to simplify protocol design. If network wants to serve mixed SCS it may do it by creating multiple cells and intra-band re-selection/HO, which can be used for any type of device in 6G.

Proposal-7: Mixed SCS in a single cell is not supported in 6G at all.

## 1 Rx for TYPE-1 IoT devices

Number of received antennas has been often a subject of disputes in 3GPP. Small number of antennas is not liked by MNOs, because only single MIMO layer can be used for such devices in DL, reducing spectral efficiency on licensed spectrum. However, in our opinion, 1Rx is a crucial feature to be able to reduce complexity of cellular IoT solution and being able to compete in low-end IoT segment with non-cellular technologies like Wi-Sun, LoRA, DECT NR+, and others. We believe that single Rx antenna should be allowed with TYPE1-IoT devices. This will ensure that impact to MNO license band spectral efficiency is minimized.

Proposal-8: For TYPE1-IoT, 1Rx support shall be the baseline in 6G unified RAT.

## Protocol overhead and configuration flexibility

One learning from NR is that protocol configuration flexibility is making it an extremely complex system, and the overhead of excessive configurability and capabilities is eating capacity which otherwise could be used for customer data. While overhead may be negligible when operating on 100MHz carrier, when 3/5MHz carrier is being deployed, overhead is significant part of available bitrate.

Generally speaking, the design of system should target heavily reducing the need for excess DL broadcast, minimize the use and size of SI messages, and aim for minimalistic DL control channel design. There should be high threshold when introducing new SI broadcast or extend their content (essential or on demand).

Finally, feature discussions carried in WGs should be completely redesigned. Instead of having one baseline capability and 20 optional on top. There should be only e.g. three capability profiles 1) minimum set of features 2) selected enhanced set of features 3) all features specified. Once profile is indicated, all features in the profile are mandatory not only for UE, but also for network. This will simplify not only ecosystem discussions and IoDT but also reduces overhead in capability exchange.

Proposal-9: Study potential techniques on how to reduce configuration complexity and system overhead (broadcast, capabilities) especially for the RF BW limited UEs in 6G unified RAT

- at least for 6G IoT consider introducing capability profiles (e.g. 3). Features within indicated profile are mandatory for both UE and network.

## Unified TN and NTN design

Unified 6G RAT should consider the following challenges of NTN in its design

- lack of wide-enough global spectrum in bands <2.5GHz

- large RTTs

- Coverage due to pathloss

- Low spectral efficiency

- GNSS dependency

Lack of wide-enough global spectrum in bands <2.5GHz: it is difficult to find large enough spectrum to deploy NTN. This is why 5G is pursuing reduction of channel BW from 5->3MHz.  This is why NTN may be even bigger driver than IoT to reduce initial access BW in bands <2.5GHz to even 3MHz.

Observation-2: NTN may be the factor reducing initial access BW to less than 5MHz.

Large RTTs: There is not much one can do about large physical RTTs in NTN. However, protocol can be optimised with assuming such constraint in mind. 5G designed HARQ-less protocol, which may work well in stable NTN link conditions. Having sufficient number of HARQ processes to cover RTT is another approach. And finally, scheduling multi-TB  physical channels with HARQ-bundling feedback is also an option. Having RLC/PDCP level ACK or multi-HARQ process single-bit acknowledgement is the same thing. The block must be retransmitted if at least one segment of it fails. While multiple solutions to the issue exist, 3GPP should pursue only one solution.

COVERAGE: Dynamic DL and UL repetitions and sub-PRB allocations are important for NTN link. sub-PRB scheduling along with DL/UL repetitions should be a part of 6G standard, and if not used in TN, used at least in NTN deployments.

Proposal-10: Unified RAT supports one sub-PRB (e.g. 3-tone) allocation in 6G, which would be beneficial at least in NTN scenarios.

Proposal-11: For all device types, support small number of UL repetitions (max 10dB) for 6G unified RAT. FFS DL.

Spectral efficiency:  Protocol overhead of NTN link must be reduced, which is not really a RAN1 topic. Except,  RAN1 study could focus mainly on protocol reduction in initial access. Assuming that UE has an exact knowledge of TA and doppler when having precise location of a cell. Initial access could start directly with MSG3 in the unified 6G RAT.

Proposal-12: In unified 6G RAT, support RACH-less initial access, assume precise location of the cell and UE is known in both TN and NTN. Study further multiple access for MSG3.

GNSS dependency: NTN cell should be accessible also with only course knowledge of position, this could be done by designing robust RACH format.

Proposal-13: Unified RAT comprises robust PRACH format to enable GNSS-less NTN access.

# Conclusions

In this contribution, we discussed the 6G IoT core design elements.  The following is observed and proposed:

Observation-1: Low-band FDD bands are the most suitable for IoT due to their good coverage. Both half-duplex and full-duplex FDD are currently deployed in cellular IoT vertical.

Proposal-1: Consider two types of IoT devices

- TYPE1-IoT: low-end IoT with battery constrain.

  - Bands below <2.5GHz

  - Peak-rates below<20Mbits

  - HD-FDD UE operated in paired and unpaired spectrum

  - non-IMS voice only

  - <10/5MHz for DL/UL

  - 1Rx baseline

- TYPE2-IoT: high-end IoT without battery constrain.

  - Bands below <6GHz

  - Peak-rates below<200Mbits

  - FD-FDD/TDD UE operated in paired and unpaired spectrum

  - IMS voice

  - <20/20MHz for DL/UL

  - 2Rx baseline

Proposal-2:  Do not optimise 6G unified RAT for unlicensed bands.

Proposal-3:  Do not optimise 6G unified RAT for Mesh/D2D. Consider standalone RAT for 3GPP Mesh in the future.

Proposal-4: For TYPE1-IoT device, support only HD-FDD in initial 6G release.  6G HD-FDD protocol should be designed such that it may be operated on both paired and unpaired spectrum.

Proposal-5: For TYPE1-IoT device, consider unbalance minimum RF BW between UL (e.g. 5MHz) and DL (10MHz for 30kHz SCS and 5MHz for 15kHz). This to simplify unified 6G RAT between eMBB, NTN and IoT devices.

Proposal-6: UL coverage of initial access of 6G unified RAT assumes single Tx antenna and <5MHz (MSG3 and NPRACH). Early UE-type identification by MSG1 is not supported in 6G.

Proposal-7: Mixed SCS in a single cell is not supported in 6G at all.

Proposal-8: For TYPE1-IoT, 1Rx support shall be the baseline in 6G unified RAT.

Proposal-9: Study potential techniques on how to reduce configuration complexity and system overhead (broadcast, capabilities) especially for the RF BW limited UEs in 6G unified RAT

- at least for 6G IoT consider introducing capability profiles (e.g. 3). Features within indicated profile are mandatory for both UE and network.

Observation-2: NTN may be the factor reducing initial access BW to less than 5MHz.

Proposal-10: Unified RAT supports one sub-PRB (e.g. 3-tone) allocation in 6G, which would be beneficial at least in NTN scenarios.

Proposal-11: For all device types, support small number of UL repetitions (max 10dB) for 6G unified RAT. FFS DL.

Proposal-12: In unified 6G RAT, support RACH-less initial access, assume precise location of the cell and UE is known in both TN and NTN. Study further multiple access for MSG3.

Proposal-13: Unified RAT comprises robust PRACH format to enable GNSS-less NTN access.

# References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, INC. (moderator)