3GPP TSG-RAN WG1 Meeting #122	R1-2506366

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	5

Source:	Ericsson

Title:	On RAN2 LS on CSI-RS based CFRA using SBFD RO

Document for:	Discussion, Decision

# Introduction

RAN2 sent the following LS to RAN1 [1]:

1. Overall Description: For the L3 handover and BFR cases, RAN2 has made the below agreement: Agreement: For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion.. 2. Actions: To RAN WG1, RAN WG4: ACTION: RAN2 respectfully asks RAN1 and RAN4 to take the above agreement into account and provide feedback if there are any concerns.

Below we provide our views on this matter.

# Discussion

The network has great flexibility in configuring dedicated RACH configuration, since the UE is provided the complete configuration in the RRC IE RACH-ConfigDedicated [2]. In fact, it is not necessary for the dedicated RACH configuration to be related to the common configuration, as provided by RACH-ConfigCommon, at all.

The RACH configuration provided by dedicated RRC must not necessarily be related to any configured common RACH configurations.

This may cause an issue in determining the CSI-RS-to-RO mapping for two reasons:

The UE may not know whether legacy or additional ROs should be used since the RO locations are implicitly provided by the RO index and the two RO types do not have unique indices, and,

In case of additional ROs, whether to validate the additional ROs according to validation rules for the single RACH configuration (Option 1) or the additional RACH configuration (Option 2).

Without this information, there will be an ambiguity in the interpretation of the RO location. This ambiguity is both wasteful and undesirable since making room for multiple interpretations will cause additional complexity and overhead while the network likely has good reasons to prefer one RO type before the other.

With the above in mind, it is reasonable to respond to RAN2 that the above information is necessary from RAN1’s perspective in order to achieve a desirable outcome.

Send a reply LS to RAN2 stating that in order to unambiguously map CSI-RS indices to RO locations, the UE must also be made aware of the following:

  - Whether the RO locations should apply to legacy or additional ROs, and,

  - In case of additional ROs, whether said ROs should be validated according to the single RACH configuration (Option 1) or the additional RACH configuration (Option 2).

# Conclusion

In the previous sections we made the following observations:

Observation 1	The RACH configuration provided by dedicated RRC must not necessarily be related to any configured common RACH configurations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Send a reply LS to RAN2 stating that in order to unambiguously map CSI-RS indices to RO locations, the UE must also be made aware of the following:

a.	Whether the RO locations should apply to legacy or additional ROs, and,

b.	In case of additional ROs, whether said ROs should be validated according to the single RACH configuration (Option 1) or the additional RACH configuration (Option 2).

# References

R1-2505121 (R2-2504951), “LS on CSI-RS based CFRA using SBFD RO,” RAN2 WG, ZTE, RAN1 #122, August 2025.

TS 38.331, “NR - Radio Resource Control (RRC) protocol specification,” 3GPP, v18.6, June 2025.