3GPP TSG RAN WG1 #122			R1-2506416

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	9.15.9

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary of discussion on Rel-19 LTE UE features for IoT-NTN Phase 3

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 9.15.9. regarding Rel-19 LTE UE features for IoT-NTN Phase 3.

[122-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO), Ralf (AT&T) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

- FGs list agreed at RAN1#121

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. IoT_NTN_Ph3 | 1-1 | OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED 2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa 3. Dynamic activation/deactivation of OCC feature via DCI | Rel-17 2-1b | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | Per UE | FDD only | N/A | Note: This FG is applicable only to IoT-NTN FDD band | Optional with capability signalling |
| 1. IoT_NTN_Ph3 | 1-2 | OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED 2. Support of CDM DMRS for NPUSCH format 1 with 15 kHz SCS 3. Dynamic activation/deactivation of OCC feature via DCI | Rel-17 2-1b | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | Per UE | FDD only | N/A | Note: This FG is applicable only to IoT-NTN FDD band | Optional with capability signalling |
| 1. IoT_NTN_Ph3 | 1-3 | GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED | 1. Support of only GSO or NGSO scenario (or both GSO and NGSO if the indication is absent) for OCC with single-tone NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED. | 1-1 or 1-2 | Yes | N/A | UE does not support GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED | Per UE | FDD only | N/A | Note: This FG is applicable only to IoT-NTN FDD band NOTE: This FG should follow the same principle as ntn-HarqEnhScenarioSupport from RAN1 perspective, based on the following agreement: Agreement: - Adopt per UE for FG 1-1 and 1-2 - Introduce a new FG 1-3 that indicates whether FG 1-1/1-2 applies to GSO/NGSO/both, following the same principle as in Rel-18 IoT-NTN - Type: Per UE | Optional with capability signalling |

- Companies’ inputs

The following inputs have been provided in companies’ contributions in this meeting:

| - CATT | UE feature for 3.75kHz SCS NPUSCH format1 In RAN1#121 meeting, for 3.75kHz SCS NPUSCH format1 with OCC, the mappings between DMRS sequence samples and active TDM DMRS slots and the TDM DMRS positions that are activated within the TDM DMRS pattern have been reached more detailed agreements. Furthermore, when OCC feature is enabled, how to indicate OCC sequence index and OCC activation/deactivation have been agreed. Hence, it is necessary to update the component of FG1-1 as shown in table1. Proposal 1: Apply the updates on the IoT NTN FG1-1 provided in the table1 to follow RAN1 #121 progress. Table 1 UE feature for 3.75kHz SCS NPUSCH format1 with OCC Features; Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 1. IoT_NTN_Ph3; 1-1; OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED; 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED. 2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa. 3.Sequential mapping of samples of the original DMRS sequence to active DMRS slots. 4. For an NPUSCH format 1 with 3.75kHz SCS allocated to start in slot m, the NPUSCH is postponed to start in the next slot, whose index satisfies (SFN * 5 + ns) mod 4 = 0. 6. Dynamic activation/deactivation of OCC feature via “Redundancy version” field in DCI. 7.OCC sequence index is indicated via the fields “subcarrier indication” and “modulation coding scheme” jointly encoding. 8.The same RV value is used within an OCC group.; Rel-17 2-1b; Yes; N/A; UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. UE feature for 15kHz SCS NPUSCH format1 In RAN1#121 meeting, for 15kHz SCS NPUSCH format1 with OCC, when OCC feature is enabled, how to indicate OCC sequence index and OCC activation/deactivation have been agreed. Hence, it is necessary to update the component of FG1-2 as table2. Proposal 2: Apply the updates on the IoT NTN FG1-2 provided in the table2 to follow RAN1 #121 progress. Table 2 UE feature for 15kHz SCS NPUSCH format1 with OCC Features; Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 1. IoT_NTN_Ph3; 1-2; OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED; 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED. 2. Support of CDM DMRS for NPUSCH format 1 with 15 kHz SCS. 3. Dynamic activation/deactivation of OCC feature and OCC sequence index via “subcarrier indication” field in DCI. 4. The same RV value is used within an OCC group.; Yes; N/A; UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band.; Optional with capability signalling. |
| --- | --- |
| - Huawei, HiSilicon | UE feature for IoT-NTN UL capacity enhancement Multi-TB scheduling has been supported for NB-IoT in Rel-16 for scheduling enhancement, the interleaving for the case of multi-TB was captured in Clause 16.5.1 of TS36.213 [2] as follows, ### 16.5.1 UE procedure for transmitting format 1 narrowband physical uplink shared channel - For NTB>1, - if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and NRep>C where C=1 for NscRU=1, C=4 otherwise. - NB-IoT UL slots ng∙c∙NTB+r+l with l=0, 1, …, g-1, c=0, 1, …, NRep/C-1, g=CNRUNslotsUL are associated with TBr+1 , r=0, 1, …, NTB-1 - otherwise, - NB-IoT UL slots nr∙NRepNRUNslotsUL+l with l=0, 1, …, NRepNRUNslotsUL-1 are associated with TBr+1 , r=0, 1, …, NTB-1. The unit of interleaving for multi-TB scheduling is g=CNRUNslotsUL in the above specification. When either inter slot or inter symbol OCC is enabled for NPUSCH format1, the unit of interleaving should be scaled by the OCC length MOCC. The OCC sequence indicated by DCI format N0 should be applied for both TBs. Thus, a new UE feature group should be added on the supporting of OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED. The prerequisite of this FG can be one of {1-1, 1-2} and npusch-MultiTB-Interleaving-r16. Proposal 1: Add a new feature 1-4 to support OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC CONNECTED, as shown in the Table in Annex. # Annex 1: UE features for Rel-19 IoT-NTN Features; Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 1. IoT_NTN_Ph3; 1-1; OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED; 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED 2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa 3. Dynamic activation/deactivation of OCC feature via DCI; Rel-17 2-1b; Yes; N/A; UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. 1. IoT_NTN_Ph3; 1-2; OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED; 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED 2. Support of CDM DMRS for NPUSCH format 1 with 15 kHz SCS 3. Dynamic activation/deactivation of OCC feature via DCI; Rel-17 2-1b; Yes; N/A; UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. 1. IoT_NTN_Ph3; 1-3; GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED; 1. Support of only GSO or NGSO scenario (or both GSO and NGSO if the indication is absent) for OCC with single-tone NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED.; 1-1 or 1-2; Yes; N/A; UE does not support GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band NOTE: This FG should follow the same principle as ntn-HarqEnhScenarioSupport from RAN1 perspective, based on the following agreement: Agreement: - Adopt per UE for FG 1-1 and 1-2 - Introduce a new FG 1-3 that indicates whether FG 1-1/1-2 applies to GSO/NGSO/both, following the same principle as in Rel-18 IoT-NTN - Type: Per UE; Optional with capability signalling. 1. IoT_NTN_Ph3; 1-4; OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED; 1. Support OCC for PUSCH with interleaved multi-TB scheduling in RRC_CONNECTED; One of {1-1, 1-2}, npusch-MultiTB-Interleaving-r16; Yes; N/A; UE does not support OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. |
| - Samsung | Discussion 1. IoT_NTN_Ph3; 1-3; GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED; 1. Support of only GSO or NGSO scenario (or both GSO and NGSO if the indication is absent) for OCC with single-tone NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED.; 1-1 or 1-2; Yes; N/A; UE does not support GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band Note: Both GSO and NGSO are applicable if the indication is absent NOTE: This FG should follow the same principle as ntn-HarqEnhScenarioSupport from RAN1 perspective, based on the following agreement: Agreement: - Adopt per UE for FG 1-1 and 1-2 - Introduce a new FG 1-3 that indicates whether FG 1-1/1-2 applies to GSO/NGSO/both, following the same principle as in Rel-18 IoT-NTN - Type: Per UE; Optional with capability signalling. - Component: While this row remains stable, "(or both GSO and NGSO if the indication is absent)" appears more appropriate for relocating the Note column. Proposal 1: Consider the above update for FG 1-3. |
| - OPPO | - Discussion on UE features for R19 NTN for IoT Phase 3 In RAN1#121 meeting, the UE features on R19 IoT-NTN OCC have been extensively discussed and all the remaining issues have been addressed thanks to the great effort of our moderator and the fruitful discussions among companies. From our perspective, there is one potential issue on the Component 2 in FG 1-1, i.e., the current TDM DMRS related description is not accurate enough, because it is based on the agreement made in the previous meeting. Actually, RAN1 has a lot of discussions regarding the detailed TDM DMRS design in the last meeting and the following agreements are made. Agreement Confirm the following working assumption from RAN1#120 Athens: “For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.” Agreement For 3.75kHz SCS, the TDM DMRS positions that are activated within the TDM DMRS pattern are associated at least with the OCC sequence index. Agreement For 3.75kHz SCS OCC for NPUSCH format 1, the following mappings between DMRS sequence samples and active TDM DMRS slots is applied: - Option 1: Sequential mapping of samples of the original DMRS sequence to active DMRS slots Agreement For the TDM DMRS positions that are activated within the TDM DMRS pattern: For an NPUSCH format 1 with 3.75kHz SCS allocated to start in slot m, the NPUSCH is postponed to start in the next slot, whose index satisfies (SFN * 5 + ns) mod 4 = 0. The UE transmits TDM DMRS in the nth slot after the start of its NPUSCH transmission according to: Criterion; DMRS activity. OCC sequence index 0 [1 1]; OCC sequence index 1 [1 -1]. n mod 4 = 0; ON; OFF. n mod 4 = 1; ON; OFF. n mod 4 = 2; OFF; ON. n mod 4 = 3; OFF; ON.. According to the above agreement, the association between the TDM DMRS position and the OCC sequence index and the sequential mapping of original DMRS sequence to active DMRS slots need to be clarified. Nevertheless, it will incur excessive description in the FGs. For simplity, we can only mention that Support of TDM DMRS for NPUSCH format 1 with 3.75 kHz SCS in Component 2 in FG 1-1, like Component 2 in FG 1-2, and the detailed description is only reflected in RAN1’s specification. Proposal 1: Component 2 in FG 1-1 is updated to “Support of TDM DMRS for NPUSCH format 1 with 3.75 kHz SCS” for alignment with component 2 in FG 1-2. |
| - ZTE Corporation, Sanechips | In normative phase, the mechanisms of UL capacity enhancement for IoT-NTN has been discussed. For UL capacity enhancement, it has been agreed that for CONNECTED mode with 3.75kHz and 15kHz configuration, OCC feature can be enabled based on network configuration for IoT over NTN as shown below. The enabling signaling is via RRC and regardless of frame structure. And after higher layer enabling, dynamic activation and deactivation by DCI can be performed. Hence, an overall FG for supporting receiving of OCC enabling configuration in RRC signaling should be added as shown in FG 1-1. And it should be pre-requisite of FG 1-2 and FG 1-3. Otherwise, the controlling of OCC is not complete. Agreement For CONNECTED mode, UE-specific RRC signalling is used for enabling of the OCC feature.. Proposal 1: Adopt the feature groups in following table for Rel-19 IoT-NTN Features; Index; Feature group; Components; Prerequisite feature groups; Need for the eNB to know if the feature is supported; Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs); Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Capability interpretation for mixture of FDD/TDD; Note; Mandatory/Optional. 1. IoT_NTN_Ph3; 1-1; OCC feature for uplink transmission for NB-IoT; 1.Support receiving enabling of the OCC feature in RRC signalling; Rel-17 2-1b; Yes; N/A; UE does not support OCC for uplink transmission for NB-IoT via satellite; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. 1. IoT_NTN_Ph3; 1-12; OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED; 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED 2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa 3. Dynamic activation/deactivation of OCC feature via DCI; Rel-17 2-1b, 1-1; Yes; N/A; UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. 1. IoT_NTN_Ph3; 1-23; OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED; 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED 2. Support of CDM DMRS for NPUSCH format 1 with 15 kHz SCS 3. Dynamic activation/deactivation of OCC feature via DCI; Rel-17 2-1b, 1-1; Yes; N/A; UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. 1. IoT_NTN_Ph3; 1-34; GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED; 1. Support of only GSO or NGSO scenario (or both GSO and NGSO if the indication is absent) for OCC with single-tone NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED.; 1-12 or 1-23; Yes; N/A; UE does not support GSO/NGSO satellite orbit differentiation for OCC with single-tone for NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band NOTE: This FG should follow the same principle as ntn-HarqEnhScenarioSupport from RAN1 perspective, based on the following agreement: Agreement: - Adopt per UE for FG 1-1 and 1-2 - Introduce a new FG 1-3 that indicates whether FG 1-1/1-2 applies to GSO/NGSO/both, following the same principle as in Rel-18 IoT-NTN - Type: Per UE; Optional with capability signalling. |

- Discussion

Summary of companies’ inputs

- FG 1-1: - Components - Add “Sequential mapping of samples of the original DMRS sequence to active DMRS slot”: CATT - Add “For an NPUSCH format 1 with 3.75kHz SCS allocated to start in slot m, the NPUSCH is postponed to start in the next slot, whose index satisfies (SFN * 5 + ns) mod 4 = 0”: CATT - Update component on dynamic activation/deactivation by clarifying “RV field in DCI”: CATT - Add “OCC sequence index is indicated via the fields “subcarrier indication” and “modulation coding scheme” jointly encoding”: CATT - Add “The same RV value is used within an OCC group.”: CATT - Update component 2 to “support of TDM DMRS for NPUSCH format with 3.75 kHz SCS”: OPPO - FG 1-2: - Components: - Update component on dynamic activation/deactivation of OCC feature by clarifying “OCC sequence index via “subcarrier indication” field in DCI”: CATT - Add “The same RV value is used within an OCC group.”: CATT - FG 1-3: - Components: - Remove from component 1 “(or both GSO and NGSO if the indication is absent)” and clarify in Note “Both GSO and NGSO are applicable if the indication is absent”: Samsung - New FG - “OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED”: HW/Hisi, 1-4; OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED; 1. Support OCC for PUSCH with interleaved multi-TB scheduling in RRC_CONNECTED; One of {1-1, 1-2}, npusch-MultiTB-Interleaving-r16; Yes; N/A; UE does not support OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling. - “OCC feature for uplink transmission for NB-IoT” - Will be a prerequisite of original 1-1 and 1-2 1-1; OCC feature for uplink transmission for NB-IoT; 1.Support receiving enabling of the OCC feature in RRC signalling; Rel-17 2-1b; Yes; N/A; UE does not support OCC for uplink transmission for NB-IoT via satellite; Per UE; FDD only; N/A; Note: This FG is applicable only to IoT-NTN FDD band; Optional with capability signalling.

Question 1:

Companies are encouraged to provide views on the need for FG 1-1 to update as follows:

- Q1-1: Add a new component “Sequential mapping of samples of the original DMRS sequence to active DMRS slot”

- Q1-2: Add a new component “For an NPUSCH format 1 with 3.75kHz SCS allocated to start in slot m, the NPUSCH is postponed to start in the next slot, whose index satisfies (SFN * 5 + ns) mod 4 = 0”

- Q1-3: Add a new component “OCC sequence index is indicated via the fields “subcarrier indication” and “modulation coding scheme” jointly encoding”

- Q1-4: Add a new component “The same RV value is used within an OCC group.”

- Q1-5: Update component 2 to “Support of TDM DMRS for NPUSCH format with 3.75 kHz SCS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa”

- Q1-6: Update component 3 to “Dynamic activation/deactivation of OCC feature via “Redundancy version” field in DCI”

| Companies | Comment |
| --- | --- |
| Moderator | In general, it seems a bit unclear the above proposals are essential or nice-to-have, and in case it is nice-to-have, my preference is not to adopt that. Hence, I would like to ask especially proponents to clarify why this change is essential. |
| Ericsson | Overall, we think that none of the above proposals are essential, in our view the last outstanding issue was resolved in the previous meeting (i.e., Type and Satellite orbit differentiation). Moreover, we have already reached the following agreement: Agreement: For FG 1-1 and 1-2, adopt the following: - Remove “[FFS: Whether to have other component(s)]” in Component column |
| Vivo1 | Same view as Ericsson |
| Apple | Same view as Ericsson and vivo. |
| Huawei, HiSilicon | Same view as commenters above |
| ZTE | As the FFS on introducing other components have been agreed removed, it is not expected to introduce additional component is not essential. |
| Nokia, NSB | We think these do not need to be added in UE FG as UE should always support them when support 3.75kHz SCS with OCC or 15kHz SCS with OCC. |
| Xiaomi | Same view as Ericsson and other companies. |
| OPPO | Same view as Ericsson. From our perspective, adopting Q1-5 may be helpful for the alignment between the description of DMRS for 3.75kHz and 15kHz. |

Question 2:

Companies are encouraged to provide views on the need for FG 1-2 to update as follows:

- Q2-1: Update component 3 to “Dynamic activation/deactivation of OCC feature and OCC sequence index via “subcarrier indication” in DCI”

- Q2-2: Add a new component “The same RV value is used within an OCC group”

| Companies | Comment |
| --- | --- |
| Moderator | In general, it seems a bit unclear the above proposals are essential or nice-to-have, and in case it is nice-to-have, my preference is not to adopt that. Hence, I would like to ask especially proponents to clarify why this change is essential. |
| Ericsson | Overall, we think none of the above proposals are essential. Moreover, we have already reached the following agreement: Agreement: For FG 1-1 and 1-2, adopt the following: - Remove “[FFS: Whether to have other component(s)]” in Component column |
| Vivo1 | Same view as Ericsson |
| Apple | Same view as Ericsson and vivo. |
| Huawei, HiSilicon | Same view as commenters above |
| ZTE | As the FFS on introducing other components have been agreed removed, it is not expected to introduce additional component is not essential. |
| Nokia, NSB | We think these do not need to be added in UE FG as UE should always support them when support 3.75kHz SCS with OCC or 15kHz SCS with OCC. |
| Xiaomi | Same view as Nokia. |
| OPPO | Same view as Ericsson |

Question 3:

Companies are encouraged to provide views on the need for FG 1-3 to update as follows:

- Q3-1: Update component 1 to “Support of only GSO or NGSO scenario (or both GSO and NGSO if the indication is absent) for OCC with single-tone NPUSCH format 1 with 15 kHz / 3.75 kHz SCS in RRC_CONNECTED.”, and add in Note column “Note: Both GSO and NGSO are applicable if the indication is absent”

| Companies | Comment |
| --- | --- |
| Moderator | In general, it seems a bit unclear the above proposals are essential or nice-to-have, and in case it is nice-to-have, my preference is not to adopt that. Hence, I would like to ask especially proponents to clarify why this change is essential. |
| Ericsson | The proposed update is not essential. Given the issue was highly controversial we prefer to keep the wording as agreed in RAN1# 121. |
| Apple | The current FG 1-3 is stable. Prefer to keep as it is. |
| Huawei, HiSilicon | We do not see any difference. Prefer to keep as it is. |
| ZTE | Not essential and current version can be kept. |
| Nokia, NSB | It should be like similar FG in legacy. |
| Xiaomi | We don’t see any difference. |
| OPPO | We also support to keep as it is. |

Question 4:

Companies are encouraged to provide views on the need for new FG for the following:

- Q4-1: FG for “OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED”

| 1-4 | OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED | 1. Support OCC for PUSCH with interleaved multi-TB scheduling in RRC_CONNECTED | One of {1-1, 1-2}, npusch-MultiTB-Interleaving-r16 | Yes | N/A | UE does not support OCC for NPUSCH format 1 with interleaved multi-TB scheduling in RRC_CONNECTED | Per UE | FDD only | N/A | Note: This FG is applicable only to IoT-NTN FDD band | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Q4-2: FG for “OCC feature for uplink transmission for NB-IoT” (Note: this FG will be a prerequisite for original FG 1-1/1-2)

| 1-1 | OCC feature for uplink transmission for NB-IoT | 1.Support receiving enabling of the OCC feature in RRC signalling | Rel-17 2-1b | Yes | N/A | UE does not support OCC for uplink transmission for NB-IoT via satellite | Per UE | FDD only | N/A | Note: This FG is applicable only to IoT-NTN FDD band | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Companies | Comment |
| --- | --- |
| Ericsson | Q4-1: There is no need for it, Multi-TB grant has not been agreed to be supported along with OCC. Q4-2: It is unclear to us the motivation behind adding yet another FG, it seems a bit late since it impacts both FG 1-1/1-2. |
| Vivo1 | Q4-1: this should be discussed in 8.11.4 first. We have no corresponding agreement to support multi-TB with OCC yet. Q4-2: the motivation of the proposal is not clear. We already introduced an OCC enabling RRC parameter, and only for UE supporting FG1-1/1-2, this RRC parameter can be configured. why a new FG 1-1 is needed? WI code; Sub-feature group; RAN1 specification; Section; RAN2 Parent IE; RAN2 ASN.1 name; Parameter name in the spec; New or existing?; Parameter name in the text. IoT_NTN_Ph3; TS36.212; 6.4.3.1; NPUSCH-ConfigDedicated-NB; npusch-OCC-Enabled; new; npusch-OCC-Enabled. |
| Apple | Before introducing this new feature, the agreements to support multi-TB with OCC need to be agreed first. |
| Huawei, HiSilicon | Q4-1, We can hold the discussion until WI agreed it. Q4-2, not clear the motivation to add this FG. |
| ZTE | Q4-1, no agreement has been achieved yet and hence no need of FG for multi-TB. Q4-2, for DL HARQ feedback disabling in Rel-17, it was introduced an overall FG 34-2 indicating whether support feedback disabling. And 3 FGs 26-6, 26-6a, 26-6b are introduced for detailed HARQ codebook enhancements and take the overall FG as pre-requisite. We think similar structure can be reused here, i.e., an overall FG indicating support of OCC, and 2 FGs for detailed enhancements related to different SCS. |
| Nokia, NSB | Q4-1: not add before any RAN1 agreement. Q4-2: not support as it can not work alone. |
| Xiaomi | Q4-1: Not add before any RAN1 agreement. Q4-2: Can’t see any necessity to add this FG. |
| OPPO | Q4-1: wait for RAN1’s process Q4-2: the motivation is not clear |

- Conclusions

This document summarizes companies’ inputs for several issues on IoT-NTN UL related UE features.

References

- R1-2505342	Discussion on UE features for NTN for Internet of Things (IoT) Phase 3	CATT

- R1-2505346	UE features for IoT-NTN Phase 3	Huawei, HiSilicon

- R1-2505568	UE features for IoT NTN Phase 3	Samsung

- R1-2505709	Discussion on UE features for NTN for Internet of Things Phase 3	OPPO

- R1-2506257	Discussion on the UE feature for IoT-NTN phase 3	ZTE Corporation, Sanechips