3GPP TSG RAN WG1 #122bis		                                         R1-2506799

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	8.7.1

Source:	Spreadtrum, UNISOC

Title:	Remaining issues on NR-NTN

Document for:	Discussion and decision

Introduction

In this paper, we share our views on remaining issues of NR-NTN.

NR-NTN downlink coverage enhancement

Common PDCCH repetition for TN

PDCCH schemes of Type0-PDCCH CSS and Type0A/1/2-PDCCH CSS defined for NTN are extended to TN. However, inter-slot PDCCH repetition of Type0-PDCCH CSS is bundled with SIB1 PDSCH repetition, i.e. they are enabled or disabled by 1 bit in MIB. NTN UE may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS. However, for TN UE supporting common PDCCH repetition, it needs to clarify the slot of SIB1 PDSCH receptions, slot n or slot n+1.

A UE capable of PDSCH repetitions for broadcast channels, which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS, with the same RV as indicated by the DCI format 1_0.

We give some analysis for the two alternatives, and propose UE assume PDSCH only exist in the latter slot, i.e. slot n+1. If assuming PDSCH in slot n, UE has to buffer all symbols in slot n. After it receives the PDCCH in the slot n+1 and does soft combining to decode PDCCH, and then come back to the PDSCH in early slot n. Clearly, it has a reversed PDSCH processing, such as later slot and then early slot. Thus, it is not friendly for UE implementation. On the contrary, if UE assuming PDSCH in slot n+1, it has a streamline PDCCH decoding and PDSCH processing operation, which is also same as in current specification that PDSCH always in same or latter slot as its own PDCCH.

Therefore, the following TP#1 can be adopted for 38.214.

- TP#1 can be adopted for 38.214.

WI Code: TEI19 [Common_PDCCH_Rep_TN] Reason for change: It is not clear the slot of PDSCH if common PDCCH repetition for TN for FR1 is enabled. Summary of change: UE assume PDSCH scheduled by DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero is in the latter slot. Consequence if not approved: UE behavior for PDSCH reception is not clear.

- 5.1 UE procedure for receiving the physical downlink shared channel *** Unchanged parts are omitted *** A UE capable of PDSCH repetitions for broadcast channels, which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS, with the same RV as indicated by the DCI format 1_0. A UE incapable of PDSCH repetitions for broadcast channels, which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCH scheduled by the DCI format 1_0 is transmitted in the latter slot as the Type0 PDCCH CSS. *** Unchanged parts are omitted ***

NR-NTN uplink capacity/throughput enhancement

CSI reference resource

For resolving the overlapping PUSCH repetitions with inter-slot OCC and P/SP CSI on PUCCH with/without repetitions, the legacy definition of CSI reference resource is impacted. The first PUSCH repetition of an OCC group can be used instead of uplink slot of P/SP CSI report for CSI reference resource determination. So on top of previous discussions we propose the following TP#2 which only limit to P/SP CSI report.

- TP#2 can be adopted for 38.214.

Reason for change: Since there may be situations where the CSI report is adjusted to start at the start of an OCC period, the corresponding CSI reference resource also needs to be adjusted accordingly. Summary of change: First PUSCH repetition of an OCC group is used instead of uplink slot of P/SP CSI report for CSI reference resource determination Consequence if not approved: UE behavior for PDSCH reception is not clear.

- 5.2.2.5 CSI reference resource definition *** Unchanged parts are omitted *** The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' , or a CSI reporting in the OCC group starting in uplink slot n' if UE would multiplex periodic and semi-persistent CSI reporting from a PUCCH transmissions in the PUSCH transmission with repetitions in an OCC group, is defined by a single downlink slot n-nCSI_ref-Koffset⋅2μDL2μKoffset, where Koffset is a parameter configured by higher layer as specified in clause 4.2 of [6, TS 38.213], and where μKoffsetis the subcarrier spacing configuration for Koffset with a value of 0 for frequency range 1 and for FR2-NTN, *** Unchanged parts are omitted ***

SR on PUCCH with repetition/high priority

UE does not expect to transmit more than one PUCCHs that provide HARQ-ACK information or CSI reports and would overlap with the PUSCH transmission in different slots in the OCC group. Similarly, it is good to reduce the complexity for UCI multiplexing of SR PUCCH with repetitions or high priority.

According to the discussion in the previous meeting, we propose the following as a conclusion.

- In case HARQ-ACK and/or CSI report(s) would be multiplexed on PUSCHs in an OCC group and at least one SR with high priority index and/or with PUCCH repetition is scheduled in a different slot in the same OCC group, the UE behavior is up to implementation.

Phase continuity and power consistency for OCC with PUSCH

RAN4 confirmed that same requirements of phase continuity for DMRS bundling can be reused for a OCC group with PUSCH.

RAN4 has discussed the phase continuity requirements for the duration of one OCC group with PUSCH, and confirmed that the same requirements for DMRS bundling can be applied. Specifically, ≤25° between adjacent slots. The side conditions will also be aligned with DMRS-bundling requirement. It is RAN4 opinion that UE should be able to indicate support for OCC independently for GSO and NGSO deployments.

From RAN1 perspective, if phase continuity and power consistency cannot be satisfied, there is an event to a nominal TDW. Thus, UE still transmit PUSCHs/PUCCHs and do not require maintaining phase continuity and power consistency across actual TDWs. However, it is different for PUSCH with OCC. Since UE cannot transmit PUSCH repetitions at all if there is an event.

Among all the events defined for DMRS bundling, only other uplink transmissions are scheduled between the two consecutive PUSCH transmissions matters phase continuity and power consistency. To simplify the discussion, we can go with other uplink transmission between any two PUSCH repetitions within an OCC group can be avoided by gNB implementation.

Second, there are additional events for redcap HD UE, including DL cancel UL or DL reception between two consecutive PUSCH in an OCC groups. We can easily go with redcap HD UE does not support PUSCH repetitions with inter-slot OCC to save time. Due to too many collision cases, it is very hard to finish them in maintenance phase.

- For maintaining phase continuity and power consistency for the duration of one OCC group with PUSCH, other uplink transmission between any two PUSCH repetitions within an OCC period can be avoided by gNB implementation.S

- Redcap HD UE does not support PUSCH repetitions with inter-slot OCC in Rel-19 NTN.

Conclusion

In this contribution, we have following proposals:

- TP#1 can be adopted for 38.214.

- TP#2 can be adopted for 38.214.

- In case HARQ-ACK and/or CSI report(s) would be multiplexed on PUSCHs in an OCC group and at least one SR with high priority index and/or with PUCCH repetition is scheduled in a different slot in the same OCC group, the UE behavior is up to implementation.

- For maintaining phase continuity and power consistency for the duration of one OCC group with PUSCH, other uplink transmission between any two PUSCH repetitions within an OCC period can be avoided by gNB implementation.

- Redcap HD UE does not support PUSCH repetitions with inter-slot OCC in Rel-19 NTN.

Reference

- RP-243300  Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

- R1-2506732 Reply LS on requirements for phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3	RAN4, vivo

- R1-	Feature lead summary #2: NR-NTN uplink capacity and throughput enhancements	Moderator (MediaTek)