3GPP TSG RAN WG1 #122bis	                                           R1-2507017

Prague, Czech, Oct 13th – 17th, 2025

Source: 	CMCC

Title:	Discussion on channel coding for 6GR interface

Agenda item:	11.4.1

Document for:	Discussion & Decision

- Introduction

In RAN1#122 meeting, 3GPP RAN1 started to study the channel coding schemes for 6GR and the following chairman guidance was provided for future study [1]:

Chairman Guidance For 6GR control channel coding, - Evaluations can be provided in form of BLER and FAR results. - Evaluations/analysis can be provided for complexity, decoding latency, - Other metrics are not precluded. - Proponent companies to provide evaluation assumptions and methodologies for respective evaluation. - Proponent companies to provide details of channel coding extension compared with NR channel coding - Proponent companies to provide justification for the channel coding extension, compared with control channel codes as defined in 5G NR. Chairman Guidance For 6GR data channel coding, - Evaluations can be provided in form of BLER results. - Evaluation/analysis on throughput, complexity, and decoding latency can be provided - Other metrics are not precluded. - Proponent companies to provide their target scenarios and requirements, evaluation assumptions and methodologies for respective evaluation/analysis, e.g., decoding algorithm and details, information sizes, code rates, HARQ scheme, channel type, modulation order, target BLER, etc. - Proponent companies to provide details of channel coding extension compared with NR channel coding. - Proponent companies to provide justification for the channel coding extension, and how to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off, compared with data channel codes as defined in 5G NR.

In this contribution, considerations of several candidate channel coding schemes are presented.

- Channel coding schemes for 6GR interface

## 2.1 Polar codes

Polar code is adopted in 5G systems for uplink/downlink control information (DCI/UCI), due to its good error-correction performance for short-block length (e.g., ~200bits). However, with the advent of 6G, the size of control information is expected to expand, posing new challenges for the application of Polar codes.

Larger UCI payload: 6G may adopt wider bandwidths such as the 6425~7125 MHz frequency-band (U6G), and may deploy massive MIMO systems with more antenna ports (e.g., 256) [2-4]. These developments will increase the payload requirements for Channel State Information (CSI) feedback, as the system needs to capture more detailed channel characteristics across broader frequency ranges and larger antenna arrays. For instance, in massive MIMO systems with 256 antennas ports, the CSI payload may exceed ~1000 bits to maintain sufficient accuracy.

Larger DCI payload: Firstly, subband precoding in uplink requires TPMI to be indicated in each subband, which largely increases the DCI payload. Secondly, in ultra-high-density user scenarios such as firework displays, concerts, or other mass gathering events, the conventional 5G PDCCH capacity faces significant challenges. To address these challenges, we expect 6G to introduce single-DCI schedule multiple UEs in order to reduce overall PDCCH overhead by common CRC across multiple UEs. In such cases, the DCI payload would significantly increase by multiple times.

While 5G Polar codes is designed for short blocks, their performance for larger DCI/UCI payload in 6G requires careful evaluation. Polar codes’ potential limitations for larger DCI/UCI payload in 6G includes:

- Higher decoding latency: SC/SCL complexity grows as O(N*logN), becoming non-negligible when the block length N extends to the hundreds of bits. This is useful especially considering reducing the number of blind decoding.

- Degraded error performance: Polar codes exhibit progressively degraded BLER performance for longer block lengths compared to alternatives like LDPC codes.

- Inherent short-block optimization: 5G Polar code implementations - including their interleaving patterns, reliability sequences, and rate matching schemes - are fundamentally designed for limited block sizes (~200 bits), making them unapplicable/suboptimal for extended 6G DCI/UCI lengths.

To maintain Polar code’s advantages while meeting 6G requirements, enhanced Polar coding schemes could be considered.

Observation 1: 5G Polar code may not be optimal for the case of longer DCI/UCI straightforwardly, due to higher decoding latency, degraded error performance and inherent short-block optimization.

Proposal 1:  5G Polar code enhancement for 6G could be considered for larger payload size.

A backward-compatible solution (leveraging 5G Polar code implementations through message segmentation) is segmenting long control messages into smaller sub-blocks, which are encoded by Polar codes separately and transmitted together. One key advantage of this backward-compatible approach is that it preserves - rather than expands - the overlapping bit-length range between Polar codes and LDPC codes, which benefits the balance between performance and implementation cost.

Proposal 2: Prefer Polar enhancement that preserves - rather than expands - the overlapping bit-length range between Polar codes and LDPC codes, such as leveraging 5G Polar code implementations through message segmentation.

The segmented Polar coding framework can be further optimized through concatenated coding schemes, where an outer/inner code provides additional error protection across the sub-blocks while preserving the inherent benefits of short Polar codes. For example, an inner code can be applied across the Polar-encoded sub-blocks, generating additional bits that establish logical relationships between sub-blocks. At the receiver, these additional constraints enable iterative decoding—initially correcting errors within each Polar-decoded sub-block, then leveraging cross-block additional constraints to refine the decoding of sub-blocks. The inner code can be inner single parity-check (SPC) code or inner Polar code.

Proposal 3: If segmented Polar coding is adopted, enhancement such as outer/inner code across sub-blocks could be considered to provide additional error protection across the sub-blocks.

Beyond the backward-compatible solution, revolutionary enhancements could also be explored, such as potential enhancement on Polar encoding to reduce decoding complexity while maintaining error-correction performance.

Proposal 4: Prefer Polar enhancement that reduces decoding complexity while maintaining error-correction performance.

## 2.2 Low density parity check (LDPC) codes

5G NR systems adopted quasi-cyclic (QC) LDPC codes for data channel encoding due to their excellent performance, high throughput, and hardware-friendly parallel decoding capabilities. The core of any LDPC code is its sparse binary parity-check matrix H, which defines the constraints between the codeword bits. The encoding process of LDPC codes is to generate codeword bits c that satisfy the equation HcT=0T. Therefore, the structure of H critically determines the LDPC codes’ error correction capability, iteration depth, and decoding parallelism. A well-designed sparse H enables efficient decoding.

To efficiently support a wide range of block lengths and code rates, 5G NR defines two fundamental base graphs (BG). BG1 is a larger matrix (46 rows x 68 columns) supporting higher code rates and optimized for larger block lengths, while BG2 is a smaller matrix (42 rows x 52 columns) supporting lower code rates and optimized for smaller block lengths. The system will dynamically choose BG1 or BG2 based on the required code rate and transport block size for each transmission. The BG itself defines a relatively small prototype code, and a lifting process of scaling up this base graph is required to create a much larger parity-check matrix H matching the required code block length. In detail, a lifting size Z is chosen from a predefined set (See Table 5.3.2-1 in TS 38.212) and each entry in the BG is replaced by a Z×Z submatrix depending on the value of the entry. In case of non-zero values, a cyclically shifted Z×Z identity matrix is used, and the specific shift values are defined in the specification (See Table 5.3.2-2/5.3.2-3 in TS 38.212).

As the 5G NR design of LDPC codes supports a wide range of block lengths and code rates, which can cover major use cases, although 6G expects a higher peak data rate, it should be the baseline for 6GR study of LDPC codes.

Proposal 5: 5G NR design of LDPC codes should be considered as the baseline scheme for the study of LDPC codes for 6GR.

By reviewing the data channel coding design in previous generations of mobile communication systems, it can be observed that enhancements were consistently driven by increasing demands for improved key performance indicators, particularly data rates. For 6GR, 3GPP TSG RAN#106 initiated a study on 6G scenarios/use cases and requirements [6]. In RAN#107, it was agreed to set a target value of 400MHz for system bandwidth, which is 4 times that of 5G NR. Furthermore, according to the agreement achieved in the recently concluded RAN#109 meeting [7], the minimum requirements of peak spectral efficiencies are 60 bps/Hz for the downlink and 30 bps/Hz for the uplink, respectively, which are 2 times that of 5G NR.

Given these increased requirements of peak spectral efficiency and system bandwidth, it is reasonable to expect that 6G will target a higher peak data rate or throughput than 5G NR. For example, considering the 2 times improvement in peak spectral efficiency and a carrier bandwidth of 200 MHz, a peak data rate 4 times that of 5G NR can be expected, i.e., reaching 80 Gbps for the downlink and 40 Gbps for the uplink.

Observation 2: Given the agreements in RAN plenary on minimum requirements of system bandwidth and peak spectral efficiency, 6GR is expected to achieve much higher peak data rates, e.g., 80 Gbps for the downlink and 40 Gbps for the uplink, compared to the 20/10 Gbps target for 5G NR.

In RAN1#122 meeting, companies discussed the evaluation formula for decoding throughput of LDPC codes [8]. From our perspective, the decoding throughput of LDPC codes can be approximated as follows:

Tput≈c×Kb×ZI×Titer [bps],

where c is the number of LDPC decoder cores, Kb is the number of columns for information bits in a base graph, Z denotes the lifting size, I denotes the number of decoding iterations, and Titer denotes the decoding time per iteration.

To meet a higher throughput requirement for 6GR, enhancements to the design of LDPC codes can be further explored. According to the above evaluation formula of decoding throughput, the potential enhancements include:

- Increased lifting size

In the 5G NR system, the maximum value of lifting size Z is 384. Expanding the maximum lifting size would enable the support of larger information block lengths. This is crucial for achieving the higher peak throughput targets of 6G. However, expanding the maximum lifting sizes may also lead to proportionally increased circuit gates in the LDPC decoder. So, considering the limited chip area, it may not be cost-efficient to improve the peak throughput by significantly expanding the maximum lifting size.

- Enhanced decoding parallelism via BG optimization

Achieving the ultra-high throughput required by 6G necessitates massive parallelism in LDPC decoding, that is, a reduced decoding time per iteration Titer. However, the current BG design in 5G NR, particularly within its kernel region, suffers from limited parallelism. This stems from the dense connections between rows and columns in this region, creating dependencies that prevent multiple rows from being processed simultaneously. Optimizing the BG structure to reduce these dependencies and increase the degree of row-layer independence is essential. The goal is to enable a larger number of rows to be decoded concurrently within each clock cycle.

- Reduced number of iterations via BG optimization

The number of iterations I required for successful decoding significantly impacts both decoding latency and overall throughput. More iterations mean longer decoding times and higher computational resource utilization per codeword. Optimizing the BG design, e.g., by improving girth characteristics to minimize detrimental message-passing loops and refining degree distributions for faster convergence can accelerate the decoding process. Reducing the number of iterations needed for convergence leads directly to lower decoding latency, improved chip area efficiency (as decoders can process more codewords per unit time), and consequently, higher effective throughput.

Proposal 6: Enhanced LDPC coding schemes can be further studied to meet the challenging requirements on peak data rate and throughput for 6GR. The following are considered but not limited to

- Increased lifting size

- New BG design including, decoding parallelism optimization, number of iterations reduction, and etc.

## 2.3 Medium payload size (3-11 bits)

In 5G systems, Reed-Muller (RM) code is adopted for UCI ranging from 3 to 11 bits. Further optimization of the medium payload size can be considered since for such use case. On example direction is to consider coding scheme that requires more complex decoding algorithm to improve the decoding performance, since the payload size is very limited.

And if RM code is replaced by an optimized coding scheme in 6G (e.g., through further enhancements or alternative codes), it may be necessary to reconsider the applicable bit range for the new scheme, particularly the upper limit of 11 bits.

Proposal 7: If RM code for UCI ranging from 3 to 11 bits is replaced by an optimized coding scheme in 6G, the upper limit of 11 bits could be reconsidered.

- Conclusion

In this contribution, considerations of channel coding schemes for 6GR interface are presented. In summary, we have the following observations and proposals:

Observation 1: 5G Polar code may not be optimal for the case of longer DCI/UCI straightforwardly, due to higher decoding latency, degraded error performance and inherent short-block optimization.

Observation 2: Given the agreements in RAN plenary on minimum requirements of system bandwidth and peak spectral efficiency, 6GR is expected to achieve much higher peak data rates, e.g., 80 Gbps for the downlink and 40 Gbps for the uplink, compared to the 20/10 Gbps target for 5G NR.

Proposal 1:  5G Polar code enhancement for 6G could be considered for larger payload size.

Proposal 2: Prefer Polar enhancement that preserves - rather than expands - the overlapping bit-length range between Polar codes and LDPC codes, such as leveraging 5G Polar code implementations through message segmentation.

Proposal 3: If segmented Polar coding is adopted, enhancement such as outer/inner code across sub-blocks could be considered to provide additional error protection across the sub-blocks.

Proposal 4: Prefer Polar enhancement that reduces decoding complexity while maintaining error-correction performance.

Proposal 5: 5G NR design of LDPC codes should be considered as the baseline scheme for the study of LDPC codes for 6GR.

Proposal 6: Enhanced LDPC coding schemes can be further studied to meet the challenging requirements on peak data rate and throughput for 6GR. The following are considered but not limited to

- Increased lifting size

- New BG design including, decoding parallelism optimization, number of iterations reduction, and etc.

Proposal 7: If RM code for UCI ranging from 3 to 11 bits is replaced by an optimized coding scheme in 6G, the upper limit of 11 bits could be reconsidered.

- References

- Chair notes, RAN1#122, Bengaluru, India, Aug 25th-29th, 2025.

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- J. Zhang, E. Björnson, M. Matthaiou, D. W. K. Ng, H. Yang and D. J. Love, “Prospective Multiple Antenna Technologies for Beyond 5G,” in IEEE Journal on Selected Areas in Communications, vol. 38, no. 8, pp. 1637-1660, Aug. 2020.

- H. Miao et al., “Sub-6 GHz to mmWave for 5G-Advanced and Beyond: Channel Measurements, Characteristics and Impact on System Performance,” in IEEE Journal on Selected Areas in Communications, vol. 41, no. 6, pp. 1945-1960, June 2023.

- E. Arikan, “Channel Polarization: A Method for Constructing Capacity-Achieving Codes for Symmetric Binary-Input Memoryless Channels,” in IEEE Transactions on Information Theory, vol. 55, no. 7, pp. 3051-3073, July 2009.

- RP-243327, New SID: Study on 6G scenarios/use cases and requirements, RAN#106, Madrid, Spain, December 9-12, 2024.

- RP-252946, Draft Reply LS to ITU-R_WP5D_TEMP_167 = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#109, Beijing, China, September 15-18, 2025.

- R1-2506590, FL summary#2 for data channel coding, RAN1#122, Bengaluru, India, Aug 25th – 29th, 2025.