3GPP TSG RAN WG1 #122-bis                                       R1-2507369

Prague, Europe, October 13th – 17th, 2025

Source:	Sharp

Title:	Views on evaluation assumptions and performance evaluation for ISAC

Agenda Item:	10.5.1

Document for:	Discussion and Decision

# Views on Metrics for Performance evaluation

In the last meeting in RAN1#122 [1] the following performance metrics were agreed: Horizontal/vertical positioning accuracy, Velocity accuracy (includes both radial and true velocity estimates), Missed detection probability and False alarm probability.

False alarm probability:

Agreement from RAN1#122 [1]:

  - False alarm probability Type 2 (targets dropped in simulation area): An object is detected but not associated with any true targets in the simulation area is considered as a false alarm.

Pf2=0≤n<NMn'≠0Dn'Mn'K

Where,

      - Dn' is the number of detected objects but not associated with any true targets in the drop n.

      - Mn' is the total number of detected objects in the drop n.

      - FFS:

        - Option 1: K is number of drops (N)

        - Option 2: K is number of drops with at least one detected object

In our view, the basic difference in Option 1 and Option 2 is when Mn' = 0 in Option 1 we count those drops in K whereas, in Option 2 we do not count those drops in K. However, in simulations mostly likely most of the drops will have Mn' > 0 (i.e. some object will be detected irrespective of whether it is associated with a target or not if there are true targets in the drop) for false alarm type 2 making Option 1 and Option 2 yield similar results with negligible difference. Therefore, generally it doesn’t make much of a difference choosing either Option 1 or Option 2.

In Option 1, K is the total number of drops (N). In some drops that contain zero detected objects (Mn'= 0) based on the formula, drops with Mn'= 0 are excluded from the summation (to avoid division by zero), but including those drops in the denominator would lead to a systematic underestimation of the true average false detection ratio. By artificially increasing the denominator with drops that did not contribute to the error summation, Option 1 makes the system look better than its actual operating average. Hence, we think Option 2 reports the true, unbiased mean false detection ratio. Thus, we prefer Option 2.

Proposal 1: Option 1 or Option 2 will ideally have negligible difference in system level simulations as most drops will have likely have Mn' > 0. However, for mathematical consistency and conservative estimates we prefer Option 2: K is number of drops with at least one detected object.

The other metrics which are defined in TS 22.837 [3] that required further discussion are:

- Sensing Resolution

  - Range resolution

    - Horizontal [m] and/or vertical [m]

  - Velocity resolution

    - Horizontal [m/s] and vertical [m/s]

- Service Latency and Refresh Rate

Sensing Resolution:

As per the definition in TS 22.837 [2] sensing resolution describes the minimum difference in the measured magnitude of target objects (e.g. range, velocity) to be allowed to detect objects in different magnitude. This in simple terms means “How well a system can separate two targets that are close together in some property like range, velocity, or angle.” In the realm of literature, the Range resolution is defined as ∆r = c/2B, while the velocity resolution is defined as ∆v = λ/(2T_CPI). Here, B represents the bandwidth, T_CPI stands for the coherent processing interval, and c denotes the speed of light. These theoretical formulas in literature are derived based on FFT. However, the utilization of super-resolution algorithms for sensing can theoretically surpass the theoretical limits achieved through FFT and yield better results. Additionally, in practical systems, noise, clutter, limited SNR, multipath, interference, non-ideal algorithms, and hardware imperfections degrade the ability to separate targets thus the “effective sensing resolution” can be worse than the theoretical limits. In real simulations, we can easily perform worse than the theoretical limits for example when SNR is low (targets get smeared into noise floor), targets are correlated in angle/velocity (harder to separate), clutter, multipath, or channel impairments exist, and windowing or processing losses reduce effective sensing resolution. Thus, in real simulations, the effective sensing resolution is often worse than the nominal system setting, and in some cases better if advanced algorithms are used under high SNR.

However, as per the agreement in RAN1#122 [1], we agreed to the following simulation assumptions:

| Parameter | Value |
| --- | --- |
| Minimum target-target (3D) distance | 10 m |
| System bandwidth | 100 MHz |
| Carrier frequency | Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz] |

The theoretical range resolution is given by ∆r = c / (2B). With the entire 100 MHz bandwidth allocated for sensing, the systems range resolution is ∆r ≈ 1.5 m. Since the assumed minimum target-target (3D) distance in simulations is 10 m, the performance is not limited by the theoretical sensing range resolution of the system.

Furthermore, with 15 MHz of bandwidth allocated for sensing, the resolution is ∆r ≈ 10 m, which remains consistent with the assumed minimum target-target (3D) distance. For any bandwidth allocated for sensing larger than 15 MHz, the theoretical sensing range resolution of the system improves further. Conversely, for bandwidth allocated for sensing smaller than 15 MHz, the theoretical sensing range resolution of the system exceeds the assumed minimum target-target (3D) distance. In such cases, even if simulations assume a 10 m minimum separation distance between targets, it cannot be guaranteed that two targets at this distance can be resolved, since the systems range resolution limit would be larger than the minimum target-target (3D) distance.

Observation 1: Under the current simulation assumptions for minimum target-target (3D) distance of 10 m and for the bandwidth allocated for sensing is at least 15 MHz, the system is not constrained by sensing range resolution for UAV detection use case as per the requirement defined in TS 22.387 [2].

For the sensing velocity resolution, to detect the existence of UAV, the sensing resolution of velocity is 10 m/s as per TS 22.387 [2]. Based on ∆v = λ/(2T_CPI) and simulations assumptions shown in the Table above (agreements from RAN1#122 [1]), the minimum CPI required to meet the KPIs listed in TS 22.387 [2] is 3.75 ms @ 4 GHz, 3.06 ms @ 4.9 GHz and 2.50 ms @ 6 GHz.

Observation 2: The CPI values are still open for discussion in RAN1 evaluation assumptions. However, for UAV detection use case the minimum value of CPI for the system should be greater than or equal to the sensing velocity resolution limits defined in TS 22.387 [2], for the system to be no limited by the sensing velocity resolution.

In summary, for any use case, if the allocated sensing bandwidth is such that the theoretical range resolution is less than or equal to the minimum target-target (3D) distance assumed in simulations, the system is not constrained by range resolution. Similarly, if the chosen CPI is greater than or equal to the value required to meet the sensing velocity resolution, the system is not constrained by velocity resolution.

Proposal 2: Sensing resolution for range and velocity is not considered for performance evaluation in NR ISAC if the following conditions are satisfied for any use case:

- Range resolution: The allocated sensing bandwidth B shall be chosen such that the theoretical range resolution ∆r = c / (2B) ≤ minimum target-target (3D) distance assumed in the simulations.

- Velocity resolution: The coherent processing interval (CPI) time (TCPI) shall be chosen such that TCPI≥λ2Δvreq , where Δvreq is the required velocity resolution.

Service Latency and Refresh Rate

TS 22.837 [2] defines max sensing service latency and refreshing rate as:

Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results.

Consequently, based on the definition of sensing service latency in SA1 aspects beyond the Physical layer/gNB-UE air interface are considered for determining the service latency.

Proposal 3: Service latency is not considered for performance evaluations for ISAC NR.

The refresh rate can be used to establish the maximum fundamental limit on the coherent processing interval (CPI) time TCPI.

TCPI <= 1/(Refresh Rate (Hz))

The refresh rate can be utilized for UAV detection and tracking as per TS 22.837 [2] and the maximum limits on TCPI can be set in the simulations accordingly. For instance, for a refresh rate of 1 Hz from Table 5.6.10-1, the TCPI is less than or equal to 1 second.

Proposal 4: RAN1 can define the upper bound on coherent processing interval (CPI) time (TCPI) as TCPI <= 1/(Refresh Rate (Hz)) to be used in simulations for various use cases.

# Views on KPI’s for Metrics for Performance evaluation

Some of the KPIs in Tables 5.10.6-1, 5.12.6-1 and Table 5.13.6-1 in TS 22.837 [2] for UAV related use cases lack predefined values such as:

- Confidence level percentage [%] in Table 5.10.6-1

- Missed detection [%] and false alarm [%] in Table 5.12.6-1

- Definition of Level 1 and Level 2 in Table 5.13.6-1

Proposal 5: The confidence intervals and values of the KPIs defined in TS 22.837 [2] – Tables 5.10.6-1, 5.12.6-1 and Table 5.13.6-1 can be used as starting point for performance evaluations. The confidence intervals and values of the KPI can be further modified/updated (if necessary) based on RAN1 evaluations at a later stage as needed based on performance evaluation results from companies.

# Views on evaluation assumptions

In TR 38.901 [3], the electrical down tilt for the UMa scenario is specified as 102° (Table 7.8-1). No mechanical down tilt is explicitly defined for the UMa scenario. It is noted, however, that in TR 38.901 all antenna elements are mapped to a single port under the electrical down tilt assumption. This makes the applied electrical down tilt logically equivalent to mechanical down tilt.

In the context of UAV detection, the specified 102° electrical down tilt angle may not be appropriate when UAV altitudes range from 25 meters to 300 meters (as per agreement in RAN1#122 [1] the UAV can be 25 m – 300 m and optionally distributed between 1.5 m – 300 m). This assumption for UMa scenario in TR 38.901 is based all antenna elements mapped to a single port thus only one single beam is used in the UMa scenario outlined in TR 38.901. Such a down tilt may be acceptable only for UAVs in the 1.5 m – 25 m height range. It is further noted that while electrical down tilt can be adjusted in practice, mechanical down tilt is fixed once deployed and is not altered like electrical down tilt.

Proposal 6: RAN1 to first clarify the number of beams assumed in simulation and adopt the following evaluation parameters for BS antenna configuration listed in the Table if only one single beam is considered.

| Parameters | Assumptions |
| --- | --- |
| BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 Table 7.3-1 in TR 38.901 |
| BS Mechanic tilt (downtilt angle) | Option 1: 90° |
| BS Electronic tilt | Up to company report |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

For mono-static sensing where the TX/RX are operating simultaneously, in our view the operation mode is in-band full duplex. Thus, in-band full duplex will have much larger self-interference compared to sub band full duplex.

Proposal 7: RAN1 to adopt both the evaluation parameters listed in following Table.

| Parameters | Assumptions |
| --- | --- |
| Antenna isolation | - Option 1: 65 dB with additional Digital SIC isolation of 0 dB - Option 2: 80 dB with Digital SIC isolation of 15 dB |
| BS Rx saturation power | -28dBm |
| BS Tx power | When Tx/Rx operates simultaneously, - BS_maxpower = BS Rx saturation power + Antenna Isolation, i.e, - Option 1: 37dBm - Option 2: 52dBm Otherwise, max BS Tx power is 56dBm |

# Others

We are option to studying both use cases for UAV detection and tracking. However, UAV detection can be treated as the mandatory baseline, and it is up to companies to provide results on UAV tracking for performance evaluations.

Proposal 8: UAV detection is mandatory and considered as baseline, UAV tracking can be optional for NR ISAC. Up to companies to report results for UAV tracking.

# Conclusion

Observation 1: Under the current simulation assumptions for minimum target-target (3D) distance of 10 m and for the bandwidth allocated for sensing is at least 15 MHz, the system is not constrained by sensing range resolution for UAV detection use case as per the requirement defined in TS 22.387 [2].

Observation 2: The CPI values are still open for discussion in RAN1 evaluation assumptions. However, for UAV detection use case the minimum value of CPI for the system should be greater than or equal to the sensing velocity resolution limits defined in TS 22.387 [2], for the system to be no limited by the sensing velocity resolution.

Proposal 1: Option 1 or Option 2 will ideally have negligible difference in system level simulations as most drops will have likely have Mn' > 0. However, for mathematical consistency and conservative estimates we prefer Option 2: K is number of drops with at least one detected object.

Proposal 2: Sensing resolution for range and velocity is not considered for performance evaluation in NR ISAC if the following conditions are satisfied for any use case:

- Range resolution: The allocated sensing bandwidth B shall be chosen such that the theoretical range resolution ∆r = c / (2B) ≤ minimum target-target (3D) distance assumed in the simulations.

- Velocity resolution: The coherent processing interval (CPI) time (TCPI) shall be chosen such that TCPI≥λ2Δvreq , where Δvreq is the required velocity resolution.

Proposal 3: Service latency is not considered for performance evaluations for ISAC NR.

Proposal 4: RAN1 can define the upper bound on coherent processing interval (CPI) time (TCPI) as TCPI <= 1/(Refresh Rate (Hz)) to be used in simulations for various use cases.

Proposal 5: The confidence intervals and values of the KPIs defined in TS 22.837 [2] – Tables 5.10.6-1, 5.12.6-1 and Table 5.13.6-1 can be used as starting point for performance evaluations. The confidence intervals and values of the KPI can be further modified/updated (if necessary) based on RAN1 evaluations at a later stage as needed based on performance evaluation results from companies.

Proposal 6: RAN1 to first clarify the number of beams assumed in simulation and adopt the following evaluation parameters for BS antenna configuration listed in the Table if only one single beam is considered.

| Parameters | Assumptions |
| --- | --- |
| BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 Table 7.3-1 in TR 38.901 |
| BS Mechanic tilt (downtilt angle) | Option 1: 90° |
| BS Electronic tilt | Up to company report |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |

Proposal 7: RAN1 to adopt both the evaluation parameters listed in following Table.

| Parameters | Assumptions |
| --- | --- |
| Antenna isolation | - Option 1: 65 dB with additional Digital SIC isolation of 0 dB - Option 2: 80 dB with Digital SIC isolation of 15 dB |
| BS Rx saturation power | -28dBm |
| BS Tx power | When Tx/Rx operates simultaneously, - BS_maxpower = BS Rx saturation power + Antenna Isolation, i.e, - Option 1: 37dBm - Option 2: 52dBm Otherwise, max BS Tx power is 56dBm |

Proposal 8: UAV detection is mandatory and considered as baseline, UAV tracking can be optional for NR ISAC. Up to companies to report results for UAV tracking

# References

[1] R1-2506480 “Summary #5 on evaluations for NR ISAC”, Moderator (Xiaomi).

[2] 3GPP TS 22.837 “Feasibility Study on Integrated Sensing and Communication (Release 19)”

[3] 3GPP TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19)”