3GPP TSG RAN WG1 #123	 R1-2508446

Dallas, US, Nov 17th – 21st, 2025

Source:	CMCC

Title:	Discussion on inter-vendor training collaboration for CSI compression

Agenda item:	10.1.2

Document for:	Discussion & Decision

Introduction

In RAN#108 meeting, the New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements[1] was approved, including the following objectives of enhancements for inter-vendor training collaboration for two-sided AI/ML models:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, potential spec impact for standardized dataset format/content plus dataset exchange for two-sided model will be discussed.

Discussion on dataset format/content for inter-vendor training collaboration for CSI compression

In Rel-19, to resolve the issues related to the inter-vendor collaboration, five options were identified.

Agreement @RAN1#116 To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options: - Option 1: Fully standardized reference model (structure + parameters) - Option 2: Standardized dataset - Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side - Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side - Option 5: Standardized model format + Reference model exchange between NW-side and UE-side Note 1: The above options may not be mutually exclusive and may be used together. Note 2: Other options are not precluded. Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc. Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Among these, despite the substantial data volume required for dataset exchange, Option 4-1 are still considered by RAN1 as a viable and effective solution to address inter-vendor training collaboration issues.

For Option 4-1 in Direction A, in the last meeting [5], we have agreed that the dataset shared between NW side and UE side support at least target CSI and CSI feedback.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support at least target CSI and CSI feedback in the exchanged dataset(s). - FFS: Target CSI type and format - FFS: CSI feedback type and format - FFS: Association between Target CSI and CSI feedback, including mapping for different number of Tx port, number of sub bands, and CSI payload size.

Besides, we agreed to support at least precoding matrix as target CSI type for inference and NW side data collection in RAN1#122 meeting [2].

Agreement: For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. - FFS whether consider raw channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc. Agreement: For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,: - Option 0: Reusing legacy e-Type II codebook - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI. - Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization. - Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix. - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) - Note: the quantizaton format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling. - Note: At least take precoding matrix as Target CSI type. FFS channel matrix. - FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

To make sure we have a common design on the target CSI for inference, data collection, monitoring and inter-vendor collaboration training, we suggest taking precoding matrix as target CSI type for inter-vendor collaboration training Option 4-1.

As illustrated in our company’s contribution discussing on inference related aspects of CSI compression [5], in Rel-19 study on AI/ML based CSI compression, when discussing the standardized model structure for Option 3a-1 and Direction A, we have agreed to take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure. So, to make sure that the standardized model structure can work well for Option 3a-1, at least precoding matrix in the spatial-frequency domain in a subband granularity should be supported as the target CSI type for inference with the assumption of inter-vendor collaboration training Option 3a-1.

Although the target CSI used in inference with the inter-vendor collaboration training assumption of Option 3a-1 and Option 4-1 can be different, to ensure that we have unified design for inference aspects for different inter-vendor collaboration training methods. It is suggested that the precoding matrix in the spatial-frequency domain in a subband granularity is also supported as the target CSI type for inference with the assumption of inter-vendor collaboration training Option 4-1.

The format for target CSI for inter-vendor collaboration training should be aligned with that for model input in inference. Then also at least precoding matrix in the spatial-frequency domain in a subband granularity should be supported as the standardized dataset format/content of target CSI for inter-vendor collaboration training Option 4-1.

Proposal 1: There should be a common design for target CSI for inference, data collection, monitoring and inter-vendor collaboration training.

Proposal 2: At least precoding matrix with port-subband domain representation should be supported as the standardized dataset content of target CSI for inter-vendor collaboration training Option 4-1.

As for the standardized data set format of target CSI, we also think there should be a common design for target CSI for inference, monitoring and inter-vendor collaboration training. For the data format of target CSI, we can use codebook-based Rel-16 eType2 as baseline. Furthermore, to get much higher quantized ground-truth CSI, the traditional Rel-16 eType-II may be enhanced for more accurate channel characterization with new/larger parameter(s) for codebook, like much larger number of beam/angle or frequency/delay basis vectors reporting (W1, Wf) and/or much higher resolution quantization for coefficient feedback (W2), and the basic structure of R16 eType II-like codebook, along with the basic concept of spatial domain and frequency domain could be reused as much as possible.

Proposal 3: In AI/ML based CSI compression, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, codebook-based Rel-16 eType2 can be considered as baseline.

Proposal 4: In AI/ML based CSI compression, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the basic codebook structure could be reused, along with the basic concept of spatial domain and frequency domain basis.

Proposal 5: In AI/ML based CSI compression, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the exact supported values of codebook parameters can be studied to make sure high resolution data transfer, e.g. larger number of beam/angle or frequency/delay basis vectors reporting (W1, Wf) and/or higher resolution quantization for coefficient feedback (W2).

For CSI feedback type and format for Option 4-1, whether it should be quantitated needs to be decided. In the last meeting, the following agreement was reached.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options: - Option 1: The exchanged CSI feedback is the latent message before quantization. - Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization. - Note: Quantization codebook is exchanged in addition if not specified in the spec. - Note: Whether or not using the Quantization codebook for model training, is up to UE implementation.

From our point of view, Option-2 is preferred. A same format for target CSI for both dataset exchange in Option 4-1and inference may help reduce spec impact. The discussion regarding the format of CSI feedback is also closely interlinked with the discussion on quantization codebook configuration. If the UE acquires the quantization codebook from the dataset, it can generate the quantized CSI feedback.

Proposal 6: For CSI feedback type and format, support Option 2, i.e. the exchanged CSI feedback is the binary sequence at the output of quantization.

The association between target CSI and CSI feedback is important for training the UE-side model. Because of the different CSI payload size configurations, a single target CSI sample may map to M CSI feedback samples. In the last meeting, the following agreement was made.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, for the association between target CSI and CSI feedback in the exchanged dataset. - Support 1:M mapping between one target CSI sample/set of samples to M CSI feedback samples/sets of samples associated with different CSI payload size configurations - FFS: Association between Target CSI and CSI feedback, including scalability related information for different number of Tx port, sub bands related information. - FFS: Whether/how Target CSI samples/set of samples within an exchanged dataset, with different Tx port and sub-band related information are organized. - FFS: Whether the Target CSI is per layer or per rank when Target CSI is precoder matrix. - FFS: Whether and how to support CSI feedback structure with different payload scalability options - FFS: Other details

To ensure flexibility for different CSI feedback samples corresponding to different configurations and to accurately indicate this information to the UE, it is feasible to assign a unique ID to each CSI feedback sample. The dataset should first include a list where each row corresponds to different scalability-related information for varying numbers of Tx ports and subband-related details. The ID bound to each CSI feedback sample can be used to locate the corresponding row in this list. A target CSI sample may correspond to one or multiple CSI feedback samples, with each CSI feedback sample having its own configuration ID. The UE can extract the appropriate target CSI sample and CSI feedback sample from the entire dataset based on its required configuration.

Proposal 7: To ensure the scalability related information, an ID could be introduced for each of the CSI feedback sample, which corresponding to the different configurations.

Discussion on additional information for inter-vendor training collaboration for CSI compression

Another additional information which should be exchanged along with dataset is pairing ID. In the RAN1#122 meeting, the exchange of single pairing ID was agreed.

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange. - One pairing ID is assigned to one dataset. - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed). - From RAN1 perspective, the uniqueness of the pairing ID needs further studied. FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

The remaining issue of pairing ID is whether it have the impact on pairing ID(s) when additional data samples are added to an exchanged dataset. From our point of view, the additional data samples could comprise another dataset with a same pairing ID as the exchanged dataset, which is generated form the same configuration and NW-side additional conditions. Nevertheless, the additional data samples can be added to the to-exchange dataset by NW implementation before exchange. Or, the additional data samples can be added to the exchange dataset after exchange happens, but the same pairing ID should be kept, otherwise how can vendor training model ensure these added data samples could be used additionally to train or update the current model.

In the last meeting, the FFS part was discussed but no agreement was made. Following is the latest version of the proposal provided by FL.

- Proposal 3-2 (r1): For Option 4-1 under Direction A in AI/ML based CSI compression, consider the following options on the pairing ID(s) signaling when additional data samples are added to an exchanged dataset or when one dataset is split into multiple sub-datasets before dataset exchange: - If the NW side CSI reconstruction model is backward compatible with the exchanged dataset, - Option 1: The same pairing ID is used as the exchanged dataset. - Option 2: The same pairing ID is used as the exchanged dataset, with additional value tag or sub-dataset ID. - Option 3: Different pairing ID is used, with additional signaling of backward compatibility with the exchanged dataset (e.g., pairing ID list that list all the compatible datasets). - Option 4: Different pairing ID is used. - In UE capability report, the UE report all the pairing IDs supported. - Note: NW does not explicitly signal backward compatibility. - If the NW side CSI reconstruction model is NOT backward compatible with the exchanged dataset, different pairing ID is used.

From our point of view, we may need to further clarify the statement "the NW side CSI reconstruction model is backward compatible with the exchanged dataset." If the additional dataset is fully consistent with the previous dataset in all aspects such as configuration, the UE does not need to distinguish between the data of the two datasets. Whether to use the additional dataset for further training of the current model is entirely up to the UE implementation. In this case, multiple datasets can use the same pairing ID, which corresponds to Option 1. If there is a need to distinguish the additional dataset (with justification for such a requirement), Option 2 appears to be a simpler and more straightforward approach than Option 3.

Proposal 8: For inter-vendor-collaboration Option 4-1, when additional data samples are added to an exchanged dataset, the pairing ID should be kept as same.

Conclusions

In this contribution, we discussed potential spec impact for standardized dataset format/content plus dataset exchange for two-sided model, and the following proposals are made.

Proposal 1: There should be a common design for target CSI for inference, data collection, monitoring and inter-vendor collaboration training.

Proposal 2: At least precoding matrix with port-subband domain representation should be supported as the standardized dataset content of target CSI for inter-vendor collaboration training Option 4-1.

Proposal 3: In AI/ML based CSI compression, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, codebook-based Rel-16 eType2 can be considered as baseline.

Proposal 4: In AI/ML based CSI compression, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the basic codebook structure could be reused, along with the basic concept of spatial domain and frequency domain basis.

Proposal 5: In AI/ML based CSI compression, regarding the standardized dataset format of target CSI for inter-vendor collaboration training Option 4-1, the exact supported values of codebook parameters can be studied to make sure high resolution data transfer, e.g. larger number of beam/angle or frequency/delay basis vectors reporting (W1, Wf) and/or higher resolution quantization for coefficient feedback (W2).

Proposal 6: For CSI feedback type and format, support Option 2, i.e. the exchanged CSI feedback is the binary sequence at the output of quantization.

Proposal 7: To ensure the scalability related information, an ID could be introduced for each of the CSI feedback sample, which corresponding to the different configurations.

Proposal 8: For inter-vendor-collaboration Option 4-1, when additional data samples are added to an exchanged dataset, the pairing ID should be kept as same.

References

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Prague, Czech Republic, June 9-13, 2025

- RP-233133, TR 38.843 v2.0.0, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, Qualcomm

- Chairman's Notes RAN1#120bis, Wuhan, China, April 7th – 11th, 2025.

- Chairman's Notes RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

- R1-2507006, Discussion on inference related aspects of CSI compression, Prague, Czech, Oct 13th – 17th, 2025

- Chairman's Notes RAN1#122bis, Prague, Czech, Oct 13th – 17th, 2025