3GPP TSG-RAN WG1 Meeting #123	R1-25xxxxx

# Dallas, USA, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.769 | CR | xxx | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclusion of outcomes of Study on enhancements for solutions for Ambient IoT in NR outdoor for active devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei (editor) |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Ambient_IoT_Outdoor_Active |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TR needs to include the outcomes of the Rel-20 SI on enhancements for solutions for Ambient IoT in NR outdoor for active devices |  |  |  |  |  |  |  |  |
| Summary of change: |  | Agreements made to-date are included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SI outcomes are unreported in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Introduction, 1, 2, 3.1, 3.3, 4.2.1, 4.2.2, 4.3.2, 4.4 (new), 5.2A (new), 6A (new), all clauses within 6A (new), 7.3, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The present version of this draft CR is not for final agreement in RAN1, and will be updated in RAN1#124, intending to a final CR. See the enclosed document for further details. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

# Introduction

In recent years, IoT has attracted much attention in the wireless communication world. More 'things' are expected to be interconnected for improving productivity efficiency and increasing comforts of life. Further reduction of size, complexity, and power consumption of IoT devices can enable the deployment of tens or even hundreds of billions of IoT devices for various applications and provide added value across the entire value chain. It is impossible to power all the IoT devices by battery that needs to be replaced or recharged manually, which leads to high maintenance cost, serious environmental issues, and even safety hazards for some use cases, for example, wireless sensors in electrical power, and petroleum industries.

Most of the existing wireless communication devices are powered by batteries that need to be replaced or recharged manually. The automation and digitization of various industries opens numerous new markets requiring new IoT technologies of supporting batteryless devices with no energy storage capability or devices with energy storage that do not need to be replaced or recharged manually.

An example type of application is asset identification, which presently has to resort mainly to barcodes and RFID in most industries. The main advantage of these two technologies is the ultra-low complexity and small form factor of the tags. However, the limited reading range of a few meters usually requires handheld scanning which leads to labor intensive and time-consuming operations, or RFID portals/gates which leads to costly deployments. Moreover, the lack of interference management scheme results in severe interference between RFID readers and capacity problems, especially in case of dense deployment. It is hard to support a large-scale network with seamless coverage for RFID. Other examples are sensor applications, which have characteristics such as the operating conditions or cost requirements, etc., which mean they cannot be addressed by 3GPP's LPWA technologies. In general, the use cases for these sensors span indoor and outdoor applications. Applications are also shown to have requirements for positioning, with accuracy targets varying between indoor and outdoor scenarios.

In contrast, this study investigates solutions for Ambient IoT, a new IoT technology to open new markets within 3GPP systems, whose number of connections and/or device density can be orders of magnitude higher than existing 3GPP IoT technologies, and which can provide complexity and power consumption orders-of-magnitude lower than existing 3GPP LPWA technologies such as NB-IoT and LTE-MTC. TSG RAN has completed a Rel-18 RAN-level SI on Ambient IoT, producing TR 38.848 which provides a terminological and scoping framework for future discussions of Ambient IoT. This has defined representative use cases, deployment scenarios, connectivity topologies, Ambient IoT devices, design targets, and required functionalities; it also conducted a preliminary feasibility assessment.

The SI reported in this present TR is now to investigate solutions in detail at RAN-WG level for Ambient IoT in 3GPP.

# 1	Scope

## 1.1	Indoor scenarios

The overall objective of the SI is to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

-	~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

-	≤ a few hundred µW peak power consumption has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both R2D and/or D2R amplification in the device. The device’s D2R transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

Referring to the definitions in [2, TR 38.848], this is done in the context of:

-	Deployment scenario 1 (indoor-to-indoor) with Topology 1, and indoor microcell basestation.

-	Deployment scenario 2 (indoor-to-outdoor) with Topology 2 and indoor UE as intermediate node under network control, and outdoor macrocell basestation.

The spectrum considered is FR1 licensed spectrum in FDD, which can be in-band to NR, in guard-band to NR/LTE, or in standalone band(s). The traffic types considered are DO-DTT and DT, focusing on indoor inventory and indoor command representative use cases. The study also assesses whether the harmonized air interface can address the DO-A use case.

Study of the design of energy harvesting signal/waveform is out of scope in Rel-19.

## 1.2	Outdoor scenarios

The objective of the SI is to study necessary and feasible changes to Ambient IoT specified in TS 38.291 [228] to support outdoor scenarios for the following devices:

-	≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

-	≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

Referring to the definitions in [2], this is done in the context of deployment scenario 4 (outdoor-to-outdoor) with Topology 1, with no external carrier wave. The spectrum considered is FR1 licensed spectrum in FDD in-band to NR and in standalone band(s) with R2D in DL spectrum and D2R in UL spectrum. The traffic types considered are DO-DTT, DT and DO-A, focusing on outdoor inventory, outdoor sensor and outdoor command representative use cases. The study assumes that the Ambient IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs and assumes the same air interface for both the device types. The study also addresses positioning for outdoor scenarios.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.848: "Study on Ambient IoT (Internet of Things) in RAN".

[3]	RP-240826: "Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

[4]	3GPP TR 38.869: "Study on low-power Wake-up Signal and Receiver for NR".

[5]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[6]	EPC Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz.

(…Omitted in this document…)

[226]	R4-1706112, "[DRAFT]Reply LS regarding 1024QAM", RAN4, RAN4#83, Hangzhou, China, May 2017.

[227]	RP-252964: "Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices".

[228]	3GPP TS 38.291: "Ambient IoT Physical layer".

[229]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

For the purposes of the study, this TR uses the following terms:

Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device's D2R transmission is backscattered on a carrier wave provided externally.

Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both R2D and/or D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

D1T1: Deployment scenario 1 with connectivity topology 1, according to TR 38.848.

D2T2: Deployment scenario 2 with connectivity topology 2, according to TR 38.848.

Inventory: The service provided by the network to discover and acquire the identifier of A-IoT device(s).

Command: The service provided by the network to send the operation instruction to the A-IoT device (e.g. read, write, etc.).

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1SB	Single sideband

2SB	Double sideband

A-IoT	Ambient IoT

A-IoT RAN	Ambient IoT Radio Access Network

BFSK	Binary frequency-shift keying

BPSK	Binary phase-shift keying

CAP	Clock-acquisition part (of R-TAS)

CFO	Carrier-frequency offset

CP	Cyclic prefix

CW	Carrier-wave

CW2D	Carrier-wave, or carrier-wave node, to device

D2R	Device to reader

DFT-s-OFDM	DFT-spread OFDM

DO-A	Device-originated autonomous

DO-DTT	Device-originated by device-terminated trigger

DT	Device-terminated

ED	Envelope detector

FAR	False alarm rate

FDR	False detection rate

FEC	Forward error-correction code

FR	Frequency Range

IF	Intermediate frequency

IoT	Internet of Things

LPWA	Low-power, wide-area

LTE-MTC	Long Term Evolution – Machine Type Communication

MCS	Modulation and coding scheme

MDR	Missed detection rate

MSK	Minimum-shift keying

NB-IoT	Narrowband IoT

OOK	On-off keying

PDRCH	Physical device-to-reader channel

PIE	Pulse interval encoding

PRDCH	Physical reader-to-device channel

R2D	Reader to device

RF	Radio frequency

RFID	Radio frequency identification

R-TAS	R2D timing acquisition signal

SER	Sample error rate

SFO	Sampling-frequency offset

SIP	Start indicator part (of R-TAS)

ZIF	Zero IF

# 4	Evaluation methodology

## 4.1	Remaining details of RAN design targets

TR 38.848 [2] sets a number of RAN design targets. In [3], in particular three aspects of design targets beyond those in TR 38.848 are to be studied:

-	Applicable maximum distance target value(s): The maximum distance targets are set separately for device 1, device 2a, device 2b, respectively. The distance target is compared to the evaluation results which assume the agreed mandatory evaluation assumptions.

For device 1 in D1T1: 15 m

For device 2a in D1T1: 25 m

For device 2b in D1T1: 50 m

For device 1 in D2T2: 10 m

For device 2a in D2T2: 15 m

For device 2b in D2T2: 40 m

-	Refined definition of latency:

-	For the use case of "inventory-only":

-	The time interval between the time that the A-IoT paging is sent from reader to a A-IoT device and the time that the inventory report is successfully received at reader from the A-IoT device, i.e., for completing Step A and Step B (as per RAN2 agreements).

-	For the use case of "inventory and command":

-	The time interval between the time that the A-IoT paging is sent from reader and the time that

-	The command is successfully received at A-IoT device, i.e., for completing Step A, Step B, Step C1 (as per RAN2 agreements), if Step C2 is optional and not used (note: pending RAN2 decision on optionality of Step C2).

-	The response is successfully received at Reader, i.e., for completing Step A, Step B, Step C1, and Step C2 (as per RAN2 agreements).

-	See Clause 6.3.1 for descriptions of the Steps of these procedures.

Expected value of latency is calculated according a X% re-attempt probability to each attempt.

Note: The "successfully received" (i.e., decoding successfully)

Note: The latency is evaluated for a single A-IoT device.

Note: Time for energy harvesting is not included in the definition of latency.

-	2D distribution of devices: A-IoT devices are dropped uniformly distributed over the horizontal area. See Table 4.2.2-2.

## 4.2	Evaluation scenarios and assumptions

### 4.2.1	Evaluation scenarios

The following scenarios are defined for the purpose of potential evaluation.

Table 4.2.1-1: Evaluation scenarios

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b/C | CW spectrum | D2R spectrum | R2D spectrum |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | CW inside topology |  | CW node inside topology 1 ‘CW’ in CW2D and ‘R2’ in D2R are different ‘CW’ in CW2D and ‘R1’ in R2D are same ‘R1’ in R2D and ‘R2’ in D2R are different | Device 1, 2a | Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL) | Same as CW | At least DL |
| D1T1-A2 |  |  | CW node inside topology 1 Same ‘CW’ and ‘R’ node for CW2D, D2R and R2D |  | Same as D1T1-A1 | Same as CW | At least DL |
| D1T1-B | CW outside topology |  | CW node outside topology 1 ‘CW’ in CW2D and ‘R’ in D2R are different ‘CW’ in CW2D and ‘R’ in R2D are different ‘R’ in R2D and ‘R’ in D2R are same |  | Case 1-4 (outside topology, UL) | Same as CW | At least DL |
| D1T1-C D4T1-C | No CW |  | No CW Node. | Device 2b Device C | N/A | UL | At least DL |
| D2T2-A1 | CW inside topology |  | CW node inside topology 2 ‘CW’ in CW2D and ‘R2’ in D2R are different ‘CW’ in CW2D and ‘R1’ in R2D are same ‘R1’ in R2D and ‘R2’ in D2R are different BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW | UL |
| D2T2-A2 |  |  | CW node inside topology 2 Same ‘CW’ and ‘R’ node for CW2D, D2R and R2D BS communicates with R |  | Same as D2T2-A1 | Same as CW | UL |
| D2T2-B | CW outside topology |  | CW node outside topology 2 ‘CW’ in CW2D and ‘R’ in D2R are different ‘CW’ in CW2D and ‘R’ in R2D are different ‘R’ in R2D and ‘R’ in D2R are same BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW | UL |
| D2T2-C | No CW |  | No CW Node. BS communicates with R | Device 2b | N/A | At least UL | UL |
| Note: This table is for the case where D2R is in the same spectrum as CW2D. |  |  |  |  |  |  |  |

### 4.2.2	Evaluation assumptions

The following tables of coverage evaluation assumptions for link-level simulation are considered for indoor scenarios and outdoor scenarios, respectively. (M) indicates a value mandatory for evaluation, (O) indicates optional for evaluation. If there are any differences between devices, they are for evaluation purposes only.

Table 4.2.2-1: Coverage evaluation assumptions for link-level simulation for indoor scenarios

| No. | Parameters |  | Assumptions |
| --- | --- | --- | --- |
| R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |
| [0b] | SCS |  | 15 kHz as baseline |
| [0c] | Block structure |  | Blocks as agreed in 9.4.2.3, or other blocks reported by companies |
| [0d] | Channel model |  | R2D: For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS, with TDL-A [0D]-Alt2: InH-Office LOS, with TDL-D For D1T1: InF-DH NLOS, with TDL-A D2R: For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS, with TDL-A [0D]-Alt2: InH-Office LOS, with TDL-D For D1T1: InF-DH NLOS, with TDL-A |
| [0e] | Delay spread |  | An RMS delay spread of 30 ns (M) and [150] ns (O) is considered for TDL-A channel model. An RMS delay spread of 30 ns is considered for TDL-D channel model. |
| [0f] | Device velocity |  | 3 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 or 4 |
| [0h2] |  | Number of TXRUs | 2 or 4 |
| [0j1] | Intermediate UE | Number of antenna elements | 1 or 2 |
| [0j2] |  | Number of TXRUs | 1 or 2 |
| [0m] | Reference data rate |  | 1 kbps (M) 5 - 7 kbps (M) 48 - 60 kbps (O) 0.1 kbps for message size of 20 bits or 96 bits (O) Other data rates can be reported by companies Note 1: Companies to report the exact data rate. Note 2: The exact data rate is close to the values listed above. Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads(e.g., CRC, pre/mid/post-ambles if present). Note 4: The exact data rate may be related to coding scheme, repetition and etc. Note 5: All data rates considered are for evaluation purpose only |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size |
| [0p] | BLER target |  | 1%, 10% |
| [0q] | Sampling frequency |  | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. CFO for device 2b: 100 ppm (M) 200 ppm (O) 1000 ppm (O, only as initial CFO) Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only |
| [0r] | Device 1/2a/2b |  | Options are as follows, Device 1, RF-ED Device 2a, RF-ED Device 2b, RF-ED/IF-ED/ZIF |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz as baseline. Other larger values are not precluded. |
| [1b] | ED bandwidth |  | The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth. |
| [1c] | BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth. Companies to report X = {3, 5}. |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK Companies to report, e.g., OOK-1, OOK-4 with M chips per OFDM symbol |
| [1f] | Line code |  | Companies to report, e.g., Manchester, PIE |
| [1g] | FEC |  | No FEC as baseline |
| [1h] | ADC bit width |  | 1-bit for device 1 4-bit for device 2 |
| [1j] | Detection/decoding method for Line code |  | Companies to report |
| D2R specific parameters |  |  |  |
| [2a1] | Transmission bandwidth |  | [2a1]-Alt1 (M): DSB X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz [2a1]-Alt2: SSB X kHz is considered for D2R transmission bandwidth. The value is for one sideband, i.e., the total transmission bandwidth for SSB is X kHz. Not applicable for at least device 1 X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies |
| [2a2] | [OOK/BPSK/BFSK chip rate] |  | Companies to report |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value, and further down-selection of the values and DSB/SSB is not precluded. |
| [2b] | Waveform (CW) |  | Companies to report waveform, e.g., unmodulated single tone, multi-tone(multiple unmodulated single tone) |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK, BFSK |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, FM0 encoding, Miller encoding, no line coding |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit |
| [2j] | D2R receiver |  | Companies to report, e.g., coherent receiver / non-coherent receiver |
|  | Other assumptions |  |  |
| [3a] | Other assumptions |  | To be reported by company |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |

Table 4.2.2-1a: Coverage evaluation assumptions for link-level simulation for outdoor scenarios

| No. | Parameters |  | Assumptions |
| --- | --- | --- | --- |
| R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |
| [0b] | SCS |  | 15 kHz as baseline |
| [0c] | Block structure |  | Blocks as agreed in Rel-19 with update reported by companies |
| [0d] | Channel model |  | TDL-C |
| [0e] | Delay spread |  | An RMS delay spread of 300 ns |
| [0f] | Device velocity |  | 3 km/h Companies can also report 10 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 or 4 |
| [0h2] |  | Number of TXRUs | 2 or 4 |
| [0m] | Reference data rate |  | 1 kbps (M) 5 - 7 kbps (M) 48 - 60 kbps (O) 0.1 kbps for message size of 20 bits or 96 bits (O) Other data rates can be reported by companies Note 1: Companies to report the exact data rate. Note 2: The exact data rate is close to the values listed above. Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads (e.g., CRC, pre/mid/post-ambles if present). Note 4: The exact data rate may be related to coding scheme, repetition and etc. Note 5: All data rates considered are for evaluation purpose only |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size |
| [0p] | BLER target |  | 1%, 10% |
| [0q] | Sampling frequency |  | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm SFO after clock sync/calibration: For Device 2b: 10^3 (M) ppm, 10^2 (O) ppm For Device C: 10 ppm Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. Initial CFO: For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm CFO after clock sync/calibration: For Device 2b: 100 (M) ppm, 200 (O) ppm, 50 (O) ppm For Device C: 10 ppm Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only |
| [0r] | Device 2b/C |  | ZIF/IF-ED |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180kHz Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report |
| [1b] | ED bandwidth |  | Up to company report with considering: R2D transmission bandwidth Device Rx LO uncertainty (e.g. CFO ppm assumption) |
| [1c] | BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at half of ED bandwidth. Companies to report X = {3, 5}. |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-4 Companies to report M chips per OFDM symbol |
| [1f] | Line code |  | Manchester |
| [1g] | FEC |  | Companies who report evaluations using FEC should also report evaluations without FEC |
| [1h] | ADC bit width |  | 4-bit |
| [1j] | Detection/decoding method for Line code |  | Companies to report |
| [1k] | Repetition |  | Companies who report R2D evaluations using repetition should also report R2D evaluations without repetition |
| D2R specific parameters |  |  |  |
| [2a1] | Transmission bandwidth |  | 15kHz Other values are not precluded and up to company report Companies to report which of 2SB or 1SB modulation is assumed for D2R transmission bandwidth |
| [2a2] | Chip rate |  | Companies to report |
| [2a3] | Receiver bandwidth |  | D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. Assume the receiver matches the transmitter's modulation, i.e., to receiver uses 1SB when transmitter uses 1SB, receiver uses 2SB when transmitter uses 2SB. Companies to report the value, and further down-selection of the values and 2SB/1SB is not precluded. |
| [2d] | Modulation |  | Companies to report modulation, e.g., OOK, BPSK |
| [2e] | Line code |  | Companies to report, e.g., Manchester encoding, no line coding |
| [2g] | FEC |  | Companies to report, e.g., CC, No FEC |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit |
| [2j] | D2R receiver |  | Companies to report, e.g., coherent receiver / non-coherent receiver |
| [2k] | Repetition |  | Companies to report based on update of Rel-19 if any |
| Other assumptions |  |  |  |
| [3a] | Other assumptions |  | To be reported by company |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |

The following layouts are used for evaluation purposes for indoor scenarios.

Table 4.2.2-2: Assumptions on layout for D1T1 and D2T2

| Parameter | Assumptions for D1T1 | Assumptions for D2T2 |  |
| --- | --- | --- | --- |
| Scenario | InF-DH | InH-office | InF-DL |
| Hall size | 120x60 m | 120 x50 m | 300x150 m |
| Room height | 10 m | 3m | 10 m |
| Sectorization | None |  |  |
| BS deployment / Intermediate UE dropping | 18 BSs on a square lattice with spacing D, located D/2 from the walls. L=120m x W=60m; D=20m BS height = 8 m | L=120m x W=50m; Intermediate UE height = 1.5 m Option 1: Intermediate UEs are assumed to be deployed in a grid with D = e.g. 10 m / 20 m. Option 2: Intermediate UEs are assumed to be uniformly distributed over the horizontal area. Number of UE is either 1 or 2. | L=300m x W=150m; Intermediate UE height = 1.5 m Option 1: The intermediate UEs are assumed to be deployed in a grid with D = e.g. 10 m / 20 m. Option 2: Intermediate UEs are assumed to be uniformly distributed over the horizontal area. Number of UE is either 1 or 2. |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Option 1: The devices within the maximum distance, which is obtained by the coverage evaluation, from each intermediate UE are involved in the evaluations. Option 2: The devices within the applicable maximum distance target are involved in the evaluations. | Device Height= 1.5m A-IoT devices drop uniformly distributed over the horizontal area Option 1: The devices within the maximum distance, which is obtained by the coverage evaluation, from each intermediate UE are involved in the evaluations. Option 2: The devices within the applicable maximum distance target are involved in the evaluations. |
| Device mobility (horizontal plane only) | 3 kph | 3 kph | 3 kph |
| NOTE: The CW distribution in D1T1-B or D2T2-B is reported by companies |  |  |  |

For evaluation purposes for outdoor scenarios, UMa and UMi both use a hexagonal grid with 19 site and 3 sectors per site.

## 4.3	Link budget

### 4.3.1	Receiver sensitivity

The study uses the following definitions for receiver sensitivity.

Budget-Alt1:	Receiver sensitivity is derived by a predefined threshold and no link-level simulation is needed for link budget calculation

The results rely on the received sensitivity and maximum transmit power, and directly calculate the maximum distance / pathloss based on these values and other related parameters. The link-level simulation performances, such as required SINR, can be satisfied for such case and no link-level simulation is needed for link budget calculation.

Budget-Alt1 receiver sensitivity values are summarized in Annex A.

Budget-Alt2:	Receiver sensitivity is derived by required SINR which is given by LLS results

The results rely on link-level simulation results, e.g., required SINR which corresponds to detail LLS assumptions (e.g., BW, coding, data rate). And based on the required SINR, the received sensitivity can be calculated and then the maximum distance / pathloss can be derived.

Note: For noise power, a noise figure value needs to be provided.

### 4.3.2	Link budget templates

Link budget is calculated according to the following Table 4.3.2-1 for indoor scenarios or Table 4.3.2-2 for outdoor scenarios. (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated.

Table 4.3.2-1: Link budget template for indoor scenarios

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | D1T1-A1/A2/B/C D2T2-A1/A2/B/C | D1T1-A1/A2/B/C D2T2-A1/A2/B/C |
| [0A1] | CW case | N/A | 1-1/1-2/1-4/2-2/2-3/2-4 |
| [0B] | Device 1/2a/2b | Device 1/2a/2b | Device 1/2a/2b |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Topology/Pathloss model | For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: InF-DH NLOS | For D2T2: BS pathloss model is reused for intermediate UE with antenna height = 1.5 m [0D]-Alt1: InF-DL NLOS [0D]-Alt2: InH-Office LOS For D1T1: InF-DH NLOS |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | For BS: 2(M) or 4(O) antenna elements for 0.9 GHz For Intermediate UE: 1(M) or 2(O) | 1 |
| [1E] | Total Tx Power (dBm) | For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), - [1E]-R2D-Alt2: 38dBm(O), - [1E]-R2D-Alt3: 24dBm(M) - Companies to report if PSD constraints are imposed (company to report the condition for applying PSD constraints in Row [5A]) For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) - [1E]-R2D-Alt5:26dBm(O) | For device 1/2a: (see note 1) - [1E]-D2R-Alt1: For scenarios ‘B’, the device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value and other related factors. - [1E]-D2R-Alt2: For scenarios ‘A1’ and ‘A2’, the device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. For device 2b: For scenarios ‘C’ - [1E]-D2R-Alt3: -20 dBm(M) - [1E]-D2R-Alt4: -10 dBm(O) |
| [1E1] | CW Tx power (dBm) | N/A | For scenario ‘A1’, ‘A2’ and ‘B’ - Report a value from the candidate values [1E]-R2D-Alt1 / [1E]-R2D-Alt2 / [1E]-R2D-Alt3 from [1E]-R2D if CW in DL spectrum - Report a value from the candidate values [1E]-R2D-Alt4 / [1E]-R2D-Alt5 from [1E]-R2D if CW in UL spectrum. Note: only applicable for device 1/2a |
| [1E2] | CW Tx antenna gain (dBi) | N/A | Company to report, the value equals: - UE Tx ant gain, or - BS Tx ant gain Note: Only applicable for device 1/2a |
| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (See note 1) D2T2-B: - 5m, - 10m, For scenarios ‘A1’ and ‘A2’: Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note 1: Only applicable for device 1/2a. Note 2: Companies to report which value(s) are evaluated. |
| [1E4] | CW2D pathloss (dB) | N/A | Calculated (see note1) Note 1: Only applicable for device 1/2a Note 2: For CW2D pathloss model, use the same pathloss model as used for R2D/D2R. |
| [1E5] | CW received power (dBm) | N/A | Calculated (see note1) Note: Only applicable for device 1/2a |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz(M), 360kHz(O), 1.08MHz(O) | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | For BS for indoor, 6 dBi(M), 2dBi(M) For intermediate UE, 0 dBi | For A-IoT device, 0dBi |
| [1H] | Ambient IoT backscatter loss (dB) due to Modulation factor | N/A | OOK: 6 dB PSK: 0 dB FSK: Y dB It is applicable for device 1 and 2a. Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss. |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0.9dB or 4.7dB |
| [1K] | Ambient IoT backscatter amplifier gain (dB) | N/A | 10 dB (M) 15 dB (O) Note: Only for device 2a |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A For intermediate UE, 1 dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver 20dB, Device 2 For IF/ZIF receiver 15dB, Device 2 Other values can be optionally used and reported by companies | For BS as reader: 5dB For intermediate UE as reader: 7dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2F] | Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt1/ Budget-Alt2 (see note1) | Budget-Alt2 |
| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. Notes: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The value for other cases (e.g. multi-tone CW) can be reported by companies. |
| [2K1] | Remaining CW interference (dBm) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2K2] | Receiver sensitivity loss(dB) | N/A | Calculated (see Note 1) Note: only applicable for device 1/2a |
| [2L] | Receiver Sensitivity (dBm) | For Budget-Alt1 For device 1 (RF-ED), for example: {-30 dBm, -36 dBm, -40 dBm, etc} For device 2 (RF-ED), for example: {-40 dBm, -45 dBm, etc} For Budget-Alt2 Calculated (see note1) | Calculated (see Note 1) Note 2L: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS | For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB Note: only applicable for D1T1 | 0 dB Note: only applicable for D1T1 |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

Note 1: Calculated values are derived according to the following.

[1E3]

-	For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss, [1E3] is derived by assuming pathloss is [1E4] and use the pathloss formula as agreed.

[1E4]

-	For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss

-	[1E4] = 0.5* ( [1E1] + [1E2] - [1N](R2D) + [2C] (R2D) - [2H](R2D) - 2*[3A] - 2*[3B] +[3D](R2D) + [1K] - [1H] + [1G] - [1J] + [2C] - [2X] - [2L] + [3C] + [3D] )

-	[1K] is only for device 2a

-	Otherwise

-	[1E4] is derived according to pathloss formula by assume distance is [1E3]

[1E5]

-	[1E5] = [1E1] + [1E2] - [1N](R2D) - [1E4] + [2C] (R2D) - [2H](R2D) - [3A] - [3B] + [3D](R2D)

[1E]

-	[1E] = [1E5]+ [1K] - [1H]

-	[1K] is only for device 2a

[1M]:

-	For R2D,

-	[1M] = [1E] + [1G] - [1N]

-	For D2R

-	[1M] = [1E] + [1G] - [1J]

[2F]:

-	[2F] = [2D] + [2E] +lin2dB([2B])

[2G]

-	For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth.

-	For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver.

-	For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth. Receiver bandwidth is the bandwidth used at the reader side to filter the D2R signals for calculating noise and interference (if any) power.

-	On/off keying backscatter loss (including DC removal loss) is not taken into account in the LLS and is included in link budget table [1H].

[2J]

-	For R2D link in the coverage evaluation, for device 1

-	Budget-Alt1 is used (note: receiver architecture is RF ED)

-	For R2D link in the coverage evaluation for device 2,

-	Budget-Alt1 is used if receiver architecture is RF ED

-	Budget-Alt2 is used if receiver architecture is IF/ZIF ED

Note A: this does not preclude to have LLS for device 1 and 2 R2D link with RF-ED if needed.

Note B: For device 2 R2D link with RF-ED, Budget-Alt1 is mandatory, Budget-Alt2 is optional.

Note C: this does not imply all M values are achievable with the sensitivity given by Budget-Alt1 for RF ED

Note D: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure in [2D] from the table, exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

[2K1]:

-	[2K1] = [1E1] + [1E2] -[1N](R2D) + [2C] - [2X] - [2K]

[2K2]:

-	2K2=lin2dB1+dB2lin([2K1])dB2lin([2F])

[2L]:

-	For R2D and Budget-Alt2,

-	[2L] = [2G] - lin2dB([2B] / [1F]) + [2F]

Note E: The term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR.

-	For D2R,

-	[2L] = [2G] + [2F] + [2K2], device 1/2a

-	[2L] = [2G] + [2F], device 2b

[4A]

-	For R2D

-	 [4A] = [1M] + [2C] - [2H] - [2L] - [3A] - [3B] + [3C] + [3D]

-	For D2R

-	 [4A] = [1M] + [2C] - [2X] - [2L] - [3A] - [3B] + [3C] + [3D]

[4B]

-	 [4B] is derived by assuming pathloss is [4A] and using the pathloss formula as agreed.

Table 4.3.2-2: Link budget template for outdoor scenarios

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | Deployment scenario 4 with topology 1 with no external carrier wave | Deployment scenario 4 with topology 1 with no external carrier wave |
| [0B] | Device 2b/C | Device 2b/C | Device 2b/C |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Pathloss model | UMa NLOS defined in TR38.901 BS antenna height: 25 m Device antenna height: 1.5 m RMa NLOS defined in TR38.901 BS antenna height: 35 m Device antenna height: 1.5 m Avg. street width: 20 m Avg. building height: 5 m Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901 BS antenna height: 10 m Device antenna height: 1.5 m | UMa NLOS defined in TR38.901 BS antenna height: 25 m Device antenna height: 1.5 m RMa NLOS defined in TR38.901 BS antenna height: 35 m Device antenna height: 1.5 m Avg. street width: 20 m Avg. building height: 5 m Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901 BS antenna height: 10 m Device antenna height: 1.5 m |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | Refer to LLS table [0h1] and [0h2] | 1 |
| [1E] | Total Tx Power (dBm) | For standalone: 43 dBm For in-band: 38 or 33 dBm over R2D transmission BW Note: companies should report any PSD limitation they assume in their RAN1 evaluations | Device 2b: {-10, -20} dBm Device C: {-3, 0, 5} dBm |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1a] | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | 17dBi | For A-IoT device, 0dBi |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0dB or 0.9dB or 4.7dB |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | 3dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | Device 2b: 15 dB Device C: 13 dB | For BS as reader: 5dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2E1] | Receiver interference density (dBm/Hz) | -169.3 dBm/Hz Companies can also report other higher value(s) for R2D or -999 dBm/Hz along with their assumption | -165.7 dBm/Hz Companies can also report -999 dBm/Hz along with their assumption |
| [2F] | Effective Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0dB or 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt2 | Budget-Alt2 |
| [2L] | Receiver Sensitivity (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB | For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| [3E] | Penetration margin (dB) | {0, 20, 80} dB | {0, 20, 80} dB |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

Note 1: Calculated values are derived according to the following.

[1M]:

-	For R2D,

-	[1M] = [1E] + [1G] - [1N]

-	For D2R

-	[1M] = [1E] + [1G] - [1J]

[2F]:

-	[2F] = lin2dB(dB2lin([2D] + [2E]) + dB2lin([2E1])) +lin2dB([2B])

[2G]

-	For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth.

-	For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver.

-	For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth. Receiver bandwidth is the bandwidth used at the reader side to filter the D2R signals for calculating noise and interference (if any) power.

[2L]:

-	For R2D,

-	[2L] = [2G] - lin2dB([2B] / [1F]) + [2F]

Note A: The term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR.

-	For D2R,

-	[2L] = [2G] + [2F]

[4A]

-	For R2D

-	 [4A] = [1M] + [2C] - [2H] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E]

-	For D2R

-	 [4A] = [1M] + [2C] - [2X] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E]

[4B]

-	 [4B] is derived by assuming pathloss is [4A] and using the pathloss formula as agreed.

## 4.4	R2D waveform generation

With reference to the R2D waveform described in Clause 6.1.1.1, for evaluation purposes the waveform for DFT-s-OFDM is generated as follows:

1.	The time domain OOK signal is the M chips of one OFDM symbol.

2.	A chip is represented (e.g. upsampled) by L samples

-	Companies to report L

3.	An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal.

-	Companies to report N’, e.g. N’=128 or equal to X

4.	Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D.

-	Companies report how to map and report X

5.	An N-points IDFT is performed to obtain the time domain signal.

-	Companies to report N, and how value was selected

Note: Companies report whether/how CP samples are added.

## 4.5	Energy harvesting/storage assumptions

The following energy harvesting/storage assumptions on the energy source and energy storage capacity used to obtain the reported data rates were provided during the study.

Table 4.4-x: Energy harvesting/storage assumptions from Source [x]

| Source | Energy harvesting/storage assumptions |
| --- | --- |
| Source [x] | Energy source: Energy storage capacity: Reported data rate: Other information (if any): |

# 5	Ambient IoT device architectures

## 5.1	~1 µW devices (Device 1)

The architecture of such a device is summarised in Figure 5.1-1, with the blocks described as follows.

-	Antenna could be either shared or separate for RF energy harvester and receiver/transmitter.

-	Matching network is to match impedance between antenna and other components (including RF energy harvester and receiver related blocks).

-	RF energy harvester can include rectifier performing RF signal (AC) to DC conversion.

-	Energy storage (e.g., capacitor) stores harvested energy from RF energy harvester.

-	Power management unit (PMU) manages storing energy to energy storage from energy harvester and supplying power to active component blocks which needs power supply.

-	Digital BB logic includes functional blocks like encoder, decoder, controller, etc.

-	Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

-	Clock generator provides required clock signal(s).

-	Reception related blocks

-	RF BPF for improving selectivity.

-	Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered.

-	RF Envelope Detector converts RF signal to baseband.

-	BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator.

-	Depending on implementation, it may not exist. Presence of BB LPF is assumed for the study.

-	Comparator determines high/low of input signal.

-	Transmission related blocks

-	Backscatter modulator switches impedance to modulate backscattered signal with transmitted signal from BB logic.

Figure 5.1-1: Architecture of device 1

## 5.2	≤ a few hundred µW devices (Device 2)

### 5.2.1	External carrier wave (Device 2a)

The architecture of device 2a is summarised in Figure 5.2.1-1, with the blocks described as follows.

-	Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter.

-	Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks).

-	Energy harvester.

-	Energy storage (e.g., capacitor) stores harvested energy from energy harvester.

-	Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply.

-	Digital BB logic includes functional blocks like encoder, decoder, controller, etc.

-	Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

-	Clock generator provides required clock signal(s).

-	Reflection amplifier can amplify reflected backscattered signal.

-	At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA.

-	Reception related blocks

-	RF BPF filter for improving selectivity.

-	Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered.

-	LNA for improving signal strength and sensitivity of receiver, if present.

-	At least one of R2D/CW2D and D2R could be amplified by either reflection amplifier or LNA.

-	RF envelope detector (RF-ED) detects envelope from RF signal.

-	BB amplifier amplifies BB signal to improve signal strength.

-	BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC.

-	Depending on implementation, it may not exist.

-	Comparator or N-bit ADC

-	Transmission related blocks

-	Backscatter modulator switches impedance to modulate backscattered signal with tx signal from BB logics.

-	Large Frequency shifter (e.g., tens of MHz) for shifting backscattered signal from one frequency (e.g., FDD-DL frequency) to another frequency (e.g., FDD-UL frequency).

Figure 5.2.1-1: Architecture of device 2a

IF envelope detector receiver and ZIF receiver for device 2a are de-prioritized.

#### 5.2.1.1	Reflection amplifier

For the reflection amplifier block, the following characteristics are considered for device 2a:

-	Direction of amplification

-	Uni-directional reflection amplifier (baseline) can amplify backscattered signal in D2R which can improve D2R link budget.

-	Bi-directional amplifier can amplify both signal in R2D and backscatter signal in D2R at least when R2D and D2R are in the same spectrum.

-	Bi-directional amplifier has higher complexity, higher noise figure, and reduced isolation between tx and rx path.

-	Amplification gain ranges from 10 to 20 dB.

-	Power consumption of reflection amplifier is in the range of a tens of uW to 100s of uW.

-	Reflection amplifier can operate in FDD frequency bands.

-	Reflection amplifier bandwidth can support 10s of MHz.

-	Note: reflection amplifier can get unstable when the input power exceeds a certain value, which may be frequency-dependent.

#### 5.2.1.2	Large frequency shifter

For the large frequency shifter block, it is observed that:

-	Large frequency shift can be used in shifting reflected signal in tens of MHz, e.g., from FDD DL to FDD UL frequency or vice versa.

-	Large frequency shift consumes 10s of uW to 100s of uW.

-	Large frequency shift is not feasible for device 1.

-	Large frequency shift requires a clock for IF generation which is accurate enough to avoid large guard band and interference to adjacent channels/bands.

-	Large frequency shift requires image suppression and may require harmonics suppression

-	Note: details of image suppression and harmonics suppression are not discussed in RAN1

-	Large frequency shift may allow the reader to avoid implementing in-band full duplex capability for scenarios e.g., D1T1-A2 and D2T2-A2.

-	Large frequency shift may result in e.g., 5 kHz - 50 kHz of frequency uncertainty in target frequency for clock accuracy of e.g., 0.01% - 0.1% assuming the large frequency shift range is 50 MHz.

Large frequency shift may be technically feasible in terms of power consumption, and device complexity.

Remaining technical feasibility of large frequency shift in terms of image suppression, harmonics suppression may be relevant to device emission requirement, spectrum usage. Additional complexity and power consumption may be needed to achieve image suppression and harmonics suppression.

The necessity or effectiveness of large frequency shift may depend on spectrum for R2D/CW2D/D2R, full duplex capability of reader, whether device 1 and 2a are supported by a single reader, spectrum regulation/usage, frequency error, coverage, etc.

### 5.2.2	Internally-generated carrier wave (Device 2b)

#### 5.2.2.1	RF envelope detector receiver

The architecture of device 2b with an RF envelope detector receiver is summarised in Figure 5.2.2.1-1, with the blocks described as follows.

-	Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter.

-	Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks).

-	Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc

-	Energy storage (e.g., capacitor) stores harvested energy from energy harvester.

-	Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply.

-	Digital BB logic includes functional blocks like encoder, decoder, controller, etc.

-	Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

-	Clock generator provides required clock signal(s).

-	Reception related blocks

-	RF BPF filter for improving selectivity.

-	Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered.

-	LNA for improving signal strength and sensitivity of receiver, if present

-	RF envelope detector (RF-ED) detects envelope from RF signal.

-	BB amplifier amplifies BB signal to improve signal strength.

-	BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC.

-	Depending on implementation, it may not exist.

-	Comparator or N-bit ADC

-	Transmission related blocks

-	Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic.

-	Digital to Analog Converter (DAC) converts digital signal to analog signal.

-	Low pass filter for filtering out undesired signal

-	Mixer performs up converting baseband signal to RF range.

-	Local oscillator (LO) for carrier frequency generation

-	FLL(/PLL) can be used for frequency synthesis

-	Depending on implementation, FLL(/PLL) may not exist.

-	Power amplifier (PA) amplifies tx signal, if present

-	Details on transmitter related blocks depends on tx waveform/modulation.

Figure 5.2.2.1-1: Architecture of device 2b with RF-ED receiver

#### 5.2.2.2	IF envelope detector receiver

The architecture of device 2b with an IF envelope detector receiver is summarised in Figure 5.2.2.2-1, with the blocks described as follows.

-	Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter

-	Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks)

-	Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc.

-	Energy storage (e.g., capacitor) stores harvested energy from energy harvester

-	Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply

-	Digital BB logic includes functional blocks like encoder, decoder, controller, etc.

-	Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only -	while energy is available in energy storage

Clock generator provides required clock signal(s).

-	Local oscillator (LO) for generating carrier frequency for Tx, or for generating carrier frequency offset by the IF for Rx

-	FLL(/PLL) can be used for frequency synthesis

-	One LO or separate LOs for Tx and Rx

-	Reception related blocks

-	RF BPF filter for improving selectivity

-	Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered

-	LNA for improving signal strength and sensitivity of receiver, if present

-	Mixer down converts RF signal to IF stage

-	Depending on implementation, there could be one or two mixers for Rx and Tx

-	IF amplifier amplifies IF signal

-	IF filter for filtering out unwanted RF and LO signals

-	IF envelope detector (IF-ED) detects envelope from IF signal.

-	BB amplifier

-	Depending on implementation, one or both of IF amplifier and BB amplifier may exist

-	BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator/ADC

-	Depending on implementation, it may not exist

-	Comparator or N-bit ADC

-	Note: image rejection is required

-	Transmission related blocks

-	Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic

-	Digital to Analog Converter (DAC) converts digital signal to analog signal

-	Low pass filter for filtering out undesired signal

-	Mixer performs up converting baseband signal to RF range

-	Power amplifier (PA) amplifies transmitted signal, if present

-	Details of transmitter related blocks depends on e.g., waveform/modulation, etc.

Figure 5.2.2.1-1: Architecture of device 2b with IF-ED receiver

#### 5.2.2.3	ZIF receiver

The architecture of device 2b with a ZIF receiver is summarised in Figure 5.2.2.3-1, with the blocks described as follows.

-	Antenna could be either shared or separate for RF energy harvester (if present) and receiver/transmitter

-	Matching network is to match impedance between antenna and other components (including RF energy harvester (if present) and receiver related blocks)

-	Energy harvester for harvesting energy from e.g., RF signal, solar, vibration/movement, temperature difference, etc

-	Energy storage (e.g., capacitor) stores harvested energy from energy harvester.

-	Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply

-	Digital BB logic includes functional blocks like encoder, detector, decoder, controller, etc.

-	Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage

-	Clock generator provides required clock signal(s).

-	Local oscillator (LO) for generating carrier frequency for Tx and Rx

-	FLL(/PLL) can be used for frequency synthesis

-	One LO or separate LOs for Tx and Rx

-	Reception related blocks

-	RF BPF filter for improving selectivity

-	Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered

-	LNA for improving signal strength and sensitivity of receiver, if present

-	Mixer down converts RF signal to BB stage

-	Depending on implementation, there could be one or two mixers for Rx and Tx

-	BB amplifier amplifies BB signal

-	BB LPF can filter out undesired frequency components to improve input signal quality to comparator/ADC.

-	Depending on implementation, it may not exist

-	Comparator or N-bit ADC

-	Transmission related blocks

-	Tx Modulator: baseband bits are modulated according to modulation scheme. This block could be the part of BB logic.

-	Digital to Analog Converter (DAC) converts digital signal to analog signal.

-	Low pass filter for filtering out undesired signal

-	Mixer performs up converting baseband signal to RF range.

-	Power amplifier (PA) amplifies transmitted signal, if present

-	Details of transmitter related blocks depend on e.g., waveform/modulation, etc.

Figure 5.2.2.1-1: Architecture of device 2b with ZIF receiver

## 5.2A	≤ 1 mW to ≤ 10 mW devices (Device C)

Device C is assumed to have similar device architectures as Device 2b.

## 5.3	Clock(s)

For A-IoT device clock, 9 sources [7], [9], [8], [16], [10], [12], [11], [15], [17] have reported that clock calibration based on at least clock counting for a known duration is feasible for A-IoT devices.

Table 5.3-1: Descriptions of clocks/LOs

| Clock purpose # | Description | Clock speed | Applicable devices | Power consumption | Initial clock Accuracy [ppm] | Accuracy after clock sync / calibration at device side [ppm] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Sampling | A few MHz | 1 (Note 3) | <1 µW |  | 105 (Note 2) |
|  |  |  |  |  |  | 104 (Note 2) the study did not determine whether or not this accuracy can be achieved with < 1 µW power consumption at least for device 1 |
|  |  |  | 2a, 2b | <10 µW | 103 - 104 | 103 (Note 2) |
| 2 | Small frequency shift | A few MHz | 1 | <1 µW |  | 105 (Note 2) |
|  |  |  |  |  |  | 104 (Note 2) the study did not determine whether or not this accuracy can be achieved with < 1 µW power consumption at least for device 1 |
|  |  |  | 2a, 2b (if applicable) | <10 µW |  | 103 (Note 2) |
| 3 | Time counting (if supported) | e.g. tens of kHz | 1 (if applicable) 2a, 2b | <0.1 μW | 104 - 105 | Calibration may not always be feasible for this clock purpose |
| 4 | Large frequency shift (if supported for device 2a) | 10s of MHz (e.g. 50 MHz) | 2a | 10s of µW | 103 - 104 | 103 (Note 2) Whether better accuracy can be achieved has not been determined. |
| 5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s - 100s of µW | 103 - 104 | Within 10s – 200 (Note 2), there is no need to determine a single value for the study |
| Note 1: This table does not necessarily imply that different purposes of LOs/clocks correspond to separate discrete LOs/clocks, which is up to implementation. Note 2: No drastic temperature change is assumed during calibration. Note 3: Device 2a, 2b could also use similar implementation. |  |  |  |  |  |  |

Several sources [10], [15], [13], [14], [16], [9] think that for clock purpose #1 (sampling), accuracy after clock sync / calibration at device side of 104 ppm is achievable with < 1 μW power consumption for device 1, and that 105 ppm may not be sufficient for certain designs.

-	Source [10] mentioned that device 1 can support basic calibration operation within a reasonable power consumption or complexity, achieving accuracy of 104 ppm.

-	Source [15] mentioned that with clock calibration based on a clock acquisition part design, 104 ppm could be achieved after clock calibration.

-	Source [13] reported that clock purpose #1 could be applied to all device types. But, considering complexity and power, device 1 and 2 could have different power, accuracy. For device1, the power consumption should be <1 µW; the timing accuracy should be 104 - 105 ppm; the calibration can reach to 104 ppm.

-	Source [14] proposed device 1 with accuracy of 104 ppm after clock sync/calibration.

-	Source [16] reported that using simple clock counting based calibration method, 1% could be achieved with power consumption <1 µW. It was reported that 1% is the sweet spot of target accuracy considering performance, complexity, signal design, etc.

-	Source [9] mentioned that device can use recommended clock calibration method to get high accuracy of 103 - 104 ppm for device 1 and 102 - 103 ppm for device 2.

# 6	Solutions for Ambient IoT

(…Omitted in this document…)

# 6A	Solutions for Ambient IoT in outdoor scenarios with active devices

Temporary note: These clauses may refer to Clauses 6.x in case same or similar terminology is being used between Rel-19 and Rel-20.

## 6A.1	R2D

### 6A.1.x	R2D waveform, modulation and bandwidth

The R2D waveform described in TS 38.291 [228] is used for the study, with the possible addition of Mchipsymb = 1 and Mchipsymb = 32 to, and the possible removal of Mchipsymb = 24 from, the Mchipsymb value set in TS 38.291 [228] being studied, as summarised in Table 6A.1.x-1:

Table 6A.1.x-1: Study of M-values for R2D

| Mchipsymb | Study aspect |
| --- | --- |
| 1 | Possible addition |
| 2 | As in Rel-19 |
| 6 |  |
| 12 |  |
| 24 | Possible removal |
| 32 | Possible addition |

A minimum R2D transmission bandwidth, Btx,R2D, of 1 PRB is studied as the baseline.

### 6A.1.x	R2D generation

#### 6A.1.x.y	PRDCH overall

PRDCH is studied via the blocks shown in Figure 6A.1.x.y. Other clauses give their detailed descriptions.

Temporary note: Following are placeholders for later agreements on block orders

Figure 6A.1.x.y-1: PRDCH generation – block order 1

Figure 6A.1.x.y-z: PRDCH generation – block order z

#### 6A.1.x.y	Channel coding

FEC for R2D is studied, with code rates of 1/2, 1/3 and 1/4 and the following candidate coding schemes:

Coding scheme 1: LTE tail biting convolutional code

Coding scheme 2: Tailed convolutional codes with the same polynomials as LTE tail biting convolutional codes

Coding scheme 3: Reed-Muller codes

#### 6A.1.x.y	Interleaving and bit collection

Interleaving for PRDCH with FEC and the use of the LTE bit collection scheme with the following candidate methods are studied:

Alt 1: Both the LTE sub-block interleaver and the LTE bit collection scheme are used.

Alt 2: Only the LTE bit collection scheme is used.

Alt 3: Only an updated bit collection scheme based on the LTE bit collection scheme is used.

#### 6A.1.x.y	Repetition

For the definitions of repetitions, see Clause 6.1.0.1. Block level, bit level type 2 and chip level repetitions are studied.

### 6A.1.x	R2D timing

#### 6A.1.x.y	Start of R2D

For the functionality of indicating the start of the R2D transmission containing PRDCH, the following options are studied:

Option 1: A binary sequence-based SIP, where the candidate sequence types are as follows:

Option 1-1: m-sequence

Option 1-2: Golay sequence

Option 2: A SIP as per TS 38.291 [228]

For Option 1, aspects such as the length of the sequence used, the number of sequences and the sequence type are studied. The same aspects are studied for an R2D midamble, if it is used together with a sequence-based SIP.

Temporary note: The sentence about R2D midamble may be moved into a dedicated clause later, depending on future agreements.

#### 6A.1.x.y	Chip duration determination

The chip duration determination of the data payload in the PRDCH and the L1 control information are studied. The following options are studied for the functionality of the chip duration determination of the data payload in the PRDCH:

Option 1-data: The chip duration of the corresponding L1 R2D control information is used.

Option 2-data: The chip duration indicated by the corresponding L1 R2D control information is used.

The following options are studied for the functionality of the chip duration determination of the L1 R2D control information:

Option 1-control: A fixed chip duration is used, with no use of CAP.

Option 2-control: A set of pre-defined chip durations, blindly detected by a device with no use of CAP.

Option 3-control: A CAP is used, where multiple alternatives are studied:

Option 3-1-control: Using the pattern of CAP defined in TS 38.291 [228].

Option 3-2-control:  Enhancing CAP by the following options:

Option 3-2a-control: Using the pattern of CAP defined in TS 38.291 [228] with repetitions,

Option 3-2b-control: Using a pattern that associates the chip duration of the clock acquisition part to different chip durations of the L1 R2D control information.

Option 4-control: A set of binary sequences with fixed or variable length is used, where multiple alternatives are studied:

Option 4-1-control: Use of the binary sequence-based SIP, with no use of CAP.

Option 4-2-control: Use of binary sequences for the CAP.

Option 5-control: Broadcast information is used to indicate the chip duration.

#### 6A.1.x.y	SFO calibration

Temporary note: This clause may be merged and/or split into other clauses, depending on later agreements.

The functionality of SFO calibration using the following options are studied:

Option 1: CAP defined in TS 38.291 [228] and/or Manchester encoding.

Option 2: A sequence-based SIP with or without R2D midamble(s).

Option 3: A CFO calibration signal.

#### 6A.1.x.y	Synchronization signal

A synchronization signal for the functionality of frequency synchronization, including frequency acquisition, and timing synchronization/tracking is studied, with the options of being periodic or aperiodic.

#### 6A.1.x.y	R2D end timing

Use of L1 R2D control information to indicate the TBS to provide the functionality of indicating the end of the PRDCH is studied.

### 6A.1.x	CFO calibration signal

A CFO calibration signal for the functionality of CFO estimation/LO calibration at the device is studied, where the signal is transmitted periodically and/or as needed. For the signal design, the following options are studied:

Option A: Unmodulated single tone sinusoid.

Option B: Unmodulated multiple single tone sinusoids.

Option C: Multiple modulated tones, where the CFO calibration signal may be the same as or different from the synchronization signal.

The time location of the CFO calibration signal in relation to a reference is studied, with the following candidate time references:

Option 1: L1 R2D control information

Option 2: Data payload of the PRDCH

Option 3: PDRCH

Option 4: Synchronization signal

Option 5: Broadcast information

### 6A.1.x	Broadcast information

Broadcast information is studied, including the information included, the time and frequency resources for its transmission, whether it is carried on the PRDCH, and whether it is transmitted periodically or aperiodically.

### 6A.1.x	R2D multiplexing

FDM for R2D from the same reader is studied, considering the complexity of a device required to support FDM, and how a device determines the frequency resource to receive the FDMed R2D transmissions.

### 6A.1.x	R2D control information

L1 R2D control information preceding the data payload of the PRDCH is studied, considering the necessary information included in the L1 R2D control, and how the control information is transmitted:

Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time.

Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

Aspects regarding the frequency resource used for its transmission and whether it is included in the PRDCH carrying the data payload, in another PRDCH or in another channel are studied.

#### 6A.1.x.1	CRC

Temporary note: This may be merged to the PRDCH clause 6A.1.x.1 depending on how the study proceeds.

R2D control information with a separate CRC from data is studied.

## 6A.2	D2R

### 6A.2.x	D2R waveform and modulation

The reuse, modification or enhancement of the D2R modulation schemes defined in TS 38.291 [228], including down-selection among them, and the potential introduction of new modulation schemes, considering the impact on spectrum shaping and D2R signals, are studied. The following candidate modulation schemes are studied: BPSK, OOK, DBPSK, MSK.

A single-carrier waveform is assumed for the study.

Temporary note: The structure of this section will be updated once more agreements on comparisons, etc. are available.

### 6A.2.x	D2R generation

#### 6A.2.x.y	PDRCH overall

PDRCH is studied via the blocks shown in Figure 6A.2.x.y, where the order of the blocks is studied and other clauses give their detailed descriptions.

Temporary note: Following are placeholders for later agreements on block orders

Figure 6A.2.x.y-1: PDRCH generation – block order 1

Figure 6A.2.x.y-z: PDRCH generation – block order z

#### 6A.2.x.y	Channel coding

Enhancements on the D2R channel coding scheme defined in TS 38.291 [228], considering additional FEC code rates and without any change to the polynomials defined in TS 38.291 [228], is studied.

#### 6A.2.x.y	Interleaving and bit collection

Interleaving for PDRCH with FEC and the use of the LTE bit collection scheme with the following candidate methods are studied.

Alt 1: Both the LTE sub-block interleaver and the LTE bit collection scheme are used.

Alt 2: Only the LTE bit collection scheme as per is used.

Alt 3: Only an updated bit collection scheme based on the LTE bit collection scheme is used.

#### 6A.2.x.y	Repetition

For the definitions of repetitions, see Clause 6.1.0.1. For D2R, a number of repetitions larger than 2, as supported in Rel-19, for block-level repetition is studied.

#### 6A.2.x.y	Scrambling

Scrambling for D2R is studied, considering the scrambling sequence type, the sequence generation method and whether it is applied after block-level repetition, channel coding and interleaving.

### 6A.2.x	D2R timing

#### 6A.2.x.y	D2R preamble

The D2R preamble length, the sequence type used, the number of sequences using the same length and other factors affecting the preamble design are studied.

#### 6A.2.x.y	D2R midambles

A D2R midamble signal is studied, considering the midambles pattern(s), the pattern length(s) and interval(s), the sequence type used and whether it is different from the sequence type used for the D2R preamble in Clause 6A.2.x.y, and other factors affecting the midamble design.

### 6A.2.x	D2R multiplexing

For D2R, enhancements to the time-domain multiple access and frequency-domain multiple access defined in TS 38.291 [228] are studied.

For TDMA, for contention-based random access and DT and DO-DTT traffic types, enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the Access Random ID message (Msg1) to greater than 2 and for the D2R Upper Layer Data Transfer message (Msg3) to greater than 1, where the messages are defined in TS 38.391 [229], are studied.

For FDMA, enhancements considering the D2R frequency resources used and how the device determines these resources, whether baseband small frequency shift is supported and whether FDMA is applied to other D2R transmissions apart from the Access Random ID message (Msg1) and D2R Upper Layer Data Transfer message (Msg3) defined in TS 38.391 [229], are studied.

Code-domain multiple access is studied, considering the sequence type used, the length of the sequence and the number of sequences.

## 6A.3	L1 procedures

Temporary note: If there is a need for a clause on ‘reader search’-like procedure, it can be added here.

### 6A.3.x	D2R power control

Power control for D2R is studied, considering open loop and/or closed loop power control, the required measurements, the required information at the reader and device, the signalling overhead and power consumption.

The following methods for open loop power control are studied, considering the device complexity:

Option 1: Transmit power is calculated based on the measured pathloss using a predefined formula and configured parameters from the reader, with the device requiring information on at least the reader’s transmit power and the received target power at the reader.

Option 2: Transmit power is determined based on the measured RSRP using an RSRP range based on RSRP thresholds that are predefined or configured by the reader, with the device requiring information on at least the RSRP threshold(s) and the corresponding D2R transmit power-related information for each RSRP range defined by the RSRP threshold(s).

## 6A.3	DO-A

Temporary note: The further structure of this clause will be updated later, and/or placed in other clauses if appropriate.

Support for the DO-A traffic type considering both periodic and event-triggered transmissions is studied, considering the time and frequency synchronization required for the device to transmit DO-A traffic, the resource allocation/determination method, the configuration/signalling of necessary information whether the reader provides this information periodically and/or aperiodically, and whether the resources themselves are periodic or aperiodic.

The following options on how the necessary information for the first D2R transmission for DO-A traffic is provided to the device are studied:

Option 1: Provided by broadcast information

Option 2: Provided by a paging-like R2D message

Option 3: Derived from the synchronization signal and provided by broadcast information.

The following options for the content of the first D2R transmission for DO-A traffic are studied:

Content type 1: Access Random ID message (Msg1) like content

Content type 2: DO-A data payload

Content type 3: DO-A scheduling request

## 6A.5	Timing relationships

The changing of timing offsets defined in TS 38.291 [228] and the introduction of new timing offsets are studied.

Temporary note: The agreed list is only examples, to date. The content of this section will be formed once agreements clarify what is being studied.

## 6A.6	Positioning

Temporary note: The content of this section will be formed once agreements on this aspect are made.

## 6A.z	Coexistence

Temporary note: Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios [RAN4].

# 7	Evaluations

## 7.1	Coverage evaluations

For an evaluation scenario:

-	For each link i,

-	Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements if Budget-Alt2 is used for this link i.

-	Step 2: Obtain the receiver sensitivity using the method Budget-Alt1 (if a predefined threshold is assumed to derive the receiver sensitivity) or Budget-Alt2 (if no predefined threshold is assumed to derive the receiver sensitivity). See Clause 4.3.1 for the Budget definition.

-	Step 3: Obtain the coverage performance for link i based on the receiver sensitivity from step 2 and link budget template.

-	The coverage results for each link are provided.

RAN1 has evaluated the coverage for InF-DH NLOS scenarios for D1T1 and InH-Office LOS/InF-DL NLOS scenarios for D2T2, across all devices 1/2a/2b. Coverage evaluation results comparing various R2D Tx power/EIRP value, R2D Rx sensitivity, BLER 1%/10% target, data rate ~1 kbps/~5-7 kbps, D2R coherent receiver/non-coherent receiver, R2D budget-Alt1 and Alt2, CW cancellation, backscatter loss and modulation schemes, D2R with/without channel coding schemes, etc, have been provided. The observations for coverage evaluation results can be found in Clauses 7.1.1.1.2, 7.1.1.2.2, 7.1.1.3.2, 7.1.2.1.2, 7.1.2.2.2, and 7.1.2.3.2.

All the evaluation results are contained in the attached spreadsheet file [189].

### 7.1.1	Link budget for D1T1

See associated file.

### 7.1.2	Link budget for D2T2

See associated file.

### 7.1.3	Link performance for D2R

See associated file.

### 7.1.4	Link performance for R2D

See associated file.

## 7.2	Latency evaluations

### 7.2.1	Single device latency

(Omitted in this document)

### 7.2.2	Inventory completion time for multiple devices

(Omitted in this document)

## 7.3	Evaluations for outdoor scenarios

For an evaluation scenario:

-	For each link j,

-	Step 1: Obtain the required SINR for the physical channels under different reference data rates for this link j.

-	Step 2: Obtain the receiver sensitivity using the method Budget-Alt2. See Clause 4.3.1 for the Budget definition.

-	Step 3: Obtain the MPL/distance for link j based on the receiver sensitivity from step 2 and link budget template.

-	The coverage results for each link are provided.

# 8	Conclusions and recommendations

## 8.1	Indoor scenarios

It is feasible to support a harmonized air interface design, with minimized differences (where necessary) for Ambient IoT, to enable the devices described in Clause 3 for addressing the objectives stated in [2]. The extent to which maximum distance targets, differentiated per device and deployment/topology scenario, as described in Clause 4.1, can be met under different sets of evaluation assumptions is reported in Clause 7.1. A latency target for inventory of a single device in TR 38.848, can be met under different evaluation assumptions according to evaluations reported in Clause 7.2.1. The addressed subset of design targets described in TR 38.848 can be met or taken into account in feasible designs, and the design targets left to WGs in Clause 5 of TR 38.848 have been refined. Evaluations have also reported the inventory completion time for multiple devices. Corresponding evaluation scenarios, assumptions, and methodologies have been developed.

The basic blocks of A-IoT device architectures based on RF-ED receiver for device 1, 2a, and 2b have been identified, and also for receiver architectures based on IF-ED or ZIF for device 2b. For device 2a, the characteristics of blocks representing reflection amplifier(s) and large frequency shifter have been studied. Aspects to consider further regarding whether the large frequency shifter block is feasible, and its necessity or effectiveness have also been identified.

Purposes for which clocks(s) may be used by the different A-IoT devices 1/2a/2b have been identified, together with descriptions of the corresponding power consumption, and clock accuracy characteristics.

Regarding the potential impact of device (un)availability due to energy harvesting, two 'Directions' were found, depending on whether the reader does not, or can, provide information to a device based on which the device may become available/unavailable. Company solutions are reported in Clause 6.2, grouped according to the Direction they apply to.

The physical channels studied are PRDCH in R2D, and PDRCH in D2R, for both of which the generation process has been described at high level. Feasible candidates for the methods involved in the processes have also been documented, together with evaluations and pros/cons reported by companies. Information related to R2D reception and D2R scheduling have been listed together with ways they could be conveyed.

The physical signals studied have potential purposes involving start indication, timing acquisition/tracking, calibration/frequency synchronization at the device and the reader, channel or interference estimation at the reader, and indicating the end of a R2D/D2R transmission. Some of those purposes could be provided by information in a PRDCH.

Pros and cons of multiplexing/multiple access options for R2D and D2R links have been studied by company reports. Options and relationships among the resource allocations for messages of the random access procedures have been identified. The needed timing relationships between corresponding transmissions have been captured, for example the minimum time between a R2D transmission and the corresponding D2R transmission following it; and others.

Carrier-wave waveforms and their characteristics have been identified, together with cases for how CW transmission could be deployed in terms of the CW node and the spectrum used. Two CW waveforms were studied: a single-tone unmodulated sinusoid; or two unmodulated single-tone sinusoids. Their characteristics were studied and compared in terms of D2R backscattering reception performance and CW interference suppression at the reader, spectrum utilization of the backscattered signal, and relative complexity of CW generation. The effects of frequency hopping and antenna hopping were included.

For Topology 2, RAN1 studied resource allocation and/or control of the intermediate UE via the Uu interface via higher-layer signaling, or L1 and higher-layer signaling.

The harmonized air interface design focuses on DT and DO-DTT use cases. The study also assessed whether the harmonized air interface design can address the DO-A use case, and concluded that it cannot. The part(s) which is/are not sufficient for the DO-A use case are summarized in Clause 6.10.

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR.

(1)	The following techniques are recommended in the first normative phase:

-	PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

-	Modulations of OOK-1 and OOK-4 in R2D

-	Multiplexing in time domain for R2D

-	Convolutional FEC in D2R and no FEC in R2D

-	R2D and D2R signal(s)

(2)	At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release:

-	Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture)

-	D1T1 and/or D2T2

-	Scenario A1 and/or A2 and/or B and/or C

-	Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

-	Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D

-	OOK and/or BPSK and/or MSK modulation for D2R

-	Carrier-wave waveform 1 and/or 2, and transmission case(s)

-	Device (un)availability direction 1 and/or 2

-	Proximity determination solution 1 and/or 2

From RAN2 perspective, RAN2 has studied Topology 1 and Topology 2, and has concluded they are feasible. In detail:

-	General AS procedures, A-IoT paging, A-IoT random access, A-IoT data transmission and Topology 2 aspects have been studied:

-	See clause 6.3.1 for the overall AS procedure and the information useful to be visible to the reader from CN, etc.

-	See clause 6.3.3 for the A-IoT paging function, including paging message content, paging monitoring behavior, multiple (subsequent) A-IoT paging messages for the same service, etc.

-	See clause 6.3.4) for the A-IoT random access, including the contention-free access and contention-based random access procedure, the message contents, the re-access solutions, etc.

-	See clause 6.3.2 and 6.3.5 for protocol stack for A-IoT radio interface between A-IoT device and reader, required functionalities, and A-IoT data transmission functionalities, etc.

-	See clause 6.3.6 for the Topology 2 specific aspects, including the architecture options and management of radio resources in A-IoT radio interface, etc.

-	From RAN2 perspective, the descriptions under clause 6.3 and 6.10 are considered as the baseline conclusions.

-	Following is further concluded:

-	At least following features are recommended for the normative phase in the A-IoT MAC layer:

-	A-IoT paging

-	A-IoT random access:

-	For contention-based random access, further down-selection between Solution 1 (3step-only) and Solution 3 (unified solution) in clause 6.3.4.

-	A-IoT data transmission

-	For Topology 2, at least the following aspects are considered:

-	Forwarding of A-IoT upper layer data and A-IoT radio resource management.

-	No further down-selection for Topology 2 architecture options (which are feasible) from RAN2 perspective (the final decision depends on TSGs/WGs outcome).

Logical system architectures have been studied, as documented in clause 6.4.

Impacts on CN-RAN interface, signaling and procedures have been studied to support A-IoT Inventory operation and Command operation for topology 1 and topology 2 respectively, as documented in clause 6.5.

Solutions for locating an Ambient IoT device with minimal specification impact are identified, as documented in clause 6.9.

In RAN3, no evaluation and down selection for T2 solutions have been performed which requires coordination among WGs.

RAN4 has performed adjacent channel co-existence studies between ambient IOT and NR for the scenario that NR macro BS outdoor and ambient IOT system indoor. RAN4 evaluated a total of 9 deployment scenarios as described in Table 6.6.2-1. For each deployment scenario, a total of 4 cases were performed as described in Table 6.6.2-1. The co-existence cases are described in Table 6.6.2-2. The performance metrics for NR system impacted by ambient IOT are throughput loss at the cell edge and cell average. The performance metric for ambient IOT system impacted by NR system is SINR degradation for inter-system interference impact analysis. Meanwhile, in addition to SINR degradation, ambient IOT intra-system performance evaluated by link level simulation is also one of the key aspects affecting ambient IOT system performance, which can be further analyzed in WI phase. The summary of co-existence evaluation can be found in clause 6.6.5.

RAN4 has studied the RF requirements impact for ambient IOT, including system parameters, ambient IoT BS, intermediate UE, devices (device 1, 2a and 2b) and CW node, which can be found in clause 6.7.1, 6.7.2, 6.7.3 and 6.7.4.

RAN4 has also studied the feasibility of CW interference cancellation for both ambient IoT BS and intermediate UE, and the feasible interference cancellation values can be found in clause 6.7.5.

Moreover, RAN4 also studied the testability issue for ambient IOT devices, which can be found in clause 6.7.4.4.

## 8.2	Outdoor scenarios

Annex A:	Device receiver sensitivity for Budget-Alt1

<Unchanged parts omitted>