3GPP TSG-RAN WG1 Meeting #123	R1-2508513

Dallas, USA, November 17th – 21st 2025

Agenda Item:	10.3.2.2

Source:	Ericsson

Title:	D2R signals, channels, waveform, and procedures

Document for:	Discussion, Decision

# Introduction

The Rel-20 study item “Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices” [1] is to study a second phase of RAN aspects for outdoor scenarios of Ambient IoT, a 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies e.g. NB-IoT including with reduced peak Tx power, nor the features of 6G relevant to IoT.

The SID [1] states the following regarding SI objective:

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. Define link budget calculation for coverage

In this contribution, we present our views on the necessary and feasible Rel-19 air-interface enhancements for Device 2b/C including waveform and modulation design, FEC and CRC configuration, repetition schemes, bandwidth considerations, timing and synchronization signals, multiplexing/multiple access, and scheduling.

# D2R: Physical channels and signals generation

## D2R waveform and modulation

RAN1#122 made the following agreement regarding D2R modulation:

Agreement Study necessity and feasibility of reuse, modifications or enhancements of existing Rel-19 D2R modulations, and the potential introduction of new modulations schemes, for Device 2b and for Device C, including at least the following aspects: - introduction of new modulation schemes (e.g., MSK or DBPSK) - down-selection of existing modulation schemes - design considerations related to spectrum shaping and D2R signals

RAN1#122b made the following agreement regarding D2R modulation:

Agreement Regarding D2R modulation for device 2b and device C, further study the following: - Option 1: BPSK - FFS the corresponding preamble and midamble design for coherent detection at reader side - Option 2: OOK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side - Option 3: DBPSK - FFS the corresponding preamble and midamble design for non-coherent detection at reader side - Option 4: MSK - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

We compared the spectral performance and the bit error rate (BER) performance of BPSK, OOK, and MSK modulations. The simulation parameters are set in Table 1.

Table 1: Evaluation parameters for the D2R modulation

| Parameters | Value |
| --- | --- |
| Sampling rate | 1.92 MHz |
| Channel type | AWGN |
| Number of transmitted bits | 96 |
| Signal bandwidth | 180 kHz |
| Symbol rate | 180 kHz |

Figure 1: Spectrum performance of different modulation schemes

Figure 2: BER performance of different modulation schemes under AWGN channel

From Figure 1 and Figure 2, the following observations can be made.

In terms of spectral efficiency, MSK is more efficient than OOK, and OOK is more efficient than BPSK.

In terms of BER performance, BPSK and MSK have the same performance and are better than OOK.

Pulse shaping is necessary to reduce spectral leakage.

We summarize the features of different modulation schemes in Table 2.

Table 2: Evaluation results of D2R modulation schemes

| Modulation scheme | MSK |  | BPSK |  | OOK |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Active | Passive | Active | Passive | Active | Passive |
| Tx complexity | Good | Good | Fair | Good | Good | Good |
| Uplink link budget | Good | Good | Good | Good | Poor | Poor |
| Spectrum efficiency | Good | Good | Fair (without pulse shaping ) | Fair (without pulse shaping ) | Poor | Poor |
| BER performance | Good | Good | Good | Good | Fair | Fair |
| TX energy consumption | Good | Good | Fair | Good | Poor | Good |
| Phase continuity | Yes | Yes | No | No | No | No |
| PAPR | Good | Good | Fair (with pulse shaping) | Fair | Poor | Poor |
| Constant envelope | Yes | Yes | Pulse shaping dependent | Pulse shaping dependent | No | No |

In Rel-19, Device 1 can use either OOK or BPSK for modulation. Down-selecting between OOK, BPSK, MSK offers several benefits, including:

- Avoid Blind Detection Complexity at Receiver: Blindly detecting modulation requires additional processing and decision logic at the receiver (e.g., energy detection, hypothesis testing).

- Improve Decoding Reliability: Blind detection is prone to modulation misclassification, especially in low SNR conditions, leading to incorrect decoding and degraded system performance.

- Minimize Latency in Contention-Based Access: In contention-based D2R, fast demodulation is needed to avoid missing subsequent signals.

- Enable Simpler System-Level Optimization: Fixing the modulation allows for better link budget planning, power control, and interference analysis.

- Down-selecting the modulation scheme eliminates the need for blind detection at the receiver, reducing complexity, power consumption, and decoding errors. It also simplifies system design, improves reliability, and enables more efficient resource implementations.

- To improve efficiency and reliability, one modulation scheme between OOK, BPSK and MSK needs to be down-selected.

## D2R line coding, FEC, CRC and repetitions

### D2R FEC

RAN1#122 made the following agreement regarding D2R FEC:

Agreement Study necessity and feasibility of enhancements on existing Rel-19 D2R FEC for Device 2b and for Device C including the following aspects: - additional FEC code rate(s) (e.g., 1/2, 1/4) - interleaver, or LTE bit collection scheme for TBCC - Note: no change to Rel-19 TBCC polynomial

RAN1#122b made the following agreement regarding D2R FEC:

Agreement Study interleaving for PDRCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver - Combination of alternatives can be studied Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

D2R supports only convolutional coding using the generator polynomials specified in TS 36.212. In addition to the mother code rate of 1/3, a coding rate of 1/2 is natively supported by Rel-19 A-IoT (or LTE) TBCC, offering additional benefits. For example, instead of relying on simple repetition, a convolutional code with a rate of 1/2 improves resource efficiency while providing coding gain. This rate is achieved through puncturing, which removes one out of every three coded bits, without increasing encoder complexity for an A-IoT device. However, a code rate of 1/4 is not natively supported by TBCC as specified in TS 36.212. Interleaving and bit collection require data buffering and, therefore, memory, which is not feasible for Device 2b/C.

Interleaving and bit collection require buffering, which is not feasible for Device 2b/C.

TBCC, as specified in TS 36.212, does not natively support a code rate of 1/4.

In addition to the mother code rate of 1/3,

      - support a coding rate of 1/2 for Device 2b/C.

  - not support a coding rate of 1/4 for Device 2b/C.

  - not support interleaver or bit collection

### D2R repetitions

RAN1#122 made the following agreement regarding D2R repetitions:

Agreement For PDRCH, study necessity and feasibility of larger (>2) number of repetitions for block-level repetitions for Device 2b and for Device C.

Block-level repetition greater than two does not provide significant gain for Device 2b due to poor clock accuracy. For Device C, the necessity of block-level repetition greater than two depends on coverage evaluation results and the need to meet coverage targets in feasible A-IoT use cases.

The necessity of block-level repetition greater than two should be discussed based on coverage evaluation results and the coverage targets for feasible A-IoT use cases.

### D2R scrambling

RAN1#122 made the following agreement regarding D2R scrambling:

Agreement For PDRCH, study necessity of scrambling for Device 2b and for Device C

RAN1#122b made the following agreement regarding D2R scrambling:

Agreement For PDRCH, further study the necessity of scrambling with following details: - Scrambling applied after block-level repetition (if applied) and channel coding (if applied, including interleaving if supported) - Scrambling sequence type and generation method

Scrambling is a bit-level operation used to randomize the transmitted bit patterns so that the signal appears statistically uniform. This prevents long runs of identical bits, which can degrade clock recovery and synchronization at the receiver. By randomizing the sequence, scrambling also ensures a more even power spectral distribution, reducing narrowband interference and avoiding spectral peaks that may occur with structured data patterns. Although scrambling does not add redundancy or perform error correction, it enhances the effectiveness of other physical-layer techniques (such as FEC) by providing a well-distributed bitstream that is more robust to channel impairments.

The scrambling and descrambling operations require only a small Linear Feedback Shift Register (LFSR) and one XOR operation per bit, resulting in negligible computational and hardware complexity at both the transmitter and receiver.

D2R scrambling is necessary to reduce interference and ensure statistical uniformity, preventing long runs of identical bits and improving clock recovery, synchronization, and spectral efficiency.

Scrambling and descrambling are feasible, requiring only a small LFSR and one XOR per bit, leading to negligible hardware complexity at both transmitter and receiver.

D2R scrambling for PRDCH is necessary and feasible.

## D2R bandwidth

In Rel-19 A-IoT, 2SB transmissions for D2R were considered to reduce the complexity. SSB (1SB) is a variation of DSB modulation, where only one of the sidebands is transmitted and the other is suppressed. This reduces the bandwidth and power consumption. To demodulate the SSB signal, the receiver needs to reconstruct the missing sideband with a phase-shifted local oscillator and then filter out the unwanted components. We summarize the difference of SSB and DSB in Table 3.

Table 3: Comparison of DSB-OOK and SSB-OOK

|  | DSB-OOK | SSB-OOK |
| --- | --- | --- |
| Spectrum |  |  |
| Advantages | - Easy to implement - High fidelity - Simpler to demodulate | - Reduced bandwidth - Reduced power consumption - Accommodating more channels |
| Disadvantages | - Low power efficiency - Higher bandwidth | - Precise filtering and synchronization - Accurate tuning and alignment - More complex |

DSB-OOK is bandwidth inefficient while easy to implement. SSB-OOK is spectrum efficient, but it is more complex. It is important to note that SSB modulation is only applicable to real-valued signals (e.g., OOK or ASK) and not be applicable for all D2R modulation candidates.

Support SSB for device 2b and C

## D2R timing and sync signals

The synchronization signal requirement for active devices can consider the Rel-19 design of preamble and midamble. The enhancements of sync blocks either in number or length can be considered first to correct large CFO and SFO. The Rel-20 A-IoT devices internally generates RF carrier by themselves, e.g., at 900 MHz and with 0.1~1% CFO the device will have an error of 900 kHz~9 MHz. To correct such a large CFO, study needs to be done for the estimation and correction of CFO by increasing the number or length of synchronization blocks.

Study the enhancements of preamble and/or midamble either in number or length to correct large CFO and SFO.

- Regarding the D2R preamble and midambles, the following agreements have been reached.

Agreement Regarding D2R preamble for PDRCH, study the following aspects of the D2R preamble. - Preamble length (e.g., longer than Rel-19 D2R preamble) - Sequence type - Number of sequences using the same length (e.g., for reader/device differentiation) - Considering other factors affecting preamble design (e.g., D2R modulation and detection schemes) Agreement Regarding D2R midamble for PDRCH, study the following aspects of the D2R midamble. - Midamble pattern(s) including the length(s) and interval(s) within (and possibly at the end of) a PDRCH - Sequence type, and whether the same or different sequence type is used for D2R preamble and D2R midamble - Considering other factors affecting midamble design (e.g., D2R modulation and detection schemes)

In Release 19, m-sequences of lengths 7 and 31 are used for D2R preamble and midamble. The higher layers indicate several parameters related to D2R amble configuration. These include the interval in bits for D2R midamble insertion, the sequence length indicator for D2R ambles, and the additional D2R midamble insertion indicator. Together, these parameters define how often midambles are inserted, the sequence length used for synchronization signals, and whether extra midambles are included to enhance channel estimation and tracking performance.

Regarding the addition of CFO estimation functionality to the D2R ambles, it is critical that the device maintains proper CFO calibration before transmitting its D2R data; otherwise, it may transmit in frequency bands that are not allocated to it. In such cases, either large guard bands would be required, or coexistence issues may occur. Therefore, it is particularly important that the R2D CFO calibration signal be carefully designed to achieve a good level of coarse CFO calibration. The remaining fine calibration can then be performed using the D2R ambles at the reader side.

Regarding the indication of the amble length and midamble intervals for the first D2R message in Do-A traffic, these parameters can still be specified either through the same approach used for indicating the message’s resource allocation information or by using fixed, predefined values.

## D2R multiplexing/multiple access

### D2R TDM(A)

RAN1#122 made the following agreement regarding D2R TDM(A).

Agreement At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following: - X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message) - X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

With regards to the above agreement, our view is that the necessity of any further enhancements on X1 should be motivated by the proponents from capacity and latency perspectives potentially using evaluations.

On discussion related to the value of X3, we think this is a bit of a futile discussion. There is a need to indicate or configure the number of time domain resources/occasions if the resources are randomly selected by a device. Unlike for Msg1 transmissions, this is not the case for Msg3 as dedicated resources should be allocated for Msg3. Each device that successfully received its random ID in Msg2 will have a dedicated time and frequency domain resource for Msg3 transmission, which can be indicated through Msg2. Therefore, we think the value of X3 should always be 1 from device perspective.

Unlike for Msg1 transmissions where there is need to indicate the number of time domain resources/occasions that can be randomly selected by a device, a dedicated time-domain resource is provided for Msg3 transmission.

Determine if X1 = 2 is sufficient to meet the capacity and latency that outdoor use cases demand. The necessity of further enhancements to X1 = 2 for Msg1 should be motivated by the proponents from capacity and latency perspectives potentially using evaluations.

From device perspective, X3 should be 1.

### D2R FDM(A)

Agreement Study necessity and feasibility of enhancements of D2R for Device 2b and for Device C, considering the following aspects or potential enhancements: - Considering that Device 2b and Device C can adjust the LO towards a desired frequency, whether to also support baseband small FS for Device 2b and Device C - D2R frequency resources for FDM(A) (e.g., considering D2R transmission bandwidth, guard-band size between FDMed D2R transmissions, CFO, SFO, etc.) - How a device determines the frequency resource for D2R transmission - FDM(A) for D2R transmissions other than Msg1 and Msg3

The Rel-19 A-IoT device uses small frequency shift to offset the backscatter signal power onto a carrier frequency away from the carrier wave to mitigate interference and facilitate FDMA. For Rel-20, the device type 2b and c have an LO which can tune into a desired carrier frequency. Therefore, we think supporting small FS for Device 2b and Device C is not necessary.

It is not necessary to support baseband small FS for Device 2b and Device C in Rel-20.

Regarding frequency and time resource allocation for D2R transmission, the device can determine the resource differently for different D2R transmissions. For Msg1 transmission for device accessing the reader or device with DO-A traffic, a broadcast information may be transmitted periodically by the reader which provides multiple occasions with frequency and time resource for the device. After the device identifies the resource sets, the device is allowed to determine the resource among the multiple occasions. For Msg1 transmission for DT and DO-DTT traffic, the device acquires the frequency and time resource from paging message. For Msg3 transmission, the frequency and time resource can be carried by msg2.

The device can determine the frequency and time resources from different messages (e.g SIB, Msg2) for different D2R transmissions (e.g., Msg1, Msg3 etc).

The device is allowed to determine the frequency and time resources for MSG1 transmission for DO-A traffic.

### D2R CDM(A)

Agreement Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures. - FFS: other cases, except for data transmission Agreement For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A) for Device 2b and for Device C, study at least the following aspects: - Sequence type (e.g. Gold sequence, m-sequence) - Length of the sequence - Number of sequences

In Rel-19, CDMA was not specified for A-IoT due to low device complexity. For Rel-20, although device 2b/c are more capable than device 1, they still suffer from large CFO/SFO and related issues which are critical for CDMA implementation. Meanwhile, CDMA also requires accurate timing and power control which are also strict for A-IoT devices. On the other hand, the Rel-20 A-IoT devices are already utilizing a combination of TDMA and FDMA for increased efficiency and reduced collisions and therefore the necessity of introducing CDMA on top of the existing mechanisms needs to be validated. Therefore, we think that TDMA and FDMA should be the baseline for D2R transmission while CDMA may not be needed due to high complexity unless the service requirement, e.g., latency, cannot be met.

CDMA may not be needed due to high complexity unless the service requirement cannot be met.

## D2R scheduling

In Rel-19, all the D2R scheduling information (related to resource allocation) is signaled by higher-layer signaling via the corresponding PRDCH.

In general, the D2R scheduling information could include the following in Rel-20:

- Payload size/TBS

- whether FEC is applied or not (and code rate, if multiple code rates are considered)

- Number of block-level repetitions

- bit duration (Tb)

- sequence length for D2R preamble and midamble (short or long, if multiple lengths are considered)

- interval bits for D2R midamble insertion

- whether the midamble is present at the end

- Frequency domain information

- Timing offset (see discussion in our companion paper [8])

In Rel-19, D2R scheduling parameters are the same for all FDM-ed devices in D2R. However, for Rel-20, there should be flexibility to provide different D2R scheduling parameters for different FDM-ed devices. Note also that scheduling information related to frequency resource allocation (including the bandwidth) for Device 1 (Rel-19) is implicit from the values of Tb and Tchip. Specifically, the center frequency of the frequency domain resource to be used by a device, or equivalently, the amount of small frequency shift is given by R/Tb = 1/(2 × Tchip), where R is codeword repetition number or the number of square wave periods. Whereas, the D2R transmission bandwidth of the device is given by 2/Tb. For active devices such as Device 2b and C, the frequency domain information can be explicitly provided in the D2R scheduling information.

For Device 2b/C in Rel-20, frequency domain resource allocation can be provided explicitly in the D2R scheduling information.

For Device 2b/C in Rel-20, there should be flexibility to provide different D2R scheduling parameters for different FDM-ed devices

# Conclusion

In the previous sections we made the following observations:

Observation 1	In terms of spectral efficiency, MSK is more efficient than OOK, and OOK is more efficient than BPSK.

Observation 2	In terms of BER performance, BPSK and MSK have the same performance and are better than OOK.

Observation 3	Pulse shaping is necessary to reduce spectral leakage.

Observation 5	Interleaving and bit collection require buffering, which is not feasible for Device 2b/C.

Observation 6	TBCC, as specified in TS 36.212, does not natively support a code rate of 1/4.

Observation 7	D2R scrambling is necessary to reduce interference and ensure statistical uniformity, preventing long runs of identical bits and improving clock recovery, synchronization, and spectral efficiency.

Observation 8	Scrambling and descrambling are feasible, requiring only a small LFSR and one XOR per bit, leading to negligible hardware complexity at both transmitter and receiver.

Observation 9	Unlike for Msg1 transmissions where there is need to indicate the number of time domain resources/occasions that can be randomly selected by a device, a dedicated time-domain resource is provided for Msg3 transmission.

Observation 10	The device can determine the frequency and time resources from different messages (e.g SIB, Msg2) for different D2R transmissions (e.g., Msg1, Msg3 etc).

Observation 11	The device is allowed to determine the frequency and time resources for MSG1 transmission for DO-A traffic.

Observation 12	For Device 2b/C in Rel-20, frequency domain resource allocation can be provided explicitly in the D2R scheduling information.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To improve efficiency and reliability, one modulation scheme between OOK, BPSK and MSK needs to be down-selected.

Proposal 2	In addition to the mother code rate of 1/3,

	support a coding rate of 1/2 for Device 2b/C.

	not support a coding rate of 1/4 for Device 2b/C.

	not support interleaver or bit collection

Proposal 3	The necessity of block-level repetition greater than two should be discussed based on coverage evaluation results and the coverage targets for feasible A-IoT use cases.

Proposal 4	D2R scrambling for PRDCH is necessary and feasible.

Proposal 5	Support SSB for device 2b and C

Proposal 6	Study the enhancements of preamble and/or midamble either in number or length to correct large CFO and SFO.

Regarding the D2R preamble and midambles, the following agreements have been reached.

Proposal 7	Determine if X1 = 2 is sufficient to meet the capacity and latency that outdoor use cases demand. The necessity of further enhancements to X1 = 2 for Msg1 should be motivated by the proponents from capacity and latency perspectives potentially using evaluations.

Proposal 8	From device perspective, X3 should be 1.

Proposal 9	It is not necessary to support baseband small FS for Device 2b and Device C in Rel-20.

Proposal 10	CDMA may not be needed due to high complexity unless the service requirement cannot be met.

Proposal 11	For Device 2b/C in Rel-20, there should be flexibility to provide different D2R scheduling parameters for different FDM-ed devices

# References

RP-251840  “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108, June 2025.

TS 38.291, “Technical Specification Group Radio Access Network; Ambient IoT Physical layer”

TR 38.769 V19.0.0, “Study on Solutions for Ambient IoT (Internet of Things)”, 3GPP.

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, 3GPP

TR 38.869 V18.0.0, “Study on low-power Wake-up Signal and Receiver for NR (Release 18)”, 3GPP, January 2024.

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), RAN#106, December 2024.

TS 22.369, “Service requirements for Ambient power-enabled IoT”, 3GPP, June 2024.

R1-2508514, “Other procedures of A-IoT”