3GPP TSG RAN WG1 #123			R1-2508908

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	11.1

Source: 	LG Electronics

Title: 	Views on overall design and techniques for 6GR air interface

Document for:	Discussion and decision

1. Introduction

In this contribution, LGE’s views on 6G physical layer design are presented. Following the guideline from RAN1 chair to avoid repeating same proposals across different agenda, this contribution focus on the following topics:

- Synchronization and Initial Access

- Bandwidth

- BWP framework/operation

- Spectrum utilization/operations

- PHY Control Channel (DL/UL)

- PHY Data Channel (HARQ and scheduling)

- MIMO

- Multi-RAT Spectrum Sharing

- NTN

- ISAC

2. Discussion

Synchronization and Initial Access

During the RAN1#122 meeting, discussions were held on the 6GR sync signal structure and associated periodicity. In RAN1#122 and RAN1#122 bis meeting, agreements for the 6GR sync signal structure were reached as follows:

Agreement (RAN1#122, 2025.08) - Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

Agreement (RAN1#122bis, 2025.10) - High-level aspects to consider for the 6GR sync signal structure include, but not limited to - Sync raster design - Spectrum allocation - smallest maximum supported RF and BB UE BW without spectrum aggregation - mobile broadband service requirements as high priority - Energy efficiency for both BS and UE - Detection/tracking performance, latency, and complexity - Including initial cell search - Coverage target - Common design for diverse device types - Consideration of the supported deployment - Consideration on whether the single sync signal structure is sufficient - Note: Aspects impacting on the periodicity is to be discussed under AI11.5

Prior to the detailed discussions under the initial access agenda item, which will commence from RAN1#124, we would like to briefly present our views on the synchronization structure and initial access aspects relevant to this topic.

Synchronization

General Aspects for synchronization

As 6G systems aim to support more diverse deployment scenarios, frequency ranges, and UE capabilities, the synchronization signal and channel design must evolve to provide low-latency access, robust synchronization, beam tracking, and high energy efficiency —while maintaining compatibility with both TN and NTN.

1. Initial Beam Management and Downlink/Uplink Coverage

In 6G, initial beam management will remain essential for access procedures. Techniques such as the association of SSB with RACH, as in NR, should be retained and extended to support: Enhanced power control of Msg1 (RACH) based on received SSB power, Beam failure recovery, and Dynamic uplink beam tracking across multiple PRACH attempts

Moreover, high-frequency operation increases uplink path loss, making uplink coverage enhancement vital. To address this, time-domain repetition-based schemes (e.g., for RACH, PUSCH, and PUCCH) should be considered from the beginning of system design.

In NTN environments, enhancing downlink (DL) coverage is essential due to long-distance propagation and limited signal availability, and techniques like repetitive transmission of synchronization signals can help address this. Increasing the periodicity of DL sync signals supports both network energy saving and NTN operation, where frequent transmissions are often impractical. However, this may reduce the number of detection samples and degrade detection probability, so complementary methods such as advanced signal processing or adaptive repetition strategies should be considered to maintain reliable access.

2. Adaptive and Energy-Efficient Synchronization

Energy efficiency is a core design objective in 6G. Therefore, dynamic control of downlink signal periodicity must be supported, including: On-demand SSB and SIB transmission, and Configurable paging cycles.

However, longer SSB periodicities, while energy-efficient, may lead to increased access latency and higher UE detection complexity, especially during initial access. Thus, synchronization design should minimize latency without imposing excessive receiver complexity.

3. Unified TN/NTN Synchronization Support

6G targets seamless operation over both TN and NTN. Synchronization signal and RACH designs must accommodate: Extended periodicity and repetition of SSBs and broadcast channels, Flexible PRACH preamble formats suitable for high-delay NTN environments, and multi-beam synchronization reception in NTN scenarios.

4. Multi-TRP and Cell Edge Enhancement

To improve reliability at the cell edge, multi-TRP operation will be crucial. Synchronization signals/channels should: Support simultaneous detection from multiple TRPs, enable joint initial access and beam management, and facilitate fast and seamless mobility across TRPs.

5. Support for Diverse UE Capabilities

6G UEs will vary significantly in their bandwidth and processing capabilities. Synchronization design should allow: Operation within narrow bandwidths (e.g., for low-tier UE), Configuration of initial BWP (Bandwidth Parts) for both downlink and uplink, and Sync raster definition to support devices with limited capability.

6. Synchronization Signal for Tracking and Measurement

Beyond cell detection, synchronization signals must also serve time and frequency tracking purposes. For improved tracking accuracy: Synchronization signals/channels should share subcarrier spacing with PDSCH/PUSCH, and Tracking Reference Signals (TRS) or other synchronization signals should be considered, particularly for UEs in IDLE/Inactive states.

7. Multi-RAT and Multi-Carrier Operation

6G UEs are expected to operate across multiple carriers and to operate multi-RATs in same carrier. Synchronization mechanisms must enable: Inter-carrier and inter-RAT cell search, Cross-carrier system information acquisition, and Spectrum sharing with minimal increase in UE-side complexity.

8. Duplexing Flexibility: SBFD-Aware Synchronization

NR introduced Subband Full Duplex (SBFD) support in later releases. In 6G, duplexing flexibility should be considered from the start, with synchronization design that enables: Contiguous and non-contiguous CORESET resource mapping, Explicit indication of SBFD/non-SBFD transitions for guard period management, and Duplex-aware synchronization scheduling across time/frequency domains.

Considerations for Synchronization Signal Periodicity and UE Detection Complexity

It is necessary to discuss the periodicity of synchronization signals transmitted by the gNB, the associated UE detection complexity, and the design of synchronization signals and PBCH.

As previously discussed, longer periodicity may be required not only for gNB energy efficiency but also to support beam switching in NTN scenarios. In 5G NR, the system was already designed to allow longer periodicity for synchronization signal transmission, and the gNB can indicate the actual transmission periodicity to the UE via signaling. The key difference lies in the default periodicity assumed by the UE. In 6G RAN, it is necessary to support an increased default periodicity assumed by the UE.

However, increasing the default periodicity beyond the 20 ms used in 5G NR requires careful evaluation of its impact on UE performance. Specifically, the SS/PBCH detection performance, latency, complexity, and accuracy of time/frequency tracking may degrade if the UE assumes a longer default periodicity before receiving system information. To compensate for this, one possible approach is to enhance the detection performance during each attempt by allocating more OFDM symbols to PSS, SSS, and PBCH. This requires further study to determine how much additional resource is needed and whether the trade-off is acceptable.

It is proposed to study the impact of increasing the default periodicity assumed by the UE for synchronization signal detection. This includes evaluating the potential benefits—such as improved network energy efficiency and support for beam switching in NTN environments—as well as the implications for UE performance, including SS/PBCH detection accuracy, latency, and complexity.

In parallel, to compensate for the increased periodicity, it is also proposed to study the number of OFDM symbols allocated to PSS, SSS, and PBCH. This aims to enhance detection performance during each attempt, and further analysis is needed to determine the required resource overhead and trade-offs.

Initial Access

In the NR system, RAN1 has studied several aspects of the initial access procedure, system information acquisition, random access procedure, paging, and measurement, particularly focusing on physical layer behaviors. Similar functionalities are expected to be essential in 6G systems. However, given the expected diversity of UE capabilities and service requirements in 6G, further enhancements and new physical layer design considerations are needed.

1. Bandwidth Configuration for Initial Access

In NR, the bandwidth used for initial access (including system information acquisition and random access procedure) typically corresponds to the CORESET bandwidth or the bandwidth part (BWP) indicated by system information. In 6G, UEs with a wide range of capabilities will need to access the network efficiently. To accommodate these differences, the bandwidth used during initial access may need to be configurable depending on UE type, enabling more flexible and scalable access mechanisms.

2. 4-step and 2-step RACH Procedure

NR supports two types of RACH procedures: the 4-step RACH designed initially for robust access, and the 2-step RACH introduced in Rel-16 for low-latency and small data transmissions. These procedures use distinct PRACH resources or RACH occasions to enable separation and efficient operation. In 6G, both procedures are expected to be relevant — 4-step RACH for extended uplink coverage and 2-step RACH for fast access in small cells. Both 4-step and 2-step RACH procedures can be studied in the early phase of 6GR study

3. Uplink Beam Selection during Random Access

In NR, UEs select an appropriate Rx beam based on SSB reception during initial access. However, for uplink transmission, selecting an optimal Tx beam is more challenging. NR relies on reusing the Tx beam used during PRACH Msg1 for subsequent Msg3 transmission. Although there were proposals for enabling multiple PRACH transmissions using different beams and selecting the best-performing one, such mechanisms were not standardized. In 6G, uplink coverage enhancement is critical. Therefore, mechanisms for uplink beam selection (e.g., via Msg1/Msg3) should be considered from the early stage of system design.

4. Time/Frequency Tracking for UEs in Idle/Inactive states

In NR, UEs in Idle/Inactive states use SSBs for time and frequency tracking. However, SSBs have relatively long transmission periods and limited bandwidth. This imposes additional wake-up time and energy burden on UEs, especially during paging occasions. NR later introduced using Tracking Reference Signals (TRS) for such UEs. In 6G, to support network energy saving, SSB transmission may become even less frequent or on-demand-based. Thus, it is necessary to design additional reference signals — beyond SSB — suitable for time/frequency tracking by UEs in Idle/Inactive states.

5. Integration of L1 and L3 Measurement

Measurement in NR initially relied on Layer 3 (L3) cell-level and beam-level reports. Subsequently, Layer 1 (L1) measurement was introduced, providing faster reporting and improved responsiveness. For 6G, it is essential to consider L1 and L3 measurement schemes from the beginning and investigate integration methods to achieve both accuracy and low latency in mobility management and beam management.

6. Energy-Efficient Procedure

Energy efficiency is a key focus area in 6G, and on-demand system information request mechanisms—introduced in 5G NR and extended to SIB1 in later releases—should be further studied in 6G, including PRACH-based request mechanisms for SIB1 and other SIBs. In addition to network-side efficiency, UE power saving is equally important, especially in the context of paging reception. To this end, PEI mechanisms based on DCI or LP-WUS should be investigated to enable low-power paging in 6G.

Study on initial access procedure, random access procedure, paging, and measurement with physical layer aspects, including the following:

  - Bandwidth configuration for the initial access,

  - RACH procedures (i.e., 4-step RACH, 2-step RACH),

  - Uplink transmission (UL Tx) beam selection via Msg1/Msg3,

  - Time/frequency tracking reference signal for UEs in IDLE/Inactive state,

  - Integration of L1/L3 measurements,

  - PRACH-based request mechanism for SIB1 and other SIBs in 6GR, PEI mechanisms based on DCI or LP-WUS for 6G paging reception

Bandwidth

During the RAN1#122 meeting, discussions were held on the minimum spectrum allocation and the smallest maximum supported RF and baseband UE bandwidth. Agreements were reached as follows -.

Agreement (RAN1#122, 2025.08) - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. Agreement (RAN1#122, 2025.08) - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Considerations for Minimum Channel Bandwidth

In Rel-18 5G Advanced, studies and standardization efforts were conducted to enable network operation even with channel bandwidth narrower than 5 MHz. Similarly, in 6G, there is requirement to support network operation not only at 5 MHz but also at 3 MHz channel bandwidth. These bandwidths are closely related to the design and performance of synchronization signals and PBCH, as well as the sync raster determination.

One possible approach is to design the SS/PBCH to meet the required performance at 3 MHz bandwidth. Alternatively, it may be designed to meet performance requirements at 5 MHz, while still being operable at 3 MHz with slightly reduced performance.

However, if SS/PBCH is designed specifically for bandwidths narrower than 3 MHz or 5 MHz, the sync raster for a 3 MHz channel bandwidth may become overly dense. If such narrow bandwidths are only used in specific frequency ranges, it may be feasible to define dedicated sync raster configurations only for those specific frequency locations.

On the other hand, when using a 5 MHz channel bandwidth, the SS/PBCH bandwidth is smaller than the channel bandwidth, allowing for a sparser sync raster configuration. This can significantly reduce the complexity of SS/PBCH search operations for the UE in the frequency domain.

After receiving SS/PBCH, if the UE intends to receive common channels, the bandwidth for those channels can be indicated within the PBCH. Based on this indication (e.g., 3 MHz, 5 MHz, 10 MHz, 20 MHz), the UE can determine the bandwidth for PDCCH and PDSCH reception, enabling proper operation without major issues.

Therefore, if the minimum channel bandwidth for FDD is defined as 3 MHz or 5 MHz, it is recommended to design common channel/signal such as SS/PBCH using 15 kHz SCS, targeting 5 MHz operation, and verify its operability at 3 MHz.

Similarly, if the minimum channel bandwidth for TDD is defined as 10 MHz, SS/PBCH should be designed using 30 kHz SCS and operate within the 10 MHz bandwidth.

Consider channel bandwidth (CBW) for narrowband operation (e.g., 3 MHz, 5MHz). It is recommended to design common channel/signal such as SS/PBCH using 15 kHz SCS, targeting 5 MHz operation, and verify its operability at 3 MHz.

Bandwidth for low-tier device type and normal device type

During the RAN1#122 meeting, discussions were held regarding the smallest maximum supported bandwidth for UEs, particularly for at least one low-tier device types. Further clarification is needed on whether such low-tier devices are intended to operate only in FDD bands, or also in higher-frequency TDD bands. For instance, low-tier devices may be expected to operate in FDD bands around 2 GHz or below 1 GHz, but may not be suitable for operation in higher-frequency TDD bands such as around 4 GHz. In this context, the smallest maximum supported RF and baseband UE bandwidth without spectrum aggregation may differ from the maximum bandwidth defined specifically for low-tier device types.

In addition, for normal device types, the maximum bandwidth requirements may also vary depending on the operating frequency band. For example, the maximum supported bandwidth is expected to differ across bands such as sub6GHz, around 7 GHz, around 15 GHz, and the mmWave range between 24.25 GHz and 52.6 GHz. Therefore, it is proposed to discuss and identify the maximum bandwidth requirements per frequency band, considering the device type (i.e., low-tier device type, normal device type).

Identify the maximum bandwidth requirements per frequency band (e.g. sub 6GHz for FDD/TDD, around 7GHz, around 15GHz, and between 24.25GHz – 52.6GHz) that UEs should support, taking into account device type (i.e., low-tier device type, normal device type).

BWP framework/operation

In RAN1#122, discussions were held on the Bandwidth Part (BWP) framework, and it was agreed to study and identify the lessons learned from the 5G NR BWP framework.

Agreement - Study and identify the lessons learned from NR BWP framework

In the beginning of 5G NR, BWP concept was originally motivated and designed to support various service types and/or UE power saving. However, the BWP framework and operation currently specified in 5G NR is over-designed (i.e., too flexible and complicated) from the perspective of NW/UE implementation. Moreover, the framework is limited in terms of assumptions and signaling capabilities in the following aspects.

- Configurations per BWP is unnecessarily heavy (e.g. consuming large signaling overhead due to large amount of RRC parameter configurations for many features)

- BWP switching latency is too large (e.g. spending a few msec due to the UE processing time required for simultaneous switching of the configurations for many features)

- DL/UL BWP combination is somewhat restrictive as it assumes that the center frequency of DL and UL BWPs must be same for unpaired spectrum, and it specifies that only contiguous bandwidth can be indicated.

Regarding this BWP aspect, at the end of RAN1#122bis meeting, the following “Proposal observation 8.1c” was suggested by the moderator as starting point for the relevant discussions in RAN1#123.

Proposed observation 8.1c: - The lessons learned from NR BWP framework include, but not limited to - excessive BWP-specific BB/RF configuration parameters, which leads to UE long BWP switch latency - SCS switching under BWP framework is complicated - Some scenarios (e.g. non-overlapped BWPs) where DCI-based BWP switching can have reliability issue - lack of early RAN4 involvement, which caused sub-optimal design

On the four sub-bullets in the above observation, the third one needs further discussion on whether to include it as one of the lesson points. In our view, there is no reliability issue with DCI-based BWP switching (even in case of switching between non-overlapped BWPs) since whether UE missed the BWP switching DCI can be confirmed by detecting the scheduled PUCCH/PUSCH on the new BWP.

For example, if the UE missed the BWP switching DCI, the PUCCH/PUSCH scheduled on the new BWP by the DCI wouldn’t be detected by gNB, then the gNB assumes the UE didn’t switch its active BWP yet (i.e., still stay in the old BWP), and it would transmit PDCCH toward the UE on the old BWP. Therefore, there is no misalignment between UE and gNB on the active BWP.

With confirmation based on the scheduled PUCCH/PUSCH detection, the switching of active BWP is delayed until the UE transmits the PUCCH/PUSCH. However, it is purely due to long BWP switch latency defined in NR which is related to the first sub-bullet (and would be reduced in 6GR anyway), not the matter of reliability. For above reasons, it is better not to include the third sub-bullet in the lessons from NR BWP.

Need to remove the following sub-bullet related to reliability issue from the “Proposed observation 8.1c” in [2] since whether UE missed the BWP switching DCI can be confirmed by detecting the PUCCH/PUSCH scheduled on the new BWP.

  - Some scenarios (e.g. non-overlapped BWPs) where DCI-based BWP switching can have reliability issue

Proposed observation 8.1c: - The lessons learned from NR BWP framework include, but not limited to - excessive BWP-specific BB/RF configuration parameters, which leads to UE long BWP switch latency - SCS switching under BWP framework is complicated - Some scenarios (e.g. non-overlapped BWPs) where DCI-based BWP switching can have reliability issue - lack of early RAN4 involvement, which caused sub-optimal design

Based on the above lessons learned from 5G NR, first of all, 6GR would require to minimize the per-BWP parameters/configurations for low BWP overhead, for example, by configuring only BW related parameters (e.g. frequency location, BW size) per BWP while configuring other features (e.g. CORESET, SS set, etc.) in cell level. Secondly, for fast BWP adaptation, 6GR would also need to reduce the BWP adaptation latency/complexity, for example, by reducing association with the features (e.g. CORESET, SS set, etc.), the number of SCSs, etc. Thirdly, it can be considered in 6GR to relax/extend DL/UL BWP combination for flexible BWP operation, for example, by allowing unaligned center frequencies (including flexible DL-UL pairing case) for DL BWP and UL BWP. Lastly, non-contiguous resource allocation may also be considered to support SBFD scenario or multi-carrier single cell scenario [1].

Consider lessons learned from 5G NR on the BWP framework/operation:

  - Configurations per BWP is unnecessarily heavy (e.g. consuming large signaling overhead due to RRC parameter configurations for many features)

  - BWP switching latency is too large (e.g. spending a few msec due to UE processing time required for simultaneous switching of many configurations)

  - DL/UL BWP combination is restrictive (e.g. center frequency of DL and UL BWPs must be same for unpaired spectrum, and only contiguous bandwidth is possible)

Study on 6G BWP framework/operation includes:

  - Minimization of per-BWP parameters/configurations

  - Reduction of BWP adaptation latency/complexity

  - Relaxation/extension of DL/UL BWP combination

  - Non-contiguous frequency resource allocation

Spectrum utilization/operations

While 5G NR standardization was designed to support various spectrum utilization/operation scenarios, several limitations or drawbacks have been observed during the deployments of practical networks based on 5G NR, which may lead to necessity of enhancements for 6G standards.

Accordingly, lessons from NR spectrum utilization and aggregation framework were discussed in RAN1#122bis meeting, and the following was drafted for further discussion [2].

Proposed observation 9.1b: - The lessons learned from NR spectrum utilization and aggregation framework include, but not limited to - CA has been a beneficial feature in previous generations - Not all functionalities are available from initial release - Operating scenarios of CA and DC have some overlap - Some functionalities are supported only on Pcell - Slow and complex activation of additional carrier - Inefficiency from coupling DL and UL carriers for a cell - Utilizing fragmented spectrum is not considered well - Features (such as HARQ) defined per carrier leads to sub-optimal performance - Signalling/configuration overhead and UE processing complexity of PHY channels due to per CC constraint - limited applicable scenario of SSB adaptation for Scell - Late introduction of UL TX switching leads to restricted applicability/performance

While we agree on the many of the points listed above, some of the points may need further discussion as follows.

  - Some functionalities are supported only on Pcell

It is not clear which functionalities supported only on Pcell cause drawback and require study for potential enhancements. More discussion on practical details seems to be needed.

  - Features (such as HARQ) defined per carrier leads to sub-optimal performance

In the context of carrier aggregation in NR standards, it is unclear whether relaxing the confinement of HARQ operation within each carrier will provide better performance and more discussion may be necessary. On the other hand, it seems natural to consider sharing data processing procedure (e.g. HARQ process, TB allocation, etc.) over multiple carriers for the study of “multiple carriers over single cell operation” for 6G.

  - Late introduction of UL TX switching leads to restricted applicability/performance

While we agree UL TX switching in NR should be one of the consideration points for spectrum aggregation in 6G from the initial phase, there are other similar points in NR spectrum aggregation to be considered as the initial framework for 6G spectrum aggregation, such as, SUL, low-band CA, etc. Therefore, it may be better to consider more general observation which helps to study generalized spectrum aggregation framework for 6G.

Based on the discussion on the lessons from NR so far, it is reasonable that 6GR study at least include enhancements regarding fast SCell activation, flexible DL-UL carrier coupling, SSB adaptation and multi carrier operation for fractional bands.

Furthermore, we think that the multi-carrier scenarios can be studied for UEs in RRC_IDLE/INACTIVE as well as in RRC_CONNECTED. For example, a cell-defining SSB can be periodically transmitted on the sparse sync raster as a primary carrier while on-demand SSB is irregularly provided on off-sync raster as a secondary carrier for better spectrum utilization on wide channel bandwidth with diverse device types. Furthermore, UE in multi-carrier scenarios can quickly move to the secondary carrier during initial access for reduced access delay compared to one in NR. Therefore, it is necessary to study framework for common channel design/procedure, idle measurements and initial access for multi-carrier scenarios. including multi-carrier single cell scenario.

Among the topics listed in “Proposed observation 9.1b [R1-2508141]” as lessons learned from NR spectrum utilization and aggregation framework, the following topics may need further verification

  - Some functionalities are supported only on Pcell

  - Features (such as HARQ) defined per carrier leads to sub-optimal performance

  - Late introduction of UL TX switching leads to restricted applicability/performance

Consider the following as the lessons learned from NR spectrum utilization and aggregation framework

  - Diverging functionalities for a similar purpose of cell/carrier adaptation/switching, such as SCell activation/deactivation, UL TX switching, LB-CA, SUL, etc. cause complex implementation at both network side and UE side

  - Multi-cell deployment over multi-carriers in NR demands cell-specific always-on overhead signals on each carrier.

Study for spectrum utilization and aggregation framework for 6GR should include the following

  - Unified framework for cell/carrier adaptation/switching

  - Fast SCell activation/deactivation procedure in 6GR including fast CSI acquisition

  - Flexible band/carrier coupling between DL and UL for both FDD and TDD in 6GR.

  - Efficient/effective/practical features of carrier ON/OFF, including SSB adaptation

  - Efficient support of aggregating large number of (fractional) carriers for both intra-band and inter-band cases in 6GR.

  - Common channel design/procedure, idle measurements and initial access for multi-carrier scenarios including multi-carrier single cell scenario.

PHY Control Channel

DL control structure

In 5G NR (as well as 4G LTE), PDCCH structure for DL control signaling has several limitations or pain points (to support the operations/requirements required for 6G system) as below, in terms of DL control resource management, UE blind decoding, and DCI formats for the composition/transmission and the monitoring of PDCCH.

- DL resources and parameters used for the composition/transmission of PDCCH are (semi-)statically provided (via CORESET configuration)

- UE blind decoding is performed as many times as the amount of PDCCH candidates given in (semi-)static manner (via SS set configuration)

- DCI formats for PDCCH have non-separate structure, and the contents in each DCI are used for one-time scheduling

In 6G system, firstly, for better cooperation with various/dynamic situations with DL resource adaptation such as MR-SS, NES, AI/ML, and SBFD operations, dynamic adaptation of DL control resources/parameters would need to be considered (e.g. adaptation of CORESET region for PDCCH). Besides, adaptive reduction of UE blind decoding on DL control channel would be required in the 6G for more effective saving of UE power consumed mainly for monitoring of the DL control channel (e.g. indication of PDCCH BD parameter by LP-WUS). Furthermore, for better support of extended requirements and various UE types for 6G such as HRLLC, massive connectivity, immersive communication, adaptive reduction of DL control overhead/resources (e.g. PDSCH/PUSCH scheduling based on two-part DCI) and unified DL control resource structure accommodating multiple UE types (e.g. various BW capabilities) may also need to be considered.

Study on 6G DL control channel includes:

  - Dynamic adaptation of DL control resources/parameters

  - Adaptive reduction of UE blind decoding and DL control overhead

  - Unified DL control resource structure accommodating multiple UE types

UL control structure

In 5G NR (and/or 4G LTE), PUCCH structure for UL control signaling also has some limitations or pain points (to support the 6G operations/requirements) as below, in the aspects of initial PUCCH resource set, UCI multiplexing procedure, and UCI modulation on PUCCH.

- Initial PUCCH resource set (e.g. PRB offset & starting symbol for PUCCH, number of PUCCH resources in the set) is statically given via SIB

- UCI multiplexing is complicated in merging overlapped PUCCHs and determining container PUCCH, and inflexible since PUCCH group and UL sub-slot structure affecting PUCCH resource composition and multiplexing is semi-static.

- Modulation scheme applied for UCI transmission on PUCCH is limited to BPSK/QPSK

In 6G system, similarly as in DL control case, flexible provision of initial UL control channel resource set and parameters (e.g. via RAR as well as SIB) would need to be considered for efficient cooperation with various/dynamic operations with UL resource adaptation such as MR-SS, NES, AI/ML, SBFD. In addition, simplification and adaptation on UCI multiplexing procedure based on simplified PUCCH resource merging/determination and dynamic PUCCH group/UL sub-slot structure, would be required in the 6G for ensuring relaxed UCI multiplexing on PUCCH/PUSCH and corresponding UE processing/implementation. Furthermore, for improving UL resource efficiency in the mapping/transmission of UCI (expected to have larger payload size in the 6G) on PUCCH, applying higher order modulation (e.g. 16QAM) for UCI on PUCCH may also need to be considered.

Study on 6G UL control channel includes:

  - Adaptation of UL control resource region and offloading

  - Simplified UCI multiplexing structure

  - Higher order modulation for UL control channel

PHY Data Channel

In both LTE and NR, multi-carrier scheduling and carrier aggregation has evolved to enable faster data rates and better network capacity. As higher performance demands are expected in 6G, efficient frequency resource utilization will become even more crucial. Moreover, as discussed above, fragmented 6G spectrum motivates enhanced usage of multi-carriers.

To enhance multi-carrier operation, multiple carriers can be combined to form a frequency resource unit, allowing cell-based operations to be implemented using this combined resource. For instance, advanced methods such as a TB mapping across carriers and cross-carrier can be explored for 6G to utilize multi-carriers more efficiently. It should be noted that TB mapping in NR is confined to a single carrier, potentially constraining throughput and diversity gain if the carrier bandwidth is insufficient. To support this, it is important to design the unit for managing frequency resources and scheduling with consideration of a multi-carrier structure, which can serve as an enhancement of the Bandwidth Part (BWP) concept in 5G NR.

Study scheduling enhancements in 6G considering enhanced multi-carrier operation, including TB mapping across multiple carriers

In 5G NR, the slot structure has been a fundamental limitation on scheduling PxSCH. While various features—such as TBoMS PUSCH, Type-B PUSCH repetition, and slot aggregation—have been introduced up to Release 19 to address this, their existence itself highlights the persistent need to overcome the slot boundary.

Consequently, the design of 6GR should aim to support cross-slot transmissions from Day 1. It can be expected that allowing transmissions to cross slot boundaries bring benefits such as reduced DMRS overhead and improve resource utilization efficiency. The benefits of this flexibility are expected to justify the additional UE complexity it may require.

Study mechanisms to support PDSCH/PUSCH transmissions crossing slot boundaries.

Similarly, 5G NR introduced diverse repetition schemes across several releases to meet various target requirements such as coverage enhancement, low latency, reliability, and unlicensed band operation. These features, including TBoMS PUSCH, Type-B PUSCH repetition, and multi-PUSCH scheduling, were added incrementally, which increased system complexity and made commercialization more challenging. Although these schemes share a lot of similarities, their minor purpose-specific differences make them difficult to use simultaneously. Furthermore, the current repetition/multi-slot scheduling is just a simple concatenation of single-slot transmissions, which leads to unnecessary overhead from excessive DMRS symbols. To alleviate these issues, a unified scheduling framework requires the integration of multi-resource scheduling and cross-slot boundary scheduling.

Study a unified scheduling approach for repetition, covering existing schemes such as TBoMS PUSCH, Type-B PUSCH repetition, and multi-PUSCH scheduling

In 5G NR, as companies reported in RAN1#122, FDRA type 0 and type 1 are generally used for DL and FDRA type 1 is primarily used for UL to ensure contiguous scheduling and maintain a low uplink PAPR. However, considering scenarios such as SBFD and multi-carrier operation within as single cell (MCSC), there may be a case where the available frequency resources for a 6G UE can be fragmented. To utilize these resources efficiently, non-contiguous resource allocation methods, such as FDRA type 0 in 5G, should be considered. Given that, adopting non-contiguous allocation can be useful to reduce scheduling overhead and improve resource configuration flexibility.

Study the applicability of 5G FDRA type 0 and type 1 as potential baselines for 6G FDRA.

In LTE and NR, advancements such as multi-carrier scheduling and carrier aggregation (CA) have enabled higher data rates and more flexible network configurations. In 6GR, the need for efficient utilization of fragmented carriers is gaining attention, for reducing signaling overhead and improving throughput efficiency. MCSC and enhanced CA are being considered as approaches to efficiently utilize these fragmented carriers and maximize spectrum utilization and improve aggregation efficiency. If the spectrum utilization and aggregation techniques are newly introduced in 6GR, the design of HARQ operations should be carefully considered to accommodate various scenarios with simplified solutions and to ensure better performance. For example, determining appropriate carrier and timing for HARQ-ACK transmission in multi-carrier operation should be discussed carefully to ensure low latency and high reliability. From these perspectives, 6GR study should include modified and enhanced HARQ processes in multi-carrier scenarios. In addition, it should be pointed out that the UCI multiplexing rules in NR were numerous and highly complex. To reduce complexity while preserving the accuracy of feedback signaling, it will be necessary to define enhanced and simplified HARQ multiplexing rules. Furthermore, effective multiplexing techniques might be required to support the transmission of large HARQ-ACK payloads with reduced latency and overhead. For the study of HARQ-ACK multiplexing, aspects such as higher efficiency, better resource utilization, and improved reliability across diverse scenarios should be carefully considered in comparison with NR.

Study the HARQ-ACK process enhancements in 6GR, including considering of enhanced multi-carrier operation, aiming for simplification and efficiency.

MIMO

General considerations

MIMO has played a key role in 5G NR system to improve spectral efficiency, reliability and capacity for both NW and UE. In 6G, we believe that MIMO will still be a key component of physical layer operation. On the other hand, due to MIMO enhancements in every NR release, we observe that there is not much room to improve “peak” system/UE performance by introducing new MIMO technologies. Rather, 6G MIMO should at least consider the following aspects:

- Applicability and forward-compatibility to AI/ML based CSI/beam operations

- Network and UE energy saving with consideration of MTRP/STxMP/large antenna array transmission

- Unified and simplified framework for CSI measurement and reporting with low configuration overhead

- MIMO channel characteristics of frequency ranges being considered for 6G

- Scalability to large number of antennas/TRPs/panels with consideration of signaling and RS overhead

6G MIMO should aim to harmonize with other 6G key features such as AI/ML and NES/UES. It is also important to extend the scale of massive MIMO and to reduce signaling/RS overhead compared to 5G NR MIMO.

CSI/beam management

NR introduced structured and flexible CSI framework which supports various MIMO operations, including beam management (BM) operations. The 5G NR CSI/BM, however, requires quite a large signaling overhead in terms of RRC configuration, CSI-RS transmission, and CSI reporting in some cases. In this regard, it is valuable to focus on how to reduce signaling overhead rather than introducing new CSI/BM mode for 6G.

CSI measurement and reporting mechanisms in previous generations rely on network’s control of the measurement and reporting. Since network has difficulty in expecting how fast each UE’s DL channel changes, network-initiated CSI report may cause unnecessarily large CSI-RS overhead and CSI report overhead if network sets the measurement/reporting periodicity too conservatively. If it is set in a naïve way, it may cause too large CSI report latency that may be outdated and cause CSI mismatch. In this sense, UE-initiated reporting mechanism was studied and introduced in 5G NR Rel-19. In 6G, we think that it is valuable to use this mode of operation from the beginning. UE-initiated reporting can be used not only for beam report but for general CSI report in 6G. Also, it is desirable to have unified design not only for UE-initiated CSI/beam reporting but also for any event based reporting such as BFR and LTM, which reduces configuration overhead and simplifies configuration.

In addition, NR supports diverse CSI report modes but it has a large configuration overhead and complicated triggering mechanism. For example, network needs to configure multiple CSI report configurations for different combinations of P/SP/AP CSI RS and P/SP/AP CSI report on PUCCH/PUSCH and also there are various triggering mechanisms such as MAC-CE based activation, DCI based activation and DCI based triggering. This overdesign makes NR specification more complicated than it needs to be. Therefore, simplified and unified CSI framework for various CSI measurement and report configurations should be supported in 6G.

For CSI codebook, 5G NR CSI codebook can be a starting point for 6G including FD/TD compression codebook for reducing CSI payload size. AI/ML based CSI should also be taken into account for 6G CSI codebook. For example, AI/ML based CSI compression can reduce feedback overhead with sufficient accuracy, and AI/ML based CSI prediction can help network to maintain good link quality without frequent reports consuming UL transmission power and UL resources. In addition, MIMO channel characteristics in 6G bands should be taken into account in the CSI codebook design. For example, DFT basis for codebook may no longer appropriate for near field since linear phase increase among antenna ports is not valid assumption.

Study CSI/BM taking into account the extension of UEI report and AI/ML prediction/compression, 6G band characteristic (e.g. near-field) and low signaling/reporting overhead.

MIMO transmission schemes

In NR, various DL/UL transmission schemes are supported including sTRP and mTRP transmission schemes, STxMP transmission schemes and DL precoder cycling with PRB bundling. However, some transmission schemes can only be applied under limited conditions so that it limits the attractiveness of deployment of these features in real system.

Specifically, 5G mTRP/STxMP schemes mainly consider homogeneous implementation of TRPs or panels. For example, multiple TRPs/panels should be configured with the same number of ports and UL panels should be configured with the same full power mode and the same antenna coherence type, which limiting the practical usage of mTRP/STxMP schemes. In our view, 6G should support various TRP/UE implementations including not only homogeneous implementation but also heterogeneous implementation. In addition, some schemes are supported only for specific physical channels/signals which would limit the usefulness of the schemes, e.g. STxMP applicable to specific UL channel types. In 6G, more than 2 TRPs/panels can also be considered for cell edge UEs and UEs with 3 or more panels like vehicles.

NR supports OL MIMO scheme with precoder cycling but it is limited to PDSCH. In our view, UL OL MIMO is also important considering high speed UE and vehicle to avoid frequent SRS transmission with a large UL resource overhead for UL MIMO channel estimation.

In general, 6G Day-1 should strive for a limited number of schemes but the schemes should be applicable to all relevant cases. In addition, the schemes should be supported under unified signaling structure as much as possible.

Study DL/UL MIMO schemes by considering various 6G MIMO application scenarios and 6G device implementation ways, e.g. heterogeneous panels/TRPs. Strive for a limited number of schemes but applicable to all relevant cases.

DL/UL reference Signal

In NR, up to 128 CSI-RS ports are supported considering large antenna array and up to 24 DMRS ports are supported by taking into account large number of MU-MIMO UEs. To improve spectral efficiency in FRs being considered in 6G, these numbers should be a starting point of study. On top of them, more than 128 CSI-RS ports and more than 24 DMRS ports can be considered to achieve more MIMO gain.

Meanwhile, in order to reduce RS overhead, frequency/time/spatial domain RS overhead reduction should be studied as well. For example, UE only measures partial CSI-RS ports but predicts full CSI-RS ports with AI/ML based approach. Also, for DMRS overhead reduction, some DMRS REs can be overlapped with PDSCH RE and cross interference between DMRS and PDSCH can be suppressed by AI/ML based cancellation/suppression technique.

Study ways to support larger numbers of CSI-RS/DMRS ports and RS overhead reduction techniques including AI/ML based techniques.

Multi-RAT Spectrum Sharing (MRSS)

Multi-RAT spectrum sharing (MRSS) is expected to play a crucial role in enabling a smooth transition from 5G to 6G. Given the limited availability of new low-frequency bands for 6G and the expected requirement for wide-area coverage, efficient spectrum reuse may help address both coverage and deployment cost challenges. Reusing existing 5G sites and radio nodes for 6G may also offer advantages in terms of infrastructure efficiency.

For the study of MRSS operation, the following FL proposal was discussed in the last meeting [2]. Based on the FL proposal, we provide our views on high-level aspects to consider for NR-6GR MRSS.

Proposal 6.2b: - High-level aspects to consider for NR-6GR MRSS include, but not limited to - UE/NW implementation complexity - Resource allocation coordination between NR-6GR - Including whether NR and 6GR TRP are always co-located or not - Radio resource utilization - Signalling overhead - Operating bands at least existing FR1 - Alignment in time/frequency resource (e.g., numerology, RB, slot, symbol, UL/DL direction in TDD operation) - Reliance on availability of specific NR functionalities Note: Focus on existing NR deployments (NW and UE)

Resource allocation coordination

One of the key aspects to consider for supporting MRSS between NR and 6GR in a TDD carrier is whether resource multiplexing can be tightly coordinated between the two systems. Without tight resource allocation coordination between NR and 6GR, it becomes challenging to support dynamic scheduling-based flexible resource sharing. As a result, semi-static TDM/FDM-based operation may need to be considered.

Scenarios that influence the feasibility of such coordination include, for example, whether the NR and 6GR TRPs are co-located, as captured in the FL proposal. Another important factor is whether NR and 6GR operate under the same vendor or across multiple vendors. In the case of multi-vendor operation, dynamic resource allocation coordination may be difficult to achieve between different vendors.

High-level aspects to consider for NR–6GR MRSS include resource allocation coordination between NR and 6GR, including whether their TRPs are co-located and whether they operate under the same or different vendors.

Signal/Channel overhead

In NR-6GR MRSS, performance impacts may arise if signal/channel transmissions are not carefully coordinated. To protect 6GR system throughput, it is necessary to reduce overhead caused by NR signal/channel transmissions in MRSS environments. Conversely, when operating both NR and 6GR systems, NR system throughput may be affected by overhead from 6GR signal/channel transmissions.

Considering this, it may be beneficial to reduce overhead caused by duplicate transmission of signals/channels that serve the same or similar purposes in both NR and 6GR. For example, synchronization signals or reference signals may be coordinated or shared to avoid redundancy.

High-level aspects to consider for NR–6GR MRSS include overhead from duplicated signal/channel transmissions in both NR and 6GR.

Radio resource utilization

For NR–6GR MRSS, it is necessary to study how radio resources are utilized between NR and 6GR. In this context, the following FL proposal was discussed in the previous meeting [2].

#### [Low]Proposal 6.3: - For the radio resource utilization for NR-6GR MRSS support, RAN1 to study the Pros/Cons of the following options - Opt0: Semi-static TDM/FDM between NR and 6GR - Opt1: NR signal sharing with 6GR - Opt2: Rate matching of 6GR signals/channels around NR signals/channels - Opt3: SDM between NR and 6GR

To support NR–6GR MRSS, rate matching for 6G transmissions is needed over the resources occupied by specific 5G signals/channels to protect their transmission. This includes signals that are periodically transmitted and not suitable for scheduling-based collision avoidance, such as 5G SSB, CSI-RS, CORESET (DL), and RO (UL). To address this issue in 6GR, NR rate-matching pattern can serve as a starting point. However, existing NR rate-matching patterns may not be optimal due to configuration overhead and limitations in handling long-period or RE-level signals. Therefore, tailored rate-matching patterns for 6G PDSCH and PUSCH should be considered.

In deployments where tight coordination between NR and 6GR is challenging (e.g., multi-vendor environments), semi-static TDM/FDM-based resource multiplexing may be necessary to ensure reliable operation and expand MRSS applicability.

To reduce overhead from duplicated signal/channel transmissions in both NR and 6GR, the feasibility of sharing NR signals/channels in 6GR should also be explored.

Additionally, scenarios where NR cells operate only within a subset of the 6GR spectrum should be studied. The system bandwidths of NR and 6GR cells may differ, with 6G expected to support wider bandwidths than NR. For example, a 6GR cell operating with 200 MHz system bandwidth may share only 100 MHz of its spectrum with a coexisting NR cell. In other deployment scenarios, a single 6GR cell may simultaneously perform MRSS with multiple NR cells, each occupying different subsets of the 6GR spectrum (e.g., two adjacent 100 MHz NR cells sharing with one 200 MHz 6GR cell).

High-level aspects to consider for NR-6GR MRSS include radio resource utilization.

For NR–6GR MRSS, study radio resource utilization aspects including rate matching of 6GR signals/channels around NR signals/channels, semi-static TDM/FDM between NR and 6GR, and NR signal/channel sharing with 6GR.

For NR–6GR MRSS, study scenarios where MRSS is performed only within a subset of the 6G cell’s frequency resources, or where different frequency segments are shared with different 5G cells.

UL/DL direction in TDD operation

For MRSS operation between NR and 6GR in a TDD carrier, considering the impact of CLI between 5G and 6G cells, it is considered necessary to align the UL/DL link direction used in the same symbol and same frequency resource between the two cells. However, whether this requires the frame format configuration applied to 5G and 6G cells to be identical is a matter that requires further discussion.

As we described in our companion contribution [1], even if the frame format configurations of 5G and 6G cells are not aligned, it may not be problematic as long as the two cells do not transmit and receive in opposite link directions on the same resource. Also, it is also worth considering scenarios where different UL/DL link directions are applied to different frequency resources within the same symbol between 5G and 6G cells.

Therefore, 6G study assumes that the UL/DL link direction used in the same symbol and same frequency resource between 5G and 6G cells is aligned. However, it should not assume that the frame format configurations applied to 5G and 6G cells are identical.

High-level aspects to consider for NR-6GR MRSS include alignment of the UL/DL link direction in the same time/frequency resource between NR and 6GR, while alignment of the frame format configurations applied to NR and 6GR is not considered.

NTN

In RAN1#122bis meeting, some technical aspects affected by NTN characteristics were agreed while other important aspects are not yet captured. In this contribution, we provide the technical aspects for 6GR NTN that need to be further considered.

In NR NTN, the EIRP is determined with an assumption that the power backoff is [5] dB. With limited EIRP, the coverage range would be further reduced due to the limited CNR. To recover the coverage range, it can be considered to adopt the repetition, but it will require more active time of a certain beam, and then the coverage ratio of the satellite would be limited. In 6GR, it is necessary to carefully investigate how to reduce the PAPR of the DL signals/channels in NTN scenario. In our view, it can be considered to support the DFT-s-OFDM as the additional DL waveform. According to our analysis, even though the DFT transform precoding can be applied to each DL channel/signal while the DFT transform precoding is not applied to common channels/signals, the remarkable PAPR reduction is observed. If we consider applying the DFT to multiple DL channels/signals jointly, the PARP can be further reduced at the expense of the additional specification works.

In case of NGSO, the relative satellite position is not fixed and the satellite speed is highly fast. Moreover, it would be necessary to further consider earth rotation. Depending on the relative location of the satellite with respect to the cell, the proper setting of the RRC configurations such as the repetition number, the value of K_offset, PRACH format could be different. In NR NTN, if the NW want to update the RRC configuration according to the satellite position, the RRC reconfiguration or SI update need to be used. It will cause large latency for updating the RRC configurations, and it will require additional resources for common signaling. In 6GR, it would be necessary to provide the adaptive RRC configuration based on the satellite position. In NR NTN, due to the satellite moving, the satellite switching is introduced for DL/UL re-synchronization while keeping the same cell ID and RRC configurations. During the satellite switching, the UE may need to monitor SSB from the source satellite and the target satellite, and the other data transmission/reception could be halt. This kind of service interruption would need to be enhanced in 6GR considering applications that require service continuity.

Meanwhile, in 6GR, TN-NTN spectrum sharing can be considered for efficient spectrum management and the common device implementation for both TN and NTN services. In this case, the interference management between TN and NTN would need to be carefully investigated. If the concept of the reserved resources is used for the TN-NTN spectrum sharing, the time drift, Doppler shift and its variations due to the satellite moving would need to be taken into account. To be specific, depending on the satellite location, the overlapping portion and location between the TN resources and the NTN resources will be changed. Alternatively, it can be considered that the satellite performs the TO/FO pre-compensation so that the DL frame boundary and the RB boundary for TN are aligned with the DL frame boundary and the RB boundary for NTN, respectively. However, since the level of the propagation delays for TN and NTN are significantly different, the differential delay within a certain area for TN and NTN can be quite different as well. In this case, even though the frame boundary and the RB boundary for NTN are aligned with those for TN on a certain reference location, it is not always ensured that the frame boundary and the RB boundary are aligned between TN and NTN on another location. Moreover, in 6G NTN, multi-orbit can be further considered for the purpose of service continuity during the satellite switching for LEO. Moreover, even in NTN, it would be necessary to enhance the throughput performance to extend the applicable scenario and the use cases.

IoT-NTN is designed to coexist with the NR-NTN. To be specific, the concept of the reserved resources is reused for this purpose. In 6GR TN, the concept of MRSS (multi-RAT spectrum sharing) is widely discuss to consider the coexistence between NR and 6GR in the same band. The migration issue would be common for both TN and NTN. In those points of views, it seems natural to consider the coexistence between IoT/NR-NTN and 6GR NTN.

The technical aspects affected by NTN characteristics further include the followings:

  - Power efficiency for DL/UL transmission

  - TN-NTN and NTN-NTN mobility

  - 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

  - Satellite moving and switching

  - Muti-carrier operation with same or different satellite(s).

ISAC

We suggest to focus on the object detection and tracking services in Rel-20. Motion/gesture detection and the environment sensing can be studied and specified in a later phase. Rel-19 ISAC channel modeling was not optimized for motion/gesture detection and the environment sensing.

Prioritize object detection and tracking use cases for ISAC study in Rel-20.

Given the focused the sensing services above, we can study all 5 target object types and 6 sensing modes, which were studied in Rel-19 ISAC channel modeling. Especially UE monostatic and UE-to-UE bistatic sensing are important for autonomous driving use cases. Depending on the outcome of Rel-20 ISAC SI, specific use cases and sensing modes can be prioritized for normative work in Rel-21.

Study all 5 target object types and 6 sensing modes, which were studied in Rel-19 ISAC channel modeling SI.

The performance requirements for 5G wireless sensing in Table 6.2-1 of TS22.137 can be reused for the requirements for ISAC study in Rel-20. Among the object detection and tracking scenario in the table, we suggest to use the sensing service categories 1 and 2 for the performance requirements in Rel-20, as we are targeting the basic framework of sensing. Other service categories and related requirements can be used in a later release with more advanced ISAC features.

The sensing service categories 1 and 2 of the performance requirements for 5G wireless sensing in Table 6.2-1 of TS22.137 can be reused for ISAC requirements in Rel-20.

Though ISAC feature discussions will happen after the basic 6G physical layer structure is discussed, it should not block the possibility of introducing a new feature necessary for ISAC later. In this sense, it is needed to consider a possible modification of 6G waveform and frame structure for adopting sensing feature.

6G waveform and frame structure needs to be designed so that any necessary modification for sensing should be easily applied in a later stage.

For the measurement report, the estimated delay, Doppler, and the angle of the target objects can be the baseline. In addition, the periodic change of Doppler can be reported for the micro-Doppler estimation. When the array antenna is used for sensing, the antenna configuration information can also be reported. To save the power and the resources, the extent of the delay, Doppler, and the angle to be reported can be configured. For the velocity estimation of the target object, we need to study to ensure that the sensing signals are transmitted with the identical time interval.

For bistatic sensing, not only the reflected path (path through sensing Tx-to-target & target-to-sensing Rx) but also the direct path (sensing Tx-to-sensing Rx path) needs to be considered to get the bistatic ranging. This is because the perfect synchronization between the sensing Tx and the sensing Rx are not always guaranteed. In this case, the time of arrival difference between the reflected and the direct path can be used for bistatic ranging.

The beam pairing operation for sensing can be different from that for the communication. For sensing, both sensing Tx and sensing Rx beam can be directional to the target object or the target sensing area. So, the metric for beam pairing can be the power level of the signal reflected by the target object, which can also be used for Tx power control.

Among the beam management for sensing, the beam failure detection and recovery may be a challenging issue. For sensing, the received power of the reflected signal can be a metric for determining a beam failure. However, before any object is detected, low reflected signal power does not always mean a beam failure. This is because it can also happen when there is no object in the target sensing area.

To support mobility of the sensing Tx, the sensing Rx, and the target object, the switching of the sensing entity and/or the sensing mode needs to be studied. For the bistatic sensing, the reflected signal power for the monostatic and the bistatic sensing can be used for determining the sensing entity or the sensing mode switching.

For TRP-to-TRP bistatic sensing, the resource allocation needs some improvement. To be consistent with the existing TRP operation (Tx on DL resources, and Rx on UL resource), TDD configuration of the sensing Tx TRP and the sensing Rx TRP can be configured so that there is some overlapping resource between UL and DL resources for sensing.

If the communication and the sensing signal is multiplexed in a same spectrum, it is inevitable that some portion of resources is used for sensing, which otherwise can used for communication. To minimize the loss of the communication resources while introducing the sensing service, the superposition of the sensing signal to the communication signal can be considered. The critical signals or channels for communication (e.g. SSB) can be excluded from the overlapping to protect the communication performance.

UE-to-UE bistatic sensing is an important sensing mode, which can be used for e.g. autonomous driving of vehicles or UAVs. The standardized UE-to-UE bistatic sensing can be a differentiator of 3GPP ISAC, from e.g. the commercial vehicular sensor devices. For UE-to-UE bistatic sensing, the easiest way is to use the sidelink operation. However, UE-to-UE bistatic sensing can also be implemented based on Uu link channel. For example, if UE2 overhears the UE1-to-TRP bistatic sensing signal, it results in UE1-to-UE2 bistatic sensing. We can study the overhearing-based UE-to-UE bistatic sensing operation.

For UE-to-TRP bistatic sensing, if the target sensing area or the target object is closer to the neighbor cell TRP than the serving cell TRP, it may be beneficial for UE of the serving cell to perform UE-to-TRP bistatic sensing with the neighbor cell TRP. For this operation, some improvement can be studied regarding the coordination between UE and the neighbor cell TRP for sensing operation.

3. Conclusion

The summary of our views is listed below.

Synchronization and Initial access

Proposal 1	Study on initial access procedure, random access procedure, paging, and measurement with physical layer aspects, including the following:

	Bandwidth configuration for the initial access,

	RACH procedures (i.e., 4-step RACH, 2-step RACH),

	Uplink transmission (UL Tx) beam selection via Msg1/Msg3,

	Time/frequency tracking reference signal for UEs in IDLE/Inactive state,

	Integration of L1/L3 measurements,

	PRACH-based request mechanism for SIB1 and other SIBs in 6GR, PEI mechanisms based on DCI or LP-WUS for 6G paging reception

Bandwidth

Proposal 2	Consider channel bandwidth (CBW) for narrowband operation (e.g., 3 MHz, 5MHz). It is recommended to design common channel/signal such as SS/PBCH using 15 kHz SCS, targeting 5 MHz operation, and verify its operability at 3 MHz.

Proposal 3	Identify the maximum bandwidth requirements per frequency band (e.g. sub 6GHz for FDD/TDD, around 7GHz, around 15GHz, and between 24.25GHz – 52.6GHz) that UEs should support, taking into account device type (i.e., low-tier device type, normal device type).

BWP framework/operation

Proposal 4	Need to remove the following sub-bullet related to reliability issue from the “Proposed observation 8.1c” in [2] since whether UE missed the BWP switching DCI can be confirmed by detecting the PUCCH/PUSCH scheduled on the new BWP.

	Some scenarios (e.g. non-overlapped BWPs) where DCI-based BWP switching can have reliability issue

Proposal 5	Consider lessons learned from 5G NR on the BWP framework/operation:

	Configurations per BWP is unnecessarily heavy (e.g. consuming large signaling overhead due to RRC parameter configurations for many features)

	BWP switching latency is too large (e.g. spending a few msec due to UE processing time required for simultaneous switching of many configurations)

	DL/UL BWP combination is restrictive (e.g. center frequency of DL and UL BWPs must be same for unpaired spectrum, and only contiguous bandwidth is possible)

Proposal 6	Study on 6G BWP framework/operation includes:

	Minimization of per-BWP parameters/configurations

	Reduction of BWP adaptation latency/complexity

	Relaxation/extension of DL/UL BWP combination

	Non-contiguous frequency resource allocation

Spectrum utilization/operations

Proposal 7	Among the topics listed in “Proposed observation 9.1b [R1-2508141]” as lessons learned from NR spectrum utilization and aggregation framework, the following topics may need further verification

	Some functionalities are supported only on Pcell

	Features (such as HARQ) defined per carrier leads to sub-optimal performance

	Late introduction of UL TX switching leads to restricted applicability/performance

Proposal 8	Consider the following as the lessons learned from NR spectrum utilization and aggregation framework

	Diverging functionalities for a similar purpose of cell/carrier adaptation/switching, such as SCell activation/deactivation, UL TX switching, LB-CA, SUL, etc. cause complex implementation at both network side and UE side

	Multi-cell deployment over multi-carriers in NR demands cell-specific always-on overhead signals on each carrier.

Proposal 9	Study for spectrum utilization and aggregation framework for 6GR should include the following

	Unified framework for cell/carrier adaptation/switching

	Fast SCell activation/deactivation procedure in 6GR including fast CSI acquisition

	Flexible band/carrier coupling between DL and UL for both FDD and TDD in 6GR.

	Efficient/effective/practical features of carrier ON/OFF, including SSB adaptation

	Efficient support of aggregating large number of (fractional) carriers for both intra-band and inter-band cases in 6GR.

	Common channel design/procedure, idle measurements and initial access for multi-carrier scenarios including multi-carrier single cell scenario.

PHY Control Channel

Proposal 10	Study on 6G DL control channel includes:

	Dynamic adaptation of DL control resources/parameters

	Adaptive reduction of UE blind decoding and DL control overhead

	Unified DL control resource structure accommodating multiple UE types

Proposal 11	Study on 6G UL control channel includes:

	Adaptation of UL control resource region and offloading

	Simplified UCI multiplexing structure

	Higher order modulation for UL control channel

PHY Data Channel

Proposal 12	Study scheduling enhancements in 6G considering enhanced multi-carrier operation, including TB mapping across multiple carriers

Proposal 13	Study mechanisms to support PDSCH/PUSCH transmissions crossing slot boundaries.

Proposal 14	Study a unified scheduling approach for repetition, covering existing schemes such as TBoMS PUSCH, Type-B PUSCH repetition, and multi-PUSCH scheduling

Proposal 15	Study the applicability of 5G FDRA type 0 and type 1 as potential baselines for 6G FDRA.

Proposal 16	Study the HARQ-ACK process enhancements in 6GR, including considering of enhanced multi-carrier operation, aiming for simplification and efficiency.

MIMO

Proposal 17	6G MIMO should aim to harmonize with other 6G key features such as AI/ML and NES/UES. It is also important to extend the scale of massive MIMO and to reduce signaling/RS overhead compared to 5G NR MIMO.

Proposal 18	Study CSI/BM taking into account the extension of UEI report and AI/ML prediction/compression, 6G band characteristic (e.g. near-field) and low signaling/reporting overhead.

Proposal 19	Study DL/UL MIMO schemes by considering various 6G MIMO application scenarios and 6G device implementation ways, e.g. heterogeneous panels/TRPs. Strive for a limited number of schemes but applicable to all relevant cases.

Proposal 20	Study ways to support larger numbers of CSI-RS/DMRS ports and RS overhead reduction techniques including AI/ML based techniques.

Multi-RAT Spectrum Sharing (MRSS)

Proposal 21	High-level aspects to consider for NR–6GR MRSS include resource allocation coordination between NR and 6GR, including whether their TRPs are co-located and whether they operate under the same or different vendors.

Proposal 22	High-level aspects to consider for NR–6GR MRSS include overhead from duplicated signal/channel transmissions in both NR and 6GR.

Proposal 23	High-level aspects to consider for NR-6GR MRSS include radio resource utilization.

Proposal 24	For NR–6GR MRSS, study radio resource utilization aspects including rate matching of 6GR signals/channels around NR signals/channels, semi-static TDM/FDM between NR and 6GR, and NR signal/channel sharing with 6GR.

Proposal 25	For NR–6GR MRSS, study scenarios where MRSS is performed only within a subset of the 6G cell’s frequency resources, or where different frequency segments are shared with different 5G cells.

Proposal 26	High-level aspects to consider for NR-6GR MRSS include alignment of the UL/DL link direction in the same time/frequency resource between NR and 6GR, while alignment of the frame format configurations applied to NR and 6GR is not considered.

NTN

Proposal 27	The technical aspects affected by NTN characteristics further include the followings:

	Power efficiency for DL/UL transmission

	TN-NTN and NTN-NTN mobility

	6G NTN coexistence with IoT-NTN or NR-NTN in same beam

	Satellite moving and switching

	Muti-carrier operation with same or different satellite(s).

ISAC

Proposal 28	Prioritize object detection and tracking use cases for ISAC study in Rel-20.

Proposal 29	Study all 5 target object types and 6 sensing modes, which were studied in Rel-19 ISAC channel modeling SI.

Proposal 30	The sensing service categories 1 and 2 of the performance requirements for 5G wireless sensing in Table 6.2-1 of TS22.137 can be reused for ISAC requirements in Rel-20.

Proposal 31	6G waveform and frame structure needs to be designed so that any necessary modification for sensing should be easily applied in a later stage.

4. Reference

[1] R1-2508395, Discussion on frame structure for 6GR, RAN1#123 LG Electronics

[2] R1-2508141, FL summary#4 on overview of 6GR air interface, RAN1#122bis, NTT DCM