| 3GPP TSG RAN WG1 Meeting #123 R1-2508918 |  |
| --- | --- |

Dallas, USA, Nov 17th – 21st, 2025

Source:	Fujitsu

Title:	Fujitsu’s view of 6GR air interface

Agenda Item:	11.1

Document for:	Discussion

# Introduction

The discussion of 6GR has been started from RAN1#122 meeting. In this contribution, we describe Fujitsu’s view of 6G radio air interface.

# Discussion of physical layer design details

## Scalable design for 6GR

Agreement - Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any Agreement - Study the device types from physical layer perspective to be supported by 6GR, subject to further discussion and confirmation in RAN

In RAN1#122 meeting, the above two agreements were reached.

To be frank, our society is less experienced in designing an extendable system, which is supposed to have very good forward compatibility. We used to work on very specific targets set for each item (either study item or work item). The only focus of the item was on how to reach the targets with a minimum/low cost (e.g., usually a specification cost). It was hard to consider whether the applicability/extendibility of the solution was wide or narrow, or anything about forward compatibility. In this way, the features newly introduced into the specifications for the support of the item were often quite unique compared with what the specifications had already had. This is a lesson learned from 5G.

Observation 1:

- 3GPP work style is used to be working on very specific targets set for each item (either study item or work item).

- The only focus of the item was on how to achieve the targets with a lowest/lower cost (e.g., usually a specification cost).

- In this way, the features newly introduced into the specifications as the output of the item were often quite unique compared with what the specifications had already had.

To stop repeating this in 6G, a scalable design is expected for 6GR.

In our understanding, to reach the scalability target, we may need to change how to proceed with the specification work, particularly the principle for choosing the technical paths. Instead of the cost for the specific feature aimed in this release, we may need to pay more attention to the sustainability and the extendibility in the future when comparing candidate solutions.

Take coverage enhancement as an example to explain the sustainability and extendibility in our understanding.

The essence of enhancing coverage is actually to improve the working SINR for the target physical channel/signal. One direction is boosting the receiving energy of signal; the other direction is to make the level of noise and interference lower. There are several ways which can help in either of or both of the two directions. For example,

- Repetition

- Gain upper bound: No. Tens dB could be typical value.

- Applicable limitation: Almost no, particularly in time domain.

- Enlarging transmitting power of the signal

- Gain upper bound: depending on the region regulation for transmitting power.

- Applicable limitation: Almost no.

- Interference handle among neighboring cells/gNBs

- Gain upper bound: when SINR can be reduced to equal to SNR.

- Applicable limitation: only for scenarios with interference.

- Frequency hopping (randomly avoid interference)

- Gain upper bound: when SINR can be reduced to equal to SNR, i.e., all interference get avoided.

- Applicable limitation: only for scenarios with interference and enough frequency resource for hoppling.

- Advanced channel coding and modulation scheme,

- Gain upper bound: a couple of dB.

- Applicable limitation: not applicable for the low tier devices which cannot afford the complexity increase due to adopting more advanced channel coding and modulation schemes.

- Increasing Tx/Rx antennas

- Gain upper bound: several dB.

- Applicable limitation: not applicable for the low tier devices or for the frequency bands which cannot afford massive antennas.

It’s obvious that repetition is the most scalable way for coverage enhancement. It can reach any gain required for the necessary enhancement and be applied for almost all devices and deployment/application scenarios.

Hence, to aim at the target of a scalable design for 6GR, for a certain feature, RAN1 needs to pay more attention to the sustainability and extendibility when comparing candidate solutions. The target solution should be applicable to most types of devices and most scenarios.

Proposal 1: Targeting at a scalable design for 6GR, for a certain feature, RAN1 needs to pay more attention to the sustainability and the extendibility when comparing candidate solutions. The target solution should be applicable to most types of devices and most scenarios.

## Minimum spectrum allocation and the smallest maximum RF/BB bandwidth

Agreement For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. Note: RAN4 involvement is necessary. Agreement Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity. Agreement Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth. - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different.

Proposal 1 (Endoresed in RAN Plenary #109): Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. FFS: whether RF and BB UE BW are same or different

Minimum spectrum allocation is a key factor to a system, which could be the minimum system bandwidth. The smallest maximum bandwidth is a key factor for devices, particularly the low tier devices. The RAN1 agreements about these two factors achieved in RAN1#122 have been re-examined in RAN#109 and gotten confirmed eventually, as shown above.

The minimum spectrum allocation and the smallest maximum RF/BB bandwidth of devices could be different. In general, our view is that the smallest maximum RF/BB bandwidth should be equal to or larger than the minimum spectrum allocation. The reasons are:

- The minimum spectrum allocation is usually restricted by the available frequency resource in operators’ hands. A moderate release for the requirement for the minimum spectrum allocation can increase the number of operators qualified to deploy 6G network. This would be good to enlarge the whole 6G market from the network side.

- Although the smallest maximum RF/BB bandwidth for devices can impact the hardware cost of devices, moderately increasing the requirement for the smallest maximum RF/BB bandwidth of devices could be a sensible choice for both the device vendors and the whole 6G ecology. The reason is that the hardware cost of devices is only a part of the overall cost of devices. There are still some other costs, e.g., for design or test. This part of cost will decrease when the market scale of a single type of device gets increasing. Moderately increasing the requirement for the smallest maximum RF/BB bandwidth can give more market opportunities to the low tier devices.

Observation 2: Moderately releasing the requirement for the minimum spectrum allocation and/or increasing the requirement for the smallest maximum RF/BB bandwidth of devices can be good for the 6G ecology.

- Moderately releasing the requirement for the minimum spectrum allocation can increase the number of operators qualified to deploy 6G network, which would be good for enlarging the whole 6G market.

- Moderately increasing the requirement for the smallest maximum RF/BB bandwidth of devices can provide more market opportunities to the low tier devices.

Proposal 2: The smallest maximum RF/BB bandwidth of devices should be equal to or larger than the minimum spectrum allocation.

As indicated in the agreements above, RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth.

First, regarding the minimum spectrum allocation, we think NR should be the baseline for 6GR. For NR, the minimum spectrum allocation was assumed to be 5MHz in the first release. Later in Rel-18, enhancements were made to support operation in spectrum allocations from approximately 3 MHz up to below 5 MHz. With that said, the minimum spectrum allocation for 6GR could be 3 MHz.

Second, regarding the smallest maximum UE bandwidth, we prefer to start with Opt 1: 3MHz, because smaller smallest maximum bandwidth UE bandwidth is beneficial for providing services to the low tier market segment.

Proposal 3: For both the minimum spectrum allocation and the smallest maximum UE bandwidth, 3 MHz is the starting point for both DL and UL.

## Coverage guarantee

Agreement - On enhanced overall coverage, identify coverage target(s) considering diverse use cases and device types

As mentioned in previous section 2.1, the repetition method is simple and the most compatible way for scalable device types to enhance / guarantee the coverage.

Proposal 4: Prioritize repetition in the discussion on how to guarantee the coverage.

## Spectrum utilization, operation and aggregation

### Multi-vendor operation for spectrum aggregation

Agreement - Study and identify the lessons learned from NR spectrum utilization and aggregation framework - DC is subject to RANP decision in June 2026 - Note: MRSS aspects are separate discussion

Spectrum aggregation is one of the most important functionalities from LTE-advanced in order to achieve higher peak data rate, higher resource utilization etc. While the importance will not be changed in 6G, we think it is important to address the “multi-vendor” issue behind the spectrum aggregation.

In the past generations, such as 4G, the most viable deployment was to use a single vendor. In that generation, generic products were not used in eNB side and hence multi-vendor operation was not considered widely. Now in 5G, vRAN concept using generic products are assumed in the network side, and hence more openness, so called Open RAN (O-RAN), drew big attention from the market. In the past, it is believed that carrier aggregation is optimum for single vendor deployment, but now, O-RAN has started their work on “D2 interface”, which enables inter-DU (hopefully multi-vendor) CA.

Given this situation, we believe the mechanism of 6G spectrum aggregation should be friendly to multi-vendor deployment, which is a difference from 4G and 5G. This means that the same mechanism as 4G and 5G carrier aggregation may not be the prerequisite in 6G, and further discussion in RAN WGs, including RAN1 and RAN2/RAN3 is necessary.

Proposal 5: Multi-vendor operation should be supported for 6G spectrum aggregation. The necessary mechanism and spec impacts in RAN WGs (including RAN2/RAN3) should be studied and discussed during the study phase.

### MRSS

FL Proposal 6.2b: - High-level aspects to consider for NR-6GR MRSS include, but not limited to - UE/NW implementation complexity - Resource allocation coordination between NR-6GR - Including whether NR and 6GR TRP are always co-located or not - Radio resource utilization - Signalling overhead - Operating bands at least existing FR1 - Alignment in time/frequency resource (e.g., numerology, RB, slot, symbol, UL/DL direction in TDD operation) - Reliance on availability of specific NR functionalities Note: Focus on existing NR deployments (NW and UE)

In previous meeting, we discussed about the high-level aspects as a guideline for NR-6GR MRSS design as shown in above FL proposal. We think the proposal is well listing to overcome the challenges based on the lessons and learned from LTE-NR DSS.

UE/NW implementation complexity

A core lesson and learned from LTE-NR DSS schemes is the need to minimize both UE and NW implementation complexity. The 6G MRSS design should adopt a mechanism that is fundamentally simple and leverages the lean design of NR. Overly complex solutions may be beneficial to have flexibility while it often leads to standardize ‘never-use feature’ and to cause interoperability issues between different vendors in multi-vendor scenario. A standardized, simplified and well-defined MRSS mechanism is essential to facilitate multi-vendor integration, ensure consistency, and reduce the likelihood of timing mismatches causing signal collisions and reduced throughput in multi-vendor environments.

Resource allocation coordination between NR-6GR

Efficient resource allocation coordination should be considered to maximize the utilization of shared spectrum. This aspect is strongly linked with the disaggregated environments such as inter-DU operation. The study should specifically address as below.

- Co-located TRPs: This scenario can provide the use case where NR and 6GR TRPs are tightly integrated, allowing for highly efficient, tightly coordinated scheduling.

- Non Co-located TRPs: The MRSS solution must be robust in scenarios where the NR and 6GR functions are hosted by different vendors or loosely coordinated due to standardized, high-latency interfaces or physical separation. This is a critical scenario to ensure multi-vendor operation is practical and efficient. The risk of timing mismatches causing signal collisions and reduced throughput must be mitigated by design, prioritizing semi-static or simple dynamic mechanisms that require less real-time inter-RAT communication. Whether NR and 6GR TRP are always co-located or not is a key discriminator for the coordination mechanism.

Radio resource utilization and signalling overhead

While NR is a more flexible baseline for spectrum sharing than LTE, the MRSS mechanism itself must be lightweight, minimizing signalling overhead. A key consideration is that the signaling (e.g., rate-matching patterns) used to coordinate resource sharing should be efficient and minimize reliance on additional information (e.g., RRC parameters) between RATs. This will prevent the degradation of overall spectrum efficiency and ensure that MRSS can be free from the limitations, which are often used to be obstacles in DSS.

Alignment in time/frequency resource

NR-6GR MRSS can start deploying from existing FR1 bands as one of the purposes for the MRSS is to enhance coverage by making use of lower frequency band. Furthermore, a smooth transition requires a high degree of alignment in time/frequency resource in terms of numerology (e.g., RB boundary, slot/ symbol boundary. Therefore, MRSS design should consider the aligned time/frequency grid to prevent the timing mismatches that lead to signal collisions in multi-vendor scenarios. As an example, a unified resource grid minimizes the risk of one vendor's 6GR transmission colliding with another vendor's NR transmission on the same carrier. Also, by aligning the UL/DL direction in TDD operation, the additional and redundant interference between UE-UE or gNB-gNB can be prevented.

Reliance on availability of specific NR functionalities

The MRSS design should carefully take into account its reliance on availability of specific NR functionalities. This essentially makes us to focus on existing NR deployments for both NW and UE. By designing 6GR MRSS to work with the most essential and widely supported NR features, 6GR ensure the solution is robust and practical for the deployment, together with operating existing multi-vendor NR networks. Also, any new MRSS mechanism should be consider features with backward compatible to ensure no impact on legacy NR UE behavior.

Proposal 6: For 6G MRSS, study high-level aspects to consider for NR-6GR MRSS include, but not limited to

- UE/NW implementation complexity

- Resource allocation coordination between NR-6GR

  - Including whether NR and 6GR TRP are always co-located or not

- Radio resource utilization

- Signalling overhead

- Operating bands at least existing FR1

- Alignment in time/frequency resource (e.g., numerology, RB, slot, symbol, UL/DL direction in TDD operation)

- Reliance on availability of specific NR functionalities

- Note: Focus on existing NR deployments (NW and UE)

### Single cell multi carriers (SCMC)

SCMC is one of the candidate solutions in 6G radio to improve spectrum utilization as well as the carrier aggregation. The concept of SCMC is to treat multiple fragmented carriers as one cell. We analyze the benefits of SCMC as below:

- Improvement of spectrum utilization

  - Unlike CA, the SCMC can utilize flexible configuration of carriers. For example, lower frequency band can be used as UL for the coverage enhancement while higher frequency band can be used as DL for the higher throughput.

- Reduction of signaling overhead

  - The unified cell configuration for SCMC can be applied to all carriers.

- Scheduling efficiency

  - As the multiple carriers are tied with one virtual carrier concept, it makes easy for both gNB and UE to enable efficiently cross-carrier scheduling among configured carriers.

Having said that, there are several issues to be studied for introducing SCMC in 6GR as below.

- Multi-carrier configuration (e.g., intra-band, inter-band, inter-FR band)

  - For study of applicable scenarios for SCMC, the frequency range of multi-carriers should be considered based on frequency characteristics, limitation of RF chain, SCS, etc.

- Co-located and/or Non co-located scenario

  - Depending on the scenario of Co-located and/or Non co-located, the parameter configuration such as TA or pathloss compensation should be different. Especially, for non-collocated scenario, since the multi-vendor operation is expected, the study should consider the corresponding mechanism to deal with it.

- Whether existing spectrum utilization framework (e.g., CA, SUL) can be integrated into SCMC framework

  - From the viewpoint of the simplified and unified 6GR, it should be studied whether existing spectrum utilization framework in 5G NR such as CA and SUL can be integrated into SCMC framework for reducing the number of features.

- Resource configuration

  - Since the multiple carriers are tied with one virtual carrier concept, it should be studied how to define the resource grid and how to indicate/allocate the resources across configured multi carriers.

Proposal 7: RAN1 should study SCMC operation to improve spectrum utilization, and at least the following points should be considered.

- Multi-carrier configuration (e.g., intra-band, inter-band, inter-FR band)

- Co-located and/or Non co-located scenario

- Whether existing spectrum utilization framework (e.g., CA, SUL) can be integrated into SCMC framework

- Resource configuration

## Bandwidth Part

Agreement - Study and identify the lessons learned from NR BWP framework

The BWP mechanism was introduced in 5G to optimize UE power consumption and enable differentiated service provisioning by network operators. Despite its benefits, BWP processing is constrained by several limitations, including elevated RRC signaling overhead, increased latency during BWP switching, and reliability concerns stemming from switching errors. In the rest of section, we analyze the lessons learned from 5G NR about BWP configuration.

RRC Overhead due to Over Defined BWP Structures

The primary contributor to excessive RRC overhead is the over definition of BWP configurations. Each BWP-ID is tightly coupled with a SCS and an extensive set of RRC parameters. Many of these parameters are not inherently related to BWP functionality, leading to unnecessary reconfiguration of the entire RRC parameter set even when only a subset of BWP-specific parameters is modified.

For example, in the context of 6GR, which promotes a single SCS per frequency band, the inclusion of SCS-related RRC parameters within the BWP structure is not desired. This necessitates a re-evaluation and simplification of the RRC parameter architecture associated with BWP to reduce signaling overhead and improve configuration efficiency.

Latency in DCI-Based BWP Switching

The latency associated with DCI-based BWP switching is significantly influenced by the volume and complexity of RRC parameters linked to each BWP-ID, as well as the retuning requirements of RF components. A larger parameter set increases the time required for re-loading all the parameters even if only a subset of these parameters is modified during BWP switching, thereby exacerbating switching delays.

Moreover, the 5G specifications define switching latency based on the worst-case scenarios, further limiting performance. To address this, there is a clear need for enhanced mechanisms that enable faster and more efficient BWP switching.

Reliability Issues in DCI-Based Switching

Although DCI-based BWP switching was designed to facilitate rapid transitions between BWPs, it suffers from reliability issues where gNB is not aware of BWP switching status. For example, due to the potential for erroneous DCI decoding, a UE may end up monitoring and transmitting over incorrect BWP. Another possible scenario could be where UE does correct decoding of DCI for BWP switching but the gNB cannot receive the corresponding channel/signal from the UE after BWP switching (e.g., due to poor UL coverage). To mitigate these issues, it is imperative to develop robust recovery mechanisms or alternative switching strategies that enhance the reliability of BWP transitions.

Proposal 8: Re-evaluate the 5G NR BWP framework to mitigate the substantial RRC overhead associated with BWP reconfigurations. A proposed optimization involves the strategic decoupling of BWP IEs into BWP-specific RRC parameters and BWP invariant RRC parameters. The BWP invariant RRC parameters can be configured once and subsequently shared across multiple configured BWPs.

Proposal 9: Investigate BWP switching mechanisms to address the below critical performance bottlenecks:

- High BWP switching latency

- Poor BWP switching reliability

## Harmonization of TN and NTN

#### Proposed observation 10.1a: - The lessons learned from NR/IoT NTN include, but not limited to - NR NTN was introduced at later releases in a “NBC” fashion - Legacy UEs not able to connect, requiring extra development efforts - Many of the NTN specific features in 5G NR were later made applicable to TN, leaving only a limited set of NTN-specific features - Achievable data rate was kept low, which limits the applicability of NTN use cases - GEO satellite is hardly supported due to coverage issues for NR-NTN - Low efficient beam hopping, severe UE power wasting - Repetition is considered for the coverage enhancement in NR NTN, but its applicable scenario can be limited due to the limited active beam ratio at the satellite. - High dependency on UE GNSS accuracy - Decreased number of UEs performing handover at the same time in the scenario of mix earth-fixed and earth-moving - Due to the separate NTN-specific SI design, the latency for initial access was high, which limits the actual service time of the satellite - The service interruption time for TN-NTN and NTN-NTN mobility was high due to the larger propagation delay

In the RAN1#122-bis, we made an agreement about the 6G design guidance for NTN based on NTN characteristics and lessons and learned from 5G NR / IoT NTN. The remaining task in NTN high-level aspects is to capture the lessons and learned from 5G NR into TR which was summarized as observation in FL summary of RAN1#122-bis as shown in above. The proposed observation is a good starting point for the discussion.

Regarding the first yellow highlighted part which was controversial in the last meeting, in our understanding, the intention of this bullet is to avoid additional specification impact due to later discussions for TN applicability. In addition, basic NTN features for timing alignment, time/frequency pre-compensation and NTN-TN mobility are still NTN-specific in the current specification while many of coverage enhancement techniques specified for NTN are applicable to TN. In this sense, we prefer to modify the wording as follow:

- Many of the NTN specific features in 5G NR were later made applicable to TN in some cases, leaving only a limited set of NTN-specific features

For the other yellow highlighted parts, these lessons should be listed in the TR because practical NTN situation would be helpful to consider the next generation NTN scenarios.

Proposal 10: RAN1 to discuss the observation about the lessons learned from NR/IoT-NTN to capture in the TR based on the discussion of the last meeting with following modification in the list.

- The NTN specific features in 5G NR were later made applicable to TN in some cases.

# Further study of physical layer design

## Initial Access and common channels

### Multiplexing between SSB and SIB1

In 5G NR, UEs determine the time and frequency resources of the PDCCH for SIB1 based on its relative location to the SSB and three multiplexing patterns between them were supported. These three multiplexing patterns are illustrated in Fig.1.

Among these three patterns, (1) pattern 1 can be considered in 6G for cases with narrow bandwidth, especially a bandwidth of 3 MHz; (2) Pattern 3 is useful in terms of achieving high energy efficiency thus it can be considered for 6G as well. For pattern 2, its benefit compared with the other two patterns is not clear. NR supports this pattern for the cases where it might not accommodate both PDCCH and PDSCH for SIB1 within the same time duration of SSB. However, this issue may not exist in 6G. Thus pattern 2 can be deprioritized.

Figure 1. Multiplexing patterns between SIB1 PDCCH and SSB in 5G NR

Proposal 11: Regarding the multiplexing pattern between SIB1 and SS/PBCH, pattern 1 (TDM pattern) and pattern 3 (FDM pattern) supported in 5G NR can be considered as a starting point.

### PRACH

To perform random access, the UE transmits a preamble on PRACH to the network. In NR, to support the features introduced in later releases, such as PRACH repetition and RedCap, RACH resource partitioning is applied which fragments the available PRACH resources. As a lesson learned from 5G, the forward scalability of PRACH resource allocation needs to be considered in 6G.

Moreover, in a multi-beam operation, mapping between PRACH resources and different beams of SS/PBCH is required for beam management. The mapping rule in 5G, however, appears overly complicated. For 6G, it is strived to design a simplified mapping rule to minimize implementation complexity on both UE side and network side and specification complexity.

Proposal 12: For PRACH, the following aspects can be considered for 6G:

- The scalability of PRACH resource allocation in time, frequency and code domains, as a forward compatibility.

- A simplified mapping rule between PRACH resources and synchronization signal/PBCH.

## MIMO operation

### Lessons learned from 5G and expectations for 6G

In 5G, some MIMO features were over-standardized. The consequence was that some specified mechanisms were quite complicated, such as CSI configuration and report. To learn this lesson, in our understanding, RAN1 may need first to discuss/clarify the basic assumptions and the fundamental standardization necessities of MIMO and, if possible, sketch a skeleton of the whole framework.

In the below table, we summarize lessons learned from 5G and expectation for 6G in terms of codebooks, deployment with TRP(s) and TCI framework.

|  | Lessons learned from 5G | Expectation for 6G |
| --- | --- | --- |
| Codebook | Several codebooks were supported in 5G, e.g., Rel-15 Type I, Rel-15 Type II, Rel-16 eType II and so on. This led to diverse paths for the development of codebooks in late releases in 5G. | - Clarify the basic assumption and the basic capability of antenna at both gNB and UEs. - Determine a single path/unified framework to design MIMO codebooks for 6G based on the basic assumption of antenna at both sides. - The aspects to be unified include SU/MU precoding procedures, CSI report procedures, CSI report overhead determination, computation capabilities and so on. Also, how to streamline the implementation and avoid market segmentation should be considered in the design. A good scalability, such as an extendibility to support a larger number of antenna ports, is expected. |
| Deployment with TRP(s) | Different designs are used for sTRP and mTRP. The design framework is not friendly in terms of extending an sTRP feature to the mTRP scenario. Although mTRP has been enhanced for many releases, it may not be extensively deployed in real life. | - Prioritize the most practical deployment scenarios and consider the realistic operation complexity. - Prioritize the sTRP scenario. Guarantee the forward compatibility of the design for sTRP to facilitate the extension of the design to mTRP later. sTRP and mTRP can be supported under a single framework, or even mTRP can be supported transparently if possible. |
| TCI framework | There are several flavors of TCI frameworks, e.g., Rel-15 TCI, Rel-17 unified TCI, Rel-18 unified TCI for mTRP. These branches required a lot of standardization efforts and, consequently, potentially complicated the configuration framework and relevant specifications. | - A single TCI framework is preferred to simplify the beam management procedures. - Rel-17 unified TCI framework can be used as a starting point, considering that the sTRP scenario is prioritized. |

Based on the above analysis, we provide our observation and proposals in below.

Observation 3: In 5G, some MIMO features were over-standardized. The consequence was that some specified mechanisms were quite complicated.

Proposal 13: Discuss and clarify the basic assumptions and the fundamental standardization necessities of MIMO first to avoid 6G MIMO from being over-standardized.

Proposal 14: Consider following expectations when RAN1 studies MIMO features:

- Clarify the basic assumption and the basic capability of antenna at both gNB and UEs and determine a single path/unified framework to design MIMO codebooks for 6G based on the basic assumption of antenna at both sides for codebooks.

- Prioritize the most practical deployment scenarios and consider the realistic operation complexity for the deployment with TRP(s) such as the sTRP scenario with the forward compatibility design to facilitate the extension of the design to mTRP later.

- A single TCI framework is preferred to simplify the beam management procedures.

- Rel-17 unified TCI framework can be used as a starting point, considering that the sTRP scenario is prioritized.

### RS design to support more antenna elements

For 6GR, an FR3 band (7~15GHz) can be also utilized. Within this frequency range, a larger number of antenna elements are feasible and thus can be supported to enhance the MIMO performance. Accordingly, a larger number of antenna ports can be formed to fully exploit the potential of the larger antenna array. With the increment of the antenna ports, more degrees of freedom can be exploited to support higher-rank SU-MIMO and higher-order MU-MIMO. More layers for SU-MIMO and more paired UEs for MU-MIMO are expected to be supported in FR3. Although the original intention is FR3, we also think the features to be standardized for FR3 can also be applied in FR1 when necessary, e.g., to further enhance MU-MIMO operation. To this end, a larger number of ports should be supported for RS such as CSI-RS/DMRS/SRS/PT-RS. When designing the new RS pattens, RS overhead should be taken into account. Also, codebook and CSI report are expected to be enhanced to support a larger number of antenna ports. When using a single path/unified framework to design MIMO codebook, it should also have a good scalability to support a larger number of antenna ports.

Proposal 15: Study how to support a larger number of antenna elements at least for FR3 considering belows

- CSI-RS/DMRS/SRS/PT-RS with a larger number of ports

- RS overhead reduction

- SU-MIMO/MU-MIMO with a larger number of layers/users

- Codebook/CSI report enhancement

#### CSI-RS

In our view, the larger number of CSI-RS ports can be introduced in 6G, e.g., up to 256 ports or 512 ports. New design for CSI-RS patterns is required for the larger CSI-RS ports. In NR system, the number of CSI-RS ports in each CSI-RS resource is at most 32 ports. Thus, 2-4 CSI-RS resources can be aggerated for 48, 64, 128 ports. It is questionable whether to configure larger CSI-RS ports in 6G also by aggregating multiple CSI-RS resources. Considering the compatibility with 5G UE, there may be three possibilities for CSI-RS pattern design in 6GR as below:

- To reuse CSI-RS RE mapping specified in 5G NR for all the possible numbers of CSI-RS ports. Design CSI-RS patterns larger than 128 ports following the design principle applied in 5G NR. In this way, the CSI-RS patterns for larger than 128 may also be achieved by the aggregation of multiple CSI-RS resources, which is configured with at most 32 ports. In this case, how to aggregate multiple CSI-RSs can be up to gNB’s implement flexibly, which is similar as 5G Rel-19 CSI-RS configuration. However, the CSI-RS resource setting may be complicated.

- To reuse CSI-RS RE mapping specified in 5G NR for the CSI-RS ports less than or equal to 128. Separately design new CSI-RS patterns for larger than 128 ports without considering how the legacy RE mapping was designed. By this way, one CSI-RS configuration method is used when the number of CSI-RS ports is larger than 128, and another CSI-RS configuration method is used when the number of CSI-RS ports is less than or equal to 128. Therefore, it is not a unified design for configuring CSI-RS patterns in 6G.

- To design all CSI-RS RE mapping with overall consideration. This method can make the structure of CSI-RS resource configuration clearer, and all the CSI-RS ports can be configured in single CSI-RS resource.  In our understanding, we prefer to design all CSI-RS patterns with overall consideration. This gives us a chance to carefully consider the overhead of CSI-RS design. At least, it should be avoided that the CSI-RS overhead linearly increases with the number of antenna ports.

Proposal 16: Study how to increase the number of CSI-RS ports.

Proposal 17: Study how/whether to reuse legacy CSI-RS patterns designed in 5G NR for the CSI-RS ports less than or equal to 128.

#### SRS

SRS ports are also expected to increase.

From the perspective of gNB, SRS capacity needs to increase to support more UEs within a slot. One feasible direction is supporting higher number of orthogonal sequences to allow more UEs to multiplex on the same SRS resources. From the perspective of UEs, such as Fixed Wireless Access (FWA) Customer Premises Equipment (CPEs), the support of more than 8 SRS ports is expected in 6GR.

FWA is a service which has been launched on the market but has not been optimized. In 5G NR, SRS ports are up to 8 antenna ports, which is sufficient for most mobile devices but may limit the potential of CPEs. FWA CPEs can support more antennas than those of common smart phones. And recently CPEs are evolving to support >8 antenna ports, especially in premium deployments aiming for higher uplink throughput and coverage. Besides, CPEs are not portals. They usually stand at a fixed place and don’t frequently experience rotation, which is also helpful for CPEs to do beamforming in uplink. Another advantage of CPEs over common smart phones is that a FWA CPE typically has continuous power supply which can guarantee a stable uplink transmission with a certain power.

In 6GR, at least for CPEs, the SRS ports should be more than 8. Either longer orthogonal sequences to enlarge the multiplexing factor or more SRS symbols within one slot could be feasible. This can allow CPEs to transmit more SRS in a short period, e.g. a slot. So that, the latency in capturing the full UL CSI from a CPE in 6GR could be shortened than that reached in 5G NR.

Proposal 18: Study how to increase SRS capacity to support more UEs within a slot.

Proposal 19: Study how to increase SRS ports per UE.

Furthermore, codebooks and CSI report are also expected to support a larger number of antenna ports. As discussed before, codebooks with a unified design to support all cases are expected. The unified codebook should have a good scalability to extend to a large number of antenna elements, e.g., 256 ports.

# Conclusion

In this contribution, we have following observations and proposals on the overview of 6GR:

Observation 1:

- 3GPP work style is used to be working on very specific targets set for each item (either study item or work item).

- The only focus of the item was on how to achieve the targets with a lowest/lower cost (e.g., usually a specification cost).

- In this way, the features newly introduced into the specifications as the output of the item were often quite unique compared with what the specifications had already had.

Proposal 1: Targeting at a scalable design for 6GR, for a certain feature, RAN1 needs to pay more attention to the sustainability and the extendibility when comparing candidate solutions. The target solution should be applicable to most types of devices and most scenarios.

Observation 2: Moderately releasing the requirement for the minimum spectrum allocation and/or increasing the requirement for the smallest maximum RF/BB bandwidth of devices can be good for the 6G ecology.

- Moderately releasing the requirement for the minimum spectrum allocation can increase the number of operators qualified to deploy 6G network, which would be good for enlarging the whole 6G market.

- Moderately increasing the requirement for the smallest maximum RF/BB bandwidth of devices can provide more market opportunities to the low tier devices.

Proposal 2: The smallest maximum RF/BB bandwidth of devices should be equal to or larger than the minimum spectrum allocation.

Proposal 3: For both the minimum spectrum allocation and the smallest maximum UE bandwidth, 3 MHz is the starting point for both DL and UL.

Proposal 4: Prioritize repetition in the discussion on how to guarantee the coverage.

Proposal 5: Multi-vendor operation should be supported for 6G spectrum aggregation. The necessary mechanism and spec impacts in RAN WGs (including RAN2/RAN3) should be studied and discussed during the study phase.

Proposal 6: For 6G MRSS, study high-level aspects to consider for NR-6GR MRSS include, but not limited to

- UE/NW implementation complexity

- Resource allocation coordination between NR-6GR

- Including whether NR and 6GR TRP are always co-located or not

- Radio resource utilization

- Signalling overhead

- Operating bands at least existing FR1

- Alignment in time/frequency resource (e.g., numerology, RB, slot, symbol, UL/DL direction in TDD operation)

- Reliance on availability of specific NR functionalities

- Note: Focus on existing NR deployments (NW and UE)

Proposal 7: RAN1 should study SCMC operation to improve spectrum utilization, and at least the following points should be considered.

- Multi-carrier configuration (e.g., intra-band, inter-band, inter-FR band)

- Co-located and/or Non co-located scenario

- Whether existing spectrum utilization framework (e.g., CA, SUL) can be integrated into SCMC framework

- Resource configuration

Proposal 8: Re-evaluate the 5G NR BWP framework to mitigate the substantial RRC overhead associated with BWP reconfigurations. A proposed optimization involves the strategic decoupling of BWP IEs into BWP-specific RRC parameters and BWP invariant RRC parameters. The BWP invariant RRC parameters can be configured once and subsequently shared across multiple configured BWPs.

Proposal 9: Investigate BWP switching mechanisms to address the below critical performance bottlenecks:

- High BWP switching latency

- Poor BWP switching reliability

Proposal 10: RAN1 to discuss the observation about the lessons learned from NR/IoT-NTN to capture in the TR based on the discussion of the last meeting with following modification in the list.

- The NTN specific features in 5G NR were later made applicable to TN in some cases.

Proposal 11: Regarding the multiplexing pattern between SIB1 and SS/PBCH, pattern 1 (TDM pattern) and pattern 3 (FDM pattern) supported in 5G NR can be considered as a starting point.

Proposal 12: For PRACH, the following aspects can be considered for 6G:

- The scalability of PRACH resource allocation in time, frequency and code domains, as a forward compatibility.

- A simplified mapping rule between PRACH resources and synchronization signal/PBCH.

Observation 3: In 5G, some MIMO features were over-standardized. The consequence was that some specified mechanisms were quite complicated.

Proposal 13: Discuss and clarify the basic assumptions and the fundamental standardization necessities of MIMO first to avoid 6G MIMO from being over-standardized.

Proposal 14: Consider following expectations when RAN1 studies MIMO features:

- Clarify the basic assumption and the basic capability of antenna at both gNB and UEs and determine a single path/unified framework to design MIMO codebooks for 6G based on the basic assumption of antenna at both sides for codebooks.

- Prioritize the most practical deployment scenarios and consider the realistic operation complexity for the deployment with TRP(s) such as the sTRP scenario with the forward compatibility design to facilitate the extension of the design to mTRP later.

- A single TCI framework is preferred to simplify the beam management procedures.

- Rel-17 unified TCI framework can be used as a starting point, considering that the sTRP scenario is prioritized.

Proposal 15: Study how to support a larger number of antenna elements at least for FR3 considering belows

- CSI-RS/DMRS/SRS/PT-RS with a larger number of ports

- RS overhead reduction

- SU-MIMO/MU-MIMO with a larger number of layers/users

- Codebook/CSI report enhancement

Proposal 16: Study how to increase the number of CSI-RS ports.

Proposal 17: Study how/whether to reuse legacy CSI-RS patterns designed in 5G NR for the CSI-RS ports less than or equal to 128.

Proposal 18: Study how to increase SRS capacity to support more UEs within a slot.

Proposal 19: Study how to increase SRS ports per UE.

# Reference

- RP-250810, “Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

- RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.