3GPP TSG-RAN WG1 Meeting #123	R1-2509028

# Dallas, USA, Nov. 17th – 21st, 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on stop condition for recovery search space set monitoring for unified TCI update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ofinno |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the existing specification, after completing beam failure recovery, the conditions for a UE to stop monitoring the search space set indicated by recoverySearchSpaceId are (i) receiving an RRC reconfiguration (e.g., tci-StatesPDCCH-ToAddList) indicating TCI states for a CORESET followed by a MAC-CE activating one of the configured TCI states, (ii) receiving an RRC reconfiguration (e.g., tci-StatesPDCCH-ToAddList) indicating a single TCI state for a CORESET, or (iii) receiving a MAC-CE activating one of the already configured TCI states. In Release 15, the conditions for a UE to stop monitoring the search space set indicated by recoverySearchSpaceId after completing beam failure recovery were defined based on the RAN1 #91 agreement copied below. RAN1 #91 Agreement: Upon receiving gNB response for beam failure recovery request transmission, UE shall - UE shall monitor CORESET-BFR for dedicated PDCCH reception until one of the following conditions is met: - Reconfigured by gNB to another CORESET for receiving dedicated PDCCH and activated by MAC-CE a TCI state if the configured CORESET has K>1 configured TCI states - FFS: if a default TCI state can be assumed for PDCCH after reconfiguration without MAC-CE activation - Re-indicated by gNB to another TCI state(s) by MAC-CE of CORESET(s) before beam failure - Until the reconfiguration/activation/re-indication of TCI state(s) for PDCCH, UE shall assume DMRS of PDSCH is spatial QCL’ed with DL RS of the UE-identified candidate beam in the beam failure recovery request - After the reconfiguration/activation/re-indication of TCI state(s) for PDCCH, UE is not expected to receive a DCI in CORESET-BFR. - Note: this applies to same carrier case.. These existing stop conditions do not account for the new Rel-17 unified TCI state mechanism where a new TCI state for a CORESET can be indicated/updated by Rel-17 unified TCI state mechanism. |
| Summary of change: |  | Add the Rel-17 unified TCI state indication mechanisms as conditions for the UE to stop monitoring recoverySearchSpace. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior on CORESET-BFR is different between Rel-15 and Rel-17 mechanism. UE behavior on recoverySearchSpace is inefficient and increase UE power consumption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 6	Link recovery procedures

===============================unchanged omitted================================

For the PCell or the PSCell, the UE can be provided, by PRACH-ResourceDedicatedBFR, a configuration for PRACH transmission as described in clause 8.1. For PRACH transmission in slot n and according to antenna port quasi co-location parameters associated with periodic CSI-RS resource configuration or with SS/PBCH block associated with index qnew provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space set provided by recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI starting from slot n+4+2μ∙kmac, where μ is the SCS configuration for the PRACH transmission and kmac is a number of slots provided by kmac [12, TS 38.331] or kmac=0 if kmac is not provided, within a window configured by BeamFailureRecoveryConfig. For PDCCH monitoring in a search space set provided by recoverySearchSpaceId and for corresponding PDSCH receptions, the UE assumes the same antenna port quasi-collocation parameters as the ones associated with index qnew until the UE receives by higher layers an activation or a DCI for a TCI state or any of the parameters tci-StatesPDCCH-ToAddList, tci-StatesPDCCH-ToReleaseList and/or dl-OrJointTCI-StateList. After the UE detects a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpaceId, the UE continues to monitor PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command or a DCI for a TCI state or tci-StatesPDCCH-ToAddList, tci-StatesPDCCH-ToReleaseList and/or dl-OrJointTCI-StateList.

===============================unchanged omitted================================