3GPP TSG RAN WG1 Meeting #123	R1-2509253

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.1

Source: 	Quectel

Title: 	Maintenance on AI/ML for NR air interface

Document for:   Discussion

- Introduction

In the RAN#106 plenary meeting [1], the objective of Rel-19 NR work item on AI/ML is to provide specification support for AI/ML based CSI prediction, and normative work for CSI prediction started from Q1 of 2025. The specification impact for CSI prediction including the following aspects.

Provide specification support for the following aspects: … - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 …

In the following, we discuss the specification support for CSI prediction of AI/ML model.

- Remaining issues for AI based CSI prediction

Reason for change:

In last meeting [2], the following TP is endorsed for TS 38.214 for Clauses 5.2.1.4.6.

Agreement

Adopt the following TP for TS 38.214.

| Reason for change | According to the agreement, Rel-16 eType II codebook is used for calculating both the precoder wk represented by PMI based on the channel measurement corresponding to the monitoring report and the precoder wk represented by PMI based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the inference report. However, the codebook for calculating wkwas not clearly captured by the specification. |
| --- | --- |
| Summary of change | Add the corresponding description of the codebook for calculating wk in TS 38.214. |
| Consequences if not approved | How to select the codebook for calculating wk is not clearly defined in TS 38.214, which is not aligned with the agreement. |
| TS38.214 5.2.1.4.6 CSI-PAI reporting << Unchanged parts are omitted >> - to report CSI-PAI for the second Reporting Setting, the UE is expected to be configured in the second Report Setting with codebookType set to ‘etypeII-r16’ and it shall: <Unrelated part omitted> - for k ∈1,2,…,v, v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as, SGCSk1=n=1NwknHwknwkn.wkn2N, SGCSk2=n=1NwknHwknwkn.wkn2N, where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting, and wk wkn is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. Both wkn and wkn are calculated based on the codebookConfig within the second Reporting Setting. The UE is expected to be configured with same csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting. << Unchanged parts are omitted >> |  |

According to the agreements, to determine non-predicted PMI for each of the subband(s), which is corresponding to the S-th time instance of the report of the first Reporting Setting, the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting is used. However, how to determine the reference signal corresponding to the S-th time instance of the report of the first Reporting Setting isn’t described in the specifications.

Summary of change:

We propose to add the description of the corresponding measurement reference for the calculating wkn in TS 38.214.

Consequence if not approved:

The selection of a measurement reference signal for wkn is not clearly defined in TS 38.214, which is not aligned for implementation at UEs and gNBs.

- For CSI-PAI reporting, adopt the following TP:

---------------------------------------------------------Start of TP for TS38.214---------------------------------------------

5.2.1.4.6	CSI-PAI reporting

<< Unchanged parts are omitted >>

to report CSI-PAI for the second Reporting Setting, the UE shall:

-	determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting ( , with N4 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting,

-	determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting.

-	determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting

-	for , v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as,

SGCSk1=n=1NwknHwknwkn.wkn2N,

SGCSk2=n=1NwknHwknwkn.wkn2N,

where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting nearest to the S-th time instance corresponding to the report of the first Reporting Setting,, and wkn is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. Both wkn and wkn are calculated based on the codebookConfig within the second Reporting Setting. The UE is expected to be configured with same csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting.

< Unchanged parts are omitted >

----------------------------------------------------------End of TP for TS38.214--------------------------------------------------

- Conclusion

In this contribution, we discussed the topics on CSI prediction with AI/ML models, and our proposal is summarized as below:

Proposal 1：

- For CSI-PAI reporting, adopt the following TP:

---------------------------------------------------------Start of TP for TS38.214---------------------------------------------

5.2.1.4.6	CSI-PAI reporting

<< Unchanged parts are omitted >>

to report CSI-PAI for the second Reporting Setting, the UE shall:

-	determine predicted PMI (see Clause 5.2.2.2.10 or Clause 5.2.2.2.11) for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband for the S-th time instance configured by timeinstanceforCSImonitoring-r19 in the second Reporting Setting ( , with N4 provided in the first Reporting Setting), based on the CSI prediction performed by the UE using the channel measurement corresponding to the first Reporting Setting,

-	determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, which is corresponding to the S-th time instance of the report of the first Reporting Setting, based on the CSI-RS resource(s) in the resource set for channel measurement for the report corresponding to the second Reporting Setting.

-	determine non-predicted PMI for each of the subband(s) configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than the CSI reference resource of the CSI report, corresponding to the first Reporting Setting

-	for , v is the RI reported for the CSI report corresponding to the first Reporting Setting, calculate SGCS value(s) as,

SGCSk1=n=1NwknHwknwkn.wkn2N,

SGCSk2=n=1NwknHwknwkn.wkn2N,

where wkn is the predicted precoder represented by predicted PMI for k-th layer, n-th subband and for S-th time instance corresponding to the report of the first Reporting Setting, wkn is the precoder represented by non-predicted PMI for k-th layer, n-th subband based on the channel measurement corresponding to the second Reporting Setting nearest to the S-th time instance corresponding to the report of the first Reporting Setting,, and wkn is the precoder represented by PMI for k-th layer and n-th subband, n∈1,2,…,N, corresponds to the subbands configured by csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband, based on the latest CSI-RS resource transmission occasion, no later than CSI reference resource, corresponding to the report of the first Reporting Setting. Both wkn and wkn are calculated based on the codebookConfig within the second Reporting Setting. The UE is expected to be configured with same csi-ReportingBand and numberOfPMI-SubbandsPerCQI-Subband configurations for the first CSI Reporting Setting and the second CSI Reporting Setting.

< Unchanged parts are omitted >

----------------------------------------------------------End of TP for TS38.214--------------------------------------------------

Reference

- RP-243244, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Madrid, Spain, December 9-12, 2024.

- Chair’s Notes RAN1#122bis, Pra gue, Czech, Oct 13th – 17th, 2025.