3GPP TSG RAN WG1 #123	R1-2509322

Dallas, USA, Nov 17th – 21st, 2025

Source:	Sharp

Title:	Study on waveform for 6GR

Agenda Item:	11.3.1

Document for:	Discussion and Decision

Background

In RAN#108, the study item on 6G Radio was agreed [1].

The detailed objectives of the study are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

Discussions

Including proposals for improving spectrum efficiency, power efficiency, coexistence and coverage, etc.

Note: Proponents are encouraged to provide more detailed information on their proposals for the next meeting, e.g.: Proponents to characterize the main motivation for modification/additional waveform proposals: - Targeted link direction, i.e. DL, UL or both - Targeted use case (e.g. NTN, specific frequency range, etc.), if any - Potential motivations metrics used, and quantified gains for a proposal, e.g. - Coverage - Network energy efficiency - UE energy efficiency - Spectral efficiency - High speed tolerance - Scheduling flexibility - Integration with ISAC Proponents provide information on the following aspects, if applicable - MRSS compatibility - Target channels/signals, e.g. all channels, PxSCH only, etc. - MIMO (SU and MU-MIMO) compatibility - Target modulations, and impact to other modulations, if applicable - Multi-user multiplexing/scheduling flexibility - Multiplexing/coexistence with baseline waveforms - Impact on synchronization and initial access - Expected specification impact - Transmitter/receiver complexity and impact to power consumption.

  - DL DFT-s-OFDM

DL DFT-s-OFDM was proposed in the previous meeting to enhance the coverage and improve the PA efficiency. For this reason, expected use cases for DL DFT-s-OFDM are IoT, NTN, FR2, and Network energy saving according to proponents.

We summarize our assessments of DL DFT-s-OFDM as the following:

Table 1: Sharp’s views for DL DFT-s-OFDM

|  | Positive | Negative | Sharp’s view |
| --- | --- | --- | --- |
| Coverage | x |  | At least, PAPR of DFT-s-OFDM is better than or equal to that of CP-OFDM. |
| UE energy efficiency |  |  |  |
| BS energy efficiency |  | x | PA efficiency is not main bottleneck. |
| Spectral efficiency |  |  | Same spectral efficiency as CP-OFDM. |
| High speed tolerance |  |  |  |
| Scheduling flexibility |  | x | DFT-s-OFDM leads to FDRA limitation. |
| Integration with ISAC |  |  |  |
| MRSS compatibility | x |  | 5G resource grid can be reused. |
| MIMO compatibility |  | x | FDRA limitation leads to higher MU-MIMO complexity. |
| User multiplexing |  | x | The possibility of supporting MU is limited. |
| Impact on initial access | x |  | Coverage extension leads to a higher probability of successful connections. |
| Transmitter/receiver complexity | x |  | DFT-s-OFDM has no significant impact on complexity. |

In our analysis, the advantage of DL DFT-s-OFDM compared to CP-OFDM is only coverage gain. On the other hand, DL CP-OFDM is better than DL DFT-s-OFDM even from MRSS compatibility and transmitter/receiver complexity aspects because it has no impact on the legacy design. Nevertheless, FDRA limitation of DL DFT-s-OFDM complicates the scheduling flexibility and limits multi-user possibilities. Particularly, due to FDRA limitation by DFT-s-OFDM, only resource allocation type 1 (i.e., SLIV) is possible for each UE and we think MU-MIMO performance would be largely degraded. For MU-MIMO, performance can be optimized such that the intra-cell interference introduced by e.g., correlation of precoders for multiplexed UEs, can be mitigated by flexibly allocating resources for each UE. For this reason, we don’t support DL DFT-s-OFDM.

Proposal 1: For 6GR waveform, RAN1 should NOT study DL DFT-s-OFDM.

  - Multi-layer UL DFT-s-OFDM

In RAN1#122bis meeting, it was agreed that the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1 is studied.

Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

First, since multi-layer UL transmission is supported only for UL CP-OFDM in 5G NR, RAN1 should compare multi-layer UL DFT-s-OFDM with UL CP-OFDM. This comparison should be treated in 6GR MIMO, and we discuss the evaluation method in 6GR waveform.

In our view, for the evaluation of multi-layer UL DFT-s-OFDM, Net Gain is useful to evaluate realistic coverage benefits while assuming the same spectrum efficiency. In each port that has a PA, whether multiple layers interfere with each other depends on the specific precoder design. For example, for non-coherent precoding, time-domain signal on each port can keep single-carrier characteristics. However, for partial- or full-coherent precoding, time-domain signals on multiple ports are interfered with each other and PAPR of each time-domain signal on each port is degraded. For this reason, the used precoder assumption for evaluation is needed to compare with the legacy UL CP-OFDM. In Net Gain, Tx power gain relative to the reference depends on a precoder and is maximized if using the non-coherent precoding. On the other hand, SNR degradation relative to the reference can be minimized by the optimized precoder. Therefore, for the evaluation, companies should report the assumed precoder.

Proposal 2: For the evaluation of multi-layer UL DFT-s-OFDM, Net Gain should be used and the assumed precoder should be reported.

  - Low-PAPR for UL DFT-s-OFDM

FDSS (Frequency domain spectrum shaping)

In Rel-15, 5G NR supports Frequency Domain Spectrum Shaping (FDSS) without spectrum extension for pi/2 BPSK to reduce the PAPR. With a smooth transition at the occupied bandwidth, PAPR for the PUSCH transmission can be reduced accordingly.

We summarize our assessments of FDSS as the following:

Table 2: Sharp’s views for FDSS

|  | Positive | Negative | Comment |
| --- | --- | --- | --- |
| Coverage | x |  | PAPR is reduced. |
| UE energy efficiency | x |  | Low PAPR leads to UE power saving. |
| BS energy efficiency |  |  |  |
| Spectral efficiency | x |  | Keep the same spectral efficiency as CP-OFDM/DFT-s-OFDM. |
| High speed tolerance |  |  |  |
| Scheduling flexibility | x |  | FDSS does not impact the scheduling flexibility compared to the legacy DFT-s-OFDM. |
| Integration with ISAC |  |  |  |
| MRSS compatibility | x |  | Rel-15 5G NR supports FDSS. |
| MIMO compatibility | x |  | FDSS does not impact MIMO compatibility. |
| User multiplexing | x |  | FDSS does not impact the multiplexing. |
| Impact on initial access | x |  | Coverage extension leads to a higher probability of successful connections. |
| Transmitter/receiver complexity | x |  | FDSS is transparent to NW. |

In our analysis, FDSS has no disadvantage, and the spectrum shaping can be easily applied to the DFT-s-OFDM. Regarding coverage, whether FDSS is realistically beneficial is evaluated in Section 2.3.2.1.

FDSS-SE (FDSS with spectrum extension)

In Rel-18, the symmetric spectrum extension option for frequency domain spectrum shaping with spectrum extension (FDSS-SE) were considered for studying MPR/PAR reduction enhancements. Figure 1 illustrates FDSS-SE scheme with symmetric extension. In particular, the excess bandwidth (M), specified by the extension factor (α) is added to the allocated bandwidth (Ndata). Subsequently, M/2 tones are symmetrically added to the start and end of Ndata to obtain the total allocated bandwidth for FDSS-SE (Nextended).

Figure 1: FDSS-SE with symmetric extension.

We summarize our assessments of FDSS-SE as the following:

Table 3: Sharp’s views for FDSS-SE

|  | Positive | Negative | Comment |
| --- | --- | --- | --- |
| Coverage | x |  | PAPR is reduced. |
| UE energy efficiency | x |  | Low PAPR leads to UE power saving. |
| BS energy efficiency |  |  |  |
| Spectral efficiency |  | x | Spectral efficiency is reduced as extended resource is needed. |
| High speed tolerance |  |  |  |
| Scheduling flexibility |  | x | Extended resource is explicitly indicated to avoid resource overlap between UEs. |
| Integration with ISAC |  |  |  |
| MRSS compatibility |  |  |  |
| MIMO compatibility | x |  | FDSS-SE does not impact MIMO compatibility. |
| User multiplexing |  |  |  |
| Impact on initial access | x |  | Coverage extension leads to a higher probability of successful connections. |
| Transmitter/receiver complexity |  |  | FDSS-SE is non-transparent to NW. |

Compared with FDSS, the scheduling for FDSS-SE is more complicated because the NW needs to manage resources for both data and excess part. However, there is no significant burden on the receiver such that NW schedules the total allocated bandwidth (N_extended) to each UE and ignores the modulation symbol in the excess part.

Regarding the spectral efficiency, we evaluate whether the coverage is improved when assuming the same spectral efficiency in Section 2.3.2.1.

Evaluation: Coverage

Agreement Draft LS R1-2508068 is endorsed with following revision: - removing “Additionally, if time permits, any feedback for CP-OFDM PAPR reduction/MPR values achievable by implementation is also appreciated.” Agreement Final LS R1-2508069 is endorsed. Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding

In RAN1#122bis meeting, it was agreed that the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference. In this section, we evaluate FDSS and FDSS-SE performance when the reference is DFT-s-OFDM.

As a simulation assumption shown in Table 4, the same TBS is considered for both FDSS and FDSS-SE.

Table 4: Simulation assumption for FDSS and FDSS-SE evaluations.

|  |  | No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TBS | #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| Case 1 | 288 | 8 | 0 (pi/2 BPSK) | 6 | 8 | 1 (pi/2 BPSK) | 1/4 |
| Case 2 | 1128 | 8 | 6 (QPSK) | 6 | 8 | 8 (QPSK) | 1/4 |

In Rel-18 Coverage enhancement, it was agreed that companies are encouraged to use the following pulse shaping filters (PSF):

  - 2-tap [1 0.28]

  - 3-tap [0.335 1 0.335]

  - 3-tap [0.28 1 0.28]

In this contribution, we assumed 2-tap [1 0.28] and its frequency response is shown in Figure 2.

Figure 2: Spectrum shaping filter of 2-tap [1 0.28] for FDSS w/ and w/o SE.

In our view, Net Gain can be calculated based on PAPR or MPR (Maximum power reduction). PAPR-based Net Gain is straightforwardly calculated, but it may differ slightly from the realistic coverage gain because nonlinear components in PA are potentially negligible. On the other hand, MPR-based Net Gain can be utilized as realistic coverage gain, but RAN4 work is needed for the accurate MPR value in the expected RAN4 LS reply. For this reason,  we evaluate PAPR-based Net Gain and MPR-based Net Gain as preliminary results in this contribution.

First, we evaluate PAPR/cubic metric (CM) performances. In this contribution, Raw CM is calculated as:

Raw CM (dB)=20logvnorm3rms-1.52/1.66

where vnorm is the normalized voltage waveform of the input signal to PA.

Figure 3 and Figure 4 show PAPR/Raw CM performances for Case 1 and Case 2, respectively.

Figure 3: PAPR and Raw CM in Case 1

Figure 4: PAPR and Raw CM in Case 2

As shown in Figure 3 and Figure 4, the spectrum shaping can significantly reduce PAPR and suppress third-order distortion for both pi/2 BPSK and QPSK. We summarize our observations as follows:

Observation 1: For Case 1 (pi/2 BPSK), PAPR improvement at 1% CCDF is 1.46 dB and Raw CM improvement at 1% CCDF is 0.70 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 2: For Case 1 (pi/2 BPSK), PAPR improvement at 1% CCDF is 1.78 dB and Raw CM improvement at 1% CCDF is 0.49 dB for FDSS-SE compared to conventional DFT-s-OFDM.

Observation 3: For Case 2 (QPSK), PAPR improvement at 1% CCDF is 1.01 dB and Raw CM improvement at 1% CCDF is 0.26 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 4: For Case 2 (QPSK), PAPR improvement at 1% CCDF is 2.44 dB and Raw CM improvement at 1% CCDF is 1.22 dB for FDSS-SE compared to conventional DFT-s-OFDM.

Figure 5 show BLER performance in Case 1 and Case 2. For this evaluation, simulation assumptions such as channel model, coding scheme are shown in Appendix.

Figure 5: BLER performance in (a) Case 1 and (b) Case 2.

In Figure 5, BLER performances for both FDSS and FDSS-SE is degraded because the spectrum shaping filter leads to distorted data on the edge of allocated resource for FDSS and the higher code rate is used to assume the same spectrum efficiency for FDSS-SE. Whether this SNR degradation relative to the reference is acceptable depends on Net Gain.

In RAN1#122bis meeting, it was agreed that Net Gain is calculated as:

  - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER

In our analysis, for PAPR-based Net Gain, Tx power gain relative to the reference is calculated as ΔPAPR. For MPR-based Net Gain, Tx power gain relative to the reference is calculated as ΔMPR.

Although not only CM but also EVM and ACSL are needed to calculate MPR precisely, in this contribution, we use MPR calculation model for DC-HSUPA in TS25.101-1 as preliminary results. We can update the new model in response to the RAN4 LS sent in the last meeting.

In particular, CM and MPR can be calculated as:

CM=CeilRaw CM,0.22

MPR=MaxCM-0.72, 0

CM=Ceilx,0.22 means rounding upwards to closest 0.22dB with 0.5dB granularity, i.e., CM = [0.22, 0.72, 1.22, 1.72, 2.22, 2.72, 3.22, 3.72].

Table 5 and Table 6 show PAPR-based Net Gain and MPR-based Net Gain, respectively. In Table 5 and Table 6, PAPR and Raw CM value is calculated at 1% CCDF, and SNR value is calculated at 10% BLER.

Table 5: PAPR-based Net Gain for FDSS and FDSS-SE compared to conventional DFT-s-OFDM (reference) in Case 1 and Case 2

|  |  | PAPR (dB) | ΔPAPR | SNR (dB) | ΔSNR | Net Gain (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | No FDSS (reference) | 5.42 |  | 7.31 |  |  |
|  | FDSS | 3.96 | 1.46 | 7.84 | -0.54 | 0.92 |
|  | FDSS-SE | 3.64 | 1.78 | 8.29 | -0.98 | 0.80 |
| Case 2 | No FDSS (reference) | 6.85 |  | 12.89 |  |  |
|  | FDSS | 5.84 | 1.01 | 13.50 | -0.61 | 0.40 |
|  | FDSS-SE | 4.41 | 2.44 | 15.01 | -2.12 | 0.33 |

Table 6: MPR-based Net Gain for FDSS and FDSS-SE compared to conventional DFT-s-OFDM (reference) in Case 1 and Case 2

|  |  | Raw CM (dB) | CM | MPR (dB) | ΔMPR | SNR (dB) | ΔSNR | Net Gain (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | No FDSS (reference) | 0.67 | 0.72 | 0 |  | 7.31 |  |  |
|  | FDSS | -0.03 | 0.22 | 0 | 0 | 7.84 | -0.54 | -0.54 |
|  | FDSS-SE | 0.18 | 0.22 | 0 | 0 | 8.29 | -0.98 | -0.98 |
| Case 2 | No FDSS (reference) | 2.00 | 2.22 | 1.5 |  | 12.89 |  |  |
|  | FDSS | 1.74 | 2.22 | 1.5 | 0 | 13.50 | -0.61 | -0.61 |
|  | FDSS-SE | 0.79 | 1.22 | 0.5 | 1 | 15.01 | -2.12 | -1.12 |

In Table 5, PAPR-based Net Gain of FDSS and FDSS-SE is improved by 0.3-0.9 dB compared to conventional DFT-s-OFDM, depending on the modulation scheme in the simulations. However, it can be observed that in our preliminary results, MPR-based Net gain is negative for both FDSS and FDSS-SE compared to the conventional DFT-s-OFDM waveform. Given that MPR-based performance metric typically reflects realistic coverage gain, we suggest to further study the net gain model by considering the input from RAN4, e.g., PA model.

It should be noted that although MPR calculation in our preliminary simulation results may not be fully optimized, we observed some link level performance degradation for FDSS and FDSS-SE compared to DFT-s-OFDM when same spectral efficiency is assumed. Therefore, we think careful investigation is needed to evaluate the potential gain of FDSS and/or FDSS-SE for 6GR waveform in RAN1.

Observation 5: For Case 1 (pi/2 BPSK), MPR-based Net Gain is -0.54 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 6: For Case 1 (pi/2 BPSK), MPR-based Net Gain is -0.98 dB for FDSS-SE compared to conventional DFT-s-OFDM.

Observation 7: For Case 2 (QPSK), MPR-based Net Gain is -0.61 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 8: For Case 2 (QPSK), MPR-based Net Gain is -1.12 dB for FDSS-SE compared to conventional DFT-s-OFDM.

  - Interlace OFDM

First, we summarize our assessments of Interlace OFDM as the following:

Table 7: Sharp’s views for Interlace OFDM.

|  | Positive | Negative | Comment |
| --- | --- | --- | --- |
| Coverage | x |  | Section 2.4.1.1. |
| UE energy efficiency |  |  | Section 2.4.1.2. |
| BS energy efficiency |  |  |  |
| Spectral efficiency |  | x | Section 2.4.1.3. |
| High speed tolerance | x |  | Section 2.4.1.4. |
| Scheduling flexibility | x |  | Section 2.4.1.5. |
| Integration with ISAC |  |  |  |
| MRSS compatibility |  |  |  |
| MIMO compatibility | x |  | Interlace OFDM is applied to MIMO. |
| User multiplexing | x |  | Interlace design improves the MU possibility by FDM. |
| Impact on initial access | x |  | Coverage extension leads to increasing connection possibility. |
| Transmitter/receiver complexity | x |  |  |

In Interlace OFDM, each modulation symbol is allocated on one subcarrier per M consecutive subcarriers, where we call the variable M an interlace factor (repetition factor). In Figure 6, we show the frequency-resource allocation of modulation symbols for Interlace OFDM on each symbol.

Figure 6: Frequency-resource allocation of modulation symbols for Interlace OFDM on each OFDM symbol when the interlace factor M is 2.

As shown in Figure 6, the frequency utilization efficiency for a UE using Interlace OFDM is degraded due to the interlace factor M compared to a wideband UE using CP-OFDM and DFT-s-OFDM. However, the number of subcarriers per resource block (RB) can be adjusted by the interlace factor, and we think this interlace allocation has four benefits: (1) power boosting, (2) mitigation of frequency offset and phase noise effects, (3) OFDM symbol repetition, (4) DMRS overhead reduction, and (5) BWP-level multiplexing.

Regarding (1) power boosting, note that the minimum frequency-resource for scheduled channel is one RB consisting of 12 subcarriers in 5G NR. For this reason, the transmission power in CP-OFDM and DFT-s-OFDM is distributed to at least 12 subcarriers in 5G NR. On the other hand, Interlace OFDM can enable the transmission power boosting under power constraints compared to CP-OFDM and DFT-s-OFDM as shown in Figure 7. This is because the signal can be allocated to 12 or less subcarriers by using the interlace factor M (i.e., NSCRB/M). Theoretically, 10*logM dB gain is achieved by Interlace OFDM compared to CP-OFDM and DFT-s-OFDM.

Figure 7: Example of power boosting by Interlace OFDM when the interlace factor M is 2.

Regarding (2) mitigation of frequency offset and phase noise effects, since each modulation symbol is allocated at every M subcarriers, the impact of inter-carrier interference on adjacent subcarrier can be ignored as shown in Figure 7. Furthermore, according to SID, “Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range)”, the single numerology is used in the same band. Even if the numerology for small SCS is configured, both frequency offset and phase noise effects can be mitigated by the interlace.

Regarding (3) OFDM symbol repetition, the interlace factor M is also expressed as the repetition factor M on the time domain as shown in Figure 8. At the transmitter side, the IFFT size can be regarded as 2048κ*2-μ/M and 10*logM dB repetition gain can be achieved theoretically. Furthermore, the slot length is not related to the repetition factor M. Therefore, multiple frequency-resource groups (e.g., BWPs) with different the repetition factor M can be multiplexed with each other on the same slot. Additionally, since the repetition also has a role of CP, the CP can be extended by the repetition factor M even if the numerology μ is large. Furthermore, for OFDM symbol repetition, the power boosting can be achieved within one slot. For this reason, Interlace OFDM can avoid the cross-slot design by slot-based repetition for power boosting such as TBoMS, DMRS bundling, power control of DMRS bundling and TBoMS, and so on.

Figure 8: OFDM symbol comparison between CP-OFDM and Interlace OFDM.

Regarding (4) DMRS overhead reduction, for Interlace OFDM, DMRS subcarriers are also sparsely distributed on the frequency domain depending on the interlace factor M. For example, when the interlace factor M is 12, the DMRS or data on each OFDM symbol is allocated per a PRB. For this, Interlace OFDM can reduce DMRS overhead and increase the number of multiplexed UEs on the frequency domain. However, if different UEs are multiplexed between the different interlaces (e.g., UE#0 is allocated on Subcarrier#0 and #2, and UE#1 is allocated on Subcarrier#1 and #3), the phase noise and frequency offset effect mitigation cannot be achieved due to interference from adjacent UEs. Furthermore, for sparse DMRS, the channel estimation accuracy is degraded.

Regarding (5) BWP-level multiplexing, multiple BWP corresponding to different interlace factors can be simultaneously multiplexed in the same band by Interlace OFDM. In 5G NR, since multiple numerologies can be configured for different BWP, they are hard to be simultaneously multiplexed with each other. In other words, although FFT size depends on OFDM symbol length determined by a numerology configuration, it is difficult to simultaneously perform FFT with different window sizes at the receiver side. On the other hand, as shown in Figure 7, each transmitter side can allocate each modulation symbol at every M subcarriers, but the receiver side considers a SCS determined by a single numerology configuration. Likewise, as shown in Figure 8, the transmitter side considers IFFT size dependently on the interlace factor M, but the receiver side considers FFT size independently from the interlace factor. For this reason, as shown in Figure 9, Interlace OFDM enables to simultaneously multiplex multiple BWPs corresponding to different interlace factors with each other.

Figure 9: Example of FDM of multiple frequency resource groups (e.g., BWP) with different interlace factors and the single numerology.

For these reasons, we think Interlace OFDM is technically beneficial for 6GR, and we support to study Interlace OFDM.

Observation 9: Interlace OFDM has the following advantages:

  - (1) Power boosting

  - (2) Mitigation of frequency offset and phase noise effects

  - (3) OFDM symbol repetition

  - (4) DMRS overhead reduction

  - (5) BWP-level multiplexing

Evaluation: Coverage

Here, we evaluate BLER performance by link-level simulation, where simulation parameters are specified in Appendix.

Figure 10: BLER performance of CP-OFDM, DFT-s-OFDM, Interlace OFDM, and Interlace DFT-s-OFDM.

In Figure 10, Interlace OFDM and Interlace DFT-s-OFDM outperform CP-OFDM and DFT-s-OFDM, respectively. This is because the interlace M=2 achieves 3dB SNR gain. For this reason, the coverage edge scenario, Interlace OFDM and Interlace DFT-s-OFDM are helpful under transmission power constraints.

Observation 10: Interlace OFDM and Interlace DFT-s-OFDM have SNR gain compared to CP-OFDM and DFT-s-OFDM, and they are helpful for the coverage edge scenario.

Evaluation: Energy efficiency

Regarding energy efficiency, we evaluate PAPR for CP-OFDM, DFT-s-OFDM, Interlace OFDM, and Interlace DFT-s-OFDM. In practice, the PA efficiency degrades as the average output power is backed off from saturation. This is because the DC power consumption of the PA, which is basically determined by bias current, remains nearly constant, but the RF output power decreases due to back-off. Therefore, the PA efficiency improvement can also improve the energy efficiency.

Figure 11: PAPR for CP-OFDM, DFT-s-OFDM, Interlace OFDM, and Interlace DFT-s-OFDM.

In Figure 11, Interlace DFT-s-OFDM can slightly reduce PAPR because the DFT spreading leads to the single carrier characteristics on the time domain and the number of subcarriers decreases by the interlace. Therefore, Interlace OFDM does not contributes significantly to the reduction in PA efficiency.

Observation 11: Interlace DFT-s-OFDM does not contributes significantly to the energy efficiency.

Evaluation: Spectral efficiency

For a given number of PRBs allocated to a UE, Interlace OFDM and Interlace DFT-s-OFDM for each UE have lower spectral efficiency than CP-OFDM and DFT-s-OFDM because the interlace reduces the allocated number of subcarriers available within each PRB. Theoretically, the spectral efficiency of the wideband UE is reduced by the interlace factor M. However, when different UEs are frequency-domain-multiplexed within M subcarriers, the overall spectral efficiency of CP-OFDM, DFT-s-OFDM, Interlace OFDM, and Interlace DFT-s-OFDM becomes equivalent.

Observation 12: Interlace OFDM and Interlace DFT-s-OFDM have lower spectral efficiency for a given number of PRBs allocated to a single UE compared to CP-OFDM and DFT-s-OFDM.

Evaluation: High speed tolerance

The maximum doppler shift fD can be calculated as vcfc, where v is speed, c is light speed, and fc is a carrier frequency. Furthermore, the normalized doppler shift α can be expressed as fD/Δf, where Δf is a subcarrier spacing. Considering the interlace factor M, α=fD/MΔf. Additionally, the effects of the normalized doppler shift depend on a modulation scheme. For example, α needs to be typically lower than 0.05, 0.03, and 0.02 for QPSK, 16QAM, and 64QAM, respectively. These correspond roughly to keeping ICI low enough for the modulation. When considering this requirement of α, the smallest one of 15/30/60/120/240/480 kHz SCS is derived as below:

Table 8: Comparison of legacy OFDM (CP-OFDM and DFT-s-OFDM) and Interlace OFDM in terms of doppler shift effects.

|  |  |  |  | Legacy OFDM |  | Interlace OFDM w/ M=2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| fc | speed | mod | fD [Hz] | Δfmin=fD/αmax | Smallest SCS | Δfmin=fD/Mαmax | Smallest SCS |
| 3.5 GHz | 3 km/h | QPSK | 9.722 | 194.4 Hz | 15 kHz | 97.2 Hz | 15 kHz |
| 3.5 GHz | 3 km/h | 16QAM | 9.722 | 324.1 Hz | 15 kHz | 162.1 Hz | 15 kHz |
| 3.5 GHz | 3 km/h | 64QAM | 9.722 | 486.1 Hz | 15 kHz | 243.1 Hz | 15 kHz |
| 3.5 GHz | 60 km/h | QPSK | 194.444 | 3,888.9 Hz | 15 kHz | 1,944.4 Hz | 15 kHz |
| 3.5 GHz | 60 km/h | 16QAM | 194.444 | 6,481.5 Hz | 15 kHz | 3,240.8 Hz | 15 kHz |
| 3.5 GHz | 60 km/h | 64QAM | 194.444 | 9,722.2 Hz | 15 kHz | 4,861.1 Hz | 15 kHz |
| 3.5 GHz | 300 km/h | QPSK | 972.222 | 19,444.4 Hz | 30 kHz | 9,722.2 Hz | 15 kHz |
| 3.5 GHz | 300 km/h | 16QAM | 972.222 | 32,407.4 Hz | 60 kHz | 16,203.7 Hz | 30 kHz |
| 3.5 GHz | 300 km/h | 64QAM | 972.222 | 48,611.1 Hz | 60 kHz | 24,305.6 Hz | 30 kHz |
| 28 GHz | 3 km/h | QPSK | 77.778 | 1,555.6 Hz | 15 kHz | 777.8 Hz | 15 kHz |
| 28 GHz | 3 km/h | 16QAM | 77.778 | 2,592.6 Hz | 15 kHz | 1,296.3 Hz | 15 kHz |
| 28 GHz | 3 km/h | 64QAM | 77.778 | 3,888.9 Hz | 15 kHz | 1,944.4 Hz | 15 kHz |
| 28 GHz | 60 km/h | QPSK | 1,555.556 | 31,111.1 Hz | 60 kHz | 15,555.6 Hz | 30 kHz |
| 28 GHz | 60 km/h | 16QAM | 1,555.556 | 51,851.9 Hz | 60 kHz | 25,925.9 Hz | 30 kHz |
| 28 GHz | 60 km/h | 64QAM | 1,555.556 | 77,777.8 Hz | 120 kHz | 38,888.9 Hz | 60 kHz |
| 28 GHz | 300 km/h | QPSK | 7,777.778 | 155,555.6 Hz | 240 kHz | 77,777.8 Hz | 120 kHz |
| 28 GHz | 300 km/h | 16QAM | 7,777.778 | 259,259.3 Hz | 480 kHz | 129,629.6 Hz | 240 kHz |
| 28 GHz | 300 km/h | 64QAM | 7,777.778 | 388,888.9 Hz | 480 kHz | 194,444.4 Hz | 240 kHz |

In the above evaluation, there are cases that 120 kHz SCS is not enough for the legacy OFDM such as 5G NR CP-OFDM and DFT-s-OFDM in 28GHz carrier frequency. Particularly, when the speed is 300km/h, at least 240kHz SCS is needed to avoid ICI. Although there are cases that 30kHz SCS is not enough for the legacy OFDM in 3.5GHz carrier frequency, lower modulation order can address this issue. Likewise, QPSK can be used to prevent it from ICI even when Interlace OFDM is used and the speed is 300km/h in 28GHz carrier frequency.

Observation 13: Interlace OFDM and Interlace DFT-s-OFDM are more robust to high speed compared to CP-OFDM and DFT-s-OFDM.

Evaluation: Scheduling flexibility

Compared to CP-OFDM and DFT-s-OFDM, Interlace OFDM complicates scheduling but improves scheduling flexibility. For Interlace OFDM, data or DMRS on each symbol is sparsely allocated on the frequency domain, and we think there are two ways to uses the blank subcarrier as the following:

  - Way 1: Multiplexing within one interlace group.

  - Way 2: Multiplexing by different interlace group.

In Figure 7, UE#1 and UE2 have the same interlace group (e.g., 0, 2, 4,…), and they are CDMed in BWP#1. UE#3-#6 have different interlace group (e.g., 0, 2, 4,… and 1, 3, 5,…), and they are CDMed and FDMed in BWP#2. In BWP#1, Way 1 can maintain robustness to both frequency offset and phase noise and boost UL transmission power for each UE, but it has lower spectral efficiency due to restriction of FDM. In BWP#2, Way 2 can boost UL transmission power for each UE and has the same spectral efficiency as CP-OFDM and DFT-s-OFDM, but cannot mitigate frequency offset and phase noise effects. Furthermore, Way 2 can increase the number of multiplexed UEs while reducing DMRS overhead. For example, the interlace factor M=12, up to 12 UEs are multiplexed on different subcarriers on a PRB.

Figure 7: Example of intra-BWP UE multiplexing when the interlace factor M is 2 for both BWP#1 and BWP#2.

In our view, both Way 1 and Way 2 are beneficial by using one of them depending on a use case. For this reason, Interlace OFDM complicates scheduling but improves scheduling flexibility.

Observation 14: Interlace OFDM complicates scheduling but improves scheduling flexibility.

Observation 15: In Interlace OFDM, one of the following ways can be selected depending on a use case:

  - Way 1: Multiplexing within one interlace group.

    - Pros: Power boosting, DMRS overhead reduction, and increasing the number of multiplexed UEs on different UEs can be achieved while keeping the same spectral utilization as the legacy CP-OFDM/DFT-s-OFDM.

    - Cons: Phase noise and frequency offset effects cannot be mitigated.

  - Way 2: Multiplexing between different interlace groups.

    - Pros: Phase noise and frequency offset effects can be mitigated.

    - Cons: Spectral utilization is reduced.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: For 6GR waveform, RAN1 should NOT study DL DFT-s-OFDM.

Proposal 2: For the evaluation of multi-layer UL DFT-s-OFDM, Net Gain should be used and the assumed precoder should be reported.

Observation 1: For Case 1 (pi/2 BPSK), PAPR improvement at 1% CCDF is 1.46 dB and Raw CM improvement at 1% CCDF is 0.70 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 2: For Case 1 (pi/2 BPSK), PAPR improvement at 1% CCDF is 1.78 dB and Raw CM improvement at 1% CCDF is 0.49 dB for FDSS-SE compared to conventional DFT-s-OFDM.

Observation 3: For Case 2 (QPSK), PAPR improvement at 1% CCDF is 1.01 dB and Raw CM improvement at 1% CCDF is 0.26 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 4: For Case 2 (QPSK), PAPR improvement at 1% CCDF is 2.44 dB and Raw CM improvement at 1% CCDF is 1.22 dB for FDSS-SE compared to conventional DFT-s-OFDM.

Observation 5: For Case 1 (pi/2 BPSK), MPR-based Net Gain is -0.54 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 6: For Case 1 (pi/2 BPSK), MPR-based Net Gain is -0.98 dB for FDSS-SE compared to conventional DFT-s-OFDM.

Observation 7: For Case 2 (QPSK), MPR-based Net Gain is -0.61 dB for FDSS compared to conventional DFT-s-OFDM.

Observation 8: For Case 2 (QPSK), MPR-based Net Gain is -1.12 dB for FDSS-SE compared to conventional DFT-s-OFDM.

Observation 9: Interlace OFDM has the following advantages:

  - (1) Power boosting

  - (2) Mitigation of frequency offset and phase noise effects

  - (3) OFDM symbol repetition

  - (4) DMRS overhead reduction

  - (5) BWP-level multiplexing

Observation 10: Interlace OFDM and Interlace DFT-s-OFDM have SNR gain compared to CP-OFDM and DFT-s-OFDM, and they are helpful for the coverage edge scenario.

Observation 11: Interlace DFT-s-OFDM does not contributes significantly to the energy efficiency.

Observation 12: Interlace OFDM and Interlace DFT-s-OFDM have lower spectral efficiency for a given number of PRBs allocated to a single UE compared to CP-OFDM and DFT-s-OFDM.

Observation 13: Interlace OFDM and Interlace DFT-s-OFDM are more robust to high speed compared to CP-OFDM and DFT-s-OFDM.

Observation 14: Interlace OFDM complicates scheduling but improves scheduling flexibility.

Observation 15: In Interlace OFDM, one of the following ways can be selected depending on a use case:

  - Way 1: Multiplexing within one interlace group.

    - Pros: Power boosting, DMRS overhead reduction, and increasing the number of multiplexed UEs on different UEs can be achieved while keeping the same spectral utilization as the legacy CP-OFDM/DFT-s-OFDM.

    - Cons: Phase noise and frequency offset effects cannot be mitigated.

  - Way 2: Multiplexing between different interlace groups.

    - Pros: Phase noise and frequency offset effects can be mitigated.

    - Cons: Spectral utilization is reduced.

References

- [1]	RP- 251881, New SID: Study on 6G Radio, June, 2025.

Appendix

Table A1: Simulation assumption for FDSS and FDSS-SE evaluations

| Parameters | Values |
| --- | --- |
| Frequency | 3.5 GHz |
| SCS configuration | 30 kHz |
| Number of PRBs | 8 |
| Number of OFDM symbols | 14 |
| Number of Tx/Rx ports | 1Tx 1Rx |
| Number of layers | 1 |
| DMRS configuration | 5GNR Configuration type 1 |
| Channel model | CDL-E |
| Modulation | (Table 4) |
| Code rate | (Table 4) |
| Coding scheme | LDPC |
| Channel estimation | Realistic |
| Channel equalization | MMSE |
| Bit detector | Soft-detection (LLR) |

Table A2: Simulation assumption for Interlace OFDM evaluation

| Parameters | Values |
| --- | --- |
| Frequency | 3.5 GHz |
| SCS configuration | 30 kHz |
| Number of PRBs | 51 |
| Number of OFDM symbols | 14 |
| Number of Tx/Rx ports | 1Tx 1Rx |
| Number of layers | 1 |
| DMRS configuration | 5GNR Configuration type 1 |
| Channel model | CDL-E |
| Modulation | QPSK |
| Code rate | 679/1024 |
| Coding scheme | LDPC |
| Channel estimation | Realistic |
| Channel equalization | MMSE |
| Bit detector | Soft-detection (LLR) |