3GPP TSG RAN WG1 #123	 R1-2509329

Dallas, United States, November 17th – November 21st, 2025

Agenda item:       10.3.2.1

Source:	IIT Kanpur

Title:	Discussion on R2D signals, channels, waveform and procedures

Document for:     Discussion and Decision

# Introduction

In Rel-19, Ambient IoT (A-IoT) was specified as a new IoT technology characterized by ultra-low complexity, ultra-low power consumption, and the capability for battery-less operation, targeting very low-end IoT applications in indoor scenarios and supporting passive device types. In RAN#108, a new SID [4] was approved to study support for outdoor deployment and active A-IoT device types. In the RAN1#122 and RAN1#122bis meetings, the enhancements of the air interface for outdoor AIoT operation were discussed. In this contribution, we discuss the general aspects of air interface design, including waveform and modulation, as well as multiple access for the R2D link.

# Discussion

According to the SID, the study scope for the active device type is limited to D4T1, adhering to DO-DTT, DT, and DO-A traffic types, to address use cases such as outdoor inventory, outdoor sensing, and outdoor command. The SID further assumes similar device architectures for the active device type, namely Device 2b and Device C.

2.1 Waveform for R2D

For D4T1 deployment in A-IoT R2D transmission, it is recommended to reuse the Rel-19 OOK modulation scheme with DFT-s-OFDM waveform. This approach ensures a harmonized design, maintains compliance with legacy standards, and simplifies implementation.

Proposal 1: Reuse the Rel-19 OOK-4 waveform for R2D D4T1 deployment scenarios.

In RAN1#122, the following was agreed for R2D M value [1]:

Agreement For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: - Addition of M=1 - Addition of M=32 - Removal of M=24

As agreed in the previous meeting, the set of M values for R2D transmissions has been revised: M = 24 has been removed, and M = 32 and M = 1 have been added. In our view, coverage performance is more important than peak data rate in outdoor scenarios, as devices may struggle to reliably achieve the high data rates associated with M = 32. Therefore, M = 1 is critical for ensuring robust coverage. The longer chip duration provides inherent time diversity, a significant advantage in outdoor environments.

Proposal 2: For Rel-20 AIoT, support M=1 for R2D transmission.

2.2 Line coding, FEC, Repetition, and Scrambling for R2D Link

Line coding

In Rel-19, Manchester line coding was used for R2D transmissions. The self-clocking property of Manchester codes enabled passive devices to calibrate SFO drift. However, Manchester coding inherently limits the throughput to a 1/2  rate. Devices of type 2b/C possess improved synchronization capabilities and therefore do not require Manchester coding for calibration.

Observation 1: Rel-19 Manchester coding for R2D enables SFO calibration, but it limits throughput to half. Devices 2b/C have improved synchronization and do not require Manchester coding.

Proposal 3: Manchester coding for Device 2b/C PRDCH transmissions should not be supported.

FEC

In RAN1#122b, we achieved the following agreements about FEC [2]

Agreement For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects: - Coding schemes - LTE TBCC - Tailed convolutional code with the same polynomial as LTE TBCC - RM code - FEC code rates: 1/2, 1/3, 1/4 Agreement Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver Combination of alternatives can be studied

In Rel-19, considering the decoding complexity and power consumption, Device 1 did not support R2D FEC. In Rel-20, Device 2b/C has a higher power budget and requires extended coverage; therefore, the need for FEC was further evaluated. In our evaluation, as shown in the results presented in our contribution [3], both Device 2b and Device C can achieve the coverage requirement of [50-500] m for the R2D link without FEC. Hence, there is no strong technical justification to introduce FEC support for these devices.

Proposal 4: For Rel-20 AIoT R2D, FEC should be de-prioritized.

Repetition for R2D

As discussed in the previous meeting, there is a need to study the feasibility and benefits of repetition on the R2D link. The following agreement was reached in RAN1#122 [1]:

Agreement Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types: - Block-level repetitions - Bit-level Type-2 repetitions - Chip-level repetitions (in case of M=1 with FEC if applied)

Block-level repetition is the most effective approach, as it provides maximum diversity gains by allowing the receiver to combine multiple copies of the data. However, this method requires devices to allocate larger memory to store all repeated copies, and it introduces processing latency, as decoding cannot begin until all repetitions are received. Considering the enhanced capabilities and increased storage of Device 2b/C, block-level repetition remains a viable option.

On the other hand, bit-level repetition offers lower diversity gains but reduces memory requirements. Furthermore, combining bit-level repetition with M = 1 can effectively extend the chip duration, thereby improving coverage performance in challenging outdoor scenarios.

Proposal 5: At least the bit-level should be considered for R2D transmissions. Additionally, the necessity of block-level repetition for R2D transmissions should be further studied.

Scrambling for R2D

In the last meeting, interest was expressed in using R2D scrambling to mitigate interference among transmissions from different readers. Scrambling randomizes interference, improving communication reliability and efficiency by optimizing bit patterns for transmission, synchronization, and decoding without increasing bandwidth. It enhances effective entropy and signal quality, supporting robust data communication from an information-theoretic perspective. In the absence of Manchester coding, scrambling can also prevent long sequences of 0’s or 1’s, thereby improving detection probability.

Proposal 6: It is proposed to adopt R2D scrambling to improve interference resilience and detection performance of R2D transmissions for device type 2b/C.

2.3 Multiple Access for R2D Link

As mentioned earlier, for a larger coverage area, the number of devices to be served by a reader in outdoor deployments is also higher. To cater to high-density device deployments, FDM can be considered for outdoor scenarios. In the Rel-19, FDM for R2D was studied for Device 2b, and various observations were captured in section 6.1.1.8 in TR 38.769 as follows [5]:

6.1.1.8 R2D multiplexing For R2D, time-domain multiplexing is the baseline. Code-domain multiplexing is not considered for device 1/2a/2b. Frequency-domain multiplexing is not considered for the devices with an RF-ED receiver (see Clause 5). For device 2b with IF-ED or ZIF receivers, the study considered the following technical aspects: Table 6.1.1.8-1: Observations on the feasibility and necessity of FDM for Device 2b Aspects to be considered for feasibility/benefit; Observations. Inventory completion time; Sources [20], [39], [21], [29] state that FDM is beneficial to reduce the inventory completion time, especially considering more devices per reader due to the larger maximum distance for Device 2b. Source [62] state that inventory latency reduction would be limited, due to difficulty of allocating frequency resources efficiently for Msg2 by reader with uncertainty of number of successful Msg 1, and difficulty of informing an A-IoT device R2D frequency location other than Msg2.. Device implementation; Sources [35], [44], [20], [39], [29] state that channel selection may be performed by a narrowband filter (IF filter or BB filter) after the mixer for Device 2b, if the LO accuracy is sufficiently good. Source [71] states that narrowband RF filtering at device side to realize R2D FDM would be challenging considering reception performance and complexity, while such filtering would also limit the deployment scenario supported by device. Sources [51], [41], [23], [19], [22] state that it would be challenging for a device using an RF-ED receiver architecture to distinguish the different incoming signal fall into the RF BW without narrowband RF filtering which may cause increasing device implementation complexity and power consumption. Source [20] states that the larger R2D responses are harder for the devices to process in the case of TDM+FDM/TDM only for D2R/R2D, respectively.. Spectrum utilization; Source [51] state that the spectral efficiency may be impacted by the guard band across the FDMed R2D transmissions to multiple devices. Source [56] state that the spectrum utilization can still be higher for non-RF-ED based devices if FDM is used, despite guard bands. Source [62] state that spectrum efficiency improvement would be limited, due to difficulty of allocating frequency resources efficiently for Msg2 by reader with uncertainty of number of successful Msg 1, and difficulty of informing an A-IoT device R2D frequency location other than Msg2.. Coverage (in the case of single reader); Source [22] states the R2D link budget of a reader is decreased due to the power splitting between the parallel R2D channels. Source [20] states the coverage target of Device 2b is still larger than that of Device 1 (with RF-ED architecture).. Reader implementation (in the case of single reader); Source [22] states that additional interference suppression may be needed to deal with the intermodulation between the parallel R2D transmissions.. Harmonized design for all devices; Sources [43], [26], [36], [61] state that it is not appropriate to include FDM only for Device 2b, while Device 1 and 2a cannot support it. Source [62] state that non-harmonized resource allocation for different device types complicates the system design. Source [56] state that a deployment supporting a combination of RF-ED devices and non-RF ED devices can be harmonized by TDMA’ing different types of R2D time slots, where some time slots can support only a single frequency occasion while other time slots support multiple frequency occasions..

For scenarios with a large number of devices, a more efficient inventory and random-access procedure is required to meet the latency requirements for completion time. FDM should be considered for R2D transmission.

Proposal 7: Enable FDM for R2D transmission, at least for active device types.

2.3 SIP of R2D transmissions

At the RAN1#122bis meeting, the following agreements were reached [2].

Agreement For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP: - Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence). - Option 2: Reuse the Rel-19 SIP.

In our understanding, a binary sequence-based SIP, compared to the Rel-19 SIP, would provide better correlation properties in noisy and multipath environments, thereby satisfying the requirements of Rel-20 for long-distance outdoor scenarios.

Proposal 8: Support binary sequence–based SIP for the Rel-20 R2D link. Further evaluation is required to down-select between Golay and M-sequence candidates.

FFS: Detection performance, Number of sequences, and sequence length.

2.3 CFO Calibration

The following was agreed in RAN1#122-bis [2],

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

For CFO estimation and LO calibration in D2R transmission, Option A is considered sufficient for device types 2b and C. This approach offers a simple implementation, where the device transmits a continuous, unmodulated sine wave for a predefined duration prior to the D2R transmission. This tone serves as both a reference for carrier frequency offset estimation and an anchor for frequency-domain resource allocation in the subsequent D2R transmission.

Proposal 9: Adopt Option A, using an unmodulated single-tone sinusoid signal for CFO estimation and LO calibration.

Regarding the transmission timing of the CFO calibration signal, our understanding is that D2R transmissions are scheduled through control information over PRDCH. Therefore, the CFO calibration signal does not need to be transmitted periodically. Instead, it is reasonable that the CFO calibration signal be transmitted in association with the D2R scheduling information.

Proposal 10: The transmission timing of the CFO calibration signal should be linked either to the L1 R2D control information or to the PRDCH data payload carrying the D2R scheduling information.

# Conclusion

This paper provides the following observations and proposals:

Proposal 1: Reuse the Rel-19 OOK-4 waveform for R2D D4T1 deployment scenarios.

Proposal 2: For Rel-20 AIoT, support M=1 for R2D transmission.

Observation 1: Rel-19 Manchester coding for R2D enables SFO calibration, but it limits throughput to half. Devices 2b/C have improved synchronization and do not require Manchester coding.

Proposal 3: Manchester coding for Device 2b/C PRDCH transmissions should not be supported.

Proposal 4: For Rel-20

Proposal 5: At least the bit-level should be considered for R2D transmissions. Additionally, the necessity of block-level repetition for R2D transmissions should be further studied.

Proposal 6: It is proposed to adopt R2D scrambling to improve interference resilience and detection performance of R2D transmissions for device type 2b/C.

Proposal 7: Enable FDM for R2D transmission, at least for active device types.

Proposal 8: Support binary sequence–based SIP for the Rel-20 R2D link. Further evaluation is required to down-select between Golay and M-sequence candidates.

FFS: Detection performance, Number of sequences, and sequence length.

Proposal 9: Adopt Option A, using an unmodulated single-tone sinusoid signal for CFO estimation and LO calibration.

Proposal 10: The transmission timing of the CFO calibration signal should be linked either to the L1 R2D control information or to the PRDCH data payload carrying the D2R scheduling information.

# References

[1] Draft_Minutes_report_RAN1#122

[2] Draft_Minutes_report_RAN1#122bis

[3] R1-2509327 “Evaluation for Rel-20 AIoT” – RAN1#123 Dallas, US

[4] RP-251884, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, Moderator (RAN1 Vice-Chair), RAN#108.

[5] TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”.