3GPP TSG-RAN WG1 Meeting #123	R1-2509626

Dallas, USA, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0380 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Rel-19 EDT for IoT-NTN for LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FUTUREWEI |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the CB-Msg3 EDT procedure the HARQ-ACK resource is indicated by MAC CMR, so the HARQ-ACK resource field in the DCI should be reserved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reserve HARQ related fields for CB-RNTI similar to RA-RNTI. In addition to the HARQ-ACK resource, these include NDI (for NB-IoT) and HARQ process number/NDI/DAI (for eMTC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity over which HARQ resource infication to use in the CB-Msg3 EDT procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.12, 6.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.3.3.1.12	Format 6-1A

#### ------- text omitted -------

When the format 6-1A CRC is scrambled with a RA-RNTI or a CB-RNTI, then the following fields among the fields above are reserved:

-	HARQ process number

-	New data indicator

-	Downlink Assignment Index

-	HARQ-ACK resource offset

#### ------- text omitted -------

#### 6.4.3.2	DCI Format N1

#### ------- text omitted -------

When the format N1 CRC is scrambled with a RA-RNTI or a CB-RNTI or a G-RNTI, then the following fields among the fields above are reserved for RA-RNTI or CB-RNTI, and not present for G-RNTI:

-	New data indicator

-	HARQ-ACK resource

#### ------- text omitted -------