3GPP TSG-RAN WG1 Meeting #123	R1-2509636

Dallas, TX, USA, November 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0172 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR_LPWUS-Core |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-11-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Incorrect higher-layer parameter names for configuring LPWUS root sequences |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correcting the higher-layer parameter names |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misaligned and incomplete specification of LPWUS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.4.4.1.1	Generation of rZC,m(n)

The sequence rZC,m(n) is defined by

rZC,m(n)=xq(n+ncs) mod NZC

xqi=e-jπqi(i+1)NZC

n=0,1,…,MZC-1

where

-	NZC is the largest prime number such that NZC<MZC

-	MZC=NscWUSMWUS

The root sequence number q∈1, …, NZC-1 is configured by the higher-layer parameter root1 or root2 in lpwus-OverlaidSeqRoots, where root1 is used for the root sequence number when cmP=0, and root2 is used otherwise.cmP∈0,1The cyclic shift ncs is given by

ncs=cm mod PNZCP

P=NseqNroot

where

-	Nseq is the number of sequences configured by the higher-layer parameter lpwus-OverlaidSeqNum or lpwus-OverlaidSeqNum-SCS-60kHz or lpwus-OverlaidSeqNum-SCS-120kHz

-	Nroot=2ϵ1,2 if root2 in lpwus-OverlaidSeqRoots is configured, otherwise Nroot=1

The sequence number cm=0 if Nseq=1, otherwise it is given by

cm=i=0δ-1f1(i+δm)2δ-1-i

δ=log2Nseq

m=0,1,…,E1δ-1

where

-	f1i and E1 are given by clause 7.4.2.2 of [4, 38.212]