3GPP TSG RAN WG1 #124			R1-2600964

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	10.7.1

Source: 	LG Electronics

Title: 	Discussion on NTN specific design for GNSS based operation

Document for:	Discussion and decision

Introduction

In RAN1#122bis meeting [1], it is agreed that the aspects to consider for supporting NTN as follows:

Agreement - The aspects to consider for supporting NTN include, but not limited to - Initial access, including cell search and SSB periodicity - Coverage - Duplexing - Capacity - Signalling overhead - GNSS-less/resilient/based operation - Large/varying doppler and propagation delay - Beamforming / beam management / beam hopping Note: - High-level aspects to consider to enable lower CAPEX/OPEX with respect to current networks include, but not limited to - UE/NW implementation complexity - UE/NW energy efficiency - MRSS - Spectrum efficiency

This contribution provides our views on NTN specific requirements and design especially for GNSS based operation.  In our view, it is also important to have unified 6GR NTN design for both GNSS based operation and GNSS less operation.

Discussion

During the RAN1 discussions for 6GR, there are some lessons from NR design. In NR, a lot of features are introduced for the same purpose which makes the system complicated and the cost increased. Meanwhile, in 6GR NTN, there are mainly two streams: one is the GNSS based operation and the other is the GNSS less operation. If we do not consider the unified design for these streams, the specification workload and the system complexity would be high even in 6GR. Therefore, even though we start studying the NTN specific requirements and design for GNSS based operation, we need to keep in mind the solutions or method should be also workable for the GNSS less scenario.

Proposal 1: RAN1 strives to design unified solution for both GNSS-based and GNSS-less operation.

  - GNSS based operation

In NTN, due to the very large altitude of the satellite, the propagation delay and the RTT (round-trip time) are much higher compared to TN scenario. Meanwhile, for a given cell or beam footprint, the differential propagation delay or RTT would be limited. In case of NGSO, since the satellite is moving, the excessive number of TA commands would be needed to cover the variations on the RTT.

In 5G NR NTN, on top of the UL timing adjustment based on TA command, the UL timing adjustments based on common TA and UE-specific TA are introduced. The common TA is used to cover the distance between the satellite position and the ULRP (UL synchronization reference point), and the UE-specific TA is used to cover the distance between the UE position and the satellite position. For the UE-specific TA, it was assumed that the UE have the GNSS capability to estimate its position until Rel-19 NR NTN. In this case, the UE can estimate the RTT variations considering the earth rotation and the satellite moving and the its own moving. On the other hand, 6G NTN will support the UE without the GNSS capability. In this case, the definition or the usage of the UE-specific TA would need to be modified accordingly. For instance, the UE-specific TA could be determined based on the reference UE location and/or TA formula provided by network.

Proposal 2: Common TA and UE-specific TA as defined in 5G NR NTN are supported as the basis for 6GR NTN.

- Enhancement/modification on UE-specific TA at least for GNSS-less/resilient operation will be studied as potential additions.

Due the large RTT in NTN scenario, the UE will apply the large timing advance for UL transmission. In this case, on top of the UL timing indicated by the scheduling DCI, the concept of K_offset was introduced to cover the large RTT. To be specific, UL grant-to-PUSCH timing will be determined by the summation of K_2 indicated by the scheduling DCI and K_offset. In NR NTN, SIB for NTN (i.e., SIB19) provides the value of cell-specific K_offset. If the cell-specific K_offset is set for the worst case for a satellite (e.g., minimum elevation angle), the NW does not need to update the value of the cell-specific K_offset regardless of the satellite moving, but it will cause the unnecessary latency during the initial access even though the actual largest RTT of a certain cell is relatively small in some time. Alternatively, if the UE-specific differential K_offset is used to update the value of K_offset due to the satellite moving, the excessive number of MAC CE would be needed. In our understanding, in 6GR NTN, it would be necessary to consider the update of the value of cell-specific K_offset considering the satellite position as shown in Figure 1. It would be useful to reduce the latency for the initial access opportunistically depending on the satellite position. To do this, it is necessary to revisit how to update the SIB-NTN (e.g., SIB19) before the UL synchronization validity timer expires. Meanwhile, since the reference location of a cell could be determined differently across different cells and it will be independent on the satellite position, it can be considered that the UE determines the value of the cell-specific K_offset based on the reference location of a cell and the satellite position.

Figure 1: Example of updating cell-specific K_offset based on the satellite position.

Next, since the exact value of RTT between the UE position and the satellite position will be different across different UEs, the UE-specific K_offset can be determined by subtracting the differential K_offset value provided by the MAC CE from the cell-specific K_offset. For the UE-specific K_offset, since the gNB needs to know how amount of the time gap between DL reception and UL transmission is needed for a UE, it was supported that the UE performs the TA reporting during the initial access and when the triggering event occurs. To be specific, UE can report its current TA value with a granularity of 1msec when the variation between the current estimate of the TA value and the last reported TA value is equal to or larger than the configured threshold. Meanwhile, since the UE-specific K_offset will be determined based on the last reported TA value, it would be possible that the UE cannot meet the UL transmission preparation time with the current TA value. In this case, the TA reporting would not be possible as shown in Figure 2. To mitigate this situation, in 6GR NTN, it would be necessary to have the fallback operation with respect to K_offset. For instance, if the PDSCH or PUSCH is scheduled by a DCI in CSS, for the corresponding PUCCH TX timing or the PUSCH TX timing, the cell-specific K_offset will be applied instead of the UE-specific K_offset even for the C-RNTI, respectively.

Figure 2: Example of TA reporting failure due to the old-fashion UE-specific K_offset.

Next, considering the transparent payload, the ULRP always does not need to be at GW or at gNB on the ground. In this case, in NR NTN, the concept of k_mac was introduced since the DL frame could be not aligned with UL frame at the gNB.

In Rel-19 NR NTN, it was discussed how to support the half-duplex UE in NR NTN. To be specific, since the NW may not know the exact value of the UE-specific TA value, the NW may not know whether the DL reception and the UL transmission are overlapped in the same time at the UE side. In Rel-19 NR NTN, rather than mitigating this situation, it was defined the UE behavior when the overlapping occurs. One simply way would be applying the GNSS less operation for the half-duplex UE. In this case, since the NW will control all the TA terms for the UE, the collisions will be managed by the NW as well. Moreover, considering that the half-duplex UE will be the low-cost UE, it would be reasonable that these UE have not GNSS capability.

Proposal 3: K_offset and k_mac as defined in 5G NR NTN are supported as the basis for 6GR NTN.

- Enhancement/modification on K_offset and its signaling will be studied as potential additions based on the satellite location and UE location.

Proposal 4: TA report as defined in 5G NR NTN is supported as the basis for 6GR NTN.

- Enhancement/modification on TA report and its triggering condition will be studied as potential additions.

For the GNSS based operation in NTN, the UE will basically use the its position and the satellite position, and the satellite position will be provided by the NW in a form of the ephemeris information. In NR NTN, the ephemeris information is a part of the SIB-NTN (e.g., SIB19), and the UE is required to reacquire the SIB-NTN before the UL synchronization validity timer expires for the UL transmission regardless of whether the UE receives SI change notification from NW. In our understanding, the components of the ephemeris information do not need to be changed compared to the NR NTN.

Meanwhile, in NR NTN, to acquire the NTN configuration including the ephemeris information, the UE needs to receive the SIB-NTN (e.g., SIB19), and the UE needs to receive SIB1 to receive SIB-NTN since the SI scheduling information is provided by SIB1. Considering the UE processing time for SIB1 reception and acquisition, the SIB-NTN would not be transmitted right after the SIB1 transmission before the SI scheduling information is available at the UE side. At least in GNSS based operation, since UE cannot start the PRACH transmission before the acquisition of the SIB-NTN including the ephemeris information, it would be necessary to study how to efficiently reduce the latency for SIB-NTN acquisition.

Moreover, considering TN-NTN harmonization, it would be necessary to align the same set of SIBs for the cell access for both TN and NTN. In NR, for TN cell access, the UE need to acquire at least SIB1, but for NTN cell access, the UE needs to acquire SIB19 in addition to SIB1. In 6GR, it can be considered that for cell access regardless of the cell type is TN or NTN, the UE would need to acquire SIB1 (and/or SIB2 depending on the SIB1/2 contents design).

In NR NTN, even though the UE receives the ephemeris information, the UE may need to receive the ephemeris information and the corresponding epoch time before the UL synchronization validity timer expires. In our understanding, even though the UE knows the satellite orbit information based on the received ephemeris information, the clock speed could be different between UE and NW. In this case, as the time goes, the estimated satellite position at the UE side will be inaccurate. So, for the calibration purpose, the UE will reacquire the new epoch time and the ephemeris information. Meanwhile, before receiving the new epoch time and the ephemeris information, the NW will control TO by using the existing TA command, and it will effectively reduce the timing error due to the clock speed misalignment. In this case, if the UE try to recalculate the UE-specific TA and common TA based on the new epoch time and the ephemeris information, the timing adjustment based on the TA command will make another error. In 6GR, these aspects also need to be carefully investigated.

Proposal 5: Ephemeris information as defined in 5G NR NTN is supported as the basis for 6GR NTN.

- Enhancement/modification on SIB-NTN will be studied as potential additions.

- Enhancement/modification on UL timing adjustment procedure based on SIB-NTN will be studied as potential additions.

In the near future, it is expected that the NR NTN is widely deployed in the world. In this case, even for the 6GR NTN, the migration from NR NTN and 6GR NTN and the coexistence between NR NTN and 6GR NTN would need to be considered. For the NTN MRSS, it can be considered that a single satellite supports both NR NTN and 6GR NTN. In this case, it would be necessary to study whether or how to share the NTN configuration including the ephemeris information and the beam management between different RAT.

In our view, it would be possible to further consider the MRSS between TN and NTN as well. Since the DL/UL frame boundary for NTN on the ground will be time varying due to the satellite moving, the management for the guard time and the guard band need to be carefully designed.

Proposal 6: Study MRSS (multi-RAT spectrum sharing) for NTN operation.

- FFS: Whether/how to share NTN configuration and beam management between different RAT.

In NR NTN, it was observed that the level of the RSRP measurements for a certain cell area is compatible, so the RSRP measurement-based operation could be inefficient in NTN scenario. Therefore, it was introduced that location-based operation. To be specific, UE can perform CHO based on its UE location and the reference location of the serving cell or the target cell. In addition, in NR NTN, there were simplified or optimized procedures by using the UE position. For instance, the UE can select a subset of SMTC for the neighbor cell measurement based on the UE location.

In 6GR, RAN1 can further study the simplified or optimization procedures based on the UE position and/or the satellite position. For a given cell, depending on the satellite position, the RTT and the level of pathloss will be different. In this case, the relevant parameters such as UL timing candidates and the repetition number would need to be updated. In NR, RRC reconfiguration can be used to update the relevant parameters, but it will require a huge latency and the inefficient fallback operation will be used during the RRC reconfiguration period since the NW and UE may have different understanding on when the new parameter will be applied. On the other hand, if the RRC configurations or higher layer parameters can be updated based on the satellite position or timer, it will reduce the signaling overhead and the latency for the RRC reconfigurations.

Another example is that the beam management could be simplified or optimized based on the UE location or satellite location. In NR, for the beam management, NW may perform beam sweeping for the reference signal transmissions, and the UE will report the beam-related information. Then, the NW can determine the best or optimal beam for the UE. In NTN scenario, due to the large RTT, this kind of beam management would be inefficient since it will cause huge latency and the beam-related information could be old-fashioned. Instead, the UE can use its location and the reference location for each beam provided by NW for the beam selection. Then, the UE can skip performing RSRP measurements for a number of beams, and the UE can simply report the beam or beam footprint based on its location.

As mentioned before, it was observed that the level of the RSRP measurements would be compatible for a certain cell or beam area within a certain duration of time. In other words, if the UE know the optimistic RSRP value, it would be possible that the UE determines whether the current RX spatial setting is optimal or not based on the estimated RSRP value. If the NW provide the reference RSRP value, the UE can adjust its RX spatial setting so that the difference between the estimated RSRP value and the reference RSRP value is sufficiently small.

Proposal 7: Study UE procedure based on UE position and NTN node position including, e.g.,

- Neighbor cell measurement management

- Higher layer parameter or configuration adaptation

- Fast beam management

Meanwhile, since the location-based operation would be no longer valid for the GNSS less operation, it would be necessary to carefully investigate the benefits and the necessity to avoid the separate design between GNSS based operation and the GNSS less operation.

  - Waveform and frame structure

In NTN scenario, a single satellite can serve few hundreds or thousands of cells or beams, and due to the limitation of RF, only few cells or beams can be activated in a time. Meanwhile, the activated cells or beams will share the total EIRP of the satellite. In this situation, when the EIRP is reduced due to the power backoff for the high PAPR, the number of activated cells or beams could be further reduced, and the coverage ratio of the satellite will be problematic.

According to TR38.821, the EIRP is determined with an assumption that the power backoff is [5] dB. For instance, if the power backoff is set to 5 dB, then the number of active cells or beams and the coverage ratio of the satellite will be 1/3. Moreover, the coverage enhancement also needs to consider the limited EIRP. If the EIRP value can be enhanced, it would be possible to save the resources for the repetitions to cover the same coverage. Meanwhile, in NTN scenario, due to the LOS dominant environment and the limited link budget, it is not expected to consider multi-rank transmission even for the DL. Moreover, in case of NTN operation in S-band, due to the limited BW (e.g., 5MHz), in most case, it is expected that the different DL channels/signals for different UEs would not be FDMed in the same time. On the other hand, in case of higher frequency range (e.g., NR FR2-NTN), it would be necessary to consider the case where the different DL channels/signals for different UEs is FDMed in the same time. It was discussed that the gNB implementation can resolve the high PAPR issue by using the DPD (digital pre-distortion). However, in NTN, especially for transparent payload, it would be difficult for the GW or gNB to have the accurate PA model at the satellite. It will result the performance degradation on the DPD scheme.

On the gNB/UE complexity, considering that the gNB support the reception of UL signals/channels with DFT-s-OFDM in NR, the gNB will have IDFT logic. Similarly, since the UE supports DFT-s-OFDM for UL transmission, the UE will have DFT logic. Meanwhile, it is well known that the DFT or IDFT can be implemented by the IDFT or DFT together with the conjugate logic, respectively. To be specific, the DFT can be implemented by applying the conjugate function to both the input and the output of the IDFT. The IDFT can be implemented by applying the conjugate function to both the input and the output of the DFT. With this approach, DFT-s-OFDM can be supported for DL waveform without additional DFT logic at the transmitter side and the IDFT module at the receiver side. At the UE side, the channel equalization would be needed, but considering the LOS channel is dominant in NTN, it is expected that the low-complexity channel estimation still works properly even with the DL DFT-s-OFDM waveform.

Meanwhile, unlike UL transmission, it is allowed that a single gNB transmits multiple DL channels/signals associated with different UEs in the same time in a FDM manner. Moreover, the UE-dedicated channels/signals can be FDMed with the common channels/signals (e.g., SS/PBCH, Reference signals, PDCCH/PDSCH for SI/paging/random access). Considering that all the UE does not support the DFT transform precoding for the DL, it would be natural that the DFT transform precoding is not applied to the common channels/signals.

Figure 3-(a) shows that the CCDF of the PAPR for the case when the common channel (PBCH) and the PDSCH are FDMed. It is assumed that the waveform of the common channel is always CP-OFDM. On the other hand, the waveform of the PDSCH is either CP-OFDM or DFT-s-OFDM. For the analysis, it is assumed that the PDSCH occupies 40 PRB with the gap of 10 PRB from the PRBs occupied by the SSB. For the modulation scheme, QPSK is considered. It is observed that the DFT-s-OFDM for PDSCH can reduce the PAPR by more than 2 dB even though the CP-OFDM is used for the FDMed common channel.

It would be also possible to consider the case where more than two DL channels are FDMed in the same slot. In this case, there are two approaches: one is to apply the DFT for each DL channel, and the other is to jointly apply the DFT for combined DL channels. For the separated DFT transform precoding, for a single UE perspective, the UE will assume that the resources for DFT will be the same as the assigned PRBs for the DL channel. However, as the number of DL channels to be FDMed increases, the PAPR reduction would not be optimized. Alternatively, it would be possible that the gNB jointly applies the DFT transform precoding to the multiple DL transmissions.

Figure 3-(b) shows that the CCDF of the PAPR for the case when three PDSCHs are FDMed. For the reference, it is assumed that all the PDSCHs are CP-OFDM. For the analysis, it is assumed to compare the case where the DFT is separately applied to each PDSCH and the case where a single DFT is jointly applied to the combined PDSCHs. For the analysis, it is assumed that each PDSCH occupies 20PRBs and the gap between different PDSCHs is 10 PRB. All the PDSCH use QPSK for the modulation scheme. It is observed that the separate DFT can reduce the PAPR by 2 dB while the joint DFT can reduce the PAPR by 3 dB compared to the CP-OFDM case.

(a) Common channel and PDSCH are FDMed                              (b) 3 PDSCHs are FDMed

Figure 3: CCDF of PARP for DL DFT-s-OFDM

Meanwhile, in case of the joint DFT, since the resources to apply the DFT transform precoding and the assigned resources for a UE can be different, it would be necessary to carefully investigate whether or how the UE knows the resources that is applied with DFT transform precoding.

Alternatively, in case of the separated DFT, the resources to apply the DFT transform precoding will be the same as the assigned resources for the UE. In this case, there is no need to have the additional indicator to indicate the DFT transform precoding resources. Moreover, the UE can perform the channel estimation only for the assigned resources. The PAPR reduction would be relatively smaller compared to the joint DFT, but the expected specification change for supporting the separated DFT would be marginal.

Proposal 8: RAN1 captures followings as a conclusion for DL DFT-s-OFDM

- MRSS compatibility

  - DL DFT-s-OFDM can be TDMed and FDMed with NR DL CP-OFDM as if UL DFT-s-OFDM can coexist with UL CP-OFDM in NR.

- MIMO (SU and MU-MIMO) compatibility

  - In NTN scenario, since LOS paths are predominantly employed, the spatial multiplexing gain to be exploited for MIMO are limited.

- Multi-user multiplexing/scheduling flexibility

  - DL DFT-s-OFDM can support TDM and FDM for multi-user multiplexing.

    - If DFT transform precoding is separately applied to each FDMed DL channel/signal, the PAPR can be reduced compared to the reference.

    - If DFT transform precoding is jointly applied to multiple FDMed DL channels/signals, the significant PAPR reduction can be achieved at the expense of the increased UE complexity.

- Multiplexing/coexistence with baseline waveform

  - DL channel(s)/signal(s) with DL DFT-s-OFDM can be TDMed and FDMed with DL channel(s)/signal(s) with DL CP-OFDM. Even in this case, non-negligible PAPR reduction can be achieved.

- Transmitter/receiver complexity and impact power consumption

  - Transmitter complexity for DL DFT-s-OFDM is marginal since the DFT transform precoding can be implemented by IDFT for receiving UL DFT-s-OFDM and the conjugate function.

  - Receiver for DL DFT-s-OFDM may need the equalization process while the channel estimation complexity would be similar compared to the reference.

Proposal 9: DFT-s-OFDM waveform is supported as the additional basis for 6GR in downlink.

- DFT transform precoding for DL is available at least for a single UE-dedicated PDSCH.

  - FFS: Whether or how the DFT transform precoding is applied to the common DL channels (e.g., PDSCH containing common signaling, SS/PBCH).

  - FFS: Whether or how the DFT transform precoding is applied to multiple PDSCHs for multiple UEs.

    - Option 1: Before multiplexing, DFT transform precoding is applied to each DL signal or DL channel among all or a subset of DL signals or DL channels in a time, i.e., DFT size is the same as the number of assigned subcarriers for each DL signal/channel or each UE

    - Option 2: After multiplexing, DFT transform precoding can be applied to all or a subset of DL signals or DL channels in the same time, i.e., DFT size can be larger than the number of assigned resources for each DL signal/channel or each UE

  - FFS: Whether or how to support multiplexing with DMRS

    - Option 1: DFT transform precoding is applied to REs excluding the RE containing DMRS

    - Option 2: UE expects that the DMRS is always TDMed with the data RE at OFDM symbol level

In 6G NTN, the multi-orbit scenario can be considered as well. To be specific, since the service time of GNSO would be limited to few minutes, the service interruption could occur every satellite switching period. To mitigate this problem, it can be considered that the UE connect to both GSO and NGSO. In this case, GSO can be used for the service continuity while the NGSO can be used for the data throughput boost up. In this scenario, the propagation delay from the GSO would be highly different compared to the propagation delay from the NGSO. Moreover, it is still open whether or how to support the dual connectivity in 6GR. In those points of views, the longer CP would be useful for the inter-working between GSO and NGSO at the UE side.

Next, it was discussed that the NTN is operated on the TN band. In this scenario, it is necessary to carefully consider the interference management between NTN and TN. In this case, it would be useful to align the frame and symbol boundary between TN and NTN. Meanwhile, due to the very large altitude of the satellite, the differential RTT for a certain geographical area would quite different between TN and NTN. In this case, even though the DL/UL frame or symbol boundaries are aligned at a certain point of the cell, it is not guaranteed that the DL/UL frame or symbol boundaries are always aligned in the whole cell area as shown in Figure 4. To overcome the misalignment on the frame or symbol boundary between TN and NTN, it can be considered to use the suitable guard time or longer or extended CP for TN and NTN when the NTN coexist with TN in the same band.

Proposal 10: Longer CP or extended CP can be considered as the additional numerology for NTN scenario at least in semi-static manner.

Figure 4: Example of frame or symbol boundary misalignment between TN and NTN.

  - Initial access and mobility

In 6GR, considering at least network energy saving, it is considered to minimize the always-on common signals/channels and the always-on common signals/channels are confined within a certain duration of time with longer periodicity to support the longer deep sleep time. In the same time, RO (RACH occasion) would be confined within the burst as well.

In Rel-19 NR NTN, it was discussed that the active beam ratio for a satellite could be very limited (e.g., 1.5%). In this case, the duty cycle or the active time ratio of a single cell or beam would be 1.5% as well in average sense. Currently, even though the 6GR NTN evaluation assumptions including the maximum number of active beams or the maximum active beam ratio are not yet decided, the clustered common signal structure needs to consider NTN scenario especially for the periodicity and the active time duration.

Moreover, in NTN scenario, due to the large RTT, even for the PRACH transmission, it would be possible that the UE applies the common TA and the UE-specific TA. In this case, the time gap between the SS/PBCH and the corresponding RO needs to be a few tens or hundreds of msec. Considering the SIB1 and/or SIB-NTN reception, the time gap will be extended further in practice. If the large RTT in NTN is not considered for the RO design for the clustered common signal structure, the UE may need to wait the next period of the clustered common signal structure for PRACH transmission after the SS/PBCH detection in the current period, and the UE may need to maintain the DL synchronization until the PRACH transmission. In this case, the UE and the network may not go to sleep for energy saving. In case of PRACH reception, since a single satellite can serve a number of cells, a number of PRACH transmissions for different cells will arrive at the same satellite. In this situation, unlike TN, the strong signal can be an interferer to another cell as shown in Figure 5. In this case, it would be necessary to have the sufficiently separated RACH resources across different cells served by the same satellite.

Figure 5: Example of UL interference at the same satellite.

In NES perspective, it was considered that the active time for DL and active time for UL are aligned to avoid the case where the network needs to turn on the RF for UL reception even though the network does not have the DL transmission during the time. Meanwhile, in NTN, the DL frame and the UL frame will be aligned at the ULRP (UL synchronization reference point), and the ULRP could be a point between a satellite position and the gNB position. In this case, the alignment on active DL time and active UL time at the ULRP does not guarantee the alignment on active DL time and active UL time at the satellite and at the gNB. To be specific, the DL subframe boundary at the satellite is delayed from the DL subframe boundary at ULRP by the propagation delay while the UL subframe boundary at the satellite is prior from the UL subframe boundary at ULRP by the propagation delay due to the applied TA as shown in Figure 6.

If the active DL time and active UL time needs to be aligned at the satellite or at the gNB, it would be necessary to have separated (starting) offset would be needed for cell DTX pattern and cell DRX pattern in 6GR NTN.

Proposal 11: Study clustered common signal design and its periodicity considering NTN scenario including the large RTT and the limited active beam ratio.

Proposal 12: Study association between SS/PBCH and the corresponding PRACH resource/occasion considering the large RTT and the UL interference management.

Figure 6: Example of the alignment between active DL time and active UL time at ULRP.

In 6GR NTN, GNSS-less operation also needs to be carefully considered especially for the PRACH format and the RACH procedure. Considering that the initial access design will be defined at the very beginning of the 6GR study, it should consider the aspects for the GNSS less operation even though the study or the GNSS-less operation is not yet started.

In Rel-20 NR NTN, for the GNSS-resilient/less operation, it can be considered to employ the UE reference location for each cell or each beam-footprint. Meanwhile, the beam diameter considered in NTN is generally much larger compared to TN scenario. To be specific, in case of GEO scenario, the beam diameter of 250km is usually considered. On the other hand, the existing NR PRACH format is designed to support up to 100km of cell radius (200km of cell diameter). In this case, even though the concept of the UE reference location is used for TO/FO pre-compensation, the residual TO cannot be covered by the existing NR PRACH format due to the insufficient CP length and guard time. Therefore, in 6GR, the maximum target cell radius for the PRACH format design need to be at least 125km for GEO.

In case of FR2 and/or FR3, the higher SCS could be used for PRACH transmission to be robust with the CFO. Meanwhile, as the SCS increases, the CP length would be reduced. With the limited CP length of the PRACH format, the target cell radius could be limited as well. According to T38.821, in case of Ka-band, the beam diameter for GEO would be reduced into 110km, but the NR short PRACH cannot cover this cell area even with the UE reference location for GNSS less operation. In this case, it would be necessary to carefully investigate whether the GNSS less operation will be supported for initial access in FR2 and/or FR3 and the possibility of using the lower SCS (e.g., 1.25kHz) for PRACH transmission in FR2 and/or FR3.

In GNSS less operation, since the UE may not pre-compensate TO/FO sufficiently for the PRACH transmission, the PRACH preamble would need to be robust with the high (residual) TO and/or FO. Meanwhile, after applying the reference TO/FO pre-compensation, depending on the satellite location or the elevation angle, there is no case where both the residual TO and the residual FO are high. In other words, when the elevation angle is low, the residual TO could be high, but the residual FO could be small. On the other hand, when the elevation angle is around 90 degrees, the residual TO could be small, but the residual FO could be high. When the elevation angle is around 45 degrees or 60 degrees, the residual TO and the residual FO would be moderate. In this case, RAN1 does not need to focus on a single optimized PRACH format to be robust with both the high TO and the high FO simultaneously. Instead, it would be reasonable approach to allow the adaptation on the RACH configuration based on the time or the satellite position.

For the PRACH preamble design to be robust with the high (residual) FO, it would be necessary to consider PRACH format or preamble design for TN in the same time. For the TN and NTN harmonization, it can be considered to have a unified PRACH format for both TN and NTN scenarios. However, in TN scenario, since the gNB position would be stationary in general, the target FO would be much smaller compared to NTN scenario. In this case, if the PRACH format for NTN scenario is directly applied to the TN scenario as well, it will cause the unnecessary signaling overhead for PRACH transmission and the unnecessarily high NW and/or UE complexity operated in TN scenario. Meanwhile, if RAN1 try to have the totally separated PRACH format between TN and NTN, it will cause the high specification work load and the complicated UE implementation to support NTN feature.

To alleviate these problems, we propose to introduce the hierarchical PRACH format structure. To be specific, RAN1 can design PRACH preamble for the TN scenario. Next, RAN1 can design additional sequence for the inter-working with the TN preamble sequence. In TN scenario, the basic preamble will be used for the PRACH transmission. On the other hand, in NTN scenario, the additional preamble will be used for the coarse TO and/or FO estimation, and the basic preamble will be used for the fine TO and/or FO estimation. For the additional sequence design, it can be considered that the basic preamble sequence with different parameter (e.g., root index) or another sequence type (e.g., chirp sequence) having the robustness for the high TO and/or FO.

Proposal 13: Study PRACH format and RACH procedure for GNSS-less operation in NTN scenario including, e.g.,

- PRACH format design for GEO scenario (e.g., 250km of cell/beam diameter)

- Whether/how to support initial access procedure for FR2 and/or FR3

- Adaptation on RACH configuration

- Hierarchical PRACH format structure with a combination of TN preamble sequence and additional sequence

  - Coverage enhancement

In NTN scenario, due to the large pathloss, the coverage enhancement scheme would need to be enabled from the common signals/channels. Moreover, if the light indoor UE is supported on top of the outdoor UE for NTN, the necessity of the coverage enhancement would be high. In NR NTN, the coverage enhancement for a number of channels and signals are developed and considered. In case of the common channels and signals and the channels for the initial access, since there is no remaining bit or field to indicate the coverage enhancement enabled, it was considered that the existing bits or fields are re-interpreted. In this case, some system flexibility could be degraded. In case of SIB1 PDCCH/PDSCH repetition in Rel-19 NR NTN, it is necessary to consider the coexistence with the legacy UE. To be specific, since the legacy UE would not know the SIB1 PDCCH/PDSCH can be repeated, the repetition resource cannot be optimized at that time. In those points of views, in 6GR, it would be necessary to carefully design the coverage enhancement for common channels/signals and for the initial access procedure at the beginning of the SI and WI.

Next, if the repetition is enabled for the common channels including PDCCH, it would be necessary to revisit the RLF procedure. In NR, the UE compares the RS measurement results and the threshold, and the threshold is determined so that the expected BLER of the reference format of PDCCH can be achieved. Meanwhile, the reference format of PDCCH does not consider that the PDCCH can be repeated. Therefore, if the basic setting of the PDCCH is that the repetition is applied, the RLF also need to be updated accordingly.

Similarly, in case of CSI measurement, the UE and NW assume that the PDSCH transmission is not repeated. With the repetition, the NW may anticipate the SNR value, but the NW still does not know the BLER curve with respect to SNR at the UE side. In this case, it would be inaccurate to convert the CQI for the single PDSCH transmission into the CQI for the PDSCH repetition.

Proposal 14: Study repetition-native 6GR design including, e.g.,

- Enabling repetition for common channels/signals and for initial access procedure

- RLF/RRM management

- CSI measurement and reporting

  - Capacity enhancement

In NTN, it would be necessary to consider the active beam ratio of a satellite is limited. In this case, the active DL/UL time for each beam or cell could be limited as well. Meanwhile, the cell or beam coverage for NTN is much wider compared to TN. Considering that diverse device types support NTN operation, it would be expected that the number of active UEs for a beam or cell could be further increased. In those situations, it would be important to efficiently manage the UL resources.

Meanwhile, in NTN scenario, it would be inefficient or difficult to employ the MU-MIMO due to the CSI aging and the difficulty on separating each UL signals/channels from the composite signal via the feeder link. In Rel-19 NR NTN, it was introduced that the capacity enhancement by using the OCC (orthogonal cover code). In this case, it would be possible that the different UL transmissions share the same time-and-frequency resources. In 6GR NTN, it would be necessary to consider that the capacity enhancement scheme at the beginning of the SI and WI.

Proposal 15: Study UL transmissions sharing the same time-and-frequency resources.

  - DL control channel, scheduling and HARQ operation

Due to the large RTT in NTN scenario, it would be necessary to increase the maximum number of HARQ processes. However, due to the limited soft buffer at the UE side, there is a restriction on the possible value for the number of HARQ processes. In this situation, in NR NTN, it was considered the possibility of disabling HARQ feedback per HARQ process. In this case, for the HARQ process with disabled HARQ feedback, the UE would not generate the acknowledgement(s) of the received TB. Meanwhile, the UE stores the received data when the UE fails to decode the received data. Therefore, if the PDSCH transmissions are conservatively scheduled with sufficiently small TBS or low code rate, it would be possible to increase the maximum number of HARQ process further even with the limited soft buffer size at the UE. In 6GR NTN, it would be necessary to consider more flexible management on the HARQ feedback disabling.

Proposal 16: Study UE procedure for DL/UL transmission with disabled HARQ feedback including HARQ combining, retransmission for DL transmission with disabled HARQ feedback.

  - CSI acquisition and reporting

Due to the large RTT, the CSI aging would be a critical problem in NTN scenario. To be specific, the time gap between the CSI reference resource and the PDSCH resource scheduled by the NW based on the CSI reporting associated with the CSI reference resource would be considerably large. Moreover, during the time duration, the satellite position could be changed accordingly, and the level of the pathloss would be changed as well. To mitigate this inefficient, the UE may need to consider the expected pathloss variations based on the ephemeris information to derive the CSI and/or CQI. For instance, if the CSI reference resource is in a slot n, the UE may calculate the CQI based on the expected RSRP in a slot m where the PDSCH scheduling is expected.

Next, as mentioned before, if the PDSCH repetition is widely used in NTN, the CSI and/or CQI would need to be updated accordingly. To be specific, in 6GR, it can be considered that the reference format of the PDSCH to derive CSI can be a form of PDSCH repetition.

Considering the large RTT and the limited soft buffer at the UE side, it can be considered to employ the disabled HARQ feedback. In this case, the UE may not expect to receive the retransmissions of the TB with the disabled HARQ feedback. Therefore, the target BLER would be different depending on whether the HARQ feedback is enabled or disabled. In the NW point of view, since the NW does not know the BLER curve with respect to SNR at the UE side, it would be good to know the separated CQI value for the case where the HARQ feedback is disabled.

In general, the NTN operation targets outdoor UE with the LOS path. Meanwhile, due to the UE moving, the UE can lose the LOS path, then the UE can continuously report the CQI#0 (out of range) with respect to the given reference format of PDSCH for a while. In this case, rather than reporting CQI#0, it would be better to send more valuable information. For instance, if UE reports the expected BLER or the expected number of (re)transmissions) for a TB, the NW can manage the retransmission resources efficiently.

Proposal 17: Study CSI acquisition and reporting for NTN operation considering

- CSI aging due to the large RTT

- CSI/CQI for a TB with the disabled HARQ feedback

- UE temporally loses the LOS path, and continuously reports CQI#0 (out of range)

Conclusion

In this contribution, we discussed NTN specific requirement and design especially for GNSS based operation. Based on the above discussion, our observations and proposals are given as follows:

Proposal 1: RAN1 strives to design unified solution for both GNSS-based and GNSS-less operation.

Proposal 2: Common TA and UE-specific TA as defined in 5G NR NTN are supported as the basis for 6GR NTN.

- Enhancement/modification on UE-specific TA at least for GNSS-less/resilient operation will be studied as potential additions.

Proposal 3: K_offset and k_mac as defined in 5G NR NTN are supported as the basis for 6GR NTN.

- Enhancement/modification on K_offset and its signaling will be studied as potential additions based on the satellite location and UE location.

Proposal 4: TA report as defined in 5G NR NTN is supported as the basis for 6GR NTN.

- Enhancement/modification on TA report and its triggering condition will be studied as potential additions.

Proposal 5: Ephemeris information as defined in 5G NR NTN is supported as the basis for 6GR NTN.

- Enhancement/modification on SIB-NTN will be studied as potential additions.

- Enhancement/modification on UL timing adjustment procedure based on SIB-NTN will be studied as potential additions.

Proposal 6: Study MRSS (multi-RAT spectrum sharing) for NTN operation.

- FFS: Whether/how to share NTN configuration and beam management between different RAT.

Proposal 7: Study UE procedure based on UE position and NTN node position including, e.g.,

- Neighbor cell measurement management

- Higher layer parameter or configuration adaptation

- Fast beam management

Proposal 8: RAN1 captures followings as a conclusion for DL DFT-s-OFDM

- MRSS compatibility

  - DL DFT-s-OFDM can be TDMed and FDMed with NR DL CP-OFDM as if UL DFT-s-OFDM can coexist with UL CP-OFDM in NR.

- MIMO (SU and MU-MIMO) compatibility

  - In NTN scenario, since LOS paths are predominantly employed, the spatial multiplexing gain to be exploited for MIMO are limited.

- Multi-user multiplexing/scheduling flexibility

  - DL DFT-s-OFDM can support TDM and FDM for multi-user multiplexing.

    - If DFT transform precoding is separately applied to each FDMed DL channel/signal, the PAPR can be reduced compared to the reference.

    - If DFT transform precoding is jointly applied to multiple FDMed DL channels/signals, the significant PAPR reduction can be achieved at the expense of the increased UE complexity.

- Multiplexing/coexistence with baseline waveform

  - DL channel(s)/signal(s) with DL DFT-s-OFDM can be TDMed and FDMed with DL channel(s)/signal(s) with DL CP-OFDM. Even in this case, non-negligible PAPR reduction can be achieved.

- Transmitter/receiver complexity and impact power consumption

  - Transmitter complexity for DL DFT-s-OFDM is marginal since the DFT transform precoding can be implemented by IDFT for receiving UL DFT-s-OFDM and the conjugate function.

  - Receiver for DL DFT-s-OFDM may need the equalization process while the channel estimation complexity would be similar compared to the reference.

Proposal 9: DFT-s-OFDM waveform is supported as the additional basis for 6GR in downlink.

- DFT transform precoding for DL is available at least for a single UE-dedicated PDSCH.

  - FFS: Whether or how the DFT transform precoding is applied to the common DL channels (e.g., PDSCH containing common signaling, SS/PBCH).

  - FFS: Whether or how the DFT transform precoding is applied to multiple PDSCHs for multiple UEs.

    - Option 1: Before multiplexing, DFT transform precoding is applied to each DL signal or DL channel among all or a subset of DL signals or DL channels in a time, i.e., DFT size is the same as the number of assigned subcarriers for each DL signal/channel or each UE

    - Option 2: After multiplexing, DFT transform precoding can be applied to all or a subset of DL signals or DL channels in the same time, i.e., DFT size can be larger than the number of assigned resources for each DL signal/channel or each UE

  - FFS: Whether or how to support multiplexing with DMRS

    - Option 1: DFT transform precoding is applied to REs excluding the RE containing DMRS

    - Option 2: UE expects that the DMRS is always TDMed with the data RE at OFDM symbol level

Proposal 10: Longer CP or extended CP can be considered as the additional numerology for NTN scenario at least in semi-static manner.

Proposal 11: Study clustered common signal design and its periodicity considering NTN scenario including the large RTT and the limited active beam ratio.

Proposal 12: Study association between SS/PBCH and the corresponding PRACH resource/occasion considering the large RTT and the UL interference management.

Proposal 13: Study PRACH format and RACH procedure for GNSS-less operation in NTN scenario including, e.g.,

- PRACH format design for GEO scenario (e.g., 250km of cell/beam diameter)

- Whether/how to support initial access procedure for FR2 and/or FR3

- Adaptation on RACH configuration

- Hierarchical PRACH format structure with a combination of TN preamble sequence and additional sequence

Proposal 14: Study repetition-native 6GR design including, e.g.,

- Enabling repetition for common channels/signals and for initial access procedure

- RLF/RRM management

- CSI measurement and reporting

Proposal 15: Study UL transmissions sharing the same time-and-frequency resources.

Proposal 16: Study UE procedure for DL/UL transmission with disabled HARQ feedback including HARQ combining, retransmission for DL transmission with disabled HARQ feedback.

Proposal 17: Study CSI acquisition and reporting for NTN operation considering

- CSI aging due to the large RTT

- CSI/CQI for a TB with the disabled HARQ feedback

- UE temporally loses the LOS path, and continuously reports CQI#0 (out of range)

Reference

- “RAN1 Chair’s Notes”, RAN1#122bis, October 13th – 17th, 2025.